IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | TG3 PHY: Channel plan for high-rate WPAN |  |
| Date Submitted | [16 January, 2001] |  |
| Source | [Stanley K. Ling] [Intel] [9750 Goethe Road, Sacramento CA 95827] | Voice: [916-855-5000-X2398] Fax: [916-854-2809] E-mail: [stan.k.ling@intel.com] |
| Re: | [] |  |
| Abstract | [This contribution proposes a transmit spectrum mask and channel plan for 802.15.3 WPAN] |  |
| Purpose | [To propose a channel plan, transmit spectrum mask, and roll-off factor for 802.15.3 high-rate PHY.] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

## Baseband Filter Characteristics

A square-root raised cosine pulse shaping filter, p(t) is used to transmit the in-phase and quadrature-phase symbols. The impulse response of the pulse-shaping filter is given as follows

p(t)=4αtπT⋅cos1+α⋅πtT+T4αt⋅sin1−α⋅πtT1−4αtT2,

where  and 1T are the spectral roll-off factor and symbol rate, respectively.

Spectral Roll-off Factor

The spectral roll-off factor, , shall be equal to 0.30.

## Transmit Spectrum Mask

The transmit spectrum mask is shown in Figure 1.

Figure 1: Transmit spectrum mask

## Operating Frequency Range

The PHY shall operate in the 2.4 – 2.4835 GHz frequency range.

## Number of Operating Channels

The resolution of the channel center frequency is 1 MHz. In the US, 61 center frequencies are defined. The channel numbers are defined as 1, …, 61, as listed in Table 1. The center frequencies are fc = 2407 MHz + i*1 MHz, where i is the channel number. In a multiple piconet topology, adjacent channels can operate simultaneously if the distance between center frequencies is ≥20 MHz.

Table 1: Operating Channel Center Frequencies (MHz)

| Channel # | Value | Channel # | Value | Channel # | Value |
| --- | --- | --- | --- | --- | --- |
| 1 | 2408 | 21 | 2428 | 41 | 2448 |
| 2 | 2409 | 22 | 2429 | 42 | 2449 |
| 3 | 2410 | 23 | 2430 | 43 | 2450 |
| 4 | 2411 | 24 | 2431 | 44 | 2451 |
| 5 | 2412 | 25 | 2432 | 45 | 2452 |
| 6 | 2413 | 26 | 2433 | 46 | 2453 |
| 7 | 2414 | 27 | 2434 | 47 | 2454 |
| 8 | 2415 | 28 | 2435 | 48 | 2455 |
| 9 | 2416 | 29 | 2436 | 49 | 2456 |
| 10 | 2417 | 30 | 2437 | 50 | 2457 |
| 11 | 2418 | 31 | 2438 | 51 | 2458 |
| 12 | 2419 | 32 | 2439 | 52 | 2459 |
| 13 | 2420 | 33 | 2440 | 53 | 2460 |
| 14 | 2421 | 34 | 2441 | 54 | 2461 |
| 15 | 2422 | 35 | 2442 | 55 | 2462 |
| 16 | 2423 | 36 | 2443 | 56 | 2463 |
| 17 | 2424 | 37 | 2444 | 57 | 2464 |
| 18 | 2425 | 38 | 2445 | 58 | 2465 |
| 19 | 2426 | 39 | 2446 | 59 | 2466 |
| 20 | 2427 | 40 | 2447 | 60 | 2467 |
|  |  |  |  | 61 | 2468 |

Operating Channels (IEEE 802.11-coexistence mode)

The North American operating channels in the IEEE 802.11-coexistence mode are defined in Table 2. (Note: TG3 will decide between Option 1 and Option 2. Option 1 utilizes the same center frequencies of the 802.11 channel plan. Option 2 utilizes the same center frequencies as the high user-density mode for the two outer channels, and the same center frequency as the 802.11 channel plan for the middle channel. The maroon dotted lines in Figure 2 and Figure 3 denote the 802.11 North American channel plan.)

Table 2: North American operating channels (IEEE 802.11-coexistence mode)

| Option 1 |  | Option 2 |  |
| --- | --- | --- | --- |
| Channel ID | Center Frequency (MHz) | Channel ID | Center Frequency (MHz) |
| 5 | 2412 | 1 | 2408 |
| 30 | 2437 | 30 | 2437 |
| 55 | 2462 | 61 | 2468 |

Figure 2: North American channel plan (IEEE 802.11-coexistence mode (Option 1))

Figure 3: North American channel plan (IEEE 802.11-coexistence mode (Option 2))

Operating Channels (high user-density mode)

The North American operating channels in the high user-density mode are defined in Table 3.

Table 3: North American operating channels (high user-density mode)

| Channel ID | Center Frequency (MHz) |
| --- | --- |
| 1 | 2408 |
| 21 | 2428 |
| 41 | 2448 |
| 61 | 2468 |

Figure 4: North American channel plan (high user-density mode)