IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | PHY Amendments |  |
| Date Submitted | [14 May, 2001] |  |
| Source | [James P. K. Gilb] [Mobilian] [11031 Via Frontera, Suite C] | Voice: [858-217-2201] Fax: [858-217-2301] E-mail: [gilb@ieee.org ] |
| Re: | [] |  |
| Abstract | [Amendments to the PHY layer of 00000d02P802.15.3__Draft-Standard.pdf] |  |
| Purpose | [To record the amendments to the PHY layer for the purpose of modifying the baseline draft standard.] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

- Change temperature from TBD to 0 to 40 C ambient – DONE 3/27/01

- Correct taps in figure 44 (they are reversed) – DONE 3/14/01

- Note that if data rate x is supported all rates less than x shall be supported. – DONE 3/27/01

- Fix OQPSK flush symbols (needs to be four bits, 0101 sent 0 first over the air).  – DONE 3/27/01

- Add OQPSK bit-stream to di-bit mapping – To go into d05, add to issues list

- Add note that CAZAC QPSK/OQPSK is under consideration. – DONE 3/27/01

- Add data scrambler, side-stream or frame synchronous scrambler with a polynomial x^15+x^14+1. – Waiting for description, should go in d05

- Modulation accuracy per document P802.15.3-01/090r2 –DONE 5/7/01

- Change to 2048 total byte packets – Will go into d05 with note about reducing packet size for bad channels

- Mobility specification, i.e. a maximum relative speed TBD m/s.  We need an analysis of the change in the enviroment during a packet and application (John Barr will lead this). – Waiting for input, add to issues list

- Change MIB to PIB.  – DONE 3/27/01

- Consider synchronizing carrier frequency to symbol clock (TBD). – On issues list, waiting for sample definition and test criteria

- Add channel switch time of 224 us. – DONE 3/27/01

- Add frame format to overview from P802.15.3-01/030r0 – Should go into d05

- Add description of transmit modes (e.g. delayed ack) to overview clause (Raju Gubbi). – Some description in d04, check d04 and make suggestions