IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Running PHY Con Call Minutes-Hilton Head to Orlando |  |
| Date Submitted | 12May2001 |  |
| Source | James D. Allen Eastman Kodak Company 4545 East River Road Rochester, NY 14650-0898 | Voice: [(716)-781-9025 ] Fax: [ -- ] E-mail: james.d.allen@kodak.com |
| Re: | Conference Call Minutes |  |
| Abstract | These running minutes capture all of the PHY conference calls from March 29th to May 10. These minutes are submitted for approval at the next meeting (May 12-18). |  |
| Purpose | To document the actions, decisions and discussions of the 802.15.3 PHY sub committee |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

March 29, 2001

ATTENDEES:

TK Tan

James Gilb

Jeyhan Karaoguz

Rick Alfvin

Jim Allen (Secretary)

Called to order at 1:15PM EST.

Agenda:

- Roll Call

- Old business

- Review Hilton Head decisions

- Action items

- Scrambler definition

- Timing synchronization

- Scramber initialization

- IFS'es time

- New business

- Adjourn

Gilb suggested a reference for Karaoguz about the scrambler lengths - Need an estimate next week.

Jeyhan took that action to work with Gubbi on max header length and whether it needs scrambling.

We still need to assign and define how long we need to listen for the CCA.  This will go out on an email request.

Karaoguz found from modeling the CAZAC sequence on OQPSK has a problem and will send a paper out explaining it.  We'll add this discussion to next week's agenda.

Adjourned 1:28PM

April 12, 2001

ATTENDEES:

Jehyan Karaoguz

James Allen

Roberts Roberts

James Gilb

Called to order by Gilb  at 1:12 DST).

The agenda is as follows

_ Roll Call

- Old business

- Action items

- Scrambler definition - ??

- Timing synchronization - S. Ling

- Modulation accuracy - New person?

- PHY SAP issues - any problems with these?

PHY-DATA.request

PHY-DATA.indication

PHY-DATA.confirm

PHY-TXSTART.request

PHY-TXSTART.confirm

PHY-TXEND.request

PHY-TXEND.confirm

PHY-CCARESET.request

PHY-CCARESET.request

PHY-CCA.indication

PHY-RXSTART.indication

PHY-RXEND.indication

- Scramber initialization

- IFS'es time

- CAZAC non-orthogonality with OQPSK - Jeyhan

- New business

- Adjourn

Scrambler -  Jeyhan mentioned it was adopted as proposed by Karaoguz and Schmidl.  He will take a section out of a different document and submit it.  It will be based on the three term format, such as  X^15+X^14+1.  Gilb said that a reference recommended by Stevenson has some suggestions and it was not this one.   It was agreed that they would pick one and propose it.

Timing Sync.  – Stan is not available so we’ll review this later.

Modulation Accuracy – Did Wayne get the document done and where is it. Jehyan said it was on the web page, but Gilb is looking for the Framemaker source.  Jeyhan will take the action to try and locate the file.

A discussion about  PHYSAP was lead by Roberts.

Section 6.7 – the intent is to go over questions to the document and the comments by Rick which are in color.

Roberts - is OK with leaving the TX vector the way it is.  Does any one have a suggestion?  Gilb thought that Gubbi was working to a fixed header length so should it be hard coded.  Jeyhan said it was 14 Bytes with 2B of CRC (no FEC).   We will add MAC header to the TX vector.

PHY  TXEnd – Rick had no problem with this but questioned if PHY TXReset  was needed.  Gilb said that was done in .11 to reset timers.   There may be no way to exit CCA without at least a start and stop command.   CCA reset signals also need to be resolved.  Gilb put all the CCA issues on hold until the CCA is defined.  Gilb will take the action to make sure it is on his “Issues” list.

PHY RXStart – Gilb asked is this is missing a “start request” which otherwise assumes the  receiver is on.  Should it be covered in the PLME? It’s not in there and should be in the PHYSAP not SME.  Gilb suggested we add RXStart request and confirm to tell the rcvr to look for valid packets and send a “confirm“ when it receives a valid packet and request the MPDU be received.  The PHY passes all data up and only checks the header and CRC.  There might not be an associated parameter, unless it some antenna issue.

Document 01186r2 added a nap node.    Send RXEnd request, then if the MAC decides that the PHY can sleep or wake, it could send the appropriate command.  There may be several sleep modes could be communicated.  The state will be implementation dependently but the command will exist.   Currently there is no way to flag on power management issues.

Scrambler initialization – since MAC header is 14 bytes.  The header is the CAZAC sequence , and things we should add another field for scrambler initialization.  At 55 Mbps plus 5 bits can tell the PHY the data rate.  How do we initialize it?   With a seed of 8 bits which is the seed of the scrambler.  One byte is wasted in this method.so it is a little more complicated but not a problem.  Alternative methods were discussed.  The solution has to work regardless of peer to peer or under master control.

IFS’es time – still need to define parameters so we will postpone this discussion.

CAZAC non-orthogonality with OQPSK – Jeyhan let the discussion.  There is still a problem using CAZAC in OQPSK does not work.  Jeyhan can run the autocorrelation to make sure things are double checked.  Jeyhan is working on this issue.

New business – additional PHY SAP or PLME issues to cover more ground.

Section 6.4.  -  Rick believes it is pretty straightforward.  But what is the PLME reset.   Normally the vendor defines it.  It is off at initialization.    The entire issue of resets is also being discussed.  Gilb will move the PL or ML reset from section 6.2 to their respective  PLME and MLME sections.

Roberts – is there a series of test (Loop Back)  modes that we should include in this too, like in 802.15.1.  In dot 1 it is done higher layer, not the PHY.   We’ll leave the test mode open and loop back modes will be a vendor specific “value-add”.

Gilb – was thinking of getting rid of the references to multiple data rate sets in the PHY.  Roberts has also had the same discussion because if you support the basic rate, the rest does not need to be specified.  It will be deleted.

Gilb also suggested that the PHY PIB min max step to PHY PIB power level vector.

Allen reminded attendees to vote on LB9 and LB10.

April 26, 2001

Attendees: James Gilb, Stan Ling, Susumu Hara, Jeyhan Karaoguz, Rick Roberts, Jim Allen

The next PHY conference call will be held on April 26, 2001 at 10 am PDT

for 1.5 hours.  The call in information is as follows:

1-888-385-5669 (US)

1-816-650-0602 (outside US)

Call ID: 60809

The call chair will be James Gilb

The agenda is as follows

- Roll Call

- Old business

- Action items

- Latest issues list should be out (01/122r3) - JPKG

- Add CCA definition to issues list - JPKG

- IFS'es: need definition - JPKG

- LQI (link quality indication) and CCA definition

- PHY SAP issues - any problems with these?

**** PHY MIB ... part 2 (page 11 of Doc 01186r0) ****

PhyMib_NumChannelsSupported

PhyMib_CurrentChannel

PhyMib_RSSI_max

PhyMib_RSSI_min

PhyMib_RSSI_StepSize

PhyMib_MPDU_LengthMax

PhyMib_CCAModesSupported

PhyMib_CurrentCCAMode

PhyMib_CCA_Threshold

**** PLME ... (page 8 of Doc 01186r0) ****

PLME-GET.request

PLME-GET.confirm

PLME-SET.request

PLME-SET.confirm

PLME-RESET.request

PLME-TESTMODE.request

PLME-TESTOUTPUT.request

- Jamming resistance

- New business

- Adjourn

Minutes:

Roberts called in to report that he is unable to attend today, layer management stuff will be put off until the call next week.

Gilb called the meeting to order 10:08

There will be an additional conference call on May 3, 2001 at 10 am PDT.

Action Item (A.I.)- Gilb will send announcement today for next meeting.

Gilb announced another conference call next week on Thursday, May 3rd.

The Issues List (document 01/122r3) has been OK'ed but is not on the web site yet, so this action item will be discussed next week.

Action- Gilb :  Add CCA definition to Issues list.

Action - Gilb: Add IFS to issues list.

Gilb asked if anyone knew what the 802.11 interframe spacing requirement is.  Ours is defined to be the Tx to Rcv turn around time as defined by Gubbi .  It  is also defined as the difference between a  frame and its ACK.   This is 10 usecs , and is the same as .11's.    Tx to Rcv will be the same duration as Rx Tx even though .11 uses different times for each.    Ling mentioned that Rcv to Tx is more difficult due to latencies in decoding, MAC calculation delays and other things that have to happen.   The 10 usec. includes the sum of the  MAC and PHY.  The PHY turn around time won't be spec'ed individually but it is on the order of 2 usec.

The Clear channel detect slot plus IFS in 802.11b (section 18.4.8.4) is10 for turn around and 15 usec energy detect for a total slot of 25 usec.  If noise is detected during the IFS, then back-off and wait.  If nothing is heard, the xmtr would turn around and start transmitting. We would like the Rcv-Tx and Tx-Rcv times to be symmetrical.

LQI needs to be a relative measure, not a magnitude level.

Action: -Allen to find out what Kodak proposed last year for MFSK.

Should LQI be a power level in a valid packet.  Jeyhan suggested we send along a Mean Square Error measurement to indicate the noise energy levels, but this mandates a digital implementation.  Jeyhan will suggest a way to measure the error rate to control the link.  This should be an average number and may be an optional feature with reserved hooks.   It may also include a valid packet indication.

Action: -Jehyan to propose an LQI method.

In channel power sense measurement increments will be 3db + 1.5db for 16 steps and over all accuracy of + 10 overall.  The radio has no sense of absolute power rating so keep in mind this adds to production line calibration.  Allen suggested we start at the terminated input Rcvr noise level an work up.  Ling suggested we look at some other standards like .11 and cell phones and Gilb will look at Bluetooth.

Action: Ling - Look at .11 and cell phone channel assessment methods and report back next week.

Action: Gilb - Look at BT channel assessment methods and report back next week.

Jamming Resistance: S. Ling presented this a few times already.  This was a discussion to figure out where to go from here.   Gilb relax the spec. in the raft.  There are no new numbers and Ling will look into see if any on at Intel has comments.  This will be discussed again at the next meeting.

New Business:

Ling indicated that the channel plan in the standard is not the same as the one he had proposed.   The one written in the draft std only 5 channels.  Ling's allowed channels every MHz but only recognized the 5 channels that are allowed.  There was a discussion about how to allow only certain channels, vs too may defined channels which cause long acquisition times.  We will consider numbering all 83 one MHz steps and only allowing use of specific ones.  This way the receiver can step in 1 MHz increments if necessary.  Still need to figure out why we should do this extra work.

OQPSK - Stan wants to discuss this at the next meeting.  A doc number will be assigned for that discussion.

The frame is currently not able to communicate what the receiver thinks the link quality is.  Should this be addressed?  We'll discuss it after the above issues are resolved.

Adjourned at 2:03 PM EDT

May 3, 2001

Attendees: James Gilb, Jeyhan Karaoguz, Rick Roberts, Jim Allen, Stan Ling, Susumu Hara

Chair: James Gilb

Secretary: James Gilb

Agenda: (from email)

- Roll Call

- Old business

- Action Items

- Issues list (01/122r3) - JPKG

- Add CCA definition to issues list - JPKG

- IFS’es: need definition - JPKG

- Discuss Stan’s document, 01/225r0

- PHY SAP issues - any problems with these?

**** PHY MIB ... part 2 (page 11 of Doc 01186r0) ****

PhyMib_NumChannelsSupported

PhyMib_CurrentChannel

PhyMib_RSSI_max

PhyMib_RSSI_min

PhyMib_RSSI_StepSize

PhyMib_MPDU_LengthMax

PhyMib_CCAModesSupported

PhyMib_CurrentCCAMode

PhyMib_CCA_Threshold

**** PLME ... (page 8 of Doc 01186r0) ****

PLME-GET.request

PLME-GET.confirm

PLME-SET.request

PLME-SET.confirm

PLME-RESET.request

PLME-TESTMODE.request

PLME-TESTOUTPUT.request

- LQI (link quality indication) and CCA definition

- Jamming resistance

- New business

- Adjourn

Call started at 10:08 PDT

A.I. = Action Item.

A.I. Header coding/HCS: need to find the person that this is assigned to.

With times 2 repetition, how do you correct for one error, alternate times two is to repeat the header twice completely.  Does the times 2 even provide gain?  Jeyhan says that the times 2 provides no gain.  Jeyhan thinks the twice header repetition will not provide much coding gain.  There is general agreement that BPSK provide 3 dB gain.

A. I. James will send a proposal for interleaving the headers to the reflector.

A.I. Jeyhan email John Barr with document numbers and titles and James will suggest a presentation time.  This includes LQI and OQPSK CAZAC.

A. I. Rick Roberts to work on the service field and determine what might go into it, James suggested it might be used for scrambler synchronization, modulation rate determination, clock/carrier synchronization and future use.

Maximum packet length, for 10-5 2048 bytes is 15% which is too high, for 10-6 it is 1.62 % PER.  Jeyhan noted that there is a wide variation in the SNR packet to packet.

Adaptive channel assignments: A.I. James will email Mohammed to ask him if we can drop the adaptive channel assignments item.

Stan Ling discussed 01/225r0, conclusions in the document, Jeyhan pointed out that the equalizer for OQPSK is slightly different.  James suggested to present the document and ask for a vote at the meeting.  James wants to think about it one more week.

Rick Roberts noted that we had finished the PLME in the previous call.  We will now do the PHY-PIB

Rick suggested changing the min/max/step to vectors (i.e. a list of discrete elements, clarification to Jim Allen).  Jim Allen suggested allowing slower data rates than in the standard.

A.I. Jim Allen to contact TG4 to ask what data rates they may want in the PIB.  Jeyhan suggested that a BPSK 11 Mb/s mode might be useful.  (ed. note 5/8:  TG4 has not decided that to use and the chair has no way to comment yet.)

Roberts: How do we enumerate regulatory domains?  James said that 15.3 has only one regulatory design.

Roberts: Do we need CCA mode?  James Gilb suggested reviewing it, Jim Allen suggested leaving it in for now with a comment that it might be removed later.

Roberts: Ranging support group - was added to the criteria document, Rick suggested putting the items in based on suggestions from his group.  Jim Allen said that perhaps he could use ranging from other methods, and putting in the PIB would help that.

Adjourned at 11:26 PDT

May 10, 2001

PHY conference call, May 10, 2001

Attendees: James Gilb, Jeyhan Karaoguz, Stan Ling, Susumu Hara, and Keith Holt

Chair: James Gilb

Secretary: James Gilb

Agenda: (from email)

- Roll Call

- Old business

- Action items

- Issues list should be out (01/122r3) - JPKG

- Add CCA definition to issues list - JPKG

- IFS’es: need definition - JPKG

- LQI (link quality indication) and CCA definition

- Jamming resistance

- TBD’s in spec

- Items to look at in latest draft

- New business

- Adjourn

Call started at 10:10 am PDT

Jamming margin: Stan discussed his analysis of the requirements.  Our current requirements appear to be more stringent than 802.11a.  Suggestion to reduce alternate channel requirements to
(O)QPSK	30 to 26		27 dB (18 Mb/s 802.11a)

16 QAM 	26 to 22

32 QAM	23 to 19

64 QAM	20 to 16 dB		15 dB (54 Mb/s 802.11a)

Suggest 5 dB for signal level above desired signal.  The problem was the linearity requirement.  The biggest alternate channel blocker was -68 dBm +10 dB + 16 dB = -42 dBm.  The consensus is to wait for some more data, but the number will be between 3 dB and 6 dB.

Adjacent channel: current requirements

O) QPSK	10

16 QAM 	6

32 QAM	3

64 QAM	0

The feeling is that these numbers are fine for now.

TX modulation accuracy: Stan is doing studies on the modulation accuracy.  Our specification is much more difficult than 802.11a.  Jeyhan stated that the current requirements are based on a calculation that has a 0.5 dB degradation in the TX SNR.  Reducing it will increase the degradation.  Consensus is to do more study, but be aware that we are more difficult than .11a.  James said that .11a vendors are having difficulty meeting the 64-QAM TX specification.

LQI: Jeyhan said that we will have RSSI, also suggested a mean square error using a digital low pass filter.  We need to specify the LPF characteristic in the standard.  James suggested that this would be an optional feature.  Stan wondered if reporting the MSE as dB was best.  Jeyhan said that a linear scale would take at most 8 bits.  Stan wondered about the RSSI possibility?  James said we are going to do it, but it is not defined yet.  Stan and Susumu looked at relative vs. absolute and suggested relative measurement.  Stan suggested that the 802.11 is fine.  Jeyhan wondered if it was relative to sensitivity, Stan indicated that it is vendor defined.

A. I. James to put 802.11 RSSI on agenda for call after Orlando, May 31 is tentatively the next call.

Other TBD’s in standard that we can discuss today.

Header coding: Jeyhan agrees that there is 1 dB gain from BPSK vs. interleaving 2 headers.  Jeyhan looked at (10,15) Hamming code, but only came up with 2.8 dB.  For twice repetition, QPSK becomes BPSK anyway.

Stan asked about a possible 11 Mb/s mode, could we still receive the CAZAC at the same sensitivity as the rest of the packet.  Straight QPSK requires 12.6 dB SNR (1e-5), but detection works as low as 7-8 dB SNR.  So this should work well even with BPSK payloads.

A. I. Jeyhan will see if there is a BPSK CAZAC and the implications.

Stan asked about section 11.2.6.3, TR/RX turnaround time.  The current draft has RX to TX of > 10 and less than 11 us.  Stan wondered if this is long enough time (11 symbols) for turnaround.  The other alternative is that we just specify the time with no upper and lower limits.  The entire CAZAC is 14 us.

Stan asked what the implication is of James not being there next week.  The implication is that there should be little PHY work done there.

Meeting adjourned at 10:59 am PDT.