IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15.3 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | TG3 System Subcommittee SAP and PIB Living List |  |
| Date Originated | 25 March 2001 |  |
| Source | Rick Roberts XtremeSpectum, Inc. 8133 Leesburg Pike, Ste. 700 Vienna, Va. 22182 | Voice: 703.749.0230 Fax: 703.749.0249 E-mail: rroberts@xtremespectrum.com |
| Re: | Revision 1 (in-progress) |  |
| Abstract | This document represents the working document for the System Subcommittee. |  |
| Purpose | Working Document |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Subsystem Committee SAP and PIB Living List

This document tracks primitives and parameters that are being passed over the SAPs.  It also tracks items that are being stored in the PIBs.

Notation Key

Blue Reflects the Text of Clause 6.0 going into the Hilton Head Meeting

Red Reflects additional text added due to the Hilton Head Meeting

Green Reflects text added between the Hilton Head and Orlando Meeting

Text that is double strikethrough has been deleted

PHY Committee Issues List

| Item | Issue |
| --- | --- |
| PLME-RESET.request() | Roberts … what is the default |
| PLME-TESTMODE.request | Roberts … Missing DATA_TYPE parameter, add it |
| PLME-TESTMODE.request | Roberts … should we have loop back testing similar to 802.15.1? |
| Add PLME Power Management | Roberts … need to add PLME power management? |
| PhyPib_RegDomainsSupported | Roberts … how do we numerate these? |
| PhyPib_MaxDataRate | Roberts … delete, unnecessary (see note below) |
| PhyPib_MinDataRate | Roberts … delete, unnecessary (see note below) |
| PhyPib_DataRateStepSize | Roberts … replace with PhyPib_DataRateVector. Index into vector returns a data rate. |
| PhyPib_CurrentDataRate | Roberts … Should this be reported instead in MAC PIB? Can this report be done on a per packet basis. Is this the only access point for the SME to determine/set data rate for scalable data rate (i.e. to combat poor channel conditions)? |
| PhyPib_CurrentPowerLevel | Roberts … Can this be done on a per packet basis is SME wants to do power control? Can the SME set the power here also? |
| PhyPib RX RSSI Group | Heberling … need to revisit policy issue during the CAP in conjunction with RSSI group. |
| PhyPib_CCAModesSupported | Roberts … how many modes will 15.3 have? More than one? |
| Need Ranging Support Group | Roberts … ranging is in the evaluation requirements document … needs to be defined in PHY PIB? |
| In the PHY SAP clause, PHY-TXSTART.request, TXVECTOR | Shvodian … should the TXVECTOR pass the DA (destination address) and the SA (source address)? |
| In the PHY SAP clause, PHY-RXSTART.request, TXVECTOR | Shvodian … should the RXVECTOR pass the DA (destination address) and the SA (source address)? |
| PHY-CCARESET.request() | Roberts … is this needed? |
| PHY-CCARESET.confirm() | Roberts … is this needed? |

PHY Committee Issues List (cont.)

| PHY-CCA.indication | Heberling … need to work out policy |
| --- | --- |
| In PHY SAP, add PHY-RXNAP.request | Shovdian … now does RX go to sleep if the incoming packet is not for this station? |
| In PHY SAP, add PHY-RXNAP.confirm | Shovdian … see above comment |

6.x MAC SAP

MA-UNITDATA.request(source address, destination address, routing information, data, priority, service class)

MA-UNITDATA.indication(source address, destination address, routing information, data, reception status, priority, service class)

MA-UNITDATA-STATUS.indicate(source address, destination address, transmission status, provided priority, provided service class)

6.3 MLME SAP

MLME-GET.request (MIBattribute) ## Requests the value of the given MIBattribute

MLME-GET.confirm (status, MIBattribute, MIBattributevalue) Returns the appropriate MIB attribute value if status = “success,” otherwise returns an error indication in the Status field. Possible error status values include “invalid MIB attribute” and “attempt to get write only MIB attribute.

MLME-SET.request (MIBattribute, MIBattributevalue) Requests that the indicated MIBattribute be set to the given value. If this MIB attribute implies a specific action, then this requests that the action be performed.

MLME-SET.confirm (status, MIBattribute) If status = “success,” this confirms that the indicated MIB attribute was set to the requested value, otherwise it returns an error condition in status field. If this MIBattribute implies a specific action, then this confirms that the action was performed. Possible error status values include “invalid MIB attribute” and attempt to set read-only MIB attribute.

MLME-POWERMGT.request (PowerManagementMode, WakeUp, ReceiveDTIMs)

PowerManagementMode = {ACTIVE, POWER_SAVE} Wakeup = {True, False} ReceiveDTIMs = {True, False}

MLME-POWERMGT.confirm (ResultCode)

ResultCode = {SUCCESS, INVALID_PARAMETERS, NOT_SUPPORTED}

MLME-SCAN.request (PiconetType, PiconetID, SSID, ScanType, ProbeDelay, ChannelList, MinChannelTime, MaxChannelTime)

PicoType = {INFRASTRUCTURE, INDEPENDENT, ANY_BSS} PiconetID = “MACAddress” SSID = “Octet String 0 – 32 octets” ScanType = {ACTIVE, PASSIVE} ProbeDelay = “Integer, in us, used with Probe frame during active scanning” ChannelList = “Specifies a list of channels examined when scanning” ## MinChannelTime = “greater/equal to Probe Delay” MaxChannelTime = “greater/equal to MinChannelTime”

MLME-SCAN.confirm (PiconetDescriptionSet, ResultCode)

PiconetDescriptionSet = “indicates the results of the scan request” (see below) ResultCode = {SUCCESS, INVALID_PARAMETERS}

PiconetDescription.PiconetID = “MACAddress” PiconetDescription.SSID = “1 – 32 octets, the SSID of the found BSS” PiconetDescription.PiconetType = {INFRASTRUCTURE, INDEPENDENT} PiconetDescription.BeaconPeriod = “The Beacon period of the found piconet (in TU)” PiconetDescription.DTIM_Period = “The DTIM period of the BSS (in beacon periods)” PiconetDescription.Timestamp = “timestamp of the received frame from found piconet” PiconetDescription.LocalTime = “value of the STA’s TSF timer at the start of reception” PiconetDescription.PHY_parameter_set = “parameter set relevant to PHY” PiconetDescription.CF = “parameter set for the CF periods, if found BSS supports CF mode” PiconetDescription.IBSS = “parameter set for the IBSS, if found BSS is an IBSS” PiconetDescription.CapabilityInformation = “The advertised capabilities of the piconet” PiconetDescription.PiconetBasicRateSet = “The set of data rates that must be supported by all STA’s that desire to join this BSS”

MLME-JOIN.request (BSSDescription, JoinFailureTimeout, ProbeDelay, OperationalRateSet)

PiconetDescription = “see above” JoinFailureTimeout = “The time limit, in units of beacon intervals, after which the join procedure will be terminated” ProbeDelay = “Delay (in us) to be used prior to transmitting a Probe frame during active scanning” OperationalRateSet = “see above”

MLME-JOIN.confirm (ResultCode)

ResultCode = {SUCCESS, INVALID_PARAMETERS, TIMEOUT}

MLME-AUTHENTICATE.request (PeerSTAAddress, AuthenticationType, AuthenticateFailureTimeout)

PeerSTAAddress = “MACAddress” AuthenticationType = {OPEN_SYSTEM, SHARED_KEY} AuthenticationFailureTimeout = “Specifies a time limit (in TU) after which the authentication procedure will be terminated”

MLME-AUTHENTICATE.confirm (PeerSTAAddress, AuthenticationType, ResultCode)

MLME-AUTHENTICATE.indication (PeerSTAAddress, AuthenticationType)

MLME-DEAUTHENTICATE.request (PeerSTAAddress, ReasonCode)

MLME-DEAUTHENTICATE.confirm (PeerSTAAddress, ResultCode)

ResultCode = {SUCCESS, INVALID_PARAMETERS, TOO_MANY_SIMULTANEOUS_REQUESTS}

MLME-DEAUTHENTICATE.indication (PeerSTAAddress, ReasonCode)

MLME-ASSOCIATE.request (PeerSTAAddress, AssociateFailureTimeout, CapabilityInformation, ListenInterval)

PeerSTAAddress = “MACAddress” AssociateFailureTimeout = “Specifies a time limit (in TU) after which the associate procedure will be terminated” CapabilityInformation = “As defined in frame format Specifies the operational capability definitions to be used by the MAC entity” ListenInterval = “Specifies the number of beacon intervals that may pass before the STA awakens and listens for the next beacon”

MLME-ASSOCIATE.confirm (ResultCode)

ResultCode = {SUCCESS, INVALID_PARAMETERS, TIMEOUT, REFUSED}

MLME-ASSOCIATE.indication (PeerSTAAddress)

MLME-REASSOCIATE.request (NewAPAddress, ReassociateFailureTimeout, CapabilityInformation, ListenInterval)

NewAPAddress = “MACAddress” ReassociateFailureTimeout = “Specifies a time limit (in TU) after which the reassociate procedure will be terminated”

MLME-REASSOCIATE.confirm (ResultCode)

ResultCode = “SUCCESS, INVALID_PARAMETERS, TIMEOUT, REFUSED”

MLME-REASSOCIATE.indication (PeerSTAAddress)

MLME-DISASSOCIATE.request (PeerSTAAddress, ReasonCode)

MLME-DISASSOCIATE.confirm (ResultCode)

ResultCode = {SUCCESS, INVALID_PARAMETERS, TIMEOUT, REFUSED}

MLME-DISASSOCIATE.indication (PeerSTAAddress, ReasonCode)

MLME-RESET.request (STAAddress, SetDefaultMIB)

MLME-RESET.confirm (ResultCode)

ResultCode = {SUCCESS}

MLME-START.request (SSID, PiconetType, BeaconPeriod, DTIMPeriod, CF parameter set, PHY parameter set, IBSS parameter set, ProbeDelay, CapabilityInformation, PiconetBasicRateSet, OperationalRateSet)

SSID = “Octet string, the SSID of the BSS” PiconetType = {INFRASTRUCTURE, INDEPENDENT} DTIM_Period = “The DTIM Period of the BSS (inbeacon periods)” CF_parameter_set = “The parameter set for CF periods, if the BSS supports CF mode” PHY_parameter_set = “The parameter set relevant to the PHY” IBSS_parameter_set = “The parameter set for the IBSS, if BSS is an IBSS” ProbeDelay = “Delay (in us) to be used prior to transmitting a Probe frame during active scanning” CapabilityInformation = “The capabilities to be advertised for the BSS” BSSBasicRateSet = “The set of data rate (in units of 500 kbit/s) that must be supported by all STAs to join this BSS” OperationalRateSet = “The set of data rate (in units of 500 kbit/s) that the STA may use for communication within the BSS”

MLME-START.confirm (ResultCode)

ResultCode = {SUCCESS, INVALID_PARAMETERS, BSS_ALREADY_STARTED_OR_JOINED}

MLME-Join-network.request

MLME-Join-network.confirm

MLME-Establish-network.request

MLME-Establish-network.confirm

MLME-Join-or-establish-network.request

MLME-Join-or-establish-network.confirm

6.4 PLME SAP

PLME-GET.request (PIBattribute) ## Requests the value of the given PIBattribute

PLME-GET.confirm (status, PIBattribute, PIBattributevalue) Returns the appropriate PIB attribute value if status = “success,” otherwise returns an error indication in the Status field. Possible error status values include “invalid PIB attribute” and “attempt to get write only PIB attribute.

PLME-SET.request (PIBattribute, PIBattributevalue) Requests that the indicated PIBattribute be set to the given value. If this PIB attribute implies a specific action, then this requests that the action be performed.

PLME-SET.confirm (status, PMIBattribute) If status = “success,” this confirms that the indicated PIB attribute was set to the requested value, otherwise it returns an error condition in status field. If this PIBattribute implies a specific action, then this confirms that the action was performed. Possible error status values include “invalid PIB attribute” and attempt to set read-only PIB attribute.

PLME-RESET.request () “what is default”

PLME-TESTMODE.request (TEST_ENABLE, TEST_MODE, DATA_TYPE, DATA_RATE)

TEST_ENABLE = {True, false} TEST_MODE = “01 = transparent receive, 02 = continuous transmit, 03 = 50% duty cycle” DATA_TYPE = “1, 2, 3 … Selects one of three data patterns to be used for the transmit portions of the tests” ### DATA_RATE = “Data Rate in Mbps” (do we want Mbps or data rate vector index) (question: should we have loop back testing similar to IEEE802.15.1)

PLME-TESTOUTPUT.request (TEST_OUTPUT)

TEST_OUTPUT = {True, false}

##### PLME Power Management ???

6.5 MAC PIB

| Managed Object | Data Type |
| --- | --- |
| MacPibStationID | MacAddress |
| MacPibMediumOccupancyLimit | INTEGER |
| MacPibCFPollable | TruthValue |
| MacPibCFPPeriod | INTEGER |
| MacPibCFPMaxDuration | INTEGER |
| MacPibAuthenticationResponseTimeOut | INTEGER |
| MacPibPrivacyOptionImplemented | TruthValue |
| MacPibPowerManagementMode | INTEGER |
| MacPibDesiredSSID | OCTET STRING |
| MacPibDesiredPiconetType | INTEGER |
| MacPibOperationalRateSet | OCTET STRING |
| MacPibBeaconPeriod | INTEGER |
| MacPibDTIMPeriod | INTEGER |
| MacPibAssociationResponseTimeOut | INTEGER |
| MacPibDisassociateReason | INTEGER |
| MacPibDisassociateStation | MacAddress |
| MacPibDeauthenticateReason | INTEGER |
| MacPibDeauthenticateStation | MacAddress |
| MacPibAuthenticateFailStatus | INTEGER |
| MacPibAuthenticateFailStation | MacAddress |

6.6 PHY PIB

| Managed Object | Default Value/Range | Static or Dynamic |
| --- | --- | --- |
| PhyPib Regulatory Group |  |  |
| PhyPib_Type | Phy Dependent | Static |
| PhyPib_RegDomainsSupported (note: how are these numerated?) | Phy Dependent | Static |
| PhyPib_CurrentRegDomain | Phy Dependent | Dynamic |
| PhyPib Rate Group |  |  |
| PhyPib_MaxDataRate | Phy Dependent | Static |
| PhyPib_MinDateRate | Phy Dependent | Static |
| ### PhyPib_DataRateStepSize (PhyPib_DataRateVector) | Phy Dependent | Static |
| PhyPib_CurrentDataRate (should this be in MAC PIB?) (can this be done on a per packet basis) | Phy Dependent | Dynamic |
| PhyPib Antenna Group |  |  |
| PhyPib_DiversitySupported | Phy Dependent | Static |
| PhyPib_CurrentTXAntenna | Phy Dependent | Dynamic |
| PhyPib_CurrentRXAntenna | Phy Dependent | Dynamic |
| PhyPib TX Power Group |  |  |
| PhyPib_MaxPowerLevel | Phy Dependent | Static |
| PhyPib_MinPowerLevel | Phy Dependent | Static |
| PhyPib_PowerLevelStepSize | Phy Dependent | Static |
| PhyPib_CurrentPowerLevel (can this be done on a per packet basis) | Phy Dependent | Dynamic |

| Managed Object | Default Value/Range | Static or Dynamic |
| --- | --- | --- |
| PhyPib Channelization Group |  |  |
| PhyPib_NumChannelsSupported | Phy Dependent | Static |
| PhyPib_CurrentChannel (note: per piconet) | Phy Dependent | Dynamic |
| PhyPib RX RSSI Group (need to revisit policy issue on CAP) |  |  |
| PhyPib_RSSI_max | Phy Dependent | Static |
| PhyPib_RSSI_min | Phy Dependent | Static |
| PhyPib_RSSI_StepSize | Phy Dependent | Static |
| PhyPib MPDU Length Group |  |  |
| PhyPib_MPDU_LengthMax | Phy Dependent | Static |
| PHYMib CCA Compliance Group |  |  |
| PhyPib_CCAModesSupported (how many modes will 15.3 have) | Phy Dependent | Static |
| PhyPib_CurrentCCAMode | Phy Dependent | StaticDynamic |
| PhyPib_CCA_Threshold | Phy Dependent | StaticDynamic |
| PhyPib Short Turnaround Group |  |  |
| PhyPib_ShortTurnaroundSupported | Phy Dependent | Static |
| PhyPib IEEE802.15.1 Support Group |  |  |
| PhyPib_IEEE802.15.1_PhySupported | Phy Dependent | Static |
| #### Ranging Support Group?????? |  |  |

6.7 PHY SAP

PHY-DATA.request (DATA)

PHY-DATA.indication (DATA)

DATA = {octet value}

PHY-DATA.confirm ()

PHY-TXSTART.request (TXVECTOR)

TXVECTOR = {DataRate, Length, TxPowerLevel, DA, SA, ???} Note: “Length” is number of octets

PHY-TXSTART.confirm ()

PHY-TXEND.request ()

PHY-TXEND.confirm ()

PHY-CCARESET.request () needed?

PHY-CCARESET.requestconfirm() needed?

PHY-CCA.indication (STATE) need to work out policy for alternate PHY’s

# STATE = “BUSY” or ”IDLE”

PHY-RXSTART.indication (RXVECTOR)

RXVECTOR = {DataRate, Length, RSSI, DA, SA, ???} Note: “Length” is number of octets SNR for RSSI?

PHY-RXEND.indication (RXERROR)

RXERROR = {NoError or FormatViolation or CarrierLost or UnsupportedRate}

PHY-RXNAP.request (NAP, WAKE)

PHY-RXNAP.confirm ()