IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Mobilian Text for Clause 6 |  |
| Date Submitted | [23 August, 2001] |  |
| Source | [Jim Lansford, Adrian P Stephens] [Mobilian Corporation] [7431 NW Evergreen Pkwy, Hillsboro, OR 97124, USA] | Voice: [ +1 (503) 681-8600 ] Fax: [ ] E-mail: [ jim.lansford@mobilian.com adrian.stephens@mobilian.com ] |
| Re: | 00308r0P802-15_TG2-Stage0_PHY_Model.ppt 00360r0P802-15_TG2-Mobilian-coexistence-proposal.ppt 01164r0P802-15_TG2-Mobilian-Symbol-802.11-Bluetooth.ppt 01172r0P802-15_TG2-Mobilian-Symbol-summary.ppt |  |
| Abstract | This document defines the analytical model for the BER resulting from interference between 802.11b and 802.15.1 transmissions. It also includes some introduction of the physical layer concepts and defines the path loss model used. |  |
| Purpose | This document contains draft text for the 802.15.2 recommended practice. |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Note, comments by the author that are not intended to be part of the final document are introduced by: “[ed – “.

# Physical Layer Models

This section contains two physical layer models.  The first is an analytical model that is suitable for extended MAC-layer simulations. The second is a numerical model that is more accurate but more computationally intensive.

## Physical Layer Model Concepts

This section introduces concepts that are common to the physical models described in this clause. The most powerful simplifying concept in this model is the period of stationarity (POS).  This is the period over which the parameters defining the transmissions of the devices being modeled do not change.

Consider the example shown in Figure 1.  Here an 802.15.1 device transmits two packets. An 802.11b device transmits a single PHY protocol data unit (PPDU) using 11Mbps modulation type for the PHY service data unit (PSDU).  The start of the PHY layer convergence protocol (PLCP) header overlaps the end of the first 802.15.1 packet.  The end of the PSDU overlaps the start of the second 802.15.1 packet.  There are six periods of stationarity.  Note that a new POS starts at the end of the PLCP header because the modulation type changes at this point.

Figure 1 - Example showing Periods of Stationarity

By definition, during the POS the transmit power and modulation type do not change, and the position of the devices (and hence link loss) is constant. So receiving nodes experience constant signal, noise and interference powers from which a BER value can be calculated or simulated.

## Path Loss Model

The path loss we use is given by Equation 1 and shown in Figure 2.  This path loss model is described in [ed- cite Kamerman, IEEE 802.11-00/162, “Coexistence between Bluetooth and IEEE 802.11 CCK Solutions to avoid mutual interference”].  Path loss follows free-space propagation (coefficient is 2) up to 8m and then attenuates more rapidly (with a coefficient of 3.3).

Note that the model does not apply below about 0.5m due to near-field and other implementation effects.

Equation 1- Path Loss versus distance (m)

| Path Loss =40.2+20log(d), | d < 8m |
| --- | --- |
| Path Loss =58.5+33log(d8), | d > 8m |

Figure 2- Path Loss (dB) versus distance (m)

## Analytical model for IEEE 802.11b and IEEE 802.15.1 transmissions

### Introduction

This section describes the analytical model that allows the bit error rate (BER) to be calculated for 802.11b and 802.15.1 packets in the presence of mutual interference.

### Model Interface

The model is supplied with device positions and transmission parameters.  The model calculates the BER derived from those parameters.

The parameters described in Table 1 are supplied to the PHY model for each transmission that is active during a POS.

Table 1 - Transmission Parameters

| Field | Description |
| --- | --- |
| Source Position | Device position specified using Cartesian Coordinates |
| Destination Position |  |
| Modulation Type | Type of modulation used by the transmitter. One of: - 802.15.1 - 802.11b 11Mbps - 802.11b 5.5 Mbps - 802.11b 1Mbps - 802.11b 2Mbps |
| Transmit Power | Transmit power |
| Frequency | Center frequency of transmission |

The output of the PHY model is a BER value at the receiver of each transmission.

### BER Calculation

Figure 3 shows the BER calculation in diagrammatic form.

The intended transmission is attenuated by the path loss (defined in section 6.2) to the receiver.  This is the signal power at the receiver.  Each interfering transmission is attenuated by its path loss to the receiver and by the spectrum factor (defined in 6.2.4) to account for the combined effect of receiver and transmitter masks and frequency offset.  The resulting interference powers are added to give the total interference power.  The signal to noise and interference (SNIR) value is the ratio of signal to total interference power at the receiver.  The BER is calculated from the SNIR as defined in 6.2.7.

Figure 3 - BER Calculation

### Spectrum Factor

The spectrum factor represents the combined effects of transmitter and receiver masks (defined in 6.2.6) and frequency offset.  It also includes the effect of any CCK coding gain.

There are four possible combinations of transmitter and receiver.  The spectrum factor is defined to be unity for like transmitter and receiver with a zero frequency offset.  Other spectrum factors are defined by Table 2 to Table 4.

Table 2 - Spectrum Factor Values for 802.15.1 Receiver

| Transmitter | Receiver | Frequency offset (d) in MHz |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | Other |
| 802.15.1 | 802.15.1 | 8.9433E-02 | 1.7943E-04 | 8.9433E-06 | 7.9433E-06 |
| 802.11b | 802.15.1 | 8.0433E-2 | 1.0794E-03 | 1.0079E-03, d<=11 1.7943E-05, d>11 |  |

Table 3 - Spectrum Factor Values for 802.15.1 to 802.11b

| Transmitter | Receiver | Frequency offset (d) in MHz |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 12 | 13 | 14 | Other |
| 802.15.1 | 802.11b (1 & 2 Mbps) | 7.3197E-02 | 3.5219E-04 | 2.5219E-04 , d=14 | 1.0, d<12 2.5119E-04, d<23 2.5119E-06, d>=23 |
| 802.15.1 | 802.11b (5.5 & 11 Mbps) | 1.1601E-02 | 5.5818E-05 | 3.9969E-05 | 1.0, d<12 3.9811E-05, d<23 3.9811E-07, d>=23 |

Table 4 - Spectrum Factor Values for 802.11b Self Interference

| Transmitter | Receiver | Frequency offset (d) in MHz |  |  |
| --- | --- | --- | --- | --- |
|  |  | 11 | 22 | Other |
| 802.11b | 802.11b | 0.5 | 6.7360E-03 | 1.2512E-05 |

### SNIR Computation

The SNIR is given by the ratio of the received signal power to the total received interference power.  Note that the powers are calculated after the spectrum factor has been applied, and so this ratio corresponds to the value after the receiver filter.

Receiver noise is not considered in this model.

### Transmit and Receive Masks

The transmit and receive masks used are defined in Table 5 and shown in Figure 4.

Table 5 - Transmit and Receive masks

|  | Transmit | Receive |
| --- | --- | --- |
| 802.15.1 | Frequency Offset (MHz); Attenuation (dB). 0; 0. 1; -20. 2; -40. 3; -60. 4 and greater; full attenuation. | Frequency Offset (MHz); Attenuation (dB). 0; 0. 1; -11. 2; -41. 3 and greater; -51. |
| 802.11b | Frequency Offset (MHz); Attenuation (dB). 0; 0. 1 to 11; -30. 12 and greater; -50. | Frequency Offset (MHz); Attenuation (dB). 0; 0. 1 to 12; -12. 13 to 36; -36. 37 and greater; -56. |

|  | Transmit |  | Receive |
| --- | --- | --- | --- |
| 802.15.1 |  |  |  |
| 802.11b |  |  |  |

Figure 4 - Transmit and Receive masks for 802.15.1 and 802.11b

When calculating the effects of an 802.15.1 packet on an 802.11b 5.5Mbps or 11Mbps packet, a CCK coding gain of 8dB is used.

### BER calculation based on SNIR

The Symbol Error Rate (SER) is calculated for each modulation type based on the SNIR at the receiver.  This is then converted into an effective BER given the number of bits per symbol.

The sections that follow describe the BER calculation for the different modulation types.

#### BER calculation for 802.15.1 Modulation

Assuming envelope detection of orthogonal FSK, the BER is given directly by.

BER802.15.1 =0.5×exp(−SNIR2)

#### BER Calculation for 802.11b 1Mbps

The probability of error in a symbol in the presence of AWGN is given by:

P=Qd22N0

Where d is the minimum distance between any two points in the signal constellation and N0 is the in-band noise power at the receiver. The Q function is defined in section 6.2.7.6.

In the case of a 802.11b 11Mbps chip, the modulation scheme is differential BPSK.  This has the effect of doubling the effective noise power at the receiver .

PDBPSK−CHIP=Qd24NC, where NC is the noise energy per chip.

The value of d can be determined by plotting the modulation constellation of BPSK placing the signal points at a distance of ECfrom the origin, where EC is the received signal energy per chip.  Thus dDBPSK-CHIP = 2EC. So now:

PDBPSK−CHIP=QECNC

This is the probability of an error in an individual 11Mbps chip.

To include the effect of the spreading code, the squared distance is summed over each chip.  In the case of 802.15.1 1Mbps modulation, the 11-chip spreading code results in the squared distance being multiplied by a factor of 11.

Giving P1MBPS−SYMBOL=Q11×SNIR, where SNIR = EC/NC.

This is the 1Mbps symbol error rate.  It is also the 1Mbps BER, because each symbol encodes a single bit.

#### BER Calculation for 802.11b 2Mbps

This calculation follows the treatment for the 1Mbps calculation with a few differences.

The 2Mbps rate uses 11Mbps DQPSK chips.  The minimum distance between points in the QPSK constellation is reduced by a factor of 2 (compared to BPSK) giving dQBPSK-CHIP = 2EC.

This substitution results inP2MBPS−SYMBOL=Q5.5×SNIR.

Each 2Mbps symbol encodes two bits.  However, because the symbols are gray coded, a decoding error between adjacent DQPSK constellation points yields only a single bit error in the decoded 2Mbps bitstream .  So this symbol error rate is also the required BER.

#### 802.11b 5.5 Mbps BER Calculation

The symbol error rate can be determined by treating the modulation as a block code in the presence of AWGN interference. The general symbol error rate isQ2×SNIR×Rc×Wm, where Rc is the code rate, Wm is the codeword distance and the sum is over all other codewords.

For 802.11b 5.5 Mbps, the symbol error rate, SER5.5, is given by:

SER5.5 =14×Q8×SNIR+Q16×SNIR

As each symbol encodes 4 bits, the effective BER is

BER=min1−(1−SER5.5)14,0.5

in which the value has also been limited to 0.5 .

#### 802.11b 11 Mbps BER Calculation

For 802.11b 11 Mbps, the symbol error rate, SER11 is given by:

SER11=24×Q4×SNIR+16×Q6×SNIR+174×Q8×SNIR+16×Q10×SNIR+24×Q12×SNIR+Q16×SNIR

As each symbol encodes 8 bits, the effective BER is

BER=min1−(1−SER11)18,0.5

#### Q Function Definition

The Q function is defined as the area under the tail of the Gaussian probability density function with zero mean and unit variance.

Q(x)=12πx∞e−t22dt

In this model, a fifth-order approximation to Q(x) is used:

Q(x)=12πexp(−x22)×8+9x2+x415x+10x3+x5

#### SNIR Limits

The simulation is simplified by assuming that above a certain SNIR the BER is effectively zero and below a certain SNIR the BER is effectively 0.5.  These limits are defined in Table 6.

Table 6 - Assumed Limits on SNIR

| Receiver | Upper limit on SNIR | Lower limit on SNIR |
| --- | --- | --- |
| 802.11b | 10dB | -3dB |
| 802.15.1 | 20dB | 1dB |

#### BER versus SNIR Results

Refer to section ??? for a presentation of the results of the analytical model.

#### Section ??? BER versus SNIR Results

(Ed - please move into results sections in merged document)

Figure 5 shows the results of calculating BER for SNIR values in the range –15 to 20dB for each modulation type .

Figure 5 – BER versus SNIR for 802.11b and 802.15.1 Modulation Types

# Annex ??? – Source Code for Analytical Model

This section contains source code for the Analytical Model.

/*---------------------------------------------------------------------------

* Title:    Analytical Physical-Layer Model for 802.15.2 BER Calculations

* Authors:  Ron Nevo, Josie Ammer, Adrian Stephens

*           Mobilian Corporation 2001

*

* This module contains the analytical PHY-layer model used to calculate

* BER values for 802.11b and 802.15.1 transmissions in the presence of

* mutual interference.

*---------------------------------------------------------------------------*/

/*-- Standard Includes ------------------------------------------------------*/

#include <stdio.h>

#include <math.h>

#include <stdlib.h>

#include <limits.h>

/*-- Type Definitions -------------------------------------------------------*/

typedef enum

{

WPAN, WLAN11, WLAN55, WLAN1, WLAN2

// WPAN is non-FEC WPAN transmissions

// WLAN11 is 11Mb/s 802.11

// WLAN55 is 5.5Mb/s 802.11

// WLAN1 is 1Mb/s 802.11

// WLAN2 is 2Mb/s 802.11

} ModulationType;

struct Node

{

double x,y;  // x and y positions in meters

};

typedef Node *aNodePtr;

struct Transmission

{

aNodePtr src, dst;

ModulationType type;

float txpower;  // power in mW

int frequency;

// freq for WPAN is a number 1-79, is the center frequency

// freq for WLAN is number 1-79, is the center frequency

double BER;     // the resulting BER

};

typedef Transmission *aTransmissionPtr;

/*---------------------------------------------------------------------------

* Function: CalculateAnalyticalBER

*

* Description:

*              This function takes a list of transmissions and calculates

*              the BER at each receiver using the analytical model.

*

* Parameters:  n       - the number of transmissions in the list

*              tlist   - an array of Transmissions each corresponding to

*                        an active transmission

*

* Returns: in every tlist element a calculated BER value

*---------------------------------------------------------------------------*/

extern void CalculateAnalyticalBER(int n, Transmission tlist[]);

/*-- MACRO Definitions ------------------------------------------------------*/

#define WLAN_BANDWIDTH 22

#define HALF_WLAN_BANDWIDTH 11

#define PI acos(-1.0)

// assume the power of WPAN reception at the same station is unit power

#define Tc 1

#define WPAN_Tx_1 inverse_db(-20.0)

// -20dB spurious emmision per MHz from WPAN Tx when freq dif is 1MHz

#define WPAN_Tx_2 inverse_db(-40.0)

// -40dB spurious emmision per MHz from WPAN Tx when freq dif is 2MHz

#define WPAN_Tx_3 inverse_db(-60.0)

// -60dB spurious emmision per MHz from WPAN Tx when freq dif is 3MHz

#define WPAN_Tx_other 0

// no spurious emmision from WPAN Tx when freq dif > 3MHz

#define WPAN_Rx_1 inverse_db(-11.0)

// -11dB adjacent channel interference from WPAN Rx when freq dif is 1MHz

#define WPAN_Rx_2 inverse_db(-41.0)

// -41dB adjacent channel interference from WPAN Rx when freq dif is 2MHz

#define WPAN_Rx_other inverse_db(-51.0)

// -51dB adjacent channel interference from WPAN Rx when freq dif >= 3MHz

#define WLAN_Tx_next inverse_db(-30)

// -30dB spurious emmision from WLAN Tx when freq dif <= half bandwidth

#define WLAN_Tx_other inverse_db(-50.0)

// -50dB spurious emmision from WLAN Tx when freq dif > half bandwidth

#define WLAN_Rx_next inverse_db(-12.0)

// -12dB adjacent channel interference from WLAN Rx, 12 MHz

#define WLAN_Rx_middle inverse_db(-36.0)

// -60dB adjacent channel interference from WLAN Rx, between 13MHz-36MHz

#define WLAN_Rx_other inverse_db(-56.0)

// -60dB adjacent channel interference from WLAN Rx, >= 37MHz

#define CCK_factor inverse_db(-8.0)

// 8dB gain for CCK coding

#define WLAN_EbNo_perfect 10.0   // if EbNo > 10dB, perfect reception

#define WLAN_EbNo_impossible 0.5 // if EbNo < -3dB, impossible to receiver

#define WPAN_EbNo_perfect 20.0   // if EbNo > 13dB, perfect reception

#define WPAN_EbNo_impossible 1.0 // if EbNo < 0dB, impossible to receive

#define MIN_DISTANCE 0.1

// two nodes in the same spot act as if they are 0.1 apart

/*---------------------------------------------------------------------------*/

#define abs(a) ((a)>0 ? (a) : -(a))

#define sqr(x) ((x)*(x))

#define min(a,b) ((a)<(b) ? (a) : (b))

/*-- Forward References -----------------------------------------------------*/

extern int isModulationTypeWPAN(ModulationType foo);

// returns 1 if the type is WPAN

// returns 0 otherwise

extern int isModulationTypeWLAN(ModulationType foo);

// returns 1 if the type is one of the WLANs

// returns 0 otherwise

extern double SER11(double);

extern double SER55(double);

extern double WLAN_BER_11(double);

extern double WLAN_BER_55(double);

extern double WLAN_BER_1(double);

extern double WLAN_BER_2(double);

extern double WPAN_BER(double);

extern double SpectrumFactor(Transmission &Src, Transmission &Dest);

extern double Distance(Transmission &Src, Transmission &Dest);

extern double PowerDistance(Transmission &Src, Transmission &Dest);

extern double inverse_db(double);

extern double db(double);

/*-- Local Functions --------------------------------------------------------*/

int isModulationTypeWPAN(ModulationType foo)

{

switch (foo){

case WPAN: return 1;

default: return 0;

}

}

int isModulationTypeWLAN(ModulationType foo)

{

switch (foo){

case WLAN11: return 1;

case WLAN55: return 1;

case WLAN1: return 1;

case WLAN2: return 1;

default: return 0;

}

}

// compute db from real

double db(double x)

{

return(10 * log10(x));

}

// compute real from db

double inverse_db(double x)

{

return(pow(10.0, x /10));

}

// compute the Q function using approximation Q_5

double Q_5(double x)

{

double x2,x3,x4,x5,x6;

x2 = x*x;

x3 = x2*x;

x4 = x3*x;

x5 = x4*x;

x6 = x5*x;

return(exp(-x2/2) * (x4+9*x2+8) /(x5+10*x3+15*x) / sqrt(2*PI));

}

// compute the codeword error probability of 802.11b 11Mbps

double SER11(double Eb_No)

{

double res;

res = 24*Q_5(sqrt(4*Eb_No)) +

16*Q_5(sqrt(6*Eb_No)) +

174*Q_5(sqrt(8*Eb_No)) +

16*Q_5(sqrt(10*Eb_No)) +

24*Q_5(sqrt(12*Eb_No)) +

Q_5(sqrt(16*Eb_No));

return(min(res,0.99999));

}

// compute bit error rate from Eb/No for 802.11b 11Mbps

double WLAN_BER_11(double Eb_No)

{

if(Eb_No > WLAN_EbNo_perfect)

return 0;

// if Eb/No more than some threshold, perfect reception

else if(Eb_No < WLAN_EbNo_impossible)

return 0.5;

// if Eb/No less than some threshold, impossible to receive

else

return(min(1-pow((1-SER11(Eb_No)),1.0/8),0.5));

}

// compute the codeword error probability of 802.11b 5.5Mbps

double SER55(double Eb_No)

{

double res;

res = 14*Q_5(sqrt(8*Eb_No)) +

Q_5(sqrt(16*Eb_No));

return(min(res,0.99999));

}

// compute bit error rate from Eb/No for 802.11b 11Mbps

double WLAN_BER_55(double Eb_No)

{

if(Eb_No > WLAN_EbNo_perfect)

return 0;

// if Eb/No more than some threshold, perfect reception

else if(Eb_No < WLAN_EbNo_impossible)

return 0.5;

// if Eb/No less than some threshold, impossible to receive

else

return(min(1-pow(1-SER55(Eb_No),1.0/4),0.5));

}

// compute the function number of choice of k elements from n

int choose(int k,int n)

{

int i;

int res = 1;

for(i=n;i>n-k;i--)

res *= i;

for(i=1;i<=k;i++)

res /= i;

return(i);

}

// compute the BER for WLAN 1Mbps, the BER is Q(sqrt(11*2*Eb_No/2))

double WLAN_BER_1(double Eb_No)

{

if(Eb_No > WLAN_EbNo_perfect)

return 0;

// if Eb/No more than some threshold, perfect reception

else if(Eb_No < WLAN_EbNo_impossible)

return 0.5;

// if Eb/No less than some threshold, impossible to receive

else

return(min(Q_5(sqrt(11*2*Eb_No/2)),0.5));

}

// compute the BER for WLAN 2Mbps, the BER is Q(sqrt(5.5*2*Eb_No/2))

double WLAN_BER_2(double Eb_No)

{

if(Eb_No > WLAN_EbNo_perfect)

return 0;

// if Eb/No more than some threshold, perfect reception

else if(Eb_No < WLAN_EbNo_impossible)

return 0.5;

// if Eb/No less than some threshold, impossible to receive

else

return(min(Q_5(sqrt(5.5*2*Eb_No/2)),0.5));

}

// compute the BER for WPAN

double WPAN_BER(double Eb_No)

{

if(Eb_No > 20)

return 0;

// if Eb/No more than 13dB, perfect reception

else if(Eb_No < 1)

return 0.5;

// if Eb/No less than 0dB, impossible to receive

else

return(min(exp(-Eb_No/2),0.5));

}

double Distance(Transmission &Src, Transmission &Dest)

{

return(sqrt(sqr(Src.src->x-Dest.dst->x) + sqr(Src.src->y-Dest.dst->y)));

}

// power as function of distance

double PowerDistance(Transmission &Src, Transmission &Dest)

{

double power_d0,dist;

dist=Distance(Src,Dest); // calc distance function

if (dist < MIN_DISTANCE)

dist = MIN_DISTANCE;

if(isModulationTypeWPAN(Src.type)) {    // transmitter is WPAN,

power_d0 = Src.txpower;

}

else // transmitter is WLAN

if(isModulationTypeWPAN(Dest.type)) // receiver is WPAN,

power_d0 = Src.txpower/WLAN_BANDWIDTH;

else

power_d0 = Src.txpower; // receiver is WLAN

if(dist < 8)  // use 40.2+20log d for <8M power loss

return(power_d0/(pow(dist,2.0) * pow(10.0, 4.02)));

else         // use 58.5 + 33log(d/8) for >8M power loss

return(power_d0/(pow(dist/8.0,3.3) * pow(10.0, 5.85)));

}

double SpectrumFactor(Transmission &Src, Transmission &Dest)

// (int src0, int src,int dest)

// dest is listenin to src0

// dest gets interference from src

{

int freq_dif;

if(isModulationTypeWPAN(Dest.type))

if(isModulationTypeWPAN(Src.type))

{

// both src and dest WPAN

freq_dif = abs(Src.frequency - Dest.frequency);

switch(freq_dif) {

case 0:

// frequency collide

return(1.0);

case 1:

return(WPAN_Tx_1+WPAN_Rx_1);

case 2:

return(WPAN_Tx_2+WPAN_Rx_2);

case 3:

return(WPAN_Tx_3+WPAN_Rx_other);

default:

return(WPAN_Tx_other+WPAN_Rx_other);

}

}

else

{

// src WLAN and dest WPAN

if(Dest.frequency >= Src.frequency + HALF_WLAN_BANDWIDTH)

freq_dif = Dest.frequency - (Src.frequency + HALF_WLAN_BANDWIDTH) + 1;

else if(Dest.frequency <= Src.frequency - HALF_WLAN_BANDWIDTH)

freq_dif = (Src.frequency - HALF_WLAN_BANDWIDTH) - Dest.frequency + 1;

else

freq_dif = 0;

switch(freq_dif) {

case 0:

// frequency collide

return(1.0);

case 1:

return(WLAN_Tx_next+WPAN_Rx_1);

case 2:

return(WLAN_Tx_next+WPAN_Rx_2);

default:

if(freq_dif<=HALF_WLAN_BANDWIDTH)

return(WLAN_Tx_next+WPAN_Rx_other);

else

return(WLAN_Tx_other+WPAN_Rx_other);

}

}

else // dest WLAN

{

if(isModulationTypeWPAN(Src.type)) {

// src WPAN, dest WLAN

double sf;

if(Src.frequency >= Dest.frequency + HALF_WLAN_BANDWIDTH)

freq_dif = Src.frequency - (Dest.frequency + HALF_WLAN_BANDWIDTH) + 1;

else if(Src.frequency <= Dest.frequency - HALF_WLAN_BANDWIDTH)

freq_dif = (Dest.frequency - HALF_WLAN_BANDWIDTH) - Src.frequency + 1;

else

freq_dif = 0;

switch(freq_dif) {

case 0:

// frequency collide

sf = 1.0;

break;

case 1:

sf = WPAN_Tx_1+WPAN_Tx_2+WPAN_Tx_3+WPAN_Tx_other*(WLAN_BANDWIDTH-3)+WLAN_Rx_next;

break;

case 2:

sf = WPAN_Tx_2+WPAN_Tx_3+WPAN_Tx_other*(WLAN_BANDWIDTH-2)+WLAN_Rx_middle;

break;

case 3:

sf = WPAN_Tx_3+WPAN_Tx_other*(WLAN_BANDWIDTH-1)+WLAN_Rx_middle;

break;

default:

if(freq_dif<HALF_WLAN_BANDWIDTH){

sf = WPAN_Tx_other*WLAN_BANDWIDTH+WLAN_Rx_middle;

break;

}

else{

sf = WPAN_Tx_other*WLAN_BANDWIDTH+WLAN_Rx_other;

break;

}

}

if (Dest.type == WLAN11 || Dest.type == WLAN55)

sf *=CCK_factor;

return(sf);

}

else { //both src and dest WLAN

switch(abs(Src.frequency-Dest.frequency)) {

case 0:

return(1.0);

case 11:

return(0.5);

case 22:

return(WLAN_Rx_next/11+WLAN_Tx_next);

default:

return(WLAN_Rx_other+WLAN_Tx_other);

}

}

}

}

/*-- Global Function --------------------------------------------------------*/

void CalculateAnalyticalBER(int n, Transmission tlist[])

{// n should be the length of tlist

double EbNo;

for (int dst= 0; dst < n ; dst++) { //for each dest

double Eb, No;

No=0;

for (int src = 0; src < n; src++) { //calculate the power from each source

double pd,sf,pwr;

sf=SpectrumFactor(tlist[src],tlist[dst]);

pd=PowerDistance(tlist[src],tlist[dst]);

pwr=sf*pd;

#if defined(_DEBUG_)

printf("rcpt power from %d to %d = %1.5g * %1.5g = %1.5g\n", src, dst, sf, pd, pwr);

#endif

if (src==dst)

// if src and dest are from the same transmission pair,

// pwr is signal power

Eb=pwr*Tc;

else

// if not from the same transmission pair,

// pwr is interference power

No+=pwr;

}

EbNo=Eb/No; // calculate the EbNo for each dest

//need to calc BER from SNR

double ber0;

switch (tlist[dst].type) {

case WPAN:

ber0 = WPAN_BER(EbNo);

break;

case WLAN11:

ber0 = WLAN_BER_11(EbNo);

break;

case WLAN55:

ber0 = WLAN_BER_55(EbNo);

break;

case WLAN1:

ber0 = WLAN_BER_1(EbNo);

break;

case WLAN2:

ber0 = WLAN_BER_2(EbNo);

break;

default:

printf("Unknown ModulationType");

}

#if defined(_DEBUG_)

printf("EbNo for Transmission %d = %1.5g\n", dst, EbNo);

printf("BER for Transmission %d = %1.5g\n", dst, ber0);

#endif

tlist[dst].BER = ber0;

}

}

This doubling is slightly pessimistic for BPSK under conditions of high SNIR.

An alternative approach giving the same result is to consider the spreading sequence to be a block code of length 11.

Errors between adjacent DQPSK constellation points are much more likely that errors between opposing constellation points.

BER values larger than 0.5 are meaningless.

The results for 802.11b 11Mbps do not show calculated values for SNIR < -2dB due to limitations of the tools used.