IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Power Management Additional Proposals |  |
| Date Submitted | 2 October, 2001 |  |
| Source | Jay Bain Time Domain 7057 Old Madison Pike Huntsville, AL 35801 Mark E. Schrader Eastman Kodak Co. 4545 East River Road Rochester, NY 14650-0898 | Voice: 256 922-9229 Fax: 256 922-0837 E-mail: jay.bain@timedomain.com Voice: 716-781-9561 FAX: 716-781-9533 E-Mail: mark.e.schrader@kodak.com |
| Re: | IEEE Draft P802.15.3/D0.7 |  |
| Abstract | This document provides additional proposals for draft text to power save clauses of the 802.15.3 MAC to be incorporated into D0.8 |  |
| Purpose | The recommendations contained in this document are to be applied to the 802.15.3 MAC baseline, if they are approved by the Task Group during the Nov, 2001 Plenary meeting |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

## Updates to d0.7 that relate to power management

### Add new section following 8.12.3.3 and before existing 8.12.3.4

8.12.3.4 Control of remote equipment power

Operations of EPS and AWAKE modes are extended into the equipment a DEV is incorporated into. MLME-POWERMGT.request and indicate primitives provide parameters to facilitate power saving in the equipment.

The MLME-POWERMGT.request Wakeup parameter causes a Switch to AWAKE CTA mode or Switch to EPS CTA mode to be sent to an EPS DEV. The EPS DEV receiving this command shall provide the MLME-POWERMGT.indicate primitive parameter PeerWakeup to its equipment.

The PeerWakeup parameter shall also provide indication that a packet has been received as a result of operation in EPS mode either with low duty cycle periodic information or momentary information. For each received packet while in EPS mode, the MLME-POWERMGT.indicate primitive shall be generated.

A DEV shall provide sufficient storage for received data pending the awakening of the equipment it is part of. The extent of this storage may be reduced through the use of the AWAKELeading parameter provided in MLME-POWERMGT.request. This parameter is conveyed using the EPS configuration request command and is presented on the receiving end using the MLME-POWERMGT.response parameter AWAKELeading. This operation is typically part of setting up power management operation but may be triggered by a new MLME-POWERMGT.request primitive at any time during an association.

# Add the following in the 7.5.11 EPS configuration request

Figure 34

Length = 10

New field – AWAKELeading of two octets following EPSPhase

New descriptions –

EPSTime has a range of 0 to 65,535 milliseconds. A value of zero has the interpretation of DEVs waking for each superframe. Depending on the value of superframe duration parameter, values of EPSTime that are a value that is less than the current value of superframe duration are interpreted as wake for each superframe. Since wake time is bounded by superframe beacon location, the beacon start point preceding the completion of EPSTime shall be the wake point.

EPSPhase is a value of beacon number as defined in Piconet synchronization parameters element. The value is determined by the EPSSync parameter in the MLME-POWERMGT.request primitive. The current beacon number when that primitive is received by the SME is used for EPSPhase in building the EPS configuration request command.

AWAKELeading is provided by each EPS set member to its peer. It is a two-octet field in milliseconds.

# Add the following in the 7.5.12 EPS configuration response

Figure 35

Length = 10

New field – AWAKELeading of two octets following EPSPhase

## Add the following in the 6.3.1.1 MLME-POWERMGT.request

Add AWAKELeading to the structure. Add after EPSSync on line 29 of page 20

Add to table 1 on page 21 a new row

| AWAKELeading | Integer | 0-65535 ms | Time interval for equipment a DEV is part of for that equipment to return from a reduced power state to fully ready to accept large volumes of data. |
| --- | --- | --- | --- |

## Replace the existing 6.3.1.3 MLME-POWERMGT.indication with the following

6.3.1.3 MLME-POWERMGT.indication

This primitive reports power management changes from a specific peer (peer, master, and slave) MAC entity. The primitive parameters are as follows

MLME-POWERMGT.indication	(

PeerPowerManagementMode,

PeerPowerManagementRole,

PeerWakeup,

PeerEPSTime,										AWAKELeading,

PNID

)

#### Table 3 – MLME-POWERMGT.indication primitive parameters

| # Name | # Type | # Valid Range | # Description |
| --- | --- | --- | --- |
| PeerPowerManagmentMode | Enumeration | POWERED, RPS, EPS | Indicates the Power management mode of the peer |
| PeerPowerManagementRole | Boolean | True, False | Indicates the Power management role of the peer |
| PeerWakeup | Enumeration | EPS, ACTIVE, SHORTTERM | An indication that a mode change has occurred based on network information as described in clause 8 |
| PeerEPSTime | Integer | 0-65,535 ms | EPSTime from a specific peer. This should match sent EPSTime. If not, this is a negotiation request |
| AWAKELeading | Integer | 0-65535 ms | Time interval for equipment a DEV is part of for that equipment to return from a reduced power state to fully ready to accept large volumes of data. |
| PNID | MAC Address | Any valid individual MAC address | Identifies the source of the indication primitive |

6.3.1.3.1 When generated

This primitive is generated by the DEV as a result of a command or activity by another DEV in the piconet.

6.3.1.3.2 Effect of receipt

The SME is notified of changes in power management configuration or to wake up for information reception.