IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Additional Interference Scenarios |  |
| Date Submitted | [August 10, 2001] |  |
| Source | [Nada Golmie] [NIST] [100 Bureau Dr. Stop 8920] [Gaithersburg, MD 20899] | Voice: [(301) 975-4190] Fax: [(301) 590-0932] E-mail: [nada.golmie@nist.gov] |
| Re: |  |  |
| Abstract | [Three additional interference scenarios are defined in order to quantify the impact of interference on Bluetooth and IEEE 802.11 devices operating in the same environment] |  |
| Purpose | [The intention is to stimulate discussion and reach consensus in TG2 on the interference scenarios to be included in the Recommended Practice document.] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Case 1: Conference Hall Scenario, 1 WLAN System, 10 BT piconets

Figure 1: Conference Hall Topology

|  | Bluetooth | IEEE 802.11b |
| --- | --- | --- |
| Traffic | Fixed size packet, exponential interarrivals | Fixed size packet, exponential interarrivals |
| Baseband Packet generation | DM1 (Slave to master and master to slave) | Packet = 8000 bits (including MAC Header) ACK = 112 bits |
| Offered Load | 30% of capacity | 30% of capacity |
| Interarrival mean |  |  |
| Placement: (x, y) coordinates in meters | 10 BT masters randomly distributed within a circle of radius r; BT slaves have a distance from their respective masters equal to d. d varies between 0.5 and 2 meters; r varies between 3 and 10 meters | WLAN Source: (0,15) m WLAN AP: (0,0) m |
| Number of devices | 10 pairs | 2 devices (1 AP , 1STA) |
| Bluetooth (MAC) Parameters |  |  |
| Mode of Operation | Connection | Connection |
| Other Parameters |  |  |
| Data Rate (Mbps) | 1 Mbit/s | 1) Direct Sequence Spread Sprectrum at 11, and 1 Mbits/s 2) Frequency Hopping at 1 Mbits/s |
| General Simulation Parameters |  |  |
| Processing delay (at transmitter and receiver including segmentation and reassembly) | 0 ms | 0 ms |
| Length of Simulation Run | 30 seconds | 30 seconds |
| Length of Run Prior to Gathering Statistics | 5% of simulated time | 5% of simulated time |

Case 2: Cubicle Scenario, 1 WLAN System, 4 BT piconets

Figure 2: Cubicle Topology

|  | Bluetooth | IEEE 802.11b |
| --- | --- | --- |
| Traffic | Fixed size packet, exponential interarrivals | Fixed size packet, exponential interarrivals |
| Baseband Packet generation | DM1 (Slave to master and master to slave) | Packet = 8000 bits (including MAC Header) ACK = 112 bits |
| Offered Load | 30% of capacity | 30% of capacity |
| Interarrival mean |  |  |
| Placement: (x, y) coordinates in meters | 4 BT masters at (-1,0.5), (1,0.5), (-1,-0.5) and (1,-0.5) meters and the 4 corresponding BT slaves at (-0.5, 1), (0.5,1), (-0.5,-1), and (0.5,-1). | WLAN Source: (0,15) m 4 WLAN AP: (-1,-1)m, (1,1)m, (-1,1)m, (1,-1)m |
| Number of devices | 4 pairs | 5 (1 AP , 4 STA) |
| Bluetooth (MAC) Parameters |  |  |
| Mode of Operation | Connection | Connection |
| Other Parameters |  |  |
| Data Rate (Mbps) | 1 Mbit/s | 1) Direct Sequence Spread Sprectrum at 11, and 1 Mbits/s 2) Frequency Hopping at 1 Mbits/s |
| General Simulation Parameters |  |  |
| Processing delay (at transmitter and receiver including segmentation and reassembly) | 0 ms | 0 ms |
| Length of Simulation Run | 30 seconds | 30 seconds |
| Length of Run Prior to Gathering Statistics | 5% of simulated time | 5% of simulated time |

Case 3: Factory/ Office Environment Scenario, 3 WLAN Systems, 1 BT piconets

Figure 3: Office Environment

|  | Bluetooth | IEEE 802.11b |
| --- | --- | --- |
| Traffic | Fixed size packet, exponential interarrivals | Fixed size packet, exponential interarrivals |
| Baseband Packet generation | DM1 (Slave to master and master to slave) | Packet = 8002 bits (including MAC Header) ACK = 112 bits |
| Offered Load | 30% of capacity | 30% of capacity |
| Interarrival mean |  |  |
| Placement: (x, y) coordinates in meters | 1 BT master at (0.5,0.5), and its BT slave at (-0.5, -0.5). | WLAN Sources: (-1,-1), (0,-1),(1,-1) and 1 WLAN AP at (1,1), (0,1) and (1,1). |
| Number of devices | 1 pair | 6 (3 AP , 3 STA) |
| Bluetooth (MAC) Parameters |  |  |
| Mode of Operation | Connection | Connection |
| Other Parameters |  |  |
| Data Rate (Mbps) | 1 Mbit/s | 1) Direct Sequence Spread Sprectrum at 11, and 1 Mbits/s – Each AP/Source pair is on a different center channel (non-overlapping bands) 2) Frequency Hopping at 1 Mbits/s |
| General Simulation Parameters |  |  |
| Processing delay (at transmitter and receiver including segmentation and reassembly) | 0 ms | 0 ms |
| Length of Simulation Run | 30 seconds | 30 seconds |
| Length of Run Prior to Gathering Statistics | 5% of simulated time | 5% of simulated time |