IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Pseudo-code of New Simple Adaptive Hopping approach described in 01/435r0 |  |
| Date Submitted | [18 September, 2001] |  |
| Source | [H. Gan, V. Sapozhnykov, B. Treister, E. Skafidas, et. al. ] [Bandspeed Inc. ] [Level 9, 500 Collins St., Melbourne, Australia] | Voice: [61 3 9614 6299] Fax: [61 3 9614 6699] E-mail: [h.gan, v.sapozhnykov, b.treister, e.skafidas@bandspeed.com.au] |
| Re: | Pseudo-code of a new simple adaptive hopping approach. The detailed approach was described in 01/435r0. |  |
| Abstract | [This document present the pseudo-code for a new simple adaptive hopping approach described in document 01/435r0] |  |
| Purpose | [The pseudo-code of new simple AFH is for review by P802.15.2] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

# Pseudo-Code for New Simple AFH

Note:

- No partition sequence generator is needed.

- Random Number Generator seamlessly integrated

Definition:

| G | Good channel |
| --- | --- |
| BN | Bad channel to remove |
| BK | Bad channel to keep |
| Classification Table | A table showing each channel’s classification, whether it is G, BK or BN |

Variables

| Variable | Description | Size |
| --- | --- | --- |
| NG | Number of ‘Good’ channels | Scalar |
| NBK | Number of ‘Bad-to-Keep’ channels | Scalar |
| NGBK | NG + NBK | Scalar |
| SG | Bank of ‘Good’ channels | Array, [0, ..., NG-1] |
| SBK | Bank of ‘Bad-to-Keep’ channels | Array, [0, ..., NBK-1] |
| SGBK | SGBK = SG + SBK | Array, [0, …, NGBK – 1] |
| SG (i) | ith channel of SG |  |
| SBK (i) | ith channel of SBK |  |
| SGBK (i) | ith channel of SGBK |  |
| Fnext | Next hop frequency – the output of selection box | Scalar |
| FMTX | Hop frequency transmitted by master at last time slot | Scalar |
| Fadp | Re-mapped hop frequency, Fadp  SG or SBK | Scalar |
| CLK1-0 | Bit 0 and 1 of Bluetooth Master Clock | Scalar |
| RAND | Generated pseudo-random number at a timeslot | Scalar |

/* Master Tx / Slave Rx timeslot – Even-numbered time slot  */

If (CLK1-0 = 11) Then,

/* Check classification of Fnext in the Classification Table */

If (Classification of Fnext is BN ), Then

/* Select a ‘Good’ or ‘BK’ channel randomly from SGBK, to maintain even usage of  ‘Good’ and ‘Bad to keep’ channels */

Fadp = SGBK(RAND mod NGBK )

Else   /*Classification of Fnext is G or BK */

Fadp = Fnext          / If F next is BK , Master may not transmit at all, just as scheduling */

End

End

-----------------------------------------------------------------------------------------------------------------------------------------

/* Slave Tx / Master Rx timeslot – Odd- numbered time slot */

If (CLK1-0 = 01) Then,

/* Check classification of Fnext in the Classification Table */

If (Classification of Fnext is BN), Then

/* Check what kind of channel FMTX WAS in last master Tx timeslot */

If (FMTX WAS BK), Then

Fadp = SBK(RAND mod NBK ) /* Select a ‘BK’ channel randomly from the SBK * /

Else, /*FMTX WAS a good channel G*/

Fadp = SG(RAND mod NG ) /*Select a ‘Good’ channel randomly from the SG */

End

End

If (Classification of Fnext is BK), Then

/* Check what kind of channel FMTX WAS in last master Tx timeslot */

If (FMTX WAS BK), Then

Fadp = Fnext

Else, /*FMTX WAS a good channel G*/

Fadp = SG(RAND mod NG ) /*Select a ‘Good’ channel randomly from the SG */

End

End

If (Classification of Fnext is G), Then

/* Check what kind of channel FMTX WAS in last master Tx timeslot */

If (FMTX WAS BK), Then

Fadp = SBK(RAND mod NBK ) /* Select a ‘BK’ channel randomly from the SBK */

Else, /*FMTX WAS a good channel G*/

Fadp = Fnext

End

End

##### End

When NO BK, Case 1, 3 ,4, 6 are processed

When NO BN, Case 1, 2, 10, 11 are processed

In case Master TX BN

replaced with a BK

In case Master TX BN

replaced with a G

Appendix: Processing channel pair cases, processed on a per channel pair basis on the fly