IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | (Combined) Mobilian Text for Clause 6 |  |
| Date Submitted | [12 September, 2001] |  |
| Source | [Jim Lansford, Adrian P Stephens] [Mobilian Corporation] [7431 NW Evergreen Pkwy, Hillsboro, OR 97124, USA] [Robert E. Van Dyck, Amir Soltanian] [NIST] [100 Bureau Drive, MS 8920] [Gaithersburg, MD 20899] | Voice: [ +1 (503) 681-8600 ] Fax: [ ] E-mail: [ jim.lansford@mobilian.com adrian.stephens@mobilian.com ] Voice: [(301) 975-2923] Fax: [(301) 590-0932] E-mail: [{vandyck,amirs}@antd.nist.gov |
| Re: | 00308r0P802-15_TG2-Stage0_PHY_Model.ppt 00360r0P802-15_TG2-Mobilian-coexistence-proposal.ppt 01164r0P802-15_TG2-Mobilian-Symbol-802.11-Bluetooth.ppt 01172r0P802-15_TG2-Mobilian-Symbol-summary.ppt 01324r1P802-15_TG2-Clause6.doc |  |
| Abstract | This document defines the analytical and simulation models for the BER resulting from interference between 802.11b and 802.15.1 transmissions. It also includes some introduction of the physical layer concepts and defines the path loss model used. |  |
| Purpose | This document contains draft text for the 802.15.2 recommended practice. |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Note, comments by the author that are not intended to be part of the final document are introduced by: “[ed – “.

APS comments on draft of 13/9/01 marked thus

# Physical Layer Models

The outline of this clause is as follows.   Section 6.1 introduces concepts that are useful for understanding the physical layer models, while Section 6.2 gives the path loss model.

Section 6.3 describes an analytical model that is suitable for extended MAC-layer simulations.

Section 6.4 discusses a simulation-based model that is more accurate but also more computationally intensive.  Presently, the results provided by the two models are not directly compared because of different definitions of signal-to-interference ratio.

## Physical Layer Model Concepts

This section introduces concepts that are common to the physical models described in this clause. The most powerful simplifying concept in this model is the period of stationarity (POS).  This is the period over which the parameters defining the transmissions of the devices being modeled do not change.

Consider the example shown in Figure 1.  Here an 802.15.1 device transmits two packets. An 802.11b device transmits a single PHY protocol data unit (PPDU) using 11Mbps modulation type for the PHY service data unit (PSDU).  The start of the PHY layer convergence protocol (PLCP) header overlaps the end of the first 802.15.1 packet.  The end of the PSDU overlaps the start of the second 802.15.1 packet.  There are six periods of stationarity.  Note that a new POS starts at the end of the PLCP header because the modulation type changes at this point.

Figure 1 - Example showing Periods of Stationarity

By definition, during the POS the transmit power and modulation type do not change, and the position of the devices (and hence link loss) is constant. So receiving nodes experience constant signal, noise and interference powers from which a BER value can be calculated or simulated.

## Path Loss Model

The path loss we use is given by Equation 1 and shown in Figure 2.  This path loss model is described in [ed- cite Kamerman, IEEE 802.11-00/162, “Coexistence between Bluetooth and IEEE 802.11 CCK Solutions to avoid mutual interference”].  Path loss follows free-space propagation (coefficient is 2) up to 8m and then attenuates more rapidly (with a coefficient of 3.3).

Note that the model does not apply below about 0.5m due to near-field and other implementation effects.

Equation 1- Path Loss versus distance (m)

| Path Loss =40.2+20log(d), | d < 8m |
| --- | --- |
| Path Loss =58.5+33log(d8), | d > 8m |

Figure 2- Path Loss (dB) versus distance (m)

## Analytical model for IEEE 802.11b and IEEE 802.15.1 transmissions

### Introduction

This section describes the analytical model that allows the bit error rate (BER) to be calculated for 802.11b and 802.15.1 packets in the presence of mutual interference.

### Model Interface

The model is supplied with device positions and transmission parameters.  The model calculates the BER derived from those parameters.

The parameters described in Table 1 are supplied to the PHY model for each transmission that is active during a POS.

Table 1 - Transmission Parameters

| Field | Description |
| --- | --- |
| Source Position | Device position specified using Cartesian Coordinates |
| Destination Position |  |
| Modulation Type | Type of modulation used by the transmitter. One of: - 802.15.1 - 802.11b 11Mbps - 802.11b 5.5 Mbps - 802.11b 1Mbps - 802.11b 2Mbps |
| Transmit Power | Transmit power |
| Frequency | Center frequency of transmission |

The output of the PHY model is a BER value at the receiver of each transmission.

### BER Calculation

Figure 3 shows the BER calculation in diagrammatic form.

The intended transmission is attenuated by the path loss (defined in section 6.2) to the receiver.  This is the signal power at the receiver.  Each interfering transmission is attenuated by its path loss to the receiver and by the spectrum factor (defined in 6.3.4) to account for the combined effect of receiver and transmitter masks and frequency offset.  The resulting interference powers are added to give the total interference power.  The signal to noise and interference (SNIR) value is the ratio of signal to total interference power at the receiver.  The BER is calculated from the SNIR as defined in 6.3.7.

Figure 3 - BER Calculation

### Spectrum Factor

The spectrum factor represents the combined effects of transmitter and receiver masks (defined in 6.3.6) and frequency offset.  It also includes the effect of any CCK coding gain.

There are four possible combinations of transmitter and receiver.  The spectrum factor is defined to be unity for like transmitter and receiver with a zero frequency offset.  Other spectrum factors are defined by Table 2 to Table 4.

Table 2 - Spectrum Factor Values for 802.15.1 Receiver

| Transmitter | Receiver | Frequency offset (d) in MHz |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | Other |
| 802.15.1 | 802.15.1 | 8.9433E-02 | 1.7943E-04 | 8.9433E-06 | 7.9433E-06 |
| 802.11b | 802.15.1 | 8.0433E-2 | 1.0794E-03 | 1.0079E-03, d<=11 1.7943E-05, d>11 |  |

Table 3 - Spectrum Factor Values for 802.15.1 to 802.11b

| Transmitter | Receiver | Frequency offset (d) in MHz |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 12 | 13 | 14 | Other |
| 802.15.1 | 802.11b (1 & 2 Mbps) | 7.3197E-02 | 3.5219E-04 | 2.5219E-04 , d=14 | 1.0, d<12 2.5119E-04, d<23 2.5119E-06, d>=23 |
| 802.15.1 | 802.11b (5.5 & 11 Mbps) | 1.1601E-02 | 5.5818E-05 | 3.9969E-05 | 1.0, d<12 3.9811E-05, d<23 3.9811E-07, d>=23 |

Table 4 - Spectrum Factor Values for 802.11b Self Interference

| Transmitter | Receiver | Frequency offset (d) in MHz |  |  |
| --- | --- | --- | --- | --- |
|  |  | 11 | 22 | Other |
| 802.11b | 802.11b | 0.5 | 6.7360E-03 | 1.2512E-05 |

### SNIR Computation

The SNIR is given by the ratio of the received signal power to the total received interference power.  Note that the powers are calculated after the spectrum factor has been applied, and so this ratio corresponds to the value after the receiver filter.

Receiver noise is not considered in this model.

### Transmit and Receive Masks

The transmit and receive masks used are defined in Table 5 and shown in Figure 4.

Table 5 - Transmit and Receive masks

|  | Transmit | Receive |
| --- | --- | --- |
| 802.15.1 | Frequency Offset (MHz) Attenuation (dB) 0 0 1 -20 2 -40 3 -60 4 and greater full attenuation | Frequency Offset (MHz) Attenuation (dB) 0 0 1 -11 2 -41 3 and greater -51 |
| 802.11b | Frequency Offset (MHz) Attenuation (dB) 0 0 1 to 11 -30 12 and greater -50 | Frequency Offset (MHz) Attenuation (dB) 0 0 1 to 12 -12 13 to 36 -36 37 and greater -56 |

|  | Transmit |  | Receive |
| --- | --- | --- | --- |
| 802.15.1 |  |  |  |
| 802.11b |  |  |  |

Figure 4 - Transmit and Receive masks for 802.15.1 and 802.11b

When calculating the effects of an 802.15.1 packet on an 802.11b 5.5Mbps or 11Mbps packet, a CCK coding gain of 8dB is used.

### BER calculation based on SNIR

The Symbol Error Rate (SER) is calculated for each modulation type based on the SNIR at the receiver.  This is then converted into an effective BER given the number of bits per symbol.

The sections that follow describe the BER calculation for the different modulation types.

#### BER calculation for 802.15.1 Modulation

Assuming envelope detection of orthogonal FSK, the BER is given directly by.

BER802.15.1 =0.5×exp(−SNIR2)

#### BER Calculation for 802.11b 1Mbps

The probability of error in a symbol in the presence of AWGN is given by:

P=Qd22N0

Where d is the minimum distance between any two points in the signal constellation and N0 is the in-band noise power at the receiver. The Q function is defined in section 6.3.7.6.

In the case of a 802.11b 11Mbps chip, the modulation scheme is differential BPSK.  This has the effect of doubling the effective noise power at the receiver .

PDBPSK−CHIP=Qd24NC, where NC is the noise energy per chip.

The value of d can be determined by plotting the modulation constellation of BPSK placing the signal points at a distance of ECfrom the origin, where EC is the received signal energy per chip.  Thus dDBPSK-CHIP = 2EC. So now:

PDBPSK−CHIP=QECNC

This is the probability of an error in an individual 11Mbps chip.

To include the effect of the spreading code, the squared distance is summed over each chip.  In the case of 802.15.1 1Mbps modulation, the 11-chip spreading code results in the squared distance being multiplied by a factor of 11.

Giving P1MBPS−SYMBOL=Q11×SNIR, where SNIR = EC/NC.

This is the 1Mbps symbol error rate.  It is also the 1Mbps BER, because each symbol encodes a single bit.

#### BER Calculation for 802.11b 2Mbps

This calculation follows the treatment for the 1Mbps calculation with a few differences.

The 2Mbps rate uses 11Mbps DQPSK chips.  The minimum distance between points in the QPSK constellation is reduced by a factor of 2 (compared to BPSK) giving dQBPSK-CHIP = 2EC.

This substitution results inP2MBPS−SYMBOL=Q5.5×SNIR.

Each 2Mbps symbol encodes two bits.  However, because the symbols are gray coded, a decoding error between adjacent DQPSK constellation points yields only a single bit error in the decoded 2Mbps bitstream .  So this symbol error rate is also the required BER.

#### 802.11b 5.5 Mbps BER Calculation

The symbol error rate can be determined by treating the modulation as a block code in the presence of AWGN interference. The general symbol error rate isQ2×SNIR×Rc×Wm, where Rc is the code rate, Wm is the codeword distance and the sum is over all other codewords.

For 802.11b 5.5 Mbps, the symbol error rate, SER5.5, is given by:

SER5.5 =14×Q8×SNIR+Q16×SNIR

As each symbol encodes 4 bits, the effective BER is

BER=min1−(1−SER5.5)14,0.5

in which the value has also been limited to 0.5 .

#### 802.11b 11 Mbps BER Calculation

For 802.11b 11 Mbps, the symbol error rate, SER11 is given by:

SER11=24×Q4×SNIR+16×Q6×SNIR+174×Q8×SNIR+16×Q10×SNIR+24×Q12×SNIR+Q16×SNIR

As each symbol encodes 8 bits, the effective BER is

BER=min1−(1−SER11)18,0.5

#### Q Function Definition

The Q function is defined as the area under the tail of the Gaussian probability density function with zero mean and unit variance.

Q(x)=12πx∞e−t22dt

In this model, a fifth-order approximation to Q(x) is used:

Q(x)=12πexp(−x22)×8+9x2+x415x+10x3+x5

#### SNIR Limits

The simulation is simplified by assuming that above a certain SNIR the BER is effectively zero and below a certain SNIR the BER is effectively 0.5.  These limits are defined in Table 6.

Table 6 - Assumed Limits on SNIR

| Receiver | Upper limit on SNIR | Lower limit on SNIR |
| --- | --- | --- |
| 802.11b | 10dB | -3dB |
| 802.15.1 | 20dB | 1dB |

#### BER versus SNIR Results

Refer to section ??? for a presentation of the results of the analytical model.

#### Section ??? BER versus SNIR Results

(Ed - please move into results sections in merged document)

Figure 5 shows the results of calculating BER for SNIR values in the range –15 to 20dB for each modulation type .

Figure 5 – BER versus SNIR for 802.11b and 802.15.1 Modulation Types

## Physical Layer Simulations

### Introduction

Aps:  I wonder if some of this description should move up into a common section before the analytical modeling section.

In this subsection, we discuss the modeling of the physical layers of the IEEE 802.15.1 (Bluetooth) and IEEE 802.11b (WLAN) systems, and then we examine their bit error rate performances in interference-limited environments.    Complex baseband models are used for both Bluetooth and WLAN, and the performance is determined using Monte Carlo simulation methods.   The resulting performance curves are quite accurate, but they are obtained at the expense of significant computation.   While the analytical model uses transmitter power and distance as input parameters, the simulation model uses the signal-to-noise ratio (SNR) and the signal-to-interference ratio (SIR).   In both cases, the output is bit error rate (BER).

The outline of the section is as follows: Section 6.4.2 describes the model for Bluetooth, while Section 6.4.3 does the same for 802.11b.    Section 6.4.4 contains results for the 802.11b system in the presence of interference from Bluetooth, and Section 6.4.5 provides the results for Bluetooth in the presence of an 802.11b interferer.    Some of the text and the figures have been taken from~\cite{soltanian:01}, which also contains additional results for flat fading channels.

The Bluetooth system operates at a channel bit rate of 1 Mbit/sec~\cite{haartsen:00,bluetooth:99}.

The modulation is Gaussian frequency shift keying (GFSK) with a nominal modulation index of hf=0.32 and a normalized bandwidth of BT=0.5, where B is the 3 dB Bandwidth of the transmitter's Gaussian low pass filter, and T is the bit period.  The Bluetooth radio employs a frequency hopping scheme in which the carrier frequency is changed on a packet by packet basis.  There are up to 79 different channels, each with 1 MHz separation. The entire structure of the simulated system is presented in Fig.~\ref{fig:BTawgn}. It includes the transmitter, the channel, the receiver and the interference source.    Note that the interferer can be set to have a different carrier frequency and a random phase offset.

\epsffile{fig/virBTAWGN.eps}

\caption{Bluetooth system model.}

\label{fig:BTawgn}

The 1 and 11 Mbits/sec modes of the IEEE 802.11b standard~\cite{ieee802:01} are implemented.  The first rate is achieved by using differential BPSK (DBPSK) with DSSS and an 11 chip Barker code; the chip rate is 11 Mchips/sec.  The last rate is obtained using complementary code keying (CCK), also at 11 Mchips/sec.  The communications system model for the 1 Mbit/sec bit rate is presented in Fig.~\ref{fig:802awgn}, again consisting of the transmitter, the channel, the receiver and the Bluetooth interference source. We explain the details of this model in the following sections.   The CCK system is shown in Fig.~\ref{fig:blkcck} and discussed in Section 6.4.3.2 below.

\epsffile{fig/vir802AWGN.eps}

\caption{802.11b DSSS system model.}

\label{fig:802awgn}

### IEEE 802.15.1 System Model

\label{sec:bluetooth}

#### The GFSK signal

The GFSK signal can be represented by~\cite{murota:81, aulin:81, steele:96}

s(t,α)=Acos(2πfct+φ(t,α)),     \label{GFSKBP}

where A=2EbT, Eb is the energy per data bit, and fc is the carrier frequency;

aps: what is T?

αis the random input stream, comprised of the data bits αi. φ(t,α) is the output phase deviation, given by

φ(t,α)=2πhfi=n−L+1nαiq(t−iT)+πhfi=−∞n−Lαi,

where q(t)=−∞tg(τ)dτ, and g(t) is the Gaussian-shaped pulse of the transmitter filter.

L is the length of g(t), and it determines the number of consecutive symbols required to transmit a single data bit.    Sending a data bit over multiple symbols makes GFSK a partial response symbol, which reduces the required bandwidth. For Bluetooth with BT=0.5, we have L=2, which means that a single data bit is spread over two consecutive symbol intervals.

#### Interference Model

\label{sec:inter}

Either a Bluetooth or an 802.11b interference signal can be represented as

SI(t,b)=Bcos(2π(fc+fd)t+φ2(t,b)),          \label{intGFSKBP}

where b is the random input data, which is independent of  α, and φ2 depends on the type of the interferer.  fdis the frequency difference between the desired signal and the interference.  The Bluetooth radio channels are 1 MHz apart, so fd can take values of 0,1,2,⋯. MHz. The bandwidth of the 802.11b system is 22 MHz, so we carried out simulations for fd≤11MHz.  The sampling rate is Ns=44samples/bit, which equals 4 samples/chip for the 802.11 DSSS system.  This sampling rate is appropriate for fdup to 22 MHz.  A uniform random delay td=[0,T) and a random phase φd∈[0,2π) are applied to the interferer signal for each packet.  It should be noted that the interference model is strictly concerned with the physical layer, and so it contains neither forward error correction nor retransmission protocols.

#### Limiter-Discriminator with Integrate and Dump (LDI) Receiver

This receiver consists of a pre-detection bandpass filter, a limiter-discriminator, and an integrate and dump filter, as shown in Fig.~\ref{fig:BTawgn}.  The final block is the hard limiter, which compares the output phase with a decision level. The pre-detection bandpass filter is a Gaussian filter with an equivalent lowpass impulse response, hr(t), given by~\cite{simon:84}

hr(t)=2πln2Bre−(2π2ln2)(Brt)2,

where Bris the 3 dB bandwidth.  According to Simon and Wang~\cite{simon:84}, the optimum bandwidth for this filter is BIF=2Br=1.1T.  The discrete impulse response of this filter is obtained by sampling and truncating hr(t).  The output of the receiver pre-detection filter can be represented using its inphase and quadrature components.   For a discussion of a digital implementation of this receiver,

please see~\cite{soltanian:01}.

### 802.11b System Model

\label{sec:w80211}

#### 1 Mbit/sec DSSS

The basic 1 Mbit/sec rate is encoded using DBPSK.   Thus, it is not necessary to have a coherent phase reference in the receiver to demodulate the received signal.

This system utilizes a spread spectrum scheme to mitigate the effect of interference. The Barker sequence with code length P = 11 is employed to spread the signal.  The bit duration, T, is exactly 11 chip periods, Tc, long.  The processing gain (PG) of this system is~\cite{proakis:95} PG=RcRb=11, where  Rb=1T is the bit rate, and Rc=1Tc is the chip rate. If we calculate the power spectrum of the Barker codes, we get~\cite{miller:98}

S(f)=k=−∞k≠0∞(P+1P2)sinc2(kP)δ(f−kPTc)+1P2δ(f).     \label{barkersp}

The function, S(f), is illustrated in Fig.~\ref{fig:barker} for P=11.  We see that a narrowband interference signal -like Bluetooth- located at the middle of the spectrum will be more attenuated than an interferer located 1 MHz away.

Ed. Note 1: replot Figure.~\ref{fig:barker} so that the points are not connected.   Also, make the y axis normalized and in dB.

\epsffile{fig/barkspec.eps}

\caption{Power Spectrum of the Barker Code.}

\label{fig:barker}

As shown in Fig.~\ref{fig:802awgn}, the input data bits are first differentially encoded.   The resulting sequence is spread by the Barker code. The output of the spreader is fed to a square-root raised-cosine pulse-shaping filter. The impulse response of this filter with a roll-off factor α can be found in~\cite{miller:98}  The discrete time impulse response of this filter is obtained by sampling it.

At the receiver, the input samples are first passed through the square-root raised-cosine matched filter.  The despreading filter is a rectangular filter that integrates the output of the multiplier during a bit period.  The differential decoder compares the phase angle of the received symbol and the previous one to generate the output bit stream.  It is assumed that the chip timing of the receiver is synchronized to the transmitter.

#### 11 Mbits/sec CCK

Complementary codes were originally conceived by M. J. Golay for infrared multislit spectrometry ~\cite{golay:61}.   The complementary codes in the 802.11b standards are defined by a set of 256 symbols.   Each symbol has a duration of 8 chips.   They are specified by~\cite:ieee802:01}

c=[ej(φ1+φ2+φ3+φ4),ej(φ1+φ3+φ4),ej(φ1+φ2+φ4),−ej(φ1+φ4),ej(φ1+φ2+φ3),ej(φ1+φ3),−ej(φ1+φ2),ejφ1],

\label{CCK1}

where

φi∈0,π2,π,3π2fori=1,2,3,4.               \label{phiCCK1}

Note that each chip of a symbol is complex, and so can be transmitted using QPSK modulation as discussed below.  The symbol rate is 11/8 Msymbols/sec, giving 11 Mchips/sec.

At 11 Mbits/sec, 8 bits (d0 to d7; d0 first in time) are transmitted per symbol. The first dibit (d0, d1) encodes φ1 based on DQPSK, which provides the possibility of employing differentially-coherent detection.  We first employ a coherent receiver, assuming that the initial phase

of the signal is known.  The dibits, (d2, d3), (d4, d5), and (d6, d7) encodeφ2,φ3, and φ4, respectively, as specified in Table~\ref{tb:table1}.

| Dibit Pattern (di, di+1) | Phase |
| --- | --- |
| 00 | 0 |
| 01 | π2 |
| 10 | π |
| 11 | 3π2 |

\caption{QPSK Encoding.}      \label{tb:table1}

The system model is presented in Fig.~\ref {fig:blkcck}.   Only an AWGN channel is considered in this case.

\epsffile{fig/virCCK.eps}

\caption{CCK System Model.}

\label{fig:blkcck}

A maximum likelihood decoder determines the valid symbol that is closest to the received symbol, and it maps that symbol back to eight data bits.   This decoding method  needs a bank of 256 correlators in the receiver.  Although optimum, this method may be considered too complex for some implementations.  There are also less complex sub-optimum algorithms. By looking at the code words of CCK, one can write these equations for the decoded phases~\cite{vannee:96}

φ2=argr1r2+r3r4+r5r6+r7r8φ3=argr1r3+r2r4+r5r7+r6r8φ4=argr1r5+r2r6+r3r7+r4r8            \label{subopt}

where

r=[r1r2r3r4r5r6r7r8]          \label{subr}

is the received symbol.   We employ the above sub-optimal receiver to measure the performance in the presence of interference.

### IEEE 802.11b in the Presence of IEEE 802.15.1

Ed. Note 3:  Replot all the curves using SNR and SIR intead of CNR and CIR, respectively.

Ed. Note 4: We measure SNR and SIR at the input to the bandpass filter.  Therefore, our measurements are different from those used by Mobilian, since they define SNIR after the receiver’s filter.   We need to explicitly mention in the text where the measurements are made.  We also need to compare the two sets of results and make sure they agree

I am not in a position to make this comparison because none of the plots are visible to me in this word document.

I think there’s also a potential confusion about terminology here.  The SNR, SIR and SNIR are ratios – but we always quote dB values.  This makes “low” ratios become negative.  Perhaps a note would clarify matters.

Now, we consider the performance of the 1 Mbit/sec 802.11b system, in an interference-limited environment with SNR = 35 dB.  Note that we measure both SNR and SIR at the input to the receiver’s bandpass filter.   Since the system takes advantage of DSSS, one observes in

Fig.~\ref{fig:bt8021} that for co-channel interference, SIR = -11 dB is adequate to suppress the effect of interference (BER 10−2).  The most disturbing interference is located at fd=1MHz, which needs a minimum SIR of -5 dB. This difference stems from the null at the middle of the spectrum of the Barker code as described before. For frequency offsets greater than 8 MHz, the SIR value must be very low in order to get a high BER. This fact is due to the bandpass filter in the 802.11b receiver having high attenuation at frequencies near 11 MHz.

\epsffile{fig/bt802curve1.eps}

\caption{1 Mbit/sec 802.11b DSSS performance with Bluetooth interference. AWGN channel.

SNR=35 dB.}

\label{fig:bt8021}

Fig.~\ref{fig:cckawgn} shows the performance of the 11 Mbits/sec 802.11b CCK receiver in the AWGN channel. The optimum receiver performs about 2 dB better than QPSK, and the sub-optimum method is nearly the same as QPSK.  The sub-optimal system provides a BER of 10−3 for an EbNo=8 dB.  It must be noted that CCK was designed explicitly for fading channels, where its gain over QPSK is much more significant.

\epsffile{fig/cckawgn.eps}

\caption{11 Mbits/sec 802.11b CCK performance in an AWGN channel.}

\label{fig:cckawgn}

Figure~\ref{fig:cckBT} illustrates the performance of the 11 Mbits/sec IEEE 802.11b receiver with Bluetooth interference.  This figure indicates that the CCK modulation is more vulnerable to the interference signal than the 1 Mbit/sec DSSS.   A minimum SIR of 3 dB must be achieved to get BER=10−2for all frequency offsets.   This result is not surprising, since the CCK provides a higher bit rate but occupies the same 22 MHz bandwidth, thereby having less of a coding gain.   Generally, the receivers used for both 1 Mbit/sec and 11 Mbits/sec are fairly simple, and improved performance can most likely be obtained using more sophisticated signal processing.   This fact is especially true for the 11 Mbits/sec CCK system.

\epsffile{fig/ccicckcurve.eps}

\caption{11 Mbits/sec 802.11b CCK performance with Bluetooth

interference.  SNR=35 dB}

\label{fig:cckBT}

### IEEE 802.15.1 in the Presence of IEEE 802.11b

\label{sec:Perfo}

The LDI receiver design meets the Bluetooth on Bluetooth interference specifications.    While this model is not based on any particular implementation, it is meant to be indicative of a real implementation.  Simulation results for the LDI receiver in the AWGN and Rician channels are presented in~\cite{soltanian:01}.

Here, we study the performance of IEEE 802.15.1 with 802.11b interference.  Again, we measure SNR and SIR at the input to the receiver’s bandpass filter.   The curves in Figs.~\ref{fig:i802AWGN} are for an interference-limited environment with SNR = 30 dB. The 802.11b signal looks like broadband noise at the input to the Bluetooth receiver.  The performance degradation for carrier frequency differences up to 4 MHz is almost the same, and so we plot the results forfd=0 as a representative case.  The null in the Barker code spectrum does not improve the performance here, as it does for the 802.11b DSSS system.  After 4 MHz, one gradually sees the effect of the pulse shaping filter of the 802.11b transmitter, which has a null at fd=11 MHz.  In fact, the SIR value at fd=11MHz has to be very low in order to cause high BER.

\epsffile{fig/AWGN802onBT.eps}

\caption{Bluetooth performance with 802.11b interference.  AWGN channel. LDI receiver.

SNR=30 dB.}

\label{fig:i802AWGN}

The roll-off factor αof the 802.11b transmitter determines the range of frequency offsets over which high BERs are observed.  In this simulation, we chose  α=1, so the interference signal will occupy the maximum available bandwidth.  Another observation from Fig.~\ref{fig:i802AWGN} is that if the SIR value is always greater than 6 dB, the BER for all frequency offsets is less than  10−3.    Note that as an interferer the bit rate of the 802.11b physical layer is not important to the performance.

As a solution to mitigate the effect of interference, we use a simple two-state Viterbi receiver for Bluetooth.    It must be noted that the main problem with this Viterbi receiver is that is assumes that the modulation index is known.   Unfortunately, the actual modulation index is allowed to vary over a large range.   Again, we assume that the phase of the transmitted signal is known to the receiver, and we measure the SNR and SIR at the input the receiver’s matched filter.   The performance for 802.11 interference is shown in Fig.~\ref{fig:viterbi1}.  A dramatic enhancement is observed in this figure, evi

dently at a cost of having a more complicated receiver.

\epsffile{fig/vtbcoh802BT.eps}

\caption{ Bluetooth Viterbi receiver performance with 802.11b interference. AWGN channel.

SNR = 30 db}

\label{fig:viterbi1}

Bibliography or References

\bibitem{soltanian:01}   A. Soltanian and R. E. Van Dyck, “Physical layer performance for coexistence of Bluetooth and IEEE 802.11b,” Proc. 11th Virginia Tech/MPRG Symposium on Wireless Personnal Communications, Blacksburg, VA,  pp. 31-41, June 6-8, 2001.

\bibitem{haartsen:00} J. C. Haartsen and S. Mattisson, ``Bluetooth - A new low-power radio interface providing short-range connectivity,'' Proc. of the IEEE, vol. 88, no. 10, pp. 1651-1661, Oct. 2000.

\bibitem{bluetooth:99} Bluetooth Special Interest Group,  Specifications of the Bluetooth System, vol. 1, v.1.0B, Dec. 1999.   Available : http:/www.bluetooth.com.

\bibitem{ieee802:01} IEEE Std. 802-11, IEEE Standard for Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specification,  2001 Edition.

\bibitem{murota:81} K. Murota and K. Hirade, “GMSK modulation for digital mobile radio telephony,” IEEE Trans. on Comm., Vol. 29, pp. 1044-1050, July 1981.

\bibitem{aulin:81} T. Aulin and C-E. Sundberg, “Continuous phase modulation – Part 2: Partial response signaling,” IEEE Trans. on Comm., Vol. 29, pp. 210-225, Mar. 1981.

\bibitem{steele:96} R. Steele (Ed.),  Mobile Radio Communications, John Wiley & Sons Inc., 1996.

\bibitem{simon:84} M. K. Simon and C. C. Wang, ``Differential detection of Gaussian MSK in a mobile radio environment, ''IEEE Trans. Veh. Tech., pp. 307-320, Nov. 1984.

\bibitem{oppenheim:89} A. L. Oppenheim and R. W. Schafer,  Discrete-Time Signal Processing, Prentice Hall, 1989.

\bibitem{proakis:95} J. G. Proakis,  Digital Communications, McGraw-Hill, 1995.

\bibitem{miller:98} J. S. Lee and L. E. Miller,  CDMA Engineering Handbook, Artech House, 1998.

\bibitem{golay:61} M. J. E. Golay,``Complementary series,'' IRE Trans. Information Theory, vol. IT-7, pp. 82-87, Apr. 1961.

\bibitem{vannee:96} R. D. J. Van Nee, ``OFDM codes for peak-to-average power reduction and error correction,'' Proc. IEEE Global Telecommun. Conf., vol. 1, London, pp. 740-744, Nov. 1996.

\bibitem{schilling:80} D. L. Schilling, L. B. Milstein, R. L. Pickholtz and  R. W. Brown, ``Optimization of the processing gain of an M-ary direct sequence spread spectrum communication system,'' IEEE Trans. Comm.,  pp. 1389-1398, Aug. 1980.

\bibitem{milstein:82} L. B. Milstein, S. Davidovici, and D. L. Schilling, ``The effect of multiple-tone interfering signals on a direct sequence spread spectrum communication system,'' IEEE Trans. Comm., vol. 30, pp. 436-446, Mar. 1982.

\bibitem{simon:83} M. K. Simon and C. C. Wang, ``Differential versus limiter discriminator detection of narrow-band FM,''  IEEE Trans. Comm., pp. 1227-1234, Nov. 1983.

\bibitem{halford:99} S. Halford, K. Halford and M. Webster, ``Complementary code keying for RAKE-based indoor wireless communications,''  Proc. IEEE Int. Conf. on Circuits and Systems, pp. 427-430, May 1999.

# Annex ??? – Source Code for Analytical Model

This section contains source code for the Analytical Model.

/*---------------------------------------------------------------------------

* Title:    Analytical Physical-Layer Model for 802.15.2 BER Calculations

* Authors:  Ron Nevo, Josie Ammer, Adrian Stephens

*           Mobilian Corporation 2001

*

* This module contains the analytical PHY-layer model used to calculate

* BER values for 802.11b and 802.15.1 transmissions in the presence of

* mutual interference.

*---------------------------------------------------------------------------*/

/*-- Standard Includes ------------------------------------------------------*/

#include <stdio.h>

#include <math.h>

#include <stdlib.h>

#include <limits.h>

/*-- Type Definitions -------------------------------------------------------*/

typedef enum

{

WPAN, WLAN11, WLAN55, WLAN1, WLAN2

// WPAN is non-FEC WPAN transmissions

// WLAN11 is 11Mb/s 802.11

// WLAN55 is 5.5Mb/s 802.11

// WLAN1 is 1Mb/s 802.11

// WLAN2 is 2Mb/s 802.11

} ModulationType;

struct Node

{

double x,y;  // x and y positions in meters

};

typedef Node *aNodePtr;

struct Transmission

{

aNodePtr src, dst;

ModulationType type;

float txpower;  // power in mW

int frequency;

// freq for WPAN is a number 1-79, is the center frequency

// freq for WLAN is number 1-79, is the center frequency

double BER;     // the resulting BER

};

typedef Transmission *aTransmissionPtr;

/*---------------------------------------------------------------------------

* Function: CalculateAnalyticalBER

*

* Description:

*              This function takes a list of transmissions and calculates

*              the BER at each receiver using the analytical model.

*

* Parameters:  n       - the number of transmissions in the list

*              tlist   - an array of Transmissions each corresponding to

*                        an active transmission

*

* Returns: in every tlist element a calculated BER value

*---------------------------------------------------------------------------*/

extern void CalculateAnalyticalBER(int n, Transmission tlist[]);

/*-- MACRO Definitions ------------------------------------------------------*/

#define WLAN_BANDWIDTH 22

#define HALF_WLAN_BANDWIDTH 11

#define PI acos(-1.0)

// assume the power of WPAN reception at the same station is unit power

#define Tc 1

#define WPAN_Tx_1 inverse_db(-20.0)

// -20dB spurious emmision per MHz from WPAN Tx when freq dif is 1MHz

#define WPAN_Tx_2 inverse_db(-40.0)

// -40dB spurious emmision per MHz from WPAN Tx when freq dif is 2MHz

#define WPAN_Tx_3 inverse_db(-60.0)

// -60dB spurious emmision per MHz from WPAN Tx when freq dif is 3MHz

#define WPAN_Tx_other 0

// no spurious emmision from WPAN Tx when freq dif > 3MHz

#define WPAN_Rx_1 inverse_db(-11.0)

// -11dB adjacent channel interference from WPAN Rx when freq dif is 1MHz

#define WPAN_Rx_2 inverse_db(-41.0)

// -41dB adjacent channel interference from WPAN Rx when freq dif is 2MHz

#define WPAN_Rx_other inverse_db(-51.0)

// -51dB adjacent channel interference from WPAN Rx when freq dif >= 3MHz

#define WLAN_Tx_next inverse_db(-30)

// -30dB spurious emmision from WLAN Tx when freq dif <= half bandwidth

#define WLAN_Tx_other inverse_db(-50.0)

// -50dB spurious emmision from WLAN Tx when freq dif > half bandwidth

#define WLAN_Rx_next inverse_db(-12.0)

// -12dB adjacent channel interference from WLAN Rx, 12 MHz

#define WLAN_Rx_middle inverse_db(-36.0)

// -60dB adjacent channel interference from WLAN Rx, between 13MHz-36MHz

#define WLAN_Rx_other inverse_db(-56.0)

// -60dB adjacent channel interference from WLAN Rx, >= 37MHz

#define CCK_factor inverse_db(-8.0)

// 8dB gain for CCK coding

#define WLAN_EbNo_perfect 10.0   // if EbNo > 10dB, perfect reception

#define WLAN_EbNo_impossible 0.5 // if EbNo < -3dB, impossible to receiver

#define WPAN_EbNo_perfect 20.0   // if EbNo > 13dB, perfect reception

#define WPAN_EbNo_impossible 1.0 // if EbNo < 0dB, impossible to receive

#define MIN_DISTANCE 0.1

// two nodes in the same spot act as if they are 0.1 apart

/*---------------------------------------------------------------------------*/

#define abs(a) ((a)>0 ? (a) : -(a))

#define sqr(x) ((x)*(x))

#define min(a,b) ((a)<(b) ? (a) : (b))

/*-- Forward References -----------------------------------------------------*/

extern int isModulationTypeWPAN(ModulationType foo);

// returns 1 if the type is WPAN

// returns 0 otherwise

extern int isModulationTypeWLAN(ModulationType foo);

// returns 1 if the type is one of the WLANs

// returns 0 otherwise

extern double SER11(double);

extern double SER55(double);

extern double WLAN_BER_11(double);

extern double WLAN_BER_55(double);

extern double WLAN_BER_1(double);

extern double WLAN_BER_2(double);

extern double WPAN_BER(double);

extern double SpectrumFactor(Transmission &Src, Transmission &Dest);

extern double Distance(Transmission &Src, Transmission &Dest);

extern double PowerDistance(Transmission &Src, Transmission &Dest);

extern double inverse_db(double);

extern double db(double);

/*-- Local Functions --------------------------------------------------------*/

int isModulationTypeWPAN(ModulationType foo)

{

switch (foo){

case WPAN: return 1;

default: return 0;

}

}

int isModulationTypeWLAN(ModulationType foo)

{

switch (foo){

case WLAN11: return 1;

case WLAN55: return 1;

case WLAN1: return 1;

case WLAN2: return 1;

default: return 0;

}

}

// compute db from real

double db(double x)

{

return(10 * log10(x));

}

// compute real from db

double inverse_db(double x)

{

return(pow(10.0, x /10));

}

// compute the Q function using approximation Q_5

double Q_5(double x)

{

double x2,x3,x4,x5,x6;

x2 = x*x;

x3 = x2*x;

x4 = x3*x;

x5 = x4*x;

x6 = x5*x;

return(exp(-x2/2) * (x4+9*x2+8) /(x5+10*x3+15*x) / sqrt(2*PI));

}

// compute the codeword error probability of 802.11b 11Mbps

double SER11(double Eb_No)

{

double res;

res = 24*Q_5(sqrt(4*Eb_No)) +

16*Q_5(sqrt(6*Eb_No)) +

174*Q_5(sqrt(8*Eb_No)) +

16*Q_5(sqrt(10*Eb_No)) +

24*Q_5(sqrt(12*Eb_No)) +

Q_5(sqrt(16*Eb_No));

return(min(res,0.99999));

}

// compute bit error rate from Eb/No for 802.11b 11Mbps

double WLAN_BER_11(double Eb_No)

{

if(Eb_No > WLAN_EbNo_perfect)

return 0;

// if Eb/No more than some threshold, perfect reception

else if(Eb_No < WLAN_EbNo_impossible)

return 0.5;

// if Eb/No less than some threshold, impossible to receive

else

return(min(1-pow((1-SER11(Eb_No)),1.0/8),0.5));

}

// compute the codeword error probability of 802.11b 5.5Mbps

double SER55(double Eb_No)

{

double res;

res = 14*Q_5(sqrt(8*Eb_No)) +

Q_5(sqrt(16*Eb_No));

return(min(res,0.99999));

}

// compute bit error rate from Eb/No for 802.11b 11Mbps

double WLAN_BER_55(double Eb_No)

{

if(Eb_No > WLAN_EbNo_perfect)

return 0;

// if Eb/No more than some threshold, perfect reception

else if(Eb_No < WLAN_EbNo_impossible)

return 0.5;

// if Eb/No less than some threshold, impossible to receive

else

return(min(1-pow(1-SER55(Eb_No),1.0/4),0.5));

}

// compute the function number of choice of k elements from n

int choose(int k,int n)

{

int i;

int res = 1;

for(i=n;i>n-k;i--)

res *= i;

for(i=1;i<=k;i++)

res /= i;

return(i);

}

// compute the BER for WLAN 1Mbps, the BER is Q(sqrt(11*2*Eb_No/2))

double WLAN_BER_1(double Eb_No)

{

if(Eb_No > WLAN_EbNo_perfect)

return 0;

// if Eb/No more than some threshold, perfect reception

else if(Eb_No < WLAN_EbNo_impossible)

return 0.5;

// if Eb/No less than some threshold, impossible to receive

else

return(min(Q_5(sqrt(11*2*Eb_No/2)),0.5));

}

// compute the BER for WLAN 2Mbps, the BER is Q(sqrt(5.5*2*Eb_No/2))

double WLAN_BER_2(double Eb_No)

{

if(Eb_No > WLAN_EbNo_perfect)

return 0;

// if Eb/No more than some threshold, perfect reception

else if(Eb_No < WLAN_EbNo_impossible)

return 0.5;

// if Eb/No less than some threshold, impossible to receive

else

return(min(Q_5(sqrt(5.5*2*Eb_No/2)),0.5));

}

// compute the BER for WPAN

double WPAN_BER(double Eb_No)

{

if(Eb_No > 20)

return 0;

// if Eb/No more than 13dB, perfect reception

else if(Eb_No < 1)

return 0.5;

// if Eb/No less than 0dB, impossible to receive

else

return(min(exp(-Eb_No/2),0.5));

}

double Distance(Transmission &Src, Transmission &Dest)

{

return(sqrt(sqr(Src.src->x-Dest.dst->x) + sqr(Src.src->y-Dest.dst->y)));

}

// power as function of distance

double PowerDistance(Transmission &Src, Transmission &Dest)

{

double power_d0,dist;

dist=Distance(Src,Dest); // calc distance function

if (dist < MIN_DISTANCE)

dist = MIN_DISTANCE;

if(isModulationTypeWPAN(Src.type)) {    // transmitter is WPAN,

power_d0 = Src.txpower;

}

else // transmitter is WLAN

if(isModulationTypeWPAN(Dest.type)) // receiver is WPAN,

power_d0 = Src.txpower/WLAN_BANDWIDTH;

else

power_d0 = Src.txpower; // receiver is WLAN

if(dist < 8)  // use 40.2+20log d for <8M power loss

return(power_d0/(pow(dist,2.0) * pow(10.0, 4.02)));

else         // use 58.5 + 33log(d/8) for >8M power loss

return(power_d0/(pow(dist/8.0,3.3) * pow(10.0, 5.85)));

}

double SpectrumFactor(Transmission &Src, Transmission &Dest)

// (int src0, int src,int dest)

// dest is listenin to src0

// dest gets interference from src

{

int freq_dif;

if(isModulationTypeWPAN(Dest.type))

if(isModulationTypeWPAN(Src.type))

{

// both src and dest WPAN

freq_dif = abs(Src.frequency - Dest.frequency);

switch(freq_dif) {

case 0:

// frequency collide

return(1.0);

case 1:

return(WPAN_Tx_1+WPAN_Rx_1);

case 2:

return(WPAN_Tx_2+WPAN_Rx_2);

case 3:

return(WPAN_Tx_3+WPAN_Rx_other);

default:

return(WPAN_Tx_other+WPAN_Rx_other);

}

}

else

{

// src WLAN and dest WPAN

if(Dest.frequency >= Src.frequency + HALF_WLAN_BANDWIDTH)

freq_dif = Dest.frequency - (Src.frequency + HALF_WLAN_BANDWIDTH) + 1;

else if(Dest.frequency <= Src.frequency - HALF_WLAN_BANDWIDTH)

freq_dif = (Src.frequency - HALF_WLAN_BANDWIDTH) - Dest.frequency + 1;

else

freq_dif = 0;

switch(freq_dif) {

case 0:

// frequency collide

return(1.0);

case 1:

return(WLAN_Tx_next+WPAN_Rx_1);

case 2:

return(WLAN_Tx_next+WPAN_Rx_2);

default:

if(freq_dif<=HALF_WLAN_BANDWIDTH)

return(WLAN_Tx_next+WPAN_Rx_other);

else

return(WLAN_Tx_other+WPAN_Rx_other);

}

}

else // dest WLAN

{

if(isModulationTypeWPAN(Src.type)) {

// src WPAN, dest WLAN

double sf;

if(Src.frequency >= Dest.frequency + HALF_WLAN_BANDWIDTH)

freq_dif = Src.frequency - (Dest.frequency + HALF_WLAN_BANDWIDTH) + 1;

else if(Src.frequency <= Dest.frequency - HALF_WLAN_BANDWIDTH)

freq_dif = (Dest.frequency - HALF_WLAN_BANDWIDTH) - Src.frequency + 1;

else

freq_dif = 0;

switch(freq_dif) {

case 0:

// frequency collide

sf = 1.0;

break;

case 1:

sf = WPAN_Tx_1+WPAN_Tx_2+WPAN_Tx_3+WPAN_Tx_other*(WLAN_BANDWIDTH-3)+WLAN_Rx_next;

break;

case 2:

sf = WPAN_Tx_2+WPAN_Tx_3+WPAN_Tx_other*(WLAN_BANDWIDTH-2)+WLAN_Rx_middle;

break;

case 3:

sf = WPAN_Tx_3+WPAN_Tx_other*(WLAN_BANDWIDTH-1)+WLAN_Rx_middle;

break;

default:

if(freq_dif<HALF_WLAN_BANDWIDTH){

sf = WPAN_Tx_other*WLAN_BANDWIDTH+WLAN_Rx_middle;

break;

}

else{

sf = WPAN_Tx_other*WLAN_BANDWIDTH+WLAN_Rx_other;

break;

}

}

if (Dest.type == WLAN11 || Dest.type == WLAN55)

sf *=CCK_factor;

return(sf);

}

else { //both src and dest WLAN

switch(abs(Src.frequency-Dest.frequency)) {

case 0:

return(1.0);

case 11:

return(0.5);

case 22:

return(WLAN_Rx_next/11+WLAN_Tx_next);

default:

return(WLAN_Rx_other+WLAN_Tx_other);

}

}

}

}

/*-- Global Function --------------------------------------------------------*/

void CalculateAnalyticalBER(int n, Transmission tlist[])

{// n should be the length of tlist

double EbNo;

for (int dst= 0; dst < n ; dst++) { //for each dest

double Eb, No;

No=0;

for (int src = 0; src < n; src++) { //calculate the power from each source

double pd,sf,pwr;

sf=SpectrumFactor(tlist[src],tlist[dst]);

pd=PowerDistance(tlist[src],tlist[dst]);

pwr=sf*pd;

#if defined(_DEBUG_)

printf("rcpt power from %d to %d = %1.5g * %1.5g = %1.5g\n", src, dst, sf, pd, pwr);

#endif

if (src==dst)

// if src and dest are from the same transmission pair,

// pwr is signal power

Eb=pwr*Tc;

else

// if not from the same transmission pair,

// pwr is interference power

No+=pwr;

}

EbNo=Eb/No; // calculate the EbNo for each dest

//need to calc BER from SNR

double ber0;

switch (tlist[dst].type) {

case WPAN:

ber0 = WPAN_BER(EbNo);

break;

case WLAN11:

ber0 = WLAN_BER_11(EbNo);

break;

case WLAN55:

ber0 = WLAN_BER_55(EbNo);

break;

case WLAN1:

ber0 = WLAN_BER_1(EbNo);

break;

case WLAN2:

ber0 = WLAN_BER_2(EbNo);

break;

default:

printf("Unknown ModulationType");

}

#if defined(_DEBUG_)

printf("EbNo for Transmission %d = %1.5g\n", dst, EbNo);

printf("BER for Transmission %d = %1.5g\n", dst, ber0);

#endif

tlist[dst].BER = ber0;

}

}

This doubling is slightly pessimistic for BPSK under conditions of high SNIR.

An alternative approach giving the same result is to consider the spreading sequence to be a block code of length 11.

Errors between adjacent DQPSK constellation points are much more likely that errors between opposing constellation points.

BER values larger than 0.5 are meaningless.

The results for 802.11b 11Mbps do not show calculated values for SNIR < -2dB due to limitations of the tools used.