IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | 01439r1P802.15-MTS_&_Static_GTS |  |
| Date Submitted | 19 Oct., 2001 |  |
| Source | William Shvodian <XtremeSpectrum> Suite 700, 8133 Leesburg Pike, Vienna, VA 22182 | Voice: 703-269-3047 Fax: 703-269-3092 E-mail: bshvodian@xtremespectrum.com |
| Re: | This document provides the detailed text for guard times. |  |
| Abstract | This document provides details on proposed changes to the 802.15.3 MAC for Management Time Slots and Static GTS Slots. |  |
| Purpose | The recommendations contained in this document are to be applied to the 802.15.3 MAC baseline. |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

1)	Management Time Slots	3

2)	Static GTS	5

3)	Private GTS Slots (was Unassigned GTS slots)	8

4)	Modified Stream Management Command	9

5)	CTA Change Bitmap Information Element	10

# Management Time Slots

## Clause 7.4.2  Piconet Synchronization IE

New Field in Table 62 called CAP Mode.  4 bits

B0 – Data in the CAP

B1 – Commands (not including Association) in the CAP

B2 – Association Command in the CAP

B3 – EPS in the CAP

Frames of each type are only allowed in the CAP if the corresponding CAP restriction bit is set to one.  If the CAP restriction bit is set to zero, those type of frames are only allowed in the CFP.  The PNC may choose to limit the types of frames in the CAP so that it can minimize the size of or eliminate the CAP altogether.

Should no CAP be default or even the only mode?

## New Clause 8.3.4 MTS slot Access

Management Time Slots (MTSs) are identical to GTSs except that the PNC address (zero) is the source or the destination address in the CTA.  A PNC can choose to use MTSs instead of the CAP for command frames.  When MTSs are used, the PNC shall ensure that sufficient MTSs are allocated to allow for transmission of commands to and from the PNC.  There can be as little as a single MTS in a superframe whose ownership changes from superframe to superframe.  At the other extreme, there can be one or more uplink and downlink MTS per associated DEV per superframe plus MTSs for associatnion.  It is up to the PNC to determine the appropriate number of MTS in a superframe the same way the PNC is responsible for choosing the CAP size if a CAP is used.  The PNC determines which DEVs to allocate MTSs to and how often.

An open MTS is an MTS where the source address in the CTA for the MTS is the broadcast address.  Any DEV associated to the piconet can attempt to send a command frame to the PNC in an open MTS.  An MTS with the association address as the SA in the CTA for the MTS is called an association MTS.  Any station not currently associated to the piconet can attempt to send an association command to the PNC in an association MTS.  Association commands are not permitted in open MTSs.  Likewise, only association commands are allowed in association MTSs.

Open MTSs enable the PNC to service a large number of DEVs with low MTS requirements using a minmum number of MTSs.   When there are few DEVs in a piconet it will be more efficient to use assigned an MTS to a DEV instead of using open MTS.  It is the PNC’s responsibility to determine how many and what type of MTSs to use.

The PNC shall assign an uplink MTS within 500 ms of a successful association command in order to support the 1 second connection target.

### 8.3.3.1 Slotted Aloha access for Open and Association MTS

Slotted Aloha is the access mechanism in an open MTS or an association MTS.  The access to an open or association MTS shall be controlled by a contention window CWa  maintained by each DEV.  The contention window shall be derived from the number a, where a is the number of retransmission attempts made by the DEV. For the first access attempt, a shall be set to 0. The size of the contention window, CWa,is defined as follows:

CWa=256    2a+1≥256

The open or association MTS used for the ath retransmission attempt shall be chosen by a uniformly distributed random integer value ra within the interval [1, CWa].  The random number generator is not specified.  The DEV shall start counting ra from the open or association MTS slot in the next superframe.  The lack of an ACK indicates the failure of the previous access attempt, but the presence of an ACK does not necessarily equal success for an association frame since all stations attempting to associate use the same unassigned SA.  See clause 8.2.5 for a description of the Association process.

This first broadcast or unassigned MTS is specified by number 'ra=1'.  The open or association MTS with number equal to ra is the slot that the DEV shall access.  The DEV shall not access the MTS before its counter has reached the open or association MTS with the number equal to ra. After receiving an ACK, a shall be reset to 0.

## Clause 8.12.3.5 Additional Traffic to EPS DEVs

Add the following paragraph:

If management time slots are used, the PNC shall assign management time slots for a device in EPS mode during the superframes when the EPS device is scheduled to be awake and the superframe before that.

# Static GTS

## Clause 7.5.21 Figure 49

Change Reason code to 4 bits.  Use 1 bits for GTS type.  3 bits reserved.

Note: Modified stream management command appears below.

## Clause 8.3.3.1

Add the following sentence:

There are two types of GTS: dynamic GTS,  and pseudo-static GTS.  The type of GTS slots are indicated in the stream management command as specified in 7.5.21.

## New Clause 8.3.3..1.1 Dynamic Guaranteed Time Slots

The PNC is free to move dynamic GTSs within the superframe on a superframe by superframe basis.  This allows the PNC the flexibility to rearrange GTS assignments to optimize the utilization of the slot assignments.  The PNC can move a dynamic GTS by simply changing the CTA parameters in the Beacon.

## New Clause 8.3.3.1.2 Pseudo-Static Guaranteed Time Slots

Pseudo static GTSs require a stream connection – non-stream GTSs cannot be pseudo-static.  Pseudo static GTSs can be moved within the CFP by the PNC, but the PNC shall notify the affected DEVs by sending them acknowledged Channel Time Grant frames with the new CTA.  As with dynamic GTSs, the PNC can rearrange pseudo static GTSs so that the GTS assignments can be optimized, but it must use the Channel Time Grant and coordinate the channel time grants with the CTAs in the beacon.

Before a pseudo static GTS is moved, the PNC shall ensure that the new position is unoccupied by another GTS.  Then, the PNC sends a directed channel time grant to the receiving DEV so that the receiving DEV is listening to both the old GTS position and the new position.  This channel time grant contains both the old and the new CTA.  If the old and the new position overlap, the CTA can be one larger CTA.

Next the PNC sends a channel time grant to the transmitting DEV that contains only the new CTA.  By moving the receiver first, the PNC ensures that no frames are lost if Channel time requests are corrupted.

Lastly, the PNC sends a channel time grant to the receiving DEV which only contains the new CTA.

Figure 1 CTA Assignment Sequence for pseudo-static GTSs

DEVs can request a change in a pseudo-static Slot channel time using the Channel Time Request command.

## Modified Clause 8.3.3.2 paragraph 1 (changes in blue)

The DEVs associated with a PNC shall send their changes in channel time requirement whenever they wish to make a change. Once a request for channel time is received from a station, the PNC shall remember that as the outstanding request for every superframe until, a change in request is received from the DEV. In addition to this the PNC shall make use of the properties of the stream provided during the stream connection process.  The slot assignments within the CFP are based on the current pending requests from all the DEVs and the currently available channel time within the CFP.  The slot assignments for dynamic GTS may change from superframe to superframe as required by the PNC.  Slot assignments for pseudo-static GTS slots require directed channel time grant commands.  All of the slot assignments are broadcast in the beacon. The PNC may announce the slot assignments in directed or broadcast channel time grant command in addition to the announcement in the beacon. However, additional announcements by the PNC shall not change from what was broadcast in the beacon. The start time of all the GTSs are with reference to the start of beacon frame, whether they were announced in beacon or channel time grant command. The algorithm used to allocate the channel time and assign slots is beyond the scope of this standard. Channel time requests that are ACKed are valid until the next channel time request is made.

## Modified Clause 8.3.3.2 paragraph 4

In any superframe there may be one or more DEVs in the piconet that receives the Beacon in error. This may not happen to the same DEV all the time but may happen to different DEVs at different times depending upon their location and type of interference they are subjected to.

If a DEV did not receive the CTA information beacon correctly, it shall not access the channel dynamic GTS during CFP.  Stations with pseudo-static GTS(s) are allowed to transmit during these GTS(s) as long as the number of consecutive lost beacons is less than or equal to MaxLostBeacons.  A DEV shall stop transmitting in its pseudo-static GTS when the number of consecutive lost beacons exceeds MaxLostBeacons.

The channel time grant command gives the flexibility to the PNC to broadcast the CTA information during the superframe in addition to the beacon. This increases the chances of all DEVs obtaining the allocation information. In addition, this also provides the flexibility to the PNC to help preserve the QoS by sending directed channel time grant command to a DEV that may be experiencing more than usual channel errors during certain time segments. The PNC may use the channel statistics to decide whether to send such a directed channel time grant command. Note that when channel becomes too severe for the DEV to receive the beacon, the directed channel time grant command or the data frame itself there is little help that can be provided to that DEV through these channel time grant commands.

# Private GTS  (was formerly known as Unassigned GTS )

## New Clause 3.3.3.3 Private GTS

A private GTS is a GTS where the same DEV is the source and destination.  A private GTS is not used for communication in the piconet.  Rather, it is used to reserve channel time for some other use.  The other use may be for another 802.15.3 piconet, or a different type of network sharing the same channel.

Private GTS slots will usually be pseudo-static GTS slots, so that the slot is periodic for the other use.  A DEV requests a private GTS slot by inserting it’s own AD-AD in the source and destination address for the stream and channel time request.

# Modified Stream Management Command

Reserved Byte for max Delayed ACK frames in stream Mgt Command (Issue 310) and source/target address field in stream management command (Issue 400)

Modified Action Type and Reason Code address Issues

## Figure 49 Modified Stream Management Command

| Octets: 2 | 2 | 2 | 1 | 1 | 1 | 1 | 1 | 1 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Command type | Length (=30) | Stream request identifier | Originator AD-AD | Target AD-AD | DSAA | Max Frames (delayed ACK) | Reason code/GTS type | reserved | Stream QoS parameters |

Note: Reserved field will not be needed if the stream QoS parameters become odd after pending changes.

## New Figure Reason code / GTS Type field (after figure 50)

| 4 bits | 2 bits | 2 bits |
| --- | --- | --- |
| Reason Code | GTS Type | reserved |

## Replace line 50 and 51 of page 95 with the following:

The Originator AD-AD is the 8 bit address of the originator of the stream management command.  The Target AD-AD is the 8 bit address of the target of the stream management command.

## Add the following text after line 29, page 96:

Max Frames specifies the maximum number of frames that can be outstanding when the ACK policy for the stream is Delayed ACK.

Modify the Action Type description as follows:

-Value of ‘0’ means that this is a request for stream connection. This request is sent from the DEV that originates the stream management request to the PNC.

— Value of ‘1’ means that this is the indication frame sent from the PNC to the target of the stream management frame.  This command contains the QoS-parameters, except the retransmission window, set by the PNC.

— Value of ‘2’ means that this is a response to the stream connection sent from the target DEV to the PNC.  The target DEV can lower the QoS-paremeters in the response.

- Value of ‘3’ means that this is a Stream Management confirm command for the stream connection. This request is sent by the PNC to the originator DEV to complete the stream connection.  It is also sent from the PNC to the originator and target DEV to reject or disconnect a stream.

Modify the reason code as follows:

The reason code is a 4 bit field that is valid in the Stream Management Confirm when a stream connection is being completed, rejected or disconnected.  It is also valid in the Stream Management response from the target to the PNC.  Otherwise this field is ignored. Valid reason codes are:

- 0 ->Success

- 1->invalid stream parameters

— 2 -> non-negotiable stream parameters

— 3 -> system resources unavailable

— 4 -> bandwidth allocation failure

— 5 -> currently disassociating from the piconet

— 6 -> too many streams

— 7 -> lack of required security

- 8 -> unauthorized stream

- 9-> Stream rejected or disconnected by other DEV

— 10-16-> reserved

# DEV CTA Status Information Element

## New Clause 7.4.x DEV CTA Status  Information Element

This element is a 256 bitmap where each bit corresponds to an AD-AD.  The PNC sends the DEV CTA Status element in the Beacon.  The purpose of this element is to enable a DEV to know if any CTAs where it is a SA or DA have change since the last Beacon.   If any of the CTAs for a given DEV has changed since the last Beacon, the bit corresponding to the AD-AD for that DEV shall be set to a one.  If none of the CTAs for that DEV have changed since the last beacon, the bit corresponding to the AD-AD for that DEV shall be set to zero.  If a broadcast CTA has changed since the last beacon, the broadcast CTA bit will be set, and not all bits in the bitmap.

| Octets:1 | 1 | 8 |
| --- | --- | --- |
| ElementID | Length=8 | lsb DEV CTA Status bitmap msb |

Figure 2 DEV CTA Status Information Element

If a DEV correctly received beacon n, it does not need to process the CTAs in beacon n+1 if its DEV CTA Status bit and the broadcast CTA status bit in beacon n+1 is set to zero.  If its DEV CTA Status bit is set to one or the broadcast CTA status bit, the DEV shall process the CTAs in that beacon.