IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Standards Text for Child and Neighbor Piconet Functionality |  |
| Date Submitted | 6 November 2001 |  |
| Source | Bob Huang Sony Electronics Inc. One Sony Drive, Park Ridge, NJ 07656 Shige Sugaya, K. Takamura, M. Suzuki, Masa Akahane Sony Corporation 6-7-35 Kitashinagawa Shinagawa-ku, Tokyo Mark E. Schrader Eastman Kodak Co. 4545 East River Road Rochester, NY 14650-0898 | Voice: 201-358-4409 Fax: 201-930-6397 E-mail: robert.huang@am.sony.com Voice: +81.3.5782.5067 Fax: +81.3.5782.6880 E-mail: sugaya@wcs.sony.co.jp takamura@wcs.sony.co.jp suzuki@wcs.sony.co.jp akahane@wcs.sony.co.jp Voice: 716-781-9561 FAX: 716-781-9533 E-Mail: mark.e.schrader@kodak.com |
| Re: | IEEE Draft P802.15.3/D0.7 01305r3P802.15_TG3-GTS-Daughter-Network-Proposal.ppt 01350r0P802-15_TG3-MAC-Static-&-Unassigned-GTS.ppt 01119r0P802.15_TG3-Modified-CFP.ppt |  |
| Abstract | The concept of a child piconet, presented in document P802.15-01/305 (through revision 4), was accepted to be added to the draft standard. This contribution provides specific text to be inserted into the draft standard to provide child and neighbor piconet functionality. |  |
| Purpose | Submitted for adoption and incorporation in the 802.15.3 draft D0.8. |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Standards Text for Child and Neighbor Piconet Functionality

Rev 40

- Removed the neighbor association request command (see rev 1)  and added text to the (normal association request format to accommodate the neighbor association.

- Removed the neighbor channel time request from figure def.

- Modified table 63 to remove the neighbor association request and the neighbor channel time request.

Rev 3

- Change ‘must’ to will in 3. Definitions.

- Change ‘will’ to should in (new) 5.4.7.

- Change ‘will’ to shall in  5.6.6.1, 7.4.11, 7.5.4, 7.5.5.2, 8.2.9, 8.2.10,

- Delete the number of records in figure ZCZ.

- Change Stream ID to Stream Index in figure YZY.

- Add a description of type ID after figure YZY.

- In 8.2.9 and 8.2.10, changed channel time allocation to private GTS

Note:  The following changes to add the private GTS simplify the text by referring to the text for the private GTS.

- In 5.4.7, 8.2.9 and 8.2.10, changed channel time grant to private GTS

- In 5.4.7, change so that child PNID is different from the parent PNID.  Correct type of PNID.

- Added ‘private’ to  guranteed time slot in  to 3. Definitions.

- Added message sequence charts to 8.2.9 and 8.2.10.

- Renumber 5.4.7 Neighbor Piconet to 5.4.8.

Note: Revision marks reflect changes from r2.

Rev 2

- In Figure RR, the alignment between the parent and daughter superfames has been corrected.

- In para. 3 page 12, two instances of the single neighbor AD-AD (0xFC) have been changed to reflect three neighbor addresses.

Rev. 1

On 25 October 2001, the IEEE 802.15.3 MAC group held a telephone conference call in which revision 0 of this document was discussed. Based on the comments and suggests made in that call, the following changes were made to the text below:

- Three AD-AD address values are specified for neighbor networks, instead of one AD-AD.

- Added a neighbor association request command, thus splitting the neighbor initiation process into association and channel time request. The IEEE802 address is now used in the neighbor association request and in the association response, thus making each association response unique.

- A special note is added to the association response commend text to give the valid AD-ADs for a neighbor PNC.

- Special channel time request for a neighbor PNC is removed.

- Child or neighbor channel time grant – explanation of fields (in figure YZX) is provided.

- Figure RR, Parent and Child Superframe Relationship, was modified to black, white and gray tone.

- Text in 8.2.10, neighbor piconet, is modified to reflect more neighbor PNC addresses and to reflect the separation of association and channel time request.

Note: Revision marks reflect changes from r0.

The concept of a child and neighbor piconet, presented in document P802.15-01/305 (through revision 4), was accepted to be added to the draft standard.  This contribution provides text to be inserted into the draft standard.  The basis for the suggested text is D0.7.

Note:  The original concept, in doc. 305, used the term “daughter network”.  For the standards text, the term “child network” is used instead.

Editorial notes:

Black text exists in the draft standard or gives editorial instructions.

Blue text is new and to be added to the draft standard.

Red text is work in progress (to be ignored).

3. Definitions

Editorial instruction: Insert new definitions

child piconet: A piconet that exist entirely within a private guaranteed time slot of another piconet (parent piconet) and is controlled by an alternate coordinator that is a member of the parent piconet.  The area of overlapping coverage between the two piconets may vary between congruent with the parent coverage area to mostly non-overlapping.

parent piconet: A piconet which allocates a private guaranteed time slot for another piconet of the same system (child piconet) or of another system (neighbor piconet).

neighbor piconet: A piconet that exist entirely within a private guaranteed time slot of another piconet (parent piconet) and is controlled by an alternate coordinator that is not a member of the parent piconet. The area of overlapping coverage between the two piconets may vary between congruent with the parent coverage area to mostly non-overlapping. The PNC of the neighbor piconet will request a private GTS from the parent piconet using the 802.15.3 protocol and will be able to accept 802.15.3 protocol requests for a private GTS (or equivalent) from a (potential) neighbor 802.15.3 piconet.

alternate coordinator: A member of the piconet that is capable of being the coordinator but is not functioning as the coordinator.

4. Acronyms and abbreviations

GTS 	guaranteed time slot

5.4 Overview of MAC functionality

IEEE 803.15.3 provides for the following functionality:

— 	Allow stations to form and terminate PANs

— 	Transport data between stations

— 	Authenticate stations with each other

— 	Allow stations (including the coordinator) to minimize power requirements and still maintain the net-work.

- A procedure for the coordinator to pass coordination to another station in the WPAN.

- A procedure to establish a child piconet (subnetwork with same ownership).

- A procedure to establish a neighbor piconet (subnetwork with different ownership).

5.4.7 Child Piconet

A child piconet may be formed under an established piconet, wherein the established piconet becomes the parent piconet. The child piconet uses its own piconet ID (PNID) but acts as an autonomous piconet except that it is dependent on a private GTS from the parent piconet.  Association, authentication, security and acknowledgments are handled within in the child piconet and do not involve the parent PNC.

The child PNC device is a member of the parent piconet and thus can exchange data with any device in the parent piconet. The child PNC device is also member of the child piconet and thus can exchange data with any device in the child piconet.

5.4.8 Neighbor Piconet

A neighbor piconet may be formed under an established piconet, wherein the established piconet becomes the parent piconet. The neighbor piconet has a different owner than the parent piconet and is autonomous piconet except that it is dependent on a private GTS from the parent piconet.  Association, authentication, security and acknowledgments are handled within in the neighbor piconet and do not involve the parent PNC.

The neighbor PNC device is not a member of the parent piconet and thus can not exchange any data with any device in the parent piconet.

The neighbor piconet mechanism may be used by other 802-compliant wireless devices as a means of sharing the frequency spectrum. A device capable of initiating (requesting status as) a 802.15.3 neighbor piconet should also be capable for supporting a 802.15.3 piconet as a neighbor.

5.6.6 Network termination

5.6.6.1 Involving Child Piconet

Upon termination of a parent piconet, the child piconet may continue to operate without disruption.  The child piconet will remove the Parent Device ID Element from its beacon frame, signifying that it is a freestanding piconet. In the case where the parent piconet was temporarily disrupted, the parent PNC may join the child piconet, challenge for control and receive a transfer of control (promotion to PNC or handover).  In this case, the child piconet should continue without disruption.

Upon termination of a parent piconet, the child piconet may also terminate.  The PNC of the (former) child piconet may immediately attempt to establish a new piconet. The termination of a child piconet shall have no effect on the parent piconet except to release resources.

5.6.6.1 Involving Neighbor Piconet

Upon termination of a parent piconet, the neighbor piconet may continue to operate without disruption.  The neighbor piconet will remove the Parent Device ID Element from its beacon frame, signifying that it is a freestanding piconet. In the case of a temporary termination of the parent piconet, the (former) parent PNC can restart as a neighbor (i.e., role reversal).

The neighbor piconet shall terminate its relationship with the parent PNC using the disassociation request command.  The termination of a neighbor piconet shall have no effect on the parent piconet except to release resources.

7.2.3 Address fields

There are two address fields in the MAC frame format and each of these fields is 8 bits in length. These fields are used to indicate the destination address (DA) and the source address (SA). An address for a DEV is assigned by the PNC during the association of the DEV. The address of a DEV is unique to an associated DEV within a piconet. The following addresses are reserved.

— The address value 0 is reserved for the PNC, for coordinator related transmissions and receptions

— The address value of all-ones (0xFF) is reserved for broadcast frames

— The address value of 0xFE is reserved for use by all new clients during their association until a unique address is allocated to each one of those new clients by the PNC.

— The address value of 0xFD is reserved for multicast frames

— The address values of  0xFA, 0xFB or 0xFC are reserved for a neighbor network

7.3.1 Beacon frame format

Editorial instruction: Add new element:

### Table 58—Beacon frame body

| ## Information element | Note |
| --- | --- |
| Device ID | Element ID + Length + 48 bit IEEE 802 address of the PNC |
| Piconet synchronization parameters | TSF element and other time duration elements |
| TPC element | Sets the max TX power level in the piconet |
| Channel change | During change to new channel |
| Channel time allocation (CTA) | All the channel time allocation in the current superframe |
| Parent Device ID (if child or neighbor piconet) | 48 bit IEEE 802 address of the Parent PNC |

7.4.11 Channel time allocation (CTA) element

:

Editorial note: insert in the position one paragraph after Figure 19.

The source DEV address indicates the DEV to whom the channel time is being allocated.

For a child PNC, the source DEV and destination DEV addresses shall both be the AD-AD of the child PNC (assigned by the parent PNC).

For a neighbor piconet, the source and destination addresses shall be the same and shall be in the range 0xFA to 0xFC.

7.5 Command types

Table 63—Command types

| Command type hex value | Command name |
| --- | --- |
| 0x0000 | Retransmission request |
| … | ….. |
| 0x001B | Momentary EPS CTA |
| 0x001C | Child or neighbor information response |
| 0x001D-0xFFFF | Reserved |

7.5.2 Association request command format

Editors note: This text at the end of 7.5.2.

The neighbor PNC association request shall use this format with the capability field filled with zeros and use a challenge/response text field of zero length.

7.5.4 Association response command format

:

:

Editors note:  insert text after the sixth paragraph. The sixth paragraph begins “ The AD-AD field…”

For the association of a neighbor PNC, the AD-AD shall be 0xFA, 0xFB or 0xFC.

Editor’s note: The follow text to inserted at the end of 7.5.5.1.

The neighbor PNC shall send a channel time request using this format.

7.5 Command Types

7.5.19.3 Child or neighbor information response command

This command, following the same response rules a the device information response command, defined in 7.5.19.2, uses the structure illustrated in Figures ZCZ and YZY

| Octets: 2 | 2 |  | 12 | 12 | …. |  |
| --- | --- | --- | --- | --- | --- | --- |
| Command Type | Length |  | Record for child or neighbor PNC | Record for child or neighbor PNC | …. |  |

Figure ZCZ - Child or neighbor information response command

| 1 | Octets: 6 | 1 | 1 | 1 | 2 |
| --- | --- | --- | --- | --- | --- |
| Type ID | Neighbor PNC Device ID | Stream Index | Duration between time slots | Minimum requested channel time in a time slot | Requested channel time per time slot |

Figure YZY - Child or neighbor PNC information response record

The type ID shall be 0x01 for a child piconet and 0x02 for a neighbor piconet.

The duration between transmissions indicates the requested frequency of time slots that needs to be allocated for the DEV. The resolution of this field is Ks.

The minimum requested channel time is the minimum duration of the time that is acceptable at the requesting DEV in any time slot. If the allocated time slot is smaller than the time indicated in this field, the PNC may not allocate that time-slot at all for the DEV. The resolution of this field is 32 s and so the range of this field is less than [0-8160] s.

The requested channel time is the time required at the DEV in every time slot for transmission of the indicated stream. Hence this field represents the time required for the given latency and not necessarily for the entire superframe. The resolution of the channel time field is 8 μs and so the range of requested time is less than [0-524280] s.

Editorial instruction: New text

8.2.9 Child piconet

When an alternate coordinator of an existing piconet wants to form a child piconet, the alternate coordinator shall request a GTS in which to operate a child piconet. By setting the source and destination addresses as the same,

The channel time request command, defined in 7.5.5.1, is used, setting the source and destination addresses to the child PNC AD-AD. The parent PNC will recognize this as a request for a child piconet.

After receiving theprivate GTS, the child PNC will start sending its beacon.  Included in the child piconet beacon is a private GTS for the parent piconet, using the PNC’s address (0x00) for the source device address. This is provided to reserve the slot, not to convey any information to the Parent PNC.

Figure RR illustrates the relationship between the parent piconet superframe and the child piconet superframe. It can be seen that the superframe duration is the same for both the child and the parent piconets.

Note: The slot positions GTS0, GTS1, et al, are illustrative.

Figure RR – Parent piconet and child piconet superframe relationship.

The message sequence chart for the initiation of the child piconet is shown in figure abc.

The child piconet is an autonomous piconet except that it is dependent on a private GTS from the parent piconet.  Association, authentication, security and acknowledgments are handled within the child piconet and do not involve the parent PNC.

The child PNC device is a member of the parent piconet and thus can exchange data with any device in the parent piconet. The child PNC device is also a member of the child piconet and thus can exchange data with any device in the child piconet.

8.2.10 Neighbor piconet

A neighbor alternate PNC (alternate PNC of a different system), wanting to start a piconet, shall first scan for a piconet to join, then finding none, shall scan for an open channel in which to initiate a piconet.  If no channels are available, then the neighbor alternate PNC may attempt to start a neighbor piconet within a private GTS of an existing piconet.  To start a neighbor piconet, the potential neighbor PNC shall send an association request, defined in 7.5.2, using the AD-AD of 0xFE (refer 7.2.3).

The parent PNC shall respond with a association response command (refer 7.5.4). If the neighbor association request is accepted, then the AD-AD shall be 0xFA, 0xFB or 0xFC (refer 7.2.3).  If not accepted, the AD-AD shall be set to 0xFE.

To obtain a private GTS, the neighbor PNC sends a channel time request, defined in 7.5.5.1

If the parent PNC permits the formation of a neighbor piconet, a private GTS is sent using 0xFA, 0xFC or 0xFC as both the source and destination. These addresses are reserved for the neighbor piconet (refer 7.2.3). After receiving this channel time allocation, the neighbor PNC will start sending its beacon.  The neighbor beacon contains the parent piconet ID (refer Table 58).

Included in the neighbor superframe is a private GTS for the parent piconet, using the parent PNC’s address (0x00) for the source device address. This is provided to reserve the slot, not to convey any information to the Parent PNC.

Figure RR illustrates the relationship between the parent piconet superframe and the neighbor piconet superframe. It can be seen that the superframe duration is the same for both the neighbor and the parent piconets.

The message sequence chart for the initiation of the neighbor piconet is shown in figure def.

The neighbor piconet is an autonomous piconet except that it is dependent on a private GTS from the parent piconet.  Association, authentication, security and acknowledgments are handled within the neighbor piconet and do not involve the parent PNC.

The neighbor PNC is not a member of the parent piconet (not authenticated and not associated) and thus is restricted to only sending a channel time request and reading the parent piconet beacon.  The parent PNC is not a member of the neighbor piconet.