IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | IEEE P802 |  |
| Date Submitted | 19 October, 2001 |  |
| Source | Mark E. Schrader Eastman Kodak Co 4545 East River Rd Rochester, NY 14650-0898 Jay Bain Time Domain 7057 Old Madison Pike Huntsville, AL 35801 | Voice: 716-781-9561 Fax: 716-781-9533 E-mail: mark.e.schrader@kodak.com Voice: 256 922-9229 Fax: 256 922-0837 E-mail: jay.bain@timedomain.com |
| Re: | IEEE Draft P802.15.3/D0.7 |  |
| Abstract | This document provides draft text for the power save clauses of the 802.15.3 MAC to be incorporated into D0.8. This presents a method of incorporating current mode (EPS or AWAKE) of a DEV in its CTA elements. It also allows each DEV to have a persistent CTA element containing the time slot allocation for that DEV or the superframe on which the next time slot allocation will occur. The addition of two bits of control in each CTA results in any DEV being able to resynchronize with its source or destination DEV by listening to only one beacon. |  |
| Purpose | The recommendations contained in this document are to be applied to the 802.15.3 MAC baseline, They are based on the presentation in Ad Hoc meeting in Rolling Meadows, IL (10/9/01-10/11/01). |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

PS editor note: the clause numbering for this document is based on draft 0.7

Clause 4 Acronyms and abbreviations

EPS	Enhanced Power Save power management mode, or CTA element type corresponding to MLME SAP, MLME-POWERMGT.request parameter PowerManagementMode value of EPS

ACTIVE  	Any non-EPS power management mode, or a CTA element type corresponding to MLME SAP, MLME-POWERMGT.request parameter PowerManagementMode value of either RPS or POWERED.

Clause 5 General Description

MkS Editor note: Text needed and not included now.

6.0 Layer Management

MkS Editor note: Additional changes will be needed to reflect EPS Set as a new parameter.  These changes are not included here.

Clause 7

7.5.11 EPS Configuration Request

MkS Editor note: Eliminate this command in D07

7.5.12 EPS Configuration Response

MkS Editor note: Eliminate this command in D07

7.5.13 EPS PS Configuration Response

MkS Editor note: Eliminate this command and replace the Action Commands shown below

7.5.13.1 EPS Action command

This command is found in Figure 1 along with the interpretation of the fields as shown in Figure 2.

7.5.13.2 EPS Action response

This command is found in Figure 1 along with the interpretation of the field as shown in Figure 3.

This is used to create and maintain EPS Sets as well as EPS Set membership.  When an EPS Set is confirmed as created, the PNC shall begin keeping the time base specified for that EPS Set.

| Octets:2 | 2 | 1 | 1 | 2 | 4 |
| --- | --- | --- | --- | --- | --- |
| Command type | Length (=2 or 8 based on request type) | Request type | EPSSet value | EPSTime | EPSNext |

Figure 1 PS PNC configuration command/response

| Request type | ## EPS set value | ## EPSTime & EPSNext |
| --- | --- | --- |
| Release request (0 value) | Value Required | Not present |
| New request (1 value) | Set to 0 | Required |
| Place me in set request (2 value) | Value Required | Not present |
| Give me information on the EPS set request (3 value) | Value Required | Not present |

Figure 2 Request entries

| Reply type | ## EPS set value | ## EPSTime & EPSNext |
| --- | --- | --- |
| Release confirm (0 value) | Value released | Not present |
| New confirm (1 value) | New value provided | Not present |
| Place me in set confirm (2 value) | Value of set place into | Not present |
| Give me information on the EPS set response (3 value) | Value Required | Part of reply |
| Failure code (values as necessary) | Set to 0 | Not present |

Figure 3 Response entries

Mks Editor Note: Failure codes values table needed for: Incorrect Command Length, Already a Member, EPS Set Does Not Exist, Cannot Create New Set, Illegal Command,

7.5.14 DEV to PNC PS information

MkS Editors Note: Keep this command in D07.  It is needed to give the PNC information about PS capabilities to the PNC.

MkS Editors Note: D07, Table 63, printed page 80, line 18-19. Fix momentary command entry.

| # Command type Hex value | # Command name |
| --- | --- |
| 0x0002 | Channel time request (enhancement) |
| 0x0003 | Channel time grant (enhancement) |
| 0x0015 | EPS configuration request |
| 0x0016 | EPS configuration response |
| 0x0017 | PS PNC configuration command |
| 0x0018 | DEV to PNC PS information |
| 0x0019 | Switch to ACTIVE CTA mode |
| 0x001a | Switch to EPS CTA mode |
| 0x001b | ## Momentary Substitute ACTIVE CTA for EPS CTA command |

MkS Editors note: D07 The channel time request block with will have to be modified to include a field for “EPS Set”.   Since the entire structure is TBD, I am not including it here.

7.5.5.1 Channel time request

.

.

.

MkS Editors note: D07 printed page 85, line 17-21.

Replace paragraph with text below.

EPS status indicates whether the CTA requested is for ACTIVE CTA mode channel time or EPS CTA mode channel time. The PNC shall maintain separate ACTIVE mode type CTAs and EPS mode type CTAs.  Values of 0, 1, or 3 are used for making ACTIVE CTA mode channel time requests, and the value of 2 is used for making EPS CTA mode channel time requests.

For a device for without EPS capability: A value of 0 or 3 shall be used for an ACTIVE CTA mode channel time request.

For a device with EPS capability: A value of 0, 1, or 3 shall be used for an ACTIVE CTA mode channel time request.  A value of 2 shall be used for an EPS CTA mode channel time request

A value of 3 shall be used for ACTIVE DEVs wishing to broadcast or multicast to an EPS DEV.

ACTIVE CTA Mode CTA Persistence: An EPS status value of 0 shall cause the PNC to delete the associated CTA when a DEV switches from ACTIVE CTA mode to EPS CTA mode. The associated CTA shall not exist when the same DEV switches back to ACTIVE CTA mode.  For the remaining EPS status values of 1 and 3, the associated ACTIVE mode CTAs shall persist after any transition to and from EPS CTA mode, and thus shall not be modified or deleted by transitions between CTA modes.

All other values of EPS status are reserved.

MkS Editors note: Do we need a table to show what’s above?

7.5.17  Momentary Substitute ACTIVE CTA for EPS CTA command.

The structure of the command is indicated in Figure zz. This command instructs the PNC to use the previously determined channel time request for ACTIVE mode during the next scheduled EPS superframe. This is the equivalent to replacing the EPS mode CTA with the AWAKE mode CTAs on the next EPS superframe. This substitution is only in effect for one EPS superframe.

7.4.11 Channel time allocation (CTA) element

#### MkS Editors note: The text does not change until D07 printed page 78, line 27.  Replace lines 27-37 with the following text.  I have taken the liberty to include the “Slot End Time” field in the CTA element and to allow for the possibility of an MTS slot designator.

The next four bytes in the CTA block are used to describe the location of the next time slot for the specified Source DEV, Destination DEV, and Stream ID.  The Beacon-Time Select bit of the CTA Control field (Figure ??) shall be set by the PNC to indicate how these four bytes should be interpreted.

If there is time slot in the current beacon frame, the Beacon-Time Select bit of the CTA Control field shall be set to 0 by the PNC.  This 0 indicates that the four bytes are set to the Slot Start time (2 Bytes) and Slot Stop time (2 Bytes). The Slot Start time field indicates the start time of the time slot. The value of this field is always relative to the start of the start of transmission of beacon frame from the PNC. The resolution of this field is 8 s and so the range is [0-524280] s.  The Slot Stop time is also relative to the start of the beacon frame and in the same units.

This is true regardless of whether the CTA mode is ACTIVE or EPS.  In EPS mode only, the values of Slot Start time and Slot Stop time may be identical, which indicates that the EPS DEV will be powered up to listen to this beacon, but not have time allocated for data transmission by this CTA block.

If there is no time slot in the current beacon frame, and a time slot scheduled for a future beacon frame, then the Beacon-Time Select bit of the CTA Control field shall be set to 1 by the PNC.  The 1 indicates that the four bytes after the Destination DEV are interpreted as the Next Slot Beacon Number (4 Bytes).  This is the Beacon Count of the next superframe in which there will be a time slot for this CTA

On the beacon frame specified, the PNC shall set Beacon-Time Select bit of the CTA Control field to 0. If the difference between of Slot Start time and Slot Stop time is non-zero, then the DEV can expect traffic in the current superframe in that slot. The Slot Stop time is Slot Start time of the next GTS slot minus the aSlotGuardTime.

The next byte contains the Stream ID assigned by the PNC in response to the channel time request associated with this CTA element.

The CTA Control field is shown in figure ??.  It contains bits that shall be set by the PNC.  The first bit is CTA Type.  This specifies whether the Source DEV and Destination DEV are in ACTIVE CTA mode (0) or EPS CTA mode (1).  The mode is controlled via the Switch to ACTIVE CTA mode command and Switch to EPS CTA mode command that are issued by a DEV.

The second bit is Beacon-Time Select.  This bit selects how the 4 bytes after the Destination DEV are interpreted. The use of this field was described previously in this section.

The 6th bit is reserved.

The first 5 bits are also reserved.

Figure 18—Channel time allocation element

MkS Editors Note: no change to figure 18.

Slot start time and Slot end time are in 8 s resolution, or the next slot beacon number is the beacon number of the next occurrence of this time slot.

Source DEV

1Octet

Destination DEV

1Octet

2Octets

Slot End Time

2Octets

Stream ID

1Octet

CTA Control

1Octet

Next Slot Beacon Number

Slot Start Time

### Figure 19—Channel time allocation block

Reserved

5 bits

Reserved for MTS/non-MTS indication

1Bit

Time-Beacon Select

1 Bit

CTA Type

1 Bit

### Figure ?? — CTA Control octet of channel time allocation block

8.3.x.x  Overview of CTA Management By the PNC

CTA elements once specified and confirmed by the PNC shall be present in every beacon.  If a DEV does not have an ACTIVE or EPS slot in a particular superframe, the PNC shall include a CTA with the Beacon Number of the next ACTIVE or EPS time slot according to the CTA block definitions of section 7.?.?.?.  The constant presence of CTA element may be used in any beacon to read the CTA mode of a device as well as the slot or beacon slot value needed to communicate with the device. (The EPS Set currently in use shall be part of the device information obtained via the discovery process.)

8.3.x.x EPS CTA  Mode and EPS Time Slots

The creation of an ACTIVE CTA in a beacon by the PNC is entirely the result of an ACTIVE Channel Time Request.  The creation of an EPS CTA in a beacon by the PNC is also the result of an EPS Channel Time Request, however, there is one additional requirement placed on a DEV for the creation of the EPS CTA.  The DEV shall be a member of at least one EPS Set.  The DEV shall first become a member of an EPS Set, and shall then issue an EPS Channel Time Request using the number of an EPS set to which it belongs.  The PNC shall decline the EPS Channel Time Request if the DEV is not a member of the EPS Set specified, which could be due to the condition that the EPS set does not exist.

The following applies to a device that is in EPS CTA mode.  The EPS Set and the EPS Channel Time Request are used together by the PNC to create the EPS CTAs, and time slots with the correct characteristics and at the correct times.

The role of the EPS Set is to establish a sharable time base for the occurrence of periodic EPS time slots. This time base shall be kept by the PNC with the same precision of an actual slot occurring at the rate specified by EPStim, and starting on the beacon whose beacon number is specified by EPSnext.

The role of the EPS CTA is to specify the size of the EPS time slots used for data, and the proportion 1:N of EPS time slots that will be of the length specified by the Channel Time Request Block, CTRB (Section 7.5.5.1, Figure 25).  For an EPS CTRB, the duration-between-slotsTBD field shall specify the value of N, such that 1:N is the fraction of EPS time slots that can be used for data.  If the value of N is 1, every EPS CTA has the size specified by the CTRB, and if the value is 4, then 1 EPS CTA has a CTRB specified size out of each 4 slots.  Each data slot is followed by N-1 EPS slots that the PNC shall specify using CTA elements with zero length.  If the device specifies N as 0, then the PNC shall create all EPS CTA time slots associated with the CTRB as zero length slots.  A zero length EPS CTA slot shall be designated by the PNC setting the Slot Start Time field to 0 in the EPS CTA block.

After 2 or more EPS mode CTA devices have submitted EPS type CTRBs with the same EPS Set value, the PNC shall give them time slots that are all close to each other in the EPS superframe.  A device that is only a destination device shall specify that all EPS slots will be of 0 length (N = 0).  The non-zero length EPS CTAs and zero length EPS CTAs may be read by other DEVs to as the indication that the destination station is listening to the beacon during that superframe.