IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | TG4_MAC_v12 |  |
| Date Submitted | 21 May 1999 |  |
| Source | Robert Poor Ember Corporation 1 Broadway, 14th Floor Cambridge, MA 02142 USA | Voice: 617 758-4262 Fax: 617 758-4101 E-mail: r@ember.com |
| Re: | “Mutlticast-friendly” CSMA for 802.11.4 |  |
| Abstract | In CSMA specification prior to draft version 12, any transmissions that trigger multicast or flood messages are guaranteed to collide: when the N neighbors of a node receive such a message, they will all try to transmit at the same time. The modifications proposed here allow "backlogged upon arrival" queuing, so transmissions are deferred for a random backoff before transmitting. |  |
| Purpose | This document is for discussion among the TG4 working group and interested parties with the intention of incorporating its contents in the final 802.15.4 specification. |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

### Overview

In the current spec, any transmissions that trigger multicast or flood messages are guaranteed to collide: when the N neighbors of a node receive a message, they will all try to transmit at the same time.  The modified spec allows “backlogged upon arrival” queuing, so transmissions are deferred for a random backoff before transmitting.

With a particular setting of parameters, the new algorithm will behave exactly like the current spec, and the extensions require very little additional code.  I strongly urge everyone who cares about the MAC level to consider these extensions.

### Particulars

1) The retry counter Nr, is now separate from the backoff exponent BE.  BE starts at BEmin, is incremented until BEmax, and then stays there.

2) BEmax, BEmin, and macCSMAPadding are parameters to be set by the implementer, which would provide for application specific performance.

3) When BEMin is set to 0, BEmax to macMaxCSMARetries, and macCSMAPadding to 2, then this system is equivalent to the v10 TG4 MAC spec.

4) For multicast applications, setting BEMin to zero results in multiple recipients firing off at exactly the same time.  Instead, set BEmin to 3 to provide some initial jitter.

5) There needs to be a rule for how long to wait after transmitting before starting on the next packet in the queue.  A simple approach would be to wait for macCSMAFrameSpace after the transmission ends.  However, the 802.15.4 PHY does not notify the MAC when it is done transmitting, so this might not be feasible.  So an even simpler approach would be to just wait macCSMAFrameSpace, which would be set to the maximum packet transmission time.  Should this be in the spec?

The diagram on the following page shows the revised CSMA algorithm.

Begin

CSMA-CA

Nr = 0

BE = BEmin

Backoff for

random(2^BE - 1)

units

Channel

idle?

Transmit

data

Nr = Nr + 1

Increment BE

up to BEmax

Nr >

macMaxCSMARetries?

Backoff

macCSMAPadding

units

Transmission

failure

Y

N

Y

N