IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | 01502r1P802.15_TG3-MAC-WMS-Issue-Resolution |  |
| Date Submitted | 12 Nov., 2001 |  |
| Source | William Shvodian <XtremeSpectrum> Suite 700, 8133 Leesburg Pike, Vienna, VA 22182 | Voice: 703-269-3047 Fax: 703-269-3092 E-mail: bshvodian@xtremespectrum.com |
| Re: | D07/D08 |  |
| Abstract | This document provides proposed text for issue resolution. |  |
| Purpose | The recommendations contained in this document are to be applied to the 802.15.3 MAC baseline. |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

1)	Remove Delayed ACK Expedite (Issue 248)	3

2)	Multicast Stream Establishment (Issue 339)	3

3)	Slotted ALOHA Reference for the Bibliography (Requested by Jim Allen)	3

4)	Channel Time Requests – only from stream source (Issue 412)	4

5)	Association Response Success Reason Code (Issue 410)	4

6)	MAC Frame Formats (Issue 393)	4

7)	Beacon Information Elements (Issue 398)	5

8)	Open Scan (Issue 317)	5

9)	Peer Discovery (Issue 365)	6

# Remove Delayed ACK Expedite (Issue 248)

Issue: This is redundant with 7.2.1.8 Del-ACK Request in the frame control field.  Section and figure numbers are from D07.

## Clause 7.2.4  Stream ID

Remove pg 75 line 1 and 2:

“The Del-ACK expedite field is used to request that the accumulated Del-ACK’s be sent as soon as possible.  This is described in more detail in 8.7.3.”

Modified Figure 5 Stream ID Field

| bits: 0 | 1:3 | 4:11 | 12:15 |
| --- | --- | --- | --- |
| Stream type | Priority | Stream index | Reserved |

# Multicast Stream Establishment (Issue 339)

Issue: There is currently no mechanism specified to set up a multicast stream

## Clause 7.5.21 Figure 49

Modify

For a broadcast stream, the involvement of intended receiver is precluded.

To

For a broadcast or multicast stream, the involvement of intended receiver is precluded.

# Slotted ALOHA Reference for the Bibliography (Requested by Jim Allen)

Stallings, Data and Computer Communications, Second Edition, Macmillan, 1988, pp300-302

# Channel Time Requests – only from stream source (Issue 412)

Issue: Since the originator of a stream request can be the source or the target of the stream, it is not clear which DEV can make Channel time requests: the originator of the request, the source of the stream, either if they are not the same?

## 8.3.3.2 Channel Time Allocation (CTA) and channel time usage

Add the following sentence the first paragraph:

Only the DEV that is the source of a stream can send Channel Time Request commands for that stream.  Tabled 11/13/01

# Association Response Success Reason Code (Issue 410)

Issue: DEV should not have to look at AD-AD field to determine success or failure of the association.  That should be in the reason code.

## Association Response Command Format 7.5.3

Remove the following sentence from the sixth paragraph:

If this field contains the association-address (0xFE), the DEV is not allowed to associate for the reason mentioned in the reason code. Keep this 11/13/01, James will modify as appropriate.

Change the reason codes as follows:

The valid reason codes are:

- 0 -> Success

- 1->Already serving maximum number of DEVs

— 2 -> Lack of available bandwidth to serve the DEV

— 3 -> Channel is severe to serve the DEV

— 4 -> PNC is turning off with no AC in the piconet

— 5 -> DEV wishes to disassociate

— 6 -> Channel change is in progress

— 7 -> PNC hand over is in progress

— 8 -> DEV authentication failed

- 9-255 -> reserved

# MAC Frame Formats (Issue 393)

Issue: Since there are now only 4 frame types, all DEVs shall be able to process all frames, except only PNC capable devices must be capable of creating beacon frames.

## Clause 7

Replace “In addition” through the end of the paragraph with:

In addition, every DEV shall be able to construct these frame formats for transmission, and to decode frame formats upon validation following reception.  The only exception is that a DEV that is not PNC capable need not be able to construct beacon frames.

# Beacon Information Elements (Issue 398)

Issue: D07 does not explicitly specify which information elements are optional or mandatory in the Beacon.

## Table 62

| Information Element | Note | Present in beacon |
| --- | --- | --- |
| Device ID | … | In Every Beacon |
| Piconet Synchronization Parameters | … | In Every Beacon |
| TPC element | … | As Needed |
| Channel change | … | As Needed |
| Channel time allocation (CTA) | … | As Needed |
| Parent Device ID (if child or neighbor piconet) | … | As Needed |

# Open Scan (Issue 317)

Issue: Need to define passive scanning for any PNID.  8.2 currently only addresses searching for a specific PNID.

## Clause 8.2.1Scanning through Channels

Replace the following sentence:

While scanning, the DEV shall ignore all the received frames with a different PNID than the one for which the DEV is searching.

With:

If Open Scan is specified in the MLME-SCAN.request,  the DEV shall perform Open Scan.  In Open Scan, the DEV searches for any PNID.  If Open Scan is not specified, the DEV shall ignore all the received frames with a different PNID than the one for which the DEV is searching.

## Clause 6.3.2.1 MLME-SCAN.request

Modify the primitive as follows:

MLME-SCAN.request (OpenScan,

PNID,

ChannelList,

ChannelScanDuration)

Add the following Entry to Table 5

| Open Scan | Boolean | TRUE/FALSE | Identifies if scan is Open Scan or not |
| --- | --- | --- | --- |

# Peer Discovery (Issue 365)

Issue: … any DEV may send any directed command frame to any other DEV in the piconet to determine if the destination DEV is still present in the piconet.”  Comment:  This sentence implies that the DEV can send a Retransmission req, Channel Status request, Association req, Disassociation req, and Sleep state req as "pings" to the destination device.  Doesn't this get confusing if all the transmitting DEV wants to do is determine whether the destination DEV is present?

## Clause 8.8, paragraph 3

Modify the sentence as follows:

In addition to the above, any DEV may send probe request command with the information request field set to zero and ACK set to immediate ACK  any appropriate directed command frame to any other DEV in the piconet to determine if the destination DEV is still present in the piconet.

James will add additional clarification to the probe request text to specify that if the request field is null, the DEV shall respond with immediate ACK only.