IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title |  |  |
| Date Submitted | November 15, 2001 |  |
| Source | HK Chen InProComm Taiwan Laboratories P.O. Box 24-226 Hsinchu, Taiwan 300 | Voice: +886-3-516-5106 Fax: +886-3-516-5108 E-mail: hkchen@inprocomm.com |
| Re: | [] |  |
| Abstract | This contribution contains the comments to P802.15.2_D01_Recommended_Practice |  |
| Purpose | For review by TG2 members. |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Change Table 15 in clause 11.3.2.3 (page51) to

Table 15-Partition sequence values for HV1 SCO connection (V = 1)

| Nv = # of HV streams | Dsco | Gi = # of good channels assigned to Fi | Partition Sequence, p(k) |
| --- | --- | --- | --- |
| 1 | 0 | 0 | [0 0] |
| 1 | 0 | 2 | [1 1] |

Change Table 16 in clause 11.3.2.3 (page51) to

Table 16-Partition sequence values for HV2 SCO connection (V = 2)

| Nv = # of HV streams | Dsco | Gi = # of good channels assigned to Fi | Partition Sequence, p(k) |
| --- | --- | --- | --- |
|  |  | 0 | [0 0 0 0] |
| 1 | 0 | 2 | [1 1 0 0] |
| 1 | 2 | 2 | [0 0 1 1] |
| 2 | 0,2 | 2 | [1 1 0 0] |
| x | x | 4 | [1 1 1 1] |

Renumbering the clauses including clause 11.3.2.2 and after  is required. See the line 4 of page 46.

Change the clause numbers:

11.3.2.2 Partition sequence generator

11.3.2.2.1 ACL connection

11.3.2.2.2 SCO connection

11.3.2.3 Re-mapping function

Insert this clause

##### 11.3.2.2.3Time alignment between partition sequence and clock

The partition sequence is periodic with superframe, which has duration of Ns slots.  In order to prevent clock wrap-around problems and align with SCO slot reservation, an initialization flag in the LMP setup message indicates whether initialization procedure 1 or 2 is being used.  The slave should apply the initialization method as indicated by the initialization flag. The master uses initialization 1 when the MSB of the current master clock (CLK27) is 0; it uses initialization 2 when the MSB of the current master clock (CLK 27) is 1.  The start of each superframe is at the slot where master clock CLK satisfies the following equation:

CLK27-1 mod Ns = 0		for initialization 1

(CLK27,CLK26−1) mod Ns = 0	for initialization 2

This clause applies to both the partition sequence generation of ACL and of SCO.  Although the partition sequence of ACL does not have to align with SCO slots, this prevents the clock wrap-around problem.