IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Draft text for AFH LMP commands |  |
| Date Submitted | [21 January, 2002] |  |
| Source | [HKKC Chen et. al., A. Batra et. al.] [IPC, TI] [refer to whitepages] | Voice: [ ] Fax: [ ] E-mail: [ ] |
| Re: | [01507r1P802-15_TG2-Draft text for AFH LMP commands, 01366r1P802-15_TG2-Clause-14-3-Adaptive-Frequency-Hopping] |  |
| Abstract | [This contribution is a draft text for AFH LMP commands of TG2 Coexistence Mechanisms.] |  |
| Purpose | [To consider this draft text for AFH of the TG2 Coexistence Mechanisms.] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

IEEE P802.15
Wireless PANs

Draft text for AFH LMP commands

Date:	21 January, 2002

Author:	HK Chen, KC Huang, YC Maa and KC Chen,
	Integrated Programmable Communications, Inc.
	Taiwan Lab: PO Box 24-226, Hsinchu 300, Taiwan
	+886 3 5165106
	{hkchen, kchuang, ycmaa, kc}@inprocomm.com

LMP Commands

#### Supported features

#### The  AFH capability of  a device is exchanged through the two commands, LMP_features_req and LMP_features_res .  Extra bit fields are allocated and shown in Table xxx.  The value of a bit is set to 1 if the corresponding feature is supported.  The reserved bits should be set to 0.

| Byte | Bit | Supported feature |
| --- | --- | --- |
| 2 | 7 | Slave-reported channel classification |
| 3 | 0 | Adaptive frequency hopping |
| 3 | 1-7 | Reserved |

Ed Note: Backward compatibility issued should be addressed, since it is an modification (extension) to the original command format.

#### AFH minimum channels

The master sends LMP_AFH_minimum_channels to the slave to synchronize the value of Nmin, the minimal number of channels in the adaptive hopping sequence.  The lower bound of Nmin is usually set by the regulation.  The value of Nmin also affects the interference between Bluetooth piconets.  The master should have the geographical and regulatory information required to set Nmin.  The master may choose a higher value of Nmin considering the interference between Bluetooth piconets.

Slave

LM

Master

LM

LMP_AFH_minimum_channels

| LMP PDU | Length (bytes) | op code | Packet type | Possible direction | Contents | Position in payload |
| --- | --- | --- | --- | --- | --- | --- |
| LMP_AFH_minimum_channels | 2 | ?? | DM1 | m  s | AFH minimum channels | 2 |

| Name | Length (bytes) | Type | Unit | Detailed |
| --- | --- | --- | --- | --- |
| AFH minimum channels | 1 | u_int8 |  | Minimum number of channels in the adaptive hopping sequence |

#### Request of Slave’s Classification Result

The master may issue the LMP_ channel_metrics_req command to the slave to request its classification result.  This function may or may not be supported by the slave.

| LMP PDU | Length (bytes) | op code | Packet type | Possible direction | Contents | Position in payload |
| --- | --- | --- | --- | --- | --- | --- |
| LMP_channel_metrics_req | 1 | ?? | DM1 | m  s | - | - |

Table 3: PDUs used for master to request slave’s classification information

Master

LM

Slave

LM

LMP_channel_metrics_reqLMP_available_channel_request

LMP_channel_metrics_resLMP_slave_available_channel

Sequence 2: Master requests and receives slaves’ classification information.

A slave that supports this function should respond with the LMP_channel_metrics_res PDU.

A slave that does not support the function of classifying channels will respond with LMP_not_accepted with the reason code PDU not allowed.

Master

LM

Slave

LM

LMP_channel_metrics_reqLMP_available_channel_request

LMP_not_accepted

Sequence 3: Master request of slaves’ classification information fails

#### Slave’s Classification Result

In response to the LMP_ channel_metrics_req, a supporting slave should reply with a packet containing its classification of all the channels. For a system supporting 79-channel mode, 79 bits will be used, each bit representing each of the 79 channels. A binary ‘0’ will represent a ‘bad’ channel and a binary ‘1’ will represent a ‘not known to be bad’ channel. The LSB of the payload will relate to the quality of channel ‘0’, the MSB of the payload will relate to the quality of the last channel.  For system supporting 23-channel mode, the 23 bits from LSB represents the channel condition, and the remaining bits should be set to a binary ‘0’.

| LMP PDU | Length (bytes) | op code | Packet type | Possible direction | Contents | Position in payload |
| --- | --- | --- | --- | --- | --- | --- |
| LMP_channel_metrics_res | 11 |  | DM1 | s  m | Channel metrics bitmap | 2 – 11:7 |

Table 4: PDUs used for Slave to send classification information to master

AFH start

After the master has the channel classification information, the master notifies the slaves to the new hopping sequence.  In case of multiple AFH-capable slaves, the master should send this command to each of them respectively.  The slaves are switched to the new hopping sequence one by one.  The switching of hopping sequence is either from the legacy hopping sequence to an adaptive hopping sequence, or from the current adaptive hopping sequence to a new adaptive hopping sequence.

LMP_ accepted

The master selects the AFH instant and queues the LMP_ AFH_start to its LC for transmission to the slave.  When the slave receives the LMP_AFH_start, it switches to the new hopping sequence after the AFH instant.  If the master receives LMP_accepted, it switches to the new hopping sequence after AFH instant.

Master

LM

Slave

LM

LMP_AFH_start

LMP_not_accepted

If the master receives LMP_not_accepted, it stops switching to the new hopping sequence.

If the master does not receive any LMP response before the AFH instant, it still switches to the new hopping sequence, but it should start a checking procedure in the new hopping sequence within an AFH_timeout period.  If the master can not get confirmation within the AFH_timeout period, it then switches back to the previous hopping sequence.  If slave has SCO channels, the time-out value is chosen by the master and should be at least 6*Tsco. If slave uses ACL only, the time-out value is chosen by the master and should be at least (6*Tpoll + WB(1)).

| LMP PDU | Length | op code | Packet type | Possible direction | Contents | Position in payload |
| --- | --- | --- | --- | --- | --- | --- |
| LMP_AFH_start | 17 |  | DM1 | m->s | AFH instant Channel metrics bitmap AFH partition sequence parameters Timing control flags | 2-5 6-15 16 17 |

| Name | Length (bytes) | Type | Unit | Detailed |
| --- | --- | --- | --- | --- |
| AFH instant | 4 | u_int32 |  | Bits 27:1 of the master Bluetooth clock value |
| Channel metrics bitmap | 10 |  |  |  |
| AFH partition sequence parameters | 1 | u_int8 |  | PSP(see AFH clause) |
| Timing control flag | 1 |  |  | Bits 1 |

Return to Regular Hopping Request

After the piconet has been adaptively hopping for a time, the master may force all the Slaves back to the normal 802.15.1 / Bluetooth hopping sequence. This command is issued by the Master and contains no payload.

| LMP PDU | Length (bytes) | op code | Packet type | Possible direction | Contents | Position in payload |
| --- | --- | --- | --- | --- | --- | --- |
| LMP_regular_hopping_req | 1 | ?? | DM1 | m  s | - | - |

Table 6: PDUs used for Master to request Slave return to regular hopping mode.

The Master will not send such a command to an unsupporting Slave, so the default response after receiving this command is an LMP_accepted message.

Master

LM

Slave

LM

LMP_regular_hopping_request

LMP_accepted

Sequence 5: Master requests slave to return to regular 802.15.1 hopping mode.

#### AFH hopping sequence check

This LMP command is for formal check of hopping sequence switching.  The master sends LMP_AFH_check_req to request the slave sending back a set of information that fully describes the slave’s adaptive hopping sequence.  Then the master is able to know which adaptive hopping sequence the slave is using.  This mechanism is more reliable than just sending a POLL packet and see if there is an Baseband-ACK.  The POLL-ACK mechanism may not be accurate and reliable because of the similarity between hopping sequences.  When

the slave receives LMP_AFH_check_req, it should respond by LMP_AFH_check_res with the set of information used to form its current hopping sequence.

Master

LM

Slave

LM

LMP_AFH_check_req

LMP_AFH_check_res

| LMP PDU | Length | op code | Packet type | Possible direction | Contents | Position in payload |
| --- | --- | --- | --- | --- | --- | --- |
| LMP_AFH_check_req | 1 |  |  | m->s | - | - |

| LMP PDU | Length | op code | Packet type | Possible direction | Contents | Position in payload |
| --- | --- | --- | --- | --- | --- | --- |
| LMP_AFH_check_res | 13 |  | DM1 | s->m | Channel metrics bitmap AFH partition sequence parameters Timing control flags | 2-11 12 13 |

| Name | Length (bytes) | Type | Unit | Detailed |
| --- | --- | --- | --- | --- |
| Channel metrics bitmap | 10 |  |  |  |
| AFH partition sequence parameters | 1 | u_int8 |  | PSP |
| Timing control flag | 1 |  |  | Bits 1 |