IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Draft text for AFH LMP commands |  |
| Date Submitted | [24 January, 2002] |  |
| Source | [HK Chen et. al.] [IPC] [refer to whitepages] | Voice: [ ] Fax: [ ] E-mail: [ ] |
| Re: | [01507r5P802-15_TG2-Draft text for AFH LMP commands, 01366r1P802-15_TG2-Clause-14-3-Adaptive-Frequency-Hopping] |  |
| Abstract | [This contribution is a draft text for AFH LMP commands of TG2 Coexistence Mechanisms.] |  |
| Purpose | [To consider this draft text for AFH of the TG2 Coexistence Mechanisms.] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

IEEE P802.15
Wireless PANs

Draft text for AFH LMP commands

Date:	24 January, 2002

Author:	HK Chen, KC Huang, YC Maa and KC Chen,
	Integrated Programmable Communications, Inc.
	Taiwan Lab: PO Box 24-226, Hsinchu 300, Taiwan
	+886 3 5165106
	{hkchen, kchuang, ycmaa, kc}@inprocomm.com

### LMP Commands

Change request 1: insert the following text to clause 10.3.2

#### Supported features

#### The  AFH capability of  a device is exchanged through the two commands, LMP_features_req and LMP_features_res .  Extra bit fields are allocated and shown in Table xxx.  The value of a bit is set to 1 if the corresponding feature is supported.  The reserved bits should be set to 0.

| Byte | Bit | Supported feature |
| --- | --- | --- |
| 2 | 7 | Slave-reported channel classification |
| 3 | 0 | Adaptive frequency hopping |
| 3 | 1-7 | Reserved |

The new format uses one undefined bit in byte 2 and one bit in an additional byte.   This causes a different payload length.   For compatibility between the two formats, the following rules should apply.  A received LMP_features_res packet with the payload length different from expected should not be treated as an error.   If the payload length of the received LMP_features_res packet is longer than expected, the extra byte(s) should be ignored.   If the payload length of the received LMP_features_res packet is shorter than expected, the bits of lacking byte(s) should  be treated as all 0 and the corresponding features should be assumed unsupported.  The non-zero values in the undefined or reserved bits of the LMP_features_res should be ignored.

Change request 2: insert the following text to clause 10.3.2

#### AFH minimum channels

The master sends LMP_AFH_minimum_channels to the slave to harmonize the value of Nmin, the minimal number of channels in the adaptive hopping sequence.  The lower bound of Nmin is usually set by the regulation.    The master should have the geographical and regulatory information required to set Nmin.  The master may choose a higher value of Nmin than regulatory requirements.

Master

LM

Slave

LM

LMP_AFH_minimum_channels

| LMP PDU | Length (bytes) | op code | Packet type | Possible direction | Contents | Position in payload |
| --- | --- | --- | --- | --- | --- | --- |
| LMP_AFH_minimum_channels | 2 | TBD | DM1 | m  s | AFH minimum channels | 2 |

| Name | Length (bytes) | Type | Unit | Detailed |
| --- | --- | --- | --- | --- |
| AFH minimum channels | 1 | u_int8 |  | Minimum number of channels in the adaptive hopping sequence |

Change request 3: insert the following text to clause 10.3.2

#### Request of Slave’s Classification Result

The master may issue the LMP_ channel_metrics_req command to the slave to request its classification result.  This function may or may not be supported by the slave.

| LMP PDU | Length (bytes) | op code | Packet type | Possible direction | Contents | Position in payload |
| --- | --- | --- | --- | --- | --- | --- |
| LMP_channel_metrics_req | 1 | TBD | DM1 | m  s | - | - |

Table 3: PDUs used for master to request slave’s classification information

Master

LM

Slave

LM

LMP_channel_metrics_req

LMP_channel_metrics_res

Sequence 2: Master requests and receives slaves’ classification information.

A slave that supports this function should respond with the LMP_channel_metrics_res PDU.  A slave that does not support the function of classifying channels will respond with LMP_not_accepted with the reason code PDU not allowed.

Master

LM

Slave

LM

LMP_channel_metrics_req

LMP_not_accepted

Sequence 3: Master request of slaves’ classification information fails

#### Slave’s Classification Result

In response to the LMP_ channel_metrics_req, a supporting slave should reply with a packet containing its classification of all the channels. For a system supporting 79-channel mode, 79 bits will be used, each bit representing each of the 79 channels. A binary ‘0’ will represent a ‘bad’ channel and a binary ‘1’ will represent a ‘good’ channel. The LSB of the payload should represent  the result of channel classification of channel ‘0’, the MSB of the payload should represent  the result of channel classification  of the last channel.  For system supporting 23-channel mode, the 23 bits from LSB represents the channel metrics, and the remaining bits should be set to a binary ‘0’.

| LMP PDU | Length (bytes) | op code | Packet type | Possible direction | Contents | Position in payload |
| --- | --- | --- | --- | --- | --- | --- |
| LMP_channel_metrics_res | 11 | TBD | DM1 | s  m | Channel metrics bitmap | 2 – 11:7 |

Table 4: PDUs used for Slave to send classification information to master

Change request 4: insert the following text to clause 10.3.2

#### AFH start

After the master has the channel classification information, the master notifies the slaves to the new hopping sequence.  In case of multiple AFH-capable slaves, the master should send this command to each of them respectively.  The slaves are switched to the new hopping sequence one by one.  The switching of hopping sequence is either from the legacy hopping sequence to an adaptive hopping sequence, or from the current adaptive hopping sequence to a new adaptive hopping sequence.

Master

LM

Slave

LM

LMP_AFH_start

The master should make sure the slave support AFH feature before sending the  LMP_AFH start.  The master queues the LMP_AFH_start to its LC for transmission to the slave.  When the master queues the LMP_AFH_start, it starts a timer for 6*Tpoll slots. If the Baseband-level acknowledgement is received before this timer expires, the master switches to the new hopping sequence at the next master Tx slot. Otherwise, the master switches to the new hopping sequence when the timer expires.

When the slave receives the LMP_AFH_start, it switches to the new hopping sequence at the next slave Rx slot such that there are at least one slave Tx slots for the Baseband-level acknowledgement to pass before the slave switches to the new hopping sequence.

If the master switches to the new hopping sequence due to timer expires (no Baseband-level acknowledgement for LMP_AFH_start), after switching to the new hopping sequence, the master should start a checking procedure in the new hopping sequence within an AFH_timeout period.  If the master cannot get confirmation within the AFH_timeout period, it then switches back to the previous hopping sequence.  If the slave has SCO channels, the time-out value is chosen by the master and should be at least 6*Tsco. If the slave uses ACL only, the time-out value is chosen by the master and should be at least (6*Tpoll + WB(1)).

The minimal number of channels in the adaptive hopping sequence, Nmin, is also distributed in the LMP_AFH_start.  The lower bound of Nmin is usually set by the regulation.    The master should have the geographical and regulatory information required to set Nmin.  The master may choose a higher value of Nmin than regulatory requirements.

| LMP PDU | Length | op code | Packet type | Possible direction | Contents | Position in payload |
| --- | --- | --- | --- | --- | --- | --- |
| LMP_AFH_start | 14 | TBD | DM1 | m->s | AFH minimum channels Channel metrics bitmap AFH partition sequence parameters Timing control flags | 2 3-12 13 14 |

| Name | Length (bytes) | Type | Unit | Detailed |
| --- | --- | --- | --- | --- |
| AFH minimum channels | 1 | u_int8 |  | Minimum number of channels in the adaptive hopping sequence |
| Channel metrics bitmap | 10 |  |  |  |
| AFH partition sequence parameters | 1 | u_int8 |  | PSP(see AFH clause) |
| Timing control flag | 1 |  |  | Bits 1(see Bluetooth spec 1.1, part C, 5.1) |

Change request 6: insert the following text to clause 10.3.2

#### Return to Regular Hopping Request

After the piconet has been adaptively hopping for a time, the master may force all the slaves back to the normal 802.15.1 / Bluetooth hopping sequence. This command is issued by the master and contains no payload other than the opcode.

| LMP PDU | Length (bytes) | op code | Packet type | Possible direction | Contents | Position in payload |
| --- | --- | --- | --- | --- | --- | --- |
| LMP_regular_hopping_req | 1 | TBD | DM1 | m  s | - | - |

Table 6: PDUs used for Master to request Slave return to regular hopping mode.

The master will not send such a command to an unsupporting slave, so the default response after receiving this command is an LMP_accepted message.

Master

LM

Slave

LM

LMP_regular_hopping_req

LMP_accepted

Sequence 5: Master requests slave to return to regular 802.15.1 hopping mode.

Change request 5: insert the following text to clause 10.3.2

#### AFH hopping sequence check

This LMP command is for formal check of hopping sequence switching.  The master sends LMP_AFH_check_req to request the slave sending back a set of information that fully describes the slave’s adaptive hopping sequence.  Then the master is able to know which adaptive hopping sequence the slave is using.  This mechanism is more reliable than just sending a POLL packet and see if there is a Baseband-ACK.  The POLL-ACK mechanism may not be accurate and reliable because of the similarity between hopping sequences.  When the slave receives LMP_AFH_check_req, it should respond by LMP_AFH_check_res with the set of information used to form its current hopping sequence.

Master

LM

Slave

LM

LMP_AFH_check_req

LMP_AFH_check_res

| LMP PDU | Length | op code | Packet type | Possible direction | Contents | Position in payload |
| --- | --- | --- | --- | --- | --- | --- |
| LMP_AFH_check_req | 1 | TBD |  | m->s | - | - |

| LMP PDU | Length | op code | Packet type | Possible direction | Contents | Position in payload |
| --- | --- | --- | --- | --- | --- | --- |
| LMP_AFH_check_res | 13 | TBD | DM1 | s->m | Channel metrics bitmap AFH partition sequence parameters Timing control flags | 2-11 12 13 |

| Name | Length (bytes) | Type | Unit | Detailed |
| --- | --- | --- | --- | --- |
| Channel metrics bitmap | 10 |  |  |  |
| AFH partition sequence parameters | 1 | u_int8 |  | PSP |
| Timing control flag | 1 |  |  | Bits 1 |