IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | 02032r0P802-15_TG3-Basic-Power-Save.doc |  |
| Date Submitted | 23 Jan., 2002 |  |
| Source | Dr. William Shvodian <XtremeSpectrum> Suite 700, 8133 Leesburg Pike, Vienna, VA 22182 | Voice: 703-269-3047 Fax: 703-269-3092 E-mail: bshvodian@xtremespectrum.com |
| Re: | D09 |  |
| Abstract | This document provides the MLMEs for power save |  |
| Purpose | The recommendations contained in this document are to be applied to the 802.15.3 D09 baseline. It is meant to replace the EPS MLMEs in D09. |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

1)	Changes to Clause 6	3

# Changes to Clause 6

This describes changes to the MLMEs for power management.

## 6.3.1.1   MLME-Sleep.request

This primitive is used to request that the DEV be allowed to transition into sleep mode.  The semantics of the primitive are as follows:

MLME-Sleep.request			(

MaxSleepDuration

)

The parameters are defined in Table 1.

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| MaxSleepDuration | Duration | 0-65535 ms | The maximum time interval that a DEV can be in BPS mode, where it enters a low power state where it does not receive Beacons or other frames. |
| RequesterDEVID | MACAddress | Any Valid MAC Address | The MAC address of the DEV whishing to sleep |

Table 1 MLME- Sleep primitive parameters

### 6.3.1.1.1 When Generated

This is generated by the DME when it wishes to enter sleep state.

### 6.3.1.1.2 Effect of Receipt

This request causes the MLME to generate and send a Sleep Request command to the PNC.

## 6.3.1.2   MLME-Sleep.indication

This primitive reports the receipt of a Sleep Request from another DEV.  The semantics of this primitive is as follows:

MLME-Sleep.indication		(

RequesterDEVID

MaxSleepDuration

)

The parameters are defined in Table 1.

### 6.3.1.2.1 When Generated

This is generated by the DME when it wishes to enter sleep state.

### 6.3.1.2.2 Effect of Receipt

This request causes the MLME to generate and send a Sleep Request command to the PNC.

## 6.3.1.3 MLME-Sleep.response

This primitive is used by the PNC to allow a DEV to transition into sleep mode.  The semantics of the primitive are as follows:

MLME-Sleep.response		(

MaxSleepDuration

)

The parameters are defined in Table 1.

### 6.3.1.3.1 When Generated

This is generated by the PNC DME in response to an MLME-Sleep.indication.

### 6.3.1.3.2 Effect of Receipt

This indication causes the MLME to generate and send a Sleep Permit command.

## 6.3.1.3 MLME-Sleep.confirm

This primitive allows a DEV to transition into sleep mode.  The semantics of the primitive are as follows:

MLME-Sleep.response		(

MaxSleepDuration

)

The parameters are defined in Table 1.

### 6.3.1.3.1 When Generated

This is generated by the DEV MLME in response to receiving a Sleep Permit command.

### 6.3.1.3.2 Effect of Receipt

This confirm allows the DEV to go into sleep mode.

## 6.3.1.3 MLME-Sleep-Continue.request

This primitive is used to request that the DEV be allowed to stay in Power Save mode.  The semantics of the primitive are as follows:

MLME-Sleep-Continue.request	(

MaxSleepDuration

)

The parameters are defined in Table 1.

### 6.3.1.1.1 When Generated

This is generated by the DME when it wishes to stay in sleep state.

### 6.3.1.1.2 Effect of Receipt

This request causes the MLME to generate and send a Sleep Continue command to the PNC.

## 6.3.1.2   MLME-Sleep-Continue.indication

This primitive reports the receipt of a Sleep Request from another DEV.  The semantics of this primitive is as follows:

MLME-Sleep-Continue.indication	(

RequesterDEVID

MaxSleepDuration

)

The parameters are defined in Table 1.

### 6.3.1.2.1 When Generated

This is generated by the PNC MLME when it receives a Sleep Continuation command.

### 6.3.1.2.2 Effect of Receipt

This request lets the PNC DME know that the requesting DEV will stay in EPS state.