IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | List of change requests to 802.15.2 Draft |  |
| Date Submitted | 16 Jan, 2002 |  |
| Source | [Bijan Treister et al., Efstratios Skafidas, Hongbing Gan, Vitaliy Sapozhnykov, Ben Jones] [Bandspeed] [Level 9, 500 Collins St., Melbourne Australia] | Voice: [ +61396146299 ] Fax: [ +61396146699 ] E-mail: [ {b.treister,e.skafidas,b.jones, h.gan, v.sapozhnykov}@bandspeed.com.au] |
| Re: | [modifications to 802.15.2 Draft standard 01000D02P802-15-__Draft_Recommended_Practice.pdf] |  |
| Abstract | [Changes to standard, in particular, changes to allow for further clarification of the AFH standard] |  |
| Purpose | [To be discussed and further voted on to be accepted as modifications and additions to the current standard] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

# Suggested changes:

The following is a list of changes for the 802.15.2 Draft standard.

## Addition #1 to clause 10.3

Device Identification

Channel Classification

Classification Information exchange

Adaptive Hopping

1

2

3

4

Figure X: Adaptive hopping outline

The mechanism can be described in the following manner:

Step 1: The devices in the piconet are identified as supporting AFH or not.

Step 2: The Master classifies the channels.

Step 3: The master distributes the information about the quality of the channels to all supporting slaves.

Step 4: The piconet starts adaptively hopping. After a predefined time, the piconet will return to channel classification.

Comment: Place at end of 10.3. NB: Step 1 may not be needed. This diagram gives a easy to understand overview of AFH. No general description of the method is explained as current.

## Modification #1 to clause 10.3.1.3

Clause 10.3.1.3 to be described as follows:

### 10.3.1.3 Frequency Re-mapping

The frequency re-mapping unit simply remaps the output of the Bluetooth / 802.15.1 selection kernel to another frequency. A list of channel onto which the output needs to be remapped is stored and when directed, the re-mapping system will choose one of those channels.

In the case where NG  Nmin, all bad channels are remapped to good channels chosen from a good channel bank. This is outlined in Figure Xc.

fhop

fadp

1 + CLK

Mod NG

.

.

GNG

GNG-1

GNG-2

GNG-3

GNG-4

GNG-5

GNG-6

GNG-7

.

G1

G2

G3

G4

G6

G5

G7

List of good channels

Figure Xc: Re-mapping of a bad channel into a good channel

7

fhop

fadp

1 + CLK

Mod Nb

.

.

BNG

BNG-1

BNG-2

BNG-3

BNG-4

BNG-5

BNG-6

BNG-7

.

B1

B2

B3

B4

B6

B5

B7

List of bad channels

Figure X: Re-mapping of a good channel into a bad channel

7

In the case where NG < Nmin and a partition sequence generator is used, the input p(k) is used to either re-map a good channel to a bad channel, to re-map a bad channel to a good channel or not to re-map at all. The input p(k) is described in 10.3.1.1, and use is described in the pseudocode below.

Note that a frequency re-mapping function is a necessity for all adaptive hopping schemes.

Include Pseudo-code from draft D02-2001 here

For a device that does not use a partition sequence generator, the pseudo-code simplifies as follows:

/* Check if this is a bad channel */

if (fhop is an element of SBK)

{

/* Replace it with a good channel */

index = (fhop + 1 + CLK) mod NG

fadp   =  SG(index)

}

else

{

/* No replacement required */

fadp	= fhop

}

## Addition #2 to clause 10.3

Comment: To expand on the statement made in the last line of the third paragraph on page 56 the following should be stated (clause 10.3):

Selection

Box

27

UAP/LAP

CLOCK

fgen

23/79 Mode

Frequency

Re-mapping

fadp

AM_ADDR

CLKE / CLKN

28

Figure X: When NG > Nmin the AFH algorithm simplifies

A system which does not include the partition sequence generator or in the case where NG > Nmin , the system simplifies to the selection kernel and a frequency re-mapping unit. Such a system remaps the ‘bad’ or ‘occupied’ channels from the hopping sequence to the set of ‘good’ channels as described in 10.3.1.3, no bad channels are utilized.

## Modification #2 to clause 10.3

Figure 37 caption to state:

“An example of an adapted hopping sequence structure when NG < Nmin “

To clarify the case where this example is valid

Addition to first line of 10.3.1.2:

The partition sequence generator’s functionality is utilized in the case where NG < Nmin

To explain the function of the partition sequence generator