IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | List of change requests to 802.15.2 Draft |  |
| Date Submitted | 16 Jan, 2002 |  |
| Source | [Bijan Treister et al., Efstratios Skafidas, Hongbing Gan, Vitaliy Sapozhnykov, Ben Jones] [Bandspeed] [Level 9, 500 Collins St., Melbourne Australia] | Voice: [ +61396146299 ] Fax: [ +61396146699 ] E-mail: [ {b.treister,e.skafidas,b.jones, h.gan, v.sapozhnykov}@bandspeed.com.au] |
| Re: | [modifications to 802.15.2 Draft standard 01000D02P802-15-__Draft_Recommended_Practice.pdf] |  |
| Abstract | [Changes to standard, in particular, changes to allow for further clarification of the AFH standard] |  |
| Purpose | [To be discussed and further voted on to be accepted as modifications and additions to the current standard] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

## Change #1 to clause 10.3

There are four main elements of the adaptive hopping procedure:

Device Identification:

Device identification occurs to inform the Master as to which Slave supports adaptive frequency hopping and the associated parameters.

Channel Classification:

Classification of the channels occurs in the Master device and optionally in the Slaves. Classification is essentially the process by which channels are classified as either ‘good’ or ‘bad’.

Exchange of Classification information

The information pertaining to the quality of the channels is distributed to the supporting Slaves in the piconet. This is done in a reliable manner using special AFH LMP commands.

Adaptive Hopping

Adaptive Hopping is the stage at which part, or all of the piconet will be using a subset of channels to hop over.

## Change #2 to clause 10.3.1

Selection

Box

27

UAP/LAP

CLK

fgen

23/79 Mode

Frequency

Re-mapping

fadp

AM_ADDR

CLK

28

Partition Sequence Generator

Channel Conditions

(SG, SBK, NG, NBK)

Traffic type

(ACL or SCO)

p(k)*

when NG  Nmin , the input p(k) is a constant ‘1’