IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | List of change requests to 802.15.2 Draft |  |
| Date Submitted | 16 Jan, 2002 |  |
| Source | [Bijan Treister et al., Efstratios Skafidas, Hongbing Gan, Vitaliy Sapozhnykov, Ben Jones] [Bandspeed] [Level 9, 500 Collins St., Melbourne Australia] | Voice: [ +61396146299 ] Fax: [ +61396146699 ] E-mail: [ {b.treister,e.skafidas,bjones, h.gan, v.sapozhnykov}@bandspeed.com.au] |
| Re: | [modifications to 802.15.2 Draft standard 01000D02P802-15-__Draft_Recommended_Practice.pdf] |  |
| Abstract | [Changes to standard, in particular, changes to allow for further clarification of the AFH standard] |  |
| Purpose | [To be discussed and further voted on to be accepted as modifications and additions to the current standard] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

## Change request #1 to clause 10.3

Comment: Add this to the end of clause 10.3 (before clause 10.3.1)

There are four main elements of the adaptive hopping procedure:

AFH Capability Discovery:

AFH capability discovery occurs to inform the Master as to which Slave supports adaptive frequency hopping and the associated parameters.

Channel Classification:

Classification of the channels occurs in the Master device and optionally in the Slaves. Classification is the process by which channels are classified as either ‘good’ or ‘bad’.

Channel Classification Information Exchange:

The channel classification information is exchanged between the Master and the supporting Slaves in the piconet. This is done in a reliable manner using special AFH LMP commands.

Adaptive Hopping:

Adaptive Hopping is the operation of hopping over a subset of channels.

## Change #2 to clause 10.3.1

Comment: to replace figure 36 in clause 10.3.1

Selection

Box

27

UAP/LAP

CLK

fhop

23/79 Mode

Frequency

Re-mapping

fadp

AM_ADDR

CLK

28

Partition Sequence Generator

Channel Conditions

(SG, SBK, NG, NBK)

Traffic type

(ACL or SCO)

p(k)*

Figure 36—Block diagram of the adaptive frequency hopping mechanism

*when NG  Nmin , p(k) = 1; when NG < Nmin , p(k)  {0,1}