- IEEE P802.15

- Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Change request for 802.15.2 Recommended Practice PHY text |  |
| Date Submitted | [January 24, 2002] |  |
| Source | [Aik Chindapol] [Siemens] [16745 W. Bernardo Drive San Diego, CA 92127] [Adrian Stephens, Jim Lansford] [Mobilian] [7431 NW Evergreen Pkwy, Hillsboro, OR 97124, USA] | Voice: [ 858-521-3546 ] Fax: [ 858-521-3108 ] E-mail: [ aik.chindapol@icm.siemens.com ] Voice: [ +1 (503) 681-8600 ] Fax: [ +1 425 671 6099 ] E-mail: [ jim.lansford@mobilian.com adrian.stephens@mobilian.com ] |
| Re: | P802.15.2_D01_Recommended_Practice |  |
| Abstract | The BER equations and figure for 5.5 and 11MB/s needs to be corrected to reflect a more accurate calculation. |  |
| Purpose | Correction to BER calculation for 5.5 and 11 Mbps 802.11b |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

- Change Request #1

- Section 5.3.6.4 - 802.11b 5.5 Mbit/s BER calculation

- Replace the following paragraph

- As each symbols encodes 4 bits, the effective BER is

BER=min(1−(1−SER5.5)14,0.5)

in which the value has been limited to 0.5.

with

- As each symbols encodes 4 bits, the average BER is

BER=24−124−1SER5.5.

- Change Request #2

- Section 5.3.6.5 - 802.11b 11 Mbit/s BER calculation

- Replace the following paragraph

- As each symbols encodes 8 bits, the effective BER is

BER=min(1−(1−SER11)18,0.5).

with

- As each symbols encodes 8 bits, the average BER is

BER=28−128−1SER11.

- Change Request #3

- Section 5.3.6.8 - BER versus SINR results

- Replace Figure 5

Change Request # 4

Change two lines in Annex A as follows:

/*---------------------------------------------------------------------------

* Title:    Analytical Physical-Layer Model for 802.15.2 BER Calculations

* Authors:  Ron Nevo, Josie Ammer, Adrian Stephens

*           Mobilian Corporation 2001

*

* This module contains the analytical PHY-layer model used to calculate

* BER values for 802.11b and 802.15.1 transmissions in the presence of

* mutual interference.

*---------------------------------------------------------------------------*/

/*-- Standard Includes ------------------------------------------------------*/

#include <stdio.h>

#include <math.h>

#include <stdlib.h>

#include <limits.h>

/*-- Type Definitions -------------------------------------------------------*/

typedef enum

{

WPAN, WLAN11, WLAN55, WLAN1, WLAN2

// WPAN is non-FEC WPAN transmissions

// WLAN11 is 11Mb/s 802.11

// WLAN55 is 5.5Mb/s 802.11

// WLAN1 is 1Mb/s 802.11

// WLAN2 is 2Mb/s 802.11

} ModulationType;

struct Node

{

double x,y;  // x and y positions in meters

};

typedef Node *aNodePtr;

struct Transmission

{

aNodePtr src, dst;

ModulationType type;

float txpower;  // power in mW

int frequency;

// freq for WPAN is a number 1-79, is the center frequency

// freq for WLAN is number 1-79, is the center frequency

double BER;     // the resulting BER

};

typedef Transmission *aTransmissionPtr;

/*---------------------------------------------------------------------------

* Function: CalculateAnalyticalBER

*

* Description:

*              This function takes a list of transmissions and calculates

*              the BER at each receiver using the analytical model.

*

* Parameters:  n       - the number of transmissions in the list

*              tlist   - an array of Transmissions each corresponding to

*                        an active transmission

*

* Returns: in every tlist element a calculated BER value

*---------------------------------------------------------------------------*/

extern void CalculateAnalyticalBER(int n, Transmission tlist[]);

/*-- MACRO Definitions ------------------------------------------------------*/

#define WLAN_BANDWIDTH 22

#define HALF_WLAN_BANDWIDTH 11

#define PI acos(-1.0)

// assume the power of WPAN reception at the same station is unit power

#define Tc 1

#define WPAN_Tx_1 inverse_db(-20.0)

// -20dB spurious emmision per MHz from WPAN Tx when freq dif is 1MHz

#define WPAN_Tx_2 inverse_db(-40.0)

// -40dB spurious emmision per MHz from WPAN Tx when freq dif is 2MHz

#define WPAN_Tx_3 inverse_db(-60.0)

// -60dB spurious emmision per MHz from WPAN Tx when freq dif is 3MHz

#define WPAN_Tx_other 0

// no spurious emmision from WPAN Tx when freq dif > 3MHz

#define WPAN_Rx_1 inverse_db(-11.0)

// -11dB adjacent channel interference from WPAN Rx when freq dif is 1MHz

#define WPAN_Rx_2 inverse_db(-41.0)

// -41dB adjacent channel interference from WPAN Rx when freq dif is 2MHz

#define WPAN_Rx_other inverse_db(-51.0)

// -51dB adjacent channel interference from WPAN Rx when freq dif >= 3MHz

#define WLAN_Tx_next inverse_db(-30)

// -30dB spurious emmision from WLAN Tx when freq dif <= half bandwidth

#define WLAN_Tx_other inverse_db(-50.0)

// -50dB spurious emmision from WLAN Tx when freq dif > half bandwidth

#define WLAN_Rx_next inverse_db(-12.0)

// -12dB adjacent channel interference from WLAN Rx, 12 MHz

#define WLAN_Rx_middle inverse_db(-36.0)

// -60dB adjacent channel interference from WLAN Rx, between 13MHz-36MHz

#define WLAN_Rx_other inverse_db(-56.0)

// -60dB adjacent channel interference from WLAN Rx, >= 37MHz

#define CCK_factor inverse_db(-8.0)

// 8dB gain for CCK coding

#define WLAN_EbNo_perfect 10.0   // if EbNo > 10dB, perfect reception

#define WLAN_EbNo_impossible 0.5 // if EbNo < -3dB, impossible to receiver

#define WPAN_EbNo_perfect 20.0   // if EbNo > 13dB, perfect reception

#define WPAN_EbNo_impossible 1.0 // if EbNo < 0dB, impossible to receive

#define MIN_DISTANCE 0.1

// two nodes in the same spot act as if they are 0.1 apart

/*---------------------------------------------------------------------------*/

#define abs(a) ((a)>0 ? (a) : -(a))

#define sqr(x) ((x)*(x))

#define min(a,b) ((a)<(b) ? (a) : (b))

/*-- Forward References -----------------------------------------------------*/

extern int isModulationTypeWPAN(ModulationType foo);

// returns 1 if the type is WPAN

// returns 0 otherwise

extern int isModulationTypeWLAN(ModulationType foo);

// returns 1 if the type is one of the WLANs

// returns 0 otherwise

extern double SER11(double);

extern double SER55(double);

extern double WLAN_BER_11(double);

extern double WLAN_BER_55(double);

extern double WLAN_BER_1(double);

extern double WLAN_BER_2(double);

extern double WPAN_BER(double);

extern double SpectrumFactor(Transmission &Src, Transmission &Dest);

extern double Distance(Transmission &Src, Transmission &Dest);

extern double PowerDistance(Transmission &Src, Transmission &Dest);

extern double inverse_db(double);

extern double db(double);

/*-- Local Functions --------------------------------------------------------*/

int isModulationTypeWPAN(ModulationType foo)

{

switch (foo){

case WPAN: return 1;

default: return 0;

}

}

int isModulationTypeWLAN(ModulationType foo)

{

switch (foo){

case WLAN11: return 1;

case WLAN55: return 1;

case WLAN1: return 1;

case WLAN2: return 1;

default: return 0;

}

}

// compute db from real

double db(double x)

{

return(10 * log10(x));

}

// compute real from db

double inverse_db(double x)

{

return(pow(10.0, x /10));

}

// compute the Q function using approximation Q_5

double Q_5(double x)

{

double x2,x3,x4,x5,x6;

x2 = x*x;

x3 = x2*x;

x4 = x3*x;

x5 = x4*x;

x6 = x5*x;

return(exp(-x2/2) * (x4+9*x2+8) /(x5+10*x3+15*x) / sqrt(2*PI));

}

// compute the codeword error probability of 802.11b 11Mbps

double SER11(double Eb_No)

{

double res;

res = 24*Q_5(sqrt(4*Eb_No)) +

16*Q_5(sqrt(6*Eb_No)) +

174*Q_5(sqrt(8*Eb_No)) +

16*Q_5(sqrt(10*Eb_No)) +

24*Q_5(sqrt(12*Eb_No)) +

Q_5(sqrt(16*Eb_No));

return(min(res,0.99999));

}

// compute bit error rate from Eb/No for 802.11b 11Mbps

double WLAN_BER_11(double Eb_No)

{

if(Eb_No > WLAN_EbNo_perfect)

return 0;

// if Eb/No more than some threshold, perfect reception

else if(Eb_No < WLAN_EbNo_impossible)

return 0.5;

// if Eb/No less than some threshold, impossible to receive

else

return (128.0/255.0) * SER11(SNIR);

}

// compute the codeword error probability of 802.11b 5.5Mbps

double SER55(double Eb_No)

{

double res;

res = 14*Q_5(sqrt(8*Eb_No)) +

Q_5(sqrt(16*Eb_No));

return(min(res,0.99999));

}

// compute bit error rate from Eb/No for 802.11b 11Mbps

double WLAN_BER_55(double Eb_No)

{

if(Eb_No > WLAN_EbNo_perfect)

return 0;

// if Eb/No more than some threshold, perfect reception

else if(Eb_No < WLAN_EbNo_impossible)

return 0.5;

// if Eb/No less than some threshold, impossible to receive

else

return (4.0/7.0) * SER55(SNIR);

}

// compute the function number of choice of k elements from n

int choose(int k,int n)

{

int i;

int res = 1;

for(i=n;i>n-k;i--)

res *= i;

for(i=1;i<=k;i++)

res /= i;

return(i);

}

// compute the BER for WLAN 1Mbps, the BER is Q(sqrt(11*2*Eb_No/2))

double WLAN_BER_1(double Eb_No)

{

if(Eb_No > WLAN_EbNo_perfect)

return 0;

// if Eb/No more than some threshold, perfect reception

else if(Eb_No < WLAN_EbNo_impossible)

return 0.5;

// if Eb/No less than some threshold, impossible to receive

else

return(min(Q_5(sqrt(11*2*Eb_No/2)),0.5));

}

// compute the BER for WLAN 2Mbps, the BER is Q(sqrt(5.5*2*Eb_No/2))

double WLAN_BER_2(double Eb_No)

{

if(Eb_No > WLAN_EbNo_perfect)

return 0;

// if Eb/No more than some threshold, perfect reception

else if(Eb_No < WLAN_EbNo_impossible)

return 0.5;

// if Eb/No less than some threshold, impossible to receive

else

return(min(Q_5(sqrt(5.5*2*Eb_No/2)),0.5));

}

// compute the BER for WPAN

double WPAN_BER(double Eb_No)

{

if(Eb_No > 20)

return 0;

// if Eb/No more than 13dB, perfect reception

else if(Eb_No < 1)

return 0.5;

// if Eb/No less than 0dB, impossible to receive

else

return(min(exp(-Eb_No/2),0.5));

}

double Distance(Transmission &Src, Transmission &Dest)

{

return(sqrt(sqr(Src.src->x-Dest.dst->x) + sqr(Src.src->y-Dest.dst->y)));

}

// power as function of distance

double PowerDistance(Transmission &Src, Transmission &Dest)

{

double power_d0,dist;

dist=Distance(Src,Dest); // calc distance function

if (dist < MIN_DISTANCE)

dist = MIN_DISTANCE;

if(isModulationTypeWPAN(Src.type)) {    // transmitter is WPAN,

power_d0 = Src.txpower;

}

else // transmitter is WLAN

if(isModulationTypeWPAN(Dest.type)) // receiver is WPAN,

power_d0 = Src.txpower/WLAN_BANDWIDTH;

else

power_d0 = Src.txpower; // receiver is WLAN

if(dist < 8)  // use 40.2+20log d for <8M power loss

return(power_d0/(pow(dist,2.0) * pow(10.0, 4.02)));

else         // use 58.5 + 33log(d/8) for >8M power loss

return(power_d0/(pow(dist/8.0,3.3) * pow(10.0, 5.85)));

}

double SpectrumFactor(Transmission &Src, Transmission &Dest)

// (int src0, int src,int dest)

// dest is listenin to src0

// dest gets interference from src

{

int freq_dif;

if(isModulationTypeWPAN(Dest.type))

if(isModulationTypeWPAN(Src.type))

{

// both src and dest WPAN

freq_dif = abs(Src.frequency - Dest.frequency);

switch(freq_dif) {

case 0:

// frequency collide

return(1.0);

case 1:

return(WPAN_Tx_1+WPAN_Rx_1);

case 2:

return(WPAN_Tx_2+WPAN_Rx_2);

case 3:

return(WPAN_Tx_3+WPAN_Rx_other);

default:

return(WPAN_Tx_other+WPAN_Rx_other);

}

}

else

{

// src WLAN and dest WPAN

if(Dest.frequency >= Src.frequency + HALF_WLAN_BANDWIDTH)

freq_dif = Dest.frequency - (Src.frequency + HALF_WLAN_BANDWIDTH) + 1;

else if(Dest.frequency <= Src.frequency - HALF_WLAN_BANDWIDTH)

freq_dif = (Src.frequency - HALF_WLAN_BANDWIDTH) - Dest.frequency + 1;

else

freq_dif = 0;

switch(freq_dif) {

case 0:

// frequency collide

return(1.0);

case 1:

return(WLAN_Tx_next+WPAN_Rx_1);

case 2:

return(WLAN_Tx_next+WPAN_Rx_2);

default:

if(freq_dif<=HALF_WLAN_BANDWIDTH)

return(WLAN_Tx_next+WPAN_Rx_other);

else

return(WLAN_Tx_other+WPAN_Rx_other);

}

}

else // dest WLAN

{

if(isModulationTypeWPAN(Src.type)) {

// src WPAN, dest WLAN

double sf;

if(Src.frequency >= Dest.frequency + HALF_WLAN_BANDWIDTH)

freq_dif = Src.frequency - (Dest.frequency + HALF_WLAN_BANDWIDTH) + 1;

else if(Src.frequency <= Dest.frequency - HALF_WLAN_BANDWIDTH)

freq_dif = (Dest.frequency - HALF_WLAN_BANDWIDTH) - Src.frequency + 1;

else

freq_dif = 0;

switch(freq_dif) {

case 0:

// frequency collide

sf = 1.0;

break;

case 1:

sf = WPAN_Tx_1+WPAN_Tx_2+WPAN_Tx_3+WPAN_Tx_other*(WLAN_BANDWIDTH-3)+WLAN_Rx_next;

break;

case 2:

sf = WPAN_Tx_2+WPAN_Tx_3+WPAN_Tx_other*(WLAN_BANDWIDTH-2)+WLAN_Rx_middle;

break;

case 3:

sf = WPAN_Tx_3+WPAN_Tx_other*(WLAN_BANDWIDTH-1)+WLAN_Rx_middle;

break;

default:

if(freq_dif<HALF_WLAN_BANDWIDTH){

sf = WPAN_Tx_other*WLAN_BANDWIDTH+WLAN_Rx_middle;

break;

}

else{

sf = WPAN_Tx_other*WLAN_BANDWIDTH+WLAN_Rx_other;

break;

}

}

if (Dest.type == WLAN11 || Dest.type == WLAN55)

sf *=CCK_factor;

return(sf);

}

else { //both src and dest WLAN

switch(abs(Src.frequency-Dest.frequency)) {

case 0:

return(1.0);

case 11:

return(0.5);

case 22:

return(WLAN_Rx_next/11+WLAN_Tx_next);

default:

return(WLAN_Rx_other+WLAN_Tx_other);

}

}

}

}

/*-- Global Function --------------------------------------------------------*/

void CalculateAnalyticalBER(int n, Transmission tlist[])

{// n should be the length of tlist

double EbNo;

for (int dst= 0; dst < n ; dst++) { //for each dest

double Eb, No;

No=0;

for (int src = 0; src < n; src++) { //calculate the power from each source

double pd,sf,pwr;

sf=SpectrumFactor(tlist[src],tlist[dst]);

pd=PowerDistance(tlist[src],tlist[dst]);

pwr=sf*pd;

#if defined(_DEBUG_)

printf("rcpt power from %d to %d = %1.5g * %1.5g = %1.5g\n", src, dst, sf, pd, pwr);

#endif

if (src==dst)

// if src and dest are from the same transmission pair,

// pwr is signal power

Eb=pwr*Tc;

else

// if not from the same transmission pair,

// pwr is interference power

No+=pwr;

}

EbNo=Eb/No; // calculate the EbNo for each dest

//need to calc BER from SNR

double ber0;

switch (tlist[dst].type) {

case WPAN:

ber0 = WPAN_BER(EbNo);

break;

case WLAN11:

ber0 = WLAN_BER_11(EbNo);

break;

case WLAN55:

ber0 = WLAN_BER_55(EbNo);

break;

case WLAN1:

ber0 = WLAN_BER_1(EbNo);

break;

case WLAN2:

ber0 = WLAN_BER_2(EbNo);

break;

default:

printf("Unknown ModulationType");

}

#if defined(_DEBUG_)

printf("EbNo for Transmission %d = %1.5g\n", dst, EbNo);

printf("BER for Transmission %d = %1.5g\n", dst, ber0);

#endif

tlist[dst].BER = ber0;

}

}