IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | SG3a Alt PHY Selection Criteria |  |
| Date Submitted | [28 May 2002] |  |
| Source | [Kai Siwiak] [Time Domain] [7057 Old Madison Pike Huntsville, AL 35806] | Voice: [256-990-9062] Fax: [256-922-0387] E-mail: [kai.siwiak@timedomain.com] |
| Re: |  |  |
| Abstract | [Definitions for the proposal evaluation for Task Group 3a] |  |
| Purpose | [This is a working document that will become the repository for the terms and definitions to be used in the selection process for a Draft Standard for TG3.] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

TABLE OF CONTENTS

1.	Introduction	6

2.	References	6

3.	General Solution Criteria	6

3.1.	Unit Manufacturing Cost (UMC)	6

3.1.1.	Definition	6

3.1.2.	Values	7

3.2.	Signal Robustness	7

3.2.1.	General Definitions	7

3.2.2.	Interference and Susceptibility	8

3.2.3.	Coexistence	14

3.3.	Technical Feasibility	17

3.3.1.	Manufacturability	17

3.3.2.	Time to Market	17

3.3.3.	Regulatory Impact	17

3.4.	Scalability	19

3.4.1.	Definition	19

3.4.2.	Parameters	19

3.5.	Location Awareness	20

3.5.1.	Definition	20

3.5.2.	Values	20

4.	MAC Protocol Supplements	20

4.1.	Required enhancements and modifications of theMAC to  accommodate the Alternative PHY	20

4.1.1.	Definition	20

4.1.2.	Values	20

4.2.	Power Management Types	21

4.2.1.	Definition	21

4.2.2.	Values	21

4.3.	Power Consumption	21

4.3.1.	Definition	21

4.3.2.	Value	22

5.	PHY Layer Criteria	22

5.1.	Size and Form Factor	22

5.1.1.	Definition	22

5.1.2.	Values	22

5.2.	PHY-SAP & PMD-SAP Bit Rate and Throughput	23

5.2.1.	Minimum Receive Bit Rate	23

5.2.2.	PHY-SAP Throughput	23

Simultaneously Operating Piconets	25

5.3.1.	Definition	25

5.3.2.	Values	25

5.4.	Signal Acquisition Timeline	26

5.4.1.	Definition	26

5.4.2.	Values	26

5.5.	Range	27

5.5.1.	Definition	27

5.5.2.	Values	27

5.6.	Sensitivity	28

5.6.1.	Definition	28

5.6.2.	Values	28

5.7.	Multi-Path Immunity	28

5.7.1.	Environment model	28

5.7.2.	Delay Spread Tolerance	28

5.8.	Power Consumption	28

5.8.1.	Definition	28

5.8.2.	Values	29

5.9.	Antenna Practicality	29

5.9.1.	Definition	29

6.	Evaluation Matrix	29

6.1.	General Solution Criteria	29

6.2.	Supplements to MAC Protocol	31

6.3.	PHY Protocol Criteria	31

7.	Pugh Matrix Comparison Value	32

7.1.	General Solution Criteria Comparison Values	32

7.2.	MAC Protocol Criteria	34

7.3.	Phy Protocol Criteria	36

8.	Annex:  Criteria Definition Clarifications based on Committee Work	37

# Introduction

Study Group 3a, the alternate PHY Study Group (SG3a) of IEEE 802.15, will define the criteria for the eventual selection of an alternate PHY Draft Standard from a set of alternate PHY Draft Proposals.  In order to accurately and consistently judge the submitted proposals, technical requirements are needed that reflect the application scenarios that were contributed in response to the call for applications.

This working document will become the repository for the requirements to be used in the selection process for an alternate PHY Draft Standard for SG3a.  The criteria presented in this document are based on document [02/104], which takes precedence, and may also contain more general Marketing Requirements on which the proposers are asked to comment.

The document is divided into four sections: General Solution Criteria, MAC Protocol Supplements Criteria, PHY Layer Criteria and Evaluation Matrix.  The evaluation matrix provides the summary of criteria assessments expected with each proposal.

# References

This recommended practice shall be used in conjunction with the following publications. If the following publications are superseded by an approved revision, the revision shall apply.

[02/104]	IEEE P802.15 Wireless Personal Area Networks 802.15-02/104rxP802-15_SG3a-Technical-Requirements

# General Solution Criteria

This section defines the system level concerns of the solution, both technical and marketing related.

## Unit Manufacturing Cost (UMC)

### Definition

It is important for the cost/complexity to be as small as possible for this type of  device used in the personal area space, see [Section 9: 02/104].  Figure 1 illustrates the logical blocks in the transceiver PHY layer.  Not all blocks are required to implement a communications system.  However, if the functionality is used (even optionally) in the specification, then the cost for implementing the functionality must be included in the estimate.  The order and contents of the blocks may vary in the chain, for example, the frequency spreading may be a part of the modulate/demodulate portion, and the encode/decode operations might split out to ‘source encode/decode’ and ‘channel encode/decode’.

Figure 1.  Logical blocks in the transceiver PHY layer

- Encode/Decode – packet formation including headers, data interleaving, error correction/detection (FEC, CRC, etc), compression/decompression, bias suppression, symbol spreading/de-spreading (e.g. DSSS), data whitening/de-whitening (or scrambling).  (merge two bullets, in figure too)

- Modulate/Demodulate – convert digital data to analog format, can include symbol filtering, frequency conversion, frequency filtering.

- Frequency Spreading/De-spreading – can include techniques to increase or decrease, respectively, the Hz/bit of the analog signal in the channel.

- Transmit/Receive – transition the signal to/from the channel.

### Values

Cost should be specified in US dollar amounts.  It is important to indicate cost as a function of volume or time.  Reasonable and conservative values are important to the present, and will be challenged by participants.  Relative comparisons to existing technology is acceptable.

## Signal Robustness

### General Definitions

An error rate criterion is the maximum bit error rate (BER). Another error rate criterion is the maximum packet error rate (PER) for a specified packet length. The proposer will be asked to indicate both the BER, and the corresponding PER, see Sections 2 and 7 of [02/104] used in the determination of this value when indicating the sensitivity of the proposed device in Section x.  Payload size for the PER test is called out in Section 2 of [02/104] and is intended to be a value between the minimum and maximum packet size. {Editor note, visit 5.3 and then revisit this}

The minimum required sensitivity is the power level of a signal, in dBm, present at the input of the receiver modulated by the proposed method with pseudo-random data for which the error rate criterion is met.  The proposer should include all the calculations used to determine the sensitivity. {Ed comment: need uniform way of collecting the link budget data} The power level should be specified at the antenna to receiver connection (i.e. it should not include any antenna gain).  The error ratio should be determined after any error correction methods required in the proposed device have been applied.  Devices may exceed the minimum required sensitivity, but the measurements in Section 3.2 are taken relative to the minimum value specified in the proposal. {this parag may need to be reviewed later}

The PHY-SAP peer-to-peer throughput of the device is the net amount of data, that is transferred from one PHY SAP to another.  The elapsed time should be at least 1 second.  The connection should already have been established and in progress prior to the 1 second interval.  The units of the throughput are in Mb/s. {This sentence to be reviewed and may be removed later}

Unless otherwise noted, the 802.15.3a transceivers are assumed to use ideal isotropic radiators (i.e. 0 dBi antennas).   =

### Interference and Susceptibility

      - Definition

Interference susceptibility refers to the impact other co-located intentional and unintentional radiators may have on a proposed physical layer solution.  This section is mainly concerned with the interference coming from other non-802.15.SG3a devices.  Although there may be a number of systems radiating RF energy in the environments envisioned for 802.15.SG3a systems, the interference from WLANs (2.4 GHz and 5 GHz), other WPANs, and microwave ovens will be the primary cases considered.

      - Interference Model

The following interferers will be considered:

- Microwave Oven

- IEEE 802.15.1 and Bluetooth

- IEEE 802.11b

- IEEE 802.15.3

- IEEE 802.11a

- Out-of-band interference from intentional or unintentional radiators

Although other wireless systems may be present, the above systems represent a broad representative set of interferers whose impact has been determined to be sufficient for the evaluation of the proposed PHY solutions based upon the IEEE 802.15.SG3a target applications.  Since this document is only concerned with trying to evaluate the capabilities, complexities, and performance implications of proposed physical layers only, it is sufficient to use more generic models of the above systems in order to ease the burden of the proposers who will need to evaluate their system against these interferers.

Towards this end, the following representative models are suggested.

        - Microwave Oven

Model for microwave oven interference is as follows. The microwave oven is transmitting at a power of 100 mW with an active period of 8 ms, followed by a dormant period of 8 ms. That is, during the active period the transmit  power is 100 mW and during the dormant period the transmit power is 0 mW. During the active period, the microwave oven output can be modeled as a continuous wave interferer with a frequency that moves over a few MHz.  At the beginning of the active period, the frequency is 2452 MHz, and a the end of the active period, the frequency is 2458 MHz.  There is a continuous sweep in frequency as the active period progresses in time.

        - Narrowband 2.4 GHz Interferer

This model is intended to represent the impact of Bluetooth or 802.15.1 device.  The following table identifies the parameters of this interferer at the receiving antenna of the proposed 802.15.SG3a system.

| Center frequency | 2.4 GHz |
| --- | --- |
| Baud rate | 1 MHz |
| Modulation | GFSK |
| Tx power | 0 dBm |
| Tx antenna gain | 0 dBi |
| Path loss (1) at 1 meter | 40 dB |
| (2) at 0.3 meters | 29.6 dB |
| Rx power (1) at 1 meter | -40 dBm |
| (2) at 0.3 meters | -29.6 dBm |

        - Wideband 2.4 GHz Interferer

This model is intended to represent the impact of an 802.11b or 802.15.3 device.  The following table identifies the parameters of this interferer at the receiving antenna of the proposed 802.15.SG3a system.

| Center frequency | 2.4 GHz |
| --- | --- |
| Baud rate | 11 MHz |
| Modulation | QPSK |
| Tx power | 20 dBm |
| Tx antenna gain | 0 dBi (handset) |
| Path loss (1) at 1 meter | 40 dB |
| (2) at 0.3 meters | 29.6 dB |
| Rx power (1) at 1 meter | -20 dBm |
| (2) at 0.3 meters | -9.6 dBm |

        - Wideband 5 GHz Interferer

This model is intended to represent the impact of an 802.11a device.  The following table identifies the parameters of this interferer at the receiving antenna of the proposed 802.15.SG3a system.

| Center frequency | 5.3 GHz |
| --- | --- |
| Baud rate | 16.6 MHz |
| Modulation Number of carriers Carrier spacing | 16-QAM OFDM 52 312.5 KHz |
| Tx power | 15 dBm |
| Tx antenna gain | 0 dBi (handset) |
| Path loss (1) at 1 meter | 46.9 dB |
| (2) at 0.3 meters | 36.5 dB |
| Rx power (1) at 1 meter | -31.9 dBm |
| (2) at 0.3 meters | -21.5 dBm |

        - Generic In-band Modulated Interferer

For ultra-wideband based proposals, there may be other wireless systems that may be near-by the 802.15.SG3a system that could cause in-band interference.  In order to understand how much protection the system will provide in this case of an unknown modulated interferer, the following model is proposed for evaluation.

I(t)=2PIcos(ω0t+θ)k=−∞∞gkv(t−kTI−τI)

where PI is the average received power of the interfering waveform, ω0=2πf0 is the carrier frequency of the “narrowband” waveform, θ is a random phase of the carrier uniformly distributed in 0,2π, {gk} are the randomly modulated BPSK symbols where gk∈±1, TI is the symbol period, τI is a random delay uniformly distributed in [0,TI], and v(t) is the baseband waveform shape.  The following table specifies the relevant parameters:

| f0 | Within the bandwidth of the proposal |
| --- | --- |
| 1TI | 5 MHz |
| Modulation | BPSK |
| Baseband waveform | Root Raised Cosine with a roll-off of 0.25 |

        - Generic In-band Tone Interferer

All systems may experience tone interference resulting from close proximity to unintentional radiators like PCs or consumer electronic devices.  .  In order to understand how much protection the system will provide in this case of an unknown modulated interferer, the following model is proposed for evaluation.

I(t)=2PIcos(ω0t+θ)

where PI is the average received power of the interfering waveform, ω0=2πf0 is the carrier frequency of the “narrowband” waveform, and θ is a random phase of the carrier uniformly distributed in 0,2π.  For evaluation, f0 should be chosen to be within the bandwidth of the proposal.

      - Evaluation Method and Minimum Criteria

The following subsections describe how the above models can be used for evaluating the performance impact on the proposal.  Since the performance of these systems may depend on particular receiver designs, and it’s not the intent to standardize certain receiver designs, the proposer should describe any special circuits that were needed to obtain these results (e.g., interference suppression algorithms, notch filters, steep roll-off filters, etc.).  Also, all of the following tests should be done using the nominal system configuration which provides ~110 Mbps.

        - Microwave Oven

When this interferer is present, using simulation results, analysis, or technical explanations, describe the impact on the proposed system performance when operating at 6 dB above the proposed systems receiver sensitivity level.  This impact should either be a reduction in throughput or rise in the BER.

Minimum criteria: Little or no impact at x meter separation distance.

        - Narrowband 2.4 GHz Interferer

When this interferer is present, using simulation results, analysis, or technical explanations, describe the impact on the proposed system performance when operating at 6 dB above the proposed systems receiver sensitivity level.  This impact should either be a reduction in throughput or rise in the BER.

Minimum criteria: Little or no impact at 1 meter separation distance.

Desired criteria: Little or no impact at 0.3 meter separation distance.

        - Wideband 2.4 GHz Interferer

When this interferer is present, using simulation results, analysis, or technical explanations, describe the impact on the proposed system performance when operating at 6 dB above the proposed systems receiver sensitivity level.  This impact should either be a reduction in throughput or rise in the BER.

Minimum criteria: Little or no impact at 1 meter separation distance.

Desired criteria: Little or no impact at 0.3 meter separation distance.

        - Wideband 5 GHz Interferer

When this interferer is present, using simulation results, analysis, or technical explanations, describe the impact on the proposed system performance when operating at 6 dB above the proposed systems receiver sensitivity level.  This impact should either be a reduction in throughput or rise in the BER.

Minimum criteria: Little or no impact at 1 meter separation distance.

Desired criteria: Little or no impact at 0.3 meter separation distance.

        - Generic In-band Modulated Interferer

When this interferer is present, using simulation results, analysis, or technical explanations, determine the average received interference power, PI, that can be tolerated before the BER increases to 10-5 or the throughput is degraded by a factor of 2 when operating at 6 dB above the proposed systems receiver sensitivity level (Pd).  Present results for a number of different center frequencies or describe how the performance changes as the center frequency is changed.

Minimum criteria: PIPd > X dB.

        - Generic In-band Tone Interferer

When this interferer is present, using simulation results, analysis, or technical explanations, determine the average received interference power, PI, that can be tolerated before the BER increases to 10-5 or the throughput is degraded by a factor of 2 when operating at 6 dB above the proposed systems receiver sensitivity level (Pd).  Present results for a number of different center frequencies or describe how the performance changes as the center frequency is changed.

Minimum criteria: PIPd > X dB.

### Coexistence

The proposer should show the level of coexistence with current 802 devices that share the same spectrum within a distance of Y m antenna separation between devices.  (Analysis with 802.11 DSSS shall suffice for 802.11b, 802.11g and 802.15.3).  Coexistence with more than one device at a time is a plus.  The analysis should consider the susceptibility of harmful interference from and to alt-PHY devices. Distance Y is 1 m, however proposers are encouraged to show their performance at shorter distances such as with Y= 30 cm and Y = 8 cm. {Ed. comment – distance Y to be studied further, in view of antenna nearfield issues} {Editor: this text comes from technical requirements doc 02/104}

Coexistence

#### Definition

Coexistence is measured by a reduction in the link budget of an alternate system in the presence of the proposed system with no other interferers or systems present. The physical layout of the network is the same as specified in section 3.2.3.1{Ed. to fix reference}.

Figure 2.  A typical wireless network environment with interfering sources.

#### Values

The value reported shall be the ratios of the net throughput of the following alternate systems in the presence of the proposed system.  The reference node of the proposed system is communicating with a desired node that is located at a distance of 3 m.  Both nodes of the proposed system shall be operating at the nominal transmitting power required for the proposal. All antennas are omnidirectional and 0 dBi. (If the proposer can not achieve these distances then the proposer should indicate what he can do).

- IC1 - Two 802.15.1 devices creating a piconet.  Both devices in the piconet shall be transmitting at 1 mW.  One device participating in the piconet shall be at a distance of 0.3 m, the other at a distance of 13 m.

- IC2 - An 802.15.3 piconet with two devices at 22 Mbps.  Both devices in the piconet shall be transmitting at 1 mW.  One participant of the piconet shall be at a distance of 0.3 m, the other at a distance of 10.3 m.

- IC3 - An 802.11b network with two devices at 11 Mbps.  Both devices shall be transmitting at 100 mW.  One participant shall be at a distance of 0.3 m, the other shall be at a distance of 100 m.

- IC4 - An 802.11a data connection with two devices at 24 Mbps. Both 802.11a devices shall be transmitting at 50 mW.  One device shall be located at a distance of 3 m, the other at a distance of 50 m.

- IC5 -  (802.15.4 to be considered here, text to follow later, to be supplied by Rick)

## Technical Feasibility

This is intended to determine if the proposal is real or academic.  Any proposal may be submitted, but demonstrated feasibility and manufacturability should receive favor over equal but untested proposals.  Proposals will be asked to comment on criteria listed in the following sections.

### Manufacturability

#### Definition

Manufacturability is defined in terms of the use of mature, cost effective manufacturing processes with evidence of effective mass production capability.

#### Values

The proposals are asked to submit proof of the claims by way of expert opinion, models, experiments, pre-existence examples, or demonstrations.  Globally accepted concepts that will be quick to market, with little risk will be favored.

### Time to Market

#### Definition

Time to Market addresses the question of when the proposed technology will be ready for integration.

#### Values

The proposal shall estimate a schedule for when the PHY would be available for integration.

Regulatory Impact

#### Definition

The proposal should specify to which geopolitical regions it applies and identify any applicable requirements with which it conflicts. Merit will be awarded for proposals with regulatory compliance of wider geopolitical scope.

#### Values

The proposer shall state which regions the proposal is in regulatory compliance where merit is awarded for each region of compliance. 
Merit awarded for each category:

- Regions adopting US FCC regulations

- Regions adopting European regulations

- Japanese regulations

- Other National Regulations

Specific conflicts and potential derogations should be detailed

## Scalability

### Definition

Scalability refers to the ability to adjust important parameters, such as those mentioned below, (if they are required by the applications) without rewriting the standard. Scalability should address evolutionary extensions to this proposal.

### Parameters

Parameters of interest include; power consumption, data rate, number of channels, cost, function, interface, range, frequencies of operation (occupied bandwidth of operation)

, and others deemed appropriate by the proposer.

## Location Awareness

### Definition

Location awareness is the ability to determine information about the range and perhaps relative location of one device with respect to another. The purpose is to improve usability of portable devices.  This data can be used to locate, identify and discriminate amongst users in crowded environments and to simplify device registration in constantly changing network topology.  Provisions must be made to propagate location information to a suitable management entity.

### Values

Proposers are to show the functional capability, level of accuracy, resolution (meters), and time (seconds) to compute.

# MAC Protocol Supplements

## Required enhancements and modifications of theMAC to  accommodate the Alternative PHY

### Definition

Supplements and modifications to the MAC may be required to accommodate the proposed alternative PHY.  It is preferred that the supplements be additions which expand the solution capability as opposed to changes in the MAC that represent an alternative way to do a particular function.

### Values

Proposals should justify and explain the supplements that may be necessary in support of additional features for the alternate PHY.

Proposals should justify and explain the modifications that may be necessary to support or enhance operation of the alternate PHY.

## Power Management Types

Power management types are provided by the 15.3 MAC. The proposal should indicate what support is provided for the methods defined in the latest version of the 802.15.3 draft standard.

### Definition

It is important to be able to reduce power consumption for consumer electronic devices.  One method is to use power management and to include protocols that allow methods for sleeping, wakeup, polling, etc.

### Values

The proposals should indicate any limitations to its support of power management as identified in the 802.15.3 MAC.

## Power Consumption

The proposer should evaluate average power consumption for the following states: transmit, receive, and power save.

Definition

Transmit

Power consumption during transmit state is defined as the average power consumed from the PHY-TX-START.request for a given MPDU to the PHY-TX-END.confirm.

Receive

#### Power consumption during receive state is defined as the average power consumed from the PHY-RX-START.request for a given MPDU to the PHY-RX-END.indication.

Power Save

The proposer should provide his values for power save group parameters as specified in 11.7.9 (PHY PIB PS group) in 802.15.3. In addition, the proposer should specify the power consumed from the PHY-PWRMGT.request with the lowest supported power level to the PHY-PWRMGT.confirm of a subsequent PHY-PWRMGT.request with no PS level.

Value

The proposals should estimate the power consumption for the PHY throughputs specified in section 5.2 and for minimum and maximum PHY frame lengths.

PHY Layer Criteria

## Size and Form Factor

### Definition

Size is important for consumer electronic systems such as PDAs and cameras.   The smaller the package, the easier it is to embed.  It is important that the device be compatible with accessory formats as well.  Antennas are not considered in the size requirements.

### Values

Proposers shall provide a time line estimate of when their proposed PHY and the 15.3 MAC will fit into the following form factors:

- PC Card

- Compact Flash

- Memory Stick

- SD Memory

PHY-SAP & PMD-SAP Bit Rate and Throughput

### Minimum Receive Bit Rate

#### Definition

The minimum burst pay load bit rate at the receiver should be provided for the PHY-SAP (after decoding) and PMD-SAP (prior to decoding).

{Editor: The PLCP Sublayer consists of , The pay load rate is

#### Values

The proposer should provide the minimum bit rates for the PHY-SAP and prior to coding. The bit rate at the PHY-SAP should be at least 100 Mbps.

{Editor note: 5.2.1 going offline- Bill to provide new text}

### PHY-SAP Throughput

The proposer should provide all of the following parameters according to clause 2 of [02/104] for each of the proposed rates. In support of these numbers, the equations and values used to derive these times should be provided by the proposer. The number of octets for the MAC Header, HCS, and FCS are specified in the latest 802.15.3 draft. The proposer should provide the description of the proposed PHY Header.

The Short Interframe Spacing (SIFS) is the time between the end of one transmission and the start of the next.  (T_SIFS in Figure 3).  The SIFS must be large enough to support the TxRx turn around time for the proposed PHY, the MAC turnaround and the RxTx turn around time. 
}

Figure 3 Timing parameters for throughput comparison

#### Values

Time values should be in nanoseconds.

### Simultaneously Operating Piconets

### Definition

The proposed PHY should operate in the presence of multiple uncoordinated, close proximity piconets at specific bit and error rates

Values

The proposal should evaluate the effect of simultaneously operating piconets as specified in clause 3 of [02/104] by providing the BER performance (as shown in the figure below)  for the environments specified in document (TBD) {editor note: this document will be derived from channel model selections} over a range of parameters (e.g. coding schemes) relevant to the proposer in accordance with clause 2.0 (at least 110 Mbps and 200 Mbps) of [02/104]  An isotropic antenna should be assumed.

{Editor note: Change “Co-located WPAN” to “Close proximity piconet” in above picture; Bob Huang and Stan Bottoms (Jason Ellis to post on reflector) to supply more text for consideration, this selection criteria is not yet resolved.}

## Signal Acquisition Timeline

### Definition

The signal acquisition methods are the techniques by which the proposed receiver acquires and tracks the incoming signal in order to correctly receive the transmitted data.

### Values

The proposer should indicate the overall acquisition timeline.  A breakdown of the time for the constitute parts of the acquisition preamble would be beneficial.  (TBD based on Applications.)

## Range

### Definition

Based on the 802.15.3a PAR, the proposed system shall be able to initiate a WPAN connection within a 10 meter radius 99.9% of the time.

### Values

Proposals should indicate the range possible with the proposed system.  Provide published references available in the open literature that provide the bases for the environment models.

## Sensitivity

### Definition

Sensitivity was defined in 3.2.1 as part of the Signal Robustness description.  It is important for the proposal to specify the sensitivity level used in the determination of the signal robustness criteria.

### Values

The proposal should indicate the power level at which the error criterion is met.  The proposal should also indicate both the PER, and the corresponding BER used in the determination of this value.

## Multi-Path Immunity

### Environment model

Propose to use “Impulse Response Modeling of Indoor Radio Propagation Channels”, Homayoun Hashemi, IEEE Journal On Selected Areas In Communications, Vol. 11, No. 7, September 1993.

### Delay Spread Tolerance

#### Definition

The delay spread tolerance is the value of TRMS for which the error rate criterion is met with the input signal 3 dB above the minimum required sensitivity using the channel model defined in section 5.12.1.  The system shall have a delay spread tolerance of at least 25ns.

#### Values

TRUE – The proposed system meets the minimum delay spread tolerance

FALSE – The proposed system does not meet the minimum delay spread tolerance

## Power Consumption

### Definition

The power consumption is defined as the total DC power required by the proposed system to operate in either transmit or receive mode.  The power consumption includes all blocks that may be required for the operation of the radio (e.g. voltage regulators, reference oscillators, digital control logic and traditional analog blocks).

### Values

Proposals should indicate the peak and average power in mW necessary to provide the minimum required MAC/PHY throughput.  Values shall be given for both transmit and receive modes for the transceiver used to calculate the unit manufacturing cost figures in section 3.1.

## Antenna Practicality

### Definition

Antenna must be compatible with applications.

# Evaluation Matrix

These matrices are the summarization of the criteria defined in the previous sections.  As proposals are submitted for consideration, these matrices should be completed based on the proposed system parameters.  All proposals should include the general solution criteria matrix.    Comments can be added by the submitter for specified explains and clarity.

Weighting values are included to inform the proposer of the opinion of the IEEE 802.15 Working Group on priority of the indicated criteria.

## General Solution Criteria

| CRITERIA | REF. | WEIGHT | VALUE |  |
| --- | --- | --- | --- | --- |
| Unit Manufacturing Cost | 2.1 |  |  |  |
| Interference and Susceptibility | 2.2.2 |  |  |  |
| Jamming Resistance | 2.2.3 |  | Source 1: Source 2: Source 3: Source 4: Source 5: Source 6: Source 7: |  |
| Multiple Access | 2.2.4 |  | Scenario 1: Scenario 2: Scenario 3: |  |
| Coexistence | 2.2.5 |  | IC1: IC2: IC3: IC4: IC5: IC6: |  |
| Manufacturability | 3.3.1 |  |  |  |
| Time to Market | 3.3.2 |  |  |  |
| Regulatory Impact |  |  | TRUE FALSE |  |
| Maturity of Solution | 3.3.3 |  |  |  |
| Scalability | 3.4 |  | Power consumption: Data rate: Cost: Function: |  |
| Location Awareness | 3.5 |  | Resolution: |  |

## Supplements to MAC Protocol

| CRITERIA | REF. | WEIGHT | VALUE |  |
| --- | --- | --- | --- | --- |
| Required supplements to MAC to accommodate the Alternate PHY | 3.1 |  |  |  |

## PHY Protocol Criteria

| CRITERIA | REF. | WEIGHT | VALUE |  |
| --- | --- | --- | --- | --- |
| Size and Form Factor | 5.1 |  |  |  |
| MAC/PHY Throughput | 4.2 |  | TRUE FALSE |  |
| Frequency Band | 5.2.9 |  |  |  |
| Number of Simultaneously Operating Full-Throughput PANs | 4.4 |  |  |  |
| Signal Acquisition Method | 4.5 |  |  |  |
| Range | 5.10 |  |  |  |
| Sensitivity | 5.11 |  | Burst Rate: PER: BER: |  |
| Delay Spread Tolerance | 5.12.2 |  | TRUE FALSE |  |
| Power Consumption | 5.13 |  |  |  |
| Antenna Practicality | 4.10 |  |  |  |

# Pugh Matrix Comparison Value

To manage the evaluation process of proposals, each proposal will be compared against the following values.  These values were determined through a combination of information from this document and discussions on conference calls.

Each proposal will be compared against the following Pugh Matrix comparison values.  To get a numerical ranking of all proposals, values can be assigned to the different value options.  A “-“ can be given a –1 value, “same” can be given a 0 value and “+” can be given a +1 value.  A relative weight for each category will also be applied to factor in the appropriate overall priority of the various attributes.

## General Solution Criteria Comparison Values

| CRITERIA | REF. | WEIGHT |  | Comparison Values |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | - 1 | 0=Same | + 1 |
| Unit Manufacturing Cost ($) as a function of time (when product delivers) and volume | 3.1 |  | > 2 x equivalent Bluetooth 1 | 1.5-2 x equivalent Bluetooth 1 value as indicated in Note #1 Notes: 1. Bluetooth 1 value is assumed to be $20 in 2H2000. 2. PHY and MAC only proposals use ratios based on this comparison | < 1.5 x equivalent Bluetooth 1 |
| Interference and Susceptibility | 2.2.2 |  | Out of the proposed band: Worse performance than same criteria In band: -: Interference protection is less than 25 dB (excluding co-channel and adjacent channel) | Out of the proposed band: based on Bluetooth 1.0b (section A.4.3) In band: Interference protection is less than 30 dB (excluding co-channel and adjacent and first channel) | Out of the proposed band: Better performance than same criteria In band: Interference protection is less greater than 35 dB (excluding co-channel and adjacent channel) |
| Jamming Resistance | 2.2.3 |  | Any 3 or more sources listed jam | 2 sources jam | No more than 1 sources jams |
| Multiple Access | 2.2.4 |  | No Scenarios work | Handles Scenario 2 | One or more of the other 2 scenarios work |
| Coexistence (Evaluation for each of the 5 sources and the create a total value using the formula shown in note #3) | 2.2.5 |  | Individual Sources: less than 40% (IC = -1) Total: < 3 | Individual Sources: 40% - 60% (IC = 0) Total: 3 | Individual Sources: greater than 60% (IC = 1) Total: > 3 |
| Manufactureability | 3.3.1 |  | Expert opinion, models | Experiments | Pre-existence examples, demo |
| Time to Market | 3.3.2 |  |  | Earliest availability after working group consensus on all relevant issues |  |
| Regulatory Impact |  |  | False | True | N/A |
| Maturity of Solution | 3.3.3 |  | Expert opinion, models | Experiments | Pre-existence examples, demo |
| Scalability (weighting of 5 areas TBD) | 3.4 |  | Scalability in 1 or less than of the 5 areas listed | Scalability in 2 areas of the 5 listed | Scalability in 3 or more of the 5 areas listed |
| Location Awareness | 3.5 |  | N/A | FALSE | TRUE |

Note 3:  Total equation for coexistence value calculation.  Individual comparison values (-, same, +) are represented by the following numbers:  - equals –1, same equals 0, + equals +1.  The individual comparison values will be represented as IC in the equation below, with the subscript representing the source number referenced.

Total = 2 * IC1 + 2 * IC2 + IC3 + IC4 + IC5

## MAC Protocol Criteria

| CRITERIA | REF. | WEIGHT |  | Comparison Values |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | - 1 | 0=Same | + 1 |
| Supplements to MAC to accommodate the Alternative PHY | 3.1 |  | Changes without added capability | No Changes | Added Capability |

## Phy Protocol Criteria

| CRITERIA | REF. | WEIGHT |  | Comparison Values |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | - 1 | 0=Same | + 1 |
| Size and Form Factor | 5.1 |  | Larger | Specific form factor TBD | Smaller |
| Minimum MAC/PHY Throughput | 5.2.34.2 |  | N/A | TBD based on applications | N/A |
| Maximum MAC/PHY Throughput | Error: Reference source not found |  | <Nominal | Nominal 100+ Mbps | >Nominal |
| Frequency Band | 5.2.9 |  | N/A (not supported by PAR) | Unlicensed | N/A (not supported by PAR) |
| Number of Simultaneously Operating Full-Throughput PANs |  |  | < Nominal | TBD based on applications | > Nominal |
| Signal Acquisition Method | 4.5 |  |  | TBD based on applications |  |
| Range | 5.10 |  | < 10 meters | > 10 meters |  |
| Sensitivity | 5.11 |  |  |  |  |
| Delay Spread Tolerance | 5.12.2 |  | < 25 ns | 25 ns - 40 ns | > 40 ns |
| Power Consumption (PHY) | 5.13 |  | > Nominal | Nominal TBD based on applications | < Nominal |

# Annex:  Criteria Definition Clarifications based on Committee Work

As appropriate this section should address any of the defined criteria that warrants clarification and/or redefinition.