IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Unified Power Save |  |
| Date Submitted | 8 July, 2002 |  |
| Source | Bill Shvodian XtremeSpectrum 8133 Leesburg Pike Suite 700 Vienna, VA 22182 | Voice: 703-269-3047 Fax: 703-269-3092 E-mail: bshvodian@REMOVETHISxtremespectrum.com |
| Re: | IEEE Draft P802.15.3/D0.10 |  |
| Abstract | This document comment resolution text for Draft D10 of the 802.15.3 MAC to be incorporated into D11 |  |
| Purpose | The recommendations contained in this document are to be applied to the 802.15.3 MAC baseline, if they are agreed to by the Task Group during the July 2002 Plenary meeting. |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

# Comment #123

This provides resolution to comment 123 from KO/ADH on clause 8.5.1.1

## Clause 8.5.1.1 page 175, line 50

Delete:  The PNC shall then wait for an ACK from the requesting DEV before updating the beacon with the newly assigned isochronous stream CTAs.

New Text:  Once the PNC sends the channel time status command it shall update the beacon with the newly assigned isochronous stream CTAs regardless of whether or not it received an ACK.  If an ACK is not received for the channel time status command the PNC shall retry up to the PNC’s retry limit.  If the requesting DEV does not receive the channel time status command from the PNC within RequestTimeout, the DEV shall retransmit the channel time request command.