tIEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | QoS Policy Proposal for TG3-MAC |  |
| Date Submitted | [October, 2001] |  |
| Edited by | [ Allen D. Heberling ] [ XtremeSpectrum, Inc. ] [8133 Leesburg Pike] [ Vienna, Va. 22182 ] | Voice:[ 703-269-3022 ] Fax: [ ] E-mail: [adheberling@ieee.org] |
| Re: | [] |  |
| Abstract | [This contribution is a draft QoS Policy for the TG3 MAC.] |  |
| Purpose | [To provide the TG3 MAC with a QoS Policy based on principles gleaned from 802.16 and applied to the 802.15.3 WPAN environment and constraints.] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |
| Revision History |  |  |

# References

# Definitions

## Quality of Service

A collective measure of the level of service delivered between DEVs. QoS can be characterized by several basic performance criteria, including availability (low downtime), error performance, response time and

throughput, lost calls or transmissions, connection set-up time, and speed of fault detection and correction.

## QoS Strictness

Three broad categories of QoS,  in a WPAN, are: best effort, differentiated, and guaranteed. The "strictness" of the QoS service describes how tightly the service can be bound by specific channel time allocations, delay, jitter, and loss characteristics.

# Abbreviations and acronyms

CPCS		Common Part Convergence  Sublayer

DME		Device Management Entity

SSCS		Service Specific Convergence Sub layer

### Stream Primitives

The IEEE 802.15.3 MLME supports these primitives at the MLME SAP:

MLME-CREATE-STREAM.request

MLME-CREATE-STREAM.indication

MLME-CREATE-STREAM.response

MLME-CREATE-STREAM.confirm

MLME-MODIFY-STREAM.request

MLME-MODIFY-STREAM.indication

MLME-MODIFY-STREAM.response

MLME-MODIFY-STREAM.confirm

MLME-TERMINATE-STREAM.request

MLME-TERMINATE-STREAM.indication

MLME-TERMINATE-STREAM.response

MLME-TERMINATE-STREAM.confirm

#### MLME-CREATE-STREAM.request (Comment ID # 918, 1425,1426,1429, 1434

This primitive is used to request the creation of a stream.  The semantics of this primitive are:

MLME-CREATE-STREAM.request

(

TargetDEvID,

StreamIndex,

CTR-Control,

CTR-Interval,

CTR-TU,

MinCTR-TUs,

DesiredCTR-TUs,

StreamReqID

)

Parm Name: TargetDEvID;	Data Type: Integer;	Valid Range: 0-255; 	Description: is the DEVID with which the initiating DEV wants to establish a stream.

Parm Name: StreamIndex ;	Data Type: Integer;	Valid Range: 0-255;	Description: In the case of the DEV requesting to establish a stream, the stream index will be either a “0” for  asynchronous traffic or “254” for  an as yet unassigned streamIndex .

Parm Name: CTR-Control	 	Data Type: Octet;		Valid Range: NA;	Description:  contains information about the CTR (xref: 7.5.10.1).

Parm Name: CTR-Interval;	Data Type: Integer; 	Valid Range: 0-32768; 	Description:  is dependent upon the value of the CTR-IntervalType.  When the CTR-IntervalType is TRUE, the CTR-Interval represents the number of superframes per GTS (e.g. 1/1 = 1 GTS /Superframe; 2/1 = 1 GTS every other Superframe; … etc). When the CTR-IntervalType is FALSE, the CTR-Interval represents the number of GTSs per Superframe.

Parm Name: CTR-TU;	Data Type: Duration; 	Valid Range: 0-65536; 	Description:  is the duration of one GTS.  The resolution of this parameter is 1 usec.

Parm Name: MinCTR-TUs;		Data Type: Integer; 	Valid Range: 0-255; 	Description:  is the minimum number of requested GTS Time Units below which the DEV can not support an isochronous stream.

Parm Name: DesiredCTR-TUs;	Data Type: Integer; 	Valid Range: 0-255; 	Description:  is the desired number of  GTS Time Units that a DEV may request for best support of its isochronous stream.

Parm Name: ARQPolicy;		Data Type: Enumeration;	Valid Range: No_ACK, Immediate_ACK, Delayed_ACK, Implied_ACK;	Description:  indicates which ACK Policy the PNC needs to consider when allocating time for  the request.

Parm Name: StreamReqID;	DataType: Integer;	Valid Range: 0-255	Description: is a unique value created by the initiating DME to correlate this primitive with the response primitive it receives from the PNC MLME.

##### When generated

This primitive is generated by an initiating DME to initiate a bi-partite stream negotiation between a DEV and its PNC.  The purpose of which is to establish a new stream between either  DEV-X and its DEV-1/PNC,  DEV-X and   peer DEV-Y, or DEV-1/PNC and DEV-X.

##### Effect of receipt

When a DEV MLME receives this primitive from its DME via the MLME-SAP, it generates a CHANNEL-TIME-REQUEST command, which is sent to its corresponding PNC MLME.  The PNC MLME upon receiving the CHANNEL-TIME-REQUEST command will send an MLME-CREATE-STREAM.indication to its PNC DME via the MLME-SAP.

#### MLME-CREATE-STREAM.indication

This primitive is used to indicate a received  CHANNEL-TIME-REQUEST command.  The semantics of this primitive are:

MLME-CREATE-STREAM.indication

(

TargetDEVID,

RequestorDEVID,

Priority

)

Parm Name:  RequestorDEVID	Data Type: Integer;	Valid Range: 0-255?;	Description: is the DEVID of the DEV requesting a stream.

All other  parameters are defined in the clause entitled:  MLME-CREATE-STREAM.request.

##### When generated

This primitive is sent by the non-initiating PNC MLME to its DME after

- receiving a CHANNEL-TIME-REQUEST command from the initiating DEV MLME

- and determining, in the PNC MLME, that the requested CTA resources are available.

##### Effect of receipt

When the PNC DME receives this primitive it will determine whether:

- the TargetDEvID is in the piconet. If it isn’t, set the ReasonCode = DEV_UNASSOCIATED.

- the TargetDEvID is authenticated.  If it isn’t, set the ReasonCode = DEV_UNAUTHENTICATED.

- the requested priority can be handled.  If it can not be handled,  then set the ReasonCode to PRIORITY_UNSUPPORTED.

before sending an MLME-CREATE-STREAM.response to its  MLME via the MLME-SAP.

#### MLME-CREATE-STREAM.response

This primitive is used to initiate a response to an MLME-CREATE-STREAM.indication. The semantics of this primitive are:

MLME-CREATE-STREAM.response

(

Priority

ReasonCode

)

Parm Name: Priority;	Data Type: Integer;	Valid Range 0-7;		Description: is … ?.

Parm Name: ReasonCode;		Data Type: Enumeration;	ValidRange: DEV_UNASSOCIATED, DEV_UNAUTHENTICATED, and  PRIORITY_UNSUPPORTED; 	Description:  is used to indicate reason for failure.

##### When generated

This primitive is generated by the PNC DME upon receiving an MLME-CREATE-STREAM.indication.

##### Effect of receipt

When the PNC MLME receives an MLME-CREATE-STREAM.response from its DME, it generates a CHANNEL-TIME-STATUS command , which is then sent to the initiating DEV MLME.

#### MLME-CREATE-STREAM.confirm (Comment ID # 1715, 466

This primitive is used to confirm the acceptance or rejection of a request to create a stream. The semantics of this primitive are:

MLME-CREATE-STREAM.confirm

(

StreamReqID,

StreamIndex,

AvailableCTR-TUs,

ReasonCode

)

See the parameters defined for MLME-CREATE-STREAM.response.

##### When generated

This primitive is sent by the initiating MLME to its DME upon receiving either:

- a RESPONSE_TIMEOUT

- a CHANNEL-TIME-STATUS command with a StreamIndex = an UnassignedStream value, or an AvailableCTR-TUs value < minCTR-TUs value, or a ReasonCode = DEV_UNASSOCIATED, DEV_UNAUTHENTICATED,  PRIORITY_UNSUPPORTED, CT_UNAVAILABLE, or REQUEST_DENIED.

- A CHANNEL-TIME-STATUS command with valid data and followed by a BEACON containing the requested CTA.

##### Effect of receipt

The initiating DME, when it receives the MLME-CREATE-STREAM.confirm primitive, is informed whether its stream request was successful or unsuccessful.  The  DME stream request is successful when the TargetDEvID is a valid associated DEVID and authenticated (if authentication is required), the StreamIndex is given an assigned value, the AvailableCTR-TUs value is equal or greater than the minCTR-TUs requested, and the ReasonCode = SUCCESS.  The DME stream request is unsuccessful when either the StreamIndex  is still equal to the UnassignedStream value,  the AvailableCTR-TUs value is less than the minCTR-TUs requested, or the ReasonCode =DEV_UNASSOCIATED, DEV_UNAUTHENTICATED, PRIORITY_UNSUPPORTED, CT_UNAVAILABLE, REQUEST_DENIED, or RESPONSE_TIMEOUT.

#### Create Stream Message Sequence Charts

Figure 1 illustrates the sequence of messages involved in successfully establishing a DEV-2 to DEV-3 stream in a piconet.

Figure 1 Bi-partite MSC for Creating a DEV-2 to DEV-3 Stream

Figure 2 illustrates the sequence of messages involved in successfully establishing a DEV-1/PNC to DEV-2 stream in a piconet.  Please note the MSC convention for DEV-1/PNC.  In 802.15.3 piconets the PNC is always assigned a DEVID value of zero.  Yet an 802.15.3 DEV can support the dual roles of PNC and logical DEV.  Consequently, the  logical DEV is assigned a non-zero DEVID to enable the 802.15.3 MAC to determine which frames are directed to the PNC(DEVID=0) and which frames are directed to the logical DEV whose ID is 0 < DEVID < 252)

Figure 2 MSC for Creating a DEV-1/PNC to DEV-2 Stream

### Stream modification

This mechanism supports the modification of the characteristics of a stream connection between DEVs.

#### MLME-MODIFY-STREAM.request (Comment ID # 920, 141, 1435,

This primitive is used to request a modification to an existing stream.  The semantics of this primitive are:

MLME-MODIFY-STREAM.request

(

TargetDEVID,

StreamIndex,

CTR-Control,

CTR-Interval,

CTR-TU,

MinCTR-TUs,

DesiredCTR-TUs

)

Parm Name: StreamIndex;		Data Type: Integer;	Valid Range: 0-255;	Description: indicates the stream to be modified.

CTR-Control, CTR-Interval, CTR-TU, MinCTR-TUs, DesiredCTR-TUs   are defined in the clause entitled MLME-CREATE-STREAM.request.

##### When generated

This primitive is generated by an initiating DME to request a modification to an existing stream.

##### Effect of receipt

When a DEV MLME receives this primitive from its DME, it will generate a CHANNEL-TIME-REQUEST command, which it will send to the PNC MLME.

The PNC MLME upon receipt of the CHANNEL-TIME-REQUEST command will perform a series of operations according to the steps outlined in 8.6.1.2

#### MLME-MODIFY-STREAM.confirm (Comment ID # 924

This primitive is used to inform the initiating DME whether the requested stream modification was successful or unsuccessful.  The semantics of this primitive are:

MLME-MODIFY-STREAM.confirm

(

StreamIndex,

AvailableCTR-TUs,

ReasonCode

)

The StreamIndex parameter is defined in clause entitled MLME-CREATE-STREAM.request.

The AvailableCTR-TUs parameter is defined in the clause entitled MLME-CREATE-STREAM.response.

##### When generated

The initiating MLME sends this primitive to its DME upon receiving either:

- a RESPONSE_TIMEOUT,

- a CHANNEL-TIME-STATUS command with either

  - the “new” AvailableCTR-TUs = the “previous” AvailableCTR-TUs, or

  - the “previous” AvailableCTR-TUs < the “new”AvailableCTR-TUs <= DesiredCTR-TUs (when requesting an increase in channel time) followed by a BEACON containing the requested CTA modification, or

  - the “new” AvailableCTR-TUs <= DesiredCTR-TUs < the “previous” AvailableCTR-TUs (when requesting a decrease in channel time),  followed by a BEACON containing the requested CTA modification .

##### Effect upon receipt

When the initiating MLME receives a RESPONSE_TIMEOUT it will send an MLME-MODIFY-STREAM.confirm with  ReasonCode =  RESPONSE_TIMEOUT to its  DME.

The initiating DME, when it receives the MLME-MODIFY-STREAM.confirm primitive, is informed whether its  request to modify a stream was successful or unsuccessful.  The  DME stream modification request is successful when the “new” AvailableCTR-TUs value is not equal to the “previous” AvailableCTR-TUs, and the change in AvailableChannelTime is confirmed with a BEACON containing the requested change in channel time allocation. The DME stream modification request is unsuccessful when the AvailableCTR-TUs value is equal to the previous AvailableCTR-TUs or the ReasonCode = RESPONSE_TIMEOUT.

#### Modify Stream Message Sequence

Figure 3 illustrates the message sequence involved when requesting a modification to an existing stream.

Figure 3 MSC for Modifying a Stream

Figure 4  illustrates  the message sequence involved when a requested stream modification for an existing stream is denied.

Figure 4 MSC for a Denied Stream Modification

### Stream termination

These commands are used to terminate a stream connection between DEVs.

#### MLME-TERMINATE-STREAM.request

This primitive is used to request the termination of a specific stream.  The semantics of this primitive are:

MLME-TERMINATE-STREAM.request

(

StreamIndex

)

The StreamIndex parameter specifies the stream to be terminated.

##### When generated

This primitive is generated by the DME to request the termination of an existing stream.

##### Effect of receipt

When a DEV MLME receives this primitive from its DME, it will send a CHANNEL-TIME-REQUEST command (DesiredCTR-TUs=0), to the PNC MLME.  The PNC MLME, upon receiving the CHANNEL-TIME-REQUEST command, will send an ACK to the initiating DEV.

#### MLME-TERMINATE-STREAM.confirm (Comment ID # 467

The primitive informs the initiating DME whether the requested stream termination was successful or not.  The semantics of this primitive are:

MLME-TERMINATE-STREAM.confirm

(

StreamIndex

ReasonCode

)

The StreamIndex parameter indicates the stream  terminated.

The ReasonCode  indicates whether the MLME-TERMINATE-STREAM.request was successful or unsuccessful.  If  successful, the ReasonCode indicates SUCCESS.  If unsuccessful, the ReasonCode indicates an ACK_TIMEOUT, where ACK_TIMEOUT indicates that the requesting DEV has not received an ACK from the PNC before the ACK-TIMEOUT timer expired.

##### When generated

The initiating DEV MLME sends this primitive to its DME after the DEV MLME receives either an ACK to its CHANNEL-TIME-REQUEST command (DesiredCTR-TUs = 0) or its ACK_TIMEOUT has expired.

##### Effect of receipt

The initiating DME, when it receives this primitive, is notified whether its stream termination request was successful or unsuccessful.  If  unsuccessful (ACK_TIMEOUT), the initiating DME will resend an MLME-TERMINATE-STREAM.request to its MLME.  The MLME will then resend a CHANNEL-TIME-REQUEST command (DesiredCTR-TUs = 0) to the PNC.  If successful, the stream is terminated.

#### MLME-TERMINATE-STREAM.indication

This primitive is used to inform the originating DEV, that the channel time allocated for its stream is no longer present in the most recently received BEACON.  The semantics of this primitive are:

MLME-TERMINATE-STREAM.indication

(

StreamIndex

)

##### When generated

This primitive is sent by the stream originating DEV’s  MLME to its DME upon receiving a BEACON which no longer contains the originating DEV’s StreamIndexed CTAs.  The absence of the originating DEV’s CTAs from the BEACON is a consequence of the PNC deallocating the originating DEV’s channel time allocation  after receiving either an  MLME-DISASSOCIATE.request from the target DEV or an ATP-TO indicating that the target DEV is no longer associated with the PNC.

##### Effect of receipt

The DME upon receiving this primitive will terminate the stream associated with the indicated StreamIndex.

#### Terminate Stream Message Sequence Chart

Figure 5 Illustrates the message sequence involved when terminating a DEV-2 to DEV-3 Stream in a piconet.

Figure 5 MSC of Source DEV-2 requesting termination of its stream

Figure 6  illustrates the stream termination process that occurs when the stream between DEV-3(originator) and DEV-2 (target) is terminated as a consequence of DEV-2 disassociating from the piconet.

Figure 6 MSC for a Target DEV-2 Disassociation Causing a Source DEV-3 Stream Termination

Figure 7 illustrates the stream termination process that occurs when the stream between DEV-3(originator) and DEV-2(target) is terminated  by DEV-2.

Figure 7 MSC of Target DEV-2 Requesting Termination of Source DEV-3’s Stream

## MAC sublayer

The MAC  provides stream-oriented service to the Packet Convergence Layer.  This service enables the Packet Convergence Layer to map Source/Target DEVAddresses to Source/Target DEVIDs and  and their associated Channel Time requirements to specific streams identified by stream indices.  This mapping of  Channel Time requirements to a specific stream is a primary attribute of this MAC protocol.  Streams, in this context, provide a mechanism for managing the Channel Time requirements of uplink (DEV->PNC), downlink (PNC->DEV), and peer link (DEV->DEV) streams.

Streams are dynamic in nature, in that they may be created, modified, and deleted.  An established stream may need to be modified due to the type of service assigned to it.  For instance, IP services may require the Channel Time requirements of the stream to be modified due to the burst nature of the service.

## MAC SAP

The MAC  SAP defines the logical interface between the MAC and the Packet Convergence Layer above it.  This logical interface description includes a list of primitives and their definitions.  Although these primitives and their definitions are informative, they provide a context in which to understand the parameters, which need to be passed between the MAC and the Packet Convergence Layer so that each sub layer may fulfill its specified functions.

The IEEE 802.15.3 MAC  supports these Asynchronous and Isochronous primitives at the MAC  SAP:

- Asynchronous primitives:

  - MAC-UNITDATA.request

  - MAC-UNIDATA.indication

- Isochronous primitives:

  - MAC-DATA.request

  - MAC-DATA.indication

### Asynchronous data primitives

#### MAC-UNITDATA.request

This primitive is used to initiate the transfer of a MAC PDU from one MAC entity to another MAC entity or entities. The semantics of this service primitive are:

MAC-UNITDATA.request

(

TargetDEVID,

SourceDEVID,

Priority,

Policy,

TransmissionTimeOut,

Length,

Data,

)

Parm: TargetDEVID	Type: Integer	Range: 0-255	Description: is the DEV ID to which the MAC PDU is to be sent.

Parm:  SourceDEVID 	Type: Integer	Range: 0-255	Descripton: is the DEV ID from which the MAC PDU is sent.

Parm: Priority		Type: Integer	Range: 0 to 7	Description:  Specifies the priority with which the data is to be sent from one MAC entity to another.  See Table xx for interpretation of priority values.

Parm: Policy	Type:  Enumeration	Range:  ACK, NO-ACK	Description: Specifies whether the MAC SDU is to be acknowledged or unacknowledged.

Parm: TransmissionTimeOut;	Type: Duration;	Range: ??;	Description: is the time interval after which a PDU in a transmit queue is purged from the queue.

Parm: Length		Type: Integer	Range: 		Description: Specifies the octet length of the MAC SDU.

Parm: Data		Type: octet	Range: 		Description: Specifies the data portion of the MAC SDU.

##### When generated

This primitive is sent by the Packet Convergence Layer to the MAC SAP after receiving a CL-UNITDATA.request from the LLC sub layer.

##### Effect upon receipt

The MAC upon receiving this primitive uses the received parameters to format an appropriate MAC PDU which is then sent to the PHY-SAP for transfer over the wireless media to a peer MAC entity or entities.

#### MAC-UNITDATA.confirm

This primitive is used to inform the CL of a successful delivery or a failed delivery due to a transmission timeout. The semantics of this primitive are:

MAC-UNIDATA.confirm

(

TargetDEVID,

SourceDEVID,

Priority,

ReasonCode

)

The TargetDEVID, SourceDEVID, and Priority parameter definitions are the same as in 6.8.1.1

Parm: ReasonCode;	Type: Enumeration;	Range: SUCCESS, Tx_TIMEOUT, NO_ACK;	Description: indicates the result of the MAC-UNIDATA.request.

##### When generated

This primitive is generated by the MAC upon either a successful delivery, or an unsuccessful delivery due to either a Tx_TIMEOUT expiration,  or the maximum number of allowed retries has been exceeded without receiving an ACK (assuming the Policy parameter was set to ACK).

##### Effect of receipt

The Packet convergence layer, upon receiving this primitive from the MAC, will send a CL_UNITDATA-STATUS.indication.

#### MAC-UNITDATA.indication

This primitive is used to indicate the reception of a MAC SDU.  The semantics of the service primitive

MAC-UNITDATA.indication

(

TargetDEVID,

SourceDEVID,

Priority,

Length,

Data

)

The TargetDEVID, sourceDEVID, Priority, Length, and Data parameter definitions are the same as in clause 6.8.1.1

##### When generated

This primitive is generated by the MAC upon successfully processing a received MAC PDU.

##### Effect upon receipt

When the Packet Convergence Layer receives this primitive from the MAC it will generate a CL-UNITDATA.indication.

#### An Asynchronous channel time reservation MSC

Figure 8 illustrates the sequence of messages involved in reserving channel time for the exchange of asynchronous data between DEV-2 and  DEV3  in a piconet.

Figure 8 MSC for Reserving Asynchronous Data Channel Time

AsynchTxPDU.req and AsynchTxPDU.cfm are informative internal primitives created to illustrate one method of initiating an Asynchronous CHANNEL-TIME-REQUEST command without sending an MLME-CREATE-STREAM.request primitive from the DME to the MLME.

### Isochronous data primitives

#### MAC-DATA.request

The semantics of this service primitive are:

MAC-DATA.request

(

TargetDEVID,

SourceDEVID,

Priority,

StreamIndex,

Length,

Data

)

The TargetDEVID,  SourceDEVID, Length, and Data  are defined in clause 6.8.1.1.

Parm: StreamIndex	Type: Integer	Range:0 to 255	Description: Specifies the stream over which the data is to be sent.

##### When generated

This primitive is sent by the Packet Convergence Layer to the MAC SAP after receiving a CL-DATA.request from the LLC sub layer and then assigning it an appropriate StreamIndex.

##### Effect upon receipt

The MAC upon receiving this primitive uses the received parameters to format an appropriate MAC PDU which is then sent to the PHY-SAP for transfer over the wireless media to a peer MAC entity or entities.

#### MAC-DATA.confirm

This primitive is used to inform the CL of a successful delivery or a failed delivery.  The semantics of this primitive are:

MAC-DATA.confirm

(

StreamIndex,

ReasonCode

)

StreamIndex is defined in clause 6.8.2.1

Parm: ReasonCode;	Type: Enumeration;	Range: SUCCESS, NO_ACK;	Description:  indicates the result of the MAC-DATA.request.

#### MAC-DATA.indication

This primitive is used to indicate the reception of a MAC SDU

##### Semantics of the service primitive

MAC-DATA.indication

(

TargetDEVID,

SourceDEVID,

Priority,

StreamIndex,

Length,

Data

)

The TargetDEVID, SourceDEVID, Length, and Data parameter definitions are the same as in clause 6.8.1.1. The StreamIndex defintion is the same as in clause 6.8.2.1

##### When generated

This primitive is generated by the MAC upon successfully processing a received MAC PDU.

##### Effect upon receipt

When the Packet Convergence Layer receives this primitive from the MAC it will generate a CL-DATA.indication.

# MAC frame formats

#### Channel time request command (Comment ID #  359,361,725,726,751,837, 839,840, 1094,1098, 1116,1118,1339,1340, 1121,1533,1716)

Text for the first paragraph of this clause is in document 02/100r4

Figure 9 CHANNEL-TIME-REQUEST command frame format

TargetDEVID is the DEV with which the stream originating DEV is requesting channel time from the PNC.

Stream Index field is defined in  clause 7.2.4.  with this additional caveat: a Stream Index field  initialized with the “UnassignedStream” value by a stream originating DEV is used to indicate to the PNC that this is a new stream.

Stream Request ID is defined in 7.5.10.3

“The SPS Set Index is used to identify the SPS set that the CTR is associated with if the CTR is an SPS CTR.”(excerpt from doc:02/100r4)

The CTR Control field shall be formatted as illustrated in Figure 10

Figure 10 CTR Control field

Excerpt from document 02/100r4:

“…The CTR Control field contains information about the CTR.

The 3-bit Priority field in Figure 10 is defined in A.3.

The Stream Type field indicates whether the CTR is for an Asynchronous or Isochronous stream.

The CTR-SPS Type field indicates if the CTR is an Active CTR or an SPS CTR.

The GTS Type field  indicates if the request is for dynamic or pseudostatic GTS

The CTR Interval Type field indicates whether the CTR Interval field represents the number of slots per superframe, or the number of superframes per slot.  If the CTR Interval Type is zero, the CTR interval is the requested number of slots per superframe.  This allows an extremely latency sensitive application to request multiple slots per superframe.  If the CTR Interval Type is one, then the CTR Interval is the number of superframes between slots, allowing a DEV to request a slot every n superframes.  Regardless of the CTR Interval Type field, if the CTR Interval field is zero, the DEV is requesting its fair share of as much channel time as is available.  This can be used for types of asynchronous traffic that can use as much bandwidth as is available.

The CTR Time Unit field indicates the unit of time that the DEV is using for its request.  This allows the PNC to know what units of GTS time the DEV can make use of so that the PNC can efficiently allocate GTS time that is more than the minimum needed by the DEV, but less than the desired.   It also enables the PNC to fragment a slot if necessary.   The resolution of this field is 1 µs and therefore has a range of [0-65535] µs.

The Minimum CTR Time Units field indicates the minimum number of CTR Time Units that are required per slot.

The Desired CTR Time Units field indicates the desired number of CTR Time Units per slot.  Desired CTR Time Units shall be greater than or equal to Minimum CTR Time Units. …  “

#### Channel time status command

The format of a CHANNEL-TIME-STATUS command shall be formatted as illustrated in Figure 11.

Figure 11 CHANNEL-TIME-STATUS command frame format

The Stream Request ID is defined in clause 7.5.10.3.

The Stream Index is defined in clause 7.2.4.

The AvailableCTR-TUs  is used by the PNC to indicate to the requesting DEV the CTR-TUs it has assigned to the requested stream.  If the AvailableCTR-TUs is greater than or equal to the MinimumCTR-TUs requested or less than or equal to the DesiredCTR-TUs requested  the requesting DEV is informed that there is channel time available.  If, however, the AvailableCTR-TUs are less than the MinimumCTR-TUs requested, the requesting DEV is informed that the PNC is unable to fulfill the DEV’s request for channel time.

Reason Code is a an octet field that is valid when a channel time request is being granted, rejected or terminated.  Valid reason codes are:

  - 0 -> SUCCESS

  - 1 -> DEV_UNASSOCIATED

  - 2 -> DEV_UNAUTHENTICATED

  - 3 –> PRIORITY_UNSUPPORTED

  - 4 –> STREAM_TERMINATED

  - 5 - 255 -> Unassigned

# MAC functional description

## Channel time management

Channel time management in an 802.15.3 piconet involves:

- the creation, modification, and termination of isochronous data streams between two or more DEVs in the piconet.

- the reserving of asynchronous channel time for the exchange of asynchronous data between two or more DEVs in the piconet.

Each DEV shall support at least one isochronous stream.  A DEV may support more than one stream depending upon the application for which it is designed to support.  A PNC needs to support as many isochronous streams as it desires to source and sink.

### Isochronous stream management  (Comment ID # 69,377,378, 379, 1144,1146,1563,

Creating, modifying, and terminating isochronous streams between  two or more DEVs in a piconet shall be initiated via a bi-partite communication/negotiation between the requesting DEV and the PNC. Once a stream index and channel time allocation are established, an isochronous stream may be sent peer to peer, modified, or terminated.

Only a DEV shall send a CHANNEL-TIME-REQUEST command to the PNC.

The values for  stream type, GTS Type, and CTR Interval Type (xref: 7.5.10.1) shall be non-negotiable and are decided by the DEV that is sending the channel time request. These values shall not be changed anytime after the first transmission of the command frame containing the request for that stream.

The channel time requirements of the stream shall be negotiated between the stream originating DEV and the PNC using the CHANNEL-TIME-REQUEST and CHANNEL-TIME-STATUS commands.

#### Isochronous stream creation

In the case where the stream originating DEV desires to establish an isochronous stream with a target DEV, the stream originating DEV shall send a CHANNEL-TIME-REQUEST command (xref: 7.5.10.1) to the PNC with these parameter values:

- Target DEVID field is set to the DEVID with which the originating DEV desires to establish a stream.

- Stream Index field is set to the “UnassignedStream” value  (xref: 7.5.10.1 )

- Stream  Request ID field is set to a unique value between 1 and 255 for the duration of the negotiation.

- Priority field is set to a value between 0b011 and 0b110 as defined in A.3

- Stream Type field is set to the bit value indicating an isochronous stream.

- All the other CHANNEL-TIME-REQUEST command parameters are set to appropriate values as defined in 7.5.10.1

The PNC upon receiving the CHANNEL-TIME-REQUEST command from the stream originating DEV shall respond with a CHANNEL-TIME-STATUS command (xref: 7.5.10.2 to the requesting DEV with these CHANNEL-TIME-STATUS command parameters values (if the requested Channel Time is available):

- Stream Index field is set to a value greater than 0 and less than 254 to indicate that the isochronous stream has been allocated channel time.

- AvailableCTR-TUs field is set to a value greater than or equal to the MinimumCTR-TUs and less than or equal to the DesiredCTR-TUs requested.

- Reason Code field is set to SUCCESS

The PNC shall then wait for an ACK from the requesting DEV before updating the  BEACON with the newly assigned isochronous stream CTAs .

If, however,  the requested Channel Time is not available, the PNC shall respond to the requesting DEV with these parameter values:

- Stream Index field set to the UnassignedStream value

- AvailableCTR-TUs field set to a value less than the MinimumCTR-TUs requested.

- Reason Code field set to the REQUEST_DENIED value.

The requesting DEV upon receiving this CHANNEL-TIME-STATUS command and indicated parameters may accept its denied request as final,  it may resend its original request, or it may modify its original request with new parameters.

If the Target DEVID is not available (its unassociated, and/or unauthenticated), the PNC shall respond to the requesting DEV with these parameter values:

- Stream Index field set to the UnassignedStream value.

- Available CTR TUs field set to a value greater than or equal to the MinimumCTR-TUs and less than or equal to the DesiredCTR-TUs requested.

- Reason Code field set to the either the DEV_UNASSOCIATED, or DEV_UNAUTHENTICATED value.

See Figure 1 and Figure 2 in clause 6.3.13.5 for the message sequence chart illustrating the described process.

#### Isochronous Stream modification

Only a stream originating DEV may  modify an established stream by requesting the PNC to either  increase or decrease the channel time allocated to one of its streams.  The stream originating DEV desiring to modify the channel time allocated to one of its streams shall send the PNC a CHANNEL-TIME-REQUEST command with these parameter values:

- TargetDEVID field is set to the DEVID with which the originating DEV has an established stream.

- StreamIndex field set to the value of the stream to be modifed

- MinimumCTR-TUs field is set to its original values

- DesiredCTR-TUs field is set to a value greater than or equal to the MinimumCTR-TUs or greater than the current AvailableCTR-TUs.

The PNC upon receiving the CHANNEL-TIME-REQUEST command shall check to see if the requested resources are available.  If the requested channel time is not available, the PNC shall:

- Send a CHANNEL-TIME-STATUS command with the AvailableCTR-TUs  = to the previously assigned AvailableCTR-TUs,

- await an ACK from the requesting DEV, and

- make no modification to the existing BEACON CTAs for the DEV requesting the modification.

If the requested channel time is available, the PNC shall:

- reserve the requested channel time,

- send a CHANNEL-TIME-STATUS command with either

  - the “previous” AvailableCTR-TUs < = “new” AvailableCTR-TUs < = DesiredCTR-TUs  (when requesting an increase in channel time), or

  - the “new” AvailableCTR-TUs < = DesiredCTR-TUs ( when requesting a decrease in channel time).

- await an ACK from the requesting DEV, and

- build a new BEACON with the requested channel time.

See Figure 3 in clause 6.3.14.3

#### Isochronous Stream termination

Only the PNC, the stream originating DEV, or the target DEV may terminate an established stream.  In the case where either the originating DEV or the target DEV  desires to terminate a specific stream, it shall send the PNC the CHANNEL-TIME-REQUEST command with these parameter values:

- TargetDEVID field is set to the DEVID of the DEV to which the originating DEV is sending isochronous data via an established stream.

- StreamIndex field shall be set to the value of the stream to be terminated

- The CTR-TU, MinimumCTR-TUs and DesiredCTR-TUs fields shall be set to zero.

The PNC, upon receiving the CHANNEL-TIME-REQUEST command from a DEV requesting stream termination, shall respond with an ACK followed by a BEACON in which the CTAs assigned to the indicated StreamIndex are set to a null CTA value for aBeaconInfoElementRepeat duration.

In the case where a stream originating DEV-1/PNC desires to terminate the stream, it shall notify the target DEV  via  the next periodic  BEACON that the CTAs assigned to the indicated StreamIndex are set to a null CTA value for aBeaconInfoElementRepeat duration.  See Figure 7 in clause 6.3.xxx

### Asynchronous channel time reservation

When a DEV desires to reserve channel time for asynchronous data transmissions with a target DEV or DEVs, the originating DEV shall send a CHANNEL-TIME-REQUEST command (xref: 7.5.10.1) to the PNC with these parameter values:

- Target DEVID field shall be set to the DEVID with which the originating DEV desires to reserve channel time.

- Stream Index field shall be set to zero.

- Priority field shall be set to a value between 0b000 and 0b001 as defined in A.3

- Stream Type field shall be set to the bit value  indicating an asynchronous stream.

- All the other CHANNEL-TIME-REQUEST command parameters are set to appropriate values as defined in 7.5.10.1

The PNC upon receiving the CHANNEL-TIME-REQUEST command from the originating DEV shall respond with a mandatory ACK to the requesting DEV.  If the requested Channel Time is available the PNC shall respond to the requesting DEV with a BEACON containing CTAs with the source and target DEVID fields appropriately set.

The originating DEV’s request for channel time is guaranteed to be scheduled.  However, there is no guarantee for how long the delay will be between the time of the request and the reception of a BEACON containing the requested channel time allocation.  It is the responsibility of the requesting DEV to maintain an “aging” function for managing its internal transmit queues.  If a frame’s TransmissionTimeout interval expires while waiting for its requested CTA in the BEACON, a MAC-UNTIDATA.confirm shall be sent with the ReasonCode set to Tx_TIMEOUT.

# Packet Convergence layer

## Overview: generic packet convergence layer

The Packet Convergence layer (PCL),detailed in Figure 12, may contain one or more Packet based  Service Specific Convergence Sublayers ( SSCS)(i.e. 802.2, 1394, USB, …) .

The Packet Convergence Layer provides these functions:

- receiving PDUs from upper protocol layers via the SSCS-SAP

- classifying PDUs received from upper protocol layers according to a classification rule set.

- delivering each Packet based PDU to the MAC SAP

- receiving Packet based PDUs from  peer Packet Convergence layers via the MAC-SAP.

Figure 12 802.15.3 Packet based Convergence Layer Model

Figure 13 illustrates the relationship between the Packet Convergence Layer and the rest of the  802.15.3 protocol  entities.

Figure 13 802.15.3 Protocol Layer Model

## IEEE 802.2 Logical Link Control Service Specific Convergence Sublayer

The IEEE 802.2 Logical Link Control (LLC) Service Specific Convergence Sublayer ( SSCS), shall:

- receive upper layer 802.2 LLC Packet PDUs via the SSCS-SAP,

- classify each received PDU according to these attributes:

- Destination MAC address, a 48-bit IEEE MAC Destination Address.

- Source MAC address, a 48-bit IEEE MAC Source Address.

- Priority, an 802.1D  hierarchical QoS scheme.

- map each received PDU to a specific StreamIndex according to the rules of the SSCS classifier

- map each received PDU 48-bit source and destination address to a corresponding  8-bit, 802.15.3 source and destination DEVID.

- deliver each valid Packet SDU to the MAC-SAP

- Receive Packet SDUs from the MAC-SAP

### Classification

The Packet SSCS PDU classification process maps each Packet SSCS PDU to a specific Stream-ID.  Each Stream-ID has associated with it a set of QoS characteristics.  Consequently, after classification, each Packet SSCS PDU will be delivered using the QoS parameters specified for the Stream-ID.

The classification process uses one or more Classification parameters sets to analyze each packet entering the 802.2 LLC SSCS.  Each set includes a classification priority, StreamIndex, and protocol specific parameters such as destination MAC address, source MAC address, and optionally a priority parameter.  If a Packet SSCS PDU, received from an upper layer protocol, matches the specified protocol specific parameters, it is then sent to the MAC-SAP.  If the Packet SSCS PDU does not match the specified protocol parameters, the packet may be delivered using either a default Stream-ID or the packet may be discarded.  The policy for deciding how to handle a packet in this instance is outside the scope of this standard.   Figure 14 provides a graphical representation of the conceptual entities involved.

In the case where more than one classification parameters set is available, the classification process first shall use the classification parameters set containing the highest valued classification priority parameter.  If no match is found with the first classification parameters set, the next highest priority classification parameters set will be applied.  This process will repeat itself until either the incoming packet is properly matched and assigned to a specific Stream-ID for subsequent delivery, or there are no more classification parameters sets available and the incoming packet is either discarded or delivered with a default delivery QoS.

Figure 14 Classifications and StreamIndex Mapping

### IEEE 802.2  Quality of Service (QoS)

The 802.2 LLC SSCS shall support one or both of these QoS schemes:

- Best Effort

This is the default QoS  that be shall be supported.  All 802.2 PDUs are handled the same (i.e. no QoS guarantees are provided regarding delivery of the received PDU).

- Hierarchical IEEE 802.1D QoS priority scheme

The 802.1D priority scheme, which describes up to eight  (0-7) different QoS levels, may be included in the rule set for the 802.2 LLC SSCS classifier. Each of  the 8 different QoS priorities are described in Table 1 IEEE 802.1.

Table 1 IEEE 802.1D QoS priority levels

| User Priority | Traffic Type | Used for: | Comments |
| --- | --- | --- | --- |
| 0 (Default) | Best Effort (BE) | Asynchronous data. | Default WPAN traffic |
| 1 | Background (BK) | Asynchronous |  |
| 2 | - | A spare. | Currently, not assigned. |
| 3 | Excellent Effort (EE) | Isochronous | For valued customers |
| 4 | Controlled Load (CL) | Isochronous | Traffic will have to conform to some higher protocol layer admission control. |
| 5 | Video (VI) | Isochronous | < 100 ms delay and jitter |
| 6 | Voice (VO) | Isochronous | < 10 ms delay and jitter |
| 7 | Network Control (NC) |  |  |

## SSCS-SAP

## Annex C

Bibliography

[Cx1] G. Xylomenos and G.C. Polyzos, “Link Layer Support for Quality of Service on Wireless Internet Links,” IEEE Personal Communications, vol. 6, no. 5, Oct. 1999, pp. 52-60.

[Cx2] IEEE Draft 802.16/D4-2001. IEEE Draft Standard for Local and Metropolitan Area Networks – Part 16: Standard Air Interface for Fixed Broadband Wireless Access Systems.

[Cx3]  QoS Forum,  “White Paper – QoS protocols and architectures,”  Stardust.com, Inc., Jul. 1999

[Cx4] QoS Forum, “White Paper – Introduction to QoS Policies,”  Startdust.com, Inc., Jul. 1999

[Cx5] QoS Forum, “Quality of Service – Glossary of Terms,” Stardust.com, Inc., May 1999