IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Comments and Change Requests for TG2 Draft D04 |  |
| Date Submitted | [10 March, 2002] |  |
| Source | [YC Maa et al.HK Chen et. al.] [InProCommIPC] [refer to whitepages] | Voice: [ ] Fax: [ ] E-mail: [ ] |
| Re: | [01000D04P802-15-2__Draft_Recommended_Practice] |  |
| Abstract | [This contribution is provides comments and change requests to 802.15 TG2 draft D04.] |  |
| Purpose | [Provide comments and suggested changes/corrections in the draft text.] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

IEEE P802.15
Wireless PANs

Comments and Change Requests for TG2 Draft D04

Date:	10 March, 2002

Author:	YC Maa, HK Chen and KC Chen,
	Integrated Programmable Communications, Inc.
	Taiwan Lab: PO Box 24-226, Hsinchu 300, Taiwan
	+886 3 5165106
	{ ycmaa, hkchen, kc}@inprocomm.com

# 1. Comment #1

The missing texts on subclause 4.1 description of the interference problem, clause 4.2 Overview of coexistence model and 4.3 Overview of coexistence mechanisms have not been formally reviewed, voted and accepted to be included into the draft.

Suggested Remedy: allocate time slots to have these subclauses presented for formal approval of inclusion into the draft text.

# 2. Change Request #1

Clause 4.3.1 Non-collaborative Proposal 1) Adaptive Frequency Hopping (AFH): Replace the last sentence, “This is done by first removing the interfered channels from the hopping sequence and secondly intelligently using the remaining bad channels (if any).” with the following:  “From traffic type and channel condition, a partition sequence is generated as input to the frequency re-mapper, which modifies hopping frequencies to avoid or minimize interference effects.” to be consistent with subclause 12.2 on AFH mechanism.

# 3. Change Request #2

Change related texts from PLR to PER terminology change:

- Clause 12.1.1.2 Paragraph 3, sentence 3: Replace “A packet loss is declared …” with “A packet error is declared …”.

- Clause 11.2.2  Paragraph 1, sentence 3: Replace “A collision results in a packet loss for both packets.” with “A collision results in a packet error for each packet.”.

- Figure 46 & 48, in the subtitle/caption, Replace all “packet loss” occurrences with “packet error”.

- Replace all “packet(s) be lost” usages by “packet(s) be in-error”, in

  - Clause11.2.1, paragraph 3, sentence 2;

  - Clause 12.1.1.1, paragraph 2, sentence 1;

  - Clause 12.1.1.2, paragraph 1, sentence 2, 3

  - Clause 12.1.1.2 paragraph 3, sentence 3;

  - Clause 12.1.2.4, paragraph 2, sentence 2,

# 4. Change Request #3

Clause 12.1.2.5 Classification during connection state: Replace the title, “Classification during connection state” with “Online classification” to be concise and parallel in style with the next subclause 12.1.2.6 title for Offline classification.

# 5. Change Request #4

Clause 12.1.2.3 Integrating slaves’ classification data: Renumber subclause 12.1.2.3 as subclause 12.1.2.2.1 since “Integrating slaves’ classification data” is a special case of “Master’s classification”. Renumber the subsequent 12.1.2 subclauses accordingly.

# 6. Change Request #5

Add the following paragraph as subclause 12.1.1.4. The text was newly added as part of the channel classification draft approved in the Jan02 Dallas Interim meeting, but somehow it was not included in D04.

### 12.1.1.4 Transmission Packet Acknowledgement

Packet acknowledgement uses acknowledgement to identify whether a channel is good or bad. If an acknowledgement (ACK) is received, the channel is deemed “good”. If a NACK is received, the channel is deemed “bad” channel. Time-based averaging should be used over a period of time to collect the channel classification result.

# 7. Change Request #6

In Table 21 for IEEE 802.15.1 ACL packet types, exchange the payload length entries of DH5 (0-224) and DM5 (0-339), for currently the values are just on the opposite position of each other.

# 8. Change Request #7

In clause 12.2 Adaptive frequency hopping mechanism, replace the “selection box” text of the diagram with “legacy hop kernel”, since the legacy hop kernel in the text and the selection box in the diagram should be harmonized.

# 9. Change Request #8

In clause 12.2 Adaptive frequency hopping mechanism, the caption for the AFH block diagram (Figure 49) is wrong. Replace the “when NG <= Nmin, …” with “when NG >= Nmin, …” in the caption.

# 10. Change Request #9

In clause 12.2.1 Channel selection. Replace the “NBK = min(0, Nmin - NG )” with “NBK = max(0, Nmin - NG )” in the text.

# 11. Change Request #10

In clause 12.2.1 Channel selection, paragraph 2. Replace the “SBK” with “SBK” in the text.

# 12. Change Request #11

In clause 12.3.4 AFH start, paragraph 2, sentence 1. Replace the “LMP_AFH start” with “LMP_AFH_start” in the text.

# 13. Change Request #12

Replace the “Table xx—” occurrences in the text with “Table xx” in the text:

xx  = 1-9, 11-17, 20, 22-25.

# 14. Change Request #13

Replace the “Figure xx—” occurrences in the text with “Figure xx” in the text:

xx  = 1-42, 44-55.

# 15. Change Request #14

Some tables, “Table xx”, are not cited in the text: Add corresponding citations for these tables.

xx  = 10, 18, 19, 21, 26-32.

# 16. Change Request #15

Some tables, “Figure xx”, are not cited in the text: Add corresponding citations for these figures.

xx  = 43, 56-60. .

# 17. Change Request #16

The cover page seems messed up, some texts on top of the others. (Same cases for D02, D03, D04).