IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | 1 Proposal |  |
| Date Submitted | [March 13, 2002] |  |
| Source | [Daniel V. Bailey and Ari Singer] [NTRU] [5 Burlington Woods Burlington, MA 01803 USA] | Voice: [+1 781 418-2522] Fax: [+1 781 418-2532] E-mail: [dbailey@ntru.com] |
| Re: | 802.15.3 TG3 Letter Ballot Draft D09, 02074r1P802.15_TG3-Security-CFP.doc |  |
| Abstract | [This document instantiates both mandatory and optional to implement security protocols found in 02130r1P802-15_TG3-NTRU-Security-Architecture-Proposal. The mandatory to implement algorithms are as proposed in 02131r0P802-15_TG3-NTRU-Security-Algorithm-Suite-Proposal and an optional to implement algorithm suite is specified using ECC MQV key agreement, SHA-256 and AES-128. The trusted root keys for verifying certificates are established by the DME. The binding between the ID and the public key are verified by the MLME.] |  |
| Purpose | [This document is intended as a proposal for algorithm suite text for inclusion in the 802.15.3 draft standard. The text from this submission may be incorporated directly into the draft standard and is offered as a set of algorithm suites for inclusion in 802.15.3.] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Table of Contents

1	Introduction	2

1.1	Scope	2

1.2	Purpose	3

1.3	Document Organization	3

1.4	Notes to the Reader	3

2	References	4

3	Mandatory Algorithm Suite Specification	5

3.1	Object Identifier	5

3.2	Security Functionality Provided	5

3.3	Data Formats	6

3.4	Cryptographic Operation Selections	7

4	Optional Algorithm Suite Specification (MQV-Implicit Certificates)	11

4.1	Object Identifier	11

4.2	Security Functionality Provided	12

4.3	Data Formats	12

4.4	Cryptographic Operation Selections	14

5	Security Analysis	19

5.1	Protocol Security Analysis	19

5.1.1	Comparison with TLS	20

5.1.2	Protection Against Attacks	22

6	Efficiency Analysis	27

6.1	Frame Protection	27

6.2	Authentication	27

6.3	Access Control List Storage	28

6.4	Secure Group State Storage	29

# Introduction

02130r1P802-15_TG3-NTRU-Security-Architecture-Proposal supplies a security architecture designed to accommodate a variety of cryptographic algorithms and trust establishment methods. To achieve this versatility, the security architecture is necessarily underspecified. This document instantiates all underspecified items to build a complete security solution using the proposed architecture and multiple different algorithm selections.

This document provides standards text for inclusion of these algorithm suites in the 802.15.3 draft standard.  Additionally, this document contains security analysis and supporting documentation for this proposal and for the security architecture proposed in 02130r1P802-15_TG3-NTRU-Security-Architecture-Proposal.

## Scope

This document covers text including NTRU’s proposed algorithm suite as the mandatory to implement algorithm suite and a proposed security suite using ECC technologies as an optional to implement security suite for the 802.15.3 draft standard.  In particular, this includes text to completely instantiate the security architecture proposed in 02130r1P802-15_TG3-NTRU-Security-Architecture-Proposal as well as additional informative text and security considerations.

## Purpose

This document is intended as an algorithm suite submission to the 802.15 TG3 for inclusion in the 802.15.3 draft standard.  The text from this submission may be incorporated directly into the draft standard and is offered as a proposal for the security suite vote announced in document 02074r1.

## Document Organization

This document contains text for proposed mandatory and optional algorithm suites for the 802.15.3 draft standard.  In addition, this submission includes informative text that may or may not be included in the draft standard to support the architecture.

The document is organized into the following categories:

- References (Normative)

- Instantiation of the algorithm suites (Normative)

- Security considerations for an informative annex (Informative)

- Efficiency considerations for working group consideration (Informative)

The reference section describes the external documents that are required in order to implement the cryptographic algorithms proposed in this document.

The instantiation section provides text and information for the aspects of the algorithm suites that must be defined in order to fully implement the security architecture.  This text may be added to the main section of the document or to annexes.

The security considerations provide security analysis and rationale for the security architecture and algorithm suite.  This section is informative and may be included in an informative annex in the standard and/or used as a basis for discussions in the working group.

The efficiency considerations are provided to enable the working group to properly assess the impact on the draft standard by the acceptance of the proposed algorithms.

## Notes to the Reader

Throughout the document, the author has included notes to the reader that are not part of the proposed text or submission.  These notes are supplied to aid in the review process of this document and the consideration for inclusion in the standard.  These notes are not intended to be a part of the standard itself.

Author’s note: Notes are underlined, indented and written in this font to indicate that they are not part of the intended draft text.

# References

[TLS] Dierks, T. and C. Allen, "The TLS Protocol, Version 1.0," RFC 2246, January 19991.

[EESS#1] EESS #1: Implementation Aspects of NTRUEncrypt and NTRUSign2.

[ISO/IEC8824] ISO/IEC 8824: Information technology -- Open Systems Interconnection -- Specification of Abstract Syntax Notation One (ASN.1)3.

[ISO/IEC9797] ISO/IEC 9797: Information technology – Security techniques – Data integrity mechanism using a cryptographic check function employing a block cipher algorithm3.

[FIPS46-3] NIST FIPS PUB 46-3, “Data Encryption Standard,” Federal Information Processing Standards Publication 46-34.

[FIPS180-1] NIST FIPS PUB 180-1, “Secure Hash Standard,” Federal Information Processing Standards Publication 180-14.

[FIPS180-2] NIST FIPS Pub 180-2, Draft Specification for the Secure Hash Standard, Federal Information Processing Standards Publication 180-2, US Department of Commerce/N.I.S.T., draft, 20014.

[AES] FIPS Pub ZZZ, Announcing the Advanced Encryption Standard (AES), Federal Information Processing Standards Publication ZZZ, US Department of Commerce/N.I.S.T., draft, 20014.

[HMAC] FIPS Pub #HMAC, The Keyed-Hash Message Authentication Code (HMAC), Federal Information Processing Standards Publication #HMAC, US Department of Commerce/N.I.S.T., draft, 20014.

[SEC4] Standards for Efficient Cryptography, SEC 4: Implicit Certificates, working draft, Version 0.2, Certicom Research, November 14, 20005.

[X9.63] ANSI X9.63-2001, Public Key Cryptography for the Financial Services Industry, Key Agreement and Key Transport Using Elliptic Curve Cryptography, American Bankers Association, January 10, 2002.

[MODES] Recommendation for Block Cipher Modes of Operation – Methods and Techniques, NIST Special Publication 800-38A, 2001 Edition, NIST4.

__________

1IETF RFCs are available from the Internet Engineering Task Force (http://www.ietf.org)

2EESS publications are available from the Consortium for Efficient Embedded Security (http://www.ceesstandards.org)

3ISO/IEC publications are available from the ISO Central Secretariat, Case Postale 56, 1 rue de Varembe, CH-1211, Geneve 20, Switzerland/Suisse (http://www.iso.ch).  Electronic copies are available in the United States from the American National Standards Institute, 11 West 42nd Street, 13th Floor, New York, NY 10036 (http://www.ansi.org)

4NIST FIPS publications are available from the National Institute for Standards and Technology, 100 Bureau Drive, Stop 8900, Gaithersburg, MD 20899-8900 (http://www.nist.gov)

5SECG publications are available from the Standards for Efficient Cryptography Group (http://www.secg.org)

# Mandatory Algorithm Suite Specification

Author’s note: Each algorithm suite would presumably be included in its own annex, however the mandatory to implement algorithm suite will contain additional statements indicating that it is the mandatory to implement.

The algorithm suite specified in this clause shall be supported by all implementations supporting security.

## Object Identifier

The object identifiers for NTRU algorithm suites will be built off of the following OID root.  This root is written in ASN.1 format.

id-ntru-security-suites OBJECT IDENTIFIER ::= {
iso(1) identified-organization(3) dod(6) internet(1) private(4) enterprises(1) ntruCryptosystems(8342) ieee802-15-3(2) securitySuites(1)}

The object identifier for the security suite proposed in this clause is:

id-ntru-security-suite1 OBJECT IDENTIFIER ::= {
id-ntru-security-suites 1}

The DER encoding of this element, which shall be included in the OID field when this OID is used is the hex value: 06 0A 2B 06 01 04 01 C1 16 02 01 01.

## Security Functionality Provided

This security suite provides the following security services.

| Security Service | Provided |
| --- | --- |
| Mutual Authentication |  |
| Verification of Public-Key |  |
| Key Establishment |  |
| Key Transport |  |
| Beacon Integrity Protection |  |
| Freshness Protection |  |
| Command Integrity Protection |  |
| ACK Integrity Protection |  |
| Data Integrity Protection |  |
| Data Encryption |  |

Table XX - Supported security services

## Data Formats

The following table specifies the length and meaning of the undefined data elements from clause 7.

| Notation | Length | Value | Description |
| --- | --- | --- | --- |
| PublicKeyObjectType | 2 | 1 | The public key object type specifies NTRUEncrypt-251 public-key encryption keys as specified in EESS #1 using parameter set ees251ep1. |
| PublicKeyObjectLength | 2 | 251 | The length of an unpacked NTRUEncrypt-251 public key as specified in EESS #1 using parameter set ees251ep1. |
| PublicKeyObject | 251 | Variable | Unpacked NTRUEncrypt-251 public-key as specified in EESS #1 using parameter set ees251ep1. |
| AuthResponseType | 2 | 1 | The auth response type specifies the result of a two-key Triple-DES message authentication code using CBC-MAC-EDE mode as defined in ISO 9797. |
| AuthResponseLength | 2 | 8 | The length of a two-key Triple-DES message authentication code using CBC-MAC-EDE mode as defined in ISO 9797. |
| AuthResponse | 8 | Variable | The result of the two-key Triple-DES message authentication code using CBC-MAC-EDE mode as defined in ISO 9797. |
| OIDLength | 1 | 13 | The length of the DER encoding of the OID 1.3.6.1.4.1.8342.2.1.1. |
| OID | 13 | OID Value | The DER encoding of the object identifier 1.3.6.1.4.1.8342.2.1.1, which is the hex value 06 0A 2B 06 01 04 01 C1 16 02 01 01. |
| ChallengeType | 2 | 1 | The challenge type specifies an NTRUEncrypt-251 encryption of a 21-octet secret that is randomly and unpredictably generated at the time of the challenge. |
| ChallengeLength | 2 | 251 | The length of an unpacked NTRUEncrypt-251 public key encryption as specified in EESS #1 using parameter set ees251ep1. |
| Challenge | 251 | Variable | NTRUEncrypt-251 public-key encryption of the random challenge using the recipient device’s public key as specified in EESS #1 using parameter set ees251ep1. |
| ChallengeResponseType | 2 | 1 | The challenge response type specifies the result of a two-key Triple-DES message authentication code using CBC-MAC-EDE mode as defined in ISO 9797. |
| ChallengeResponseLength | 2 | 8 | The length of a two-key Triple-DES message authentication code using CBC-MAC-EDE mode as defined in ISO 9797. |
| ChallengeResponse | 8 | Variable | The result of the two-key Triple-DES message authentication code using CBC-MAC-EDE mode as defined in ISO 9797. |
| KeyPurpose | 1 | 0 | The type of key requested in key request protocols. Only seeds are transmitted in this security suite. |
| EncryptedKeyType | 2 | 1 | The encrypted key type specifies the result of two-key Triple-DES encryption of a 21-byte seed using CBC-EDE mode with initialization vector IV as defined in FIPS 46-3. |
| EncryptedKeyLength | 2 | 32 | The length of an encrypted 21-byte seed encrypted using two-key Triple-DES encryption using CBC-EDE mode with initialization vector IV as defined in FIPS 46-3. |
| EncryptedKey | 32 | Variable | The result of the two-key Triple-DES encryption using CBC-EDE mode with initialization vector IV as defined in FIPS 46-3. |

Table XX – Frame object formats

## Cryptographic Operation Selections

The security architecture is instantiated by the following algorithms.

- NTRUEncrypt encryption and decryption using parameter set ees251ep1 as specified in EESS #1 [EESS#1] for public key operations

- Two-key Triple-DES CBC-EDE mode as specified in FIPS 46-3 [FIPS46-3] for all symmetric encryption

- Two-key Triple-DES CBC-MAC-EDE message authentication codes as specified in ISO/IEC 9797 [ISO/IEC9797] for symmetric integrity protection.

- SHA-1 cryptographic hash as specified in FIPS 180-1 [FIPS180-1] for all cryptographic hashing.

The use of the cryptographic algorithms for each of the security operations performed in the piconet are specified in the following table:

| Use | Operation |
| --- | --- |
| Verification of Public-Key | The ID and public-key received during the authentication protocol is verified by generating the SHA-1 hash of the device address concatenated with the public key of the device and comparing it to the stored hash of the ID and public key stored in the MAC PIB. |
| Challenge generation | The challenges generated during the authentication protocol are computed by performing an NTRUEncrypt encryption on a fresh, randomly generated 21-byte challenge using the other device’s public key. |
| Challenge decryption | The challenge decryption operation is the NTRUEncrypt decryption of the challenge received. |
| Seed generation (for authentication protocol) | The 42-byte seed for the authentication protocol consists of the decrypted challenge from the security manager, concatenated with the decrypted challenge of the DEV. |
| Integrity Key Derivation | All integrity keys are generated from a seed by first calculating the SHA-1 hash of the seed concatenated with the byte 0x00 and then setting the key to be the first 112-bits of the result. |
| Encryption Key Derivation | All encryption keys are generated from a seed by first calculating the SHA-1 hash of the seed concatenated with the byte 0x01 and then setting the key to be the first 112-bits of the result. |
| Challenge response generation | The challenge response is computed by computing the TDES message authentication code on the entire authentication protocol up to that point using the integrity key. |
| Authentication response generation | The authentication response is computed by computing the TDES message authentication code on the entire authentication protocol up to that point using the integrity key. |
| Beacon message authentication code generation (Integrity Code information element) | The message authentication code included in the beacon is computed as the TDES message authentication code on the entire beacon up to the integrity code information element using the integrity key. |
| Command message authentication code generation | The message authentication code included in command frames is computed as the TDES message authentication code on the entire command up to the message authentication code using the integrity key. |
| ACK message authentication code generation | The message authentication code included in ACK frames is computed as the TDES message authentication code on the entire ACK up to the message authentication code using the integrity key. |
| Data message authentication code generation | The message authentication code included in data frames is computed as the TDES message authentication code on the entire data frame up to the message authentication code after encryption has been performed using the integrity key. |
| Seed encryption operation (for request key and distribute key) | The seed for key transport is encrypted using TDES with a random IV using the encryption key. |
| Data encryption generation | Data in a data frame is encrypted using TDES with a random IV using the encryption key. |

Table XX – Security related operations

The following table specifies the instantiation of the protocols in the notation from clause XX.

| Notation | Definition |
| --- | --- |
| Enc(m, K) | NTRUEncrypt-251 public-key encryption of the message m using the public key K as specified in EESS #1 using parameter set ees251ep1. |
| ID_D | The 48-bit IEEE MAC address uniquely identifying the device. |
| ID_SM | The 48-bit IEEE MAC address uniquely identifying the device. |
| PKObj_D | The device’s NTRUEncrypt-251 public key as specified in EESS #1 using parameter set ees251ep1. |
| PKObj_SM | The security manager’s NTRUEncrypt-251 public key as specified in EESS #1 using parameter set ees251ep1. |
| Pub_D | The device’s NTRUEncrypt-251 public key as specified in EESS #1 using parameter set ees251ep1. |
| Pr_D | The device’s NTRUEncrypt-251 private key as specified in EESS #1 using parameter set ees251ep1. |
| Pub_SM | The security manager’s NTRUEncrypt-251 public key as specified in EESS #1 using parameter set ees251ep1. |
| Pr_SM | The security manager’s NTRUEncrypt-251 private key as specified in EESS #1 using parameter set ees251ep1. |
| C1 | 21-octet secret that is randomly and unpredictably generated at the time of the challenge. |
| C2 | 21-octet secret that is randomly and unpredictably generated at the time of the challenge. |
| OID | Uniquely identifies the security suite. The object identifier is the ASN.1 DER encoding of the OID as defined by ISO/ITU 8824. For this security suite, this is the hex value 0x060A2B06010401C116020101. |
| SSID_D | 8-octet random value chosen by the security manager to uniquely identify the keys used to communicate with device D. |
| SSID_G | 8-octet random value chosen by the security manager to uniquely identify the keys used to communicate in the piconet. |
| Seed_G | 21-octet random value associated with a particular SSID_G used to generate the encryption key Enc_G and integrity key Int_G. |
| Seed_D | 21-octet random value associated with a particular SSID_D used to generate the encryption key Enc_D and integrity key Int_D. |
| Enc_D | 112-bit two-key Triple-DES key associated with a particular SSID_D, to be used in TCBC-EDE mode as defined in FIPS 46-3. |
| Enc_G | 112-bit two-key Triple-DES key associated with a particular SSID_G, to be used in TCBC-EDE mode as defined in FIPS 46-3. |
| Int_D | 112-bit two-key Triple-DES key associated with a particular SSID_D, to be used in TCBC-MAC mode as defined in ISO 9797. |
| Int_G | 112-bit two-key Triple-DES key associated with a particular SSID_G, to be used in TCBC-MAC mode as defined in ISO 9797. |
| seq_num_SM | 4-octet integer in network byte order associated with a particular SSID_D, used to count commands sent by the SM using that key. The sequence number shall begin counting with 0. |
| seq_sum_D | 4-octet integer in network byte order associated with a particular SSID_D, used to count commands sent by the device using that key. The sequence number shall begin counting with 0. |
| SymE(m, Enc, IV) | The result of two-key Triple-DES encryption of the message m with the Triple-DES key Enc using CBC-EDE mode with initialization vector IV as defined in FIPS 46-3. |
| SymI(m, Int) | The result of two-key Triple-DES message authentication code on the message m with the Triple-DES key Int using CBC-MAC-EDE mode as defined in ISO 9797. If m is “…”, the message authentication code is computed over all preceding fields in the frame. |
| PP(h, m, K1, K2) | - The result of the header h concatenated with the encryption e of message m, using the key K1, concatenated with the integrity code on h concatenated with e using the key K2. |
| H(m) | The 20-octet result of SHA-1 hash on the message m as defined in FIPS 181. |
| m¦¦n | The concatenation of two messages m and n. |
| Key(m) | The 112-bit result of truncating the message m to be used as a two-key Triple-DES key. |
| AReq | Authentication Request command header |
| CReq | Challenge Request command header |
| CRes | Challenge Response command header |
| ARes | Authentication Response command header |
| KUReq | Key Update Request command header |
| KURes | Key Update Response command header |
| KRReq | Key Request command header |
| KRRes | Key Request Response command header |
| finished1 | SymI(m, Int) where m is the entire set of data in order in the preceding protocol up to the point of the message authentication code and Int is Int_D. |
| finished2 | SymI(m, Int) where m is the entire set of data in order in the preceding protocol up to the point of the message authentication code and Int is Int_D. |

# Optional Algorithm Suite Specification (MQV-Implicit Certificates)

Author’s note: This algorithm suite is designed to correspond to the proposed algorithm selections specified in 02114r4, 02113r0, 02144r1 and 02135r0.  Since these proposals were made based on a different architecture, some additional details are filled in by the author.  Wherever possible, the changes will be noted and explained.

The algorithm suite specified in this clause may be supported by implementations supporting security.

## Object Identifier

Author’s note: An object identifier is being provided here for this algorithm suite for completeness.  This OID may be replaced by an appropriate OID specified by the algorithm proposers.

The object identifiers for this algorithm suite will be built off of the following OID root.  This root is written in ASN.1 format.

id-ntru-security-suites OBJECT IDENTIFIER ::= {
iso(1) identified-organization(3) dod(6) internet(1) private(4) enterprises(1) ntruCryptosystems(8342) ieee802-15-3(2) securitySuites(1)}

The object identifier for the security suite proposed in this clause is:

id-ntru-security-suite1 OBJECT IDENTIFIER ::= {
id-ntru-security-suites 2}

The DER encoding of this element, which shall be included in the OID field when this OID is used is the hex value: 06 0A 2B 06 01 04 01 C1 16 02 01 02.

## Security Functionality Provided

This security suite provides the following security services.

| Security Service | Provided |
| --- | --- |
| Mutual Authentication |  |
| Verification of Public-Key |  |
| Key Establishment |  |
| Key Transport |  |
| Beacon Integrity Protection |  |
| Freshness Protection |  |
| Command Integrity Protection |  |
| ACK Integrity Protection |  |
| Data Integrity Protection |  |
| Data Encryption |  |

Table XX - Supported security services

## Data Formats

The following table specifies the length and meaning of the undefined data elements from clause 7.

Author’s Note: The following comments apply to the table entries below.

Author’s Note: Multiple key strengths were proposed for the ECC algorithm suites, however most specified a key strength equivalent to 128-bits (symmetric) security.  This algorithm suite uses the 128-bit strengths. In the case of the ECC operations this is done using the ANSI-approved curve ansit283k1. It should be relatively simple to define an alternative algorithm suite using 80-bit strength.

Author’s Note: The implicit certificate along with additional information is used as the public key object.  The additional information may contain information such as issuer, issue date, expiry date, serial number. This is currently left undefined in this proposal.

Author’s Note: The text in 02130r1 includes identifying numbers for ECC algorithms at the 80-bit strength rather than the 128-bit strength.  This document uses the 80-bit numbers for the 128-bit values.  If the 128-bit algorithm suite is accepted, these numbers should be changed in the architecture document.

Author’s Note: The HMAC specification defines full HMAC or truncated HMAC.  The non-truncated HMAC was included, but this may be changed if desired.

Author’s Note: The length of the seed for generating keys has been set to 256 bits.

| Notation | Length | Value | Description |
| --- | --- | --- | --- |
| PublicKeyObjectType | 2 | 5 | The public key object type consists of the following elements: additional information that is used in the construction of the implicit certificate and an ECC implicit certificate as specified in SEC 4 using the elliptic curve ansit283k1 as specified in ANSI X9.63. |
| PublicKeyObjectLength | 2 | 37 + length of add. info | The length of the additional information concatenated with the implicit certificate. |
| PublicKeyObject | 37 + length of add. info | Variable | The additional information for construction of the implicit certificate and an ECC implicit certificate as specified in SEC 4 using the elliptic curve ansit283k1 as specified in ANSI X9.63. |
| AuthResponseType | 2 | 3 | The auth response type specifies the result of an HMAC computation as defined in FIPS #HMAC using a 32-byte key and the SHA-256 hash algorithm as defined in FIPS 180-2. |
| AuthResponseLength | 2 | 32 | The length of an HMAC computation using a 32-byte key as defined in FIPS #HMAC. |
| AuthResponse | 32 | Variable | The result of the HMAC computation using a 32-byte key as defined in FIPS #HMAC. |
| OIDLength | 1 | 13 | The length of the DER encoding of the OID 1.3.6.1.4.1.8342.2.1.2. |
| OID | 13 | OID Value | The DER encoding of the object identifier 1.3.6.1.4.1.8342.2.1.2, which is the hex value 06 0A 2B 06 01 04 01 C1 16 02 01 02. |
| ChallengeType | 2 | 4 | The challenge type specifies an ECC MQV ephemeral key, which is a point on the curve ansit283k1 as specified in ANSI X9.63, that is randomly and unpredictably generated at the time of the challenge. |
| ChallengeLength | 2 | 37 | The length of an ECC MQV ephemeral key, which is a point on the curve ansit283k1 as specified in ANSI X9.63. |
| Challenge | 37 | Variable | The ECC MQV ephemeral key, which is a point on the curve ansit283k1 as specified in ANSI X9.63. |
| ChallengeResponseType | 2 | 4 | The challenge response type specifies the result of an HMAC computation as defined in FIPS #HMAC using a 32-byte key and the SHA-256 hash algorithm as defined in FIPS 180-2. |
| ChallengeResponseLength | 2 | 32 | The length of an HMAC computation as defined in FIPS #HMAC using a 32-byte key and the SHA-256 hash algorithm as defined in FIPS 180-2. |
| ChallengeResponse | 32 | Variable | The result of the HMAC computation as defined in FIPS #HMAC using a 32-byte key and the SHA-256 hash algorithm as defined in FIPS 180-2. |
| KeyPurpose | 1 | 0 | The type of key requested in key request protocols. Only seeds are transmitted in this security suite. |
| EncryptedKeyType | 2 | 2 | The encrypted key type specifies the result of AES-256 CBC encryption of the 256-bit seed with random IV as specified in FIPS Pub ZZZ and NIST Special Publication 800-38A. |
| EncryptedKeyLength | 2 | 64 | The length of an encrypted 256-bit seed encrypted using AES-256 CBC encryption with random IV as specified in FIPS Pub ZZZ and NIST Special Publication 800-38A. |
| EncryptedKey | 64 | Variable | The result of the encryption of the 256-bit seed using AES-256 CBC encryption with random IV as specified in FIPS Pub ZZZ and NIST Special Publication 800-38A. |

Table XX – Frame object formats

## Cryptographic Operation Selections

The security architecture is instantiated by the following algorithms.

- A four-pass version of MQV based on the three-pass version specified in ANSI X9.63 [X9.63] for public-key key agreement and authentication

- 128-bit AES CBC-EDE mode as specified in [AES] for all symmetric encryption

- HMAC with SHA-256 message authentication codes as specified in [HMAC] for symmetric integrity protection.

- SHA-256 cryptographic hash as specified in [FIPS180-2] for all cryptographic hashing.

- Implicit certification as specified in [SEC4] for establishing authenticity of public keys.

- Elliptic curve point arithmetic on the elliptic curve ansit283k1 defined in appendix J.4.5 of ANSI X9.63. Note that all points are to be transmitted in compressed form.

Author’s Note: The diagram given in 02130r1P802-15_TG3-NTRU-Security-Architecture-Proposal describes only the mandatory to implement cipher suite (or any other cipher suite based on public key encryption and decryption, rather than discrete log operations). The following diagram describes the equivalent processing and message contents for MQV with implicit certificates. Note that in this case, the notation “Pub_SM” refers not to the Security Manager’s public key, but to its implicit certificate. This and other notation decisions are clarified in the table below.

The use of the cryptographic algorithms for each of the security operations performed in the piconet are specified in the following table:

| Use | Operation |
| --- | --- |
| Verification of Public-Key | The verification of the public key is assured by the certification by the appropriate CA and is implicitly verified if the MQV authentication and key exchange protocol succeeds. |
| Challenge generation | The challenges generated during the authentication protocol consist of an ephemeral public/private key pair, where the private key is an integer and the public key is a point on the curve. |
| Seed generation (for authentication protocol) | The seed K for the authentication protocol consists of the result of the each party’s MQV computation on the challenges, the other party’s certification information, the CA’s public key, and the party’s own private key. |
| Integrity Key Derivation | All integrity keys are generated from the seed K by calculating the SHA-256 hash of the seed concatenated with the byte 0x00 and then setting the key to be the first 128-bits of the result. |
| Encryption Key Derivation | All encryption keys are generated from the seed K by calculating the SHA-256 hash of the seed concatenated with the byte 0x01 and then setting the key to be the first 128-bits of the result. |
| Challenge response generation | The challenge response is computed by computing the HMAC message authentication code on the entire authentication protocol up to that point using the integrity key. |
| Authentication response generation | The authentication response is computed by computing the HMAC message authentication code on the entire authentication protocol up to that point using the integrity key. |
| Beacon message authentication code generation (Integrity Code information element) | The message authentication code included in the beacon is computed as the HMAC message authentication code on the entire beacon up to the integrity code information element using the integrity key. |
| Command message authentication code generation | The message authentication code included in command frames is computed as the MHAC message authentication code on the entire command up to the message authentication code using the integrity key. |
| ACK message authentication code generation | The message authentication code included in ACK frames is computed as the HMAC message authentication code on the entire ACK up to the message authentication code using the integrity key. |
| Data message authentication code generation | The message authentication code included in data frames is computed as the HMAC message authentication code on the entire data frame up to the message authentication code after encryption has been performed using the integrity key. |
| Seed encryption operation (for request key and distribute key) | The seed for key transport is encrypted using AES with a random IV using the encryption key. |
| Data encryption generation | Data in a data frame is encrypted using AES with a random IV using the encryption key. |

Table XX – Security related operations

The following table specifies the instantiation of the protocols in the notation from clause XX.

Author’s Note: The exact format of the public key object (implicit certificate with identifying information) as with the previous table is not specified.

| Notation | Definition |
| --- | --- |
| ID_D | The 48-bit IEEE MAC address uniquely identifying the device. |
| ID_SM | The 48-bit IEEE MAC address uniquely identifying the device. |
| PKObj_D | The device’s implicit certificate object. This contains the device’s implicit certificate Pub_D. It may additionally contain information such as issuer, issue date, expiry date, serial number. This is considered to be the string AdditionalInfo, and may or may not be present. |
| PKObj_SM | The security manager’s implicit certificate object. This contains the security manager’s implicit certificate Pub_SM. It may additionally contain information such as issuer, issue date, expiry date, serial number. This is considered to be the string AdditionalInfo, and may or may not be present. |
| Pub_D | The device’s implicit certificate, which is a point on the elliptic curve ansit283k1 defined in appendix J.4.5 of ANSI X9.63. |
| Pr_D | The device’s private key, which is an integer less than the order of the elliptic curve ansit283k1 defined in appendix J.4.5 of ANSI X9.63. |
| Pub_SM | The security manager’s implicit certificate, which is a point on the elliptic curve ansit283k1 defined in appendix J.4.5 of ANSI X9.63. |
| Pr_SM | The security manager’s private key, which is an integer less than the order of the elliptic curve ansit283k1 defined in appendix J.4.5 of ANSI X9.63. |
| P | The base point of the elliptic curve ansit283k1 defined in appendix J.4.5 of ANSI X9.63. |
| x | A random positive integer, generated by the device, greater than 2 and less than the order of the elliptic curve ansit283k1 defined in appendix J.4.5 of ANSI X9.63. |
| X | The elliptic curve point xP, calculated according to the techniques of ANSI X9.63 |
| y | A random positive integer, generated by the security manager, greater than 2 and less than the order of the elliptic curve ansit283k1 defined in appendix J.4.5 of ANSI X9.63. |
| Y | The elliptic curve point yP, calculated according to the techniques of ANSI X9.63 |
| OID | Uniquely identifies the security suite. The object identifier is the ASN.1 DER encoding of the OID as defined by ISO/ITU 8824. For this security suite, this is the hex value 0x060A2B06010401C116020102. |
| SSID_D | 8-octet random value chosen by the security manager to uniquely identify the keys used to communicate with device D. |
| SSID_G | 8-octet random value chosen by the security manager to uniquely identify the keys used to communicate in the piconet. |
| Seed_G | 256-bit random value associated with a particular SSID_G used to generate the encryption key Enc_G and integrity key Int_G. |
| K | 256-bit seed value agreed on during the MQV authentication process associated with a particular SSID_D used to generate the encryption key Enc_D and integrity key Int_D. |
| Enc_D | 128-bit AES key associated with a particular SSID_D, to be used in AES-CBC mode as defined in [AES] and [MODES]. |
| Enc_G | 128-bit AES key associated with a particular SSID_G, to be used in AES-CBC mode as defined in [AES] and [MODES]. |
| Int_D | 128-bit HMAC-with-SHA-256 key associated with a particular SSID_D |
| Int_G | 128-bit HMAC-with-SHA-256 key associated with a particular SSID_G |
| seq_num_SM | 4-octet integer in network byte order associated with a particular SSID_D, used to count commands sent by the SM using that key. The sequence number shall begin counting with 0. |
| seq_sum_D | 4-octet integer in network byte order associated with a particular SSID_D, used to count commands sent by the device using that key. The sequence number shall begin counting with 0. |
| SymE(m, Enc, IV) | The result of AES encryption of the message m with the AES key Enc using CBC mode with initialization vector IV as defined in [AES] and [MODES]. |
| SymI(m, Int) | The result of calculating the HMAC-with-SHA-256 message authentication code on the message m with the 128-bit HMAC key Int. If m is “…”, the message authentication code is computed over all preceding fields in the frame. |
| PP(h, m, K1, K2) | - The result of the header h concatenated with the encryption e of message m, using the key K1, concatenated with the integrity code on h concatenated with e using the key K2. |
| H(m) | The 32-octet result of SHA-256 hash on the message m as defined in [FIPS180-2]. |
| m¦¦n | The concatenation of two messages m and n. |
| Key(m) | The 128-bit result of truncating the message m to be used as a 128-bit AES key. |
| AReq | Authentication Request command header |
| CReq | Challenge Request command header |
| CRes | Challenge Response command header |
| ARes | Authentication Response command header |
| KUReq | Key Update Request command header |
| KURes | Key Update Response command header |
| KRReq | Key Request command header |
| KRRes | Key Request Response command header |
| finished1 | SymI(m, Int) where m is the entire set of data in order in the preceding protocol up to the point of the message authentication code and Int is Int_D. |
| finished2 | SymI(m, Int) where m is the entire set of data in order in the preceding protocol up to the point of the message authentication code and Int is Int_D. |

# Security Analysis

Author’s note: The security analysis on the comparison with TLS and some of the attack scenarios also apply to the security architecture document.  Some of this text may be included in the security considerations section of the draft standard.

## Protocol Security Analysis

### Comparison with TLS

The following describes some of the differences between the TLS protocol and the protocols defined in clause XX. We first describe the differences between the TLS protocol and our design, and then the differences between the proposed security suite and ciphersuites supported in TLS.

- TLS provides integrity with HMAC and PRF using SHA-1 and MD5. This proposal instead uses TDES CBC-MAC. 

SHA-1 involves many 32-bit operations. On the relatively powerful platforms that TLS is commonly used on, SHA-1 is very fast. However, in constrained environments, especially ones with 8-bit processors, Triple DES is considerably faster. The Triple DES CBC-MAC is an international standard means of providing message integrity and is highly suited to the 802.15.3 scenario.

- TLS requires certificates. This proposal does not.

In TLS, the typical use case is that a user’s browser communicates to an e-commerce site and requires assurance that the site belongs to a legitimate business. Certificates are highly suitable for this situation. In 802.15.3, a DEV is obtaining a connection to a network, which contains a wide variety of devices with different functionality and requirements. In this situation, certificates are not of great practical value.

Additionally, the proper use of certificates introduces financial, technical and administrative complexities for device manufacturers. All manufacturers together need to decide which root certificates to install and what the trust model is. Device manufacturers need to provision each device with its own certificates and with root certificates. Users need to be able to understand certificate information and make decisions based on it; a wrong decision compromises the security architecture. Timely revocation information needs to be provided or the compromise of a single device compromises every system. Unless all of these factors are taken on board by all manufacturers, the use of certificates simply results in a more complex system, with all the security vulnerabilities that implies.

- TLS typically authenticates the server with an encrypted challenge-response, while client authentication is done by means of a signed challenge from the server. This proposal instead uses encrypted challenge-response to authenticate both parties.

This change is desirable for several reasons:

  - If both parties use an encrypted challenge-response, both parties can contribute to the freshness of the key that is established. This is a desirable security feature.

  - In price-sensitive devices such as 802.15.3, gates and cost can be saved by not requiring devices that might act as a security manager to implement signing as well as encryption and decryption.

  - For the specific NTRU cipher suites proposed, encryption and decryption are considerably faster than signing and verification. This increases the probability that authentication of a device to the network can be completed quickly and efficiently.

- TLS has cipher suite agreement inline. This proposal does not have cipher suite agreement inline for devices authenticating to the network.

For a group networking protocol, all devices must use the same cipher suite. A device that wants to join the network should not be able to compel all other devices to change cipher suite. Therefore, there is no requirement for cipher suite negotiation when a device is joining the group. 

For peer-to-peer communications, we support cipher suite negotiation, as this is appropriate in this context.

- In TLS, the client (device) commits to a public random value in step 1. In this proposal, there are no public random values.

TLS requires the use of public random values to ensure that its Diffie-Hellman-based ciphersuites produce a different seed every time. The use of random challenges generated by each party ensures that a unique seed is produced every time.

- In TLS, the server (security manager) commits to a public random value in step 2. In this proposal, there are no public random values.

See the previous comment.

- TLS offers optional compression. This proposal does not.

TLS is an application-layer protocol. The data it produces is passed down through the protocol stack. However, because it is encrypted, it appears random, which means that it cannot be compressed by any process running further down the stack than TLS. TLS therefore has to offer compression. In contrast, 802.15.3 is a networking protocol. If data is to be compressed, it will be done at a higher level than the MAC layer.

- In TLS, the sequence number used for payload protection is not explicitly sent (it is stored internally and incremented). This proposal transmits the sequence number, cryptographically protected, in the messages.

TLS uses a reliable communications channel between two applications, and can assume that packets arrive in order and that both parties are still involved in the communication. 802.15.3 is a standard for ad hoc networking and can make no such assumptions. The safest way to ensure all devices have access to the data they need is to explicitly include it in the messages.

- TLS does not require additional key transport messages after the handshake (authentication) is complete.

In TLS, once a shared key is established between the two end-entities, no further exchange of cryptographic material is necessary. In this proposal, once the shared key has been established, the joining device must have the group key securely transmitted to it before it can participate in the group communications. For this reason, the current proposal requires the additional key transport message.

- Key derivation from the seed is different in TLS from this proposal.

TLS requires the use of two different hash functions, MD5 and SHA-1, to derive the key from the seed.

### Protection Against Attacks

Attackers herein are assumed to be non-members of the secure piconet.  An authenticated member of the piconet has easy avenues of attack, including revealing decrypted data.  More difficult first-party attacks are not considered further.

Similarly, non-members of the secure piconet have easy denial of service attacks: simply jam the radio.  More difficult denial of service attacks are not considered further.

| Type |  | Attack | Defense |
| --- | --- | --- | --- |
| Attacks on the Public-key Binding |  |  |  |
|  | Public-key Substitution, or “the man in the middle” | When the public key is sent from one device to another, an attacker may try to insert her public key instead. If this key is accepted by the recipient, the attacker would gain all access privileges that the legitimate device would have. | The DME bears the responsibility for establishing a binding between a public key and a device’s identity, using the method best suited to the device. |
|  | Multiple Public-key/ID Pairs with Same Binding Hash | The DME binds a public key and ID together by writing H(ID_D ¦¦ Pub_D) to the ACL. An attacker may try to find another pair of Pub_D and ID_D values that give the same hash. | Using the SHA-1 cryptographic hash algorithm, an attacker needs to try 280 values before finding any two values with the same hash. |

| Attacks on Authentication Protocol |  |  |  |
| --- | --- | --- | --- |
|  | Replay attack | An attacker may attempt to authenticate by recording a legitimate command from a previous transaction and using it in the current protocol. | Each authentication protocol is unique due to each device choosing its own random challenge value. During the final steps of the protocol, the finished message derived from the entire protocol ensures that only a party that knows the fresh secret (which can only be the owner of the corresponding private key) participated in the protocol. The fact that the whole message is protected by the finished message also prevents the insertion of a message from a different protocol into the current run of the protocol. |
|  | Decryption of challenges | An attacker may attempt to decrypt the challenge in order to obtain the secret. | Using the LLL lattice-reduction algorithm to find the NTRUEncrypt private key or decrypt an individual message requires 5 x 1010 MIPS-Years. |
|  | Brute force on shared key | An attacker may attempt to guess the value of a key. | Randomly guessing a 112-bit Triple-DES key requires 2111 guesses on average. |
|  | Challenge substitution | An attacker may attempt to substitute a symmetric key in the protocol instead of the one sent by the intended party. | The finished messages provide integrity protection on all data in the protocol, including the challenges. Both parties in the protocol would detect the tampering of the key and reject the protocol’s outcome when the finished messages do not match their expected values. |
|  | Security Suite Substitution | An attacker may attempt to trick a device into using keys with the wrong algorithms. | The finished messages provide integrity protection on all data in the protocol, including the OID. Both parties in the protocol would detect the tampering and reject the protocol’s outcome when the finished messages do not match their expected values. |
|  | Authentication States Not Synchronized | An attacker may attempt to inject an authentication command frame. | During the authentication process, an attacker can issue a Disassociate command frame and the device will return to the Unassociated state. Without knowing either the device’s public key or decrypted challenges (see above), the attacker is unable to impersonate the device. |
|  | Dictionary Attacks | An attacker builds a table of messages from successful authentications | If an attacker can’t break NTRUEncrypt given only one message, he can’t break NTRUEncrypt no matter how many messages he collects. |

| Attacks on the Beacon |  |  |  |
| --- | --- | --- | --- |
|  | Beacon Replay | The attacker could replay the beacon in order to replay messages. | A device rejects any beacon with a TimeToken less than or equal to a TimeToken it has already received. |
|  | PNC Spoofing | The attacker could broadcast a fake beacon to confuse devices. | Randomly guessing the Triple-DES integrity code on a message requires 263 guesses on average. Since a device will only accept a message with a valid TimeToken, the TimeToken changes with every superframe, and much less than 263 frames can be sent in a superframe, this attack is infeasible. Randomly guessing the 112-bit Triple-DES key would allow the attacker to forge integrity codes. This requires 2111 guesses on average. |
|  | Time Rollback | The attacker could broadcast an old or fake beacon with an old TimeToken | In addition to the integrity code problems listed under PNC Spoofing, devices only accept strictly increasing TimeToken values. |

| Attacks on the Distribute Key and Request Key Protocols |  |  |  |
| --- | --- | --- | --- |
|  | Command Spoofing | An attacker could attempt to inject a command telling a device to update its key. | The security manager computes an integrity code over the command using a key known to only the device and the security manager. |
|  | Command Replay | An attacker could replay a command | PNC-DEV commands include a sequence number. Any received command with a sequence number less than or equal to the last received sequence number is rejected. A DEV-DEV command may only be replayed while its TimeToken is valid: typically the duration of one superframe. Replay within a superframe is not prevented, but appears to offer little threat. |

| Attacks on Other Commands |  |  |  |
| --- | --- | --- | --- |
|  | Message Replay | An attacker could replay a message | A message may only be replayed while its TimeToken is valid: typically the duration of one superframe. Replay within a superframe is not prevented, but appears to offer little threat. |

| Attacks on Data Frames |  |  |  |
| --- | --- | --- | --- |
|  | Brute Force on Data Keys | An attacker may attempt to guess the value of a key. | Randomly guessing a 112-bit Triple-DES key requires 2111 guesses on average. |
|  | Source Spoofing From a Non-group Member | An attacker may try to authenticate as another device. | The DME bears the responsibility for establishing a binding between a public key and a device’s identity, using the method best suited to the device. |
|  | Source Spoofing From a Group Member | An attacker may try to guess the correct integrity code on a message. | Randomly guessing the Triple-DES integrity code on a message requires 263 guesses on average. Since a device will only accept a message with a valid TimeToken, the TimeToken changes with every superframe, and much less than 263 frames can be sent in a superframe, this attack is infeasible. |
|  | Replay of Data | An attacker could replay a message. | A message may only be replayed while its TimeToken is valid: typically the duration of one superframe. Replay within a superframe is not prevented, but appears to offer little threat. |
|  | Traffic Analysis | An attacker could attempt to determine information about the data based on the length of the encrypted data. | The encryption mechanism pads the message to the nearest 8-byte length. Traffic analysis protection is not provided beyond the padding. If traffic analysis protection is desired, the DME should ensure that data packets each have the same length before they are encrypted. |
|  | Cut-and-paste of Ciphertext | An attacker could insert ciphertext sent to one device into a message sent to another device, which might then display the decrypted ciphertext. | This attack is not possible unless the attacker knows the message integrity key or can forge the message integrity code. Randomly guessing the Triple-DES integrity code on a message requires 263 guesses on average. |
|  | Dictionary Attacks | The attacker collects a table of messages and their integrity codes. | The construction of the Triple-DES integrity code means that no matter how many messages an attacker sees, they have no advantage in guessing the integrity code on a message they have not previously seen. |
|  | Message Authentication Code Attacks | An attacker may try to guess the correct integrity code on a message. | Randomly guessing the Triple-DES integrity code on a message requires 263 guesses on average. Since a device will only accept a message with a valid TimeToken, the TimeToken changes with every superframe, and much less than 263 frames can be sent in a superframe, this attack is infeasible. |
|  | Replay Attacks | An attacker could replay a message | A message may only be replayed while its TimeToken is valid: typically the duration of one superframe. Replay within a superframe is not prevented, but appears to offer little threat. |

# Efficiency Analysis

## Frame Protection

Triple-DES for encryption and integrity on command and data frames should be implemented in hardware.  The DES algorithm processes 64 bits at a time, has 16 rounds, and a typical implementation instantiates two rounds of DES in 9,796 gates.  Thus encrypting 64 bits with DES takes 8 clock cycles while Triple-DES takes 24 cycles.  Adding another DES round for integrity, 32 cycles are required to provide both encryption and integrity.  Thus, to achieve a data rate of 55 Mbps, a clock rate of 30 MHz is required.

## Authentication

Software computational efficiency results in this section are based on a commercially available 8-bit microcontroller running at 2.66 MHz.  We assume a superframe lasts 65 msec.  On this processor, encryption and decryption using NTRUEncrypt can complete in the duration of a single superframe: encryption takes 42 msec, decryption takes 60 msec.  A SHA-1 hash takes 8.2 msec.  These functions are only used during authentication.  The following table details the computation and communication needed for authentication on a superframe-by-superframe basis.

| Super- frame Number | Action | Time (msec) | Octets |
| --- | --- | --- | --- |
| 1 | DEV sends Authenticate Request Command to SM |  | 277 |
| 2 | SM checks if H(ID_D, Pub_D) is in its ACL. If not, send failure message and abort. | 41 |  |
|  | Generate challenge C2 | 16.4 |  |
| 3 | Encrypt C2 with Pub_D | 42 |  |
|  | SM sends Challenge Request Command to DEV |  | 554 |
| 4 | Dev checks if H(ID_SM, Pub_SM) is in its ACL. If not, send failure message and abort. | 41 |  |
| 5 | Decrypt C2 | 60 |  |
| 6 | Dev generates challenge C1 | 16.4 |  |
|  | Compute Enc_D=H(C1¦¦C2¦¦0x00) | 8.2 |  |
|  | Compute Int_D=H(C1¦¦C2¦¦0x01) | 8.2 |  |
|  | Compute finished1 = SymI(…) | .14 |  |
| 7 | DEV encrypts C1 with Pub_SM | 42 |  |
|  | DEV sends Challenge Response Command to SM |  | 283 |
| 8 | SM decrypts C1 | 60 |  |
| 9 | SM computes Enc_D=H(C1¦¦C2¦¦0x00) | 8.2 |  |
|  | Compute Int_D=H(C1¦¦C2¦¦0x01) | 8.2 |  |
|  | Verify IC on finished1 | .14 |  |
|  | Compute finished2 = SymI(…) | .14 |  |
| 10 | SM sends Authentication Response to DEV | 29 |  |
|  | DEV computes finished2 = SymI(…) | .14 |  |

The Authentication protocol completes in ten superframes, or 650 msec.  The total processing time for both the DEV and the SM is approximately 176 ms.  A total of 1,143 octets are exchanged.

## Access Control List Storage

A device keeps a table of the other devices with which it has a security relationship.  To facilitate reintroduction of devices, one can use persistent store such as EEPROM for parts of the ACL.  The DME is responsible for maintaining the ACL by adding trusted devices and deleting untrusted devices.  Items in the ACL:

- Device address, the 48-bit IEEE MAC address of the other device.  Written only by the DME.

- 160-bit hash of device address and public key.  Written only by the DME.

- 1-bit indicator for the other device’s role in the relationship: SM or device.  Written only by the MLME.

- 8-bit Device ID for the piconet address.  Read and written only by the MLME.

- 112-bit management encryption key shared with that device.  Read and written only by the MLME.

- 112-bit management integrity key shared with that device.  Read and written only by the MLME.

- 64-bit SSID for management keys.  Read and written only by the MLME.

- 112-bit payload encryption key shared with that device.  Read and written only by the MLME.

- 112-bit payload integrity key shared with that device.  Read and written only by the MLME.

- 64-bit SSID for payload protection keys.  Read and written only by the MLME.

- 64-bit sequence number for this device’s use of the management keys.  Read and written only by the MLME.

- 64-bit sequence number for the other device’s use of the management keys.  Read and written only by the MLME.

These items total 921 bits per security relationship.  Of these, items 1 and 2 should be kept in persistent storage.

## Secure Group State Storage

A device must store state information for the piconet-wide security session.  These include:

- Device address of the PNC, the 48-bit MAC address of the piconet’s security manager.  Written only by the DME.

- 160-bit hash of device address and public key.  Written only by the DME.

- 112-bit management encryption key for the piconet.  Read and written only by the MLME.

- 112-bit management integrity key for the piconet.  Read and written only by the MLME.

- 64-bit SSID for management keys.  Read and written only by the MLME.

- 112-bit payload encryption key for the piconet.  Read and written only by the MLME.

- 112-bit payload integrity key for the piconet.  Read and written only by the MLME.

- 64-bit SSID for payload protection keys.  Read and written only by the MLME.

- 64-bit sequence number for this device’s use of the management keys.  Read and written only by the MLME.

- 64-bit sequence number for the other device’s use of the management keys.  Read and written only by the MLME.

- 64-bit TimeToken.  Read and written only by the MLME.

- 1-bit indicator of current or failed validity for this superframe.

These items total 977 bits.