IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | 02100r5P802-15_TG3-Comment-Resolution-Text.doc |  |
| Date Submitted | 26 March., 2002 |  |
| Source | Dr. William Shvodian <XtremeSpectrum> Suite 700, 8133 Leesburg Pike, Vienna, VA 22182 | Voice: 703-269-3047 Fax: 703-269-3092 E-mail: bshvodian@xtremespectrum.com |
| Re: | D09 |  |
| Abstract | This document provides the Text for D09 LB12 Issue Resolution |  |
| Purpose | The recommendations contained in this document are to be applied to the 802.15.3 D09 baseline. |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

1)	MLME-Sync-LOST.indication (Comment 1413)	3

2)	Multicast Text (Comment 305, 321, 377, 408, 475)	3

3)	CTR Change (Comment 1333)	3

4)	8 s to 1s time unit change (Comments 1491 and 792)	6

5)	Guard Time (Comment 1488, 1542, 1547, 1557, 505, 315)	7

6)	Slot Duration added to CTA (Comment 1488, 1492, 502, 505)	10

7)	Delayed ACK  (Comment  1464)	11

8)	Support for Asynchronous Traffic (Comment 1628)	12

# MLME-Sync-LOST.indication (Comment 1413)

This describes a new MLME primitive that is used to inform the DME that the DEV can no longer hear the Beacon.

## 6.3.3.3 MLME-SYNC-LOST.indication

This primitive is used to initiate that the DEV can no longer hear the Beacon. The semantics of the primitive are as follows:

MLME-SYNCH-LOST.indication 	(

)

## 6.3.3.3.1 When Generated

This primitive is generated by a non-PNC MLME when it does not correctly receive a beacon for greater than the Association Timeout Period (ATP) interval of time.

## 6.3.3.3.2 Effect of receipt

The DME is notified that the Association Timeout Period has expired and it responds accordingly.

# Multicast Text (Comment 305, 321, 377, 408, 475)

A full description of the usage of multicast was missing in D09.

## 8.6.1.1 Multicast Streams

Unique DEV IDs are not provided for individual multicast streams.  All multicast streams in a piconet shall use the multicast ID, in 7.2.3.  A DEV that is receiving a multicast stream should receive all frames addressed to the multicast destination address.  The DEV may choose not to receive multicast streams based on the stream index or source ID.

# CTR Change (Comment 1333)

Max Allocation Delay removed.  Also CTRB modified as specified in 02/076r2.

Clause 7.5.10.1 Modification

The channel time request command structure shall be formatted as illustrated in Figure 71. Each block corresponds to channel time requested for a particular stream. The DEV that sends this command is the originator and is seeking to allocate channel time to communicate with the target DEV. The format of a channel time request block for a given stream shall be formatted as illustrated in Figure 1.

Figure 71 12 octets per CTRB changed to 8 octets, no pad needed

Figure 1 Channel time request block for a particular stream

The target DEV ID indicates the DEV for which the originator DEV is requesting channel time for communication.

The stream index indicates the stream for which the channel time is being requested or modified. This field is defined in 7.2.4.

Figure 2 CTR Control Field

The CTR Control field contains information about the CTR.  The Stream Parameter field indicates if the CTR is new or if it is a modification of an existing stream.  The CTR Type field indicates if the CTR is an Active CTR or an SPS CTR.   The CTR Interval Type field indicates whether the CTR Interval field represents the number of slots per superframe, or the number of superframes per slot.  If the CTR Interval Type is zero, the CTR interval is the requested number of slots per superframe.  This allows an extremely latency sensitive application to request multiple slots per superframe.    Regardless of the CTR Interval Type field, if the CTR Interval field is zero, the DEV is requesting its fair share of as much channel time as is available.  This can be used for types of asynchronous traffic that can use as much bandwidth as is available.  The GTS type indicates if the request is for dynamic or pseudostatic GTS.  The CTR negotiation parameter indicates if the negotiation is bipartite or tripartite (TBR).

The SPS Set Index is used to identify the SPS set that the CTR is associated with if the CTR is an SPS CTR.

The CTR Time Unit field indicates the unit of time that the DEV is using for its request.  This allows the PNC to know what units of GTS time the DEV can make use of so that the PNC can efficiently allocate GTS time that is more than the minimum needed by the DEV, but less than the desired.   It also enables the PNC to fragment a slot if necessary.   The resolution of this field is 1 µs and therefore has a range of [0-65535] µs.

The Minimum CTR Time Units field indicates the minimum number of CTR Time Units that are required per slot.

The Desired CTR Time Units field indicates the desired number of CTR Time Units per slot.  Desired CTR Time Units shall be greater than or equal to Minimum CTR Time Units.

8.4.3.x Channel Time Requests

Each DEV sends Channel Time Requests to the PNC to indicate the amount of channel time required for transmission.  Channel time requests are made on a per stream basis.  As an example of how the fields of a channel time request frame are used, a DEV can request CTR Interval slots per superframe, where each slot has a minimum size of CTR Time Unit times Minimum CTR Time Unit.

The requesting DEV shall include frame transmission time (if known a priori) and ACK (if used) transmission time and one SIFS time per frame or ACK when calculating channel time requests.   Figure 3 shows an example of a channel time request when immediate ACK is not used.

Figure 3 Channel Time Request with no ACKs

Figure 4 shows an example of channel time being requested for a GTS slot where immediate ACKs are used.

Figure 4 Channel Time Request for Frames with Immediate ACKs

A GTS Time Unit in the CTA may cover more than one frame as shown in Figure 5.

Figure 5 GTS Time Unit covering multiple frames

If the frame size is not known a priori, it is up to the requesting DEV to decide how much channel time to request for the GTS.

8.4.3.1: add the following sentence

If multiple slots per superframe are requested, the PNC shall attempt to spread the slots out evenly within the superframe.

# 8 s to 1s time unit change (Comments 1491 and 792)

## Overall: Do a global search on 8 µs and replace:

From:

The resolution of this field is 8 µs and so the range of this field is [0-524280] µs.

To:

The resolution of this field is 1 µs and so the range is [0-65535] µs.

Note: some of the sentences are worded slightly different.

# Guard Time (Comment 1488, 1542, 1547, 1557, 505, 315)

Text was approved last fall, but never inserted into the draft because it needed to be updated to reflect slot durations in the CTAs.   This has been modified based on slot duration in the CTAs.

Note: Issue 1547 is covered by the addition of CAP End Time.

## New clause 8.4.3.x Guard Time,

In a TDMA system, guard times are required to keep transmissions in adjacent slots from colliding.  In addition, SIFS is required to ensure sufficient turnaround time between transmissions.    A GTS is defined by the start time and the duration as specified in the CTA.  Guard time is the time between the end of one GTS and the start of the next GTS.  Including SIFS as part of GTS slot assignments and allocating guard time between GTS slots ensures that transmissions are spaced by at least a SIFS.  Figure 6 is an illustration of how guard time must be allocated such that transmissions are separated by at least a SIFS if the owners of adjacent slots drift towards the other slot.

Figure 6 Guard time

The required guard time depends on the maximum drift between a DEV’s local time and the ideal time. This drift is a function of the time elapsed since a synchronizing reference event.   In an 802.15.3 piconet, the synchronizing event is the start of the preamble of a beacon. The maximum drift, MaxDrift, is calculated as follows:

MaxDrift=Clock accuracy (ppm)1,000,000 * interval duration (μsec).

Propagation delay will also affect timing uncertainty, but in a piconet, the 10 meter range limits propagation delay to around 33ns, or even 66 ns for devices 20 meters apart at opposite ends of a piconet.  This is much lower than the resolution of the GTS slot timing and can be ignored when calculating guard time.

The PNC shall have the option to calculate a single worst case guard time for all slots in the superframe, or it may calculate and assign guard time based on the type of slot (dynamic or pseudo-static) and the position of the slot in the superframe.

Pseudo-static slots require longer guard times than dynamic slots because pseudo-static slots allow transmission when up to aMaxLostBeacons are missed by the transmitting DEV.  Guard times are calculated based on the worst-case drift in a superframe and the maximum allowed number of lost beacons for each of the adjacent slots.  Guard time may be calculated by the PNC as follows:

GuardTime=(MaxLostBeaconsslotn+MaxLostBeaconsslotn+1+2)∗MaxDrift+SIFS

MaxLostBeacons for each slot depends on whether the slot is pseudo-static or dynamic.  MaxLostBeacons is zero for a dynamic slot and is equal to aMaxLostBeacons if the slot is pseudo-static.  The PNC calculates the MaxDrift using the superframe duration and the clock accuracy, aPHYClockAccuracy (see table 68).  The PNC then calculates the start time and duration of each GTS so that there is sufficient guard time between the end of one GTS and the start of the next GTS.

Channel time requests shall allow sufficient time for one SIFS per frame and one per ACK if used.  A DEV transmitting in a GTS starts transmission of the preamble for the first frame at the point where it calculates the start of the GTS slot to be based on its local clock.  In the case of no-ACK or delayed-ACK, the transmitting DEV shall ensure that there is enough time remaining in the slot to transmit the frame and allow for a SIFS before the end of the slot as calculated by that DEV, as shown in Figure 7.

Figure 7 SIFS and Guard time at the end of a GTS - No ACK

If immediate ACK is used, the transmitting DEV shall also ensure there is enough time for the ACK and another SIFS as shown in Figure 8.

Figure 8 SIFS, ACK, SIFS and Guard time at the end of a GTS – Immediate ACK

If implied ACK is used the responding DEV shall also ensure there is enough time for a SIFS, the immediate ACK and another SIFS as shown in Figure 9.

Figure 9 End of GTS timing - Implied ACK

As with any slot, the PNC shall include sufficient guard time between the last GTS in the superframe and the beacon as shown in  Figure 10.

Figure 10 Guard time at the end of the Superframe

The PNC shall begin transmission of the beacon preamble at the point in time that it calculates the start of the superframe to be based on its local clock.   All DEVs will resynchronize their clock based on the beacon arrival.  A DEV transmitting in the CAP shall start transmitting no sooner than the end of the beacon + SIFS.  If the Beacon is fragmented, the CAP does not begin until after the beacon with the frag end bit set.

Because the clock in one DEV may be fast and another may be slow relative to the ideal time, a DEV that is expecting to receive either the beacon or a frame during the CAP or in a GTS slot shall begin receiving before the time that it calculates to be the start of the beacon, CAP or GTS slot to be and shall continue receiving after the time that it calculates to be within one SIFS of the end of the slot.  The amount of time that the DEV listens before the start of the slot and at the end of the slot is up to the implementer.  It may be calculated based on the type of slot, the superframe duration and aPHYClockAccuracy.  The DEV shall be able to receive a frame that is transmitted within the bounds of allowable transmission for the GTS, including worst-case drift.

Addition to Table 68:

aPHYClockAccuracy       PHY dependent, defined in 11.5.5 for the 2.4 GHz PHY

## 7.4.2 (CAP End Time)

Currently there is no straightforward way to specify guard time for the CAP.  In order to do this, we need to replace CFP Duration with CAP End Time.  Put CAP end time into piconet synchronization IE.  Replace CFP Duration, which is not needed.

Remove the description of CFP duration in lines 47-50 and replace it with:

The CAP end time specifies the end of the CAP interval in the superframe. The resolution of this field is 1µs, which gives a range of [0-65535] µs.   CAP end time is used instead of CAP duration because the end of the CAP is static, while the CAP duration can fluctuate as the beacon size changes.  The CAP begins a SIFS after the end of the beacon and continues until to the CAP end time.

## Figure 19

Replace CFP Duration with CAP End Time.

# Slot Duration added to CTA (Comment 1488, 1492, 502, 505)

Background: Originally there was a duration field in the CTAs.  Slot Duration was removed to make the CTAs smaller.  However, due to several factors it is necessary to add slot duration back into the CTA.  One reason is that the PNC may allocate guard time on a slot-by-slot basis, depending on the location and whether the slot is static or pseudo static.  Another reason is that without duration, the end of one slot is determined by the start of the next.  Therefore, slots need to be continuously packed against each other in order to keep the slot size from growing.  Otherwise, the PNC would need to insert Dummy CTAs whose start time would mark the end of the previous GTS.

## Figure 30.

Remove CTA control field due to changes in the EPS agreed to in Dallas and modify figure 30 as follows:

| Octets: 1 | 1 | 1 | 2 | 2 |
| --- | --- | --- | --- | --- |
| Source DEV ID | Destination DEV ID | Stream Index | Slot Start Time | Duration |

## 7.4.10

Remove the following text (page 107, line 46)

The end of each GTS slot is the start time of the next GTS slot minus the guard time indicated in the beacon.

Add the following Text:

The duration field specifies the duration of the GTS.  The resolution of this field is 1 s, so the valid range is [0-65535]s.   The end time of a GTS is the start time of the GTS plus the duration.

# Delayed ACK  (Comment  1464)

A mechanism is needed to allow the sender and receiver of a delayed ACK stream to negotiate the buffer size for the stream.

## 7.5.9.3 Delayed ACK Setup Request

The Delayed ACK Setup Request command is used by the source of a stream to request the use of the delayed ACK mode for the stream or to renegotiate the Stream Buffer size if the fragment size changes.

| Octets: 2 | 2 | 1 | 2 |
| --- | --- | --- | --- |
| Command Type | Length | Stream Index | Fragment Size |

Figure xx. Delayed ACK Setup Response Command

The Stream Index field contains the index of the stream for which delayed ACK policy is being requested.

The Fragment Size Field contains the fragmentation size for the stream in octets.  .

## 7.5.9.4 Delayed ACK Setup Response

The Delayed ACK Setup Response Command is used to respond to a Delayed ACK Setup Request.

| Octets: 2 | 2 | 1 | 2 |
| --- | --- | --- | --- |
| Command Type | Length | Stream Index | Rx Stream Buffer Size |

Figure xx. Delayed ACK Setup Response Command

The Rx Stream Buffer Size field contains the integer number of sequential fragments that the receiver of the stream is able to process.  This is calculated based on the fragment size specified in the delayed ACK Setup Request Command and limitations of the receiver.  If the RxStream Buffer Size is equal to one, the target of the stream is rejecting delayed ACK mode and the source of the stream shall only use No ACK or Immediate ACK policies.

## Clause 8.8.3 Delayed Acknowledgement

Delayed ACK shall only be used for transmitting stream traffic.  Before using the Delayed ACK mode, the source of the stream shall send a Delayed ACK Setup Request command to the target of the stream.  The target of the stream shall respond with a Delayed ACK Setup Response command.

The intended recipient of the directed data frames is allowed to target of the stream shall group the acknowledgement indications into a delayed-ACK command described in 7.5.9.1. The negotiated retransmission window for the stream and the available transmission opportunities for the intended recipient of the stream govern the frequency of the delayed-ACK command.  The intended recipient of the stream is free to send the delayed-ACK command anytime before the expiration of the retransmission window. However the recipient of the stream shall also send the delayed-ACK command at the expiration of the retransmission window. In addition to this,   The source DEV for the stream shall solicit the delayed-ACK command by setting the Del-ACK request bit in the FC of any frame of a stream that is employing the Del-ACK policy.  The source of the stream shall wait aPHYACK time before transmitting another frame in the GTS.

# Support for Asynchronous Traffic (Comment 1628)

Add a More Data bit to the header.

## 7.2.1 Frame Control Field

Replace bit 13 (reserved) with More Data in Figure 12, Frame Control Field

## 7.2.1.11 More Data Field

The more data field is one bit in length and shall be set to one in frames when the sender has more data to transmit in the current GTS slot.

## 8.4.3.1Guaranteed Time Slots

Add the following text:

If a DEV has more frames to transmit in the current GTS, it shall set the More Data field in the frame control field of the header to one.  If the transmitting DEV has no more frames to transmit in the current GTS, it shall set the More Data bit to zero.  If a DEV receives a frame addressed to it with the More Data field set to one, it shall continue receiving in the GTS slot, after sending an ACK if appropriate, because the owner of the GTS has more frames to send.  If the More Data field is set to zero, the receiving DEV may save power by not listing to the rest of the GTS slot.  In order to allow for the possibility of retransmissions, the receiving DEV should listen for the start of a frame before halting reception.