IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | SG3a Technical Requirements |  |
| Date Submitted | [11 March, 2000] |  |
| Source | [Kai Siwiak] [Time Domain Corporation] [7057 Old Madison Pike, Huntsville, AL 35806] | Voice: [256-990-9062] Fax: [256-922-0387] E-mail: [kai.siwiak@timedomain.com] |
| Re: |  |  |
| Abstract | [Definitions for the proposal evaluation for Study Group 3a] |  |
| Purpose | [This is a working document that will become the repository for the terms and definitions to be used in the selection process for a Draft Standard for TG3.] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Table of Contents

1.0 	PHY Layer Technical Requirements	3

2.0 	Data Throughput and  Range	3

3.0 	Data Rate and Capacity and Addressing	4

4.0 	Coexistence, Interference Resistance, Multipath	4

5.0 	Channel Model	5

6.0 	Power Consumption	5

7.0 	Quality of Service	5

8.0 	Form Factor	6

9.0 	Cost	6

# 1.0 	PHY Layer Technical Requirements

Physical layer requirements for the Alternate PHY layer are often combinations: High data rates, but only at low ranges, lower rates at higher ranges; low power consumption, but only with battery use; high capacity, in a shared sense.

The requirements reflect a PHY layer that is scalable across multiple dimensions including range, throughput, shared capacity, size, and power consumption. There are additional requirements in Quality of Service.

# 2.0 	Data Throughput and  Range

A data rate of at least 110 Mbps at 10 meters is required.  The packet error rate used for this requirement is 10% for 512 byte packets.  The assumption is the transmit EIRP is fixed by emission limits and the receiver antenna has a 0 dBi gain. Higher data rates are desired at shorter ranges. Data rates are “information rate” at the PMD-SAP.

An additional higher data rate of 200 Mbps at 4 meters is required and scalability to rates in excess of 480 Mbps is desirable at reduced range.

{Editor’s Note}: we are waiting for an applications table here.

| Requirement | Source |
| --- | --- |
| 100 / 200 / 400 and 480 Mb/s up to 4.5 m range | 02/031r0 |
| 20 Mb/s to 10 m range | 00/075r0 |
| 60 Mb/s, 90, >100 Mb/s (kiosk) | 02/043r0 |
| 2-25 Mb/s; 31-63 Mb/s 10 m and 30 m | 02/047r0 |
| 6-32 Mb/s; 15-50 Mb/s; 20-70 Mb/s; 30-100 Mb/s 10 m or 30-50 m range | 02/119r0 |
| 2 Mb/s to 300 Mb/s: scalable >10 m range to 3 m range | 02/137r1 |
| 1.5 Mb/s, 12 Mb/s, 480 Mb/s to < 5 m range; 10 -1000 Mb/s desired | 02/139r0 |
| 50 Mb/s – 500 Mb/s 1 m on body to 5 m ranges: scalable – very low bit rate to high bit rate | 02/143r0 |

# 3.0 	Aggregate Data Rates and Capacity and Addressing

The data rate of 110 Mbps at 10 meters, as indicated in section 2.0, shall be achievable on each piconet for operation with at least 4 non-overlapping piconets.  The ability to support a larger number of simultaneous over-lapping piconets is desirable even with some degree of performance degradation.  The proposer shall specify the degree of degradation.

The definition of non-overlapping piconets is one or more piconets where there are no interfering devices directly between any DEV in the piconet, including the PNC.  This is illustrated below.

An overlapping piconet has DEVs from two or more piconets “embedded” within each other as shown below.

| Requirement | Source |
| --- | --- |
| 3 non-overlap channels, min effective throughput 50 Mb/s each | 02/031r0 |
| multiple simultaneous channels | 02/119r0 |
| 4 to 8 non-overlap channels, to 300 Mb/s shared | 02/137r1 |
| addressing of <127 devices per host | 02/139r0 |
| Large capacity, many devices (private addresses) | 02/143r0 |

# 4.0 	Coexistence, Interference Resistance

The proposer shall show the level of coexistence with current 802 devices, cellular, PCS, GPS and 802.15.1 within one foot separation from the device.  (Analysis with 802.11 DSSS shall suffice for 802.11b, 802.11g and 802.15.3).  Coexistence with more than one device at a time is a plus.  The proposer should provide data to show the level of coexistence with these devices.  Analysis should consider both ingress and egress scenarios.

An example of coexistence with more than one device at a time is operating the alternate PHY within one foot of an 802.11 radio and a cellular radio at the same time.

| Requirement | Source |
| --- | --- |
| Coexistence with other 802, BlueTooth, Cellular, PCS, GPS, 2.4 GHz and 5 GHz devices | 02/043r0, 02/137r1, 02/139r0, 02/143r0 |
| Operates within 1 m of 802, BlueTooth, Cellular, PCS and GPS | 02/119r0 |
| Must work with incumbent networks | 02/137r1 |
| Must support combinations of network technologies | 02/137r1 |
| Shall be capable of (802.15.3) 22 Mb/s QPSK mode | 02/102r0 |
| Robust in multipath (indoor) | 02/143r0 |

# 5.0 	Channel Model

{Ed. note}: Jeff Foerster to provide text for a call for channel models ... we can use 802.11 channel models for 2.4 GHz and 5 GHz but we need UWB and other models (i.e. 20 GHz if applicable).  Matt to suggest text for section 5.0 and submit it for comments before next week.

| Requirement | Source |
| --- | --- |
| Wide band channel model (to be defined) | 02/137r1 |
| JTC residential A | 02/031r0 |

# {Ed. note: sections 6-9 were previously 7-10; #6 omitted}

# 6.0 	Power Consumption

| Requirement | Source |
| --- | --- |
| 350-450 mW on CamCorder; 150-200 mW Still Digital camera; <100 mW in PDA; 90mW in (toy) wireless robot control | 02/043r0 |
| Possible on battery operated devices | 02/047r0, 02/143r0 |
| Low power and advanced power management; low power density | 02/102r0 |
| 40-80 mW battery power; and 150-500 mW mains | 02/119r0 |
| Power management | 02/139r0 |

# 7.0 	Quality of Service

| Requirement | Source |
| --- | --- |
| “Best remedy” for QoS | 02/043r0 |
| 10-9 BER: video; isochronous capable | 02/047r0 |
| 10-12 BER: USB-LL | 02/139r0 |
| Secure communications; privacy in PAN | 02/143r0 |
| High quality audio / video at high bit rate | 02/147r0 |

# 8.0 	Form Factor

| Requirement | Source |
| --- | --- |
| Not a factor in some applications | . |
| easy to integrate into consumer devices, especially portable | 02/119r0 |
| memory stick, SD card, Media Card compatible size | 02/102r0 |
| 5cm x 2.15cm x 0.28 cm without antenna but with MAC: cameras | 02/043r0 |

# 9.0 	Cost

| Requirement | Source |
| --- | --- |
| Cost and BOM similar to BlueTooth | 02/119r0, 02/043r0, 02/031r0 |