IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | SG3a Technical Requirements |  |
| Date Submitted | [11 March, 2000] |  |
| Source | [Kai Siwiak] [Time Domain Corporation] [7057 Old Madison Pike, Huntsville, AL 35806] | Voice: [256-990-9062] Fax: [256-922-0387] E-mail: [kai.siwiak@timedomain.com] |
| Re: |  |  |
| Abstract | [Definitions for the proposal evaluation for Study Group 3a] |  |
| Purpose | [This is a working document that will become the repository for the terms and definitions to be used in the selection process for a Draft Standard for TG3.] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Table of Contents

1.0 	PHY Layer Technical Requirements	3

2.0 	Data Throughput and  Range	3

3.0 	Aggregate Data Rates and Capacity and Addressing	4

4.0 	Coexistence, Interference Resistance	4

5.0 	Channel Model	5

6.0 	Power Consumption	5

7.0 	Quality of Service	6

8.0 	Form Factor	6

9.0 	Cost	7

# 1.0 	PHY Layer Technical Requirements

Physical layer requirements for the Alternate PHY layer are often combinations: High data rates, but only at low ranges, lower rates at higher ranges; low power consumption, but only with battery use; high capacity, in a shared sense.

The requirements reflect a PHY layer that is scalable across multiple dimensions including range, throughput, shared capacity, size, and power consumption. There are additional requirements in Quality of Service.

# 2.0 	Data Throughput and  Range

A data rate of at least 110 Mbps at 10 meters is required.  The packet error rate used for this requirement is 8% for 1024 octet frame body.  The assumption is the transmit EIRP is fixed by emission limits and the receiver antenna has a 0 dBi gain. Higher data rates are desired at shorter ranges. Data rates are “information rate” at the PMD-SAP.

An additional higher data rate of 200 Mbps at 4 meters is required and scalability to rates in excess of 480 Mbps is desirable at reduced range.

{Editor’s Note}: we are waiting for an applications table here.

| Requirement | Source |
| --- | --- |
| 100 / 200 / 400 and 480 Mb/s up to 4.5 m range | 02/031r0 |
| 20 Mb/s to 10 m range | 00/075r0 |
| 60 Mb/s, 90, >100 Mb/s (kiosk) | 02/043r0 |
| 2-25 Mb/s; 31-63 Mb/s 10 m and 30 m | 02/047r0 |
| 6-32 Mb/s; 15-50 Mb/s; 20-70 Mb/s; 30-100 Mb/s 10 m or 30-50 m range | 02/119r0 |
| 2 Mb/s to 300 Mb/s: scalable >10 m range to 3 m range | 02/137r1 |
| 1.5 Mb/s, 12 Mb/s, 480 Mb/s to < 5 m range; 10 -1000 Mb/s desired | 02/139r0 |
| 50 Mb/s – 500 Mb/s 1 m on body to 5 m ranges: scalable – very low bit rate to high bit rate | 02/143r0 |

# 3.0 	Aggregate Data Rates and Capacity and Addressing

The data rate of 110 Mbps at 10 meters, as indicated in section 2.0, shall be achievable on each piconet for operation with at least 4 non-overlapping piconets.  The ability to support a larger number of simultaneous over-lapping piconets is desirable even with some degree of performance degradation.  The proposer shall specify the degree of degradation.

The definition of non-overlapping piconets is one or more piconets where there are no interfering devices directly between any DEV in the piconet, including the PNC.  This is illustrated below.

An overlapping piconet has DEVs from two or more piconets “embedded” within each other as shown below.

| Requirement | Source |
| --- | --- |
| 3 non-overlap channels, min effective throughput 50 Mb/s each | 02/031r0 |
| multiple simultaneous channels | 02/119r0 |
| 4 to 8 non-overlap channels, to 300 Mb/s shared | 02/137r1 |
| addressing of <127 devices per host | 02/139r0 |
| Large capacity, many devices (private addresses) | 02/143r0 |

4.0 	Coexistence, Interference Resistance

The proposer shall show the level of coexistence with current 802 devices, cellular, PCS, GPS and 802.15.1 within 0.3 m antenna separation between devices.  (Analysis with 802.11 DSSS shall suffice for 802.11b, 802.11g and 802.15.3).  Coexistence with more than one device at a time is a plus.  The proposer should provide data to show the level of coexistence with these devices.  Analysis should consider both ingress and egress scenarios.

An example of coexistence with more than one device at a time is operation of the alternate PHY with its antenna within 0.3 m of the antennas of an 802.11 radio and a cellular radio at the same time.

| Requirement | Source |
| --- | --- |
| Coexistence with other 802, BlueTooth, Cellular, PCS, GPS, 2.4 GHz and 5 GHz devices | 02/043r0, 02/137r1, 02/139r0, 02/143r0 |
| Operates within 1 m of 802, BlueTooth, Cellular, PCS and GPS | 02/119r0 |
| Must work with incumbent networks | 02/137r1 |
| Must support combinations of network technologies | 02/137r1 |
| Shall be capable of (802.15.3) 22 Mb/s QPSK mode | 02/102r0 |
| Robust in multipath (indoor) | 02/143r0 |

# 5.0 	Channel Model

Performance in a multipath environment is a critical feature for the SG3a ALT PHY. The SG3a study group intends to issue a call for contributions to provide input for the group to select a reference ALT PHY channel model. This selection process will include consideration of models for the 2.4 GHz ISM band, the 5.2 GHz UNII band and for UWB.  It is strongly suggested that individual proposers make use of the SG3a models, as they become available, as a basis for evaluating the performance of proposals. Individuals submitting proposals that are not addressed by the above list of RF channels are encouraged to provide specific details of the channel model used, and should support the use of that model with references to literature, other standards, and/or research.

| Requirement | Source |
| --- | --- |
| Wide band channel model (to be defined) | 02/137r1 |
| JTC residential A | 02/031r0 |

# 6.0 	Power Consumption

The alternate PHY power consumption goal is 100 mW averaged over the duration of an entire packet burst as defined at the PMD SAP for the data rate and error rate given in clause 2.0 of this document. The power averaged over a data burst shall be stated for two conditions: (1) while transmitting data, (2) while receiving data. Power consumption shall also be stated for the case of ‘neither transmitting nor receiving.’ The proposer is encouraged to show power consumption numbers for additional modes of operation that may include combinations of higher/lower data rates and lower error rates.

| Requirement | Source |
| --- | --- |
| 350-450 mW on CamCorder; 150-200 mW Still Digital camera; <100 mW in PDA; 90mW in (toy) wireless robot control | 02/043r0 |
| Possible on battery operated devices | 02/047r0, 02/143r0 |
| Low power and advanced power management; low power density | 02/102r0 |
| 40-80 mW battery power; and 150-500 mW mains | 02/119r0 |
| Power management | 02/139r0 |

# 7.0 	Quality of Service

The uncorrected (i.e., without re-transmissions) error rate as defined in clause 2 is 8% packet error rate. The proposer is encouraged to show techniques that will yield an ‘equivalent’ bit error rate of <10-9 at the PHY SAP. Data should be provided that shows the added complexity, and power consumption, and latency due to any relevant signal processing. In addition, estimates should be given to indicate the PHY overhead due to preambles and PHY headers.

| Requirement | Source |
| --- | --- |
| “Best remedy” for QoS | 02/043r0 |
| 10-9 BER: video; isochronous capable | 02/047r0 |
| 10-12 BER: USB-LL | 02/139r0 |
| Secure communications; privacy in PAN | 02/143r0 |
| High quality audio / video at high bit rate | 02/147r0 |

# 8.0 	Form Factor

The PHY components should be capable of fitting into a form factor consistent with a camera, PDA, NIC cards or other small form factors of hand-held devices. The antenna size and form factor should be consistent with a small form factor without deviating significantly from the original device size.

| Requirement | Source |
| --- | --- |
| Not a factor in some applications | . |
| easy to integrate into consumer devices, especially portable | 02/119r0 |
| memory stick, SD card, Media Card compatible size | 02/102r0 |
| 5cm x 2.15cm x 0.28 cm without antenna but with MAC: cameras | 02/043r0 |

# 9.0 	Cost

Cost/complexity estimates should be provided in terms of die size estimates, semiconductor process requirements, gate count estimates, and major external components. The target complexity is to be stated relative to Bluetooth.

Additionally, it is important to have the possibility of mixing very low complexity devices and very low cost devices that trade off performance with cost.

| Requirement | Source |
| --- | --- |
| Cost and BOM similar to Bluetooth | 02/119r0, 02/043r0, 02/031r0 |