IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | SG3a Technical Requirements |  |
| Date Submitted | [May 3, 2000] |  |
| Source | [Kai Siwiak] [Time Domain Corporation] [7057 Old Madison Pike, Huntsville, AL 35806] | Voice: [256-990-9062] Fax: [256-922-0387] E-mail: [kai.siwiak@timedomain.com] |
| Re: |  |  |
| Abstract | [Definitions for the proposal evaluation for Study Group 3a] |  |
| Purpose | [This is a working document that will become the repository for the terms and definitions to be used in the selection process for a Draft Standard for TG3.] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Table of Contents

Introduction	4

1.0 	PHY Layer Technical Requirements	4

2.0 	Data Throughput and  Range	4

Wireless Peripherals – H.323 / T.120 Video Conferencing	5

Home Theater	6

Gaming	6

Content Downloading	6

3.0 	Aggregate Data Rates and Capacity and Addressing	7

4.0 	Coexistence, Interference Resistance	9

5.0 	Channel Model	9

6.0 	Power Consumption	10

7.0 	Quality of Service	11

8.0 	Form Factor	11

9.0 	Cost	11

10.0 Supplements to 802.15.3 functionality	12

11.0 Regulatory	12

# Introduction

This document and the Selection Criteria 02/105 provide the technical content for the project to develop an alternate physical layer (alt-PHY). This alt-PHY shall be a supplement to the nearly complete 802.15.3 standard. This document serves two purposes. It is first a summary of the 8 application presentations from the Study Group 3a call for applications. Second it defines the fundamental requirements implied by applications but not necessarily stated explicitly. The selection criteria document, 02/105 will guide proposers on how to respond to a call for proposals. It also provides amplification on key areas not addressed in 02/104. The study group may update this document as discoveries are made while working on the 02/105 document.

# 1.0 	PHY Layer Technical Requirements

Physical layer requirements for the alt-PHY layer are: high data rates, but only at low ranges, lower rates at higher ranges; low power consumption; and high capacity in a shared sense.

The requirements reflect a PHY layer that is scalable across multiple dimensions including range, throughput, shared capacity, size, and power consumption. There are additional requirements in Quality of Service.

# 2.0 	Data Throughput and  Range

A data rate of at least 110 Mbps at 10 meters is required.  The packet error rate used for this requirement is 8% for 1024 octet frame body.  The transmit EIRP is fixed by regulatory emission limits and the receiver antenna has a 0 dBi gain. Higher data rates are desired at shorter ranges. Data rates are “information rate” at the PMD-SAP.  Figure 1 shows the IEEE802.15 partitioning.

Figure 1 – IEEE802.15 Partitioning.

An additional higher data rate of 200 Mbps at 4 meters is required and scalability to rates in excess of 480 Mbps is desirable at reduced range.

[The following applications tables are sourced from: 02/170r2]

## Wireless Peripherals – H.323 / T.120 Video Conferencing

This application involves a set of wireless peripherals (at range <= 4m) being used for a H.323 videoconferencing session (e.g., NetMeeting) using video, audio, and application sharing over a broadband connection or a corporate Intranet Ethernet connection.

| Application Ingredients | Data Format | Min Data Rate | Max Data Rate |
| --- | --- | --- | --- |
| Video Camera | MPEG2 resolution w/o compression | 75Mbps | 150Mbps |
| Monitor | Varying compression | 63Mbps | 1Gbps |
| H.323 comm (broadband) | H.261/H.263, G.711/G.723, T.120 | 128kbps | 10Mbps |
| Audio (mono) | PCM | 64kbps | 64kbps |
| Mass storage |  | 50Mbps | 240 Mbps |
| Mouse |  | < 9600 bps | < 9600 bps |
| Keyboard |  | < 9600 bps | < 9600 bps |
| Total |  | 188+ Mbps | 1.4+ Gbps |

## Home Theater

This application involves a set of wireless consumer electronics devices (at range <= 10 m) that might be used in a home theater room.  The key focus of this collection of devices is the video and audio signals.  A wireless web pad would allow the user to explore details about the programs being viewed using an Internet connection.

| Application Ingredients | Data Format | Min Data Rate | Max Data Rate |
| --- | --- | --- | --- |
| Monitor/Projector | HD Video | 19.2 Mbps | 19.2 Mbps |
| Audio (speakers) | 5.1 to 10.2 channels (24 bits) | 13.8 Mbps | 27.6 Mbps |
| Web Pad | Internet (streaming media) | 10 Mbps | 10 Mbps |
| Remote Controller |  | ?? | ?? |
| Total |  | 43 Mbps | 56.8 Mbps |

## Gaming

{Ed. Note, FCC regulations may inhibit the use of a UWB alt-PHY in games and toys}

This application has significant market appeal in that wireless links will not only improve the ease of setup/positioning and storage, but they will also promote safer operation (fewer wires to trip over).  The requirements are listed here in order of priority, with the display and audio transmission having a higher priority than the game controllers.

| Application Ingredients | Data Format | Min Data Rate | Max Data Rate |
| --- | --- | --- | --- |
| Monitor/Projector | XGA w/ Varying compression | 63 Mbps | 1 Gbps |
| Audio (speakers) | 5.1 to 10.2 channels (24 bits) | 13.8 Mbps | 27.6 Mbps |
| Game Pads | (Reaction Sensitive) | ?? | ?? |
| Total |  |  |  |

## Content Downloading

These applications concentrate on reducing the user’s waiting time when downloading large loads of information, such as video, audio and digital images.  The transfer time indicates the amount of time the user spends waiting for a typical data transfer to complete.

| Content Downloaded | Data Format | Data Rate | Transfer Time |
| --- | --- | --- | --- |
| Audio | MP3 | 60 Mbps | 10 sec |
|  | CD | 90 Mbps | 60 sec |
| Photos | Varied Compression | 90 Mbps | 10 sec |
| Movies | DV | ?? (higher) | ?? |
| Total |  |  |  |

These are minimum download rates for an initial implementation.  Consumer desire is always that faster downloads are better and thus significant importance is attached to the ability of the proposed technology to significantly increase download rates in the future.

| Requirement | Source |
| --- | --- |
| 100 / 200 / 400 and 480 Mb/s up to 4.5 m range | 02/031r0 |
| 20 Mb/s to 10 m range | 00/075r0 |
| 60 Mb/s, 90, >100 Mb/s (kiosk) | 02/043r0 |
| 2-25 Mb/s; 31-63 Mb/s 10 m and 30 m | 02/047r0 |
| 6-32 Mb/s; 15-50 Mb/s; 20-70 Mb/s; 30-100 Mb/s 10 m or 30-50 m range | 02/119r0 |
| 2 Mb/s to 300 Mb/s: scalable >10 m range to 3 m range | 02/137r1 |
| 1.5 Mb/s, 12 Mb/s, 480 Mb/s to < 5 m range; 10 -1000 Mb/s desired | 02/139r0 |
| 50 Mb/s – 500 Mb/s 1 m on body to 5 m ranges: scalable – very low bit rate to high bit rate | 02/143r0 |

# 3.0 	Aggregate Data Rates and Capacity and Addressing

The data rate and range specified in section 2.0 shall be achievable on each piconet for operation with at least 3 other non-overlapping piconets.  The ability to support multiple simultaneous uncoordinated (multiple independent parent piconets) over-lapping piconets is desirable even with some degree of performance degradation.  The proposer shall specify the degree of degradation.

A non-overlapping piconet is defifed as a piconet in which the useable RF footprint of each of the members does not overlap the useable RF footprint of any member of an adjacent piconet.  This is illustrated below.

An overlapping piconet is one in which the useable RF footprint of at least one of the members of the piconet overlaps the useable RF footprint of at least one member of another piconet as shown below.

| Requirement | Source |
| --- | --- |
| 3 non-overlap channels, min effective throughput 50 Mb/s each | 02/031r0 |
| multiple simultaneous channels | 02/119r0 |
| 4 to 8 non-overlap channels, to 300 Mb/s shared | 02/137r1 |
| addressing of <127 devices per host | 02/139r0 |
| Large capacity, many devices (private addresses) | 02/143r0 |

# 4.0 	Coexistence, Interference Resistance

The proposer shall show the level of coexistence with current 802 devices, cellular, PCS, GPS and 802.15.1 within a distance of Y m antenna separation between devices.  (Analysis with 802.11 DSSS shall suffice for 802.11b, 802.11g and 802.15.3).  Coexistence with more than one device at a time is a plus.  The analysis should consider the susceptibility of harmful interference from and to alt-PHY devices. Distance Y is 1 m, however proposers are encouraged to show their performance at shorter distances such as with Y= 30 cm and Y = 8 cm.

| Requirement | Source |
| --- | --- |
| Coexistence with other 802, BlueTooth, Cellular, PCS, GPS, 2.4 GHz and 5 GHz devices | 02/043r0, 02/137r1, 02/139r0, 02/143r0 |
| Operates within 1 m of 802, BlueTooth, Cellular, PCS and GPS | 02/119r0 |
| Must work with incumbent networks | 02/137r1 |
| Must support combinations of network technologies | 02/137r1 |
| Shall be capable of (802.15.3) 22 Mb/s QPSK mode | 02/102r0 |
| Robust in multipath (indoor) | 02/143r0 |

# 5.0 	Channel Model

Performance in a multipath environment is a critical feature for the SG3a alt-PHY. The SG3a study group will issue a call to provide a reference alt-PHY channel model. The intention is to use existing narrow band channel models (such as the 802.11 model) for non-UWB alt-PHY proposals. The individual proposers make use of the SG3a models as they become available as a basis for evaluating the performance of their proposals. Individuals submitting proposals that are not addressed by the RF channels selected by the Study Group are encouraged to provide specific details of the channel model used with references to literature, use in other standards, and/or research.

| Requirement | Source |
| --- | --- |
| Wide band channel model (to be defined) | 02/137r1 |
| JTC residential A | 02/031r0 |

# 6.0 	Power Consumption

The alt-PHY power consumption goal is 100 mW averaged over the duration of an entire packet burst as defined at the PMD SAP for the data rate and error rate given in clause 2.0 of this document. The power averaged over a data burst shall be stated for two conditions: (1) while transmitting data, (2) while receiving data. Power consumption shall also be stated for the case of ‘neither transmitting nor receiving.’ The proposer is encouraged to show power consumption numbers for additional modes of operation that may include combinations of higher/lower data rates and lower error rates.

| Requirement | Source |
| --- | --- |
| 350-450 mW on CamCorder; 150-200 mW Still Digital camera; <100 mW in PDA; 90mW in (toy) wireless robot control | 02/043r0 |
| Possible on battery operated devices | 02/047r0, 02/143r0 |
| Low power and advanced power management; low power density | 02/102r0 |
| 40-80 mW battery power; and 150-500 mW mains | 02/119r0 |
| Power management | 02/139r0 |

# 7.0 	Quality of Service

The uncorrected (i.e., without re-transmissions) error rate as defined in clause 2 is 8% packet error rate. The proposer is encouraged to show techniques that will yield an ‘equivalent’ bit error rate of <10-9 at the PHY SAP. Data should be provided that shows the added complexity, and power consumption, and latency due to any relevant signal processing. In addition, estimates should be given to indicate the PHY overhead due to preambles and PHY headers.

| Requirement | Source |
| --- | --- |
| “Best remedy” for QoS | 02/043r0 |
| 10-9 BER: video; isochronous capable | 02/047r0 |
| 10-12 BER: USB-LL | 02/139r0 |
| Secure communications; privacy in PAN | 02/143r0 |
| High quality audio / video at high bit rate | 02/147r0 |

# 8.0 	Form Factor

The PHY components should be capable of fitting into a form factor consistent with a camera, PDA, NIC cards or other small form factors of hand-held devices. The antenna size and form factor should be consistent with a small form factor without deviating significantly from the original device size.

| Requirement | Source |
| --- | --- |
| Not a factor in some applications | . |
| easy to integrate into consumer devices, especially portable | 02/119r0 |
| memory stick, SD card, Media Card compatible size | 02/102r0 |
| 5cm x 2.15cm x 0.28 cm without antenna but with MAC: cameras | 02/043r0 |

# 9.0 	Cost

Cost/complexity estimates should be provided in terms of die size estimates, semiconductor process requirements, gate count estimates, and major external components. The target complexity is to be stated relative to Bluetooth. Additionally, it is important to have the possibility of mixing very low complexity devices and very low cost devices that trade off performance with cost.

| Requirement | Source |
| --- | --- |
| Cost and BOM similar to Bluetooth | 02/119r0, 02/043r0, 02/031r0 |

# 10.0 Supplements to 802.15.3 functionality

It is envisioned that the alt-PHY project will allow for specific supplements to 802.15.3. In addition to the PHY itself, the project may include MAC functionality necessary to support the selected PHY.

It is required that the alt-PHY operate with the 802.15.3 MAC without modification. It is also required that any additional capabilities will still allow the

defined PHY in 802.15.3 to operate in the presence of supplements introduced with the alt-PHY project.

As an example, from the project request, document 01/532r0 (alt-PHY study group proposal), location awareness is considered as a possible component of certain alt-PHY technologies and is an example of supplemental type of functionality. Such a provision may be supported by the PHY PIB parameters in the current 802.15.3 standard or via supplemental MAC capabilities provided by the alt-PHY project.

# 11.0 Regulatory

It is a requirement that a resulting alt-PHY standard comply with the regulatory requirements of the regions for which the standard addresses. Proposers are required to address known regulatory requirements.