IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Throughput-range-requirement-application-table |  |
| Date Submitted | [27 Mar, 2002”] |  |
| Source | [Chuck Brabenac] [Intel] [2111 NE 25th Ave, Hillsboro OR 97124] | Voice: [503-264-8575 ] Fax: [503-264-3483 ] E-mail: [chuck.brabenac@intel.com ] |
| Re: | [02104r2P802-15_SG3a-Technical-Requirements.doc] |  |
| Abstract | [This contribution proposes the addition of application tables to be included in 02104r2P802-15_SG3a-Technical-Requirements, that supports stated throughput and range requirements. It includes one completed application scenario table, and provides a framework for others to be added] |  |
| Purpose | [Use this as a starting point for completing application driven tables to support stated PHY throughput and range requirements.] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

## Wireless Peripherals – H.323 / T.120 Video Conferencing

This application involves a set of wireless peripherals (at range <= 4m) being used for a H.323 videoconferencing session (e.g., NetMeeting) using video, audio, and application sharing over a broadband connection or a corporate Intranet Ethernet connection.

| Application Ingredients | Data Format | Min Data Rate | Max Data Rate |
| --- | --- | --- | --- |
| Video Camera | MPEG2 resolution w/o compression | 75Mbps | 150Mbps |
| Monitor | Varying compression | 63Mbps | 1Gbps |
| H.323 comm (broadband) | H.261/H.263, G.711/G.723, T.120 | 128kbps | 10Mbps |
| Audio (mono) | PCM | 64kbps | 64kbps |
| Mass storage |  | 50Mbps | 240 Mbps |
| Mouse |  | < 9600 bps | < 9600 bps |
| Keyboard |  | < 9600 bps | < 9600 bps |
| Total |  | 188+ Mbps | 1.4+ Gbps |

## Home Theater

(Description to go here.  Intent is to show an application that would run at range <= 10m).

| Application Ingredients | Data Format | Min Data Rate | Max Data Rate |
| --- | --- | --- | --- |
| Monitor | HD Video |  |  |
| Audio (mono) | Dolby 5.1 |  |  |
| Total |  |  |  |

## Gaming

<Description to go here>.

| Application Ingredients | Data Format | Min Data Rate | Max Data Rate |
| --- | --- | --- | --- |
| Total |  |  |  |