IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | IEEE P802-15_TG3 Security Text Recommendations Rationale |  |
| Date Submitted | [April 26, 2002] |  |
| Source | [Daniel V. Bailey, Ari Singer, William Whyte] [NTRU] [5 Burlington Woods Burlington, MA 01803 USA] | Voice: [+1 781 418-2522] Fax: [+1 781 418-2532] E-mail: [dbailey@ntru.com] |
| Re: | 802.15.3 TG3 Letter Ballot Draft D09, 02074r1P802.15_TG3-Security-CFP.doc, 02130r1P802-15_TG3-NTRU-Security-Architecture-Proposal.doc |  |
| Abstract | [This document offers the rationale for the recommended text for the ECC security algorithm suite for the 802.15.3 draft standard defined in 02171r0P802-15_TG3-Security-Text-Recommendations.doc and 02210r0P802-15_TG3-NTRU-Full-Security-Text-Proposal.pdf. The ECC suite performance and security is analyzed.] |  |
| Purpose | [This document offers guidance to the 802.15.3 working group on assessing the suite in 02/171r0.] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Table of Contents

1	Executive Summary	2

1.1	Summary	2

1.2	Changes from 173r0	2

2	Overview	3

3	Efficiency Analysis	5

3.1	Summary	5

3.2	Implementation Choices	6

3.2.1	Choice of Curve	6

3.2.2	Omission of Point Compression	6

3.3	Detailed Analysis	6

4	Security Considerations	10

4.1	General	10

4.2	Key Validation in ECIES: Reasons for Omission	11

4.2.1	What is Key Validation?	11

4.2.2	How is Key Validation Performed in X9.63?	11

4.2.3	Is Key Validation Required in X9.63?	12

4.2.4	Is Key Validation Required in Other Standards?	12

4.2.5	What are the Security Risks if Key Validation is Not Performed?	12

4.2.6	Is Key Validation Subject To Patents?	13

4.2.7	In Conclusion:	14

5	Review of Protocols	14

5.1	ECIES Protocol	14

5.2	MQV Protocol	15

6	References	16

# Executive Summary

## Summary

The document [02210] proposes an ANSI X9.63-based Elliptic Curve Cryptography authentication protocol for 802.15.3. This protocol appears to be patent unencumbered, in accordance with the unanimous vote of the Working Group. The protocol presented has performance characteristics similar to the MQV protocol proposed in [02200], and appropriate security characteristics for the intended use. If adopted, it will be possible for 802.15.3 device manufacturers to write their own cryptographic toolkits, or purchase toolkits from one of multiple competing vendors. The protocol is based on ANSI standard X9.63-2001 [X963] and provides 128-bit security, as mandated by the Working Group.

This document provides a performance and security analysis of the proposed authentication protocol, comparing it to the MQV-based protocol as specified in [02200].

## Changes from 173r0

- Updated references: 173r0 compared [02171] to [02144], this document compares [02210] to [02200].

- Added discussion (section 4.2) of the decision to make public key validation optional, inline with practice in ISO, IEEE P1363 proposed standards.

- Enlarged discussion (section 3.2.1) of the decision to implement over a prime curve rather than a binary Koblitz curve.

- Added discussion (section 3.2.2) of the decision to omit point compression.

- Removed references to underspecification of point representation in [02144]. [02200] specifies the point representation as over a polynomial basis.

- Updated performance analysis (section 3) to reflect the fact that [02210] does not use point compression.

- Added comment to section to the effect that the presentation of the MQV protocol (section 5.2) may not exactly coincide with the embodiment intended by [02200]. If the two statements of the protocol are not the same we will happily correct this document.

# Overview

On March 14th, 2002, the 802.15.3 working group passed the following motion unanimously, with no abstentions:

- Place complete security suite description based on ANSI X9.63-2001 and NTRUEncrypt in the draft

  - A security suite based on ANSI X9.63-2001 will be mandatory when security modes 2 and 3 are enabled. The goal of the mode selected will be to have no issued worldwide patents.

  - NTRUEncrypt using eess251ep1, 02/131r0, will be optional when security modes 2 and 3 are enabled.

  - Complete security suite text due to technical editor (cc to TG3 chair) in a form similar to 02131r0, for both items by 5PM PST, April 5

  - If complete text specifying an implementation based on ANSI X9.63-2001 security suite is not received and posted to the 802.15.3 email reflector on time, the draft will proceed with the proposal in 02/131r0, as the mandatory security suite

- The security modes are as defined on slide 19, 02/096r4

- Add the ability to use access control lists without security enabled that allow for the selection of a set of devices allowed to associate via user configured means.

The Certicom proposal outlined in [02200] appears highly patent-encumbered and certainly does not meet the goal of having no issued worldwide patents. Specific areas of concern include:

- It uses the MQV protocol to establish keys. This protocol is patented in the US (number 5,761,305 and others).

- It uses implicit certificates. Although these are not subject to any issued patent, Certicom have acknowledged that other parties may be concerned about their patent status.

- It uses point compression, a means of saving bandwidth when transmitting or storing elliptic curve points. Certicom has been assigned US patents number 6,141,420 and 6,199,086, which cover point compression.

An unencumbered protocol must obviously avoid the use of MQV, implicit certificates, point compression over binary fields, and any required use of normal basis representation. Additionally, Certicom has stated [Dec99] that it has patents and patent applications that include the use of the following:

- Methods for efficient implementation of elliptic curve arithmetic over finite fields, including fast methods for calculating inverses.

- Methods for point compression.

- Methods to improve performance of private key operations.

- Versions of the MQV key agreement protocols

- Methods to avoid the small subgroup attack.

- Methods to improve performance of elliptic curve arithmetic; in particular, fast efficient multiplication techniques

- Methods to improve performance of finite field multiplication.

- Methods for efficient implementation of arithmetic modulo n.

- Methods to perform validation of elliptic curve public keys.

- Methods to perform efficient basis conversion.

It is clear that an unencumbered security suite must avoid these as much as possible. Our proposal, in outline, is the following:

- Two-way ECIES as the key agreement method: ECIES [BR97, ABR01] is the Elliptic Curve Integrated Encryption Scheme. The inventors have stated that it has not been patented. It is included in X9.63.

- Use the NIST curve ansip256r1: This curve is defined over a prime field, not over a binary field. This has the following advantages:

  - The order of the curve is the same as the order as any point on the curve. This eliminates small subgroup attacks, and removes the risk of infringing patents that protect against small subgroup attacks.

  - Any point on the curve is a valid public key. This reduces (but does not entirely eliminate) the risk that an implementation will infringe a key validation patent.

  - Efficient implementations on this curve are possible; implementations that do not infringe patents are also possible.

  - Patents on efficient implementations are owned by the NSA, not by any private company.

  - There is only one representation of points on prime curves, so basis conversion is not an issue.

The rest of this note analyses the proposed protocol in more detail.

We provide a comparison of the most efficient implementation of ECIES with the most efficient implementation of MQV over the same curve. The ECIES-based system is comparable in speed to the encumbered MQV system, typically taking only 30% more superframes to complete than MQV operations on the same devices. We additionally compare ECIES to MQV on the curve proposed in [02200]. The comparisons are in software; we make no statement about the relative performances in hardware.

We also analyze the security services provided by both ECIES and MQV and demonstrate that the services provided are equivalent in all cases unless multiple private keys belonging to devices within a single piconet are compromised.

# Efficiency Analysis

This section analyses the efficiency of the ECIES-based protocol described here, and compares it to the efficiency of the MQV-based protocol described in [02200]. We show that the ECIES operation is of comparable efficiency to MQV, completing in about 30% more superframes. The implementations compared are the most efficient ones known, and may be patent encumbered; a patent-unencumbered implementation of this protocol may run more slowly.

## Summary

The following table summarizes the performance characteristics of three authentication protocols. A detailed justification of the figures presented follows in subsequent sections. The authentication protocols are:

- The proposed ECIES-based protocol outlined in [02210] over the curve ansip256r1.

- The MQV protocol outlined in [02200] over the same curve.

- The MQV protocol outlined in [02200] over the curve ansit283k1.

The times presented here are derived from figures in peer-reviewed, published papers co-authored by members of Certicom staff. The times for operations on ansip256r1 were obtained by taking the figures on a 400 MHz Pentium presented in [BHLM01] and scaling down for clock speed. The times for operations on ansit283k1 were obtained by taking the figures on a 400 MHz Pentium presented in [HHM00] and scaling down for clock speed. This table does not use the figures in [02144] because we could not confirm them from any other source.

These figures may not realistically reflect what is achievable in software; hardware acceleration may be necessary to achieve acceptable performance. Additionally, it is possible that the most efficient implementations of the operations are subject to patents. The intent of these figures is to demonstrate that the current proposal performs acceptably compared to MQV.

|  | Number of 65-ms superframes required to complete authentication for: |  |  |
| --- | --- | --- | --- |
| Clock Speed | ECIES over ansip256r1 | MQV over ansip256r1 | MQV over ansit283k1 |
| 30 MHz | 7 | 5 | 6 |
| 13 MHz | 13 | 11 | 12 |
| 2.66 MHz | 59 | 43 | 53 |

## Implementation Choices

### Choice of Curve

The NIST prime curve ansip256r1 was chosen for the following reasons:

- It admits of particularly fast software operations (using patented techniques)

- The order of the point is the order of the curve. This has the following consequences.

  - There is no need for cofactor multiplication to avoid small subgroup attacks. This avoids the use of patented techniques.

  - There is no need to perform key validation (see section 4.2 for more details). This avoids the use of patented techniques.

- There is only one representation of points on a prime curve. This avoids patents based on basis conversion methods.

It is unclear how the performance of operations on this curve in hardware (in gates and cycles per operation) compares to the performance of operations on curves over binary fields, especially binary curves.

### Omission of Point Compression

Point compression is a technique that can be used to save data. An elliptic curve point has the form

y2 = f(x, y).

Therefore, for any given value of x, there are only two possible values of y. Point compression is a process in which the sender of a point represents the y coordinate by a single bit, and the receiver uses the x coordinate and the bit representing y to recover the full value of y.

In the case of ECC over 256-bit primes, point compression saves 32 bytes of representation. This reduces bandwidth for transmission, and also the amount of data that must be input to the hash functions. However, these gains are minor.

Point compression, over both binary and prime fields, is claimed by Certicom in US Patent Number 6,141,420. It is therefore omitted from [02210], in accordance with the motion passed in St Louis.

## Detailed Analysis

This section analyses the efficiency of this protocol, compared to the efficiency of the patented MQV protocol presented in [02200].  The data presented in this section should be considered in the context of comparing the relative efficiency of different algorithm and curve selections and should not be taken to reflect the running times of actual devices.

It is difficult to obtain precise performance figures for operations on the NIST curve ansip256r1 on constrained microprocessors or in hardware. In this analysis we rely on [BHLM01], which presents figures for software implementation on a 400 MHz Pentium II. We present figures obtained by scaling for clock speed. We also use the figures from [HNM], in their summary form as presented in [LD00].The figures for SHA-256 are obtained from Wei Dai’s Crypto++ 4.0 benchmark page [Dai00].  These figures are for an 850 MHz Pentium processor and have been scaled down accordingly.

The timing data presented here are difficult to compare with the data presented in [02144]. That document did not cite the source of the data presented: scaling down the figures in [HHM00] gives results that differ from [02144] by a factor of four. Additionally, [02144] assumes that certain calculations can be performed off-line; this will not always be the case, and our timing figures here take this fact into account.

Many implementations of ECC will require custom hardware.  We emphasize again that the purpose of the figures below is simply to compare the relative speeds of the two protocols under consideration, not to suggest the speeds that these protocols will actually run in practice.

To start the analysis, we outline the cryptographic operations that must be performed in each of the protocols. A detailed description of these operations can be found in the appendix below.

| Calculation | Cryptographic Operations |  |
| --- | --- | --- |
|  | ECIES | MQV |
| DEV (step 1) | - | - |
| PNC (step 2) | Public key management operation (see notes 1, 2). 1 point multiply without precomputation. 1 point multiply with precomputation (see notes 3, 4). 6 hashes (see notes 5, 6, 7, 8). | Public key management operation (see notes 1, 2). 1 point multiply with precomputation (see notes 3, 4). |
| DEV (step 3) | Public key management operation (see notes 1, 2). 2 separate point multiplies without precomputation. 1 point multiply with precomputation (see notes 3, 4). 22 hashes (see notes 5, 6, 7, 8). | Public key management operation (see notes 1, 2). 1 point multiply with precomputation (see notes 3, 4). 1 MQV operation. 4 hashes (see notes 5, 6, 7, 8). |
| PNC-2 (step 4) | 1 point multiply without precomputation. 24 hashes (see notes 5, 6, 7, 8). | 1 MQV operation. 5 hashes (see notes 5, 6, 7, 8). |

Notes:

- Key Management: In the 802.15.3 architecture specified in [02130], key management is carried out by the DME. The operations may be simple or complex and may incur considerable non-cryptographic overhead. Therefore, we do not consider the time for key management operations in this analysis.

- Certificate Type: For both ECIES and MQV, the protocol may be used with no certificates, implicit certificates, or other (typically X.509) certificates. The choice of certificate type is independent of the choice of protocol. We do not include times related to certificate processing in this analysis.

- Point Multiply with Precomputation: Both ECIES and MQV involve the computation of an ephemeral keypair. This requires an elliptic curve point multiplication on a known base point. In the case of both protocols, precomputation techniques can be used to trade off memory (RAM or ROM) for speed. [BHLM01] provides details of the optimal tradeoff. We have assumed that this precomputation is used. Note that this is not the same as pre-calculation of the key pairs, defined in note 4. Precomputation refers to the case where the ephemeral keypair is calculated during execution of the protocol, but using pre-computed data to speed up this calculation; pre-calculation refers to the case where the ephemeral keypair is calculated before execution of the protocol begins.

- Pre-calculation and Storage of Ephemeral Keypairs: In both ECIES and MQV, the calculation of the ephemeral keypair can be performed before the protocol is initiated, and the results stored in RAM. We refer to this as pre-calculation. Pre-calculation may be used in either protocol to speed up individual authentications. However, if a device has to perform many authentications in a short period of time it may run out of stored pre-calculated ephemeral keys. Our performance figures are therefore calculated for peak loads, on the assumption that ephemeral keypairs have not been pre-calculated.

- Hash Input: SHA-1 and SHA-256 take input blocks 64 bytes in length. We use this in the calculations presented above.

- Point Compression and Length of Hash Input: In both schemes, elliptic curve points may be transmitted compressed or uncompressed. Here, by “compressed”, we refer to the practice of representing the y coordinate by a single bit, as described in section 3.2.2 above. In the MQV protocols, we assume that the points are represented using point compression. In the ECIES protocols, we assume that the points are represented without using point compression. The representation of the point affects the amount of data passed to the hash function. This is taken into account in the figures given below: with compression, the representation of a point on ansip256r1 is 33 bytes, and without it the representation is 65 bytes.

- Certificate Format and Length of Hash Input: In the protocols presented, the certificate information is input to the symmetric integrity function, and so affects the running time of the cryptographic operations. In this document, we assume for the sake of argument that the public key objects are public keys. Certificates are longer, and the use of certificates would make the symmetric integrity operations take more time. However, the majority of the running time is due to the asymmetric operations, so the effect on the overall time for cryptographic operations would be minor.

- Number of Hash Operations: To perform an ECIES encryption or decryption of a 32-byte block of data without point compression requires 6 hashes: two calls to SHA-256 on a two-block input to generate EncKey||MacKey (in the notation of [X963], section 5.8), then two calls to generate or check the HMAC. For HMACs on general messages, we model the HMAC as requiring two more hashes than the number of blocks in the message. The message lengths for ECIES are derived from section 5.1 of this document. The message lengths for MQV are taken from [02200].

These calculations give the following running times for different microprocessors. Columns 1, 2 and 4 provide the running times obtained by scaling down the figures obtained in [BHLM01] from a clock speed of 400 MHz to the clock speed given. Columns 3 and 5 provide the running times obtained from the figures obtained in [HNM98], first by multiplying by (256/160)3 to allow for the increase in keylength compared to that paper, and second by scaling the clock speed from 10 MHz to the speed given.

We emphasize again that these times should be compared to give the relative speeds of ECIES and MQV, not necessarily to reflect the running times on actual devices.

| Computation | Time (ms) on 30 MHz device by [BHLM01] |  |  | Time (ms) on 13 MHz microprocessor by [BHLM01] |  | Time (ms) on 10 MHz microprocessor by [HNM98] |  | Time (ms) on 2.66 MHz microprocessor by [BHLM01] |  | Time (ms) on 2.66 MHz microprocessor by [HNM98] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ECIES |  | MQV | ECIES | MQV | ECIES | MQV | ECIES | MQV | ECIES | MQV |
| PNC (step 2) | 93.2 |  | 35.3 | 215 | 82 | 2562 | 973 | 1051 | 398 | 9633 | 3659 |
| DEV (step 3) | 165.0 |  | 120.9 | 381 | 279 | 4512 | 3554 | 1861 | 1364 | 16967 | 13363 |
| PNC (step 4) | 72.0 |  | 85.7 | 166 | 198 | 1951 | 2581 | 812 | 967 | 7335 | 9705 |
| Total superframes | 7 |  | 5 | 13 | 11 | 141 | 110 | 59 | 43 | 524 | 413 |
| Base times used (ms) |  |  |  |  |  |  |  |  |  |  |  |
| Point multiply |  | 70.6 |  | 163 |  | 1946 |  | 796 |  | 7318 |  |
| Point multiply with precomp. |  | 22.3 |  | 51 |  | 614 |  | 251 |  | 2310 |  |
| MQV operation |  | 85.3 |  | 197 |  | 2580 |  | 962 |  | 9701 |  |
| SHA-256 hash (64-byte block) |  | 0.07 |  | 0.17 |  | 0.22 |  | 0.83 |  | 0.83 |  |
| Superframe |  | 65 |  | 65 |  | 65 |  | 65 |  | 65 |  |

# Security Considerations

## General

This section provides an overview of the security properties of our proposal. First we review the security of the ECIES encryption scheme as a building block for secure protocols; then we review the properties of the ECIES encryption scheme against the security properties defined in Section 2.1 and identified in Appendix H, Table H-2 of [X963]. See section 5.1 for an explanation of the variable names used when referring to the ECIES protocol.

- ECIES: ECIES provides provable security against adaptive chosen ciphertext attacks. This is the strongest property that an encryption scheme can have. To paraphrase [ABR01], it is a scheme that combines a symmetric encryption method, a message authentication code, and a hash function, in a way that is intended to provide security against chosen ciphertext attacks. The proofs of security are based on the assumption that the underlying symmetric primitives are secure and on appropriate assumptions about the Diffie-Hellman problem.

- Key Authentication: The protocol provides explicit key authentication through the use of the shared secret Int_D.

- Entity Authentication: The protocol provides entity authentication through the inclusion of fresh challenges in each direction.

- Known-Key Security: Each session key is generated by combining two fresh random challenges using a one-way function. The compromise of one session key cannot endanger another session key, barring catastrophic random number generator failure on both sides. This is also true of MQV.

- Forward Secrecy: MQV provides “perfect forward secrecy”, which is the property that if a device is compromised and its private key recovered, an attacker is still unable to read any previous messages received by that device (provided that short-lived security data has been securely disposed of). ECIES does not provide this exact property, but the implementation in this protocol has the property that in order to read any messages that two devices exchanged in the past, an attacker must record all those messages and then compromise the private keys of both of the devices.

- Key Compromise Impersonation: An attacker who obtains Alice’s private key cannot use it to impersonate any other entity to Alice.

- Unknown Key Share: Certain implementations of MQV and other key exchange protocols are subject to an unknown key-share attack. In this attack, an interceptor, Ivan, can manipulate messages between Alice and Bob such that Alice and Bob share a key with each other (and this key is unknown to Ivan), Alice knows she shares the key with Bob, but Bob thinks he shares it with Ivan. This attack fails against the ECIES authentication protocol provided here, because each party calculates an integrity code on data including the identity of both parties they think are participating in the protocol and Ivan, not knowing the shared key Int_D, cannot forge this integrity code. We note that the version of MQV proposed in [02150] is not vulnerable to unknown key-share attacks either.

It therefore appears that this proposal provides security against all practical attacks.

## Key Validation in ECIES: Reasons for Omission

### What is Key Validation?

Key Validation is a technique used in Discrete Logarithm and Elliptic Curve Discrete Logarithm cryptosystems, to guarantee that a public key is “valid” – in other words, that it is a mathematical object of the right type. Note that key validation does not guarantee that the public key belongs to the right person or entity, or that even that a corresponding private key exists. Key validation is simply a statement that the object purporting to be a public key has certain mathematical properties. X9.63 (Appendix H, section H.5, note 4) describes public key validation as a process that “states that ‘it looks like this particular public key makes sense’.”

### How is Key Validation Performed in X9.63?

In the case of an elliptic curve E, where there is a generating point G of prime order n and the curve is of order .n, the procedure for key validation on a purported public key (xQ’, yQ’) is as follows:

Actions: The following checks are made:

- Verify that Q’ is not the point at infinity O.

- Verify that xQ’ and yQ’ are elements in the field Fq. (That is, verify that xQ’ and yQ’ are integers in the interval [0,p-1] in the case that q=p is an odd prime, or that xQ’ and yQ’ are bit strings of length m bits in the case that q=2 m .)

- If q=p is an odd prime, verify that (yQ’)2 (xQ’)3 +axQ’ +b (mod p). If q=2 m , verify that (yQ’)2+  yQ’ xQ’ = (xQ’)3 +axQ’ +b in F2m .

- Verify that nQ’=O.

Output: If any one of the above verifications fail, then output ‘invalid’ and reject the public key. Otherwise, output ‘valid’ and accept the public key.

Of these actions, the first three simply demonstrate that the string (xQ’, yQ’) actually represents a point on the curve. The final one, multiplying by n, demonstrates that the point is one of those generated by G.

### Is Key Validation Required in X9.63?

The standard is ambiguous. The description of ECIES decryption in [X963] section 5.8.2 states that key validation shall be performed (step 2). However, Appendix H, section H.5, note 4 states:

The decision whether to do any public key validation and what kind(s) to do is a cost/benefit decision. Public Key Validation is optional, but doing some form of validation is recommended, especially for high security and/or high risk applications.

Thus, it appears that a body adopting X9.63 can decide, on the basis of an informed cost/benefit decision, not to use key validation.

### Is Key Validation Required in Other Standards?

#### ISO

An authoritative overview of issues in the use of ECIES is provided by Victor Shoup, the editor of the forthcoming ISO/IEC standard 18033, in [Sho02]. Section 15 of this document concerns ECIES. In it he defines two modes, CofactorMode and CheckMode, each of which is set to 1 if the relevant operation (cofactor multiplication and public key validation, respectively) is to be performed, and 0 if it is not. In section 15.1 he states:

If  = 1, then both of these modes should be 0.

In the case of the NIST prime curves, the value of  is always 1. Therefore, omitting key validation for ECIES over the NIST curves is entirely consistent with the security arguments of ISO 18033.

(We note that in section 15.6.1 of his document, Shoup states that the ephemeral public key of the encrypter must be passed to the key derivation function. This corresponds to passing the public key as SharedData1 in step 5 of ECIES encryption and decryption, sections 5.8.1 and 5.8.2 of X9.63. The ECIES security suite proposal enforces this requirement).

#### IEEE

IEEE P1363 marks key validation steps as optional in all cryptographic schemes.

### What are the Security Risks if Key Validation is Not Performed?

Opinions differ as to whether or not there is a security risk in omitting key validation. To quote again from [X963] Appendix H, section H.5, note 4:

The decision whether to do any public key validation and what kind(s) to do is a cost/benefit decision. Public Key Validation is optional, but doing some form of validation is recommended, especially for high security and/or high risk applications. There are two main reasons to consider doing public key validation:

- To increase the chances of detecting an inadvertent key generation calculation error that could void any or all security objectives. This objective can be seen as a way to follow “good engineering practice” by checking the output of the key pair generation process.

- To increase the possibility of detecting a deliberate error introduced by an adversary and/or possibly make the task of creating a candidate public key with a deliberate error more difficult. Depending on other possible constraints, this may mean that an otherwise feasible attack becomes infeasible or uneconomic

and

However, note that just because a public key is valid does not mean it is secure, there are other considerations involved, such as ensuring the key pair was selected from a large enough keyspace and whether the private key is protected properly. Furthermore, the recipient of a message may decrypt it and reveal it to others that were not intended by the sender to see it.

To assess the danger from omitting key validation, we must consider the possible attacks. These are:

- An attacker manages to cause a random fault in key generation, resulting in nonsense being sent instead of a public key. This is the case caught by steps 1-3 of validation.

- An attacker manages to cause a random fault in key generation, resulting in a public key being sent which is contained in an incorrect subgroup of the main group (this is known as a “small subgroup attack”). This is the case caught by step 4 of validation.

- An attacker manages to substitute a public key for the legitimate one, knowing that the public key is in an incorrect subgroup or is carefully formed nonsense.

It is unclear that any of these attacks constitute a realistic threat for 802.15.3.

- There are no known attacks on elliptic curve cryptosystems that involve getting a victim to use nonsense, instead of a valid point. The chances of such nonsense having a useful (to the attacker) structure seem vanishingly small.

- If Shoup’s requirement ( = 1) is met, there is no small subgroup attack.

- If we posit an attacker who can insert key material of their choice, that attacker might as well generate a valid public and private key pair and insert the valid public key into the message; that way, they can mount an effective man-in-the-middle attack. Key validation does not prevent this attack.

### Is Key Validation Subject To Patents?

Certicom has stated that it has applied for patents on techniques including key validation. It is unclear what specific techniques are covered. This uncertainty, coupled with the fact that it was hard to argue for a significant gain in security from the use of key validation, is one reason why IEEE P1363 decided not to require the use of key validation

### In Conclusion:

Both ISO and IEEE 1363 have presented strong arguments that the possible security gains from key validation are so small that its use should not be required in standards. Even X9.63 makes a statement to the effect that it may be optional. Additionally, the patent situation regarding the use of key validation is unclear. For all these reasons, it is a reasonable choice not to require key validation in 802.15.3.

# Review of Protocols

This section reviews the details of the protocols under consideration.

## ECIES Protocol

This section outlines the ECIES protocol. The key Int_D, used to provide symmetric authentication, can be a key for HMAC-SHA-256, as mandated by the 802.15.3 working group. We note that HMAC-SHA-1 (truncated to 128 bits) and AES-CBC-MAC may also be used to provide 128-bit security, at considerably higher efficiency. See [02130] for a more detailed description of the meaning of the other variables.

| Step | Calculation performed / Message sent |
| --- | --- |
| Device  SM | AReq, ID_D, PKObjD |
| SM Calculation | - Extracts Pub_D from PKObj_D - Selects a unique SSID_D - Generates random C2 - Encrypts C2 with Pub_D |
| SM  Device | CReq, OID, SSID_D, ID_SM, PKObj_SM, Enc(C2, Pub_D) |
| DEV Calculation | - Extracts Pub_SM from PKObj_SM - Decrypts C2 using Pr_D - Generates random C1 - Encrypts C1 using Pub_SM - Generates Enc_D and Int_D using the formulas: Enc_D = Key(H(K¦¦0x00)) Int_D = Key(H(K¦¦0x01)) - Generates message authentication code on the entire protocol up to this point using Int_D |
| Device  SM | CRes, Enc(C1, Pub_SM), SymI (AReq¦¦ ID_D¦¦ PKObj_D¦¦ CReq¦¦ OID¦¦ SSID_D¦¦ ID_SM¦¦ PKObj_SM¦¦ Enc(C2, Pub_D) ¦¦ CRes ¦¦ Enc (C1, Pub_SM), Int_D) = finished1 |
| SM Calculation | - Decrypts C1 using Pr_SM - Generates Enc_D and Int_D using the formulas: Enc_D = Key(H(K¦¦0x00)) Int_D = Key(H(K¦¦0x01)) - Checks message authentication code finished1 using Int_D - Generates message authentication code on the entire protocol up to this point using Int_D. - Set seq_num_SM = 0 - Set seq_num_D = 0 |
| SM  Device | ARes, SymI (AReq¦¦ ID_D¦¦ PKObj_D¦¦ CReq¦¦ OID¦¦ SSID_D¦¦ ID_SM¦¦ PKObj_SM¦¦ Enc(C2, Pub_D) ¦¦ CRes ¦¦ Enc (C1, Pub_SM), finished1 ¦¦ C2 ¦¦ c1, Int_D) = finished2 |
| Device Calculation | - Checks message authentication code finished2. - Sets seq_num_SM = 0 - Sets seq_num_D = 0 |

## MQV Protocol

This section recaps the MQV protocol described in [02200] optimized for the protocol described in [02130]. The operations presented here may differ slightly from the operations of [02200], which does not present its operations in this form. We believe this to be the most efficient possible embodiment of the operations of [02200] consistent with [02130]. If the operations can be embodied more efficiently we will happily update this section in a future revision of this document.

| Step | Calculation performed / Message sent |
| --- | --- |
| Device  SM | AReq, IDD, PKObjD |
| SM Calculation | - Extracts Pub_D from PKObj_D - Calculates random y, ephemeral public key Y. - Selects a unique SSID_D |
| SM  Device | CReq, OID, SSID_D, ID_SM, PKObj_SM, Y |
| DEV Calculation | - Extracts Pub_SM from PKObj_SM - Calculates random x, ephemeral public key X. - Calculates K = (x+Pr_D.map(X)) . (Y + map(Y).Pub_SM) - Generates Enc_D and Int_D using the formulas: Enc_D = Key(H(K¦¦0x00)) Int_D = Key(H(K¦¦0x01)) - Generates message authentication code on the entire protocol up to this point using Int_D |
| Device  SM | CRes, X, SymI (AReq¦¦ ID_D¦¦ PKObj_D¦¦ CReq¦¦ OID¦¦ SSID_D¦¦ ID_SM¦¦ PKObj_SM¦¦ Y) = finished1 |
| SM Calculation | - Calculates K = (y+Pr_SM.map(Y)) . (X + map(X). Pub_D) - Generates Enc_D and Int_D using the formulas: Enc_D = Key(H(K¦¦0x00)) Int_D = Key(H(K¦¦0x01)) - Generates message authentication code on the entire protocol up to this point using Int_D. - Set seq_num_SM = 0 - Set seq_num_D = 0 |
| SM  Device | ARes, SymI (AReq¦¦ ID_D¦¦ PKObj_D¦¦ CReq¦¦ OID¦¦ SSID_D¦¦ ID_SM¦¦ PKObj_SM¦¦ Y¦¦ finished1¦¦ ARes, IntD) = finished2 |
| Device Calculation | - Checks message authentication code. - Sets seq_num_SM = 0 - Sets seq_num_D = 0 |

# References

[02130]	D. V. Bailey and A. Singer, IEEE P802-15_TG3 NTRU Security Architecture Proposal, submitted to IEEE P802.15.3 as 02130r1P802-15_TG3-NTRU-Security-Architecture-Proposal.doc

[02144]	G. Rasor, Efficient Public Key Cryptosystem for WPANs, submitted to IEEE P802.15.3 as 02144r1P802.15_TG3-Efficient-Public-Key-Cryptosystem-for-WPANs2.ppt.

[02150]	D. V. Bailey and A. Singer, IEEE P802-15_TG3 Consolidated Security Architecture Proposal, submitted to IEEE P802.15.3 as 02150r0P802-15_TG3-Consolidated-Security-Architecture-Proposal.doc

[02171]	D. V. Bailey, A. Singer, W. Whyte, IEEE P802-15_TG3 Security Text Recommendations, submitted to IEEE P802.15.3 as 02171r0P802-15_TG3-Security-Text-Recommendations.doc

[ABR01]	M. Abdalla, M. Bellare and P. Rogaway, DHIES: An encryption scheme based on the Diffie-Hellman Problem. Extended abstract, entitled The Oracle Diffie-Hellman Assumptions and an Analysis of DHIES, was in Topics in Cryptology - CT-RSA 01, Lecture Notes in Computer Science Vol. 2020, D. Naccache ed, Springer-Verlag, 2001. Available from http://www-cse.ucsd.edu/users/mihir/

[BDLM0100]	M. Brown, D. Hankerson, J. Lopez, A. Menezes, Software Implementation of the NIST Elliptic Curves Over Prime Fields, Topics in Cryptology - CT-RSA 2001, Lecture Notes in Computer Science, 2020 (2001), Springer-Verlag, 250-265

[BM99]	S. Blake-Wilson, .A.Menezes, Unknown key-share attacks on the Station-to-Station (STS) Protocol. In Public Key Cryptography (PKC ’99) Proceedings, H. Imai and Y. Zheng, Eds., vol. 1560, Lecture Notes in Computer Science, Springer-Verlag, New York, 154–170. 1999.

[BR97]	M. Bellare and P Rogaway, Minimizing the use of random oracles in authenticated encryption schemes, Proc. PKS ’97, 1997.

[Dai00]	Wei Dai, Crypto++ 4.0 Benchmarks, available from http://www.eskimo.com/~weidai/benchmarks.html.

[Dec99]	P. Deck, Certicom patent letter to SECG, available from http://www.secg.org/collateral/certicom_secg_patent.pdf.

[HHM00]	D. Hankerson, J. L. Hernandez, A. Menezes, Software Implementation of Elliptic Curve Cryptography over Binary Fields, Cryptographic Hardware and Embedded Systems – CHES 2000, LNCS 1965, C. K. Koc and C. Paar (eds), Springer-Verlag, 2000, pp. 1-19

[HNM98]	T. Hasegawa, J. Nakajima, M. Matsui, A Practical Implementation of Elliptic Curve Cryptosystems over Fp on a 16-bit microcomputer, Public Key Cryptography – Proceedings of PKC ’98, LNCS 1431, pp. 182-194, 1998.

[Kal01]	B. Kaliski, An Unknown Key-Share Attack on the MQV Key Agreement Protocol, ACM Transactions on Information and System Security, Vol. 4, No. 3, August 2001, Pages 275–288.

[LD00]	J. Lopez and R. Dahab, Performance of Elliptic Curve Cryptosystems, Technical report IC-00-08, May 2000. Available at http://www.dcc.unicamp.br/ic-main/publications-e.html.

[X963]	American Bankers Association, ANSI X9.63-2001, Public Key Cryptography for the Financial Services Industry, Key Agreement and Key Transport Using Elliptic Curve Cryptography, January 10, 2002.

[Sho02] 	V. Shoup, A Proposal for an ISO Standard for Public Key Encryption (available from http://www.shoup.net)