IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Security and Security Architectural Recommendations for the IEEE 802.15.3 High-Rate WPAN |  |
| Date Submitted | October 8, 2002 |  |
| Source | [Rene Struik, Certicom Corp.] 5520 Explorer Drive, 4th Floor Mississauga, ON Canada L4W 5L1 | Voice: (905) 501-6083 Fax: (905) 507-4230 E-mail: rstruik@certicom.com |
| Re: | 802.15.3 TG3 Draft Specification D11 |  |
| Abstract | This document provides some recommendations to improve the security of the current IEEE 802.15.3 WPAN specification and assists with Letter Ballot #19 comment resolution. |  |
| Purpose | Improve the security provisions in the current draft and assist with Letter Ballot #19 comment resolution. |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

# Comment Resolution of Letter Ballot #19

[Added October 8, 2002, 3:30pm EDT]

Comment #92 of LB 19 (again)

Please find below an additional rationale to my response to the “Modes of operation comment” (#92 of LB19). See also my initial comment in 02/364r0 (as of Sep 19, 2002, 1:30pm EDT).

Access of a device A to the piconet is determined by a process involving A and the acting PNC of the piconet at hand. When admitted to the piconet, one would like to have the following guarantees, depending upon the perspective of each device:

- Device A. The entering device would like to have evidence about the true identity of the PNC. Moreover, it would like to have assurances as to the bit-level of security under which the piconet operates and under which circumstances the other devices have been admitted to the piconet, so that it can have certain minimum expectation levels as to the true identity of other devices in the piconet. It can then determine based on its own security policy whether it is justified to be(come) a member of the piconet.

- Piconet controller. The acting PNC would like to have evidence about the true identity of the entering device. Moreover, it would like to maintain a certain minimum bit-level of security and maintain certain standards under which devices are admitted to the piconet. It can determine based on its own security policy whether it is justified to grant the entering device access.

- Other member devices. The devices that have already been granted access to the piconet would like to be assured that that the access policy exercised by the PNC is consistent. Moreover, it would like to have assurances as to the bit-level of security under which the piconet operates and under which circumstances the other devices have been admitted to the piconet, so that it can have certain minimum expectation levels as to the true identity of other devices in the piconet. It can then determine based on its own security policy whether it is justified to remain a member of the piconet.

These guarantees can be provided as follows:

- Any device that is in the piconet can impersonate the acting PNC and make the entering device believe that he is the acting PNC. The entering device can only detect that after completion of the authentication protocol and potential additional checks, e.g., during the key distribution protocol or when issued commands to the true PNC fail. This hijacking of the PNC role cannot be ruled out, although the other devices in the piconet will almost surely detect it.

- Devices that a members of the piconet can only be assured about the minimum bit-level of security and the type of cryptographic identity checks if this information is available to all devices and not changed during the operation of the piconet (see also Para 2.1 of 02/364r0, which spells this out as security policy).

  - In the current Draft, the PNC communicates a set of OIDs to the entering device and vice-versa. From this set of OIDs, one can only determine a range of bit-levels and authentication mechanisms and, thus, one has uncertainty about the exact cryptographic assurances provided (the security suites define either 80-bit or 128-bit security, using certificates or not). Moreover, each device needs to be aware of all OIDs and their security properties, for otherwise it cannot determine the properties of the network.

  - In the current draft, each device can determine whether the authentication protocol fits its own security policy, although this does not necessarily extend to subsequent authentication protocols between the PNC and devices that are admitted afterwards or have been admitted beforehand.

- To make the security bit-level at which the piconet operates and the authentication methods persistent, one can do the following:

  - Each device in the piconet checks whether this information, contained in the beacon, is persistent.

  - The piconet communicates this info to the entering device in an authentic way. (If one does not do this, the entering device can still check for persistence by virtue of a., although this is less direct.

[Added October 8, 2002, 1:30pm EDT]

Comment #93 of LB 19

Please find below my response to the “MAC PIB Table comment” (#93 of LB19). Comments on the e-mail comment resolution are in italics. For my proposed solution, see below the extracted e-mail fragment.

[beginning of extract of e-mail John Barr as of October 7, 2002, 7:39pm]

93 (Gilb, TR) [SEC/PIB] The security suite and public-key verification

information have been removed from the MAC PIB. This implies that there is

no accessible information that the MLME can use to perform verification on

the public key. In particular, the CA certificate or hash of the public key

are not available. In clause 10, each security sub-suite specifies that the

ACL indicates which public-keys shall be accepted and which shall be

rejected. Add public-key verification information back into the ACL and

specify in each security suite how that ACL information is to be used. If

there are operations to be performed by the DME to verify the public key,

those should be mentioned in the security suites as well. This applies to

6.5.6 as well. Suggest accept in principle. The public key verification

operations are performed by the DME and are hence outside the scope of the

standard. Rather than adding this information to the PIB, the description of

the security suites in clause 10 should be modified to reflect that this is

out of scope. Make the following changes to clause 10:

RS: if operations are outside of scope one should not go into to much detail in describing how these are to be performed. One should describe the minimum context, though, that allows one to draw conclusions on the security of the system.

In clause 10.3.1.4.3, change the paragraph in lines 18-19 to: "The

certificate shall be generated using the digital signature algorithm ECDSA

as specified in 10.3.1.4.4. The validation of the certificate is outside the

scope of this standard."

Note RS: The proper cryptographic notion is certificate verification, not certificate validation.

In 10.3.2.2.2, remove step 2) and the final sentence and add the following

text:

"Processing shall be aborted if the public key is not successfully

extracted.

The DEV should perform additional checks such as comparing the DEV address

in the ManCert to the DEV address in the authentication request or comparing

the received key and ID to values stored in its ACL to verify the

authenticity of the public key."

In 10.3.2.3.2, remove step 2) and the final sentence and add the following

text:

"Processing shall be aborted if the public key is not successfully

extracted.

The DEV should perform additional checks such as comparing the DEV address

authenticated in the ImplCert with the DEV address stored in its ACL to

verify that the device is authorized."

Note RS: One should define what authorization entails to. Currently, it is such a vaguely defined term that one cannot really draw security conclusions about the system.

A device A is authorized by the PNC to act as a member of the piconet if it successfully passes the following two tests:

- Successful completion of the key establishment protocol;

- Successful completion of the membership test of the Access Control List (ACL membership).

Note RS: The types of potential checks one has to perform with implicit certificates and X509 certificates are very similar, depending upon the security policy (details hereof outside scope) that is in place. So, descriptions for handling of implicit certificates and X509 certificates should be similar. This is currently not the case (see e-mail text in the paragraph below).

In 10.3.2.4.2, remove step 2) and the following paragraphs and add the

following text:

"Processing shall be aborted if the public key is not successfully

extracted.

The DEV should perform additional checks such as signature verification as

specified in 10.3.1.4.3, CRL checking, validity period verification, key use

checking and comparing the DEV address in the X.509 certificate with the DEV

address stored in its ACL to verify that the device is authorized."

In 10.4.2.2, change the table entry for Verification of Public-Key to the

following text:

"The ID and public-key received during the authentication protocol should be

verified by the DME using checks such as generating the SHA-1 hash of the

device address concatenated with the public-key and comparing that to the

hash of the ID and public key stored in the ACL."

Note RS: the SHA1-hash has no security functionality. Its only reason for being there is to provide for data compression, such as to allow lengthy public key information to be stored in a condensed format. (One might as well just take the Fourier Transform and truncate). Issues that have to do with the exact representation format of public key information should not be discussed in the 802.15.3 standard, since these depend on decisions of how to architect memory by each individual manufacturer/producer of 802.15.3 compliant devices.

In 10.5.2.2, change the table entry for Verification of Public-Key to the

following text:

"The ID and public-key received during the authentication protocol should be

verified by the DME using checks such as generating the SHA-1 hash of the

device address concatenated with the public-key and comparing that to the

hash of the ID and public key stored in the ACL."

Note RS: see comment above.

In 10.5.3.2, change the table entry for Verification of Public-Key to the

following text:

"The X.509 certificate received in the authentication protocol should be

verified by performing checks such as signature verification as specified in

10.5.1.7, CRL checking, validity period verification, key use checking and

comparing the DEV address in the X.509 certificate with the DEV address

stored in its ACL to verify that the device is authorized."

Table until it can be reviewed with Rene.

[end of extract of e-mail John Barr as of October 7, 2002, 7:39pm]

Comment RS:

Suggest reject (i.e., do not incorporate as MAC PIB entries.)

ACL

ACL

ACL

Maintenance

ACL

Maintenance

ACL

initialization

ACL

initialization

B

A

authentication

key distribution

data transfer

Public key

verification

Public key

verification

Wrapped

public

key info

Extracted

public key

info

Wrapped

public

key info

Extracted

public key

info

Below are my proposed changes to the Draft D11 text.

Clause 10.1.2, Page 273: remove the reference to hash values. As already pointed out above, this is a representation issue of data objects in local memory.

Clause 10.1.3, Page 273, line 49: change this line into ‘the cryptographic operations that shall be performed on the certificate’.

Note RS: the non-cryptographic checks are outside the scope of the standard. Besides, these depend on the security policy that is defined (also outside of scope) for the particular device.

Note RS: The extraction of the public key and the device identifier might be outside the scope of the standard. Nevertheless, the resulting information should become available to the authenticated key establishment protocol in the DME (with ECC implicit certificates, this is one process and part of the inherent ECMQV protocol operations). So, whereas the verification of the validity of the certificate (authenticity, validity, etc.) can be handled outside scope, the most natural location to check whether the DEVID of the device that sent the public key information really corresponds to the device identifier that is part of the public key certificate (be it manual, implicit or X509 certificate) is inside the scope of the standard. See, also the figure above that shows how, conceptually, wrapped public key information is extracted and how the information that comes available is passed back.

This leads to the following suggestions for modifications of the D11 text.

Clause 10.3.2.2.2, Page 284:

Add the following text, at the end of the paragraph:

‘The mechanism by which Step 1) of the above process is implemented involves both cryptographic and non-cryptographic checks. The non-cryptographic checks depend on the security policy that is defined for the particular device and are outside the scope of this standard.’

Clause 10.3.2.3.2, Page 285:

Add the following text, at the end of the paragraph:

‘The mechanism by which Step 1) of the above process is implemented involves both cryptographic and non-cryptographic checks. The non-cryptographic checks depend on the security policy that is defined for the particular device and are outside the scope of this standard.’

Clause 10.3.2.4.2, Page 285:

Add the following text, at the end of the paragraph:

‘The mechanism by which Step 1) of the above process is implemented involves both cryptographic and non-cryptographic checks. The non-cryptographic checks depend on the security policy that is defined for the particular device and are outside the scope of this standard.’

Clause 10.3.2.5.2, Page 286:

Change the first entry in Table 94 (‘Verification of the public key’), by adding the following text, at the end of this entry: ‘The implementation of this mechanism is outside the scope of this standard.’

Clause 10.4.2.2, Page 290:

The first entry in Table 101 (‘Verification of the public key’) could be changed along the lines as the suggestion for Table 94 (see previous paragraph above). As already stated before,

Issues that have to do with the exact representation format of public key information should not be discussed in the 802.15.3 standard, since these depend on decisions of how to architect memory by each individual manufacturer/producer of 802.15.3 compliant devices. Hence, references to the SHA-1 hash function are a little bit odd.

Clause 10.5.2.2, Page 293:

The first entry in Table 103 (‘Verification of the public key’) could be changed along the lines as the suggestion for Table 94 (see previous paragraph above).

Clause 10.5.3.2, Page 295:

The first entry in Table 105 (‘Verification of the public key’) could be changed along the lines as the suggestion for Table 94 (see previous paragraph above).

[Added September 19, 2002, 1:30pm EDT]

Comment #92 of LB 19

Please find below my response to the "Modes of Operation Comment" (#92 of LB19).

[beginning of extract of document 02/391r9]

#92 (James Gilb)

Comment:

There is really only two security modes, no security and encryption with authentication.  Mode 3 is just a different authentication procedure, like the OIDs in mode 2.

Suggested remedy:

Change the draft to be only 2 security modes with multiple, optional authentication methods determined by the OIDs.

Response:

ACCEPT IN PRINCIPLE. Suggest a table that has security levels (i.e. claimed bits) and if the OID offers cryptographic authenticiation of public keys for each of the OIDs. Merge Mode 1 and Mode 2 services offered, pointing out that some OIDs use certificates, some don’t. Throughout the draft, use only mode 0 or mode 1 or security off or security on. Change the SEC mode field in the beacon to be only one bit.

[end of extract of document 02/391r9]

Response:

Suggest reject, for both (1) security reasons; (2) ease of implementation.

Below follows a short tutorial on IEEE 802.15.3 WPAN security that motivates this response. For simplicity, we restrict ourselves to piconets where all message traffic (except commands) is broadcast traffic.

1. Security for IEEE 802.15.3 WPANs

The IEEE 802.15.3 WPAN technology is a wireless technology and, thus, is vulnerable to passive eavesdropping of communications. Moreover, devices can join and leave a piconet in an ad-hoc fashion and the role of the PNC can be assigned dynamically, based on capabilities rather than security considerations. It is clear that in this constellation, devices have no assurances whatsoever about whom they are communicating with and that their messages cannot be eavesdropped upon, or altered, undetected. Hence, the need for security.

Security within 802.15.3 WPANs serves the following two objectives:

- Proper operation of the piconet. Scarce network resources are only allocated to authorized devices, thus ensuring the proper allocation of bandwidth and quality of service.

- Privacy of message traffic between piconet devices. Messages shared between (a group of) devices remain private to these devices only and can be verified for proper formatting, thus ensuring that no external party can learn the content of exchanged messages (confidentiality) or modify or inject messages in an undetectable way (data integrity).

1.1 Proper operation of the piconet

The PNC is responsible for access control to the piconet, message control, and allocation of bandwidth.

## 1.1.1 Access control to the piconet

Access to the piconet is administered by the PNC, who determines whether a joining or resident device is authorized to be part of the piconet. Authorization for a joining device is based on the execution of an authentication protocol between this device and the PNC – to verify the true identity of the joining device and the PNC – and, possibly, on other mechanisms that are outside the scope of the standard, such as checking whether the Access Control List (ACL) maintained by the PNC contains the device address of the joining device. Continued authorization of a resident device is not described in the current standard; it could be arranged, however, via, e.g., determining whether resident devices are still authorized to be part of the piconet, once the ACL changes. Admission of a device to the piconet depends upon successful completion of this authorization mechanism. If so, the authorization mechanism results not only in the admittance of the joining device to the piconet, but also in the establishment of a shared link key between the PNC and this device (unless operating in non-secure mode of course).

The 802.15.3 WPAN standard specifies three authentication methods, the suitability of which depending on the actual implementation environment hereof. The 3 different authentication mechanisms provide protection as follows:

(1) Mode 0: security only in the absence of active or passive adversaries. In particular, one assumes that there are no eavesdroppers. (This basically does not offer any security, since wireless traffic can easily be snooped upon, e.g., via a ‘parking-lot’ attack).

(2) Mode 1: security in the absence of active adversaries. In particular, one has security if one only has eavesdroppers, but one might loose security completely if one has adversaries that have the capability to modify or inject messages, initiate protocol runs, etc.

(2) Mode 2: security even in the presence of active adversaries. No matter how adversaries would like to attack the communication channel between devices, they will always fail to divert the security objectives.

| Security mode | Authentication result |  | Other mechanisms |
| --- | --- | --- | --- |
|  | Device authentication | Key establishment | Out of scope (via Access Control Lists, etc.) |
| Mode 0: | Non-cryptographic/manual (via button-push, etc.) | - |  |
| Mode 1: |  | Authentic link key (if passive attacks only) |  |
| Mode 2: | Cryptographic/automatic (via public-key certificates) | Authentic link key (in all attack scenarios) |  |

Figure 1: summary of security modes for authorization of devices.

Remark In Modes 0 and 1, establishing the true identity of the communicating devices requires initial user intervention, whereas in Mode 2 this is automatic, via the use of certificates. In Modes 1 and 2, the authentication protocol yields a shared link key, to be used for securing future communications between the communicating devices. Note, however, that only in security mode 2 devices automatically know with whom they share this link key; in Mode 1, this can only be concluded manually, after user intervention.

The actual implementation of the authentication for the different security modes is as follows:

- Mode 0 – no security. No cryptographic mechanisms for establishing the true identity of the communicating devices are deployed.

- Mode 1 – unauthenticated public keys.  Authentication is based on a public-key based challenge response protocol, resulting in the establishment of a shared link key between the joining device and the PNC. No cryptographic mechanisms for establishing the true identity of the communicating devices are deployed.

- Mode 2 – authenticated public keys. Authentication is based on a public-key based challenge response protocol, resulting in the establishment of a shared link key between the joining device and the PNC. The true identity of the communicating devices is established cryptographically, via the verification of certificates of both devices.

The link keys established in Modes 1 and 2 are used for securing subsequent communications between the communicating devices. A summary of the authorization mechanisms specified in the standard is provided in Figure 1.

The current 802.15.3 WPAN standard specifies three public-key mechanisms for key agreement. A summary of these authentication mechanisms, including the claimed bit-security level offered hereby, is provided in Figure 2.

| Public-key agreement mechanism | Security mode (as defined by threat model above) |  |  | Bit-strength established key (claimed) |
| --- | --- | --- | --- | --- |
|  | Mode 0 | Mode 1 | Mode 2 |  |
| ECMQV Koblitz-283 | n.a. | ECMQV, without certificates | ECMQV with certificates (implicit, or X509) | 128 bits |
| NTRUEncrypt251-1 | n.a. | NTRUEncrypt, without certificates | Not provided | 80 bits |
| RSA-OAEP 1024-1 | n.a. | RSA-OAEP, without certificates | RSA-OAEP with certificates (X509) | 80 bits |

Figure 1: summary of public-key mechanisms included in the current IEEE 802.15.3 WPAN draft standard, including claimed bit-strength of established symmetric key.

## 1.1.2 Message control and allocation of bandwidth

[DELETED]

## 1.2 Privacy protection of message traffic between devices

Protection of message traffic between devices is based on data keys that are transported to the intended audience using the link keys derived during the execution of the authorization mechanism.

The current 802.15.3 WPAN standard uses the same symmetric-key algorithm, irrespective of the public-key mechanism used. In particular, all data protection is based on AES-CCM using a 128-bit length secret key. Thus, from an interoperability standpoint the public-key mechanism deployed for setting up link keys between devices is not important. From a security-perspective it is, however, due to dependencies between the cryptographic strength of public-key based mechanisms and that of the derived symmetric key (see also 02/290). The relationship between the bit-level security for message protection and the suitability of the public-key mechanisms currently specified is depicted in Figure 3.

| Message bit-level security | Security mode (as defined by threat model above) |  |  |
| --- | --- | --- | --- |
|  | Mode 0 | Mode 1 | Mode 2 |
| 0 bits | Defined | n.a. | n.a. |
| 80 bits | n.a. | ECMQV and RSA-OAEP (with or without certificates) NTRUEncrypt | ECMQV and RSA-OAEP (with certificates) |
| 128 bits | n.a. | ECMQV (with or without certificates) | ECMQV (with certificates) |

Figure 3: Suitability of different public-key based mechanisms specified in the current IEEE 802.15.3 WPAN draft standard, as a function of the required bit-strength for protecting message traffic (based on claimed security bit-levels).

Remark Note that a public-key mechanism that has a 128-bit security level may be deployed to protect messages at a 80-bit security level, but not the other way around!

2.1 Security policy

During the life-time of the piconet, the following security policy shall apply:

- At any given moment of time, all devices in the piconet shall be able to unambiguously determine the actual security properties of the piconet and of all the security relationships they partake in.

The rationale for this policy is that devices have to be able to determine the protection level of the information they communicate at all times.

Failure to do so, e.g., by no distinguishing between Modes 1 and 2, leads to the highly undesirable situation where some devices are admitted to the piconet by checking their identity via manual mechanisms (button push, etc.) or even without checking, whereas others are admitted via cryptographic checks (certificate that evidences the binding between a device Id and its public key).

As a result, one shall have the following requirements:

- The message bit-level security and the security mode shall remain the same during the lifetime of the piconet. In particular, it shall not change between the admission of various devices to the piconet or at piconet handover.

- The PNC shall make available to all devices that want to join the piconet the message bit-level of security and the security mode under which it operates. This requires it to specify in its beacon which of the 5 nontrivial options is supported (see the non-empty boxes of Figure 3).

The cost of enforcing this security policy is that the Mode information field shall allow specification of the 5 relevant security options. This requires 3 bits of information, rather than 2 as currently specified.

- At any given moment of time, access to information shared between members of a group of piconet devices shall be restricted to precisely these group members. As such, this includes access to integrity information.

[DELETED – already discussed at length]

We assume the root CA key needed for verification of the certificates to be installed during manufacturing of the device or at some other time before operational usage, e.g., at personalization of the device.