IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Channel Modeling Sub-committee Report Final |  |
| Date Submitted | [18 November, 2002] |  |
| Source | [Jeff Foerster, Sub-committee Chair] [Intel R&D] [JF3-206 2111 N.E. 25th Ave. Hillsboro, OR 97124] | Voice: [503-264-6859] Fax: [503-264-3483] E-mail: [jeffrey.r.foerster@intel.com] |
| Re: | [] |  |
| Abstract | [] |  |
| Purpose | [The purpose of this report is to summarize the work of the channel modeling sub-committee and provide some final recommendations on how the channel model can be used to help evaluate PHY submissions to IEEE 802.15.3a.] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Channel Modeling Sub-committee Report (Final)

Date: December 3, 2002

This report has been approved by the study group IEEE 802.15.SG3a for use as a method to evaluate the physical layer performance of proposals expected to be submitted to the IEEE 802.15.3a task group.

Revision History of Final Recommendations

| Revision Number | Date | Comments |
| --- | --- | --- |
| 1.0 | 12/03/2002 | Creation of first version of FINAL recommendations. Previous revisions of document 02/368 (revisions 1-4) represented working DRAFT documents and do not constitute final recommendations. |

Table of Contents

1	Introduction	4

1.1	Desired characteristics of channel model	4

2	Narrowband Channel Model	5

3	UWB Channel Model	5

3.1	Summary of measurements and proposed models	5

3.1.1	“The Ultra-wideband Indoor Path Loss Model,” S. Ghassemzadeh and V. Tarokh [1]	6

3.1.2	“Empirically Based Statistical Ultra-Wideband Channel Model,” M. Pendergrass [2]	6

3.1.3	“UWB Channel Modeling Contribution from Intel,” J. Foerster and Q. Li [3]	7

3.1.4	“A proposal for a selection of indoor UWB path loss model,” G. Shor, et. al. [4]	7

3.1.5	“Radio Channel Model for Indoor UWB WPAN Environments,” J. Kunisch and J. Pamp [5]	8

3.1.6	“The Ultra-wideband Indoor Multipath Loss Model,” S. Ghassemzadeh and V. Tarokh [6]	9

3.1.7	“The Ultra-Wide Bandwidth Indoor Channel: from Statistical Model to Simulations,” A. Molisch, M. Win, and D. Cassioli [7]	9

3.1.8	“Evaluation of an Indoor Ultra-Wideband Propagation Channel,” J-M Cramer, R. Scholtz, and M. Win [8]	9

3.1.9	“UWB Propagation Phenomena,” Kai Siwiak [9]	9

3.1.10	“Propagation notes to P802.15 SG3a from IEEE Tutorial,” Kai Siwiak [10]	10

3.2	Proposed UWB channel model	10

3.2.1	Path Loss Model	10

3.2.2	Multipath Model	11

3.3	Realizations from channel model	13

3.3.1	Channel characteristics desired to model	13

3.3.2	Discrete Time Model	14

3.3.3	How to handle different sample times	14

3.3.4	Channel Realizations	15

3.4	Time variability of channel	31

3.5	How to use the model and realizations	31

3.5.1	Meeting the Selection Criteria	31

3.5.2	Level of disclosure desired by PHY proposals	31

4	List of Contributors	32

5	References	32

Appendix	33

# Introduction

The purpose of this report is to summarize the final recommendations of a channel modeling sub-committee that was formed to establish a commonly agreed upon channel model that could be used to evaluate the performance of physical (PHY) layer proposals to the IEEE 802.15.3a task group.  Since many companies plan on submitting PHY proposals using the newly legal UWB frequency spectrum, a new channel model needed to be created that would help evaluate the performance of these systems in realistic channels, since no well-known model existed at the time the sub-committee was created.  Also, for completeness, a narrowband channel model is also defined, based upon a previously used and well-known model, for evaluating the performance of proposals using a narrowband systems in the 2.4 GHz, 5 GHz, 60 GHz, or other unlicensed spectrum.

The establishment of a channel model for IEEE 802.15 Study Group 3a High Rate WPAN alternative PHY presented a number of challenges.  These challenges included the creation of sufficient and accurate criteria for comparing proposals while keeping the model relatively simple and straightforward to allow implementation in a short period of time.

A sub-committee was formed which established an open approach and process to include as broad a set of perspectives as possible including a general call for proposals, presentations, meetings in conjunction with 802.15 SG3a sessions, conference calls and email reflector traffic.  Assignment and tracking of action items were used to maintain focus and assure key issues were addressed.  There was a constant theme of creating a platform for comparison vs. a complete and exhaustive treatment of channel model characteristics.

The fundamental parameters to be included in the model were path loss and multipath in a variety of environments (office and residential as well as LOS and NLOS as a function of distance).  The aspects of multipath such as delay spread, decay profiles, number of paths and associated thresholds presented the most challenge.  Various models were presented based on significant data collection efforts and assessment was made of the best fit given the objectives for establishing comparison criteria.  This data driven effort helped establish key parameters of the model.

Results of the sub-committee’s efforts are captured through formal posting of presentations, links to measurement databases, minutes of meetings and conference calls as well as email reflector history.  Analysis and realizations of the channel model with standard tools such as MATLAB are also available along with information on how to use the model.

Significant appreciation goes to all contributors from commercial, research and academic organizations around the world that made the channel model represented here a reality.

For completeness, since the IEEE 802.15.3a task group is expected to consider both UWB based physical layer proposals as well as narrowband based proposals (i.e., using the 2.4 GHz or 5 GHz unlicensed bands), both channel models are included in this report.

## Desired characteristics of channel model

The goal of the channel model is to capture both the path loss and multipath characteristics of ‘typical’ environments where IEEE 802.15.3a devices are expected to operate.  The model should be relatively simple to use in order to allow PHY proposers to use the model, and, in a timely manner, evaluate the performance of their PHY in ‘typical’ operational environments.  In addition, it should be reflective of actual channel measurements.  Since it may be difficult for a single model to reflect all of the possible channel environments and characteristics, the group chose to try and match the following primary characteristics of the multipath channel:

- RMS delay spread

- Power decay profile

- Number of multipath components (defined as the number of multipath arrivals that are within 10 dB of the peak multipath arrival)

Note that the actual channels resulting from the model may have several paths that are much weaker than 10 dB from the peak, while the above characteristic was simply used to compare to measurement results.

# Narrowband Channel Model

For physical layer proposals using a narrowband channel in the 2.4 GHz, 5 GHz, 60 GHz, or other unlicensed spectrum, a narrowband channel model is described here based upon the IEEE 802.11 channel model using an exponentially decaying, Rayleigh fading tap delay line (TDL). This model assumes that each of the channel taps is drawn from an independent complex Gaussian random variable with an average power profile that decays exponentially. The probability distribution of the k-th tap of the channel impulse response hk is given by N(0,12σk2)+jN(0,12σk2), where N(0,12σk2) is a zero-mean Gaussian random variable with variance 12σk2, k = 1, …, kmax = 10τRMS/Ts, τRMS is the RMS delay spread, and Ts is the sampling time. The energy of each channel tap is given by:

σk2=σ02e−kTs/τRMS,

where σ02 is chosen in such a way as to ensure that average received energy is unity:

σ02=1−e−Ts/τRMS1−e−(kmax+1)Ts/τRMS.

It is assumed that the sampling time Ts is shorter than the symbol time (or chip time) by at least a factor of four.

Due to the simplicity of this mathematical model, no example code (e.g., Matlab) is provided to

illustrate its implementation or usage.

# UWB Channel Model

## Summary of measurements and proposed models

The following subsections summarize the channel model contributions that were considered by the sub-committee.

### “The Ultra-wideband Indoor Path Loss Model,” S. Ghassemzadeh and V. Tarokh [1]

The authors presented a statistical path loss model for indoor UWB signals of nominal center frequency of 5 GHz in indoor environments.  The model is based on extensive propagation study in 23 homes. The model makes distinction between the main parameters of the propagation path loss from one home to another. The model has capability of even more refinement as more data becomes available.

### “Empirically Based Statistical Ultra-Wideband Channel Model,” M. Pendergrass [2]

An ultra-wideband (UWB) channel measurement and modeling effort, targeted towards the short-range, high data rate wireless personal area network (WPAN) application space, has been described.  Results of this project include a measurement database of UWB channel soundings, including both line of sight and non line of sight channels, and a statistical model of this database.

The channel soundings were taken using an ultra-wideband pulse with a bandwidth of approximately 2 GHz, and a center frequency of approximately 4 GHz.  The measurement database consisted of 429 channel soundings:  369 measurements taken in 11 different rooms in an office complex with metal stud construction, and 60 measurements taken in a residence with wood stud construction.

Channel impulse response functions were extracted from all the channel soundings using the CLEAN algorithm, a serial time-domain deconvolution technique.  An 85% energy-capture criterion was used to determine when to terminate the channel extraction process.  The 85% threshold was chosen to minimize the average modeling error over the entire data set.  The average modeling error over the data set was less than 1 dB.

Of the 369 office channel soundings, there were 51 channels for which the CLEAN algorithm could not achieve an 85% energy capture threshold.  These channels were discarded.  The remaining channels were grouped into three classes, based on the transmitter-to-receiver distance and whether the direct path was obstructed:

Case 1:  0 to 4 meters, line of sight (120 channels)

Case 2: 0 to 4 meters, non line of sight (79 channels)

Case 3: 4 to 10 meters (119 channels)

Statistics for the following channel parameters were computed in for each case:

- Number of paths required to meet the 85% energy capture threshold

- Mean excess delay

- RMS delay spread

- Probability of occupancy vs. excess delay

- Average power decay profile

For each of the above three cases, the statistics of these five parameters comprise a statistical model for the given scenario.

A comparison of the measurement data to simulated data produced by the IEEE 802.11 channel model, and the -K model was made, based on these same statistics.  On the basis of the statistics of the above channel parameters, both models were judged as insufficient for the purposes of modeling the UWB channel measurements.    It was recommended that a better model than the -K be used, and that any model should be judged by it’s ability to fit the statistics of the measured data.

### “Channel Modeling Sub-committee Report,” J. Foerster and Q. Li [3]

A method for evaluating the distance capability was proposed based on using a link budget analysis and a free space path model.  This would allow proposers to state the link margin that is available in order to compensate for path losses beyond free space, including distortion, floor or wall attenuation, multipath fading, and any other additional implementation losses.  Justification for using the Friis equation for path loss was provided as an apparent good approximation for various UWB waveforms.  A multipath model was also proposed, which was based on a number of measurements that were made in a condo setting.  The measurements were based upon a frequency sweep from 2-8 GHz yielding a minimum path resolution of 167 psec and included 870 channel realizations.  Distances from 1-20 meters were considered, which included both LOS and NLOS.  The main channel characteristics that were used to compare various indoor models included the mean excess delay, mean RMS delay, and mean number of significant paths defined as the mean number of paths within 10 dB of the peak multipath arrival.  Three channel models were considered: the Rayleigh tap delay line model (same as the one used in 802.11), the -K model, and the Saleh-Valenzuela (S-V) model.  The clustering of the multipath arrivals was observed in the measurements, which supported the use of the -K and S-V model.  The comparisons showed that the S-V model was able to best fit the measured channel characteristics.  In addition, the Rayleigh and lognormal amplitude distribution was compared with measurement data, and the results showed that the lognormal distribution best fit the characteristics of the measurement data.  Therefore, the final model that was proposed was the S-V model with a lognormal fading distribution on the amplitudes.  Model parameters were found that best fit the characteristics of the channel, including the cluster arrival rate, ray (intra-cluster) arrival rate, cluster decay factor, ray decay factor, and standard deviation of the lognormal distribution.

### “A proposal for a selection of indoor UWB path loss model,” G. Shor, et. al. [4]

The presentation describes the measurement campaign carried out by Oulu University. The measurements were taken using a network analyzer covering the 2-8 GHz band. The measurements were taken in Oulu University representing a European campus environment. The measurements considered different Rx and Tx antennas heights. The results show that a double slope path loss model is relevant also for wide band signals. The presentation includes the calculation of the slopes for single and double slope models for each of the measurement environments. The measurements will be used for further modeling of UWB path-loss and multi-path properties.

### “Radio Channel Model for Indoor UWB WPAN Environments,” J. Kunisch and J. Pamp [5]

The proposed radio channel model aims at WPAN investigations for indoor UWB environments. It has been derived from measurements that have been performed in an office environment with line-of-sight, non-line-of-sight, and intermediate conditions for intra-office and inter-office scenarios with omni antennas on both transmitter and receiver side. The frequency range covered by the measurements is 1 to 11 GHz; the transmitter-receiver separation was in the range 3 to 10 m. For a given transmitter position, transfer functions were measured on a rectangular area using a grid of receiver positions with a spacing of 1 cm to achieve spatial over sampling even at the highest frequencies in the band.

The model is based on the Saleh-Valenzuela (S-V) approach, i.e., a single impulse response is basically composed of exponential decay clusters to model dense multipath components.

A distinguishing feature of the UWB indoor radio channel is that certain individual echoes are recognizable and resolvable in the measurements, which correspond to dominant reflections or diffractions. In contrast to surrounding dense multipath contributions, these echoes exist individually over distances larger than a wavelength. Unlike the traditional S-V approach, the model accounts for such strong individual echoes. These echoes are generated according to a simple algorithm based on the imaging method, which is applied to an idealised, generic model room. Consequently, the model allows determining space-variant impulse responses; in particular, delay changes of individual echoes caused by receiver movement are accounted for. This means that the Doppler behaviour of the channel, as induced by the receiver movement, is reproduced by the model.

The model can be used to produce either baseband or passband impulse responses or transfer functions, for frequency bands not exceeding app. 1-11 GHz. A number of parameter profiles are available corresponding for some of the measured environments.

### “The Ultra-wideband Indoor Multipath Loss Model,” S. Ghassemzadeh and V. Tarokh [6]

The authors presented a simple statistical multipath model that is easily integrated with the path loss model. The model is based on over 300,000 UWB frequency responses at 712 locations in 23 homes. The model regenerates the statistical properties of the indoor channel with high accuracy. This model can be used for simulation and performance evaluation of any UWB system with nominal bandwidth less than or equal 1.25 GHz, and it can be upgraded with further measurements.

### “The Ultra-Wide Bandwidth Indoor Channel: from Statistical Model to Simulations,” A. Molisch, M. Win, and D. Cassioli [7]

The model submitted by Mitsubishi (Win, Cassioli, and Molisch) is a stochastic tapped-delay-line (STDL) model. The model is based on a measurement campaign in an office building in California. The measurements were done in baseband; the delay resolution is 2ns. The measurement procedure allows the separation of small-scale and large scale fading.

The most important properties of the model are the following: For the large-scale fading, we find that

- The power decay profile (averaged over the small-scale fading) is modeled by a single exponential decay; no multiple-clusters were observed.

- The decay constant is modeled as a stochastic variable, varying from room to room.

- The total received energy experiences a lognormal shadowing around the mean energy given by the path-loss law. For the small-scale fading, we find that

- The small-scale statistics of path gains follow Gamma distributions (equivalent to Nakagami fading of the amplitudes)

- The m-parameters of the Nakagami fading are truncated Gaussian variables with mean values and standard deviations decreasing with delay.

- The fading of the taps is uncorrelated.

Numerical values for all the parameters were given. It is noteworthy that the observed delay spread was around 40ns, which is quite a bit larger than e.g., in residential areas.

### “Channel Modeling Sub-committee Report,” J-M Cramer, R. Scholtz, and M. Win [8]

The main goal of this work was to develop an understanding of the indoor UWB propagation channel, including the time-of-arrival, angle-of-arrival and level distributions of a collection of received signals. To accomplish this, a set of algorithms, based upon the sensor-CLEAN algorithm, suitable for processing UWB signals incident on an array of sensors was developed. These techniques were applied to the measured propagation data. From this, models for the propagation of UWB signals in an indoor channel were generated.  The proposed channel model, based upon the measurements and analysis using the sensor-CLEAN algorithm, is based upon the Salah-Valenzuela (S-V) model which captured the clustering phenomenon observed in the data.  Parameters characterizing the S-V model are also presented.

The channel models presented in this work are based on a set of measurement made at a number of locations within an office building. It has been noted that the geometry of the situation and the building architecture can have a significant effect on the received signals.  Therefore, further work remains in the collection and processing of propagation data from different buildings, to increase the significance of and augment the results presented in this work. It is possible therefore, that the strongest contribution of this work is in the development of the processing algorithms, and that as more measurements are taken in different environments, the parameters of the UWB channel model presented here will change to reflect this new information.

### “UWB Propagation Phenomena,” Kai Siwiak [9]

A path loss model in the multipath environment is proposed based on multipath delay spread. The model provides a transition function between free space and another power law  based on a connection between measured multipath delay spread rms(d) and the propagation law. It leads to a theory for a generalized propagation law model, and also offers a better understanding of multipath dispersion. The theoretical model appends the multiplying factor

[1exp(-t0/rms(d))]

to free space propagation where rms(d) is the rms delay spread as a function of distance, and where rays arrive at intervals t0 on average. When rms(d) can be expressed as some power (2) of distance d, the multiplier generally can be expressed as a transition function

[1exp((dt/d )2)]

between free space square law propagation and  power propagation beyond the transition distance dt. Parameters dt =12 m and =3 seem appropriate for 802.15.3a selection purposes.

### “Propagation notes to P802.15 SG3a from IEEE Tutorial,” Kai Siwiak [10]

This contribution shows a fundamental system limit in UWB, 173.3dB/Hz, which is bounded by thermal noise, the 3.1-10.6 GHz band, and the FCC emission limit. It is shown that practical systems operate as much as 25 dB from the limit, and that consequently the 802.15.3a data rates can be achieved only in radio propagation path loss environments that are moderate or benign.

## Proposed UWB channel model

### Path Loss Model

The main purpose of this path loss model is to fairly compare different physical layer proposals at the target operating distances, and to ensure adequate performance of the final standard.  This can be done by simply adopting the free space path loss model and asking the proposers to provide the resulting link margin that will be available to make up for additional channel losses, implementation losses, waveform distortion, imperfect multipath energy capture, etc.  It will be up to the evaluators to determine whether or not the resulting link margin is suitable for the intended applications.  The table below identifies the parameters needed from the proposer and how those parameters could be used to compute the final link margin.  This model is based on the narrowband path loss calculations (known as the Friis transmission formula), and justification for its use is provided in [3].

The following table lists the parameters and equations that should be used by the proposers to demonstrate the ability of the PHY to close the link at the throughputs and target operating ranges desired for the standard.  The highlighted parameters below are up to the proposers to define, while all other parameters will be consistent with all proposals so easy comparisons can be made.

Table 1: Link Budget Analysis Table

| Parameter | Value | Value |
| --- | --- | --- |
| Throughput (Rb) | > 110 Mbps | > 200 Mbps |
| Average Tx power (PT) | dBm | dBm |
| Tx antenna gain (GT) | 0 dBi | 0 dBi |
| fc'=fminfmax: geometric center frequency of waveform (fmin and fmax are the -10 dB edges of the waveform spectrum) | Hz | Hz |
| Path loss at 1 meter (L1=20log10(4πfc'c)) c=3×108 m/s | dB | dB |
| Path loss at d m (L2=20log10(d)) | 20 dB at d=10 meters | 12 dB at d=4 meters |
| Rx antenna gain (GR) | 0 dBi | 0 dBi |
| Rx power (PR=PT+GT+GR−L1−L2 (dB)) | dBm | dBm |
| Average noise power per bit (N=−174+10*log10(Rb)) | dBm | dBm |
| Rx Noise Figure (NF) | 7 dB | 7 dB |
| Average noise power per bit (PN=N+NF) | dBm | dBm |
| Minimum Eb/No (S) | dB | dB |
| Implementation Loss1 (I) | dB | dB |
| Link Margin (M=PR−PN−S−I) | dB | dB |
| Proposed Min. Rx Sensitivity Level2 | dBm | dBm |

1 Implementation loss is defined here for the AWGN channel only, and could include such impairments as filter distortion, phase noise, frequency errors, etc.

2 The minimum Rx sensitivity level is defined as the minimum required average Rx power for a received symbol in AWGN, and should include effects of code rate and modulation.

Although the proposers may need to alter the above table for their specific UWB PHY proposal, it gives the minimum justification and detail that should be part of the proposal.  Proposers are encouraged to give detailed justifications for their parameter values.  The final desired output of the link budget should be a final Link Margin that will be needed to account for additional losses due to the multipath channels such as additional path loss, additional implementation losses, additional waveform distortion, imperfect multipath energy capture, amplitude fading, etc.  Finally, evaluators are encouraged to reference indoor path loss measurements contributed in [1, 4, 9, and 10] to estimate required link margins for particular applications and deployment scenarios that may include NLOS propagation.

### Multipath Model

Based on this clustering phenomenon observed in several channel measurements, we propose an UWB channel model derived from the Saleh-Valenzuela model [11] with a couple of slight modifications.  We recommend using a lognormal distribution rather than a Rayleigh distribution for the multipath gain magnitude, since our observations show that the lognormal distribution seems to better fit the measurement data.  In addition, independent fading is assumed for each cluster as well as each ray within the cluster.  Therefore, the multipath model consists of the following, discrete time impulse response:

hi(t)=Xil=0Lk=0Kαk,liδ(t−Tli−τk,li)

where {αk,li} are the multipath gain coefficients, {Tli} is the delay of the lth cluster,  {τk,li} is the delay of the kth multipath component relative to the lth cluster arrival time (Tli),{Xi} represents the log-normal shadowing, and i refers to the ith realization.

Finally, the proposed model uses the following definitions:

Tl = the arrival time of the first path of the l-th cluster;

k,l = the delay of the k-the path within the l-th cluster relative to the first path arrival time, Tl;

 = cluster arrival rate;

 = ray arrival rate, i.e., the arrival rate of path within each cluster.

By definition, we have τ0,l=0.  The distribution of cluster arrival time and the ray arrival time are given by

The channel coefficients are defined as follows:

αk,l=pk,lξlβk,l,

20log10(ξlβk,l)∝Normal(μk,l,σ12+σ22), or ξlβk,l=10(μk,l+n1+n2)20

where n1∝Normal(0,σ12) and n2∝Normal(0,σ22) are independent and correspond to the fading on each cluster and ray, respectively,

Eξlβk,l2=Ω0e−TlΓe−τk,lγ,

where Tl is the excess delay of bin l and Ω0 is the mean energy of the first path of the first cluster, and pk,l is equiprobable +/-1 to account for signal inversion due to reflections. The k,l is given by

μk,l=10ln(Ω0)−10TlΓ−10τk,lγln(10)−(σ12+σ22)ln(10)20

In the above equations, ξl reflects the fading associated with the lth cluster, and βk,l corresponds to the fading associated with the kth ray of the lth cluster.  Note that, a complex tap model was not adopted here.  The complex baseband model is a natural fit for narrowband systems to capture channel behavior independently of carrier frequency, but this motivation breaks down for UWB systems where a real-valued simulation at RF may be more natural.

Finally, since the log-normal shadowing of the total multipath energy is captured by the term, Xi, the total energy contained in the terms {αk,li} is normalized to unity for each realization.  This shadowing term is characterized by the following:

20log10(Xi)∝Normal(0,σx2).

## Realizations from channel model

### Channel characteristics desired to model

As shown above, there are 6 key parameters that define the model:

 = cluster arrival rate;

 = ray arrival rate, i.e., the arrival rate of path within each cluster;

 = cluster decay factor;

 = ray decay factor;

σ1= standard deviation of cluster lognormal fading term (dB).

σ2= standard deviation of ray lognormal fading term (dB).

σx= standard deviation of lognormal shadowing term for total multipath realization (dB).

These parameters are found by trying to match important characteristics of the channel.  Since it’s difficult to match all possible channel characteristics, the main characteristics of the channel that are used to derive the above model parameters were chosen to be the following:

- Mean excess delay

- RMS delay spread

- Number of multipath components (defined as the number of multipath arrivals that are within 10 dB of the peak multipath arrival)

- Power decay profile

Since the model parameters are difficult to match to the average power decay profile, the main channel characteristics that are used to determine the model parameters are the first three above.  The following table lists some initial model parameters for a couple of different channel characteristics that were found through measurement data.

Table 2 Example multipath channel characteristics and corresponding model parameters.

| Target Channel Characteristics5 | CM 11 | CM 22 | CM 33 | CM 44 |
| --- | --- | --- | --- | --- |
| Mean excess delay (nsec) (τm) | 5.05 | 10.38 | 14.18 |  |
| RMS delay (nsec) (τrms) | 5.28 | 8.03 | 14.28 | 25 |
| NP10dB |  |  | 35 |  |
| NP (85%) | 24 | 36.1 | 61.54 |  |
| Model Parameters |  |  |  |  |
|  (1/nsec) | 0.0233 | 0.4 | 0.0667 | 0.0667 |
|  (1/nsec) | 2.5 | 0.5 | 2.1 | 2.1 |
|  | 7.1 | 5.5 | 14.00 | 24.00 |
|  | 4.3 | 6.7 | 7.9 | 12 |
| σ1 (dB) | 3.3941 | 3.3941 | 3.3941 | 3.3941 |
| σ2 (dB) | 3.3941 | 3.3941 | 3.3941 | 3.3941 |
| σx (dB) | 3 | 3 | 3 | 3 |
| Model Characteristics5 |  |  |  |  |
| Mean excess delay (nsec) (τm) | 4.9 | 9.4 | 13.8 | 26.8 |
| RMS delay (nsec) (τrms) | 5 | 8 | 14 | 26 |
| NP10dB | 13.3 | 18.2 | 25.3 | 41.4 |
| NP (85%) | 21.4 | 37.2 | 62.7 | 122.8 |
| Channel energy mean (dB) | -0.5 | 0.1 | 0.2 | 0.1 |
| Channel energy std (dB) | 2.9 | 3.3 | 3.4 | 3.2 |

1 This model is based on LOS (0-4m) channel measurements reported in [2].

2 This model is based on NLOS (0-4m) channel measurements reported in [2].

3 This model is based on NLOS (4-10m) channel measurements reported in [2], and NLOS measurements reported in [3].

4 This model was generated to fit a 25 nsec RMS delay spread to represent an extreme NLOS multipath channel.

5 These characteristics are based upon a 167 psec sampling time.

One hundred (100) actual realizations for each channel model are derived from the model above and are provide in this folder.

### Discrete Time Model

The output of the model is a continue time arrival and amplitude value.  The following section describes how this sampling time can be discretized in a consistent manner without losing the essence of the multipath model.

### How to handle different sample times

First, just quantize time in the continuous-time model's (time,value) pairs to ts/N where ts is the desired resolution and N is a suitably large integer.  If multiple (time,value) pairs get quantized into the same time bin, their values should be added.  Next, in normal discrete-time processing, do low-pass or band-pass or transmit-pulse filtering as appropriate (including complex frequency down conversion if desired) and decimate down by N.  A suggested rule for choosing N is to ensure that N*fs=N/ts is at least 100 GHz and that N≥1.  Thus, N=max(1, ceil(ts * 100 GHz)).  The MatlabTM code provided in the appendix provides a means for achieving this for a specified desired sample time ts, which is the only parameter that needs change in the code. The current code has anti-aliasing filtering before decimation, the user may add extra filtering or complex down-conversion as appropriate.

### Channel Realizations

The following statistics are based upon a sampling time of 167 psec and an N value specified above.

#### CM 1

#### CM 2

#### CM 3

#### CM 4

## Time variability of channel

Due to the lack of empirical data characterizing the time variation expected in UWB channels, the channel is assumed to be time-invariant during the transmission of each packet.  Regarding channel correlation from packet to packet, the model is agnostic as reflected further in Section 3.5.1.1 which recommends that PHY proposals be evaluated without any a priori information about channel correlation.  For proposals that wish to take advantage of channel correlation, proposers are required to describe a suitable model for use in evaluating the performance of their proposal in a realistic environment, and are encouraged to build upon the ideas presented in contributions [11] and [12].

## How to use the model and realizations

### Meeting the Selection Criteria

#### PER Determination

The following approach should be used to determine the required Eb/No to meet a desired PER < 8e-2 and BER < 1e-5.  It is suggested that at least 200, 1024 byte packets be simulated for each channel realization (100 channel realizations in total), and the receiver should assume no apriori knowledge of the channel upon detection of each packet.  The results should be provided in the following ways:

- Provide a single curve of PER vs. Eb/No that represents the average PER, averaged over the best 90 out of 100 channel realizations, for each of the four channel environments.

#### Time varying channels

For systems that rely on time correlation in the channel impulse response, the proposers should provide a means of evaluating the performance of the system in a realistic, time-varying channel.  Some possible suggestions are provided in contributions [11] and [12], and the proposers are encouraged to build upon these ideas to evaluate their proposals.

### Level of disclosure desired by PHY proposals

Clearly, the performance of any PHY partly depends on the receiver implementation, which is outside the scope of the standard.  However, in order to properly evaluate the relative merits and complexity required for the different PHY proposals in a multipath channel, it is desired to have an understanding of the level of complexity needed in the receiver in order to achieve the provided performance results.  Therefore, it is desired that the proposers provide, at a minimum, the following receiver characteristics that was able to achieve the given results:

- Complexity of receiver (number of gates, die area required, or other parameters that help quantify the receiver complexity)

- Power consumption of the receiver

In addition, it would be desirable, although not required, to provide additional information relating to the complexity of the receiver.  For example, some additional items could include:

- Number of taps in an equalizer, if used

- Rate at which the equalizer needs to run (minimum clock rate)

- Equalization algorithm used (LMS, RLS, etc.)

# List of Contributors

The following have participated in the 802.15 SG3a Channel Model Sub-Committee activities.

| Roberto Aiello | Naiel Askar | Edul Batliwala |
| --- | --- | --- |
| Anuj Batra | Bill Beeler | Stan Bottoms (secretary) |
| Jean-Marc Cramer (presentation) | Anond Dubak | Michael Dydyk |
| Jeff Foerster (chair, presenter) | Saeed Ghassemzadeh (presenter) | Jeyhan Karaoguz |
| Joy Kelly | Jurgen Kunisch (presenter) | Dave Leeper |
| Andy Molisch (presenter) | Marcus Pendergrass (presenter) | Henry Nielsen |
| Ivan Reede | Glyn Roberts | Rick Roberts |
| Steve Schell (presenter, Matlab code contributor) | Gadi Shor (presenter) | Kai Siwiak (presenter) |
| Matt Welborn | Hirohisa Yamaguchi | Anthony Zwilling |

# References

Responses to the Call for Contributions on UWB Channel Models:

[1] 	S. Ghassemzadeh and V. Tarokh, “The Ultra-wideband Indoor Path Loss Model,” IEEE

P802.15-02/277-SG3a and IEEE P802.15-02/278-SG3a.

[2]	M. Pendergrass, “Empirically Based Statistical Ultra-Wideband Channel Model,” IEEE

P802.15-02/240-SG3a.

[3]	J. Foerster and Q. Li, “Channel Modeling Sub-committee Report,” IEEE P802.15-

02/279-SG3a.

[4]	G. Shor, et. al., “A proposal for a selection of indoor UWB path loss model,” IEEE

P802.15-02/280-SG3a.

[5]	J. Kunisch and J. Pamp, “Radio Channel Model for Indoor UWB WPAN Environments,”

IEEE P802.15-02/281-SG3a.

[6] 	S. Ghassemzadeh and V. Tarokh, “The Ultra-wideband Indoor Multipath Loss Model,”

IEEE P802.15-02/282-SG3a and IEEE P802.15-02/283-SG3a.

[7]	A. Molisch, M. Win, and D. Cassioli, “The Ultra-Wide Bandwidth Indoor Channel: from

Statistical Model to Simulations,” IEEE P802.15-02/284-SG3a and IEEE P802.15-

02/285-SG3a.

[8]	J-M Cramer, R. Scholtz, M. Win, “Channel Modeling Sub-committee Report,” IEEE P802.15-02/286-SG3a and IEEE P802.15-02/325-SG3a.

[9]	Kai Siwiak, “UWB Propagation Phenomena,” IEEE P802.15-02/301-SG3a.

[10]	Kai Siwiak, “Propagation notes to P802.15 SG3a from IEEE Tutorial,” IEEE P802.15-

02/328-SG3a.

Additional Contributions

[11] 	Stephan V. Schell, "Analysis of Time Variance of a UWB Propagation

Channel", IEEE P802.15-02/452-SG3a and IEEE P802.15-02/453-SG3a.

[12]	A. Molisch, “Time variance for UWB wireless channels,” IEEE P802.15-02/461-SG3a

and IEEE P802.15-02/462-SG3a.

[13]	G. Valera, M. Lobeira, R.P. Torres, J.L. Garcia

, and A. Alvarez

“UWB Channel

Modeling Contribution from University of Cantabria and ACORDE”, IEEE P802.15-

02/445-SG3a.

[14]	J.Keignart, JB.Pierrot, N.Daniele

, and Ph.Rouzet

, “UWB Channel Modeling Contribution

from CEA-LETI and STMicroelectronics”, IEEE P802.15-02/444-SG3a.

Other relevant references:

[15]	A. Saleh and R. Valenzuela, “A Statistical Model for Indoor Multipath

Propagation,” IEEE JSAC, Vol. SAC-5, No. 2, Feb. 1987, pp. 128-137.

[16]	T. S. Rappaport and S. Sandhu, “Radio-Wave Propagation for Emerging Wireless Personal Communication Systems,” IEEE Antennas and Propagation Magazine, Vol. 36, No. 5, pg. 14-24, Oct. 1994 and the references therein.

[17]	K. Pahlavan and A. Levesque, Wireless Information Networks, John Wiley and

Sons, 1995.

[18]	K-W Cheung, J. Sau, and R. Murch, “A New Empirical Model for Indoor Propagation Prediction,” IEEE Trans. On Vehic. Tech.,Vol. 47, No. 3, pp. 996-1001, Aug. 1998.

[19]	J.M. Cramer, R.A. Scholtz, and M.Z. Win, “On the analysis of UWB communication channel,” Proceedings of MILCOM 1999, Vol. 2,  pp. 1191-1195, 1999.

[20]	IEEE 802.15.2 channel model for measuring coexistence.

[21]	S. Ghassemzadeh, R. Jana, C. Rice, W. Turin, and V. Tarokh, “A Statistical Path Loss

Model for In-Home UWB Channels,” IEEE UWBST, May 2002.

[22]	S. Soliman, “Report of Qualcomm Incorporated,” In the matter of revision of Part 15 of

the Commission’s Rules Regarding Ultra-Wideband Transmissions Systems, ET Docket

No. 98-153, March 5, 2001.

[23]	H. Hashemi, “Impulse Response Modeling of Indoor Radio Propagation

Channels,” IEEE JSAC, Vol. 11, No. 7, Sept. 1993, pp. 967-978.

[24]	H. Suzuki, “A Statistical Model for Urban Radio Propagation,” IEEE Transactions on

Communications, pp. 673-680, July 1977.

# Appendix

Following is the MatlabTM code that was used to generate the channel model realizations (generated by contributions from Jeff Foerster and Steve Schell).

######################  Main program

% S-V channel model evaluation

clear;

no_output_files = 0;  % non-zero: avoids writing output files of continuous-time responses

ts = 0.167;             % sampling time (nsec)

num_channels = 100;      % number of channel impulse responses to generate

randn('state',12);    % initialize state of function for repeatability

rand('state',12);     % initialize state of function for repeatability

cm_num = 1;  % channel model number from 1 to 4

% get channel model params based on this channel model number

[Lam,lambda,Gam,gamma,std_ln_1,std_ln_2,nlos,std_shdw] = uwb_sv_params( cm_num );

fprintf(1,['Model Parameters\n' ...

'  Lam = %.4f, lambda = %.4f, Gam = %.4f, gamma = %.4f\n' ...

'  std_ln_1 = %.4f, std_ln_2 = %.4f, NLOS flag = %d, std_shdw = %.4f\n'], ...

Lam, lambda, Gam, gamma, std_ln_1, std_ln_2, nlos, std_shdw);

% get a bunch of realizations (impulse responses)

[h_ct,t_ct,t0,np] = uwb_sv_model_ct( Lam, lambda, Gam, gamma, std_ln_1, std_ln_2, nlos, ...

std_shdw, num_channels );

% now reduce continuous-time result to a discrete-time result

[hN,N] = uwb_sv_cnvrt_ct( h_ct, t_ct, np, num_channels, ts );

% if we wanted complex baseband model or to impose some filtering function,

% this would be a good place to do it

if N > 1,

h = resample(hN, 1, N);  % decimate the columns of hN by factor N

else

h = hN;

end

% correct for 1/N scaling imposed by decimation

h = h * N;

% channel energy

channel_energy = sum(abs(h).^2);

h_len = size(h,1);

t = [0:(h_len-1)] * ts;  % for use in computing excess & RMS delays

excess_delay = zeros(1,num_channels);

RMS_delay = zeros(1,num_channels);

num_sig_paths = zeros(1,num_channels);

num_sig_e_paths = zeros(1,num_channels);

for k=1:num_channels

% determine excess delay and RMS delay

sq_h = abs(h(:,k)).^2 / channel_energy(k);

t_norm = t - t0(k);  % remove the randomized arrival time of first cluster

excess_delay(k) = t_norm * sq_h;

RMS_delay(k) = sqrt( ((t_norm-excess_delay(k)).^2) * sq_h );

% determine number of significant paths (paths within 10 dB from peak)

threshold_dB = -10;   % dB

temp_h = abs(h(:,k));

temp_thresh = 10^(threshold_dB/20) * max(temp_h);

num_sig_paths(k) = sum(temp_h > temp_thresh);

% determine number of sig. paths (captures x % of energy in channel)

x = 0.85;

temp_sort = sort(temp_h.^2);  % sorted in ascending order of energy

cum_energy = cumsum(temp_sort(end:-1:1));  % cumulative energy

index_e = min(find(cum_energy >= x * cum_energy(end)));

num_sig_e_paths(k) = index_e;

end

energy_mean = mean(10*log10(channel_energy));

energy_stddev = std(10*log10(channel_energy));

mean_excess_delay = mean(excess_delay);

mean_RMS_delay = mean(RMS_delay);

mean_sig_paths = mean(num_sig_paths);

mean_sig_e_paths = mean(num_sig_e_paths);

fprintf(1,'Model Characteristics\n');

fprintf(1,'  Mean delays: excess (tau_m) = %.1f ns, RMS (tau_rms) = %1.f\n', ...

mean_excess_delay, mean_RMS_delay);

fprintf(1,'  # paths: NP_10dB =  %.1f, NP_85%% = %.1f\n', ...

mean_sig_paths, mean_sig_e_paths);

fprintf(1,'  Channel energy: mean = %.1f dB, std deviation = %.1f dB\n', ...

energy_mean, energy_stddev);

figure(1); clf; plot(t,h); grid on

title('Impulse response realizations')

xlabel('Time (nS)')

figure(2); clf; plot([1:num_channels], excess_delay, 'b-', ...

[1 num_channels], mean_excess_delay*[1 1], 'r--' );

grid on

title('Excess delay (nS)')

xlabel('Channel number')

figure(3); clf; plot([1:num_channels], RMS_delay, 'b-', ...

[1 num_channels], mean_RMS_delay*[1 1], 'r--' );

grid on

title('RMS delay (nS)')

xlabel('Channel number')

figure(4); clf; plot([1:num_channels], num_sig_paths, 'b-', ...

[1 num_channels], mean_sig_paths*[1 1], 'r--');

grid on

title('Number of significant paths within 10 dB of peak')

xlabel('Channel number')

figure(5); clf; plot([1:num_channels], num_sig_e_paths, 'b-', ...

[1 num_channels], mean_sig_e_paths*[1 1], 'r--');

grid on

title('Number of significant paths capturing > 85% energy')

xlabel('Channel number')

temp_average_power = sum(h'.*(h)')/num_channels;

temp_average_power = temp_average_power/max(temp_average_power);

average_decay_profile_dB = 10*log10(temp_average_power);

figure(6); clf; plot(t,average_decay_profile_dB); grid on

axis([0 t(end) -60 0])

title('Average Power Decay Profile')

xlabel('Delay (nsec)')

ylabel('Average power (dB)')

figure(7); clf

figh = plot([1:num_channels],10*log10(channel_energy),'b-', ...

[1 num_channels], energy_mean*[1 1], 'g--', ...

[1 num_channels], energy_mean+energy_stddev*[1 1], 'r:', ...

[1 num_channels], energy_mean-energy_stddev*[1 1], 'r:');

xlabel('Channel number')

ylabel('dB')

title('Channel Energy');

legend(figh, 'Per-channel energy', 'Mean', '\pm Std. deviation', 0)

if no_output_files,

return

end

%%% save continuous-time (time,value) pairs to files

save_fn = sprintf('cm%d_imr', cm_num);

% A complete self-contained file for Matlab users

save([save_fn '.mat'], 't_ct', 'h_ct', 't0', 'np', 'num_channels', 'cm_num');

% Two comma-delimited text files for non-Matlab users:

% File #1: cmX_imr_np.csv lists the number of paths in each realization

dlmwrite([save_fn '_np.csv'], np, ',');  % number of paths

% File #2: cmX_imr.csv can open with Excel

%     n'th pair of columns contains the (time,value) pairs for the n'th realization

th_ct = zeros(size(t_ct,1),2*size(t_ct,2));

th_ct(:,1:2:end) = t_ct;  % odd columns are time

th_ct(:,2:2:end) = h_ct; % even columns are values

fid = fopen([save_fn '.csv'], 'w');

if fid < 0,

error('unable to write .csv file for impulse response, file may be open in another application');

end

for k = 1:size(th_ct,1)

fprintf(fid,'%.4f,%.6f,', th_ct(k,1:end-2));

fprintf(fid,'%.4f,%.6f\r\n', th_ct(k,end-1:end)); % \r\n for Windoze end-of-line

end

fclose(fid);

return; % end of program

###################### end

###################### Function uwb_sv_model_ct

function [h,t,t0,np] = uwb_sv_model_ct(Lam, lambda, Gam, gamma, std_ln_1, std_ln_2, nlos, ...

std_shdw, num_channels)

% IEEE 802.15.3a UWB channel model for PHY proposal evaluation

%      continuous-time realization of modified S-V channel model

% Input parameters:

%   Lam      Cluster arrival rate in GHz (avg # of clusters per nsec)

%   lambda   Ray arrival rate in GHz (avg # of rays per nsec)

%   Gam      Cluster decay factor (time constant, nsec)

%   gamma    Ray decay factor (time constant, nsec)

%   std_ln_1 Standard deviation of log-normal variable for cluster fading

%   std_ln_2 Standard deviation of log-normal variable for ray fading

%   nlos     Flag to specify generation of Non Line Of Sight channels

%   std_shdw  Standard deviation of log-normal shadowing of entire impulse response

%   num_channels  number of random realizations to generate

% Outputs

%   h is returned as a matrix with num_channels columns, each column

%     holding a random realization of the channel model (an impulse response)

%   t is organized as h, but holds the time instances (in nsec) of the paths whose

%     signed amplitudes are stored in h

%   t0 is the arrival time of the first cluster for each realization

%   np is the number of paths for each realization.

% Thus, the k'th realization of the channel impulse response is the sequence

%   of (time,value) pairs given by (t(1:np(k),k), h(1:np(k),k))

% initialize and precompute some things

std_L = 1/sqrt(2*Lam);      % std dev (nsec) of cluster arrival spacing

std_lam = 1/sqrt(2*lambda); % std dev (nsec) of ray arrival spacing

mu_const = (std_ln_1^2+std_ln_2^2)*log(10)/20;  % pre-compute for later

h_len = 1000;  % there must be a better estimate of # of paths than this

ngrow = 1000; % amount to grow data structure if more paths are needed

h = zeros(h_len,num_channels);

t = zeros(h_len,num_channels);

t0 = zeros(1,num_channels);

np = zeros(1,num_channels);

for k = 1:num_channels       % loop over number of channels

tmp_h = zeros(size(h,1),1);

tmp_t = zeros(size(h,1),1);

if nlos,

Tc = (std_L*randn)^2 + (std_L*randn)^2;  % First cluster random arrival

else

Tc = 0;                   % First cluster arrival occurs at time 0

end

t0(k) = Tc;

ln_xi = std_ln_1*randn;   % set cluster fading

path_ix = 0;

while (Tc < 10*Gam)

% Determine Ray arrivals for each cluster

Tr = 0;  % first ray arrival defined to be time 0 relative to cluster

while (Tr < 10*gamma)

t_val = (Tc+Tr);  % time of arrival of this ray

mu = (-10*Tc/Gam-10*Tr/gamma)/log(10) - mu_const;

ln_beta = mu + std_ln_2*randn;

pk = 2*round(rand)-1;

h_val = pk * 10^((ln_xi+ln_beta)/20);  % signed amplitude of this ray

path_ix = path_ix + 1;  % row index of this ray

if path_ix > h_len,

% grow the output structures to handle more paths as needed

%        fprintf(1,'Growing CIR length from %d paths to %d\n', length(tmp_h)+[0 ngrow]);

tmp_h = [tmp_h; zeros(ngrow,1)];

tmp_t = [tmp_t; zeros(ngrow,1)];

h = [h; zeros(ngrow,num_channels)];

t = [t; zeros(ngrow,num_channels)];

h_len = h_len + ngrow;

end

tmp_h(path_ix) = h_val;

tmp_t(path_ix) = t_val;

Tr = Tr + (std_lam*randn)^2 + (std_lam*randn)^2;

end

Tc = Tc + (std_L*randn)^2 + (std_L*randn)^2;

end

np(k) = path_ix;  % number of rays (or paths) for this realization

[sort_tmp_t,sort_ix] = sort(tmp_t(1:np(k)));    % sort in ascending time order

t(1:np(k),k) = sort_tmp_t;

h(1:np(k),k) = tmp_h(sort_ix(1:np(k)));

% now impose a log-normal shadowing on this realization

fac = 10^(std_shdw*randn/20) / sqrt( h(1:np(k),k)' * h(1:np(k),k) );

h(1:np(k),k) = h(1:np(k),k) * fac;

end

return

###################### end

###################### Function uwb_sv_params

function [Lam,lambda,Gam,gamma,std_ln_1,std_ln_2,nlos,std_shdw] = uwb_sv_params( cm_num )

% Return S-V model parameters for standard UWB channel models

% Lam    Cluster arrival rate (clusters per nsec)

% lambda Ray arrival rate (rays per nsec)

% Gam    Cluster decay factor (time constant, nsec)

% gamma  Ray decay factor (time constant, nsec)

% std_ln_1 Standard deviation of log-normal variable for cluster fading

% std_ln_2 Standard deviation of log-normal variable for ray fading

% nlos   Flag for non line of sight channel

% std_shdw  Standard deviation of log-normal shadowing of entire impulse response

if cm_num == 1,        % based on TDC measurements for LOS 0-4m

Lam = 0.0233;  lambda = 2.5;

Gam = 7.1; gamma = 4.3;

std_ln_1 = 4.8 / sqrt(2);

std_ln_2 = 4.8 / sqrt(2);

nlos = 0;

std_shdw = 3;

elseif cm_num == 2,    % based on TDC measurements for NLOS 0-4m

Lam = 0.4;  lambda = 0.5;

Gam = 5.5; gamma = 6.7;

std_ln_1 = 4.8 / sqrt(2);

std_ln_2 = 4.8 / sqrt(2);

nlos = 1;

std_shdw = 3;

elseif cm_num == 3,    % based on TDC measurements for NLOS 4-10m

Lam = 0.0667;  lambda = 2.1;

Gam = 14.00; gamma = 7.9;

std_ln_1 = 4.8 / sqrt(2);

std_ln_2 = 4.8 / sqrt(2);

nlos = 1;

std_shdw = 3;

elseif cm_num == 4,    % 25 nsec RMS delay spread bad multipath channel

Lam = 0.0667;  lambda = 2.1;

Gam = 24; gamma = 12;

std_ln_1 = 4.8 / sqrt(2);

std_ln_2 = 4.8 / sqrt(2);

nlos = 1;

std_shdw = 3;

end

return

###################### end

###################### Function uwb_sv_cnvrt_ct

function [hN,N] = uwb_sv_cnvrt_ct( h_ct, t, np, num_channels, ts )

% convert continuous-time channel model h_ct to N-times oversampled discrete-time samples

% h_ct, t, np, and num_channels are as specified in uwb_sv_model

% ts is the desired time resolution

%

% hN will be produced with time resolution ts / N.

% It is up to the user to then apply any filtering and/or complex downconversion and then

% decimate by N to finally obtain an impulse response at time resolution ts.

min_Nfs = 100; % GHz

N = max( 1, ceil(min_Nfs*ts) );  % N*fs = N/ts is the intermediate sampling frequency before decimation

N = 2^nextpow2(N); % make N a power of 2 to facilitate efficient multi-stage decimation

% NOTE: if we force N = 1 and ts = 0.167, the resulting channel hN will be identical to

% the results from earlier versions that did not use continuous-time.

Nfs = N / ts;

t_max = max(t(:));  % maximum time value across all channels

h_len = 1 + floor(t_max * Nfs);  % number of time samples at resolution ts / N

hN = zeros(h_len,num_channels);

for k = 1:num_channels

np_k = np(k);  % number of paths in this channel

t_Nfs = 1 + floor(t(1:np_k,k) * Nfs);  % vector of quantized time indices for this channel

for n = 1:np_k

hN(t_Nfs(n),k) = hN(t_Nfs(n),k) + h_ct(n,k);

end

end

###################### end