IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | SG3a Alt PHY Selection Criteria |  |
| Date Submitted | 18 September 2002, rev18; xx September 2002, rev 19 |  |
| Source | [Kai Siwiak, technical editor] [Time Domain] [Jason Ellis- technical editor alternate] [General Atomics] | Voice: [256-990-9062] E-mail: [kai.siwiak@timedomain.com E-mail: [jason.ellis@ga.com] Voice: [+1 858 457 8749] |
| Re: |  |  |
| Abstract | [Definitions for the proposal evaluation for Task Group 3a] |  |
| Purpose | [This is a working document that will become the repository for the terms and definitions to be used in the selection process for a Draft Standard for TG3.] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

TABLE OF CONTENTS

1.	Introduction	6

2.	References	6

3.	General Solution Criteria	6

3.1.	Unit Manufacturing Complexity (UMC)	6

3.1.1.	Definition	6

3.1.2.	Values	7

3.2.	Signal Robustness	8

3.2.1.	General Definitions	8

3.2.2.	Interference and Susceptibility	8

3.2.3.	Coexistence	14

3.3.	Technical Feasibility	17

3.3.1.	Manufacturability	17

3.3.2.	Time to Market	17

3.3.3.	Regulatory Impact	17

3.4.	Scalability	19

3.4.1.	Definition	19

3.4.2.	Values	19

3.5.	Location Awareness	19

3.5.1.	Definition	19

3.5.2.	Values	19

4.	MAC Protocol Supplements	20

4.1.	Required MAC enhancements and modifications to  accommodate Alternate PHY	20

4.1.1.	Definition	20

4.1.2.	Values	20

5.	PHY Layer Criteria	20

5.1.	Size and Form Factor	20

5.1.1.	Definition	20

5.1.2.	Values	20

5.2.	PHY-SAP Payload Bit Rate and Data Throughput	21

5.2.1.	Minimum Receive Payload Bit Rate	21

5.2.2.	PHY-SAP Data Throughput	21

5.3.	Simultaneously Operating Piconets	23

5.3.1.	Definition	23

5.3.2.	Values	23

5.4.	Signal Acquisition Timeline	23

5.4.1.	Definition	23

5.4.2.	Values	23

5.5.	Link budget	24

5.5.1.	Definition	24

5.5.2.	Values	24

5.6.	Sensitivity	24

5.6.1.	Definition	24

5.6.2.	Values	24

5.7.	Multi-Path Immunity	24

5.7.1.	Environment model	24

5.7.2.	Delay Spread Tolerance	24

5.8.	Power Management Modes	25

5.8.1.	Definition	25

5.8.2.	Values	25

5.9.	Power Consumption	25

5.9.1.	Definition	25

5.9.2.	Value	26

5.10.	Antenna Practicality	26

5.10.1.	Definition	27

5.10.2.	Value	27

# Introduction

This is the criteria for the selection of the alternate PHY Draft Proposals.  In order to accurately and consistently judge the submitted proposals, technical requirements are needed that reflect the application scenarios that were contributed in response to the call for applications.

This working document will become the repository for the requirements to be used in the selection process for an alternate PHY Draft Standard for 802.15.3a.  The criteria presented in this document are based on document [02/104], which takes precedence, and may also contain more general marketing requirements on which the proposers are asked to comment.

The document is divided into three sections: General Solution Criteria, MAC Protocol Supplements Criteria, and PHY Layer Criteria.  An evaluation matrix in document 02/365 additionally provides the summary of criteria assessments expected with each proposal.

This document and the Requirements document 02/104 provide the technical content for the project to develop an alternate physical layer (alt-PHY). This alt-PHY shall be a supplement to the proposed IEEE 802.15.3 Standard. Revision 15 of the Selection Criteria Document references draft 10 of the proposed IEEE 802.15.3 Standard.

# References

Draft 10 of the proposed IEEE 802.15.3 Standard

IEEE P802.15-02/104, SG3a Technical Requirements

IEEE P802.15-02/365, SG3a Evaluation Matrix of Selection Criteria

IEEE P802.15-02/368, SG3a Channel Modeling Subcommittee Report

# General Solution Criteria

This section defines the technical and marketing system level concerns of the solution.

## Unit Manufacturing Complexity (UMC)

### Definition

The complexity of the device must be as minimal as possible for use in the personal area space, see [02/104].  Figure 1 illustrates the logical blocks in the transceiver PHY layer.  Not all blocks are required to implement a communications system.  However, if the functionality is used (even optionally) in the specification, then the complexity for implementing the functionality must be included in the estimate.  The order and contents of the blocks may vary, for example, the frequency spreading may be a part of the modulate/demodulate portion, and the encode/decode operations might split out to ‘source encode/decode’ and ‘channel encode/decode’.

Figure 1:  Logical blocks in the transceiver PHY layer

- Encode/Decode – packet formation including headers, data interleaving, error correction/detection (FEC, CRC, etc), compression/decompression, bias suppression, symbol spreading/de-spreading (DSSS), data whitening/de-whitening (or scrambling).  Modulate/Demodulate – convert digital data to analog format, can include symbol filtering, frequency conversion, frequency filtering.

- Frequency Spreading/De-spreading – can include techniques to increase or decrease, respectively, the Hz/bit of the analog signal in the channel.

- Transmit/Receive – transition the signal to/from the channel.

### Values

Complexity estimates should be provided in terms of both analog and digital die size estimates, semiconductor processes, specified year for process technologies, gate count estimates, and major external components. {Additionally, the proposer should specify how the previous quantities scale with performance where lower performance devices are interoperable on the same piconet as higher performing devices.—EDITOR NOTE “This sentence is unresolved. The first sentence has been accepted.”} Similar considerations should be made with regard to MAC enhancements. Reasonable and conservative values should be given. Relative comparisons to existing technologies are acceptable.

## Signal Robustness

### General Definitions

An error rate criterion is the maximum bit error rate (BER). Another error rate criterion is the maximum packet error rate (PER) for a specified packet length. The proposer will be asked to indicate both the BER, and the corresponding PER, see Sections 2 and 7 of [02/104] used in the determination of this value when indicating the sensitivity of the proposed device. Payload size for the PER test is called out in Section 2 of [02/104] and is intended to be a value between the minimum and maximum packet size.

The minimum required sensitivity is the power level of a signal, in dBm, present at the input of the receiver modulated by the proposed method with pseudo-random data for which the error rate criterion is met.  The proposer should include all the calculations used to determine the sensitivity. The link budget details should be provided in a worksheet format to include such detail as the assumed noise figure and antenna gain. The power level should be specified at the antenna to receiver connection (i.e. it should not include any antenna gain).  The error ratio should be determined at the PHY-SAP interface, after any error correction methods required in the proposed device have been applied. Devices may exceed the minimum required sensitivity, but the measurements in Section 3.2 are taken relative to the minimum value specified in the proposal.

The PHY-SAP peer-to-peer data throughput of the device is the net amount of data that is transferred from one PHY SAP to another.  The elapsed time should be at least 1 second. The connection should already have been established and in progress prior to the 1 second interval. The units of the data throughput are in Mb/s. The packet length should be that referenced in document 02/104, section 2, and the throughput should include the normal overhead associated with a packet transmission. Unless otherwise noted, the 802.15.3a transceivers are assumed to use 0 dBi antennas.

### Interference and Susceptibility

      - Definition

Interference susceptibility refers to the impact other co-located intentional and unintentional radiators may have on a proposed physical layer solution.  This section is mainly concerned with the interference coming from other non-802.15.SG3a devices. Although there may be a number of systems radiating RF energy in the environments envisioned for 802.15.SG3a systems, the interference from WLANs (2.4 GHz and 5 GHz), other WPANs (such as 802.15.1 and 802.15.4), cordless phones (2.4 GHz and 5 GHz), and microwave ovens will be the primary cases considered.

      - Interference Model

The following interferers will be considered:

- Microwave Oven

- IEEE 802.15.1 (Bluetooth)

- IEEE 802.11b

- IEEE 802.15.3

- IEEE 802.11a

- Out-of-band interference from intentional or unintentional radiators

Although other wireless systems may be present, the above systems represent a broad representative set of interferers whose impact has been determined to be sufficient for the evaluation of the proposed PHY solutions based upon the IEEE 802.15.SG3a target applications.  Since this document is concerned only with evaluating the capabilities, complexities, and performance implications of proposed physical layers, it is sufficient to use generic models of the above systems in order to ease the burden on the proposers.

The following representative models are suggested.

        - Microwave Oven

The microwave oven is modeled as transmitting at an EIRP of 100 mW with an active period of 8 ms, followed by a dormant period of 8 ms. That is, during the active period the transmit  power is 100 mW and during the dormant period the transmit power is 0 mW. During the active period, the microwave oven output can be modeled as a continuous wave interferer with a frequency that moves over a few MHz.  At the beginning of the active period, the frequency is 2452 MHz, and a the end of the active period, the frequency is 2458 MHz.  There is a continuous sweep in frequency as the active period progresses in time.

        - Narrowband 2.4 GHz Interferer

This model is intended to represent the impact of Bluetooth or 802.15.1 device.  The following table identifies the parameters of this interferer at the receiving antenna of the proposed 802.15.SG3a system.

| Center frequency | 2.4 GHz |
| --- | --- |
| Baud rate | 1 MHz |
| Modulation | GFSK |
| Tx power | 0 dBm |
| Tx antenna gain | 0 dBi |
| Path loss (1) at 1 meter | 40 dB |
| (2) at 0.3 meters | 29.6 dB |
| Rx power (1) at 1 meter | -40 dBm |
| (2) at 0.3 meters | -29.6 dBm |

        - Wideband 2.4 GHz Interferer

This model is intended to represent the impact of an 802.11b or 802.15.3 device. The following table identifies the parameters of this interferer at the receiving antenna of the proposed 802.15.SG3a system.

| Center frequency | 2.4 GHz |
| --- | --- |
| Baud rate | 11 MHz |
| Modulation | QPSK |
| Tx power | 20 dBm |
| Tx antenna gain | 0 dBi (handset) |
| Path loss (1) at 1 meter | 40 dB |
| (2) at 0.3 meters | 29.6 dB |
| Rx power (1) at 1 meter | -20 dBm |
| (2) at 0.3 meters | -9.6 dBm |

        - Wideband 5 GHz Interferer

This model is intended to represent the impact of an 802.11a device.  The following table identifies the parameters of this interferer at the receiving antenna of the proposed 802.15.SG3a system.

| Center frequency | 5.3 GHz |
| --- | --- |
| Baud rate | 16.6 MHz |
| Modulation Number of carriers Carrier spacing | 16-QAM OFDM 52 312.5 KHz |
| Tx power | 15 dBm |
| Tx antenna gain | 0 dBi (handset) |
| Path loss (1) at 1 meter | 46.9 dB |
| (2) at 0.3 meters | 36.5 dB |
| Rx power (1) at 1 meter | -31.9 dBm |
| (2) at 0.3 meters | -21.5 dBm |

        - Generic In-band Modulated Interferer

For ultra-wideband based proposals, there may be other wireless systems that may be near the 802.15.SG3a system that could cause in-band interference.  In order to understand how much protection the system will provide in this case of an unknown modulated interferer, the following model is proposed for evaluation.

I(t)=2PIcos(ω0t+θ)k=−∞∞gkv(t−kTI−τI)

where PI is the average received power of the interfering waveform, ω0=2πf0 is the carrier frequency of the “narrowband” waveform, θ is a random phase of the carrier uniformly distributed in 0,2π, {gk} are the randomly modulated BPSK symbols where gk∈±1, TI is the symbol period, τI is a random delay uniformly distributed in [0,TI], and v(t) is the baseband waveform shape.  The following table specifies the relevant parameters:

| f0 | Within the bandwidth of the proposal |
| --- | --- |
| 1TI | 5 MHz |
| Modulation | BPSK |
| Baseband waveform | Root Raised Cosine with a roll-off of 0.25 |

        - Generic In-band Tone Interferer

All systems may experience tone interference resulting from close proximity to unintentional radiators like PCs or consumer electronic devices. In order to understand how much protection the system will provide in this case of an unknown modulated interferer, the following model is proposed for evaluation.

I(t)=2PIcos(ω0t+θ)

where PI is the average received power of the interfering waveform, ω0=2πf0 is the carrier frequency of the “narrowband” waveform, and θ is a random phase of the carrier uniformly distributed in 0,2π.  For evaluation, f0 should be chosen to be within the bandwidth of the proposal.

      - Evaluation Method and Minimum Criteria

The following subsections describe how the above models can be used for evaluating the performance impact on the proposal.  Since the performance of these systems may depend on particular receiver designs, and it is not the intent to standardize certain receiver designs, the proposer should describe any special circuits that were needed to obtain these results (e.g., interference suppression algorithms, notch filters, steep roll-off filters, etc.).  Also, all of the following tests should be done using the nominal system configuration which provides ~110 Mbps.

        - Microwave Oven

{EDITOR NOTE: reconcile BER with PER as appropriate throughout the document}

When this interferer is present, using simulation results, analysis, or technical explanations, describes the impact on the proposed system performance when operating at 6 dB above the proposed systems receiver sensitivity level.  This impact should either be a reduction in data throughput or rise in the BER.

Minimum criteria: Proposed system should be able to maintain a PER < 8% for 1024 byte packets when the interference is present at a distance separation of 1 meter.  If this criteria cannot be met, proposers should define the operating power level of the proposed system (in terms of the number of dB’s above the receiver sensitivity level) at which a PER < 8% can be achieved when the interference is present at a distance separation of 1 meter.

        - Narrowband 2.4 GHz Interferer

When this interferer is present, using simulation results, analysis, or technical explanations, describes the impact on the proposed system performance when operating at 6 dB above the proposed systems receiver sensitivity level.  This impact should either be a reduction in data throughput or rise in the BER.

Minimum criteria: Proposed system should be able to maintain a PER < 8% for 1024 byte packets when the interference is present at a distance separation of 1 meters.  If this criteria cannot be met, proposers should define the operating power level of the proposed system (in terms of the number of dB’s above the receiver sensitivity level) at which a PER < 8% can be achieved when the interference is present at a distance separation of 1 meters.

Desired criteria: Proposed system should be able to maintain a PER < 8% for 1024 byte packets when the interference is present at a distance separation of 0.3 meters.

        - Wideband 2.4 GHz Interferer

When this interferer is present, using simulation results, analysis, or technical explanations, describes the impact on the proposed system performance when operating at 6 dB above the proposed systems receiver sensitivity level.  This impact should either be a reduction in data throughput or rise in the BER.

Minimum criteria: Proposed system should be able to maintain a PER < 8% for 1024 byte packets when the interference is present at a distance separation of 1 meters.  If this criteria cannot be met, proposers should define the operating power level of the proposed system (in terms of the number of dB’s above the receiver sensitivity level) at which a PER < 8% can be achieved when the interference is present at a distance separation of 1 meters.

Desired criteria: Proposed system should be able to maintain a PER < 8% for 1024 byte packets when the interference is present at a distance separation of 0.3 meters.

        - Wideband 5 GHz Interferer

When this interferer is present, using simulation results, analysis, or technical explanations, describes the impact on the proposed system performance when operating at 6 dB above the proposed systems receiver sensitivity level.  This impact should either be a reduction in data throughput or rise in the BER.

Minimum criteria: Proposed system should be able to maintain a PER < 8% for 1024 byte packets when the interference is present at a distance separation of 1 meters.  If this criteria cannot be met, proposers should define the operating power level of the proposed system (in terms of the number of dB’s above the receiver sensitivity level) at which a PER < 8% can be achieved when the interference is present at a distance separation of 1 meters.

Desired criteria: Proposed system should be able to maintain a PER < 8% for 1024 byte packets when the interference is present at a distance separation of 0.3 meters.

        - Generic In-band Modulated Interferer

When this interferer is present, using simulation results, analysis, or technical explanations, determine the average received interference power, PI, that can be tolerated before the BER increases to 10-5 or the data throughput is degraded by a factor of 2 when operating at 6 dB above the proposed systems receiver sensitivity level (Pd).  Present results for a number of different center frequencies or describe how the performance changes as the center frequency is changed.

Minimum criteria: PIPd > X dB. PIPd > 6 dB (Pd is the received power which is defined here as 6 dB above the receiver sensitivity level)

        - Generic In-band Tone Interferer

When this interferer is present, using simulation results, analysis, or technical explanations, determine the average received interference power, PI, that can be tolerated before the BER increases to 10-5 or the data throughput is degraded by a factor of 2 when operating at 6 dB above the proposed systems receiver sensitivity level (Pd).  Present results for a number of different center frequencies or describe how the performance changes as the center frequency is changed.

Minimum criteria: PIPd > 6 dB (Pd is the received power which is defined here as 6 dB above the receiver sensitivity level)

### Coexistence

      - Definition

Coexistence, in this context, refers to the co-location of IEEE 802.15.SG3a devices with other, non-802.15.SG3a devices.  The criteria described in this section focuses only on the impact the 802.15.SG3a devices have on other non-802.15.SG3a devices that may be sharing the same frequency bands.  The impact of the non-802.15.SG3a devices on an 802.15.SG3a receiver is addressed in Section 3.2.2.

      - Coexistence Model

The following devices, which may be co-located with 802.15.SGa devices, will be considered here:

- Bluetooth (IEEE 802.15.1)

- IEEE 802.15.3

- IEEE 802.11b

- IEEE 802.11a

Although other wireless systems may be present, the above systems represent a broad representative set of systems whose impact has been determined to be sufficient for the evaluation of the proposed PHY solutions based upon the IEEE 802.15.SG3a target applications.  The following figure shows the coexistence scenario and parameters that will be addressed in this section.

Figure 1: Reference Coexistence Model

The main parameter for this model is the nominal Rx power for the reference device that is representative of typical operating scenarios for the targeted applications for the systems.  Each of the devices described below operate in an unlicensed frequency band, and therefore should expect other interferers to be present.  In order to provide a reasonable trade-off between strict coexistence requirements for 802.15.SG3a devices and the interference susceptibility of the reference receivers, we have adopted an ‘interference margin’ of 6 dB.  This means that coexistence is measured when the reference device is operating at 6 dB above the reference systems minimum receiver sensitivity level published in the appropriate standard.  This ‘interference margin’ has also been proposed to the 802.15.2 coexistence task group as a PHY Coexistence Characterization (PCC) and is currently being used by the 802.15.4 task group to study coexistence (see IEEE802.15-02/148r0).  The above path loss model is used to verify that the nominal Rx power is sufficient to meet the typical range requirements for the reference system.  Note that this 6 dB margin is also used when computing the adjacent channel rejection for the 802.11b receiver, while a 3 dB margin is used for 802.11a adjacent channel rejection.

The following sections describe in more detail the reference systems that must be considered by each PHY proposal.

        - Bluetooth and IEEE 802.15.1 Devices

This model is intended to represent a Bluetooth or 802.15.1 WPAN device.  The following table identifies the relevant parameters of the reference system and the nominal Rx power at the input to the receiver.

| Center frequency | 2.4 GHz |
| --- | --- |
| Baud rate | 1 MHz |
| Modulation | GFSK |
| Tx Power | 0 dBm |
| Rx Antenna Gain | 0 dBi |
| Rx Sensitivity | -70 dBm |
| Interference Margin | 6 dB |
| Nominal Rx Power | -64 dBm |

3.2.3.2.2	IEEE 802.15.3 Device

This model is intended to represent a high rate 802.15.3 WPAN device.  The following table identifies the relevant parameters of the reference system and the nominal Rx power at the input to the receiver.

| Center frequency | 2.4 GHz |
| --- | --- |
| Baud rate | 11 MHz |
| Modulation | DQPSK |
| Tx Power | 0 dBm |
| Rx Antenna Gain | 0 dBi |
| Rx Sensitivity | -75 dBm |
| Interference Margin | 6 dB |
| Nominal Rx Power | -69 dBm |

        - IEEE 802.11b Device

This model is intended to represent an 802.11b WLAN device.  The following table identifies the relevant parameters of the reference system and the nominal Rx power at the input to the receiver.

| Center frequency | 2.4 GHz |
| --- | --- |
| Baud rate | 11 MHz |
| Modulation | CCK (11 Mbps) |
| Tx Power | 20 dBm |
| Rx Antenna Gain | 0 dBi |
| Rx Sensitivity | -76 dBm |
| Interference Margin | 6 dB |
| Nominal Rx Power | -70 dBm |

        - IEEE 802.11a Device

This model is intended to represent an IEEE 802.11a WLAN device.  The following table identifies the relevant parameters of the reference system and the nominal Rx power at the input to the receiver.

| Center frequency | 5.3 GHz |
| --- | --- |
| Baud rate | 11 MHz |
| Modulation Number of carriers Carrier spacing | 16-QAM Coded-OFDM (24 Mbps mode) 52 312.5 KHz |
| Tx Power | 15 dBm |
| Rx Antenna Gain | 0 dBi |
| Rx Sensitivity | -74 dBm |
| Interference Margin | 6 dB |
| Nominal Rx Power | -68 dBm |

      - Evaluation Method and Minimum Criteria

A single parameter (dint) will be used to quantify the coexistence capability of the proposed PHYs, and free space propagation shall be assumed between the IEEE 802.15.SG3a device and the reference device.  This parameter is defined as the minimum distance that can be achieved while the reference device enjoys a BER < 10e-5 (including any coding) while receiving at the nominal Rx power level based on a 6 dB interference margin.  The following minimum and desired criteria will be used for evaluating the PHY proposals.

Minimum criteria (1): Under normal operating conditions, the proposed PHY must be able to provide a minimum level of coexistence such that dint < 1 meter with 6 dB interference margin for the reference receiver.

Minimum criteria (2): The proposed PHY must be able to provide an enhanced level of coexistence such that dint < 0.3 meter with 6 dB interference margin for the reference receiver.  This means that, if an individual implementation wants to achieve this level of coexistence, it must be able to do so and still be able to communicate with a standard 802.15.3a receiver.

Desired criteria: The proposed PHY should be able to provide an enhanced level of coexistence such that dint < 0.3 meter with 0 dB interference margin for the reference receiver.  This means that, if an individual implementation wants to achieve this level of coexistence, it must be able to do so and still be able to communicate with a standard 802.15.3a receiver.

The motivation for the above criteria is the following.  Future 802.15.3a devices will definitely be located in close proximity with other IEEE 802.11 and IEEE 802.15 devices, based upon the set of applications that this standard is trying to address.  Minimum criteria (1) is for devices that knowingly will be very unlikely to be in close proximity to these other devices or for devices that will be deployed in environments where easy separation between devices can be achieved.  The minimum criteria (2) is for systems that would like to implement better coexistence for applications that will definitely be in close proximity with other 802.11 or 802.15 devices.  This could be done on an ‘as needed’ basis, and set either adaptively, with MAC control, or with user control.  The purpose of these criteria is to allow vendors to build devices that can operate in ‘enhanced’ coexistence mode while still being able to communicate with standard 802.15.3a receivers.  Therefore, the PHY proposals must possess the flexibility to offer this type of device capability.  Finally, the desired criteria is to suggest that an even better enhanced coexistence mode would be desirable for many scenarios to account for applications where it may be difficult or undesirable to get more than 0.3 meter separation, to account for antenna gains of the reference receiving device, and to account for highly efficient receivers that are able to operate below the Rx sensitivity level.

In addition, it is desirable for the proposed PHY to have ‘good neighbor’ capabilities like adaptive power control (don’t transmit more power than necessary), time division multiplexing (don’t transmit for more time than necessary), or dynamic frequency selection (ability to detect and avoid interference).  If the system cannot achieve these minimum separation distances, proposers should state what dint can be achieved given the above criteria, and what it would take to achieve the above dint values (notch filters, etc.) and corresponding cost.T

## Technical Feasibility

This is intended to determine if the proposal is real or academic.  Any proposal may be submitted, but demonstrated feasibility and manufacturability should receive favor over equal but untested proposals.  Proposals will be asked to comment on criteria listed in the following sections.

### Manufacturability

#### Definition

Manufacturability is defined in terms of the use of mature, cost effective manufacturing processes with evidence of effective mass production capability.

#### Values

The proposals are asked to submit proof of the claims by way of expert opinion, models, experiments, pre-existence examples, or demonstrations.  Globally accepted concepts that will be quick to market, with little risk will be favored.

### Time to Market

#### Definition

Time to Market addresses the question of when the proposed technology will be ready for integration.

#### Values

The proposal shall estimate a schedule for when the PHY would be available for integration.

### Regulatory Impact

#### Definition

The proposal should specify to which geopolitical regions it applies and identify any applicable requirements with which it conflicts. Merit will be awarded for proposals with regulatory compliance of wider geopolitical scope.

#### Values

The proposer shall state which regions the proposal is in regulatory compliance where merit is awarded for each region of compliance. 
Merit awarded for each category:

- Regions adopting US FCC regulations

- Regions adopting European regulations

- Japanese regulations

- Other National Regulations

Specific conflicts and potential derogations should be detailed.

## Scalability

### Definition

Scalability refers to the ability to adjust important parameters, such as those mentioned below, (if they are required by the applications) without rewriting the standard. Scalability should address evolutionary extensions to this proposal. The MAC should be able to support the scaling of the PHY (for example: 480 Mb/s at the PHY-SAP).

### Values

Scalability parameters include; power consumption, payload bit rate and data throughput both measured at the PHY-SAP, channelization (physical or coding), complexity, range, frequencies of operation, occupied bandwidth of operation, and other functions deemed appropriate. Proposers are encouraged to show power consumption levels scaling with reduced ranges and reduced bit rates. Proposers are further encouraged to show scalability up to 480 Mb/s and beyond, as well as 110Mb/s and below, as consistent with the table of applications in Section 2 of 02/104.

## Location Awareness

### Definition

Location awareness is the ability to determine information about the range and perhaps relative location of one device with respect to another. The purpose is to improve usability of portable devices.  This data can be used to locate, identify and discriminate amongst users in crowded environments and to simplify device registration in constantly changing network topology.  Provisions must be made to propagate location information to a suitable management entity.

### Values

Proposers are to show the functional capability, level of accuracy, resolution (meters), and time (seconds) to compute.

# MAC Protocol Supplements

## Required MAC enhancements and modifications to accommodate the Alternate PHY

### Definition

Supplements and modifications to the MAC may be required to accommodate the alternate PHY.  It is preferred that the supplements be additions which expand the solution capability as opposed to changes in the MAC that represent an alternative way to do a particular function.

### Values

Proposals should justify and explain the supplements that may be necessary in support of additional features for the alternate PHY.

Proposals should justify and explain the modifications that may be necessary to support or enhance operation of the alternate PHY.

# PHY Layer Criteria

## Size and Form Factor

### Definition

Size is important for consumer electronic systems such as PDAs and cameras.   The smaller the package, the easier it is to embed.  It is important that the device be compatible with accessory formats as well.  Antennas are not considered in the size requirements.

### Values

Proposers shall provide a time line estimate of when their proposed PHY and the 15.3 MAC will fit into the following form factors:

- PC Card

- Compact Flash

- Memory Stick

- SD Memory

## PHY-SAP Payload Bit Rate and Data Throughput

### Payload Bit Rates

#### Definition

The payload bit rate is defined as the bit rate at which the frame body, FCS and any stuffing bits and tail symbols are transmitted. For 802.15.3, examples of optional payload bit rates at the PHY-SAP are 11, 33, 44, 55 and the mandatory payload bit rate is 22 Mbps.

#### Values

The proposer should provide the mandatory and optional payload bit rates for the PHY-SAP. The payload bit rate at the PHY-SAP should be at least 110 Mbps at a 10 meter range, as supported in Section 5.5 (Link Budget).

### PHY-SAP Data Throughput

#### Definition

The proposer should provide all of the following parameters according to clause 2 of [02/104] for each of the proposed rates. In support of these numbers, the equations and values used to derive these times should be provided by the proposer. The number of octets for the MAC Header and FCS are specified in the MAC clauses of the proposed 802.15.3 standard while the HCS is part of the Alt-PHY. The proposer should provide the description of the proposed PHY Header and description and specification of the functional parts of the PHY preamble. Note: the HCS is a CRC that protects both the MAC and the PHY header.

The Short Interframe Spacing (SIFS) is the time between the end of one transmission and the start of the next. (T_SIFS in Figure 3). The SIFS must be large enough to support the TxRx turn around time for the proposed PHY, the MAC turnaround and the RxTx turn around time.

Figure 2 Timing parameters for data throughput comparison

#### Values

Time values should be stated in nanoseconds.

## Simultaneously Operating Piconets

### Definition

The proposed PHY should operate in the close proximity of multiple uncoordinated piconets, at specific bit and error rates.

### Values

The proposal should evaluate the effect of simultaneously operating piconets as specified in clause 3 of [02/104] for the following specified parameters: 
	-Packet length of 1024 octet frame body
	-PHY-SAP bit rates (110 Mb/s and 200 Mb/s) 
	-At a minimum, the proposal should demonstrate the performance required in Clause 5.2 of 		[02/105] (bit rates and BER/PER) in a free space AWGN channel without any 				uncoordinated alt-PHY transmitters present.
	-It is desired that a proposal also demonstrate this level of performance (see figure 4) for the 		environments specified in document [02/368] such that the reference link achieves 10-5 		BER without any uncoordinated alt-PHY transmitters present.

The proposer should indicate the values of dint that cause the PER and BER to degrade to specific levels (for example, 10-3 BER). At a minimum, a free space channel is to be used for all links. It is desired the environments specified in document [02/368] also be used for the interfering link. The acquisition time should also be stated for all tests.  A 0 dBi antenna gain should be assumed throughout.

Figure 4 Test geometry for simultaneously operating piconets

Evaluation geometry and procedure

An interfering transmitter is an uncoordinated transmitter operating at the same power as the reference transmitter. There are two cases to be considered: (1) a co-channel interferer, occupying the same channel and (2) adjacent channel interferer, occupying adjacent channels. If the interfering PHY would have a different impact on the receiver at different supported data rates, the PHY proposer should quantify this.

Single co-channel interferer separation distance is defined as the threshold distance separation (dint) of an interfering co-channel transmitter from the test receiver such that the test receiver BER and PER degrade to specified error rates (for example, 10-3 BER).

Multiple adjacent channel interferers separation distance is defined as the threshold distance separation (dint) of multiple interfering transmitters on different adjacent channels equidistant from the test receiver such that the test receiver BER and PER degrade to specified error rates (for example, 10-3 BER). 

Multi-channel separation distance test procedure 
1. Establish a test link with a test receiver at a fixed distance from the reference transmitter. Continue by sending packets to the test receiver for a specified modulation format and data rate and range, 110 Mb/s at 10m and 200 Mb/s at 4m.

2. Verify BER and PER at the test receiver.

3. Begin transmitting with N different adjacent channel interfering alt-PHY transmitters at a large distance from the test receiver.

4. Continue BER and PER verification at the test receiver.

5. Incrementally move the N different adjacent channel interfering alt-PHY transmitters closer to the test receiver uniformly until the BER and PER exceed the allowable rates.

6. Record the distance associated with the last acceptable BER and PER as the multi-channel separation distance (dint) for the selected test receiver.

7. Since the proposal includes multiple data rates (110 and 200 Mb/s) and may include multiple modulation types or other factors that may affect close proximity operation of uncoordinated piconets, the proposer should repeat the test procedures and include sufficient test combinations to characterize system operation under these conditions.

## Signal Acquisition

### Definition

The signal acquisition methods are the techniques by which the proposed receiver acquires and tracks the incoming signal in order to correctly receive the transmitted data.

### Values

The proposer should indicate a time-line showing the overall acquisition process, according to the preamble resources devoted to acquisition as specified in  02/105 Clause 5.2.2, at the payload bit rates and ranges specified in document 02/104 Clause 2 subject to the channel model provisions in 02/104 Clause 5. Target acquisition times, reflecting what is specified in the proposed IEEE 802.15.3 Standard, are <6 s for piconet CCA (referenced to the beginning of the preamble) and <20 s for acquisition from the beginning of the preamble to the beginning of the header. Additional information concerning how well the acquisition process scales with payload bit rate would be beneficial. The proposer should provide the probability of link success (the ability to acquire and pass data with the specified packet length and BER at minimum payload bit rates for the PHY-SAP for both AWGN and for the SG3a channel model relative to distance). The proposer should further indicate the range at which the proposed PHY can acquire and meet the bit rate packet length and BER requirements of clause 2.0 of 02/104 for the channel model specified in clause 5.0 of document 02/104 for a link success probability of 90%.  The proposer should indicate BER, PER and acquisition performance as a function of the distance. The acquisition parameters (signaling and duration) should be noted for all scenarios.

## Link Budget

### Definition

Link budget is used to determine proposal capabilities under certain operating conditions for the standards specified bit rates, ranges, and bit error rate.

### Values

The table below, which has roots from document 02/368r2, identifies the necessary parameters and equations that should be used to compute the final link margin. Proposers should complete this link budget table and identify and explain all assumptions. Although the proposers may need to make minor alterations to this table to more adequately reflect their proposal, the table identifies the minimum expected level of thoroughness, detail, and justification. The required C/N is the linear average performance using the channel models defined in document 02/368.

Sensitivity

### Definition

Sensitivity is defined in 3.2.1.  It is important for the proposal to specify the sensitivity level used in the determination of the signal robustness criteria.

### Values

The proposal should indicate the power level at which the error criterion is met, consistent with the link budget as presented in document 02/279r0-SG3a, Table 1. The proposal should also indicate both the PER, and the corresponding BER used in the determination of this value.

## Multi-Path Immunity

### Environment model

Use the multi-path parameters from the channel model referred to in 02/104 and 02/368.

### Delay Spread Tolerance

#### Definition

The delay spread tolerance is the value of rms for which the error rate criterion (referenced in section 2.0 of 02/104) is met with the input signal 3 dB above the minimum required sensitivity using the channel model defined in section 5.7.1.  The system shall have a delay spread tolerance of at least 25ns.

#### Values

The proposal will show what minimum delay spread tolerance can be achieved under various channel model conditions.

## Power Management Modes

The ability to reduce power consumption for consumer electronic devices is important.

### Definition

Power management modes and protocols allow device sleep, wakeup, and poll. The proposed 802.15.3 standard provides such power management capabilities.

### Values

The proposal should explain if it supports each of the power management methods as defined in the proposed 802.15.3 standard.

## Power Consumption

### Definition

Power consumption is defined as the total average power required by the proposed system to operate in transmit, receive, clear channel assessment, or power saving modes. It includes the power consumed by all components necessary to implement all of the functionality of the proposed alternate PHY from the PHY-SAP interface, defined in the proposed 802.15.3 standard, down to an antenna, where the gain is disclosed by the proposer. No components supporting operation above the PHY- SAP interface are included in the average power consumption value.

#### Transmit

Power consumption during transmit state is defined as the average power consumed from the PHY-TX-START.request for a given MPDU, to the PHY-TX-END.confirm.

#### Receive

Power consumption during receive state is defined as the average power consumed from the PHY-RX-START.request for a given MPDU, to the PHY-RX-END.indication where the PHY-RX-START.request is assumed to be coincident with the remote transmission beginning.

#### Clear Channel Assessment

Power consumption during clear channel assessment (CCA) is defined as the average power consumed from the PHY-CCA-START.request to the PHY-CCA-END.confirm.

#### Power Save

Power consumption during the power save state is defined as the power consumed from the PHY-PS.request to the PHY-PS.confirm resulting from a subsequent PHY-PS.request with a PSLevel value of 0. Methods for achieving power save modes and the impact to the operation (acquisition, time to come ‘awake”, etc…) of the PHY should be described.

### Value

Power consumption values are to be disclosed with sufficient explanation of how the numbers are derived. These numbers should reflect operation at the rf power necessary to achieve the continuous full bit rate/throughput at the maximum range including the disclosed antenna gain. To help aid comparison among proposals, disclosure should include parameters such as technology process, clock rate, voltage, etc.

#### Transmit

The proposal should estimate the power consumption for the PHY throughputs specified in section 5.2 with proposed minimum and maximum PHY frame lengths.

#### Receive

The proposal should estimate the power consumption for the PHY throughputs specified in section 5.2 with proposed minimum and maximum PHY frame lengths.

#### Clear Channel Assessment

The proposal should state the estimated power consumed during both channel "busy" periods and channel "idle" periods.

#### Power Save

The proposal should specify the power consumption associated with the lowest supported power consumption level (PwrMgtLevel). The proposal should also provide values for power save group parameters as specified in 802.15.3. Proposals should provide justification for its stated power save values (for example, circuits disabled, clocks turned off, etc…).

## Antenna Practicality

### Definition

The antenna form factor should be consistent with the following form factors:

- PC Card

- Compact Flash

- Memory Stick

- SD Memory

### Value

Antenna form factor should be described with reference to expected size. Any additional information the proposer desires to provide on the antenna such as size, frequency response, impulse response and radiation characteristics would be beneficial.