IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Mandatory ECC Security Algorithm Suite |  |
| Date Submitted | August 2, 2002 |  |
| Source | [Rene Struik, Certicom Corp.] 5520 Explorer Drive, 4th Floor Mississauga, ON Canada L4W 5L1 [Gregg Rasor, Motorola] 1500 Gateway Blvd. Boynton Beach, Florida 33426 | Voice: (905) 501-6083 Fax: (905) 507-4230 E-mail: rstruik@certicom.com Voice: (561) 739-2952 Fax: (561) 739-3715 E-mail: Gregg.Rasor@motorola.com |
| Re: | 802.15.3 TG3 Security call for proposals/ TG3 Security Compromise (02/323r0) |  |
| Abstract | An elliptic curve cryptography (ECC) security suite is defined including a full public key cryptosystem with digital signatures and implicit certificates. Binding between a device’s identity and public key using digital certificates may be established manually through direct user intervention, at device provisioning by a distributor or manufacturer, or by a trusted third party. |  |
| Purpose | This document defines the security elements necessary to implement the mandatory elliptic curve cryptography (ECC) security suite. |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

This document provides drop-in text for inclusion in Draft D10 that will fully define the optional ECMQV security suite and its sub-suites.

Table of Contents

7.5.2	Authentication and security commands	4

10.2.1	OID selections	4

10.3	ECMQV security suite	4

10.3.1	Cryptographic building blocks	5

10.3.1.1	Symmetric cryptographic building blocks	5

10.3.1.1.1	Cryptographic hash function	5

10.3.1.1.2	Keyed hash function for message authentication	5

10.3.1.1.2.1	Truncated output	5

10.3.1.2	Public-key cryptographic building blocks	5

10.3.1.2.1	Elliptic curve ansit283k1	5

10.3.1.2.2	Elliptic curve point representation	5

10.3.1.2.3	Elliptic curve public key pair	5

10.3.1.3	ECMQV key agreement protocol	6

10.3.1.4	ECC certificates	6

10.3.1.4.1	ECC manual ‘certificates’	6

10.3.1.4.2	ECC implicit certificates	6

10.3.1.4.3	ECC X509 certificates	7

10.3.1.4.3.1	ECDSA Digital Signatures	8

10.3.2	Cryptographic implementation	8

10.3.2.1	Symmetric cryptography implementation	8

10.3.2.1.1	Symmetric cryptography data formats	8

10.3.2.1.2	Symmetric cryptography operations	9

10.3.2.2	ECC certificate processing	9

10.3.2.2.1	ECC manual certificate processing	9

10.3.2.2.1.1	ECC manual certificate data format	9

10.3.2.2.1.2	Processing of ECC manual certificates	9

10.3.2.2.2	ECC implicit certificate processing	10

10.3.2.2.2.1	ECC implicit certificate data format	10

10.3.2.2.2.2	Processing of ECC implicit certificates	10

10.3.2.2.3	ECC X509 certificate processing	10

10.3.2.2.3.1	ECC X509 certificate data format	10

10.3.2.2.3.2	Processing of ECC X509 certificates	11

10.3.2.3	ECMQV key agreement protocol implementation	11

10.3.2.3.1	ECMQV key agreement protocol data formats	11

10.3.2.3.2	ECMQV key agreement protocol operations – informal description	11

10.3.2.3.3	Public key and authentication cryptographic operations – formal description	12

10.3.2.3.3.1	Authentication request command	12

10.3.2.3.3.2	Challenge request command	13

10.3.2.3.3.3	Challenge response command	13

10.3.2.3.3.4	Authentication response command	13

10.3.3	ECMQV security suite implementation	13

10.3.3.1	ECMQV Manual security sub-suite implementation	13

10.3.3.2	ECMQV Implicit security sub-suite implementation	14

10.3.3.3	ECMQV X509 security sub-suite implementation	14

APPENDIX A	15

10.4	Implicit Certificate Scheme	15

10.4.1	Scheme Setup	15

10.4.2	Key Deployment	16

10.4.3	Implicit Certificate Generation Protocol	16

10.4.4	Implicit Certificate Processing Operation	18

APPENDIX B – ECC X509 Certificates	20

APPENDIX C – References	21

11.	Definitions, Abbreviations, and References	21

11.2	Definitions and Abbreviations	21

11.3	Symbols and Notation	21

11.3.1	Normative References	21

11.3.2	Informative References	22

### Authentication and security commands

Editorial notes:

- In Clause 7.5.2, replace all occurrences of ECC 256p by ECMQV (or by ECMQV Koblitz-283 if one really wants to).

- In Clause 7.5.2.1, replace all occurrences of the word ‘raw’ by ‘manual’.

- In Clauses 7.5.2.2 and Clauses 7.5.2.4, add an enumeration of the Challenge response types and Authentication response types that includes {CCM integrity check value, truncacted HMAC value}.

- In Clauses 7.5.2.3 and Clauses 7.5.2.4, add an enumeration of the Challenge types that includes {ECMQV Public key}.

- In Clauses 7.5.2.3, duplicate the enumeration of the Public key types of Clause 7.5.2.1.

### OID selections

Editorial notes:

- In Table 78, replace all occurrences of ECIES 256-Prime 1 by ECMQV (or by ECMQV Koblitz-283 if one really wants to). Put the security suites in alphabetical order.

| Sub-suite name | OID name | OID number | DER-encoding |
| --- | --- | --- | --- |
| ECMQV | Ecmqv-sec-suite-1 | Id-802-15-3-security-suites 1 | 0x060728C4620F030101 |

- In Table 79, replace the ECC-relevant portions of the table by the following:

| Sub-suite name | OID name | OID number | DER-encoding |
| --- | --- | --- | --- |
| ECMQV manual | ecmqv-manual | ecmqv-sec-suite-1 1 | 0x060728C4620F03010101 |
| ECMQV implicit | ecmqv-implicit | ecmqv-sec-suite-1 2 | 0x060728C4620F03010102 |
| ECMQV X509 | ecmqv-X509 | ecmqv-sec-suite-1 3 | 0x060728C4620F03010103 |

(Note: in the Draft D10, the DER encoding has format 0x060828C4620F030x0y. Isn’t this in error?)

## ECMQV security suite

Editorial note:

Replace the ECIES 256-prime-1 security suite, as specified in Clause 10.3 of the current Draft D10 by the ECMQV security suite specified below.

This sub-clause specifies the cryptographic building blocks of the ECMQV security suite and the operations that shall be performed in each of its sub-suites.

### Cryptographic building blocks

#### Symmetric cryptographic building blocks

The following symmetric cryptographic primitives and data elements are defined for use with all sub-suites of the ECMQV security suite.

##### Cryptographic hash function

The cryptographic hash function used in this security suite shall be SHA-256, as specified in the Draft FIPS Pub 180-2 standard [{xref} FIPS 180-2]. SHA-256 has message digest size hashlen=256.

##### Keyed hash function for message authentication

The keyed hash message authentication code (HMAC) used in this security suite shall be HMAC, as specified in the FIPS Pub 198 [{xref} FIPS 198], with the following instantiations:

- Each entity shall use the cryptographic hash function H as specified in Clause 10.3.1.1.1 {xref};

- The block size B shall have the integer value 16 (this block size specifies the length of the data integrity key, in bytes, that is input to the keyed hash function, i.e., it uses a 128-bit data integrity key);

- The output size HMAClen of the HMAC function shall have the same integer value as the message digest parameter hashlen as specified in Clause 10.3.1.1.1 {xref}.

###### Truncated output

Applications of the keyed hash function may truncate the output of the keyed hash message authentication code specified in Clause 10.3.1.1.2 (i.e., the length of the HMAC is less than the length HMAClen of the HMAC function). In this security suite, the output of the HMAC function shall be truncated by omitting all but the leftmost 128 bits hereof.

#### Public-key cryptographic building blocks

The following public-key cryptographic primitives and data elements are defined for use with all sub-suites of the ECMQV security suite.

##### Elliptic curve ansit283k1

All elliptic curve points (and operations hereon) used in this security suite shall be on the curve ansit283k1 as specified in Appendix J4.5, Example 1, of ANSI X9.63-2001 [{xref} X9.63].

##### Elliptic curve point representation

All elliptic curve points shall be represented in polynomial notation as specified in Section 4.1.2.1 of ANSI X9.63-2001 [{xref} X9.63}]. All elliptic curve points shall be transmitted in compressed form, as specified in Section 4.2.2 of ANSI X9.63-2001 [{xref}X9.63].

##### Elliptic curve public key pair

An elliptic curve key pair consists of an integer q and a point Q on the curve determined by multiplying the generating point G of the curve by this integer, i.e., Q=qG, as specified in ANSI X9.63-2001 [{xref} X9.63]. Here, Q is called the public key, whereas q is called the private key. The private key shall be represented as specified in Section 4.3.1 of ANSI X9.63-2001 [{xref} X9.63]. The public key shall be represented as defined in Sub-clause 10.3.2.2{xref}.

#### ECMQV key agreement protocol

The ECMQV key agreement protocol used in this security suite shall be the full MQV with key confirmation scheme as specified in Section 6.11 of ANSI X9.63-2001 [{xref} X9.63], with the following instantiations:

- Each entity shall be a DEV;

- Each entity’s identifier shall be its 48-bit device address {xref}; the parameter entlen shall have the integer value 48;

- Each entity shall use the HMAC-scheme as specified in Clause 10.3.1.1.2 {xref}, with truncated output as specified in Clause 10.3.1.1.2.1 {xref};

- Each entity shall use the cryptographic hash function as specified in Clause 10.3.1.1.1 {xref};

- The parameter keydatalen shall have the integer value 128;

- The parameter SharedData shall be the empty string; the parameter shareddatalen shall have the integer value 0;

- The optional parameters Text1 and Text2 as specified in Sections 6.11.1 and 6.11.2 of ANSI X9.63-2001 [{xref}X9.63] shall both be the empty string.

#### ECC certificates

The ECMQV security suite may use different certificate types, depending upon the applicable sub-suite hereof. The following public-key objects and data elements are defined for use with the ECMQV security suite.

##### ECC manual ‘certificates’

The manual ‘certificate’ ManCert used with the ECMQV security suite shall be specified as follows:

ManCert=(PublicKey || Subject), where

- PublicKey shall be the representation of an elliptic curve point, which is a public key as specified in Clause 10.3.1.2.3 {xref};

- Subject shall be the purported owner of the private key corresponding to the public key represented by PublicKey, which is a DEV, as represented by its 48-bit device address {xref}.

The manual certificate is no ‘real’ digital certificate, since the binding between a public key and its purported owner is to be established and verified by non-cryptographic means.

##### ECC implicit certificates

The implicit certificate scheme used with the ECMQV security suite shall be the implicit certificate scheme with associated implicit certificate generation protocol and implicit certificate processing protocol as specified in Appendix A {xref}, with the following instantiations:

- Each entity shall be a DEV;

- Each entity’s identifier shall be its 48-bit device address {xref}; the parameter entlen shall have the integer value 48;

- Each entity shall use the cryptographic hash function as specified in Clause 10.3.1.1.1 {xref};

- The format of the implicit certificate ImplCert shall be specified as follows:

  - ImplCert shall be the representation of the string ICU as specified in the implicit certificate generation protocol;

  - The string ImplCert shall contain, as a minimum, the following information:

    - PublicReconstrKey: the representation of the public-key reconstruction data BEU as specified in the implicit certificate generation protocol, which is an elliptic curve point as specified in Clause 10.3.1.2.3 {xref};

    - Subject: the identifier of the entity U that is bound to the public-key reconstruction data BEU during the execution of the implicit certificate generation protocol;

    - Issuer: the identifier of the entity CA that creates the implicit certificate during the execution of the implicit certificate generation protocol (the so-called Certificate Authority).

The exact format of ImplCert depends on the established infrastructure.

- The string IU as specified in Step 6 of the actions of the CA in the implicit certificate generation protocol shall contain information including the Subject and the Issuer, where Subject and Issuer are as specified above. The exact format of IU depends on the established infrastructure.

The following additional information shall have been unambiguously established between devices operating the implicit certificate scheme:

- Each entity shall have obtained information regarding the infrastructure that will be used for the operation of the implicit certificate scheme – including a certificate format and certificate generation and processing rules (i.e., rules need to be in place that determine how to generate and process the strings ImplCert and IU above (see Appendix A, Scheme Set-up, 10.4.1 {xref}));

- Each entity shall have access to an authentic copy of the elliptic curve public keys of one or more certificate authorities that act as CA for the implicit certificates scheme (see Appendix A, Key Deployment, 10.4.2 {xref}).

The methods by which this information is to be established are outside the scope of this standard.

##### ECC X509 certificates

The explicit certificate scheme used with the ECMQV security suite shall be the ECC X509 certificate scheme, with associated ECC X509 certificate generation protocol and ECC X509 certificate processing protocol, as specified in PKIX [{xref} RFC 2459, {xref} RFC 3279, {xref}RFC 3280] (see also Appendix B {xref}), with the following instantiations:

- Each entity shall be a DEV;

- Each entity’s identifier shall be its 48-bit device address {xref}; the parameter entlen shall have the integer value 48;

- Each certificate shall include, as a minimum, the following information:

- PublicKey: an elliptic curve public key as specified in Clause 10.3.1.2.3;

- Subject:  the identifier of the entity that is bound to the public key during the execution

of the certificate generation rule;

- Issuer: the identifier of the entity CA that creates the X509 certificate during the

execution of the X509 certificate generation protocol (the so-called Certificate

Authority).

The certificate shall be generated and verified using the digital signature algorithm ECDSA as specified in Clause 10.3.1.4.3.1.

The following additional information shall have been unambiguously established between devices operating the implicit certificate scheme:

- Each entity shall have access to an authentic copy of the elliptic curve public keys of one or more certificate authorities that act as CA for the X509 certificates.

The methods by which this information is to be established are outside the scope of this standard.

###### ECDSA Digital Signatures

The ECC X509v3 certificate scheme shall use the ECDSA signature scheme, with associated signature generation protocol and signature verification protocol, as announced in FIPS Pub 186-2 [{xref} FIPS 186-2 rev] and fully specified in ANSI X9.62-1998 [{xref} X9.62]. The ECDSA key generation protocol shall use the random number generation algorithm as specified in FIPS Pub 186-2, including Change Notice 1 [{xref} FIPS186-2 Rev].

### Cryptographic implementation

#### Symmetric cryptography implementation

All sub-suites of the ECMQV security suite shall perform all symmetric operations of Clause 7.5.2 {xref} in the same manner.

##### Symmetric cryptography data formats

The following table specifies the length and the intended meaning of the symmetric-cryptography-related security-suite-specific data elements from Clause 7.5.2 {xref}. The operations performed to obtain the variable data values are specified in a separate sub-clause.

Table XXX – Symmetric cryptography frame object formats

| Notation | Length | Value | Description |
| --- | --- | --- | --- |
| AuthResponseType | 2 | See Table 7.5.2.2 {xref} | The numerical value of the HMAC-output type in Table 7.5.2.2{xref} |
| AuthResponseLength | 2 | 16 | Length of the truncated output of the HMAC function as specified in Clause 10.3.1.1.2 {xref} |
| AuthResponse | 16 | HMAC value | Instance of the truncated HMAC output as specified in Clause 10.3.1.1.2 {xref} |
| ChallengeResponseType | 2 | See Table 7.5.2.4 {xref} | The numerical value of the HMAC-output type in Table 7.5.2.4{xref} |
| ChallengeResponseLength | 2 | 16 | Length of the truncated output of the HMAC function as specified in Clause 10.3.1.1.2 {xref} |
| ChallengeResponse | 16 | HMAC value | Instance of the truncated HMAC output as specified in Clause 10.3.1.1.2 {xref} |

##### Symmetric cryptography operations

The following table specifies the symmetric-cryptography-related operations performed in Clause 7.5.2 {xref}:

Table 5 – Symmetric cryptographic operations

| Use | Description |
| --- | --- |
| Challenge response generation | Computation of message authentication code value over part of the communicated strings during the execution of the authentication protocol, to be used for entity authentication and key confirmation. |
| Authentication response generation | Computation of message authentication code value over part of the communicated strings during the execution of the authentication protocol, to be used for entity authentication and key confirmation. |

#### ECC certificate processing

##### ECC manual certificate processing

###### ECC manual certificate data format

The following tables specify the length and intended meaning of the certificate-related data elements from Clause 7 for the ECC security suite {xref}.

Table 14 – ECC manual certificate frame object formats

| Notation | Length | Value | Description |
| --- | --- | --- | --- |
| PublicKeyObjectType | 2 | See Table 7.5.2.1 {xref} | The numerical value of the ECMQV-manual public key type in Table 7.5.2.1{xref} |
| PublicKeyObjectLength | 2 | 43 | Length of the ECC manual mertificate |
| PublicKeyObject | 43 | Manual certificate value | Instance of ECC manual certificate as specified in Clause 10.3.4.1 |
| DEV address | 6 | Variable | Device address of the purported owner of the ECC manual certificate |

###### Processing of ECC manual certificates

The ECC manual certificate shall be processed by executing the following steps (in order):

- Extract the public key QU and the identifying string of its purported owner U from the ManCert string (according to the formatting rules for manual certificates as specified in Clause 10.3.1.4.1);

- Compare the identifying string U of the purported owner with the DEV address. If these do not match, abort the operation.

Processing is aborted, if any of these steps fail.

##### ECC implicit certificate processing

###### ECC implicit certificate data format

The following tables specify the length and intended meaning of the certificate-related data elements from Clause 7 for the ECC security suite {xref}.

Table 14 – ECC implicit certificate frame object formats

| Notation | Length | Value | Description |
| --- | --- | --- | --- |
| PublicKeyObjectType | 2 | See Table 7.5.2.1 {xref} | The numerical value of the ECMQV-implicit public key type in Table 7.5.2.1{xref} |
| PublicKeyObjectLength | 2 | Variable | Length of the ECC Implicit Certificate |
| PublicKeyObject | Variable | Implicit certificate value | Instance of ECC Implicit certificate as specified in Clause 10.3.4.2 |
| DEV address | 6 | Variable | Device address of the purported owner of the ECC implicit certificate |

###### Processing of ECC implicit certificates

The ECC implicit certificate shall be processed by executing the following steps (in order):

- Extract the public key QU and the identifying string of its purported owner U from the ImplCert string, using the implicit certificate processing operation as specified in Appendix A and the formatting and processing rules as set forward in Clause 10.3.1.4.2;

- Compare the identifying string U of the purported owner with the DEV address. If these do not match, abort the operation.

Processing is aborted, if any of these steps fail.

##### ECC X509 certificate processing

###### ECC X509 certificate data format

The following table specifies the length and intended meaning of the certificate-related data elements from Clause 7 for the ECC security suite {xref}.

Table 16 – ECC X509 certificate frame object formats

| Notation | Length | Value | Description |
| --- | --- | --- | --- |
| PublicKeyObjectType | 2 | See Table 7.5.2.1 {xref} | The numerical value of the ECMQV X509 1 public key type in Table 7.5.2.1{xref} |
| PublicKeyObjectLength | 2 | Variable | Length of the ECC X509 Certificate |
| PublicKeyObject | Variable | X509 certificate value | Instance of ECC X509 certificate as specified in Clause 10.3.4.3 |
| DEV address | 6 | Variable | Device address of the purported owner of the ECC X509 certificate |

###### Processing of ECC X509 certificates

The ECC X509 certificate shall be processed by executing the following steps (in order):

- Extract the public key QU and the identifying string of its purported owner U from the X509 certificate string, using the certificate processing operation as specified in formatting and processing rules as set forward in Clause 10.3.1.4.3;

- Compare the identifying string U of the purported owner with the DEV address. If these do not match, abort the operation.

Processing is aborted, if any of these steps fail.

#### ECMQV key agreement protocol implementation

##### ECMQV key agreement protocol data formats

The following table specifies the length and intended meaning of the public-key cryptography and authentication related security suite specific data elements from Clause 7 {xref}. The operations to obtain the variable data values are specified in a separate sub-clause.

Table 12 – Public-key frame object formats

| Notation | Length | Value | Description |
| --- | --- | --- | --- |
| PublicKeyObject | variable | Certificate value of type according to OID | See Clause 10.3.2.2 for public key formats, as a function of the applicable sub-suite of the ECMQV security suite. |
| OIDLength | 1 | 10 | Length of the DER encoding of the applicable OID |
| OID | 10 | OID value | DER encoding of the OID of the applicable sub-suite of the ECMQV security suite as specified in Table 79 of Clause 10.2.1 {xref} |
| ChallengeType | 2 | See Table 7.4.1.3 {xref} | The numerical value of the ChallengeType ephemeral elliptic curve public key in Table 7.5.2.3 {xref} |
| ChallengeLength | 2 | 37 | Length of ephemeral public key |
| Challenge | 37 | Point value | Ephemeral public key as specified in Clause 10.3.1.2.3 |

Editorial note:

Modify clause 10.2.1 of D10 (below Table 79):

The value of the OID field for any command is the DER encoding of the OID. For each OID used in this standard, the length of this encoding is 10 bytes. This does away with the need to specify the OID in this subclause.

##### ECMQV key agreement protocol operations – informal description

The following table specifies the public-key cryptography and authentication-related operations for the authentication protocol frames as specified in Clause 7 {xref}:

Table XXX – Authentication-related functions

| Use | Description |
| --- | --- |
| Authorization of public key | Verification of authorization status of the other party, by checking whether the public key information as contained in the applicable certificate as specified in Clause 10.3.1.4 complies with information in the access control list (ACL). |
| Challenge generation | Communication of random short-term (ephemeral) public key to the other party, to be used for key establishment and for entity authentication. |
| Key establishment | Derivation of a shared key, based on the ephemeral public keys contributed by both parties and long-term (static) public-keying material of both parties. |
| Key authentication | Verification of the authenticity of the static key of the other party via extraction of the public key hereof from the applicable certificate communicated by the other party, via Clause 10.3.2.2 and evidence of possession of the corresponding private key hereby (as witnessed by successful key confirmation). Note: for manual certificates, this only yields non-cryptographic evidence as to the authenticity of the key sharing parties. |
| Mutual key confirmation | Verification of possession of the shared key by the other party, via the computation of a message authentication code over part of the data communicated with the other party. |
| Mutual entity authentication | Verification of the actual and real-time involvement of the other party during the authenticated key agreement protocol. |

##### Public key and authentication cryptographic operations – formal description

The following sub-clauses describe the mapping between elements of the command frames for authentication, as specified in Clause 7.5.2 {xref}, and corresponding elements of the protocol messages in the full MQV with key confirmation scheme, as specified in Section 6.11 of ANSI X9.63-2001 [{xref} X9.63]. This mapping formally defines the workings of the authentication protocol within the context of the ECMQV security suite.

With Cert(Q), we indicate the embedding of a public key Q in one of the certificate formats specified in Clause 10.3.1.4 {xref}. The public Q is extracted from the certificate Cert(Q) by applying the applicable certificate processing operation as specified in Clause 10.3.2.2 {xref}.

###### Authentication request command

| Command frame element (7.5.2.1): | MQV specification (ANSI X9.63-2001) | Description | Reference {xref} |
| --- | --- | --- | --- |
| Public key object | Cert(Qs,V) | ECC certificate of the appropriate type | 10.3.4 |
| DEV address | V | Device address of the initiating device | {xref} |

The public key object shall represent the public key of the entity indicated by the DEV address.

###### Challenge request command

| Command frame element (7.5.2.3): | MQV specification (ANSI X9.63-2001) | Description | Reference{xref} |
| --- | --- | --- | --- |
| Public key object | Cert(Qs,U) | ECC certificate of the appropriate type | 10.3.4 |
| Challenge | QEU | Ephemeral public key | 10.3.2.3 |
| DEV address | U | Device address of the responding device | {xref} |

###### Challenge response command

| Command frame element (7.5.2.4): | MQV specification (ANSI X9.63-2001) | Description | Reference {xref} |
| --- | --- | --- | --- |
| Challenge | QEV | Ephemeral public key | 10.3.2.3 |
| Challenge response | MACTag1 | Truncated HMAC output | 10.3.1.2 |

###### Authentication response command

| Command frame element (7.5.2.2): | MQV specification (ANSI X9.63-2001) | Description | Reference {xref} |
| --- | --- | --- | --- |
| Authentication response | MACTag2 | Truncated HMAC output | 10.3.1.2 |

### ECMQV security suite implementation

Each sub-suite of the ECMQV security suite shall execute the ECMQV protocol in the same manner. The distinction between the ECMQV sub-suites is confined to the use and processing of certificates only, as specified in Table XXX. The processing of certificates is specified in Clause 10.3.2.3, whereas the operation of the ECMQV protocol is specified in Clause 10.3.2.3.

| ECMQV security sub-suite | Public key protocol | Certificate format used |
| --- | --- | --- |
| ECMQV Manual | ECMQV (Clause 10.3.5.2) | ECC manual certificate (10.3.1.4.1) |
| ECMQV Implicit | ECMQV (Clause 10.3.5.2) | ECC implicit certificate (10.3.1.4.2) |
| ECMQV X509 | ECMQV (Clause 10.3.5.2) | ECC X509 certificate (10.3.1.4.3) |

Table XXX: Overview of the different ECMQV security sub-suites

#### ECMQV Manual security sub-suite implementation

The ECMQV Manual security sub-suite of the ECMQV security suite  shall be implemented by executing the following steps:

- Process the manual certificate as specified in Clause 10.3.1.4.1 using the ECC manual certificate processing operation specified in Clause 10.3.2.2.1.

- Execute the ECMQV protocol with the OID for the ecmqv-manual sub-suite.

Processing is aborted, if any of these steps fail.

Remark: these steps do not need to be executed strictly in order. The only requirement is that information on the public key of the communicating party becomes available in time to be used for the derivation of the shared key during the execution of the ECMQV key agreement protocol (this allows lazy evaluation techniques).

#### ECMQV Implicit security sub-suite implementation

The ECMQV Implicit security sub-suite of the ECMQV security suite shall be implemented by executing the following steps:

- Process the implicit certificate as specified in Clause 10.3.1.4.2 using the ECC implicit certificate processing operation specified in Clause 10.3.2.2.2.

- Execute the ECMQV protocol with the OID for the ecmqv-implicit sub-suite.

Processing is aborted, if any of these steps fail.

Remark: these steps do not need to be executed strictly in order. The only requirement is that information on the public key of the communicating party becomes available in time to be used for the derivation of the shared key during the execution of the ECMQV key agreement protocol (this allows lazy evaluation techniques).

#### ECMQV X509 security sub-suite implementation

The ECMQV Manual security sub-suite of the ECMQV security suite shall be implemented by executing the following steps:

- Process the X509 certificate as specified in Clause 10.3.1.4.3 using the ECC X509 certificate processing operation specified in Clause 10.3.2.2.3.

- Execute the ECMQV protocol with the OID for the ecmqv-X509 sub-suite.

Processing is aborted, if any of these steps fail.

Remark: these steps do not need to be executed strictly in order. The only requirement is that information on the public key of the communicating party becomes available in time to be used for the derivation of the shared key during the execution of the ECMQV key agreement protocol (this allows lazy evaluation techniques).

# APPENDIX A

## Implicit Certificate Scheme

This section specifies the ECQV implicit certificate scheme based on ECC supported in this standard.

Implicit certificate schemes are designed to be used by three entities – a Certification Authority CA, a certificate requester U, and a certificate processor V, where U wants to obtain an implicit certificate from CA in order to convey U's public key to V.

Here implicit certificate schemes are described in terms of a certificate generation protocol, a certificate processing operation, and associated setup and CA key deployment procedures. CA, U, and V shall use the schemes as follows, when they wish to communicate.

CA, U, and V should use the setup procedure to establish which options to use the scheme with. CA should use the key deployment procedure to select a key pair and U and V should obtain CA's public key – CA will use the key pair during the certificate generation protocol, U will use the public key during the certificate generation protocol, and V will use the public key during the certificate processing operation. When U wants to obtain an implicit certificate, U and CA should perform the certificate generation protocol to obtain a key pair known to U and an implicit certificate IC. Finally, when V wants to obtain U's public key, U should convey IC to V and V should apply the certificate processing to IC under CA's public key to obtain U's public key. V concludes that the public key is genuine, provided U provides evidence that it possesses the corresponding private key.

The specification of ECQV in this document relies heavily on the mathematical foundations and cryptographic components specified in Sections 2 and 3 of [SEC1].

The setup procedure for ECQV is specified in Section 8.1, the key deployment procedure is specified in Section 8.2, the certificate generation protocol is specified in Section 8.3, and the certificate processing operation is specified in Section 8.4.

### Scheme Setup

CA, U, and V shall perform the following setup procedure to prepare for the use of ECQV.

- An infrastructure shall have been established for the operation of the scheme – including a certificate format, certificate processing rules, and unique identifiers. An example of such an infrastructure is described by PKIX [PKIX-X509].

- Each entity has an authentic copy of the system’s elliptic curve domain parameters D=(p,a,b,G,n,h) or D=(m,f(x),a,b,G,n,h). These parameters shall have been generated using the parameter generation primitive in Sections 3.1.1.1 or the primitive specified in Section 3.1.2.1, both of [SEC1]. Furthermore, the parameters shall have been validated using the parameter validation primitives in Sections 3.1.1.2 or Section 3.1.2.2 of [SEC1].

- The CA shall have decided which cryptographic hash function to use when generating implicit certificates. Let Hash denote the hash function chosen, and let hashlen denote the length in bits of the output value of this hash function. Each entity shall have an authentic copy of this hash function.

- Each entity shall be bound to a unique identifier (e.g. distinguished names). All identifiers shall be bit strings of the same length entlen bits. Entity U’s identifier will be denoted by the bit string U. Entity V’s identifier will be denoted by the bit string V. Entity CA’s identifier will be denoted by the bit string CA.

- The CA shall be bound to a static public key pair associated with the system’s elliptic curve domain parameters D. The binding process shall include the validation of the static public key as specified in Section 3.2.2 of [SEC1]. The key binding shall include the unique identifier CA for the entity involved.

- Each entity shall have decided how to represent elliptic curve points as octet strings (i.e., compressed form, uncompressed form, or hybrid form).

### Key Deployment

CA, U, and V shall use the following key deployment procedure to prepare for the use of ECQV.

- CA shall establish a static elliptic curve key pair (WCA, wCA) associated with D to use with the certificate generation and processing protocols. The key pair shall be generated using the primitive specified in Section 3.2.1 of SEC 1.

- U and V shall obtain in an authentic manner the elliptic curve public key WCA selected by CA.

### Implicit Certificate Generation Protocol

This section specifies the protocol for generating implicit certificates (self-certified public keys).

Figure 12 – Implicit Certificate Generation Scheme

U

CA

WCA

QEU

ICU, WU, se

Figure 12 illustrates the messaging involved in the use of the certificate generation protocol.

CA and U shall generate an implicit certificate for U using the keys and parameters established during the setup procedure and the key deployment procedure as follows:

Input: None.

Output for CA: An octet string ICU, which is U's implicit certificate, and an elliptic curve public key WU corresponding to the elliptic curve domain parameters D, which is U's public key.

Output for U: An octet string ICU, which is U's implicit certificate, and an elliptic curve key pair (WU, wU) corresponding to the elliptic curve domain parameters D, which is U's public key pair.

Actions: U proceeds as follows:

            - Select a random ephemeral elliptic curve key pair (QU, qU) associated with the elliptic curve domain parameters D. The key pair shall be generated using the primitive specified in Section 3.2.1 of SEC 1. (qU is referred to as U's private request value; QU as U's public request value.)

            - Convert QU to an octet string QEU as specified in Section 2.3.5 in SEC 1, and send it to CA.

            - Obtain the values ICU and se from CA. Compute the hash value H = Hash(ICU) using the established hash function. Derive an integer e from H following the conversion routine of Step 5 in Section 4.1.3 of [SEC1].

            - Verify the content of ICU according to the established infrastructure. This includes verifying the contents of the certificate, e.g., the subject’s name and the validity period.

            - Derive BEU and IU from ICU according to the procedures of the established infrastructure.

            - Derive WCA from IU, according to the certificate format specified during the setup procedure.

            - Convert BEU to an elliptic curve point BU as specified in Section 2.3.4 of SEC 1 and convert se to an integer s as specified in Section 2.3.8 of SEC 1.

            - Compute the secret key wU := s + qU  e (mod n) and the public key WU := wU G.

            - Reconstruct the public key WU’ = e BU + WCA.

            - If WU' = WU, accept the certificate and output the computed key pair (WU, wU) and the implicit certificate ICU; otherwise output reject.

Actions: CA proceeds as follows:

- Verify the authenticity of the request received from U according to the procedures of the established infrastructure. The checks performed shall include, as a minimum, checking that U is indeed the origin of the request, and checking that U is authorized to obtain a certificate.

- Receive QEU from U and convert it to an elliptic curve point QU as specified in Section 2.3.4 in SEC 1.

- Select an ephemeral elliptic curve key pair (QCA, qCA) associated with the elliptic curve domain parameters D. The key pair shall be generated using the primitive specified in Section 3.2.1 of [SEC1].

- Compute the elliptic curve point BU:= QU + QCA.

- Convert the elliptic curve point BU to an octet string BEU as specified in Section 2.3.3 in SEC 1.

- Construct the ‘to-be-signed-certificate’ data, which is an octet string IU. IU shall contain identification information according to the procedures of the established infrastructure and may also contain other information, such as the intended use of the public key, the serial number of the implicit certificate, and the validity period of the implicit certificate. The exact form of IU depends on the certificate format being used.

- Construct according to the procedures of the established infrastructure U's implicit certificate, which is an octet string ICU. ICU should contain IU and BEU. The exact form of ICU depends on the certificate format being used.

- Compute the hash value H = Hash(ICU) using the established hash function. Derive an integer e from H following the conversion routine of Step 5 in Section 4.1.3 of [SEC1].

- Compute the integer s = qCA e+wCA (mod n). (s is referred to as the private-key reconstruction data.)

- Convert s to an octet string se as specified in Section 2.3.7 of SEC 1, and send ICU and se to U.

- Compute WU = e BU + WCA. Output the implicit certificate ICU and the elliptic curve public key WU.

### Implicit Certificate Processing Operation

V shall process U's implicit certificate using the keys and parameters established during the setup procedure and the key deployment procedure as follows:

Input: U's purported implicit certificate ICU.

Output: U's purported public key WU.

Actions: V proceeds as follows:

- Verify the content of ICU according to the established infrastructure. This includes verifying the contents of the certificate, such as the subject’s name and the validity period.

- Derive BEU and IU from ICU, according to the certificate format specified during the setup procedure.

- Derive WCA from IU, according to the certificate format specified during the setup procedure.

- Convert the octet string BEU to an elliptic curve point BU as specified in Section 2.3.4 of SEC 1.

- Compute the hash value H = Hash(ICU) using the established hash function. Derive an integer e from H following the conversion routine of Step 5 in Section 4.1.3 of [SEC1].

- Compute the public key WU = e BU + WCA.

After performing the certificate processing operation, V can conclude that WU is genuine, provided U evidences knowledge of the corresponding private key wU.

# APPENDIX B – ECC X509 Certificates

The IETF profile PKIX of X.509 certificates can be found in RFC 2459 and in the draft RFC 3280 (soon to be published). The various algorithm identifiers necessary to use ECC with PKIX can be found in the draft RFC 3278 (soon to be published).

# APPENDIX C – References

# Definitions, Abbreviations, and References

## Definitions and Abbreviations

Our definitions follow those as defined in [ANSI X9.63-2001, §2.1].

## Symbols and Notation

Our notation follows [ANSI X9.63-2001, §2.2].

### Normative References

- ANSI X9.30-1997, The Digital Signature Algorithm (DSA) (Revised), Annex F: Parameter Settings and Security, American Bankers Association, July 1, 1999. Available from http://www.ansi.org.

- ANSI X9.62-1998, Public Key Cryptography for the Financial Services Industry: The Elliptic Curve Digital Signature Algorithm (ECDSA), American Bankers Association, January 7, 1999. Available from http://www.ansi.org.

- ANSI X9.63-2001, Public Key Cryptography for the Financial Services Industry – Key Agreement and Key Transport Using Elliptic Curve Cryptography, American Bankers Association, November 20, 2001. Available from http://www.ansi.org.

- FIPS Pub 180-1, Secure Hash Standard, Federal Information Processing Standards Publication 180-1, US Department of Commerce/N.I.S.T., Springfield, Virginia, April 17, 1995 (supersedes FIPS Pub 180). Available from http://csrc.nist.gov/.

- FIPS Pub 180-2, Draft Specification for the Secure Hash Standard, Federal Information Processing Standards Publication 180-2, US Department of Commerce/N.I.S.T., draft, May 30, 2001. Available from http://csrc.nist.gov/.

- FIPS Pub 186-2, Digital Signature Standard (DSS), Federal Information Processing Standards Publication 186-2, US Department of Commerce/N.I.S.T., Springfield, Virginia, January 27, 2000. Available from http://csrc.nist.gov/.

- FIPS Pub 197, Advanced Encryption Standard (AES), Federal Information Processing Standards Publication 197, US Department of Commerce/N.I.S.T, November 26, 2001. Available from http://csrc.nist.gov/.

- FIPS Pub 198, The Keyed-Hash Message Authentication Code (HMAC), Federal Information Processing Standards Publication 198, US Department of Commerce/N.I.S.T., March 6, 2002. Available from http://csrc.nist.gov/.

- ISO/IEC 9798-1, Information Technology - Security Techniques - Entity Authentication Mechanisms – Part 1: General Model, International Standardization Organization, Geneva, Switzerland, 1991 (first edition). Available from http://www.iso.org/

- ISO/IEC 9798-2, Information Technology - Security Techniques - Entity Authentication Mechanisms – Part 2: Mechanisms Using Symmetric Encipherment Algorithms, International Standardization Organization, Geneva, Switzerland, 1994 (first edition). Available from http://www.iso.org/

- ISO/IEC 9798-3, Information Technology - Security Techniques - Entity Authentication Mechanisms – Part 3: Entity Authentication using a Public Key Algorithm, International Standardization Organization, Geneva, Switzerland, 1993 (first edition). Available from http://www.iso.org/

- NIST Pub 800-38A 2001 ED, Recommendation for Block Cipher Modes of Operation – Methods and Techniques, NIST Special Publication 800-38A, 2001 Edition, US Department of Commerce/N.I.S.T., December 2001. Available from http://csrc.nist.gov/.

- PKIX, L. Bassham, R. Housley, W. Polk, Algorithms and Identifiers for the Internet X.509 Public Key Infrastructure Certificate and CRL Profile, Internet Draft, PKIX Working Group,

October 2001.

- RFC 2104, HMAC: Keyed-Hashing for Message Authentication, Internet Request for Comments 2104, H. Krawczyk, M. Bellare, R. Canetti, February 1997.

- RFC 2119, Key Words for Use in RFCs to Indicate Requirement Levels, Internet Request for Comments 2119, S. Bradner, March 1997.

- RFC 2459, R. Housley, W. Ford, W. Polk, D. Solo, ‘Internet X.509 Public Key Infrastructure: Certificate and CRL Profile’, January 1999.

- RFC 3279, L. Bassham, R. Housley, W. Polk, ‘Algorithms and Identifiers for the Internet X.509 Public Key Infrastructure Certificate and CRL profile’, PKIX Working Group Internet-Draft, October 2001. See also draft-ietf-pkix-ipki-pkalgs-05.txt.

- RFC 3280, W. Ford, R. Housley, W. Polk, D. Solo, ‘Internet X.509 Public Key Infrastructure Certificate and CRL Profile’, PKIX Working Group Internet-Draft, January 2002. See also draft-ietf-pkix-new-part1-12.txt.

- Standards for Efficient Cryptography, SEC 1: Elliptic Curve Cryptography, Version 1.0, Certicom Research, September 20, 2000. Available from http://www.secg.org/.

- Standards for Efficient Cryptography, SEC 2: Recommended Elliptic Curve Domain Parameters, Version 1.0, Certicom Research, September 20, 2000. Available from http://www.secg.org/.

### Informative References

- D.R.L. Brown, R. Gallant, S.A. Vanstone, Provably Secure Implicit Certificate Schemes, in Proceedings of Financial Cryptography 2001, Lecture Notes in Computer Science, Vol. 2339, Berlin: Springer-Verlag, 2002.

- L. Law, A.J. Menezes, M. Qu, J. Solinas, S.A. Vanstone, An Efficient Protocol for Authenticated Key Agreement, Technical Report CORR 1998-05, CACR, University of Waterloo, 1998.  Available from http://www.cacr.math.uwaterloo.ca.

- A.K. Lenstra, Unbelievable Security: Matching AES Security using Public Key Systems, in Proceedings of Advances in Cryptology – ASIACRYPT 2001, December 10-13, 2001.

- A.J. Menezes, P.C. van Oorschot, S.A. Vanstone, Handbook of Applied Cryptography, Boca Raton: CRC Press, 1997.

- L.A. Pintsov, S.A. Vanstone, Postal Revenue Collection in the Digital Age, Technical Report CORR 2000-43, CACR, University of Waterloo, 2000.  Available from http://www.cacr.math.uwaterloo.ca.