IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Comments D10, Heberling/Odman |  |
| Date Submitted | [1 July 2002] |  |
| Source | [Allen Heberling, Proxy] [Knut Odman, Author] [XtremeSpectrum Inc.] [8133 Leesburg Pike] [Suite 700] [Vienna VA 22182] | Voice: [ (703) 269 3058 ] Fax: [ ] E-mail: [aheberling@xtremespectrum.com] E-mail: [kodman@xtremespectrum.com] |
| Re: | [This document provides the Text for D10 LB14 Issue Resolution] |  |
| Abstract | [Proposed changes for acknowledgements in 802.15.3] |  |
| Purpose | [The recommendations contained in this document are to be applied to the 802.15.3 D10 baseline.] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Revision history

r0 – 1-July-2002

original comment document for comment resolution LB14

r1 – 10-July-2002

Added rationale for not combining delayed ACK with other ACK policies, resolution [16].

r2 – 12-July-2002

Updated document reflecting accepted changes during the Vancouver meeting. Comments that were rejected or withdrawn after the suggestions had been solved or obsoleted by other accepted comments are deleted/modified.

r3 – 17-July-2002

Cross referenses to LB17 comment number.

Changes to resolution [12] and [15] based on the following proposals:

02/316r0, Bob Huang, recommend ACCEPT. See resolution [17], page 38

02/289r0, Jay Bain, recommend ACCEPT IN PRINCIPAL, See resolution [18] on page 39-40

Acceptance of James Gilb’s delayed ACK proposal of 7/16/02, minor rev, see resolution [1]

r4 – 23-July-2002

Changes to resolution [14]. Simplified power save based on 
02296r1P802.15_TG3-Powersave_intro.ppt

Changes to resolution [06]. NullCTA is kept in stream termination.

Acknowledgement policies in 802.15.3

comment 1,3

Resolution [01] [WITHDRAW parts regarding ACK request parameters to MLME_DATA.request and forced termination of stream where del-ACK has been declined by destination]

6.3.14.1 MLME_CREATE_STREAM.request

<Add parameter> ACK-policy

Table 18

<Add parameter>

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ACK-policy | Enumeration | no-ACK, imm-ACK, del-ACK | Default ACK request type for stream |
| ResultCode | Enumeration | SUCCESS,TIMEOUT,ILLEGAL_ACK_POLICY | Indicates the result of the MLME request |

6.3.14.4.1 When generated

<add>

- ReasonCode ILLEGAL_ACK_POLICY is returned if a multicast or broadcast stream was opened with any other ACK-Policy than no-ACK.

TE note: The rest of this page is a prerequisite for withdraing comment #3.

6.6, Table 33 – MAC-ASYNC-DATA and MAC-ISOCH-DATA primitive parameters

<add page 86, line 26, TR>

ResultCode, add value DEL_ACK_DENIED

8.8.3 Acknowledgement and retransmission

<change page 186, line 9, TR>

Delayed acknowledgement (Del-ACK) shall be used only for directed isochronous data frames where the Del-ACK mechanism has been set up with bi-partite negotiation between the source and destination DEVs.

<add page 186, line 23, TR>

If the destination DEV wants to decline the use of the Del-ACK mechanism policy, it shall reply with an Imm-ACK frame. The source upon reception of the Imm-ACK shall send a MLME_ISOCH_DATA.confirm with the ResultCode set to DEL_ACK_DENIED to the FCSL. This result shall be interpreted as an acknowledgment of the data frame and additionally an indication that the delayed ACK policy has been refused by the destination.

<add page 186, line 42, TR, text from James Gilb’s proposal 7/16/02>

If the destination DEV receives a frame with any other ACK policy set in the header than delayed ACK once a delayed ACK burst has started but before a frame with the the delayed ACK request bit set in the header, it shall abort the building of a delayed ACK frame and revert to the new indicated policy. Any previously received frames with the delayed ACK policy set shall not be acknowledged. If the source DEV at a later time desires to revert to the delayed ACK policy it shall initiate a new burst size negotiation by sending a single data frame with the ACK-policy set to delayed ACK and the delayed ACK request bit set.

Delayed-ACK

comment 13

Resolution [02] [ACCEPT]

It is not possible to let the source send any number of frames up to

aMaxDelACKBurstSize if they are smaller than aMaxFrameSize and their

combined frame bodies is <= max-burst * aMaxFrameSize. The reason is that

it's not known how much of the header and other overhead the destination

will store. Neither is it a good solution to force all receiver implementations to support a fictive max value set in the standard.

7.3.2.3 Delayed ACK (Del-ACK) frame

Figure 13 – Del-ACK frame format

| 12 | 1 | 1 | 1 | 2 | … | 2 | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MAC-Header+HCS | max frame size burst | max burst size | MPDU ACKed | MPDU Id block | … | MPDU Id block | FCS |

<add>

Max frame size burst. Indicates how many frames of the size aMaxFrameSize can be sent in one burst.

Max burst: Indicates the maximum amount of frames, regardless of size, that can be sent in a burst.

8.8.3 Delayed acknowledgement

<line 16 ff>

If the destination DEV accepts the use of del-ACK, it shall respond with a del-ACK frame, acknowledging the received data frame and setting the max-burst max-frame-size-burst field to a value representing the maximum amount of aMaxFrameSize MPDUs the source DEV may send in one burst. Since the receiver buffer size is equal to big anough to handle max-burst max-frame-size-burst aMaxFrameSize, the source may send as many smaller frames as will fit in the receive buffer window, up to a maximum of aMaxDelACKBurstSize max-burst frames.

<Move from line 23> The destination may change the max-burst max-frame-size-burst in each del-ACK frame, while the max-burst corresponds to an unchangeable physical limitation in the destination DEV.

<line 30 ff>

The source DEV may send any MPDU including retransmissions and new MPDUs, up to the max-burst value set up by the destination DEV in a burst.

Table 54 – MAC sublayer parameters

<delete>

| Parameter | Value |
| --- | --- |
| aMaxDelACKBurstSize | 255 |

Handover in 802.15.3

comment 43

Resolution [03] [ACCEPT in PRINCIPAL]

Handover procedure needs change. You cannot have an MLME timeout expiring in the midst of a synchronized transfer. There must be rules for how many beacons each side sends and how the handover should be done. The handover must be synchronized to a number of beacons with known intervals, not to any timeout.

6.3.11.1 MLME_PNC_HANDOVER.request

<line 39-42, TR>

Change parameters to:

(

AC DEVID,

DEVAssociation-list,

NbrOfHandoverBeacons,

HandoverTimeout

)

Table 15 – MLME_PNC_HANDOVER and MLME_NEW_PNC primitive parameters

<Changes, TR>

NumberOfDEVs  // deducted from DEVAssociation-list

AC-DEVaddress  AC-DEVID

PNCResponse

NewPNCTimeout

<Add>

| DEVInfoSet | implementation dep. | list of association records | see 7.5.4.2 |
| --- | --- | --- | --- |
| NbrOfHandoverBeacons | Integer | 4-255 | Number of beacon the leaving PNC will send with the handover IE present, see |

6.3.11.2 MLME_PNC_HANDOVER.indication

<change parameter 2, TR>

HandoverTimeout

6.3.11.3.2 Effect of receipt

<merge text with this:, TR>

This primitive is sent by the MLME of the PNC leaving the network that the last beacon of the NbrOfHandoverBeacons has been sent and the handover procedure is is completed.

6.3.11.4 MLME_NEW_PNC.indication  [ACCEPT in PRINCIPAL],

<change all of this clause, TR> change to be triggered by last HO beacon by MLME of DEV hearing HO beacons, del all params, see MSC]

<3 new primitives needed, TR >

6.3.11.x MLME_PNC_HANDOVER.response

no parameter.

This primitive ise sent by the DME of the new PNC when the DME is ready to assume PNC responsibility.

6.3.11.x MLME_PNC_HANDOVER_INFO.indication

(DEVInfoSet)

This primitive sent by the MLME to its DME to indicate that a complete list of DEV has been sent from the old PNC during the handover process.

7.4 Table 46 – Information elements

<Add element, TR>

| 0x18 | PNC Handover IE | 7.4.25 |
| --- | --- | --- |

7.4.25 PNC Handover

The PNC Handover element is sent in the NbrOfHandoverBeacons last beacons by the old PNC. The handover countdown element shall count down from NbrOfHandoverBeacons –1 to 0 before the old PNC stops transmitting beacons.

| 1 | 1 | 1 |
| --- | --- | --- |
| Element ID | Length = 1 | handover countdown |

7.5.3.1 New PNC announcement command

<delete all parameters to command., page 136, line 10-30, TR>

7.5.3.21 PNC handover request command

Figure 6362- PNC handover request command format

| 2 | 2 | 1 | 1 |
| --- | --- | --- | --- |
| Command Type | Length=2 | Number of DEVs | Number of CTRB |

<add to first paragraph under figure, page 136, line 44, TR>

The number of DEVs field indicates the total number of DEVs that are currently associated and, if required, authenticated with the PNC, members of the piconet and which information records will be transferred using PNC information command frame(s).

<add page 136, line 46, TR>

The number of CTRB indicates the total number of CTRB, excluding requests for asynchronous channel time, that is currently being served by the PNC and that will be transferred using PNC handover CTRB information command frame(s).

<delete whole paragraph, page 136, line 47-49, TR>

The handover timeout is the time in milliseconds … immediately previous beacon from the current PNC.

<add clause, TR>

7.5.3.2 PNC handover response command

The new PNC shall send this command to the old PNC after all information blocks indicated with the PNC handover request command have been received and the new PNC is ready to take over PNC responsibility.

Figure 63 – PNC handover response command format

| 2 | 2 |
| --- | --- |
| Command Type | Length=4 |

The old PNC, upon receiving this command, will start the handover beacon sequence, see 8.2.3

7.5.3.3 PNC handover information CTRB command

<problem: currenty subrates are impossible to hand over because the start beacon is not known by the new PNC., TR >

Figure 64 – PNC handover information CTRB command format

| 2 | 2 | 1 | 12 | 4 | … | 1 | 12 | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CommandType | Length=(17n) | DEVID | CTRB 1 | Next BCN | … | DEVID | CTRB n | Next Bcn |

<add page 137, line 13, TR>

This command frame may be fragmented.

<add page 137, line 20, TR> The next beacon field is set to the next beacon where this CTA will be allocated.

<delete page 137, line 13-15, TR>

The last field is set to 0 if there are more CTRBs (channel time request blocks) that need to be transferred to the AC. The last field shall be set to 1 in the last command that is required for transferring the last of the CTRBs.

8.2.3 PNC Handover

<Change line 25.5-30, TR> [ACCEPT]

The PNC shall send a PNC handover request command, 7.5.3.2, to it’s chosen DEV with an indication of how the handover timeout many DEVs and how many CTRBs it will hand over. The minimum handover timeout is aMinHandOverTO and the maximum handover timeout is aMaxHandOvrTO. The DEV shall always accept the nomination and be prepared to receive the DEV information and the CTRB list records from the current PNC within the indicated timeout period.

<change line 37ff, TR>

The PNC shall indicate that the transfer is complete by sending a PNC handover information command with the last field set appropriately, 7.5.3.3. The selected DEV shall indicate that it has received all information to be handed over as indicated in the PNC handover request command by sending a PNC handover response command to the old PNC. The old PNC shall ACK the frame and then start the handover beacon sequence.

The new PNC shall announce its new responsibility as PNC in at least aCHFrameRepeat of the superframes before the indicated timeout period using the new PNC announcement command, 7.5.3.1. The new PNC shall send its first beacon at the first expected beacon transmission time after the timeout period indicated in the PNC handover command. The old PNC shall insert the PNC Handover Information Element, 7.4.25, in its beacons. It shall send exactly NbrOfHandoverBeacons, 6.3.11.1, beacons with the handover countdown field of the PNC handover IE counting down from (NbrOfHandoverBeacons –1 ) to 0. The old PNC shall then stop sending beacons. At exactly one Superframe duration,7.4.3, after the old PNC has sent its last beacon with handover countdown=0, the new PNC shall send its first beacon. During the handover beacon sequence, the old PNC shall not increment the Beacon number, 7.4.3. The new PNC shall start counting from old PNC last beacon number + 1. The new PNC shall begin using the PNCID…

8.2.3.2 Not handed over parameters

<add paragraph, TR>

The handover procedure will transfer all information necessary for the new PNC to take over except:

- no asynchronous CTRB will be transferred. All DEVs with asynchronous data to send needs to reissue a new CTR to the new PNC once it has sent its first beacon

- no CTA location is transferred, only duration. This means that all pseudo-static CTA may have changed from the old to the new PNC. All DEVs using pseudo-static streams shall not transmit until a CTA IE is received from the new PNC.

Figure 94 – PNC Handover MSC

<page 154, TR>

General MLME MSC 802.15.3

Resolution [05] [ACCEPT]

Figure xx- general interaction between DME and MLME

Channel Time management in 802.15.3

comment 123

Resolution [06]

8.5 Channel Time management  [ACCEPT]

<Page 175, line 3, TR>

- The reservation creation, modification and termination of asynchronous channel time

8.5.1 Isochronous Stream management

<Page 175, line 13-15, Editorial>

Once a stream index and channel time allocation are established, an isochronous stream may be sent peer to peer, modified or terminated.

<Page 175, line 19-20, TR> [ACCEPT IN PRINCIPAL]

The values for GTS Type, and CTR Interval Type Figure 74 shall be non-negotiable and are Only the values for the desired TU and Stream index, Figure 73 , are negotiable. The rest are decided by the DEV that is sending the channel time request, . These values and shall not be changed anytime after the first transmission of the command frame containing the request for that stream.

<Page 175, line 24, Ed>

channel time status response commands.

<page 177, line 46, TR>

delete PRIORITY_UNSUPPORTED

Figure 115 – MSC for modifying a stream [ACCEPT]

<revert to old, TR>

Chk-resources and allocate resources belongs to MLME

No MLME-MODIFY_CTA.ind/rsp to DME, DME not involved in modify.

Figure 117 – MSC of source DEV-2 requesting termination of its stream [ACCEPT]

<TR>

<Delete BEACON w/ NULL CTA>

<Delete MLME_TERMINATE_CTA.ind  >  // not in dest!

Figure 118 – MSC for a target DEV-2 disassociating causing a source DEV-3 stream termination [ACCEPT]

<delete whole MSC, TR>

<not needed. The source DEV uses the Association IE to see if a peer is disassociated. If so it will send a DISASSOCIATE.indication to DME and terminate all streams to this target>

8.5.2.1 Asynchronous channel time creation and modification reservation [ACCEPT]

<Page 181, line 26, TR>

<delete “CTR Interval type”>

<Page 181, line 37, TR> [ACCEPT]

<delete “and the same destination”>

<Page 181, line 40, E>

However, there is no guarantee of what how long the delay… [?]

<Page 181, line 49, E>

send a channel time status response command…

Figure 119 – MSC for reserving asynchronous data channel time. [ACCEPT]

<the whole MSC completely wrong, change as follows, TR>

- Only use instance heads DEV-2 CL, DEV-2 MAC, PNC, DEV-3 MAC, DEV-3 CL.

- There are no MLMEs involved for asynch CTR. Only the frame is reused.

- No DME activity is involved for async CTR

- Show PNC is one instance to get rid of confusion where Chk-resources etc should be.

8.5.2.1 Asynchronous GTS termination [ACCEPT refuse remote termination]

<add text, page 182, line 37-51, TR>

Termination of asynchronous CTA can only be intitated by the requesting DEV or by the PNC.

Implied ACK in 802.15.3

comment 131

Resolution [07]  [ACCEPT]

< Implied ACK is not needed anymore now when we have efficient asynchronous data support. Besides, implied ACK is a solution for polling protocols, not for time slotted protocols., TR>

5.3.7 Communicating data between DEVs

<Page, line 13-14, TR>

Implied-ACK, 8.8.4, allows the destination of the frame to ACK receipt of the

frame with another command or data frame rather than with an Imm-ACK frame.

Table 37 – Valid ACK request field type values

<Page 104, TR>

Delete the ACK Request type “Implied ACK”, delete its description and replace with “Reserved”

7.5 MAC Command types

<Page 128, line 9, TR>

The PNC may ACK the frame if the ACK request is set to either Imm-ACK or implied-ACK.

<Page 128, line 15, TR>

It may ACK the frame if the ACK request is set to either Imm-ACK or implied-ACK.

<Page 129, line 30, TR>

Also, the ACK request field shall be set to either Imm-ACK or implied-ACK for all commands

unless otherwise stated in the command description.

8.4.1 Inter-frame spacing (IFS)

<Page 165, line 27, TR>

All ACKs, i.e. Imm-ACK frames, frames sent as implied-ACK and Dly-ACK frames, …

8.4.2 Contention based channel access

<Page 165, line 47, TR>

During the CAP a DEV is allowed to transmit one frame at a time with backoff being applied to every frame

attempted during CAP, except for the Imm-ACK frame or a frame sent as an implied-ACK.

8.4.3 Backoff procedure

<Page 166, line 10, TR>

Except when transmitting an Imm-ACK or a frame sent as the implied-ACK,

Figure 111 – End of GTS timing – implied ACK

<Page 174, TR>

Delete the figure and the text above it.

8.7 Fragmentation and defragmentation

<page 185, line 31, TR>

For frames with Imm-ACK or Implied-ACK policy mechanism request, a DEV shall not send another fragment or frame with the same stream number to the same DEV until the sending DEV has received an Imm-ACK frame or

Implied-ACK frame in response to that frame or it has timed out on sending the frame.

8.8 Acknowledgement and retransmission

<page 185, line 402 TR>

There are four three types of acknowledgement types policies defined for this standard; no acknowledgement (no-

ACK), immediate acknowledgment (Imm-ACK), and delayed acknowledgment (Del-ACK) and implied

acknowledgement (implied-ACK).

8.8.4 Implied Acknowledgement

<page 187, TR>

<delete subclause 8.8.4>

Figure 124, Figure 125

<page 188, TR>

<delete Figure 124 and Figure 125>

8.8.5 Retransmissions

<page 188, line 37, TR>

During the time slots within a CFP when either Imm-ACK or the implied-ACK del-ACK is expected,

8.9.2 Probe

<page 189, line 53, TR>

Each time the probe command is sent, it shall have the ACK request set to either Imm-ACK or implied-ACK.

8.9.3 Channel Status request

<page 190, line 53 to page 191, line 1, TR>

Both the channel status request command, 7.5.6.1, and the channel status response command, 7.5.6.2, shall have the ACK policy set to either Imm-ACK or implied-ACK.

Table 53 – Example frame exchanges

<page 206, TR>

delete all implied-ACK entries, delete complete table rows.

Table F.4 – MAC sublayer functions

<page 345, line 52 , TR>

delete MLF 20.4 / implied-ACK entry

Moving pseudo-static GTS 802.15.3

comment 180

Resolution [08] [No resolution in Vancouver, verbal principal agreement]

Informing the receiver when a pseudostatic CTA is moved will be so much overhead that it's unmanagable. Besides, the constructions in there to avoid transmitter contention. Receiver contention is ok! Let whoever missed the CTA in the beacon listen to the whole superframe. Besides, ig the intended receiver misses the CTA in the beacon, how is it going to find out when the PNC wants to inform it about the change?

2nd problem: A PNC must have the authority to arrange CTA as it pleases. It cannot be stopped by a DEV not responding. Especially if it needs to rearrange CTA to fit in a request from a new DEV in a timely fashion. The PNC shall make an effort to inform the transmitter but it shall always proceed with the change.

A third problem is that once the PNC has decided to change something, it must proceed. PNC may have a stronger signal than the DEV, hence the PNC doesn't hear the acknowledgements but the DEVs have heard the order to change. Consequently the PNC must finish what it has started.

8.4.4.1 Guaranteed time slot (GTS)

Changes:

<page 167, line 28-32, TR>

Pseudo-static GTSs shall be allocated only for stream connections. Asynchornous GTSs shall not be pseudo-static. Pseudo-static GTSs may be moved within the CFP by the PNC, but the PNC needs to notify the affected source DEVs by sending the probe command, 7.5.4.3, with the new CTA. As with dynamic GTSs, the PNC may rearrange pseudo-static GTSs so that the GTS assignments will be optimized. However, the PNC uses the probe command and coordinates the CTAs in the probe command with the CTAs in the beacon.

<page 167, line 34-40, TR>

Before a pseudo-static GTS is moved, the PNC shall ensure that the new position is unoccupied by another GTS. Then, the PNC shall send a probe command with the new CTAs to the receiving DEV so that the receiving DEV is listening to bot the old GTS position as well as the new position. The probe command shall be acknowledged with an imm-ACK by the DEV if received correctly. If the PNC does not receive an imm-ACK, it may resend the probe command, but it shall not continue with the process until the acknowledgement is received. The probe command sent by the PNC to the receiving DEV contains both the old and the new CTA. If the old and the new position overlap, the CTA may be one larger CTA.

<page 167, line 42-46, TR>

The PNC shall then send ta probe command to the transmitting DEV that contains only the new CTA. The transmitting DEV shall acknowledge with an Imm-ACK it received correctly. If the PNC does not receive the Imm-ACK, it may resend the probe command, but it shall not move the pseudo-static GTS until the acknowledgement is received. By moving the receiver first, the PNC ensures that no frames are lost if probe commands are corrupted. In either case it shall move the pseudo-static GTS after an acknowledgement is received or the retry limit is reached.

<page 167, line 48-52, TR>

Finally, the PNC shall send a probe command to the receiving DEV which only contains the new CTA. The receiving DEV shall acknowledge this command with an Imm-ACK if it is received correctly. If the PNC does not correctly receive the acknowledgement, it may resend the probe command. However, the PNC may reallocate the original pseudo-static GTS even if it does not receive the acknowledgement to the command.

<page 168, line 1-2, Editorial>

Throughout the process, the PNC will ensure that the beacon contains the current old CTAs for the pseudo-static GTS that is moving.

<page 167, line 4-5, TR>

DEVs may request a change in a pseudo-static GTS channle time useing the channel time request command, 7.5.5.1. // doesn't belong here. Besides, they can request a change of duration of any kind of GTS, but not of location

8.4.4.1 , Figure 104 – CTA assignement for pseudo-static GTSs

<Figure 104: delete. Replace with MSC illustrating move of pseudo-static GTS., TR>

Changing channels and other system parameters in 802.15.3

comment 194

Resolution [10]  [ACCEPT in PRINCIPAL]

When PNC switches channels, the DEVS cannot be free to change at any time within a timeout. If they switch early (or late) they will not hear a beacon and eventually issue a MLME_SYNCH_LOST.indication to their DME. The switch must be synchronized. A similar manner as the new handover proposal is used with a count down to switch. All DEVs must stay awake to receive the beacons and they must switch at the same time. No DEV shall be allowed to transmit until they have correctly received a CTA from their PNC on the new channel.

Changing beacon interval and moving beacon in 802.15.3

comment 196

Resolution [11] [ACCEPT]

A method of moving the beacon is needed. During hand-over, the private CTA of child and neighbor PNC might have been moved. A method of changing the superframe duration is needed. All this must be synchronized throughout the piconet and possible child and neighbor piconets.

NOTE! To make the text readable, this change is a superset of changes in Resolution [10] and [11]

NOTE to TE: The general rule to always have timeout on all MLME doeswn’t apply here since no other DEV is acknowledging anything.

5.3.9 Dynamic channel selection

<delete second bullet, page 19, line 22, TR>

Performing a passive scan of channels, 8.11

6.3.17 Changing channels

<change headline to Changing Piconet Parameters, Editorial>

6.3.17 Changing channels Piconet Parameters

<change table 21, page 71, line 35-53, TR>

This set of primitives allows the PNC to change certain characteristics of the piconet, i.e. the PNID, BSID Superframe Duration, Beacon position or PHY channel. The parameters used for these primitives are defined in Table 21.

Table 21 – MLME_PICONET_PARM_CHANGE primitive parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| NewChannelIndex | Integer | PHY dependent | Index of the new PHY channel |
| ChannelChangeTimeout | Duration | 0-65535 | The time in milliseconds in which the operation initiated by the MLME request needs to be completed before responding with a ResultCode of TIMEOUT |
| NbrOfChangeBeacons | Integer | 2-8 | The number of beacons that will be sent with the Piconet Parameter Change IE before the change takes effect. |
| PNID | Integer | 0-65535 | The ID of the piconet. |
| BeaconDuration | Duration | 0-65535 | The change of superframe duration or beacon position in milliseconds |
| Change type | Enumeration | PARENT_SYNC, PARENT_CHAN,CHANNEL, MOVE, SIZE, PNID | Indicating if the position of the beacon is moved, the size of the superframe is changed or the PNID is changed. PARENT_SYNC and PARENT_CHAN is used by child and neighbor PNC to indicate to their Piconets that their parent is in progress of changing parameters. |
| BSID | String | As defined in 7.4.2 | A text string that identifies the piconet |
| ResultCode | Enumeration | SUCCESS, TIMEOUT, INVALID_PARAMETERS | Indicates the result of the MLME request |

6.3.17.1 MLME_PICONET_PARM_CHANGE.request

<change page 72, line 6-11, TR>

This primitive initiates changing either the piconet PHY channel , the superframe duration, the beacon position or the PNID or BSID. The semantics of the primitive are as follows:

MLME_PICONET_PARM_CHANGE.request

(

NewChannelIndex,

PNID,

BSID,

ChannelChangeTimeout,

BeaconDuration,

ChangeType,

NbrOfChangeBeacons

)

6.3.17.1.1 When generated

<change page 72, line 18-19, TR>

The PNC DME sends this primitive to its MLME after the PNC DME determines that it wishes to change either the piconet PHY channel , the superframe duration, the beacon position or the PNID or BSID. The parameters are described in 7.4.5.

6.3.17.1.2 Effect of receipt

<change page 72, line 23-25, TR>

The PNC MLME upon receiving this primitive initializes the piconet parameter change information element, 7.4.5, with the NewChannelIndex, PNID, BSID and ChannelChangeTimeout, BeaconDuration and ChangeType parameters. The PNC MLME then sends the NbrOfChangeBeacons beacon frames with the piconet parameter change information element as described in 7.4.5, 8.2.7, 8.11, 8.6.5 and 8.6.6.

6.3.17.2.1 When generated

<change text page 72, line 42, TR>

The PNC MLME sends this primitive ti its DME when the PNC has broadcasted the first beacon on the new channel or with the new PNID with the change in effect or BSID or the ChannelChangeTimeout expires

7.4.5 Piconet parameter change

Figure 27 – Piconet parameter change information element format

| octets: 1 | 1 | 1 | 1 | 1 | 2 | 2 | 6-32 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Element ID | Length = 13-39 | Change type | Change countdown | New channel index | Beacon Duration | PNID | BSID |

<insert text page 119, line 44, TR>

The change type field indicates the type of change and shall be encoded as follows:

0 => PNID

1 => BSID

2 => MOVE

3 => SIZE

4 => CHANNEL

5 => PARENT_SYNC

6 => PARENT_CHAN

7-255 Reserved

<change page 119, line 49-51, TR>

The change timeout is the time witing when the DEVs shall expect a beacon from PNC in the channel indicated by the new channel indicated by the new channel index with the PNID and BSID specified in this element. The time duration is in milliseconds.

The Change countdown field shall count down from NbrOfChangeBeacons –1 to 0 before the PNC stops transmitting on the old channel, with the old PNID or with the old superframe size or beacon position.

If Change type is set to CHANNEL,The New channel index field is set to the channel where the PNC will send its first beacon after NbrOfChangeBeacons beacons have been sent on the old channel.

If Change type is set to PARENT_SYNC, the child or neighbor PNC is informing its piconet about a change in the parent piconet’s timing that may affect the child or neighbor piconet.

If Change type is set to PARENT_CHAN, the child or neighbor PNC is informing its piconet about a change in the parent piconet’s PHY channel that will affect the child or neighbor piconet.

If Change type is set to PNID, the PNID field is the new PNID that will take effect when the change timeout has expired beginning with the first beacon after the beacon with Change countdown =0 has been sent. Thus iIf the PNID is not changing, this field is set to the current PNID.

If the Change type is set to MOVE, the Beacon duration is the offset in milliseconds with which the first beacon after after the beacon with Change countdown =0 has been sent will be sent relative the previous beacon position. The Superframe duration field of the synchronization IE shall remain unchanged.

If the Change type is set to SIZE, the Beacon duration field contains the new superframe duration, with which the first beacon after after the beacon with Change countdown =0 has been sent will be sent. The first beacon with the new superframe duration shall have its Superframe duration field of the synchronization IE set to the same value as the Beacon duration field in this element.

If Change type is set to BSID, the BSID field is the new BSID that will take effect when the change timeout has expired beginning with the first beacon after the beacon with Change countdown =0 has been sent. Thus iIf the BSID is not changing, this field is set to the current BSID

8.2.3 PNC handover

<add page 153, line 49, TR>

<this solves the problem that the private CTA may have been moved during handover>

A child or neighbor PNC receiving a parent beacon with a PNC handover information element shall immediately insert the Piconet parameter change element into its beacons with the Change type set to PARENT_SYNC, 7.4.5 and force its DEVs to the AWAKE state.

Any DEV in the child or neighbor piconet shall keep listening until it has received a beacon with CTA from its PNC without the Piconet parameter change IE present.

8.2.7 Setting the BSID and PNID [ACCEPT

<add page 159, line 43, TR>

The PNC shall not change its PNID to the same value as any of its child or neighbor PNCs.

8.6 Synchronization

< add three new subclauses, TR>

8.6.5 Moving Beacon [ACCEPT]

The PNC may need to move its beacon. Moving a beacon means that the superframe duration is unchanged while the position of the beacon is moved to before or after the normal beacon interval. During handover, the private CTA for a child or neighbor piconet may have been moved. Thus the child or neighbor PNC needs to inform its DEVs about where to find future beacons from the child or neighbor PNC.

If the PNC wishes to move its beacon position, it shall insert the piconet parameter change IE, 7.4.5, into NbrOfChangeBeacons beacons with the Change type set to MOVE and the BeaconDuration set to the delay of the first beacon after this sequence compared to previous beacon transmission time.

Figure nnn – moving the beacon position

8.6.6 Changing superframe duration [ACCEPT]

The PNC may need to change the duration of its superframe. This means that the beacon may be sent with larger or smaller intervals than the present beacons..

If the PNC wishes to change its superframe duration, it shall insert the piconet parameter change IE, 7.4.5, into NbrOfChangeBeacons beacons with the Change type set to SIZE and the BeaconDuration set to the size of the superframe following the first beacon after this sequence.

Figure nnn – changing superframe duration

8.6.7 Maintaining synchronization in child and neighbor piconets.

<this solves the problem that the private CTA may have been moved during superframe size change or move>

A child or neighbor PNC receiving a parent beacon with a Piconet parameter change information element, where the Change type is being set to MOVE or SIZE, shall immediately insert the Piconet parameter change element into its beacons with the Change type set to PARENT_SYNC, 7.4.5 and force its DEVs to the AWAKE state.

Any DEV in the child or neighbor piconet shall keep listening until it has received a beacon with CTA from its PNC without the Piconet parameter change IE present.

8.11 Dynamic channel selection [ACCEPT IN PRINCIPAL]

<change page 193, line 46-52, TR>

When the PNC returns to the current channel it shall send a beacon at the appropriate time. If a better channel is available and the PNC decides to switch channels, the PNC shall send the piconet parameter change element in its beacon for NbrOfChangeBeacons beacons, 6.3.17.1. The channel change shall take effect starting with the first beacon after the last of the beacons with the piconet parameter change IE has been sent.

the PNC may send the piconet parameter change element indicating the new channel and the time-out for

changing the channel. The DEVs that received the beacon with piconet parameter change element shall

change the channel to the new channel within the time-out duration immediately before the first expected beacon on the new channel and wait for beacons in the new channel. All DEVs shall not transmit on the new channel until a beacon with correct FCS from the same PNC has been received on the new channel.

Whenever another channel is not available, the PNC may decide to stay in the same channel and hence may

not send piconet parameter change element in its beacon.

<add clause, page 194, TR>

8.11.1 Maintaining child and neighbor piconets during a channel change.

<this solves the problem that the child / neighbor piconet will get lost during a channel change>

A child or neighbor PNC receiving a parent beacon with a Piconet parameter change information element, where the Change type is being set to CHANNEL, shall immediately insert the Piconet parameter change element into its beacons with the Change type set to PARENT_CHAN and the NewChannel set to the same value as in the parent beacon, 7.4.5 and force its DEVs to the AWAKE state. The child or neighbor PNC shall also set the Coundown value to the same as in the parent beacon, regardless of how many beacons per parent superframe the child or neighbor PNC sends.

Any DEV in the child or neighbor piconet that receives a beacon with this IE shall stay awake and keep listening for an IE from its PNC with the countdown field set to 0, or if applicable the absense of beacons. It shall then switch channels to the channel indicated in the IE. It shall not transmit on the new channel until it has correctly received a beacon from its PNC on the new channel.

A neighbor PNC may opt not to switch channels for its piconet. In that case it shall just stay on its current channel and eventially it will be disassociated by the parent PNC, once its ATP runs out. If the neighbor stays on the current channel it shall remove the parent PNID element from its beacon after the piconet parameter change countdown sequence is completed.

A child or neighbor PNC cannot initiate a channel change.

Selection of AC for handover in 802.15.3

comment 206

Resolution [12]  [REJECT handover to child/neighbor. Other unresolved]

It is essential that the AC can handle all associations and streams of the current PNC. SEC and PSAVE if used, Rate, comes last. Table 51 is based on some legacy, AC announcement doesn’t exist and Transmit power is not known.

6.3.5.1 MLME_ASSOCIATE.request

<add parameters, page 34, line 48-51, TR>

MLME_ASSOCIATE.request

(

CapabilityInformation,

AssociationTimeoutPeriod,

PNCMaxAssociations,

PNCMaxStreams,

DEVPIconetServicesIE,

AssocTimeout

)

Table 9 – MLME_ASSOCIATE primitive parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PNCMaxAssociations | Integer | 0-252 | If the DEV is capable of becoming PNC, this value should be set to the maximum number of associations the DEV can handle, otherwise set to 0 |
| PNCMaxStreams | Integer | 0-252 | If the DEV is capable of becoming PNC, this value should be set to the maximum number of streams the DEV can handle, otherwise set to 0 |

7.5.1.1 Association request command

<add fields on page 129 line 46, TR>

Figure 48 –Association request command frame format

| octets:1 | 1 | 6 | 2 | 2 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- |
| Element ID | length=12 | DEV address | ATP | Capability field | Max associations | Max streams |

<insert page 129, line 50, TR>

Max associations is the maximum number of associated DEVs this DEV can handle as a PNC. If the AC bit in the capability field is set to 0, the Max association field shall also be set to 0.

Max streams is the maximum number of streams this DEV can maintain as a PNC. If the AC bit in the capability field is set to 0, the Max streams field shall also be set to 0.

8.2.3 PNC handover

<insert page 153, line 23-26, TR>

The AC bit in the capability field, 7.4.4, is used to indicate that a DEV is capable of being a PNC. Before handing over to the AC, the current PNC shall assure that the new PNC is capable of handling all the currently assigned  associations and streams, as indicated by all DEVs in the association command frame. If the PNC cannot find an AC that is capable of handling all associations and streams, it may choose to disassociate DEVs and terminate streams down to an amount the most capable AC can handle. Alternatively it may choose to do a piconet shutdown, 8.2.6.

If any child or neighbor piconets are present the parent PNC shall only use the shutdown procedure, 8.2.6, using the shutdown IE to indicate which of the remaining PNC’s that will be able to continue.

The PNC shall use the capabilities information of the other DEVs in the piconet with the evaluation criteria defined in Table 51 to select the most qualified AC that is currently associated in the piconet to be the new PNC.

Table 51 – comparison order of fields in AC announcement command for selection of AC

<delete these items, since they are not known on page 155, line 3-16, TR>

| Order | Information | Note |
| --- | --- | --- |
| 5 | Max number GTS | Higher value is preferred |
| 6 | Transmit power level (PHY dependent) | Higher value is preferred |

<change page 155, line 22, TR>

If only one DEV has the PNC Des-Mode bit set, then that DEV would become PNC,provided all other parameters are fulfilled.

System change piconet wakeup in 802.15.3

comment 361

Resolution [13] [Tabled, no resolution in Vancouver]

The current wakeup mechanisms are not sufficient to wake up a DEV when a major system change occurs. Examples are channel change, PNC handover, beacon duration or location change, shutdown and PNID change. A method is needed to allow all APS and SPS devices to easily check if a system change is in progress. The intervals for such checks must be decided by PNC.

Rather than complicating the standard with more programmable constants, we just specify that there is a system change bit that must be read every 4th beacon. If the bit is set, the DEV needs to read the entire beacon to get the information about what kind of system change is occuring. PNC may only issue one system change at a time. Once the DEV has read the beacon it may sleep, as long as it is prepared to comply with the system change when it occurs. If the bit is not set and the DEV has nothing else to do, it doesn’t need to read the rest of the beacon. To facilitate the maximum amount of power save, we put the bit in the MAC header. Even though it’s normally not kosher to use the header for only PNC use, the saved power and increased efficiency makes it worth while!

7.2.1 Frame control field.

<Replace bit 11, formerly Pad bit, with System Change, page 102, line 48, TR>

Figure 7 – Frame control field format

| b15-b14 | b13-b11 | b10 | b9-b8 | b7 | b6 | b5 | b4 | b3-b0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Protocol version | Frame type | SEC | ACK request | Delayed ACK policy | Retry | More data | System change | Reserved |

7.2.1.8 System change field

<change, formerly Pad field, page 104, line 26-30, TR>

This bit is set to 1 by PNC in the header of the beacon frame every time a major change in the piconet is in progress. When no system change is in progress, this bit is set to 0. Each DEV that receives a beacon which header has this bit set, shall set this bit in all its own headers in the following superframe. A DEV that receives a frame with this bit set shall fully interpret the current or following beacon to inform itself about the kind of system change in progress.

The PNC use of this bit is described in 8.6.8.

8.6.8 Maintaining piconet synchronization during system change

<add clause page 183, TR>

Every DEV is required to wake up and receive the header of minimum every 4th beacon, regardless of power save state. If the System change bit is set, the DEV shall read the complete beacon frame to find out the type of system change. It shall continue to do so until the system change bit is no longer set in the header of a beacon. If the bit is not set, the DEV may opt not to interpret the rest of the beacon if it’s in the SLEEP state.

The PNC shall set this bit  during 4 consequtive beacons during a system change, ending with the beacon with the change information element countdown =0, or in the case of a shutdown with its last beacon.

To increase the probability of a successful change, every DEV shall copy this bit from the previous beacon when sending their own frames. If a DEV transmits in a pseudo-static GTS after having missed the preceding beacon, it shall not set this bit. A child or neighbor PNC shall also set this bit in its beacons as long as the parent beacons has this bit set.

A DEV that receives a frame other than a beacon with the System change bit set shall stay awake and fully interpret the following beacon. It shall continue to read the beacons until the system change bit is no longer set in the header of a beacon.

<if the clause 8.6.8 is accepted, the following line shall replace all references to AWAKE state, PCTM bits, etc in the following clauses:, TR>

<new text>

During this procedure the PNC shall set the System change bit, as described in 8.6.8.

<clauses:>

8.2.3 PNC Handover

8.2.6 Stopping piconet services

8.2.7 Setting the BSID and PNID

8.6.5 Moving Beacon

8.6.6 Changing superframe duration

8.6.7 Maintaining synchronization in child and neighbor piconets

8.11 Dynamic channel selection

8.11.1 Maintaining child and neighbor piconets during a channel change

8.12 Power management

<add sentence page 195, line 24, TR>

Regardless of power save mode, the DEV shall always wake up to chack the system change bit as described in 8.6.8.

Power save modes in 802.15.3

comment 365

Resolution [14] [No resolution in Vancouver]

The beacon shares no info about when a certain SPS is awake. There is no handover procedure. A DEV can join several SPS but how does it know when to be awake? How do you send to broadcast of DEVs are in different SPS? What are you supposed to do with "suspended CTA?"? How does transmitters know when an intended receiver is awake? How does it fit wit with ATP? With pseudostat? with subrate? How is PNC supposed to calculate available CTA when DEvs of different SPS may end up with all their CTA needs in the same superframe at some intervals? The idea with PCTM for APS is wrong since PNC should accept or reject CTR for isochronous channel time instantly. The combinations of CTR interval and SPS interval is (mentioned in clause 8 but never defined) will force extremely complicated implementations.

5.3.10 Power management

<change page 19, line 34-36, TR>

This standard provides two techniques a power save mode to enable DEVs to turn off for one or more superframes: asynchronous power save (APS) mode and sychronous power save(SPS) mode. In the piconet, DEVs operate in one of three two modes, ACTIVE mode, APS mode or SPS or PSAVE mode.

<change page 19, line 38-44, TR>

APS PSAVE mode, 8.12.1, is used by DEVs that have no current time allocations to go to sleep for multiple superframes. The DEV sends a request to the PNC when it wants to enter the PSAVE mode. The PNC terminates all streams to the PSAVE DEV and informs the piconet by setting the DEV’s bit in its PS status bitmap in the beacon. with a wakeup time and the PNC responds either with the time requested, a shorter time period or a rejection. If the request is accepted, the DEV switches to APS mode and is not required to listen to beacons. If another DEV in the piconet requests channel time with the APS DEV, the PNC indicates this in the pending channel time map (PCTM) information element in the beacon. When the APS DEV sees this element with its DEVID indicated, it will switch into ACTIVE mode. If the DEV wishes to stay in APS mode, it needs to send a request, 7.5.7.1, to the PNC before the wakeup time expires. PNC will at intervals send wake beacons. All DEVs, including DEVs in PSAVE mode are required to listen to the wake beacons. All asynchronous traffic to DEVs in PSAVE mode will be allocated in the wake beacon.

<delete page 19, line 46, to page 20, line 2, TR>

6.3.19 Power save operation

<delete whole clause, page 74, line 45 – page 79, line 11, TR>

6.3.19 Power save operation

<make new clause, page 74, line 45, TR>

Table 23 – MLME_ PS_MODE_CHANGE primitive parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PSMode | Enumeration | ACTIVE, PSAVE | The PS Mode requested by the DEV DME |
| ModeChangeTimeout | Duration | 0-65535 | duration in millisecond after which the operation shall be considered failed and return TIMEOUT |
| ResultCode | Enumeration | SUCCESS,TIMEOUT | Indicates the result of the operation |

6.3.19.1 MLME_ PS_MODE_CHANGE.request

MLME_ PS_MODE_CHANGE.request

(

PSMode,

RequestedWakeBeaconInterval,

ModeChangeTimeout

)

6.3.19.1.1 When generated

This primitive is generated by the DME when it desires to change the PS mode.

6.3.19.1.2 Effect of receipt

Upon receipt of this primitive, the MLME will send the PS mode change command to PNC indicating the new PS mode. If the MAC is already in the ACTIVE state when the primitive is passed with the PSMode ACTIVE, MLME shall take no action before responding with the MLME_PS_MODE_CHANGE.confirm

6.3.19.2 MLME_ PS_MODE_CHANGE.confirm

MLME_ PS_MODE_CHANGE.confirm

(

ResultCode

)

6.3.19.2.1 When generated

This primitive is sent by the MLME with the ResultCode set to SUCCESS if it got an acknowledgement from the PNC on the PS mode change command, otherwise TIMEOUT.

6.3.19.2.2 Effect of receipt

The DME is informed about the result of the MLME_PS_MODE_CHANGE.request

6.5.4 , Table 28, MAC PIB characteristic group parameters

<delete page 83, line 9-18, TR>

<delete Table entries:>

MACPIB_PowerManagementMode

MACPIB_PowerManagmentAllowed

7.4 Information Elements

Table 46 – Information elements

<change entries on page 116, line 3-5, TR>

7.4.3 Piconet synchronization parameters

Table 23

<add 1 octet, TR>

| b7 | b6-b0 |
| --- | --- |
| wake beacon | wake-beacon countdown |

The wake-beacon bit is set to 1 if the current beacon is a wake beacon, otherwise it’s set to 0.

The wake-beacon countdown indicates how many beacons there will be before the next wake beacon. At the beacon where the countdown would have reached 0, the countdown value is set to the the value of the next count down. If PNC wishes to chain multiple wake beacons, it shall set the wake beacon bit to 1 and the countdown field to 0 in all except the last wake beacon. The last beacon is coded as a normal wake beacon.

//This fits as a information field according to Bill Shvodian’s proposal.

| 0x0E | Traffic Indication Map | 7.4.15 |
| --- | --- | --- |
| 0x0F | SPS Status | 7.4.16 |

7.4.15 Pending channel time map (PCTM)

<delete this subclause, page 124, line 23 – 50, TR>

7.4.16 SPS Status

<delete this subclause, page 125, line 1-14, TR>

7.4.16 CTR Interval announcement IE

<add subclause, page 125, line 1-14, TR>

| octets :1 | 1 | 1 | 1 | 4 |
| --- | --- | --- | --- | --- |
| Information IE | length = 6 | Stream index | CTR interval | Start beacon number |

Figure 40 – PS status information element format

This IE is put into the first wake beacon after a creation or modification of a stream.

The CTR interval is set to the number of beacons between every GTS. If one or multiple GTS per superframe is allocated, this value is set to 0.

The Start beacon number is set to the beacon number of the first beacon where the first GTS of the new or modified stream will be allocated.

7.4.16 PS Status

<add subclause, page 125, line 1-14, TR>

| octets :1 | 1 | 1 | 2-33 |
| --- | --- | --- | --- |
| Information IE | length = 2-33 | Start DEVID | PS bitmap |

Figure 40 – PS status information element format

The bits in the bitmap indiates which DEVs are currently in PSAVE mode. A bit for a DEV is set to 1 if the DEV is in PSAVE mode, otherwise it’s set to 0.

7.5.5.1 Channel time request command

Figure 73 – Channel time request block for a particular stream

<delete field SPS Set index, page 140, line 37, TR>

<delete page 140, line 50-51, TR>

7.5.5.2 Channel time status response command

<add page 142, line 50, TR>

8 => DEST_PSAVE

7.5.7 Power management commands

<delete whole clause, page 145, line 38 – page 150, line 19, TR>

7.5.7 Power management commands

This command is used by a DEV that wants to change its power save mode.

7.5.7.1 PS mode change command

This command shall only be sent by an associated DEV to its PNC.

The PS mode command shall be formatted as illustrated in Figure 83

| octets:2 | 2 | 1 | 1 |
| --- | --- | --- | --- |
| Command type | length = 2 | PSMode | RequestedWakeBeaconInterval |

Figure 83 – PS Mode change command format

The PSMode field indicates the current powersave mode field of the source DEV

0 => ACTIVE

1 => PSAVE

2-255 Reserved

The RequestedWakeBeaconInterval indicates the DEVs preferred interval between wake beacons.

8.2.7 Setting the BSID and PNID

<delete page 159, line 53 – page 160, line 3, TR>

8.4.4.1 Guaranteed Time slots

<delete page 167, line 24-26, TR>

8.5.1.1 Isochronous stream creation

<insert page 175, line 52, TR>

Creation or modification of isochronous streams shall be announced in the wake beacon using the CTR Interval announcement IE, 7.4.16. The PNC shall issue the first GTS for the stream in the superframe following the announcement. If the destination DEV is in the PSAVE mode, or if any DEV is in PSAVE mode in the case of streams with broadcast or unicast destination, the CTR Interval announcement IE and the first allocation shall be inserted in the next wake beacon.

<add page 176, line 5, TR>

,DEST_PSAVE,

8.5.2.1 Asynchronous channel time creation and modification reservation

<insert page 181, line 39, TR>

If any of the DestID is currently in PSAVE mode, PNC shall allocate the CTA with the PSAVE DEVs in the next wake beacon. It’s the responsibility of the source DEV to be able to handle different CTA for DEVs in PSMode from DEVs in ACTIVE mode.

If at any time PNC finds that one or more of the DEVs of a destination list is in PSAVE mode, it shall allocate all the requested CTA in wake beacons if there is enough available CT. Otherwise it shall allocate all the CTA in non wake beacons and allocate as much as possible in the wake beacon.

8.6.2 Beacon generation

<add page 183, line 34, TR>

At intervals to be chosen by the PNC, the PNC shall send a wake beacon. The wake beacon is a normal beacon with the difference that all DEVs are required to listen to it regardless of power save mode. The PNC shall set and decrement the wake-beacon countdown information field, 7.4.3, to enable DEVs to synchronize their sleep period with the next wake beacon.

8.11 Dynamic channel selection

<delete page 194, line 1 – 5, TR>

8.12 Power management

<delete whole clause, page 195, line 1 – page 204, line 46, TR>

8.12 Power management

There are two states defined for this standard. They are AWAKE and SLEEP state. There are two modes of powersave defined for this standard. They are ACTIVE and PSAVE mode. Being in the AWAKE state is defined as listening for traffic. Being in the SLEEP state is defined as having the receiver is shut off. In the ACTIVE mode, a DEV listens to all beacons and will listen in all CTA where its DEVID, BcstId or McstID is indicated as destination. In the PSAVE mode, the DEV is only required to listen to wake beacons and listen to asynchrnous CTA where its DEVID, BcstId or McstID is indicated as destination. Regardless of power save mode, a DEV is allowed to go to the SLEEP state during all GTS where it is not a destination. A DEV is also allowed to switch to the AWAKE state during any time when it’s in PSAVE mode.

Wake beacons are identified with a down counter and it’s the responsibility of the DEV using PSAVE mode to synchronize with the beacon’s wake beacon countdown field before entering the sleep state.

A DEV shall always associate and authenticate with the piconet starting in the ACTIVE mode.

8.12.1 PSAVE mode

A DEV shall send a PS mode command to the PNC with the PSMode field set to PSAVE, and get this command acknowledged, before entering the PSAVE mode.

In the PSAVE mode a DEV cannot send or receive on isochronous streams. If there are any streams allocated where the PSAVE DEV was either source or destination, these streams shall be terminated by the PNC.

The PS status bitmap in the beacon shall serve as indication to a DEV that its peer has gone into PSAVE mode and all streams to this DEV is terminated. The DEV shall indicate to its DME all terminated streams with the MLME_STREAM_TERMINATE.indication.

A DEV in PSAVE mode shall listen to all wake beacons, as announced by PNC. CTA where the PSAVE DEV is destination will be allocated in the wake beacon.

A DEV may listen during other times even if it’s in the PSAVE mode.

The DEV may leave the PSAVE mode by sending a PS mode command to the PNC with the PSMode field set to ACTIVE. Once this command is send the DEV shall regard itself as in the ACTIVE mode regardless if the command was acknowledged by the PNC or not.

Figure 131 – DME initiating change to the PSAVE mode

Figure 132 – DME initiating change to the ACTIVE mode

8.12.2 Live check

Every DEV, regardless of power save mode, shall send a PS mode command or a probe command with all parametrs set to 0 at a minimum once per ATP to maintain association status. If no change is in progress, the DEV needs to send the PS mode command within one ATP of its last acknowledged command sent to the PNC.

Table F.4—MAC sublayer functions

<page 346, line 24-31, TR>

<delete table entries:>

MLF24.1 - Asynchronous power save

MLF24.2 - Synchronous powersave (support for at least one SPS set)

MLF24.3 - Synchronous power save (support for at least four SPS sets)

Handover DEVID conflicts in 802.15.3

comment 422

Resolution [15]  [partial REJECT, no handover to child / neighbor]

Note to TE: Even if the handover to child/neighbor is rejected, the passing of the PNID of the potential child/neighbor to its parent is still needed. The reason is that the parent shall not be allowed to change its PNID to the same as one of its children/neighbors, hence the parent needs to know this onformation. I have deleted all the parts that was only needed for handover to child/neighbor. In addition, a mechanism for parent to decline the formation of a child neighbor is needed (other comment).

<Add to clause 6, TR>

Table x – MLME_INIT_DEPENDENT_PNC primitive parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| OrigID | Integer | Any valid DEVID as defined in 7.2.3 | Specifies the DEVID of the originator of the MLME request |
| PiconetType | Enumeration | CHILD,NEIGHBOR | Specifies the type of piconet as defined in 8.2.4 and 8.2.5 |
| NewPNID | Integer | 0-65535 | The PNID of the new subnet |
| ReasonCode | Enumeration | SUCCESS, CHILD_UNSUPPORTED, NEIGHBOR_UNSUPPORTED, CT_UNAVAILABLE |  |

MLME_INIT_DEPENDENT_PNC.indication

(

OrigID,

PiconetType,

NewPNID

)

When generated:

This primitive is sent by the PNC MLME after receiving a start subnet request command.

Effect of receipt:

The PNC DME is notified about a request to start a child or neighbor piconet.

MLME_INIT_DEPENDENT_PNC.response

(

ReasonCode

)

When generated:

This primitive is sent by the PNC DME as a result of the receipt of an MLME_INIT_DEPENDENT_PNC.indication

Effect of receipt:

The PNC MLME creates a init dependent pnc response command and sends to the indicated DEV.

Clause 7, new frames:

create init dependent pnc request command

| octets:2 | 2 | 2 | 1 |
| --- | --- | --- | --- |
| Command ID | Length=3 | NewPNID | SubnetType |

SubnetType:

0 => Client

1 => Neighbor

2-255 Reserved

init dependent pnc response command

| octets:2 | 2 | 1 |
| --- | --- | --- |
| Command ID | Length=1 | ReasonCode |

ReasonCode:

0 => SUCCESS

1 => CHILD_UNSUPPORTED

2 => NEIGHBOR_UNSUPPORTED

3 => CT_UNAVAILABLE

4-255 Reserved

Vendor specific addditions to 802.15.3

Resolution [16] [ACCEPT]

We cannot have unspecified vendor specific commands and IE.

Solution:

Provide a mechanism for DEVs to find out if they are of the same vendor. Reserve an area for vendor specific commands and IE.

<Make New IE in the standard defined area, TR:>

Figure nn - Vendor Information Element

| 1 | 1 | 8-63 |
| --- | --- | --- |
| IE ID | Length=8-63 | Vendor string |

<Add IE subclause in clause 7, TR:>

Vendor element, if used, shall be exchange during association.

The vendor IE may also be exchange by the probe command to enable vendor specific command exchange in peer to peer frame exchanges.

Before using vendor specific commands and IE, both DEVs shall have used the vendor IE to assure that they will have the same interpretation of the commands and IE.

<7.4, Table 46 page 116, TR>

| Element ID | Element | Subclause |
| --- | --- | --- |
| 0x17-0x7F | Reserved |  |
| 0x80-0xFF | Vendor specific |  |

<7.5, Table 48, page 128-129, TR>

| Command Type | Command Name | Subclause | Associated | Authenticated |
| --- | --- | --- | --- | --- |
| 0x0020-0x00FF | Reserved |  |  |  |
| 0x0100-0xFFFF | Vendor Specific |  |  |  |

<Add Vendor IE to, TR>:

7.5.1.1 Association request command

Vendor IE = Vendor string of associating DEV

7.5.1.2 Association response command

Vendor IE = Vendor string of PNC

Comments on 02/316r0, Bob Huang

comment: ?

Resolution [17] [ACCEPT]

7.4.13: suggest accept

if accepted, the following comments can be withdrawn:

358, 101, 67, 69

8.2.4: suggest reject

Reason: DEVID overlap between two piconets. See comment 350

8.2.6.x: suggest accept

addition: See my resolution [12], added two lines to 8.2.3, handover:

If any child or neighbor piconets are present the parent PNC shall only use the shutdown procedure, 8.2.6, using the shutdown IE to indicate which of the remaining PNC’s that will be able to continue.

Comments on 02/289r0, Jay Bain

comment: 986, 988, 989

Resolution [18] [ACCEPT IN PRINCIPLE]

6.3.x.1: Suggest ACCEPT

Editorial: Parameters should be MACParameter and PiconetDescription, since the PNC candidate only intends to start one PNC with one set of parameters.

Editorial: MACParameterSet would be better as an element of PiconetDescription

Technical: Add parameter CTRB so that MLME independently can request the private channel time. Only needed parameters passed, see below. Why combined? If CTR is done before the request to start subnet, the CTR is wasted upon parent PNC refusal. If the CTR is done after the approval to start a subnet, the requestor’s MLME will not know in which CTA to start sending beacons.

MLME_INIT_DEPENDENT_PNC.request

(

PiconetDescription,  // if it includes MACParameterSet

PiconetMaxTXPower,  // why not in PiconetDescription?

CTR-TU,

MinNumTUs,

DesiredNumTUs,

RequestTimeout

)

6.3.x.2: Suggest ACCEPT

Use ResultCode from Resolution[15], MLME_INIT_INDEPENDENT_PNC.response on page 35 in this document.

Add stream-ID so that child/neighbor PNC may ask parent PNC to get its private CTA modified.

MLME_INIT_DEPENDENT_PNC.confirm

(

StreamIndex,

ResultCode

)

8.2.x Dependent Piconets: suggest REJECT

Editorial: the idea that non 802.15.3 compliant DEVs should be able to do 802.15.3 handshakes is a little bit too optimistic. We should skip all notion of “non conforming neighbors”. How do we ask a HiperLAN 2 base station to form a sub-cluster?

8.2.4: Suggest ACCEPT IN PRINCLIPLE

To make the flow smooth we need the following sequence:

Note that permission to start child/piconet is different from asking for channel time. This is for two reasons:

- Parent PNC should be able to reject the formation of child/neighbor because:
1a) it can only support N of them
1b) it is itself a child or neighbor PNC

- Parent PNC should have a way of shutting down client neighbor PNC. It cannot just withdraw the CTA, then the DEVs associated with the client or neighbor will be left hanging, see comments 352, 354, 356.

Editorial: Figures on page 4 not readable and thus excempt from this comment.

8.2.5: Suggest ACCEPT IN PRINCIPAL

The same MLME can be used for both CTRB and PNC parameters. The PNC startup handshake with PNC (see MSC), needs to come before the CTR.

Editorial: replace NbrID, page 5 line 19, with DEVID.

Editorial: delete line 28-33 about non-compliant neighbor PNC.

Editorial: Figures on page 6 not readable and thus excempt from this comment.

8.2.6: Suggest REJECT

A better child neighbor shutdown process is needed, see comment 352, 354, 356