IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Path Loss Proposed Text and S-V Model Information |  |
| Date Submitted | [4 September, 2002] |  |
| Source | [Jeff Foerster] [Intel Research and Development] [JF3-206 2111 N.E. 25th Ave. Hillsboro, OR 97124] | Voice: [503-264-6859] Fax: [503-264-3483] E-mail: [jeffrey.r.foerster@intel.com] |
| Re: | [] |  |
| Abstract | [] |  |
| Purpose | [The purpose of this contribution is to provide some recommendations for the channel modeling sub-committee final report, and a summary of some of the current S-V model parameters and resulting realizations.] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Path Loss Proposed Text and S-V Model Information

# Path Loss Recommended Text

I would like to propose the following text be included in the final sub-committee recommendations for taking into account large scale path losses.

## Path Loss Model

The main purpose of a path loss model is to fairly compare different physical layer proposals at the target operating distances, and to ensure adequate performance of the final standard.  This can be done by simply adopting the free space path loss model and asking the proposers to provide the resulting link margin that will be available to make up for additional channel losses, implementation losses, waveform distortion, imperfect multipath energy capture, etc.  It will be up to the evaluators to determine whether or not the resulting link margin is suitable for the intended applications.  The table below identifies the parameters needed from the proposer and how those parameters could be used to compute the final link margin.  This model is based on the narrowband path loss calculations (known as the Friis transmission formula), and justification for its use is provided in [3].

The following table lists the parameters and equations that should be used by the proposers to demonstrate the ability of the PHY to close the link at the throughputs and target operating ranges desired for the standard.  The highlighted parameters below are up to the proposers to define, while all other parameters will be consistent with all proposals so easy comparisons can be made.

Table 1: Link Budget Analysis Table

| Parameter | Value | Value |
| --- | --- | --- |
| Throughput | > 110 Mbps | > 200 Mbps |
| Average Tx power (PT) | dBm | dBm |
| Tx antenna gain (GT) | 0 dBi | 0 dBi |
| fc: center frequency of waveform | Hz | Hz |
| Path loss at 1 meter (L1=20log10(4πfcc)) c=3×108 m/s | dB | dB |
| Path loss at d m (L2=20log10(d)) | 20 dB at d=10 meters | 12 dB at d=4 meters |
| Rx antenna gain (GR) | 0 dBi | 0 dBi |
| Rx power (PR=PT+GT+GR−L1−L2 (dB)) | dBm | dBm |
| Noise Bandwidth at antenna port (W) | Hz | Hz |
| Noise power (N=−174+10*log10(W)) | dBm | dBm |
| Rx Noise Figure (NF) | 7 dB | 7 dB |
| Rx Noise Power (PN=N+NF) | dBm | dBm |
| Processing gain (PG : please explain how this is derived) | dB | dB |
| Minimum C/N (S) | dB | dB |
| Link Margin (M=PR+PG−PN−S) | dB | dB |
| Proposed Min. Rx Sensitivity Level | dBm | dBm |

Although the proposers may need to alter the above table for their specific UWB PHY proposal, it gives them an initial framework of the kind of justification and detail that should be part of the proposal.  The final desired output of the link budget should be a final Link Margin that will be needed to account for additional channel losses, implementation losses, waveform distortion, imperfect multipath energy capture, amplitude fading, etc.

# Multipath Model Proposed Text

Following is some proposed text for the multipath model, if the S-V model is agreed upon.  Also, I have included more information about the channel model parameters that have been developed to date and several graphs showing the characteristics of the channel model.

## Multipath Model

Based on this clustering phenomenon observed in the measurements, we propose an UWB channel model derived from the Saleh-Valenuela model with one slight modification.  We recommend using a lognormal distribution rather than a Rayleigh distribution for the multipath gain magnitude, since our observations show that the lognormal distribution seems to better fit the measurement data.  Therefore, the multipath model consists of the following, discrete time impulse response:

h(t)=l=0Lk=0Kαk,lδ(t−Tl−τk,l)

where αk,l is the multipath gain coefficient, Tl is the delay of the lth cluster, and τk,l is the delay of the kth multipath component relative to the lth cluster arrival time (Tl).

Finally, the proposed model uses similar definitions as previously described for the S-V model, and are repeated here for completeness.

Tl = the arrival time of the first path of the l-th cluster;

k,l = the delay of the k-the path within the l-th cluster relative to the first path arrival time, Tl;

 = cluster arrival rate;

 = ray arrival rate, i.e., the arrival rate of path within each cluster.

By definition, we have .  The distribution of cluster arrival time and the ray arrival time are given by

The channel coefficients are defined as follows:

αk,l=pk,lβk,l

20log10(βk,l)∝Normal(μk,l,σ2), or βk,l=10n20 where n∝Normal(μk,l,σ2)

Eβk,l2=Ω0e−TlΓe−τk,lγ

where Tl is the excess delay of bin l and Ω0 is the mean power of the first path of the first cluster, and pk,l is equiprobable +/-1. The k,l is given by

μk,l=10ln(Ω0)−10TlΓ−10τk,lγln(10)−σ2ln(10)20

### Possible modifications to the above channel model

Change 1: Use Nakagami fading distribution rather than log-normal.

Change 2: Allow clusters to fade independently of rays.  For example, each multipath arrival would have a fading term associated with the cluster arrival and a fading term associated with the ray arrival.  This modification could change the channel coefficients in the following way, if the fading for both the cluster and ray amplitudes are log-normal (note that the product of two log-normal random variables results in a log-normal random variable):

αk,l=pk,lξlβk,l,

20log10(ξlβk,l)∝Normal(μk,l,σ12+σ22), or ξlβk,l=10(μk,l+n1+n2)20

where n1∝Normal(0,σ12) and n2∝Normal(0,σ22) are independent,

Eξlβk,l2=Ω0e−TlΓe−τk,lγ,

where Tl is the excess delay of bin l and Ω0 is the mean power of the first path of the first cluster, and pk,l is equiprobable +/-1. The k,l is given by

μk,l=10ln(Ω0)−10TlΓ−10τk,lγln(10)−(σ12+σ22)ln(10)20

In the above equations, ξl reflects the fading associated with the lth cluster, and βk,l corresponds to the fading associated with the kth ray of the lth cluster.

Change 3: In addition to the model realizations, provide some additional channel realizations that capture channel characteristics that are not captured by the statistical model.

Change 4: For LOS, assume first cluster arrival is at time 0.  For NLOS, the first cluster arrival should be random.

### Channel Model parameters

As shown above, there are 5 key parameters that define the model:

 = cluster arrival rate;

 = ray arrival rate, i.e., the arrival rate of path within each cluster;

 = cluster decay factor;

 = ray decay factor;

 = standard deviation of lognormal fading term (dB).

This parameters are found by trying to match important characteristics of the channel.  The main characteristics of the channel that are used to derive the above model parameters are the following:

- Mean excess delay

- RMS delay spread

- Number of multipath components (defined as the number of multipath arrivals that are within 10 dB of the peak multipath arrival)

- Power decay profile

Since the model parameters are difficult to match to the average power decay profile, the main channel characteristics that are used to determine the model parameters are the first three above.  The following table lists some initial model parameters for a couple of different channel characteristics that were found through measurement data.

Table 2 Example multipath channel characteristics and corresponding model parameters.

| Target Channel Characteristics | CM 11 | CM 22 | CM 33 | CM 44 | CM 5 |
| --- | --- | --- | --- | --- | --- |
| Mean excess delay (nsec) (τm) | 5.05 | 10.38 | 14.18 |  |  |
| RMS delay (nsec) (τrms) | 5.28 | 8.03 | 14.28 | 20 |  |
| NP10dB |  |  | 35 |  |  |
| NP (85%) | 24 | 36.1 | 61.54 |  |  |
| Model Parameters |  |  |  |  |  |
|  (1/nsec) | 0.0233 | 0.4 | 0.0667 | 0.0667 |  |
|  (1/nsec) | 3.75 | 1 | 3 | 3 |  |
|  | 7.1 | 5.2 | 14.93 | 17 |  |
|  | 4.37 | 6.5067 | 7.03 | 12 |  |
|  (dB) | 4.8* | 4.8* | 4.8* | 4.8* |  |
| Model Characteristics |  |  |  |  |  |
| Mean excess delay (nsec) (τm) | 5.2737 | 9.8188 | 15.705 | 22.198 |  |
| RMS delay (nsec) (τrms) | 5.5691 | 8.2946 | 14.792 | 19.835 |  |
| NP10dB | 19.3000 | 20.6500 | 33.690 | 50.840 |  |
| NP (85%) | 24.7100 | 34.9800 | 62.460 | 99.86 |  |

* This model is based upon Proposed model change 2.

1 This model is based on LOS (0-4m) channel measurements made by Time Domain.

2 This model is based on NLOS (0-4m) channel measurements made by Time Domain.

3 This model is based on NLOS (4-10m) channel measurements made by Time Domain, and NLOS measurements made by Intel.

4 This model is was generated to fit a 20 nsec RMS delay spread to represent an extreme multipath channel.

## Realizations from channel model

### CM 1

### CM 2

### CM 3

### CM 4

# Appendix

Following is the Matlab code that was used to generate the channel model realizations.

% S-V channel model generation

clear;

% S-V model parameters

Lam=0.0667;               % Cluster arrival rate

lambda=3;          % Ray arrival rate

Gam=14.93;                 % Cluster decay factor

gamma=7.03;              % Ray decay factor

ts=0.167;               % sampling time (nsec)

std_ln=4.8;             % total std of log-normal shadowing for each multipath arrival

std_ln_1=4.8/sqrt(2);             % Standard deviation of log-normal variable associated with Clusters

std_ln_2=4.8/sqrt(2);             % Standard deviation of log-normal variable Rays within a cluster

std_L=sqrt(1/(2*Lam));

std_lam=sqrt(1/(2*lambda));

num_channels=100;

index_imp_resp=floor((10*Gam+10*gamma)/ts)+1;

imp_response=zeros(index_imp_resp,num_channels);

for k=1:num_channels

if (mod(k,10)==0)

k

end;

% Determine Cluster arrivals

Tc=0;                   % First cluster arrival occurs at time 0

% Tc=(std_L*randn)^2+(std_L*randn)^2;  % allow for random first arrival

while (Tc<10*Gam)

tmp_imp_response=zeros(floor((10*Gam+10*gamma)/ts)+1,1);

tmp_imp_response_index=floor(Tc/ts)+1;

mu=(-10*Tc/Gam)/log(10)-(std_ln)^2*log(10)/20;

ln_rv_temp=std_ln_1*randn;                          % Cluster shadowing

ln_rv=mu+std_ln_1*randn+ln_rv_temp;                 % first cluster arrival fading

% ln_rv=mu+std_ln*randn;

pk=2*round(rand)-1;

tmp_imp_response(tmp_imp_response_index)=10^(ln_rv/20)*pk;

% Determine Ray arrivals for each cluster

Tr=(std_lam*randn)^2+(std_lam*randn)^2;

while (Tr<10*gamma)

tmp_imp_response_index=floor((Tc+Tr)/ts)+1;

mu=(-10*Tc/Gam)/log(10)+(-10*Tr/gamma)/log(10)-(std_ln)^2*log(10)/20;  % this assumes log(Omega)=0

ln_rv=mu+std_ln_2*randn+ln_rv_temp;             % Ray fading

% ln_rv=mu+std_ln*randn;

pk=2*round(rand)-1;

tmp_imp_response(tmp_imp_response_index)=10^(ln_rv/20)*pk;

Tr=Tr+(std_lam*randn)^2+(std_lam*randn)^2;

end;

imp_response(:,k)=imp_response(:,k)+tmp_imp_response;   % this sums cluster and ray arrivals

Tc=Tc+(std_L*randn)^2+(std_L*randn)^2;

end;

imp_response(:,k)=imp_response(:,k)/sqrt(imp_response(:,k)'*imp_response(:,k));

% determine excess delay

sq_imp_response=imp_response(:,k).^2;

max_tap=floor((10*Gam+10*gamma)/ts)+1;

t=(0:(max_tap-1))'*ts;

excess_delay(k)=t'*sq_imp_response;

% determine RMS delay

RMS_delay(k)=sqrt((t.^2)'*sq_imp_response-(excess_delay(k))^2);

% determine number of significant paths (paths within 10 dB from peak)

threshold_dB=-10;   % dB

threshold=10^(threshold_dB/20);

temp_imp_resp=abs(imp_response(:,k));

max_imp=max(temp_imp_resp);

norm_imp=temp_imp_resp/max_imp;

a=find(norm_imp>threshold);

num_sig_paths(k)=length(a);

% determine number of sig. paths (captures x % of energy in channel)

x=0.85;

energy=0;

index_e=0;

temp_sort=sort(temp_imp_resp);

max_index=index_imp_resp;

while (energy<x)

energy=energy+temp_sort(max_index-index_e)^2;

index_e=index_e+1;

end;

num_sig_e_paths(k)=index_e;

end;    % num_channels

mean_excess_delay=mean(excess_delay)

mean_RMS_delay=mean(RMS_delay)

mean_sig_paths=mean(num_sig_paths)

mean_sig_e_paths=mean(num_sig_e_paths)

figure(1)

plot(t,imp_response)

grid

title('Impulse response realizations')

% axis([0 50 -0.5 0.5])

xlabel('Delay (nsec)')

ylabel('Amplitude')

figure(2)

plot(excess_delay)

grid

title('Excess delay')

xlabel('Multipath realization number')

ylabel('Excess delay (nsec)')

figure(3)

plot(RMS_delay)

grid

title('RMS delay')

xlabel('Multipath realization number')

ylabel('RMS Delay (nsec)')

figure(4)

plot(num_sig_paths)

grid

title('Number of significant paths > 10 dB from peak')

xlabel('Multipath realization number')

ylabel('Number of significant paths')

figure(5)

plot(num_sig_e_paths)

grid

title('Number of significants capturing > 85% energy')

xlabel('Multipath realization number')

ylabel('RMS Delay (nsec)')

temp_average_power=sum(imp_response'.*(imp_response)')/num_channels;

temp_average_power=temp_average_power/max(temp_average_power);

average_decay_profile_dB=10*log10(temp_average_power);

figure(6)

plot(t,average_decay_profile_dB)

grid

title('Average Power Decay Profile')

% axis([0 20 -20 0])

xlabel('Delay (nsec)')

ylabel('Average power (dB)')

figure(7)

plot(t,imp_response(:,1))

grid

title('Example impulse response realization')

axis([0 100 -0.5 0.5])

xlabel('Delay (nsec)')

ylabel('Amplitude')

% save sv_channel2_90402 imp_response excess_delay RMS_delay num_sig_paths num_sig_e_paths average_decay_profile_dB t;