IEEE P802.15

Wireless Personal Area Networks™

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks™ |  |
| --- | --- | --- |
| Title | P802.15. TG3a Alt PHY Selection Criteria |  |
| Date Submitted | 27 December 2002 |  |
| Source | Ellis/Siwiak/Roberts | jason.ellis@ga.com, kai.siwiak@timedomain.com, rroberts@xtremespectrum.com |
| Re: |  |  |
| Abstract | 03/031r0 is the TG3a document forwarded from SG3a as 02/105r25, which is the last revision of the SG3a document. |  |
| Purpose | [This is a working document that will become the repository for the terms and definitions to be used in the selection process for a Draft Standard for TG P802.15.3a.] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

TABLE OF CONTENTS

1.	Introduction	6

2.	References	6

3.	General Solution Criteria	7

3.1.	Unit Manufacturing Cost/Complexity (UMC)	7

3.1.1.	Definition	7

3.1.2.	Values	8

3.2.	Signal Robustness	8

3.2.1.	General Definitions	8

3.2.2.	Interference and Susceptibility	9

3.2.3.	Coexistence	16

3.3.	Technical Feasibility	20

3.3.1.	Manufacturability	21

3.3.2.	Time to Market	21

3.3.3.	Regulatory Impact	21

3.4.	Scalability	22

3.4.1.	Definition	22

3.4.2.	Values	22

3.5.	Location Awareness	22

3.5.1.	Definition	22

3.5.2.	Values	22

4.	MAC Protocol Supplements	23

4.1.	Alternate PHY Required MAC Enhancements and Modifications	23

4.1.1.	Definition	23

4.1.2.	Values	23

5.	PHY Layer Criteria	24

5.1.	Size and Form Factor	24

5.1.1.	Definition	24

5.1.2.	Values	24

5.2.	PHY-SAP Payload Bit Rate and Data Throughput	24

5.2.1.	Payload Bit Rates	24

5.2.2.	Packet Overhead	24

5.2.3.	PHY-SAP Throughput	25

5.3.	Simultaneously Operating Piconets	26

5.3.1.	Definition	26

5.3.2.	Values	26

5.4.	Signal Acquisition	29

5.4.1.	Definition	29

5.4.2.	Values	29

5.5.	System Performance	30

5.5.1.	Definition	30

5.5.2.	Values	30

5.6.	Link Budget	31

5.6.1.	Definition	31

5.6.2.	Values	31

5.7.	Sensitivity	33

5.7.1.	Definition	33

5.7.2.	Values	33

5.8.	Power Management Modes	33

5.8.1.	Definition	33

5.8.2.	Values	33

5.9.	Power Consumption	33

5.9.1.	Definition	33

5.9.2.	Value	34

5.10.	Antenna Practicality	35

5.10.1.	Definition	35

5.10.2.	Value	35

6.	Annex A:  Criteria Self-Evaluation Method and Matrix	36

6.1.	General Solution Criteria	37

6.2.	PHY Protocol Criteria	38

6.3.	MAC Protocol Enhancement Criteria	39

# Introduction

This is the criteria for the selection of the alternate PHY Draft Proposals. In order to accurately and consistently judge the submitted proposals, technical requirements are needed that reflect the application scenarios that were contributed in response to the call for applications.

This working document will become the repository for the requirements to be used in the selection process for a PHY Draft Standard for P802.15.3a. The criteria presented in this document are based on document [02/030], which takes precedence, and may also contain more general marketing requirements on which the proposers are asked to comment.

The document is divided into four sections: General Solution Criteria, MAC Protocol Supplements Criteria, PHY Layer Criteria, and Annex A. Annex A includes a self evaluation, expected with each proposal, and the evaluation process. Document [02/487] provides the down selection process.

This document and the Requirements document [02/030] provide the technical content for the project to develop an alternate physical layer (alt-PHY). This alt-PHY shall be a supplement to the proposed IEEE 802.15.3 Standard. Revision 0 of this Selection Criteria Document references draft 15 of the proposed IEEE 802.15.3 Standard.

In this document, as per 02/030, the reader will see reference to 110 Mb/s, 200 Mb/s and an optional 480 Mb/s.  The associated distance for these data rates are, respectively, 10 meters, 4 meters and a distance given by the presenter.  The mentioned data rates are minimums and data rates in the actual proposals may be higher than the minimums.

# References

[15.3]	Draft 15 of the proposed IEEE 802.15.3 Standard

[02/104]	IEEE P802.15-02/104, SG3a Technical Requirements

[03/030]	IEEE P802.15-03/030, TG3a Technical Requirements

[02/487]	IEEE P802.15-02/487, SG3a Down Selection Process

[02/490]	IEEE P802.15-02/490, Channel Modeling Sub-committee Report (Final)

# General Solution Criteria

This section defines the technical and marketing system level concerns of the proposals.

## Unit Manufacturing Cost/Complexity (UMC)

### Definition

The cost/complexity of the device must be as minimal as possible for use in the personal area space, see [02/030]. Fig. 1 illustrates the logical blocks in the transceiver PHY layer. Not all blocks are required to implement a communications system. However, if the functionality is used (even optionally) in the specification, then the complexity for implementing the functionality must be included in the estimate. The order and contents of the blocks may vary, for example, the frequency spreading may be a part of the modulate/demodulate portion, and the encode/decode operations might split out to ‘source encode/decode’ and ‘channel encode/decode’.

Figure 1: Logical blocks in the transceiver PHY layer

- Encode/Decode – packet formation including headers, data interleaving, error correction/detection (FEC, CRC, etc.), compression/decompression, bias suppression, symbol spreading/de-spreading (DSSS), data whitening/de-whitening (or scrambling). Modulate/Demodulate – convert digital data to analog format, can include symbol filtering, frequency conversion, frequency filtering.

- Frequency Spreading/De-spreading – can include techniques to increase or decrease, respectively, the Hz/bit of the analog signal in the channel.

- Transmit/Receive – transition the signal to/from the channel.

### Values

Complexity estimates should be provided in terms of both analog and digital die size estimates, semiconductor processes, specified year for process technologies, gate count estimates, and major external components. Similar considerations should be made with regard to MAC enhancements. Reasonable and conservative values should be given. Relative comparisons to existing technologies are acceptable.

## Signal Robustness

### General Definitions

The error rate criterion is the maximum packet error rate (PER) for a specified packet length. The proposer will be asked to indicate the PER, see Sections 2 and 7 of [02/030] used in the determination of this value when indicating the sensitivity of the proposed device. Payload size for the PER test is called out in Section 2 of [02/030] and is intended to be a value between the minimum and maximum packet size.

The receiver sensitivity is the power level of a signal in dBm present at the input of the receiver for which the error rate criteria are achieved in the AWGN environment at the minimum data rates of 110 Mb/s, 200 Mb/s and at the optional data rate of 480 Mb/s. The proposer should include all the calculations used to determine the receiver sensitivity. The power level should be specified at the receiver antenna connection (that is, 0 dBi antenna gain assumed). The error ratio should be determined at the PHY-SAP interface, after any error correction methods required in the proposed device have been applied. The minimum required receiver sensitivity is that sensitivity which produces PER less than 8% for 1024 byte packets when receiving a transmitted signal compliant with regulatory emission levels and producing the data rates of 110 Mb/s, 200 Mb/s and the optional data rate of 480 Mb/s at the PHY-SAP interface over the respective free space distance of 10, 4 meters and a presenter specified distance. Devices may exceed the minimum required sensitivity performance; however, the measurements in Section 3.2 are taken relative to the proposed system receiver sensitivity. The proposed system receiver sensitivity is defined relative to AWGN. The receiver sensitivity is calculated in clause 5.6.2.

The PHY-SAP peer-to-peer data throughput of the device is the net amount of data that is transferred from one PHY SAP to another. Throughput should be measured over at least 200 packets. The connection should already have been established and in progress. The units of the data throughput are in Mb/s. The packet length should be that referenced in document [02/030], section 2, and the throughput should include the normal overhead associated with a packet transmission. Unless otherwise noted, the P802.15.3a transceivers are assumed to use 0 dBi antennas.

### Interference and Susceptibility

#### Definition

Interference susceptibility refers to the impact that other co-located intentional and unintentional radiators may have on a proposed alt-PHY. This section is mainly concerned with the interference coming from other non-P802.15.3a devices. Although there may be a number of systems radiating RF energy in the environments envisioned for P802.15.3a systems, the interference from WLANs (2.4 GHz and 5 GHz), other WPANs (such as 802.15.1, 802.15.3, and 802.15.4), cordless phones (2.4 GHz and 5 GHz), and microwave ovens will be the primary cases considered.

#### Interference Model

The following interferers will be considered:

- Microwave Oven

- IEEE 802.15.1 (Bluetooth)

- IEEE 802.11b

- IEEE 802.15.3

- IEEE 802.11a

- IEEE 802.15.4

- Out-of-band interference from intentional or unintentional radiators

Although other wireless systems may be present, the above systems represent a broad representative set of interferers whose impact has been determined to be sufficient for the evaluation of the proposed alt-PHY solutions based upon the IEEE P802.15.SG3a target applications. Since this document is concerned only with evaluating the capabilities, complexities, and performance implications of proposed physical layers, it is sufficient to use generic models of the above systems in order to ease the burden on the proposers.

The following representative models are suggested.

##### Microwave Oven

The microwave oven is modeled as transmitting at an EIRP of 100 mW with an active period of 8 ms, followed by a dormant period of 8 ms. That is, during the active period the transmit power is 100 mW and during the dormant period the transmit power is 0 mW. During the active period, the microwave oven output can be modeled as a continuous wave interferer with a frequency that moves over a few MHz. At the beginning of the active period, the frequency is 2452 MHz, and at the end of the active period, the frequency is 2458 MHz. There is a continuous sweep in frequency as the active period progresses in time. Pseudorandom data should be used for the modulation of the interferers.

##### Bluetooth™ and IEEE 802.15.1 Interferer

This model is intended to represent the impact of a Bluetooth™ or 802.15.1 device. The following table identifies the parameters of this interferer at the receiving antenna of the proposed P802.15.3a system. Pseudorandom data should be used for the modulation of the interferers.

| Center frequency | 2.4 GHz |
| --- | --- |
| Baud rate | 1 MHz |
| Modulation | GFSK |
| Tx power | 0 dBm |
| Tx antenna gain | 0 dBi |
| Path loss (1) at 1 meter | 40 dB |
| (2) at 0.3 meters | 29.6 dB |
| Rx power (1) at 1 meter | -40 dBm |
| (2) at 0.3 meters | -29.6 dBm |

##### IEEE 802.11b and IEEE 802.15.3 Interferer

This model is intended to represent the impact of an 802.11b or 802.15.3 device. The following table identifies the parameters of this interferer at the receiving antenna of the proposed P802.15.3a system. Pseudorandom data should be used for the modulation of the interferers.

| Center frequency | 2.4 GHz |
| --- | --- |
| Baud rate | 11 MHz |
| Modulation | QPSK |
| Tx power | 20 dBm |
| Tx antenna gain | 0 dBi (handset) |
| Path loss (1) at 1 meter | 40 dB |
| (2) at 0.3 meters | 29.6 dB |
| Rx power (1) at 1 meter | -20 dBm |
| (2) at 0.3 meters | -9.6 dBm |

##### IEEE 802.11a Interferer

This model is intended to represent the impact of an 802.11a device. The following table identifies the parameters of this interferer at the receiving antenna of the proposed P802.15.3a system. Pseudorandom data should be used for the modulation of the interferers.

| Center frequency | 5.3 GHz |
| --- | --- |
| Baud rate | 16.6 MHz |
| Modulation Number of carriers Carrier spacing | 16-QAM OFDM 52 312.5 KHz |
| Tx power | 15 dBm |
| Tx antenna gain | 0 dBi (handset) |
| Path loss (1) at 1 meter | 46.9 dB |
| (2) at 0.3 meters | 36.5 dB |
| Rx power (1) at 1 meter | -31.9 dBm |
| (2) at 0.3 meters | -21.5 dBm |

##### IEEE 802.15.4 Interferer

This model is intended to represent the impact of an 802.15.4 device. The following table identifies the parameters of this interferer at the receiving antenna of the proposed P802.15.3a system. Pseudorandom data should be used for the modulation of the interferers.

| Center frequency | 868 MHz | 915 MHz | 2.450 GHz |
| --- | --- | --- | --- |
| Chip rate | 300 kc/s | 600 kc/s | 1.0 Mc/s |
| Modulation | BPSK | BPSK | O-QPSK |
| Tx power | 3 dBm | 3 dBm | 3 dBm |
| Tx antenna gain | 0 dBi | 0 dBi | 0 dBi |
| Path loss (1) at 1 meter | 31.2 dB | 31.7 dB | 40.2 dB |
| (2) at 0.3 meters | 20.8 dB | 21.2 dB | 29.8 dB |
| Rx power (1) at 1 meter | -28.2 dBm | -28.7 dBm | -37.2 dBm |
| (2) at 0.3 meters | -17.8 dBm | -18.2 dBm | -26.8 dBm |

##### Generic In-band Modulated Interferer

For ultra-wideband based proposals, there may be other wireless systems that may be near the P802.15.3a system that could cause in-band interference. In order to understand how much protection the system will provide in this case of an unknown modulated interferer, the following model is proposed for evaluation.

I(t)=2PIcos(ω0t+θ)k=−∞∞gkv(t−kTI−τI)

where PI is the average received power of the interfering waveform, ω0=2πf0 is the carrier frequency of the “narrowband” waveform, θ is a random phase of the carrier uniformly distributed in 0,2π, {gk} are the randomly modulated BPSK symbols where gk∈±1, TI is the symbol period, τI is a random delay uniformly distributed in [0,TI], and v(t) is the baseband waveform shape. The following table specifies the relevant parameters:

| f0 | Within the bandwidth of the proposal |
| --- | --- |
| 1TI | 5 MHz |
| Modulation | BPSK |
| Baseband waveform | Root Raised Cosine with a roll-off of 0.25 |

##### Generic In-band Tone Interferer

All systems may experience tone interference resulting from close proximity to unintentional radiators like PCs or consumer electronic devices. In order to understand how much protection the system will provide in this case of an unknown modulated interferer, the following model is proposed for evaluation.

I(t)=2PIcos(ω0t+θ)

where PI is the average received power of the interfering waveform, ω0=2πf0 is the carrier frequency of the “narrowband” waveform, and θ is a random phase of the carrier uniformly distributed in 0,2π. For evaluation, f0 should be chosen to be within the bandwidth of the proposal.

#### Evaluation Method and Minimum Criteria

The following subsections describe how the above models can be used for evaluating the performance impact on the proposal. Since the performance of these systems may depend on particular receiver designs, and it is not the intent to standardize certain receiver designs, the proposer should describe any special circuits that were needed to obtain these results (e.g., interference suppression algorithms, notch filters, steep roll-off filters, etc.). Also, all of the following tests should be done using the nominal system configuration which provides the following data rates: 110 Mb/s, 200 Mb/s and an optional 480 Mb/s.

##### Microwave Oven

When this interferer is present, using simulation results, analysis, or technical explanations, describes the impact on the proposed system performance when operating at 6 dB above the proposed systems receiver sensitivity level. This impact should either be a reduction in data throughput or rise in the PER.

Minimum criteria: Proposed system should be able to maintain a PER < 8% for 1024 byte packets when the interference is present at a distance separation of 1 meter from the receiver. If this criteria cannot be met, proposers should define the minimum separation distance between the interferer and the proposed system at which a PER < 8% can be achieved when operating at 6 dB above receiver sensitivity.

##### Bluetooth™ and IEEE 802.15.1 Interferer

When this interferer is present, using simulation results, analysis, or technical explanations, describes the impact on the proposed system performance when operating at 6 dB above the proposed systems receiver sensitivity level. This impact should either be a reduction in data throughput or rise in the PER.

Minimum criteria: Proposed system should be able to maintain a PER < 8% for 1024 byte packets when the interference is present at a distance separation of 1 meters from the receiver. If this criteria cannot be met, proposers should define the minimum separation distance between the interferer and the proposed system at which a PER < 8% can be achieved when operating at 6 dB above receiver sensitivity.

Desired criteria: Proposed system should be able to maintain a PER < 8% for 1024 byte packets when the interference is present at a distance separation of 0.3 meters from the receiver. If this criteria cannot be met, proposers should define the minimum separation distance between the interferer and the proposed system at which a PER < 8% can be achieved when operating at 6 dB above receiver sensitivity.

##### IEEE 802.11b and IEEE 802.15.3 Interferer

When this interferer is present, using simulation results, analysis, or technical explanations, describes the impact on the proposed system performance when operating at 6 dB above the proposed systems receiver sensitivity level. This impact should either be a reduction in data throughput or rise in the PER.

Minimum criteria: Proposed system should be able to maintain a PER < 8% for 1024 byte packets when the interference is present at a distance separation of 1 meters from the receiver. If this criteria cannot be met, proposers should define the minimum separation distance between the interferer and the proposed system at which a PER < 8% can be achieved when operating at 6 dB above receiver sensitivity.

Desired criteria: Proposed system should be able to maintain a PER < 8% for 1024 byte packets when the interference is present at a distance separation of 0.3 meters from the receiver. If this criteria cannot be met, proposers should define the minimum separation distance between the interferer and the proposed system at which a PER < 8% can be achieved when operating at 6 dB above receiver sensitivity.

##### IEEE 802.11a Interferer

When this interferer is present, using simulation results, analysis, or technical explanations, describes the impact on the proposed system performance when operating at 6 dB above the proposed systems receiver sensitivity level. This impact should either be a reduction in data throughput or rise in the PER.

Minimum criteria: Proposed system should be able to maintain a PER < 8% for 1024 byte packets when the interference is present at a distance separation of 1 meters from the receiver. If this criteria cannot be met, proposers should define the minimum separation distance between the interferer and the proposed system at which a PER < 8% can be achieved when operating at 6 dB above receiver sensitivity.

Desired criteria: Proposed system should be able to maintain a PER < 8% for 1024 byte packets when the interference is present at a distance separation of 0.3 meters from the receiver. If this criteria cannot be met, proposers should define the minimum separation distance between the interferer and the proposed system at which a PER < 8% can be achieved when operating at 6 dB above receiver sensitivity.

##### IEEE 802.15.4 Interferer

When this interferer is present, using simulation results, analysis, or technical explanations, describes the impact on the proposed system performance when operating at 6 dB above the proposed systems receiver sensitivity level. This impact should either be a reduction in data throughput or rise in the PER.

Minimum criteria: Proposed system should be able to maintain a PER < 8% for 1024 byte packets when the interference is present at a distance separation of 1 meters from the receiver. If this criteria cannot be met, proposers should define the minimum separation distance between the interferer and the proposed system at which a PER < 8% can be achieved when operating at 6 dB above receiver sensitivity.

Desired criteria: Proposed system should be able to maintain a PER < 8% for 1024 byte packets when the interference is present at a distance separation of 0.3 meters from the receiver. If this criteria cannot be met, proposers should define the minimum separation distance between the interferer and the proposed system at which a PER < 8% can be achieved when operating at 6 dB above receiver sensitivity.

##### Generic In-band Modulated Interferer

When this interferer is present, using simulation results, analysis, or technical explanations, determine the average received interference power, PI, that can be tolerated by the receiver, after it has executed any interference mitigation algorithms, while still maintaining a PER less than 8% for 1024 byte packets where the data rates are 110 Mb/s, 200 Mb/s and an optional 480 Mb/s. The proposer is to show results for a number of different center frequencies or describe how the performance changes as the center frequency changes.

Minimum criteria: PI−Pd > 3 dB (Pd is the received power which is defined here as 6 dB above the receiver sensitivity level) If this criteria cannot be met, proposers should specify the necessary PI−Pd to meet a PER less than 8% for 1024 byte packets where the data rates are 110 Mb/s, 200 Mb/s and the optional 480 Mb/s.

##### Generic In-band Tone Interferer

When this interferer is present, using simulation results, analysis, or technical explanations, determine the average received interference power, PI, that can be tolerated by the receiver, after it has executed any interference mitigation algorithms, while still maintaining a PER less than 8% for 1024 byte packets where the data rates are 110 Mb/s, 200 Mb/s and the optional 480 Mb/s. The proposer is to show results for a number of different center frequencies or describe how the performance change as the center frequency changes.

Minimum criteria: PI−Pd > 3 dB (Pd is the received power which is defined here as 6 dB above the receiver sensitivity level) If this criteria cannot be met, proposers should specify the necessary PI−Pd to meet a PER less than 8% for 1024 byte packets where the data rates are 110 Mb/s, 200 Mbps and the optional 480 Mbps.

##### Out-of-Band Interference from Intentional or Unintentional Radiators

Proposers should report the minimum out-of-band rejection in dB provided by the proposed system. This will provide a minimum standard for out-of-band interferer immunity.

### Coexistence

#### Definition

Coexistence, in this context, refers to the co-location of IEEE P802.15.3a devices with other, non-P802.15.3a devices. The criteria described in this section focuses only on the impact the P802.15.3a devices have on other non-P802.15.3a devices that may be sharing the same frequency bands. The impact of the non-P802.15.3a devices on an P802.15.3a receiver is addressed in Section 3.2.2.

#### Coexistence Model

The following victim receivers which may be co-located with P802.15.3a devices, will be considered here:

- Bluetooth™ (IEEE 802.15.1)

- P802.15.3

- IEEE 802.11b

- IEEE 802.11a

- IEEE 802.15.4

Although other wireless systems may be present, the above systems represent a broad representative set of systems whose impact has been determined to be sufficient for the evaluation of the proposed PHY solutions based upon the IEEE P802.15.SG3a target applications.

Each of the victim receivers listed above operates in unlicensed spectrum and, according to FCC, 47 C.F.R. Sec. 15.5(b), may not cause and must accept harmful interference. For this reason these systems have been specified to operate in presence of other devices sharing the same spectrum. The P802.15.3a coexistence model is consistent with this principle, limited to devices sharing the same frequency band of operation.

For example, proposers using the 5 GHz ISM band are required to show coexistence with 802.11a, not with 802.11b; proposers using the 2.4 GHz ISM band are required to show coexistence with 802.11b, not with 802.11a; proposers using UWB in the 3.1-10.6 GHz bands are required to show coexistence with 802.11a if their system intentionally emits power in the 5 GHz U-NII (Unlicensed National Information Infrastructure) band, not with 802.11b.

The coexistence model, evaluation method and criteria are based on victim receiver’s performance in presence of P802.15.3a transmitters sharing the same frequency of operation, not on P802.15.3a transmit power. This model is consistent with new FCC interference recommendations, described in Spectrum Policy Task Force report, ET Docket No. 02-135, Nov 2002.

The following sections describe in more detail the reference systems that must be considered by each PHY proposal.

##### Bluetooth™ (IEEE 802.15.1) Devices

This model is intended to represent a Bluetooth™ (802.15.1) WPAN™ device. The following table identifies the relevant parameters of the reference system.

| Center frequency | 2.4 GHz |
| --- | --- |
| Baud rate | 1 MHz |
| Modulation | GFSK |
| Tx Power | 0 dBm |
| Rx Antenna Gain | 0 dBi |
| Rx Sensitivity | -70 dBm |

##### IEEE P802.15.3 Devices

This model is intended to represent a high rate P802.15.3 WPAN™ device. The following table identifies the relevant parameters of the reference system.

| Center frequency | 2.4 GHz |
| --- | --- |
| Baud rate | 11 MHz |
| Modulation | DQPSK |
| Tx Power | 0 dBm |
| Rx Antenna Gain | 0 dBi |
| Rx Sensitivity | -75 dBm |

##### IEEE 802.11b Devices

This model is intended to represent an 802.11b WLAN device. The following table identifies the relevant parameters of the reference system.

| Center frequency | 2.4 GHz |
| --- | --- |
| Baud rate | 11 MHz |
| Modulation | CCK (11 Mb/s) |
| Tx Power | 20 dBm |
| Rx Antenna Gain | 0 dBi |
| Rx Sensitivity | -76 dBm |

##### IEEE 802.11a Device

This model is intended to represent an IEEE 802.11a WLAN device. The following table identifies the relevant parameters of the reference system.

| Center frequency | 5.3 GHz |
| --- | --- |
| Baud rate | 11 MHz |
| Modulation Number of carriers Carrier spacing | 16-QAM Coded-OFDM (24 Mb/s mode) 52 312.5 KHz |
| Tx Power | 15 dBm |
| Rx Antenna Gain | 0 dBi |
| Rx Sensitivity | -74 dBm |

##### IEEE 802.15.4 Devices

This model is intended to represent an 802.15.4 WPAN device. The following table identifies the relevant parameters of the reference system.

| Center frequency | 868 MHz | 915 MHz | 2.45 GHz |
| --- | --- | --- | --- |
| Chip rate | 300 kc/s | 600 kc/s | 1.0 Mc/s |
| Modulation | BPSK | BPSK | O-QPSK |
| Tx power | 3 dBm | 3 dBm | 3 dBm |
| Rx antenna gain | 0 dBi | 0 dBi | 0 dBi |
| Rx Sensitivity | -92 dBm | -92 dBm | -85 dBm |

#### Evaluation Method and Minimum Criteria

In order to simplify the criteria, the Interfering Average Power generated by the P802.15.3a transmitter and measured in the relevant bandwidth of the victim receiver at any frequency at which that receiver operates should be used as a parameter to evaluate the coexistence capability of the proposed PHY. This power received by a 0 dBi antenna at the victim receiver frequency should be calculated at 1 m and 0.3 m distance separation between P802.15.3a transmitter and victim receiver.

For example, the minimum receiver sensitivity for an IEEE 802.11a device is –82 dBm in the 6 Mb/s mode and –65 dBm in the 54 Mb/s mode, according to Clause 17.3.10 in IEEE P802.11a/D5.0. The minimum receiver sensitivity for an IEEE 802.11b is –76 dBm in the 11 Mb/s CCK mode, according to Clause 18.4.8 in IEEE P802.11b/D15.The impact on the victim receiver may be estimated by stating the Interfering Average Power in relation to the victim receiver’s minimum sensitivity. Furthermore, the impact on PER can be estimated from curves published in the specifications and standards for the victim receiver.

##### IEEE 802.11a Interferer

Minimum Criteria: The interfering average power generated by the P802.15.3a transmitter and measured in the relevant bandwidth of the victim receiver should be at least 6 dB below the minimum sensitivity level of the 802.11a device operating in the 6 Mb/s mode, when the separation between the P802.15.3a transmitter and victim receiver is 1 m.

Desired Criteria: The interfering average power generated by the P802.15.3a transmitter and measured in the relevant bandwidth of the victim receiver should be at least 6 dB below the minimum sensitivity level of the 802.11a device operating in the 6 Mb/s mode, when the separation between the P802.15.3a transmitter and victim receiver is 0.3 m.

##### IEEE 802.11b Interferer

Minimum Criteria: The interfering average power generated by the P802.15.3a transmitter and measured in the relevant bandwidth of the victim receiver should be at least 6 dB below the minimum sensitivity level of the 802.11b device operating in the 11 Mb/s mode, when the separation between the P802.15.3a transmitter and victim receiver is 1 m.

Desired Criteria: The interfering average power generated by the P802.15.3a transmitter and measured in the relevant bandwidth of the victim receiver should be at least 6 dB below the minimum sensitivity level of the 802.11b device operating in the 11 Mb/s CCK mode, when the separation between the P802.15.3a transmitter and victim receiver is 0.3 m.

## Technical Feasibility

This is intended to determine if the proposal is real or academic. Any proposal may be submitted, but demonstrated feasibility and manufacturability should receive favor over equal but untested proposals. Proposers will be asked to comment on criteria listed in the following sections.

### Manufacturability

#### Definition

Manufacturability is defined in terms of the use of mature, cost effective manufacturing processes with evidence of effective mass production capability.

#### Values

The proposers are asked to submit proof of the claims by way of expert opinion, models, experiments, pre-existence examples, or demonstrations. Globally accepted concepts that will be quick to market, with little risk will be favored.

### Time to Market

#### Definition

Time to Market addresses the question of when the proposed technology will be ready for integration.

#### Values

The proposal shall include an estimate of a schedule for when the PHY would be available for integration.

### Regulatory Impact

#### Definition

The proposal should specify to which geopolitical regions it applies and identify any applicable requirements with which it conflicts. Merit will be awarded for proposals with regulatory compliance of wider geopolitical scope.

#### Values

The proposer shall state which regions the proposal is in regulatory compliance. Merit is awarded for each region of compliance.

Merit awarded for each category:

- Regions adopting US FCC regulations

- Regions adopting European regulations

- Japanese regulations

- Other National Regulations

Specific conflicts and potential derogations should be detailed.

## Scalability

### Definition

Scalability refers to the ability to adjust important parameters, such as those mentioned below, (if they are required by the applications) without rewriting the standard. Scalability should address evolutionary extensions to this proposal and lower throughput modes of operation. The MAC should be able to support the scaling of the PHY (for example: 480 Mb/s at the PHY-SAP).

### Values

Scalability parameters include; power consumption, payload bit rate and data throughput both measured at the PHY-SAP, channelization (physical or coding), complexity, range, frequencies of operation, occupied bandwidth of operation, and other functions deemed appropriate. Proposers are encouraged to show power consumption levels scaling with reduced ranges and reduced bit rates. Proposers are further encouraged to show scalability up to 480 Mb/s and beyond, as well as 110Mb/s and below, as consistent with the table of applications in Section 2 of [02/030].

## Location Awareness

### Definition

Location awareness is the ability to determine information about the relative location of one device with respect to another. The purpose is to improve usability of portable devices. This data can be used to locate, identify and discriminate amongst users in crowded environments and to simplify device registration in constantly changing network topology. Provisions must be made to propagate location information to a suitable management entity.

### Values

Proposers should show that they have the capability to estimate the location of devices and the level of accuracy that can be achieved.

# MAC Protocol Supplements

## Alternate PHY Required MAC Enhancements and Modifications

### Definition

Supplements and modifications to the MAC may be required to accommodate the alternate PHY. It is preferred that the supplements be additions which expand the solution capability as opposed to changes in the MAC that represent an alternative way to do a particular function.

### Values

Proposals should justify and explain the supplements that may be necessary in support of additional features for the alternate PHY.

Proposals should justify and explain the modifications that may be necessary to support or enhance operation of the alternate PHY.

# PHY Layer Criteria

## Size and Form Factor

### Definition

Size is important for consumer electronic systems such as PDAs and cameras.  The smaller the package, the easier it is to embed. It is important that the device be compatible with accessory formats as well. Antennas are not considered in the size requirements.

### Values

Proposers shall provide a time line estimate of when their proposed PHY and the P802.15.3 MAC will fit into the following form factors:

- PC Card

- Compact Flash

- Memory Stick

- SD Memory

## PHY-SAP Payload Bit Rate and Data Throughput

### Payload Bit Rates

#### Definition

The payload bit rate is defined as the bit rate at which the frame body, FCS and any stuffing bits and tail symbols are transmitted. For P802.15.3, examples of optional payload bit rates at the PHY-SAP are 11, 33, 44, 55 and the mandatory payload bit rate is 22 Mb/s.

#### Values

The proposer should provide the payload bit rates to meet the mandatory and optional payload bit rates for the PHY-SAP as defined in clause 2 of [02/030].

### Packet Overhead

#### Definition

For each of the proposed rates the proposer should provide all of the following packet overhead times (illustrated in Fig. 3):

T_PA_INITIAL,

T_PHYHDR,

T_MIFS, and

T_PA_CONT,

where T_PA_CONT is the time for a potentially shortened preamble to be used for subsequent data packets in a single CTA (channel time allocation). The proposer should also provide times proposed for the other interframe spacings, SIFS, RIFS, HCS, and BIFS, as defined in the proposed 802.15.3 standard.

In addition, the proposer should indicate the MPDU times, T_DATA for a 1024-octet data packets (MPDU data bits and FCS combined) and T_MACHDR. In support of these numbers, the proposer should provide the equations and values used to derive these times. The number of octets for the FCS is specified in the MAC clauses of the proposed 802.15.3 standard while the HCS is part of the Alt-PHY. The proposer should provide the descriptions of the proposed PHY Header and MAC Header (if different from the proposed 802.15.3 standard) and description and specification of the functional parts of the PHY preamble. The overhead for interleavers and FEC should be included.  Note: the HCS is a CRC that protects both the MAC and the PHY header.

Figure 3 Packet overhead parameters for data throughput comparison

#### Values

Time values should be stated in microseconds.

### PHY-SAP Throughput

#### Definition

The PHY-SAP data throughput is defined as the bit rate at which a series of 5 MPDUs are transferred from the MAC to the PHY across the PHY-SAP. This is to be computed for the NO-ACK case. The data throughput rate will be lower than the payload bit rate due to packet overhead as defined in 5.2.2.1. The relation of the payload throughput, Payload_Throughput_PHY_SAP, to the payload bit rate, R_Pay, for n frame throughput is given by:

Payload_Throughput_PHY_SAP = n Payload_bits/[T_PA_INITIAL+T_SIFS + (n-1) (T_PA_CONT+T_MIFS) + n (Payload_bits/R_Pay+T_MACHDR + T_PHYHDR+T_HCS+T_FCS)]

Or equivalently:

Payload_Throughput_PHY_SAP = n Payload_bits / [T_PA_INITIAL+T_SIFS+(n-1)  (T_PA_CONT+T_MIFS) + n  (T_DATA+T_MACHDR + T_PHYHDR+T_HCS+T_FCS)]

#### Values

The proposed data throughput rates should be specified in Mb/s for both single frame and the multiframe transmission.

## Simultaneously Operating Piconets

### Definition

The proposed PHY should operate in the close proximity of multiple uncoordinated piconets, at specific bit and error rates.

### Values

The proposal should evaluate the effect of simultaneously operating piconets as specified in clause 3 of [02/030] for the following specified parameters:

- Packet length of 1024 octet frame body

- PHY-SAP bit rates (110 Mb/s, 200 Mb/s and the optional 480 Mb/s)

- Random initial symbol alignment between reference link and interferers

- Meet the baseline performance as indicated in clause 5.5.

- A minimum of 200 packets should be used in estimating the packet error rate.

- The proposer should indicate the values of dint that cause the PER to degrade to a specific level. At a minimum, a free space channel is to be used for all links. It is desired that the environments specified in document [02/490] also be used for the interfering link. The acquisition time should also be stated for all tests. A 0 dBi antenna gain should be assumed throughout.

Figure 4 Test geometry for simultaneously operating piconets

Evaluation geometry and procedure

An interfering transmitter is an uncoordinated transmitter operating at the same power as the reference transmitter. There are two cases to be considered: (1) a co-channel interferer, occupying the same channel and (2) adjacent channel interferer, occupying adjacent channels. If the interfering PHY would have a different impact on the receiver at different supported data rates, the PHY proposer should quantify this.

Single co-channel interferer separation distance is defined as the threshold distance separation (dint) of an interfering co-channel transmitter from the test receiver such that the test receiver PER degrades to a specified error rate.

Multiple adjacent channel interferers separation distance is defined as the threshold distance separation (dint) of multiple interfering transmitters on different adjacent channels equidistant from the test receiver such that the test receiver PER degrades to a specified error rate.

Single Co-channel separation distance test procedure

1. 	Establish a test link with a test receiver at a fixed distance from the reference transmitter, such that the receiver power is 6 dB above the receiver sensitivity level. Continue by sending packets to the test receiver for a specified modulation format and the data rates of 110 Mb/s, 200 Mb/s and the optional 480 Mb/s. When doing the test in multipath channels, at a minimum the 10 required channel realizations from each of the four TG3a channel model scenarios should be used for the test link.

2. 	Verify PER at the test receiver.

3. 	Begin transmitting with a single co-channel interfering alt-PHY transmitter at a large distance from the test receiver. Three pre-specified channel realizations from [02/490] will be used for the interfering links: a very low multipath channel (CM1 delay), a “typical” multipath channel (CM3 delay), and a high multipath channel (CM4 delay). The simultaneous piconet operation shall be assessed for each of the three specified interference channels.

4. 	Continue PER verification at the test receiver.

5. 	Incrementally move the co-channel interfering alt-PHY transmitter closer to the test receiver until the PER exceed the allowable rates.

6. 	Record the distance associated with the last acceptable PER as the single-channel separation distance (dint) for the selected test receiver.

7. 	Since the proposal may include multiple modulation types or other factors that may affect close proximity operation of uncoordinated piconets, the proposer should repeat the test procedures and include sufficient test combinations to characterize system operation under these conditions.

Multi-channel separation distance test procedure

1. 	Establish a test link with a test receiver at a fixed distance from the reference transmitter. Continue by sending packets to the test receiver for a specified modulation format and data rate and range, 110 Mb/s at 10m and 200 Mb/s at 4m. The optional 480 Mb/s has no specified distance. When doing the test in multipath channels, at a minimum the 100 required channel realizations from each of the four TG3a channel model scenarios should be used for the test link.

2. 	Verify PER at the test receiver.

3. 	Begin transmitting with N different adjacent channel interfering alt-PHY transmitters at a large distance from the test receiver. At a minimum, the proposer should consider the cases N = 1, 2, and 3. Three pre-specified channel realizations from [02/490] will be used for the interfering links: a very low multipath channel (CM1 delay), a “typical” multipath channel (CM3 delay), and a high multipath channel (CM4 delay). For N = 1 interferer, the simultaneous piconet operation shall be assessed for each of the three specified interference channels. For N = 2 interferers, all 3 combinations of 2 interference channels shall be assessed. For N = 3 interferers, each interferer will be assigned one of the three unique interference channels.

4. 	Continue PER verification at the test receiver.

5. 	Incrementally move the N different adjacent channel interfering alt-PHY transmitters closer to the test receiver uniformly until the PER exceed the allowable rates.

6. 	Record the distance associated with the last acceptable PER as the multi-channel separation distance (dint) for the selected test receiver.

7. 	Since the proposal includes multiple data rates (110, 200 and optional 480 Mb/s) and may include multiple modulation types or other factors that may affect close proximity operation of uncoordinated piconets, the proposer should repeat the test procedures and include sufficient test combinations to characterize system operation under these conditions.

## Signal Acquisition

### Definition

The signal acquisition methods are the techniques by which the proposed receiver acquires and tracks the incoming signal in order to correctly receive the transmitted data.

### Values

The proposer should provide the false alarm probability and the miss detect probability for the proposed preamble design in both AWGN and the environment specified by the channel model in document [02/490].  The proposer should consider both the single piconet and multiple uncoordinated piconet environment. The proposer should indicate a time-line showing the overall acquisition process, according to the preamble resources devoted to acquisition as specified in this document, at the payload bit rates and ranges specified in document [02/030] Clause 2 subject to the channel model provisions in [02/030] Clause 5. Target acquisition times, reflecting what is specified in the proposed IEEE 802.15.3 Standard, are <6 s for piconet CCA (referenced to the beginning of the preamble) and <20 s for acquisition from the beginning of the preamble to the beginning of the header. Additional information concerning how well the acquisition process scales with payload bit rate would be beneficial.

## System Performance

### Definition

System performance refers to the ability of the system to successfully acquire and demodulate data packets at the required data rates and bit and packet error rates, both in the free space AWGN channel and in the multipath channels specified by the channel model document [02/490].

Performance of the proposed system as measured in various multipath environments defined in the latest revision of document [02/490]. The multipath performance can be used to evaluate the losses incurred due to multipath.

The proposer will be asked for 90% PER link success probability where a 90% outage PER is defined as the PER averaged over the channels which result in the 90% best performance at a given Eb/N0 for a particular channel environment, i.e., the PER performance due to the worst 10% channels at a given Eb/N0 should not be included in the average PER calculation.

Values

The proposer should provide the probability of link success (the ability to acquire and pass data with the specified packet length and PER at minimum payload bit rates for the PHY-SAP for both AWGN and the channel model specified in document [02/490], relative to distance). The proposer should further indicate the range at which the proposed PHY can acquire and meet the bit rate packet length and PER requirements of clause 2.0 of [02/030] for the channel model specified in document [02/490] for a link success probability of 90%. The proposer should indicate PER and acquisition performance as a function of the distance. The acquisition parameters (signaling and duration) should be noted for all scenarios.

The proposal must include the 90% outage packet error rate (PER) as a function of Eb/N0 for each of the multipath environments (CM1 through CM4).  Eb is computed as the average multi-path signal energy, averaged over the 100 channel realizations for each channel environment.

## Link Budget

### Definition

Link budget is used to determine proposal capabilities under certain operating conditions for the standards specified bit rates, ranges, and bit error rate.

### Values

The table below, see [02/490], identifies the necessary parameters and equations that should be used to compute the final link margin. Proposers should complete this link budget table and identify and explain all assumptions. Although the proposers may need to make minor alterations to this table to more adequately reflect their proposal, the table identifies the minimum expected level of thoroughness, detail, and justification.

Source: [02/490]

| Parameter | Value | Value | Value |
| --- | --- | --- | --- |
| Throughput (Rb) | > 110 Mb/s | > 200 Mb/s | > 480 Mb/s (optional) |
| Average Tx power (PT) | dBm | DBm | dBm |
| Tx antenna gain (GT) | 0 dBi | 0 dBi | 0 dBi |
| fc'=fminfmax: geometric center frequency of waveform (fmin and fmax are the -10 dB edges of the waveform spectrum) | Hz | Hz | Hz |
| Path loss at 1 meter (L1=20log10(4πfc'c)) c=3×108 m/s | dB | DB | dB |
| Path loss at d m (L2=20log10(d)) | 20 dB at d=10 meters | 12 dB at d=4 meters | presenter specified |
| Rx antenna gain (GR) | 0 dBi | 0 dBi | 0 dBi |
| Rx power (PR=PT+GT+GR−L1−L2 (dB)) | dBm | DBm | dBm |
| Average noise power per bit (N=−174+10*log10(Rb)) | dBm | DBm | dBm |
| Rx Noise Figure Referred to the Antenna Terminal (NF)1 | dB | DB | dB |
| Average noise power per bit (PN=N+NF) | dBm | DBm | dBm |
| Minimum Eb/N0 (S) | dB | DB | dB |
| Implementation Loss2 (I) | dB | DB | dB |
| Link Margin (M=PR−PN−S−I) | dB | DB | dB |
| Proposed Min. Rx Sensitivity Level3 | dBm | DBm | dBm |

1 Per text book definition, the NF is the ratio of the SNR at the antenna output with respect to the SNR at the demodulator input (reference figure 1).  The NF should include not only the LNA but also cascaded stages as per Friis’ equation.  Each proposer should justify the proposed noise figure number, or else use a default value of 11 dB.

2 Implementation loss is defined here for the AWGN channel only, and could include such impairments as filter distortion, phase noise, frequency errors, etc.

3 The minimum Rx sensitivity level is defined as the minimum required average Rx power for a received symbol in AWGN, and should include effects of code rate and modulation.

Sensitivity

### Definition

Sensitivity is defined in 3.2.1. It is important for the proposal to specify the sensitivity level used in the determination of the signal robustness criteria.

### Values

The proposal should indicate the power level at which the error criterion is met, consistent with the link budget as presented in document [02/490], Table 1. The proposal should also indicate the PER used in the determination of this value.

## Power Management Modes

The ability to reduce power consumption for consumer electronic devices is important.

### Definition

Power management modes and protocols allow device sleep, wakeup, and poll. The proposed 802.15.3 standard provides such power management capabilities.

### Values

The proposal should explain if it supports each of the power management methods as defined in the proposed 802.15.3 standard.

## Power Consumption

### Definition

Power consumption is defined as the total average power required by the proposed system to operate in transmit, receive, clear channel assessment, or power saving modes. It includes the power consumed by all components necessary to implement all of the functionality of the proposed alternate PHY from the PHY-SAP interface, defined in the proposed 802.15.3 standard, down to an antenna, where the gain is disclosed by the proposer. No components supporting operation above the PHY- SAP interface are included in the average power consumption value.

#### Transmit

Power consumption during transmit state is defined as the average power consumed from the PHY-TX-START.request for a given MPDU, to the PHY-TX-END.confirm.

#### Receive

Power consumption during receive state is defined as the average power consumed from the PHY-RX-START.request for a given MPDU, to the PHY-RX-END.indication where the PHY-RX-START.request is assumed to be coincident with the remote transmission beginning.

#### Clear Channel Assessment

Power consumption during clear channel assessment (CCA) is defined as the average power consumed from the PHY-CCA-START.request to the PHY-CCA-END.confirm.

#### Power Save

Power consumption during the power save state is defined as the power consumed from the PHY-PS.request to the PHY-PS.confirm resulting from a subsequent PHY-PS.request with a PSLevel value of 0. Methods for achieving power save modes and the impact to the operation (acquisition, time to come ‘awake”, etc…) of the PHY should be described.

### Value

Power consumption values are to be disclosed with sufficient explanation of how the numbers are derived. These numbers should reflect operation at the RF power necessary to achieve the continuous full bit rate/throughput at the maximum range including the disclosed antenna gain. To help aid comparison among proposals, disclosure should include parameters such as technology process, clock rate, voltage, etc.

#### Transmit

The proposal should estimate the power consumption for the PHY throughputs specified in section 5.2 with proposed minimum and maximum PHY frame lengths.

#### Receive

The proposal should estimate the power consumption for the PHY throughputs specified in section 5.2 with proposed minimum and maximum PHY frame lengths.

#### Clear Channel Assessment

The proposal should state the estimated power consumed during both channel "busy" periods and channel "idle" periods.

#### Power Save

The proposal should specify the power consumption associated with the lowest supported power consumption level (PwrMgtLevel). The proposal should also provide values for power save group parameters as specified in 802.15.3. Proposals should provide justification for its stated power save values (for example, circuits disabled, clocks turned off, etc…).

## Antenna Practicality

### Definition

The antenna form factor should be consistent with the following form factors:

- PC Card

- Compact Flash

- Memory Stick

- SD Memory

### Value

Antenna form factor should be described with reference to expected size. Any additional information the proposer desires to provide on the antenna such as size, frequency response, impulse response and radiation characteristics would be beneficial.

# Annex A:  Criteria Self-Evaluation Method and Matrix

Each proposer will evaluate their own proposal, based on the selection criteria referenced in the tables below, and will include this information in their proposal presentation. In order to indicate the importance level of each of the criteria, an ABC leveling scheme is used, as was determined by the mutual consent of SG3a during the November 2002 meeting.

- A – Most Important Requirement

- B – Important Desired Requirement

- C – A “Nice to Have” Requirement

Once the proposals have been presented, each token-bearing TG3a committee member will have an opportunity to score each proposal. This information will not be collected and is intended solely to help individual committee members with their own evaluations of each of the proposals. Scoring will be achieved by rating each proposal against each of the criteria indicated below. Three scoring levels are possible:  -, 0, +.

## General Solution Criteria

| CRITERIA | REF. | IMPORTANCE LEVEL | PROPOSER RESPONSE |
| --- | --- | --- | --- |
| Unit Manufacturing Complexity (UMC) | 3.1 | B |  |
| Signal Robustness |  |  |  |
| Interference And Susceptibility | 3.2.2 | A |  |
| Coexistence | 3.2.3 | A |  |
| Technical Feasibility |  |  |  |
| Manufacturability | 3.3.1 | A |  |
| Time To Market | 3.3.2 | A |  |
| Regulatory Impact | 3.3.3 | A |  |
| Scalability (i.e. Payload Bit Rate/Data Throughput, Channelization – physical or coded, Complexity, Range, Frequencies of Operation, Bandwidth of Operation, Power Consumption) | 3.4 | A |  |
| Location Awareness | 3.5 | C |  |

## PHY Protocol Criteria

| CRITERIA | REF. | IMPORTANCE LEVEL | PROPOSER RESPONSE |
| --- | --- | --- | --- |
| Size And Form Factor | 5.1 | B |  |
| PHY-SAP Payload Bit Rate & Data Throughput |  |  |  |
| Payload Bit Rate | 5.2.1 | A |  |
| PHY-SAP Data Throughput | 5.2.2 | A |  |
| Simultaneously Operating Piconets | 5.3 | A |  |
| Signal Acquisition | 5.4 | A |  |
| Link Budget | 5.5 | A |  |
| Sensitivity | 5.6 | A |  |
| Multi-Path Immunity | 5.7 | A |  |
| Power Management Modes | 5.8 | B |  |
| Power Consumption | 5.9 | A |  |
| Antenna Practicality | 5.10 | B |  |

## MAC Protocol Enhancement Criteria

| CRITERIA | REF. | IMPORTANCE LEVEL | PROPOSER RESPONSE |
| --- | --- | --- | --- |
| MAC Enhancements And Modifications | 4.1. | C |  |