IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Preamble, Phy header protection & ACK aided AMC |  |
| Date Submitted | May 5, 2003 |  |
| Source | [Michael Park] [Samsung Electronics co., Ltd] [416, Maetan 3-dong, Paldal-gu, Suwon-si, Korea] | Voice: [+82-31-279-5108] Fax: [+82-31-279-5130] E-mail:[si0519.park@samsung.com] |
| Re: | [None] |  |
| Abstract | Propose Preamble Structure & Phy header Protection Scheme |  |
| Purpose | Make a proposal |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Table of Contents

Introduction	3

Part I	3

1	Introduction	3

2	Criterion	3

3	Proposed scheme	3

4	Performance	5

4.1Correlation property	5

4.2Detection probability	7

5	Implementation issue	8

6	Conclusion	8

Part II	9

7	Introduction	9

8	Criterion	9

9	Proposed scheme	10

10	Performance	12

11	Conclusion	17

Part III	17

12	Introduction	17

13	Criteria	18

14	Proposed scheme	18

15	Conclusion	22

Conclusion of This Proposal	22

- Introduction

This contribution consists of two parts, the first part is for the preamble structure and the second part is for the Phy header protection scheme.

- Part I

Title: Preamble structure for synchronization

- Introduction

In this contribution, we propose ARM sequence for synchronization in preamble of UWB system. The preamble is used for the synchronization and the channel estimation. In 802.15.3 specification, QPSK is used for the transmit modulation. But QPSK is not adaptive for UWB system. So, Bi-phase modulation is desirable in UWB system. In this proposal, the proposed preamble will be a Bi-phase sequence. In Bi-phase modulation, the symbol length of CAZAC sequence is 4. The short code length leads to the performance degradation in the view of the synchronization.

As the preamble is used for the synchronization and the channel estimation, we separate the preamble into two parts. One is a synchronization part, and the other is a channel estimation part. We propose the synchronization part of preamble in Bi-phase modulated UWB system. The aperiodic sequence is used for synchronization part. We propose the ARM sequence as a synchronization part in the preamble.

- Criterion

The preamble considers below:

- Synchronization

Generally, the aperiodic property of sequence is a measure of synchronization performance. So, it is important to use sequence with a good aperiodic property.

- Channel Estimation

The channel estimation is also important function of preamble. It, however, is out of scope in our contribution.

- Proposed scheme

We suggest ARM sequence for the preamble in UWB system. ARM sequence is aperiodic sequence. The code generation is very simple. Generally, finding good aperiodic sequence with large code length is very complicated. Therefore, the ARM sequence has a good property in terms of hardware complexity. ARM sequence has the property that the aperiodic correlation values in the even shift case are equal to ‘0’. In Fig.1, the structure of ARM sequence generator is depicted. For example of ARM sequence with length 128, the initial value is set to ‘00’, and other inputs to XORs are ‘01…’ patterns. The output sequence with length 128 is as follows:

0001 0010 0001 1101 0001 0010 1110 0010

0001 0010 0001 1101 1110 1101 0001 1101

0001 0010 0001 1101 0001 0010 1110 0010

1110 1101 1110 0010 0001 0010 1110 0010

Figure 1 ARM sequence generator

In 802.15.3 specification, the preamble code length is 160 symbols. So we propose the preamble structure with length 160. We separate two parts, one is synchronization part, and the other is channel estimation part. Length of preamble code set is 160 symbols. First 128symbols are to synchronize. Second 32symbols are to estimate channel. Our proposed preamble structure is shown in Fig 2.

Figure 2 Preamble structure

- Performance

  - Correlation property

The correlation property of ARM sequence with length 128 is shown in Fig 3. In case of in-phase, it has a peak correlation value 128. In the other hand, the correlation value in case of out-of-phase are very low as shown in Fig 3.

Figure 3 Aperiodic correlation of ARM sequence with length 128

In Fig 4, we compare the correlation property between ARM sequence and Binary CAZAC sequence in term of the power of the correlation value. In this contribution, Binary CAZAC sequence means 32times repetition of binary CAZAC sequence with length 4. Generally, the merit factor is used to measure the performance of aperiodic sequence. As shown in (1), the merit factor is the ratio of peak power on time and sum of the side-lobe powers.

(1)

If we have a high merit factor, the peak power is enoughly larger than side-power. In the case of the length 128, the merit factor of ARM sequence is 2.9767(=16384/(21888-16384)) and the merit factor of Binary CAZAC sequence is 0.0491(= 16384/(349824-16384)). As a result,in the sense of the merit factor, the performance of ARM sequence is better than that of Binary CAZAC sequence.

Figure 4 Comparison of Power of correlation between ARM sequence and Binary CAZAC sequence

In Fig 5, we comparecorrelation values between ARM sequence and Binary CAZAC sequence.  Similar to the case of the correlation power comparison in Fig 4, it is easily known that ARM sequence has better performance.

Figure 5 Comparison of Correlation values between ARM sequence and Binary CAZAC sequence

  - Detection probability

In this section, we will show detection probabilities of ARM sequence in 5 channel models.

In Fig. 6, it shows detection probabilities of ARM sequence and Binary CAZAC sequence when the probability of false alarm is set to 0.1%. The channel models are AWGN, CM1, and CM4. The CM1 and the CM4 represent IEEE 802.15.3a channel model 1 and channel model 4, respectively. ARM sequence has better detection probabilities than those of the Binary CAZAC sequence for all cases.

Figure 6 Comparison of Detection probabilities between ARM sequence and Binary CAZAC sequence

We show the simulation results of detection probabilities of 5 channel models in Fig 7. The sequence is ARM sequence and length of the sequence is 128.

Figure 7 Detection probabilities of the ARM sequence with length 128 in each Channel Models

- Implementation issue

We propose ARM sequence for synchronization part of preamble. As shown in Fig 1, the structure of ARM sequence generator is so simple. We need 6 MUXs and 6 adders to generate ARM sequence.

- Conclusion

The proposed structure of preamble has two parts; synchronization part and channel estimation part. For synchronization part, we proposed ARM sequence with length 128. The proposed ARM has a good aperiodic property. Moreover, ARM sequence is simple to generate. It is very suitable for preamble of UWB to synchronize.

- Part II

Title: Error Correcting Code for Physical Header

- Introduction

The purpose of this proposal is to suggest the protection of physical header by error correcting code. Generally, the physical header contains the information of physical frame format. So, recovering physical header information is very important in communication systems because in case of receiving failure of physical header entire frame can not be recovered. An irrecoverable error in physical header causes whole frame to be discarded even though the rest of frame is uncorrupted. To tackle this problem, an error correcting code is considered. By using the error correcting code, the physical header that is corrupted by noise can be recovered. It will improve the efficiency of channel and increase the throughput of network. Conclusively we suggest that physical header be protected by proposed error correcting code with optimal minimum distance.

- Criterion

We have to consider main three criteria to select the channel coding scheme. These are the minimum distance, the availability of the soft decision decoding scheme, and the decoding complexity.

1. Minimum distance

The minimum distance is the smallest Hamming distance of code among codes that in same code group. The minimum distance is a good measurement of the block code performance. The larger its minimum distance, the better performance. So the minimum distance is a very important factor when designing an error correcting code.

2. Availability of the soft decision decoding scheme

Most of the block code can not use the soft decision decoding algorithm. The soft decision decoding algorithm is superior to hard decision decoding algorithm. In AWGN channel environment, the soft decision decoding algorithm has the coding gain more than 2dB when compared to the hard decision decoding algorithm. It is desirable to select the block coding scheme that the soft decision decoding algorithm can be adapted.

3. Decoding complexity

It is desirable to design the block code which has the simple decoding scheme. Well designed coding schemes have merits of simple encoding and decoding structures.

- Proposed scheme

In this proposal, we propose the sub-code of 2nd order Reed Muller code. The suggesting code has three main advantages. At first, in the sense of minimum distance, the proposed coding scheme is optimal. The second, it can use the soft decision decoding. The third, based on IFHT (Inverse Fast Hadamard Transform) technique, the hardware complexity can be reduced.

The sub-code of 2nd order Reed-Muller code is optimal code. In case of (32, 11) block code, the optimal bound of the minimum distance is 12. The sub-code of 2nd order Reed-Muller code has the minimum distance 12. So it is optimal code. It is natural that the optimal code shows best performance. Sub-code of 2nd order Reed-Muller code is soft decision capable block code by using IFHT technique. As describe before soft decision decoding algorithm is superior to hard decision decoding algorithm in coding gain. In addition IFHT technique can reduce hardware complexity by simplified calculation amount.

G=01010101010101010101010101010101001100110011001100110011001100110000111100001111000011110000111100000000111111110000000011111111000000000000000011111111111111111111111111111111111111111111111101101111000001100011010101011100000111010111101100100001101110000000101001100011010111111100100100100001110111100111010010001011

Figure 8 generation matrix of sub-code of 2nd order Reed-Muller

Figure 8 is generating matrix of sub-code of 2nd Reed-Muller code. It consists of Walsh code (upper part) and code that makes the 2nd order Reed-Muller code’s minimum distance large (lower part). We call this code ‘mask function’ from now on. Walsh code and the mask function are linear code so all codes are presented by combination of basis.

Figure 9 structure of encoder

Figure 9 shows the encoder structure of proposed coding scheme. In encoding state input bit is divided into two parts. One is consist of 6 bits that make Walsh combination. Another consists of 5 bits that make mask function. Proposed code is summation of two combined codes.

Figure 10 frame structure of example system

Figure 10 is example of frame structure that can applied proposed coding scheme. MAC layer attaches MAC header and MAC payload and FCS (Frame Check Sequence) and Physical layer calculate HCS (Header Check Sequence) of PHY and MAC header and attaches HCS and preamble to MAC header and MAC header. After attaching process, physical header is coded by proposed coding scheme and the rest of frame may be coded by convolution or turbo code.

- Performance

The fallowing figures are BER and FER performances for the coded and uncoded physical header as a function of Eb/N0 (dB). Channel model is based on ‘IEEE P802.15-02/368r5-SG3a’

Figure 11 Performance in AWGN

Figure 11 shows the error performance for AWGN channel. It is noticed that the BER and FER performance of coded PHY header outperform. Especially gain of FER is significant compared to that of BER. For example, at BER 10-2 , coding gain is 2dB.

Figure 12 Performance in CM1 at 200 Mb/s

Figure 12 is error performance at channel model 1 data rate of 200 Mb/s. Trend is similar with previous result. For example at BER 10-2, coding gain is 2.5 dB.

Figure 13 Performance in CM2 at 200 Mb/s

Figure 13 is error performance at channel model 2 data rate of 200 Mb/s. Trend is similar with previous result. For example at BER 10-2, coding gain is larger than 2.5 dB.

Figure 14 Performance in CM3 at 200 Mb/s

Figure 14 is error performance at channel model 3 data rate of 200 Mb/s. Trend is similar with previous result. For example at BER 10-2, coding gain is 2.5 dB.

Figure15 Performance in CM4 at 200 Mb/s

Figure 15 is error performance at channel model 4 data rate of 200 Mb/s. Trend is similar with previous result. For example at BER 10-2, coding gain is approximately 2.5 dB.

Figure 16 Performance in CM1 at 100 Mb/s

Figure 16 is error performance at channel model 1 data rate of 110 Mb/s. Trend is similar with previous result. For example at BER 10-2, coding gain is approximately 2 dB.

Figure 17 Performance in CM2 at 100 Mb/s

Figure 17 is error performance at channel model 2 data rate of 110 Mb/s. Trend is similar with previous result. For example at BER 10-2, coding gain is approximately 2 dB.

Figure 18 Performance in CM3 at 100 Mb/s

Figure 18 is error performance at channel model 3 data rate of 110 Mb/s. Trend is similar with previous result. For example at BER 10-2, coding gain is 2 dB.

Figure 19 Performance in CM4 at 100 Mb/s

Figure 19 is error performance at channel model 4 data rate of 110 Mb/s. Trend is similar with previous result. For example at BER 10-2, coding gain is approximately 2 dB.

- Conclusion

In this proposal, we suggest the error correcting code for physical header. Physical header is quite important because it is impossible for the receiver to recover received frame when the physical header is failed. So the error correcting code is needed for physical header protection method. Among the error correcting code, we suggested the sub-code of 2nd order Reed-Muller code. The 2nd order Reed-Muller code is optimal code and easy to implement. The simulation shows that the coding gain of 2nd order Reed-Muller code is more than 2dB in almost conditions. Therefore, the sub-code of 2nd order Reed-Muller code brings a lot of performance improvement for the reliability of physical header.

- Part III

Title: The ACK Aided Adaptive Modulation and Coding

- Introduction

In this part, we propose one of the AMC (Adaptive Modulation and Coding) techniques that obtains channel state information by counting the frequency of the acknowledgement frame (hereinafter referred to as ACK). Wireless channels vary with time and other factors. For example, when a moving obstacle interrupts between a transmitter and a receiver, the channel attenuation becomes larger so the channel condition becomes severe. In case of instant channel degradation, adaptive transmission technique can be considered as a countermeasure. The main concept of adaptive transmission is to maintain a constant bit error rate against degraded channel condition by varying the transmitted power level, modulation scheme, coding scheme, coding rate and etc.

There are various adaptive transmission techniques from combination of the transmitter parameters. With well designed adaptive transmission technique, wireless communication system can be robust to varying channel condition. But there are some limitation in particular systems. By the UWB emission limits of the FCC (Federal Communications Commission), the UWB communication systems are restricted in emission power (i.e. transmit power). As a result, some adaptive transmission techniques described previously are not available in UWB system. Especially, the power control which varies the transmit power level is practically forbidden. So among various kind of adaptive transmission technique, the AMC technique is an influential candidate.

To apply AMC, the channel state information must be known to the transmitter. This means that the transmitter or the receiver needs additional hardware block for obtaining the channel state information and for converting modulation or coding scheme. But it is definite that simple hardware structure is recommended.

Consequently we suggest the ACK aided AMC for more efficient management of channel resource with simple hardware and minimum impact to IEEE 802.15.3 MAC structure.

- Criteria

The followings are main criteria to consider with introduction of adaptive transmission technique.

- minimum impact on conventional MAC structure

According to ‘the technical requirements’, it is required that the alt-PHY operate with the 802.15.3 MAC without fundamental change in operation. In detail, it is preferred that alt-PHY operates with minimum supplement and minimum modification to 802.15.3 MAC.

- throughput

The throughput represents the efficiency of channel utilization. In ARQ (Automatic Repeat Request) scheme, the more repeat request, the worse channel efficiency. So the throughput is important factor in point of channel resource management.

- hardware complexity

The adaptive transmission technique increases complexity of hardware. Namely, channel state monitoring device and converting modulation or coding scheme device are supplemented. Increment of hardware complexity causes high cost and more power consumption. So the system with simple hardware structure is desirable.

- Proposed scheme

In this proposal, we suggest the ACK aided AMC technique with simple hardware structure.

Changing the coding rate can compensate the channel degradation.

Figure 20 is frame error rate (hereinafter referred to as FER) performances with several coding rates. These 4 curves are in the order of coding rate. Code rate 1 is the highest coding rate. Code rate 4 is the lowest coding rate. It is clear that as coding rate becomes lower, the FER is reduced. In case of the channel degradation, FER is increased even though the bit energy is constant. In general, increase of bit energy makes it possible to maintain same FER in degraded channel. And the conversion of coding rate has same effect so the conversion of coding rate can be an alternative method. For example, at the 3 dB of Eb/No, FER of code rate 2 is approximate 0.006 but FER of code rate 4 is approximate 0.002. To be more particular, converting to higher coding rate has same effect of increase of bit energy at the cost of redundancy.

Figure 20 Frame error rate performances with various coding rates

The AMC technique requires the channel state information. The measurement of carrier to interference ratio (C/I) is a practical method of obtaining channel estimation value. However, the additional hardware block is essential to measurement of C/I and conventional MAC structure ought to be modified. So, we suggest that the transmitter estimate the channel state by measurement of ACK.

Figure 21 is the procedure of ACK aided AMC at immediately ACK. The transmitter counts the number of ACK then calculates the success probability (number of ACK/number of whole transmission) during fixed period i.e. update period.

Figure 91 The procedure of ACK aided AMC at immediately ACK

It is reasonable that the transmitter considers the success probability as channel state information. Because required packet error rate by the selection criteria is less than 8%.  The 8% of FER can be calculated relatively less trial. In proposed ACK aided AMC, If the success probability is in fixed probability range, the transmitter maintains current coding rate. In case of the low probability of success, the transmitter decides that the channel condition is severe then converts the coding rate lower for the reliable communication. In case of high success probability, the transmitter decides that the channel condition is favorable then converts the coding rate higher for efficiency of channel resource.

Figure 22 is the procedure of ACK aided AMC at delay ACK. The only difference is that in delay ACK, the transmitter decides the channel state by information of Dly-ACK frame.

Figure 22 The procedure of ACK aided AMC at delay ACK

Figure 23 is Dly-ACK frame format. The MPDUs ACKed field indicates the number of MPDUs that are being ACKed. Success probability calculation in delay ACK is processed by the information of MPDUs ACKed field.

Figure 23 Dly-ACK frame format of IEEE 802.15.3

Figure 24 is Frame structure of proposed system. It is exact same with IEEE 802.15.3 except supplement of the coding rate information which informs the coding rate to the receiver. As described before, it is strongly recommended that impacts on conventional MAC frame structure are as little as possible. In addition, MAC structure of the receiver remains identical with that of conventional.

Figure 24 Frame structure of proposed system

- Conclusion

In this proposal, we propose ACK aided AMC technique. Estimating the channel status by ACK information is one of the simple methods which have minimum impact to conventional system structure. Only adding the “coding rate information” field in the PHY header, we can achieve adaptive coding. As a result, we can expect the more throughput performance in IEEE 802.15.3a system.

- Conclusion of This Proposal

In this proposal we propose preamble structure, the Phy header protection scheme and ACK aided AMC.