Wireless Personal Area Network (WPAN) APPLICATION MATRIX

| APPLICATION | DATA RATE | RANGE | POWER | UNIT ASP | TOPOLOGY | COMMENTS |
| --- | --- | --- | --- | --- | --- | --- |
| CAN | 19.2-100K | 0-5M | 20mW 10/90% | $40 | P-P Bus | Courier Area Network |
| SRVC | .2-10Mb | 0-5M | 1W 20/80% | $500 | P-P Bus | Short range Video Comm. |
| PMF | 1-8Kb | 0-3M | 600mW 15/85% | $5 | P-P | Physiological Medical F. |
| Inv. Cntl | 1-19.2Kb | 0-15M | 20mW | $50 | P-P | Inventory Control |
| Co-Maint. | 1-250Kb | 0-15M | 250mW 20/80% | $500 | P-P/MP | Collaborative Maintenanc |
| Ent. Audio | 10Kb | 0-15M | 20mW 10/90% | $50 | P-P | Entertainment Audio |
| Target | 1-500Kb | 0-10M | 20mW 10/90% | $50-100 | P-P | Gateway Appl. Pot. |

Note: Target Frequency: 2400-2500MHz (US 2400-2483.5, EU 2400-2500, JP 2471-2497)

-----------------------------------------------------------

Source: IEEE Ad-Hoc WPAN Committee, Cleveland, 12/2/97