IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | TG1-Errata-Submitted-to-Bluetooth-SIG |  |
| Date Submitted | [The date the document is contributed, in the format “21 May, 1999”] |  |
| Source | [Tom Siep] [Texas Instruments] [12500 TI Blvd, m/s 8723] | Voice: [214-480-6786] Fax: [972-761-5581] E-mail: [siep@ti.com] |
| Re: | [Original document; processing resulting from 117r10.] |  |
| Abstract | [Comments from LB8 that generated inputs to the Bluetooth Errata system] |  |
| Purpose | [Documentation of disposition of Unsatisfied comments] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

| Control #: | 217 |
| --- | --- |
| Submitter: | Gilb, James |
| IEEE Clause: | 8.10.6.4.1 |
| Bluetooth Spec Section: | V1.1, Part B, page 104, paragraph 2 |
| Type | Editorial |
| Problem: | The nomenclature for the timing parameter here, pagerespTO differes from earlier timing, e.g. Tw page scan. Select one method (T_parameter is best) and keep it consistent throughout for all timing paramters (e.g. newconnectionTO). Link all of the usages of the word with cross references to where the numeric definition can be found. |
| Proposed Solution: | We agree that it is preferable to maintain a consistent nomenclature. We will submit an official Bluetooth erratum to call out this deficit. We do not believe that this problem will prevent the proper implementation of a system based on this Standard. |
| Submission to Bluetooth Errata System: |  |
| Requested by | Tom Siep |
| Company | IEEE |
| Requestor email | siep@ti.com |
| Version | 1.1 |
| Part | B |
| Page | 104 |
| Section | 10.6.4.1 |
| Paragraph | 2 |
| Subject heading | Consistent nomenclature for parameters |
| Problem | Editorial: The nomenclature for the timing parameter here, pagerespTO differes from earlier timing, e.g. Tw page scan. |
| Suggested Change | Select one method (T_parameter is preferred) and keep it consistent throughout for all timing paramters (e.g. newconnectionTO). Link all of the usages of the word with cross references to where the numeric definition can be found. |
| Errata system # | 2135 |

| Control #: | 279, 280, 281 |
| --- | --- |
| Submitter: | Cypher, David |
| IEEE Clause: | 9.3.19 |
| Page | 179 |
| Line | 35 |
| Type | technical |
| Problem: | The Labels for the sequences 44, 45, & 46 do not appear to be correct. Label 44 should be deleted and replaced with label from 45. Label 45 should be deleted and replaced with label 46. Label 46 should be deleted and replaced with "Master notifies slave of quality of service" |
| Proposed Solution: | Label 44 should be deleted and replaced with label from 45. |
| Submission to Bluetooth Errata System: |  |
| Requested by | Tom Siep |
| Company | IEEE |
| Requestor email | siep@ti.com |
| Version | 1.1 |
| Part | C |
| Page | 226, 227 |
| Section | 3.19, 3,20 |
| Paragraph | Sequences 44, 45, 46 |
| Subject heading | Sequence chart label problem |
| Problem | The Labels for the sequences 44, 45, & 46 do not appear to be correct. |
| Suggested Change | Label 44 should be deleted and replaced with label from 45. Label 45 should be deleted and replaced with label 46. Label 46 should be deleted and replaced with "Master notifies slave of quality of service" |
| Errata system # | 2137 |

| Control #: | 311 |
| --- | --- |
| Submitter: | Cypher, David |
| IEEE Clause: | G.0.0.2 |
| Page | 898 |
| Line | 1 through 40 |
| Type | technical |
| Problem: | Figure 0.6 and 0.7 are the same, yet they are supposed to represent different MSCs. Is an MSC missing or are these two operations truly the same? |
| Proposed Solution: | Correct as appropriate based on the answer to the question |
| Submission to Bluetooth Errata System: |  |
| Requested by | Tom Siep |
| Company | IEEE |
| Requestor email | siep@ti.com |
| Version | 1.1 |
| Part | IX |
| Page | 1046, 1047 |
| Section | 3.2.1, 3.2.2 |
| Paragraph | Figures 3.3, 3.4 |
| Subject heading | MSC chart duplication? |
| Problem | Figure 3.3 and 3.4 are the same, yet they are supposed to represent different MSCs. Is an MSC missing or are these two operations truly the same? |
| Suggested Change | Correct as appropriate based on the answer to the question |
| Errata system # | 2136 |

| Control #: | 331 |
| --- | --- |
| Submitter: | Cypher, David |
| IEEE Clause: | 8.10.7.4 |
| Page | 93-94 |
| Line | 36-54 |
| Type | Technical |
| Problem: | The new procedures either prohibit entering or leaving Inquiry Response. After RAND slots return to inquiry scan on first inquiry message go to inquiry response and return FHS. At this point device is in inquiry response. Now what triggers the device out of this state? |
| Proposed Solution: | The new procedures either prohibit entering or leaving Inquiry Response. After RAND slots return to inquiry scan on first inquiry message go to inquiry response and return FHS. At this point device is in inquiry response. Now what triggers the device out of this state? |
| Submission to Bluetooth Errata System: |  |
| Requested by | Tom Siep |
| Company | IEEE |
| Requestor email | siep@ti.com |
| Version | 1.1 |
| Part | B |
| Page | 109 |
| Section | 10.7.4 |
| Paragraph | 1 |
| Subject heading | State change problem |
| Problem | The new procedures define two possible paths for leaving Inquiry Response, with no inputs for making a determination which one to take. RAND slots return to inquiry scan on first inquiry message go to inquiry response and return FHS. At this point device is again in inquiry response. |
| Suggested Change | It now appears that all actions are done in the inquiry scan substate and there is no need for the inquiry response state, so delete it and it related description. |
| Errata system # | 2138 |

| Control #: |  |
| --- | --- |
| Submitter: |  |
| IEEE Clause: |  |
| Page |  |
| Line |  |
| Type | Editorial Technical |
| Problem: | What does this statement mean? It is in conflict with the statement in section 8.10.8.5.1 on page 102 line 20. I have asked this on LB#3 and the reponse from BSIG is that to remove what is confusing to me would only confuse others. So please some one, any one, tell me what it means. |
| Proposed Solution: | Re-submit Bluetooth Errata 1253, which was rejected, text is still unclear. Is the SCO slave always allowed to transmit in it's SCO, slot? We believe that the Slave is always allowed to send it's data in it's reserved slot. However, the current V1.1 text indicates a possible exception for slave transmission in it's reserved slot. To remove this ambiguity, delete "unless a different slave was aaddressed int the previous master-to-slave slot". or provide the conditions under which the exception will occur. Ref. Bluetooth Core v1.1 Part B Sec 3.2, 2nd Paragraph |
| Submission to Bluetooth Errata System: |  |
| Requested by | Tom Siep |
| Company | IEEE |
| Requestor email | siep@ti.com |
| Version | 1.1 |
| Part | B |
| Page | 45 |
| Section | 3.2 |
| Paragraph | 2 |
| Subject heading | SCO Transmissions |
| Problem | Text is unclear. Is the SCO slave always allowed to transmit in its SCO, slot? We believe that the Slave is always allowed to send data in its reserved slot. However, the current V1.1 text indicates a possible exception for slave transmission in it's reserved slot. |
| Suggested Change | To remove this ambiguity, delete "unless a different slave was addressed in the previous master-to-slave slot" or provide the conditions under which the exception will occur. |
| Errata system # | 2139 |

| Control #: | 353 |
| --- | --- |
| Submitter: | Cypher, David |
| IEEE Clause: | C.5.1.1.1 |
| Page | 843 |
| Line | 50 |
| Type | Technical |
| Problem: | INQUIRY_SCAN is the state that may be periodically entered if a device wants to be available to repond to inquirys. |
| Proposed Solution: | Change INQUIRY_RESPONSE to INQUIRY_SCAN |
| Submission to Bluetooth Errata System: |  |
| Requested by | Tom Siep |
| Company | IEEE |
| Requestor email | siep@ti.com |
| Version | 1.1 |
| Part | K:1 GAP |
| Page | 30 |
| Section | 4.1.1.1 |
| Paragraph | 1 |
| Subject heading | State of Non-discoverable mode |
| Problem | INQUIRY_SCAN is the state that may be periodically entered if a device wants to be available to repond to inquirys. |
| Suggested Change | Change INQUIRY_RESPONSE to INQUIRY_SCAN |
| Errata system # | 2140 |

| Control #: | 355 |
| --- | --- |
| Submitter: | Cypher, David |
| IEEE Clause: | G.8.2 |
| Page | 920 |
| Line | 7 through 46 |
| Type | Technical |
| Problem: | The Figure shows that after invoking HCI_Write_loopback_Mode creating ACL and SCO connections are done. Yet reading the text of HCI the ACL and SCO must be already established in order for the data paths to be remotely looped back. |
| Proposed Solution: | Figure needs to move these blocks before and after the remote loopback MSC |
| Submission to Bluetooth Errata System: |  |
| Requested by | Tom Siep |
| Company | IEEE |
| Requestor email | siep@ti.com |
| Version | 1.1 |
| Part | IX, MSCs |
| Page | 1071 |
| Section | 8.1 |
| Paragraph | Figure 8.1 |
| Subject heading | MSC Inconsistent with HCI text |
| Problem | The Figure shows that after invoking HCI_Write_loopback_Mode creating ACL and SCO connections are done. Yet reading the text of HCI the ACL and SCO must be already established in order for the data paths to be remotely looped back. |
| Suggested Change | Figure needs to move these blocks before and after the remote loopback MSC |
| Errata system # | 2141 |

| Control #: | 318 |
| --- | --- |
| Submitter: | Cypher, David |
| IEEE Clause: | 11.2.7.14 |
| Page | 143 |
| Line | 52 |
| Type | Editorial Tecnical |
| Problem: | Current hex coding does not equal 5 seconds. |
| Proposed Solution: | Change value 0x1FA0 to 0x1F40 to make 5 seconds. |
| Submission to Bluetooth Errata System: |  |
| Requested by | Tom Siep |
| Company | IEEE |
| Requestor email | siep@ti.com |
| Version | 1.1 |
| Part | H:1 |
| Page | 646 |
| Section | 4.7.14 |
| Paragraph | Command Parameters; Parameter Description |
| Subject heading | Math Error |
| Problem | Current hex coding does not equal 5 seconds. |
| Suggested Change | Change value 0x1FA0 to 0x1F40 to make 5 seconds. Assume that it is a typo and that either the “A” should be a “4” OR 5 seconds is really 5.06 seconds |
| Errata system # | 2142 |

| Control #: | 319 |
| --- | --- |
| Submitter: | Cypher, David |
| IEEE Clause: | 11.2.7.21 |
| Page | 151 |
| Line | 14&16 |
| Type | Technical |
| Problem: | The range value 0x0012 does not agree with Range value 0.625 msec. |
| Proposed Solution: | Either change 0x0012 to 0x0001 or change 0.625 to 11.25. |
| Submission to Bluetooth Errata System: |  |
| Requested by | Tom Siep |
| Company | IEEE |
| Requestor email | siep@ti.com |
| Version | 1.1 |
| Part | H:1 |
| Page | 651 |
| Section | 4.7.19 |
| Paragraph | Page_Scan_Interval parameter description |
| Subject heading | Math Error |
| Problem | The range value 0x0012 does not agree with Range value 0.625 msec. |
| Suggested Change | Either change 0x0012 to 0x0001 or change 0.625 to 11.25. |
| Errata system # | 2143 |

| Control #: | 209 |
| --- | --- |
| Submitter: | Gilb, James |
| IEEE Clause: | 8.10.6.2 |
| Page | 85 |
| Line | 39 |
| Type | Editorial |
| Problem: | There is an inconsistent use of all-caps for system states. The state of page scan, page, etc. are lower cased while STANDBY and CONNECTION are upper cased. |
| Proposed Solution: | Change all state indications to either lower case or upper case. |
| Submission to Bluetooth Errata System: |  |
| Requested by | Tom Siep |
| Company | IEEE |
| Requestor email | siep@ti.com |
| Version | 1.1 |
| Part | B |
| Page | 97 |
| Section | 10.6.2 |
| Paragraph | 1 |
| Subject heading | Consistent case for States (editorial) |
| Problem | There is an inconsistent use of all-caps for system states. The state of page scan, page, etc. are lower cased while STANDBY and CONNECTION are upper cased. |
| Suggested Change | Change all state indications to either lower case or upper case. |
| Errata system # | 2144 |