Dr. Robert F. Heile

Verizon Technology Organization

Chair IEEE 802.15

40 Sylvan Road

Waltham,  MA  02451 USA

TEL: +1 781 466 2057

FAX: +1 781 466 2575

E-M: bheile@bbn.com

Pager: 800-759-8888  PIN 1109355

Mr. David Cypher

NIST, M/S 8920

100 Bureau Drive

Gaithersburg, MD 20899-8920  USA

Saturday, November 11, 2000

Subject: IEEE P802.15.1 Letter Ballot #3 Comment Resolution Disposition

Dear Mr. Cypher,

Thank you for participating in the Letter Ballot #3 that was held from 24May00 to 3Jul00.  As you learned this Letter Balloted Motion failed with 18/15/0 (P802-15/D0.7.2):

- There were 49 Voting members. 33 submitted their vote.

- The return ratio is 33/49 = 67 % (50 % is required) and the abstention rate was less than 30% of those voting.  The ballot is valid.   16 failed to vote.

- Motion 1 failed with 18/15/0 or 55 %.

In reviewing your comments we have decided to decline the following based on:

| DEC022 |  |  |  | 8.4.6.2 |  |  |  |  |  | e |  |  |  |  | N |  |  |  |  | sentence split unexpectedly |  |  |  |  |  | Undo split |  | D |  |  |  | Style convention for labeling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DEC024 |  |  |  |  | 8.10.7.4 |  | T |  |  |  |  |  |  | Y |  |  | Figure 47 is missing links. |  |  |  |  |  |  |  | Add Link from CONNECTION to inquiry response; from inquiry response to Page Scan; and from STANDBY to inquiry response. |  |  |  |  | F |  | Location of problem: State diagram of BT baseband controller. On page 103 REJECTED: These links doesn't add anything from an implementation perspective and doesn't bring any clarity to the state diagram |
| DEC021 |  | 8.3.2 |  |  |  |  |  | T |  |  |  | Y |  |  |  |  |  | Text is unclear about the slave being allowed to transmit SCO data. |  |  |  |  |  |  | Delete "unless a different slave was addressed in the previous master-to-slave slot" |  |  |  |  | F |  | REJECTED: This errata request is simply not correct and will just lead to possible misunderstanding. |
| DEC097 |  |  | C.2.5 |  |  |  |  |  | t |  |  |  | Y |  |  |  |  |  | This profile has nothing to do with co-existence |  |  |  |  |  | Delete "and co-existence" |  |  |  |  | F |  | REJECTED: This profile handles co-existence between different Bluetooth devices so that they can operate in the vicinity of each other although they will never have any reason to be connected. Although baseband is designed to handle this, some high-level requirements on use of the baseband and link policies need to be provided to ensure that applications can execute between some devices unharmed from radio emissions and connection attempts occuring from other device in the vicinity. |
| DEC101 | C.5 |  |  |  |  | T |  |  |  |  |  |  |  |  |  | Y |  |  |  |  | The statement made in C1 is a violation of profiling. If a feature is mandatory at a lower layer a profile cannot undo that requirement. |  | Change item 1 to M for mandatory |  |  |  | F |  |  |  | It is suggested that the note for C1 be removed. Also what's the real purpose of item 1? Although LMP-authentication and LMP-pairing are mandatory, there are several ways they can be combined. GAP section 5.1 defines one way to combine them, but this way is only required in case a device implements a higher security level than defined by security mode 1. Although a device has implemented authentication according to LMP, it is not required that it has an operational mode where it ever initiates authentication. Proposal: No action on C1. In case a device do want to implement a security level higher than provided by security mode 1, it has to implement security mode 2 or security mode 3(or both). This is expressed by C2. Proposal: No action on C2. |  |
| DEC102 | C.5 |  |  |  |  | T |  |  |  |  | Y |  |  |  |  |  |  |  |  |  |  | Item 2 lists three possible security modes, but places C2 on two of them relating to the first. If this is the real association then the question must not be a list, but separate items. |  | No specific suggestion is made due to the confusion of the current specification. |  |  |  |  | F |  |  | What is the real meaning of "C2"? References for security modes 1, 2, and 3 are missing. See also comment on report nr 1271 for use of C1 and C2. Could maybe be clarified, but not necessarily needed. It is common in the conformance tables to refer to the sections where the features are defined. In table 5.1, it is seen that the security modes are defined in sub-section 5.2. Proposal: No action needed. |

The IEEE 802.15 Working Group for WPANs™ appreciates your interest and we look forward to your participation in the next Letter Ballot.  For future information on LB3 status please point your browser here: http://ieee802.org/15/pub/LB3/LB3.html

Thank you for your assistance in this matter.

Sincerely,

Bob Heile, Chair 802.15

cc: Ian Gifford, Chatschik Bisdikian, Tom Siep, Pat Kinney, WG File