| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed response to the public consultation on draft amended ECC decision (06)12 |  |
| Date Submitted | 2007-07-14 |  |
| Source(s) | Mike Hart UK Broadband Ltd. 6 The Sqaure, Stockley Park East Uxbridge, UB11 1FW | Voice: +44 20 3178 5835 E-mail: mike.hart@ukbroadband.com |
| Re: |  |  |
| Abstract | This contribution contains a proposed response for the Liaison Adhoc to consider to the aforementioned consultation. |  |
| Purpose | Input for Liaison Adhoc to discuss and approve. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed response to the public consultation on draft amended ECC decision (06)12

Mike Hart

UK Broadband Ltd.

# Introduction

The is contribution contains a proposed response to the ECC consultation on the draft amended ECC decision (06)12 relating to the operation of UWB devices with DAA in the 3.4 to 4.2GHz band in Europe.

# Proposed Response Letter

IEEE 802.18

DRAFT

Radio Regulatory Technical Advisory Group Homepage at: http://ieee802.org/18/

Mr. Fatih Mehmet Yurdal ERO

Nansensgade 19-3

DK-1366 Copenhagen Denmark

yurdal@ero.dk

Subject: Public Consultation on Draft amended ECC/DEC/(06)12, “Complementary regulatory provisions for devices using Ultra-Wideband (UWB) technology”

Dear Mr. Yurdal,

IEEE 802.18, the Radio Regulatory Technical Advisory Group (“the RR-TAG”) within IEEE 802 hereby submits comments to this consultation. This document was prepared by the IEEE 802.16TM Working Group on Wireless Metropolitan Area Networks (http://ieee802.org/16/), reviewed and approved by the RR-TAG, and also was reviewed by the IEEE 802 Executive Committee.

IEEE 802.16 develops standards for mobile and fixed broadband wireless access systems that operate in various frequency bands, including those below 9 GHz. We have reviewed the proposed amendments to ECC Decision (06)12 and ECC report 120 paying particular attention to the protection that “Detect And Avoid” (DAA) affords to both mobile and fixed BWA terminals in the 3.4-4.2 GHz band.

As previously indicated in our comments on the consultation on draft ECC report 120, which appear in the main to not have been addressed in the formation of the draft amendment to the decision, we reiterate that we have some concerns that DAA may not be able to practically provide the required level of protection to mobile BWA systems.

The main reasons for our concerns are that in the development of DAA it was not considered that:

Network deployments that are based on mobile BWA using OFDMA operate with lower receive power levels than fixed BWA deployments.  In the development of DAA, and in particular the specification of the thresholds that feature in Table 1 of the draft amended decision, it appears that the performance of realistic mobile BWA deployments were not considered. In particular, the sensitivity levels used to derive the thresholds are taken from a non-public conformance (not performance) test document that is referenced in the report.  Consequently, it is not possible for us to review this document to provide an accurate statement. However, we are concerned that the performance degradation caused by UWB devices with DAA is likely to be more severe in mobile BWA systems, unless the thresholds are calculated considering typical receive sensitivities of mobile devices.

Mobile stations (MSs) will support power saving features such as idle and sleep modes, and/or multicast and broadcast services (MBS). Such stations cannot be presumed to provide a continuous or periodic uplink transmission. The current definition of DAA will not protect devices in these modes or using these services because, based on the parameters in Table 1 of the draft amended decision, after 5.1s the UWB will assume it can occupy the channel. This will impact the ability of the devices to exit power saving modes and could degrade multicast and broadcast services, particularly for users close to the limit of the coverage of the BWA system. To provide sufficient protection to such devices and services DAA needs to include downlink detection mechanisms with appropriate detection thresholds for these services.

Unlike a fixed BWA terminal, an MS typically transmits using a large dynamic range  based on uplink power control, and the MS is not always required to support high order modulation in the uplink. Mobile BWA devices are required to support 45dB, whereas only 3.1dB was considered in ECC report 120. Furthermore, a base station (BS) may reduce the effective transmit power delivered to an MS close to the BS in order to boost the power delivered to a MS at the cell edge. Due to these points not being considered in the specification of the signal thresholds for each zone that appear in Table 1 of the draft amended decision, the current definition of DAA could result in an effective error in the distance calculation larger than accounted for in ECC Report 120. Therefore it is likely that DAA will not provide sufficient protection to these types of mobile devices.

Consequently, it is likely that by adopting the thresholds in Table 1 and only accounting for uplink detection, UWB with DAA will not provide sufficient protection to BWA services. The IEEE 802.16 TM WG are willing to assist the ECC in the revision of the DAA protocol, in particular the thresholds and methods for providing protection to MBS and devices utilizing power saving features to ensure the concerns outlined in this response are addressed. The IEEE 802.16™ WG are prepared to provide specific proposals for amendments to the protocol upon request.

In response to other aspects of the draft amended decision, we were pleased to note in ECC report 120 that DAA is intended to provide protection for more than 99.75% of the time. We suggest that this is reflected in the decision by amending decision 2b as indicated: “DAA UWB devices are defined as devices using UWB technology that meet the technical requirements for Detect And Avoid (DAA) mitigation technique given in Annex 2 and provide protection to BWA TSs for more than 99.75% of the time;”

We are pleased to note consideration i) in the draft amended ECC decision “that technical parameters of DAA will need to be reviewed taking into account future technological development” however this does not seem to be reflected in the decisions made. We request that this is addressed by accommodating for a defined review cycle of at least every 18 months to account for a realistic rate of technology evolution by adding a new decision: “that technical parameters of DAA will be reviewed taking into account future technological development on at least an 18 montly basis”.

As requested in our response to the consultation on draft ECC report 120, we would appreciate being advised of any future activities in this area, and we repeat this request in this response.

In the interim, we request that the ECC removes or delays changes to the parts of the decision that relate to the 3.4 – 4.2GHz band until an assessment of the above points has been conducted and accounted for through revision of the DAA protocol.

Regards,

Michael Lynch /s/

Michael Lynch

Chair, IEEE 802.18 Radio Regulatory Technical Advisory Group IEEE-SA Technical Liaison to ITU-R

cc:	Terry deCourcelle, IEEE-SA
Paul Nikolich, Chair, IEEE 802

The IEEE Local and Metropolitan Area Networks Standards Committee (“IEEE 802” or the “LMSC”). This document represents the views of IEEE 802.16 and 802.18. It does not necessarily represent the views of the IEEE as a whole or the IEEE Standards Association as a whole.