| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | IEEE 802.16m Evaluation Results for LBT for IMT-Advanced |  |
| Date Submitted | 2009-09-24 |  |
| Source(s) | Wookbong Lee, Sunam Kim, Sungho Park, Seunghyun Kang, DongCheol Kim, Jinsam Kwak LG Electronics Apostolos Papathanassiou, Roshni Srinivasan, Shailender B Timiri, Alexei Davydov, Takashi Shono Intel Corporation Jeongho Park, Kichun Cho, Jaeweon Cho, Hokyu Choi Samsung Electroincs Kwangjae Lim, Wooram Shin ETRI Mitsuo Nohara, Satoshi Imata KDDI R&D Laboratories Kenji Saito UQ Communications Michiharu Nakamura, Zhou Hua, Yan Hongzhong Fujitsu I-Kang Fu, Kelvin Chou, Yu-Hao Chang, Pei-Kai Liao, Chih-Yuan Lin, Yih-Shen Chen, Paul Cheng MediaTek Inc. Mark Cudak, Bill Hillery, Anup Talukdar, Eugene Visostsky, Fred Vook Motorola, Inc. | wbong@lge.com apostolos.papathanassiou@intel.com jeongho.jh.park@samsung.com kjlim@etri.re.kr sa-imata@kddilabs.jp kenji@uqc.jp zhouhua@cn.fujitsu.com ik.fu@mediatek.com bill.hillery@motorola.com |
| Re: |  |  |
| Abstract | This contribution provides an input to the link budget template (LBT) for IMT-Advanced submission |  |
| Purpose | To assist preparation of the link budget templates for IMT-Advanced submission |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

IEEE 802.16m Evaluation Results for LBT for IMT-Advanced

Wookbong Lee, Sunam Kim, Sungho Park, Seunghyun Kang, DongCheol Kim, Jinsam Kwak
LG Electronics

Apostolos Papathanassiou, Roshni Srinivasan, Shailender B Timiri, Alexei Davydov, Takashi Shono

Intel Corporation

Jeongho Park, Kichun Cho, Jaeweon Cho, Hokyu Choi

Samsung Electroincs

Kwangjae Lim, Wooram Shin

ETRI

Mitsuo Nohara, Satoshi Imata

KDDI R&D Laboratories

Kenji Saito

UQ Communications

Michiharu Nakamura, Zhou Hua, Yan Hongzhong

Fujitsu

I-Kang Fu, Kelvin Chou, Yu-Hao Chang, Pei-Kai Liao, Chih-Yuan Lin, Yih-Shen Chen, Paul Cheng

MediaTek Inc.

Mark Cudak, Bill Hillery, Anup Talukdar, Eugene Visostsky, Fred Vook

Motorola, Inc.

- Introduction

This contribution provides IEEE 802.16m evaluation results generated for the purpose of filling the Link Budget Template (LBT) of IMT-Advanced. All evaluation results were generated by following the guidelines in [1] and the System Evaluation Details of IEEE 802.16m for IMT-Advanced according to [2].

The contribution is organized as follows: In Section 2, we describe the main assumptions. In Section , we provide the link budget tables for all mandatory test environments: TDD indoor hotspot in Table 2, FDD indoor hotspot in Table 3, TDD urban micro-cell in Table 4, FDD urban micro-cell in Table 5, TDD urban macro-cell in Table 6, FDD urban macro-cell in Table 7, TDD rural macro-cell in Table 8 and FDD rural macro-cell in Table 9.

In the tables of Sections 2 to 4, the different sources of the evaluation results correspond to contributors from the different affiliations according to the following mapping:

| Source 1 | Intel Corporation |
| --- | --- |
| Source 2 | Clearwire |
| Source 3 | ETRI |
| Source 4 | Fujitsu Laboratories Ltd. |
| Source 5 | KDDI R&D Laboratories |
| Source 6 | LG Electronics |
| Source 7 | MediaTek Inc. |
| Source 8 | Motorola Inc. |
| Source 9 | Samsung Electronics |
| Source 10 | Alcatel-Lucent Shanghai Bell |
| Source 11 | UQ Communications |

- Main assumptions for the LBT

The guidelines of [3] were followed in the development of the link budget tables presented in Section 4. In the following, the main assumptions used for generating the LBT are:

- Number of transmit/receive antennas

  - ABS: 4 transmit antennas and 4 receive antennas

  - AMS: 1/2 transmit antennas and 2 receive antennas

- Target packet error rate

  - Data channel: 10% (for initial transmission)

  - Control channel: 1%

- Control channel

  - DL: A-A-MAP with QPSK 1/8 MCS

  - UL: 6-bit P-FBCH

- Data channel

  - Modulation: QPSK

  - Channel coding: Convolutional Turbo Coding with the number of resource units (RUs),  ISizeOffset, and data rates as shown in the following two tables:

| InH | Coderate | DL Burst BW (Number of RUs) | ISizeOffset | DL Burst size (bits) | DL MPDU Payload (bits) | DL MAC kbps (at initial transmission) | DL MAC kbps (at 4th retransmission) | DL MAC SE bps/Hz (at initial transmission) | DL MAC SE bps/Hz (at 4th retransmission) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL TDD | 0.61699 | 24ⅹ5 (120) | 17 | 15104 | 15024 | 3004.80 | 2528.824 | 1.017 | 0.856 |
| DL FDD | 0.64706 | 24ⅹ4 (96) | 17 | 12672 | 12592 | 5036.80 | 4238.945 | 1.066 | 0.897 |
| UL TDD | 0.60131 | 16ⅹ3 (48) | 17 | 5888 | 5824 | 1164.80 | 980.290 | 0.986 | 0.830 |
| UL FDD | 0.57843 | 16ⅹ4 (64) | 17 | 7552 | 7488 | 2995.20 | 2520.745 | 0.951 | 0.800 |

| UMi/UMa/RMa | Coderate | DL Burst BW (Number of RUs) | ISizeOffset | DL Burst size (bits) | DL MPDU Payload (bits) | DL MAC kbps (at initial transmission) | DL MAC kbps (at 4th retransmission) | DL MAC SE bps/Hz (at initial transmission) | DL MAC SE bps/Hz (at 4th retransmission) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL TDD | 0.46275 | 8ⅹ5 (40) | 15 | 3776 | 3728 | 745.60 | 627.493 | 0.757 | 0.637 |
| DL FDD | 0.45098 | 8ⅹ4 (32) | 15 | 2944 | 2896 | 1158.40 | 974.903 | 0.735 | 0.619 |
| UL TDD | 0.53595 | 4ⅹ3 (12) | 16 | 1312 | 1264 | 252.80 | 212.755 | 0.856 | 0.720 |
| UL FDD | 0.57108 | 4ⅹ4 (16) | 16 | 1864 | 1816 | 726.40 | 611.334 | 0.922 | 0.776 |

- Subchannelization (permutations) and MIMO transmission scheme (wherever applicable)

  - DL data: M-CRU and rank-1 wideband beamforming

  - DL control: DRU and SFBC with non-adaptive precoding

  - UL data: M-CRU and rank-1 wideband beamforming

  - UL control: P-FBCH structure with single transmit antenna

- Pilot boosting: 2 dB pilot boosting over data tone for DL and no pilot boosting for UL

- Transmitter array gain is included in the required SNR

- No HARQ is assumed for the control channel

- 0.5 dB HARQ combining gain is assumed for the data channel

- Shadowing fade margin: The shadowing fade margin is determined as a function of the cell-edge coverage reliability and the standard deviation of the log-normal shadow fading (including penetration losses if applicable). The cell-edge coverage reliability is determined for the given area coverage reliability as a function of the shadow fading standard deviation and the pathloss exponent obtained from the pathloss model. The cell-edge reliability can be determined using simulations or using traditional numerical methods. Table 1 contains the shadowing fade margin information required for the link budget tables of Section .

Table 1 Shadowing fade margin information

|  | InH NLoS | UMi NLoS | UMa NLoS, In-car | RMa NLoS, In-car |
| --- | --- | --- | --- | --- |
| Shadow fading standard deviation | 4 dB | 4 dB | 7.8 dB | 9.4 dB |
| Pathloss exponent | 4.33 | 3.67 | 3.91 | 3.86 |
| Shadowing fade margin | 2.8 dB | 3.1 dB | 8.1 dB | 10.4 dB |

- Link level simulation results

In the following embedded spreadsheet, the derivation of the required SNR values used in the link budget tables of Section 4 is shown based on the evaluation results from the different sources.

- Link budget tables

In this section, the link budget tables for all mandatory test environments (InH, UMi, UMa, RMa), links (downlink, uplink), and duplex modes (TDD, FDD) of IEEE 802.16m are presented from Table 2 to Table 9. The NLoS channel models were considered for each test environment since those models constitute the limiting factor for coverage in the IMT-Advanced evaluation methodology, i.e., the coverage with the LoS channel models for each test environment is better than the one presented for the NLoS channel models in this section.

Table 2 Link budget template for InH, TDD

| Item | Downlink | Uplink |
| --- | --- | --- |
| System configuration |  |  |
| Carrier frequency (GHz) | 3.4 | 3.4 |
| BS antenna heights (m) | 6 | 6 |
| UT antenna heights (m) | 1.5 | 1.5 |
| Cell area reliability(1) (%) (Please specify how it is calculated.) | 95% | 95% |
| Transmission bit rate for control channel (bit/s) | 89,600 | 1,200 |
| Transmission bit rate for data channel (bit/s) | 20,230,593 | 980,290 |
| Target packet error rate for the required SNR in item (19a) for control channel | 10-2 | 10-2 |
| Target packet error rate for the required SNR in item (19b) for data channel | 10-1 | 10-1 |
| Spectral efficiency(2) (bit/s/Hz) for data | 0.856 | 0.830 |
| Pathloss model(3) (select from LoS or NLoS) | NLoS | NLoS |
| Mobile speed (km/h) | 3 | 3 |
| Feeder loss (dB) | 2 | 2 |
| Transmitter |  |  |
| (1) Number of transmit antennas. (The number shall be within the indicated range in Table 6 of Report ITU-R M.2135) | 4 | 2 |
| (2) Maximal transmit power per antenna (dBm) | 18 | 18 |
| (3) Total transmit power = function of (1) and (2) (dBm) (The value shall not exceed the indicated value in Table 6 of Report ITU-R M.2135) | 24 | 21 |
| (4) Transmitter antenna gain (dBi) | 0 | 0 |
| (5) Transmitter array gain (depends on transmitter array configurations and technologies such as adaptive beam forming, CDD (cyclic delay diversity), etc.) (dB) | 0 | 0 |
| (6) Control channel power boosting gain (dB) | 0 | 0 |
| (7) Data channel power loss due to pilot/control boosting (dB) | 0.2734 | 0 |
| (8) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for downlink) | 3 | 1 |
| (9a) Control channel EIRP = (3) + (4) + (5) + (6) – (8) dBm | 21 | 20 |
| (9b) Data channel EIRP = (3) + (4) + (5) – (7) – (8) dBm | 20.73 | 20 |
| Receiver |  |  |
| (10) Number of receive antennas (The number shall be within the indicated range in Table 6 of Report ITU-R M.2135) | 2 | 4 |
| (11) Receiver antenna gain (dBi) | 0 | 0 |
| (12) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for uplink) | 1 | 3 |
| (13) Receiver noise figure (dB) | 7 | 5 |
| (14) Thermal noise density (dBm/Hz) | –174 | –174 |
| (15) Receiver interference density (dBm/Hz) | –174 | –174 |
| (16) Total noise plus interference density = 10 log (10^(((13) + (14))/10) + 10^((15)/10)) dBm/Hz | -166.21 | -167.81 |
| (17) Occupied channel bandwidth (for meeting the requirements of the traffic type) (Hz) | 37808640 | 3150720 |
| (18) Effective noise power = (16) + 10 log((17)) dBm | -90.43 | -102.82 |
| (19a) Required SNR for the control channel (dB) | -0.56 | -2.48 |
| (19b) Required SNR for the data channel (dB) | 1.41 | -0.24 |
| (20) Receiver implementation margin (dB) | 2 | 2 |
| (21a) H-ARQ gain for control channel (dB) | 0 | 0 |
| (21b) H-ARQ gain for data channel (dB) | 0.5 | 0.5 |
| (22a) Receiver sensitivity for control channel = (18) + (19a) + (20) – (21a) dBm | -89.00 | -103.30 |
| (22b) Receiver sensitivity for data channel = (18) + (19b) + (20) – (21b) dBm | -87.52 | -101.57 |
| (23a) Hardware link budget for control channel = (9a) + (11) − (22a) dB | 110.00 | 123.30 |
| (23b) Hardware link budget for data channel = (9b) + (11) − (22b) dB | 108.25 | 121.57 |
| Calculation of available pathloss |  |  |
| (24) Lognormal shadow fading std deviation (dB) | 4 | 4 |
| (25) Shadow fading margin (function of the cell area reliability and (24)) (dB) | 2.8 | 2.8 |
| (26) BS selection/macro-diversity gain (dB) | 0 | 0 |
| (27) Penetration margin (dB) | 0 | 0 |
| (28) Other gains (dB) (if any please specify) | 0 | 0 |
| (29a) Available path loss for control channel = (23a) – (25) + (26) – (27) + (28) – (12) dB | 106.20 | 117.50 |
| (29b) Available path loss for data channel = (23b) – (25) + (26) – (27) + (28) – (12) dB | 104.45 | 115.77 |
| Range/coverage efficiency calculation |  |  |
| (30a) Maximum range for control channel (based on (29a) and according to the system configuration section of the link budget) (m) | 87.39 | 159.42(4) |
| (30b) Maximum range for data channel (based on (29b) and according to the system configuration section of the link budget) (m) | 79.65 | 145.37 |
| (31a) Coverage Area for control channel = (π (30a)2) (m2/site) | 23993.85 | 79846.00 |
| (31b) Coverage Area for data channel = (π (30b)2) (m2/site) | 19929.95 | 66391.76 |
| (1) Cell area reliability is defined as the percentage of the cell area over which coverage can be guaranteed. It is obtained from the cell edge reliability, shadow fading standard deviation and the path loss exponent. The latter two values are used to calculate a fade margin. Macro diversity gain may be considered explicitly and improve the system margin or implicitly by reducing the fade margin. (2) The spectral efficiency of the chosen modulation scheme. (3) The pathloss models are summarized in TABLE A1-2 of Report ITU-R M.2135. (4) InH NLoS pathloss model is only defined for less than 150m. Even though the maximum range for uplink control channel exceeds 150m, same pathloss model is used. |  |  |

Table 3 Link budget template for InH, FDD

| Item | Downlink | Uplink |
| --- | --- | --- |
| System configuration |  |  |
| Carrier frequency (GHz) | 3.4 | 3.4 |
| BS antenna heights (m) | 6 | 6 |
| UT antenna heights (m) | 1.5 | 1.5 |
| Cell area reliability(1) (%) (Please specify how it is calculated.) | 95% | 95% |
| Transmission bit rate for control channel (bit/s) | 89,600 | 1,200 |
| Transmission bit rate for data channel (bit/s) | 16,955,780 | 2,520,745 |
| Target packet error rate for the required SNR in item (19a) for control channel | 10-2 | 10-2 |
| Target packet error rate for the required SNR in item (19b) for data channel | 10-1 | 10-1 |
| Spectral efficiency(2) (bit/s/Hz) for data | 0.897 | 0.800 |
| Pathloss model(3) (select from LoS or NLoS) | NLoS | NLoS |
| Mobile speed (km/h) | 3 | 3 |
| Feeder loss (dB) | 2 | 2 |
| Transmitter |  |  |
| (1) Number of transmit antennas. (The number shall be within the indicated range in Table 6 of Report ITU-R M.2135) | 4 | 2 |
| (2) Maximal transmit power per antenna (dBm) | 15 | 18 |
| (3) Total transmit power = function of (1) and (2) (dBm) (The value shall not exceed the indicated value in Table 6 of Report ITU-R M.2135) | 21 | 21 |
| (4) Transmitter antenna gain (dBi) | 0 | 0 |
| (5) Transmitter array gain (depends on transmitter array configurations and technologies such as adaptive beam forming, CDD (cyclic delay diversity), etc.) (dB) | 0 | 0 |
| (6) Control channel power boosting gain (dB) | 0 | 0 |
| (7) Data channel power loss due to pilot/control boosting (dB) | 0.2734 | 0 |
| (8) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for downlink) | 3 | 1 |
| (9a) Control channel EIRP = (3) + (4) + (5) + (6) – (8) dBm | 18 | 20 |
| (9b) Data channel EIRP = (3) + (4) + (5) – (7) – (8) dBm | 17.73 | 20 |
| Receiver |  |  |
| (10) Number of receive antennas (The number shall be within the indicated range in Table 6 of Report ITU-R M.2135) | 2 | 4 |
| (11) Receiver antenna gain (dBi) | 0 | 0 |
| (12) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for uplink) | 1 | 3 |
| (13) Receiver noise figure (dB) | 7 | 5 |
| (14) Thermal noise density (dBm/Hz) | –174 | –174 |
| (15) Receiver interference density (dBm/Hz) | –174 | -174 |
| (16) Total noise plus interference density = 10 log (10^(((13) + (14))/10) + 10^((15)/10)) dBm/Hz | -166.21 | -167.81 |
| (17) Occupied channel bandwidth (for meeting the requirements of the traffic type) (Hz) | 18904320 | 3150720 |
| (18) Effective noise power = (16) + 10 log((17)) dBm | -93.44 | -102.82 |
| (19a) Required SNR for the control channel (dB) | -0.56 | -2.48 |
| (19b) Required SNR for the data channel (dB) | 1.23 | -0.50 |
| (20) Receiver implementation margin (dB) | 2 | 2 |
| (21a) H-ARQ gain for control channel (dB) | 0 | 0 |
| (21b) H-ARQ gain for data channel (dB) | 0.5 | 0.5 |
| (22a) Receiver sensitivity for control channel = (18) + (19a) + (20) – (21a) dBm | -92.01 | -103.30 |
| (22b) Receiver sensitivity for data channel = (18) + (19b) + (20) – (21b) dBm | -90.71 | -101.82 |
| (23a) Hardware link budget for control channel = (9a) + (11) − (22a) dB | 110.01 | 123.30 |
| (23b) Hardware link budget for data channel = (9b) + (11) − (22b) dB | 108.44 | 121.82 |
| Calculation of available pathloss |  |  |
| (24) Lognormal shadow fading std deviation (dB) | 4 | 4 |
| (25) Shadow fading margin (function of the cell area reliability and (24)) (dB) | 2.8 | 2.8 |
| (26) BS selection/macro-diversity gain (dB) | 0 | 0 |
| (27) Penetration margin (dB) | 0 | 0 |
| (28) Other gains (dB) (if any please specify) | 0 | 0 |
| (29a) Available path loss for control channel = (23a) – (25) + (26) – (27) + (28) – (12) dB | 106.21 | 117.50 |
| (29b) Available path loss for data channel = (23b) – (25) + (26) – (27) + (28) – (12) dB | 104.64 | 116.02 |
| Range/coverage efficiency calculation |  |  |
| (30a) Maximum range for control channel (based on (29a) and according to the system configuration section of the link budget) (m) | 87.44 | 159.42(4) |
| (30b) Maximum range for data channel (based on (29b) and according to the system configuration section of the link budget) (m) | 80.46 | 147.37 |
| (31a) Coverage Area for control channel = (π (30a)2) (m2/site) | 24020.14 | 79846.00 |
| (31b) Coverage Area for data channel = (π (30b)2) (m2/site) | 20337.43 | 68224.73 |
| (1) Cell area reliability is defined as the percentage of the cell area over which coverage can be guaranteed. It is obtained from the cell edge reliability, shadow fading standard deviation and the path loss exponent. The latter two values are used to calculate a fade margin. Macro diversity gain may be considered explicitly and improve the system margin or implicitly by reducing the fade margin. (2) The spectral efficiency of the chosen modulation scheme. (3) The pathloss models are summarized in TABLE A1-2 of Report ITU-R M.2135. (4) InH NLoS pathloss model is only defined for less than 150m. Even though the maximum range for uplink control channel exceeds 150m, same pathloss model is used. |  |  |

Table 4 Link budget template for UMi, TDD

| Item | Downlink | Uplink |
| --- | --- | --- |
| System configuration |  |  |
| Carrier frequency (GHz) | 2.5 | 2.5 |
| BS antenna heights (m) | 10 | 10 |
| UT antenna heights (m) | 1.5 | 1.5 |
| Cell area reliability(1) (%) (Please specify how it is calculated.) | 95% | 95% |
| Transmission bit rate for control channel (bit/s) | 89,600 | 1,200 |
| Transmission bit rate for data channel (bit/s) | 7,529,917 | 212,755 |
| Target packet error rate for the required SNR in item (19a) for control channel | 10-2 | 10-2 |
| Target packet error rate for the required SNR in item (19b) for data channel | 10-1 | 10-1 |
| Spectral efficiency(2) (bit/s/Hz) for data | 0.637 | 0.720 |
| Pathloss model(3) (select from LoS or NLoS) | NLoS | NLoS |
| Mobile speed (km/h) | 3 | 3 |
| Feeder loss (dB) | 2 | 2 |
| Transmitter |  |  |
| (1) Number of transmit antennas. (The number shall be within the indicated range in Table 6 of Report ITU-R M.2135) | 4 | 2 |
| (2) Maximal transmit power per antenna (dBm) | 38 | 21 |
| (3) Total transmit power = function of (1) and (2) (dBm) (The value shall not exceed the indicated value in Table 6 of Report ITU-R M.2135) | 44 | 24 |
| (4) Transmitter antenna gain (dBi) | 17 | 0 |
| (5) Transmitter array gain (depends on transmitter array configurations and technologies such as adaptive beam forming, CDD (cyclic delay diversity), etc.) (dB) | 0 | 0 |
| (6) Control channel power boosting gain (dB) | 0 | 0 |
| (7) Data channel power loss due to pilot/control boosting (dB) | 0.2734 | 0 |
| (8) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for downlink) | 3 | 1 |
| (9a) Control channel EIRP = (3) + (4) + (5) + (6) – (8) dBm | 58 | 23 |
| (9b) Data channel EIRP = (3) + (4) + (5) – (7) – (8) dBm | 57.73 | 23 |
| Receiver |  |  |
| (10) Number of receive antennas (The number shall be within the indicated range in Table 6 of Report ITU-R M.2135) | 2 | 4 |
| (11) Receiver antenna gain (dBi) | 0 | 17 |
| (12) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for uplink) | 1 | 3 |
| (13) Receiver noise figure (dB) | 7 | 5 |
| (14) Thermal noise density (dBm/Hz) | –174 | –174 |
| (15) Receiver interference density (dBm/Hz) | -165 | -166 |
| (16) Total noise plus interference density = 10 log (10^(((13) + (14))/10) + 10^((15)/10)) dBm/Hz | -162.88 | -164.24 |
| (17) Occupied channel bandwidth (for meeting the requirements of the traffic type) (Hz) | 18904320 | 787680 |
| (18) Effective noise power = (16) + 10 log((17)) dBm | -90.11 | -105.27 |
| (19a) Required SNR for the control channel (dB) | -1.57 | -4.10 |
| (19b) Required SNR for the data channel (dB) | -1.05 | -0.96 |
| (20) Receiver implementation margin (dB) | 2 | 2 |
| (21a) H-ARQ gain for control channel (dB) | 0 | 0 |
| (21b) H-ARQ gain for data channel (dB) | 0.5 | 0.5 |
| (22a) Receiver sensitivity for control channel = (18) + (19a) + (20) – (21a) dBm | -89.68 | -107.37 |
| (22b) Receiver sensitivity for data channel = (18) + (19b) + (20) – (21b) dBm | -89.66 | -104.73 |
| (23a) Hardware link budget for control channel = (9a) + (11) − (22a) dB | 147.68 | 147.37 |
| (23b) Hardware link budget for data channel = (9b) + (11) − (22b) dB | 147.39 | 144.73 |
| Calculation of available pathloss |  |  |
| (24) Lognormal shadow fading std deviation (dB) | 4 | 4 |
| (25) Shadow fading margin (function of the cell area reliability and (24)) (dB) | 3.10 | 3.10 |
| (26) BS selection/macro-diversity gain (dB) | 0 | 0 |
| (27) Penetration margin (dB) | 0 | 0 |
| (28) Other gains (dB) (if any please specify) | 0 | 0 |
| (29a) Available path loss for control channel = (23a) – (25) + (26) – (27) + (28) – (12) dB | 143.58 | 141.27 |
| (29b) Available path loss for data channel = (23b) – (25) + (26) – (27) + (28) – (12) dB | 143.29 | 138.63 |
| Range/coverage efficiency calculation |  |  |
| (30a) Maximum range for control channel (based on (29a) and according to the system configuration section of the link budget) (m) | 1030.29 | 891.56 |
| (30b) Maximum range for data channel (based on (29b) and according to the system configuration section of the link budget) (m) | 1011.89 | 755.50 |
| (31a) Coverage Area for control channel = (π (30a)2) (m2/site) | 3334807.34 | 2497180.50 |
| (31b) Coverage Area for data channel = (π (30b)2) (m2/site) | 3216769.97 | 1793159.90 |
| (1) Cell area reliability is defined as the percentage of the cell area over which coverage can be guaranteed. It is obtained from the cell edge reliability, shadow fading standard deviation and the path loss exponent. The latter two values are used to calculate a fade margin. Macro diversity gain may be considered explicitly and improve the system margin or implicitly by reducing the fade margin. (2) The spectral efficiency of the chosen modulation scheme. (3) The pathloss models are summarized in TABLE A1-2 of Report ITU-R M.2135. |  |  |

Table 5 Link budget template for UMi, FDD

| Item | Downlink | Uplink |
| --- | --- | --- |
| System configuration |  |  |
| Carrier frequency (GHz) | 2.5 | 2.5 |
| BS antenna heights (m) | 10 | 10 |
| UT antenna heights (m) | 1.5 | 1.5 |
| Cell area reliability(1) (%) (Please specify how it is calculated.) | 95% | 95% |
| Transmission bit rate for control channel (bit/s) | 89,600 | 1,200 |
| Transmission bit rate for data channel (bit/s) | 5,849,421 | 611,334 |
| Target packet error rate for the required SNR in item (19a) for control channel | 10-2 | 10-2 |
| Target packet error rate for the required SNR in item (19b) for data channel | 10-1 | 10-1 |
| Spectral efficiency(2) (bit/s/Hz) for data | 0.619 | 0.776 |
| Pathloss model(3) (select from LoS or NLoS) | NLoS | NLoS |
| Mobile speed (km/h) | 3 | 3 |
| Feeder loss (dB) | 2 | 2 |
| Transmitter |  |  |
| (1) Number of transmit antennas. (The number shall be within the indicated range in Table 6 of Report ITU-R M.2135) | 4 | 2 |
| (2) Maximal transmit power per antenna (dBm) | 35 | 21 |
| (3) Total transmit power = function of (1) and (2) (dBm) (The value shall not exceed the indicated value in Table 6 of Report ITU-R M.2135) | 41 | 24 |
| (4) Transmitter antenna gain (dBi) | 17 | 0 |
| (5) Transmitter array gain (depends on transmitter array configurations and technologies such as adaptive beam forming, CDD (cyclic delay diversity), etc.) (dB) | 0 | 0 |
| (6) Control channel power boosting gain (dB) | 0 | 0 |
| (7) Data channel power loss due to pilot/control boosting (dB) | 0.2734 | 0 |
| (8) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for downlink) | 3 | 1 |
| (9a) Control channel EIRP = (3) + (4) + (5) + (6) – (8) dBm | 55 | 23 |
| (9b) Data channel EIRP = (3) + (4) + (5) – (7) – (8) dBm | 54.73 | 23 |
| Receiver |  |  |
| (10) Number of receive antennas (The number shall be within the indicated range in Table 6 of Report ITU-R M.2135) | 2 | 4 |
| (11) Receiver antenna gain (dBi) | 0 | 17 |
| (12) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for uplink) | 1 | 3 |
| (13) Receiver noise figure (dB) | 7 | 5 |
| (14) Thermal noise density (dBm/Hz) | -174 | -174 |
| (15) Receiver interference density (dBm/Hz) | -165.00 | -166.00 |
| (16) Total noise plus interference density = 10 log (10^(((13) + (14))/10) + 10^((15)/10)) dBm/Hz | -162.88 | -164.24 |
| (17) Occupied channel bandwidth (for meeting the requirements of the traffic type) (Hz) | 9452160 | 787680 |
| (18) Effective noise power = (16) + 10 log((17)) dBm | -93.12 | -105.27 |
| (19a) Required SNR for the control channel (dB) | -1.57 | -4.10 |
| (19b) Required SNR for the data channel (dB) | -1.68 | -1.14 |
| (20) Receiver implementation margin (dB) | 2 | 2 |
| (21a) H-ARQ gain for control channel (dB) | 0 | 0 |
| (21b) H-ARQ gain for data channel (dB) | 0.5 | 0.5 |
| (22a) Receiver sensitivity for control channel = (18) + (19a) + (20) – (21a) dBm | -92.69 | -107.37 |
| (22b) Receiver sensitivity for data channel = (18) + (19b) + (20) – (21b) dBm | -93.30 | -104.91 |
| (23a) Hardware link budget for control channel = (9a) + (11) − (22a) dB | 147.69 | 147.37 |
| (23b) Hardware link budget for data channel = (9b) + (11) − (22b) dB | 148.03 | 144.91 |
| Calculation of available pathloss |  |  |
| (24) Lognormal shadow fading std deviation (dB) | 4 | 4 |
| (25) Shadow fading margin (function of the cell area reliability and (24)) (dB) | 3.10 | 3.10 |
| (26) BS selection/macro-diversity gain (dB) | 0 | 0 |
| (27) Penetration margin (dB) | 0 | 0 |
| (28) Other gains (dB) (if any please specify) | 0 | 0 |
| (29a) Available path loss for control channel = (23a) – (25) + (26) – (27) + (28) – (12) dB | 143.59 | 141.27 |
| (29b) Available path loss for data channel = (23b) – (25) + (26) – (27) + (28) – (12) dB | 143.93 | 138.81 |
| Range/coverage efficiency calculation |  |  |
| (30a) Maximum range for control channel (based on (29a) and according to the system configuration section of the link budget) (m) | 1030.96 | 891.56 |
| (30b) Maximum range for data channel (based on (29b) and according to the system configuration section of the link budget) (m) | 1053.45 | 763.94 |
| (31a) Coverage Area for control channel = (π (30a)2) (m2/site) | 3339120.21 | 2497180.50 |
| (31b) Coverage Area for data channel = (π (30b)2) (m2/site) | 3486434.72 | 1833460.41 |

(1) 	Cell area reliability is defined as the percentage of the cell area over which coverage can be guaranteed. It is obtained from the cell edge reliability, shadow fading standard deviation and the path loss exponent. The latter two values are used to calculate a fade margin. Macro diversity gain may be considered explicitly and improve the system margin or implicitly by reducing the fade margin.

(2) 	The spectral efficiency of the chosen modulation scheme.

(3)  The pathloss models are summarized in TABLE A1-2 of Report ITU-R M.2135.

Table 6 Link budget template for UMa, TDD

| Item | Downlink | Uplink |
| --- | --- | --- |
| System configuration |  |  |
| Carrier frequency (GHz) | 2 | 2 |
| BS antenna heights (m) | 10 | 10 |
| UT antenna heights (m) | 1.5 | 1.5 |
| Cell area reliability(1) (%) (Please specify how it is calculated.) | 95% | 95% |
| Transmission bit rate for control channel (bit/s) | 89,600 | 1,200 |
| Transmission bit rate for data channel (bit/s) | 7,529,917 | 212,755 |
| Target packet error rate for the required SNR in item (19a) for control channel | 10-2 | 10-2 |
| Target packet error rate for the required SNR in item (19b) for data channel | 10-1 | 10-1 |
| Spectral efficiency(2) (bit/s/Hz) for data | 0.637 | 0.720 |
| Pathloss model(3) (select from LoS or NLoS) | NLoS | NLoS |
| Mobile speed (km/h) | 30 | 30 |
| Feeder loss (dB) | 2 | 2 |
| Transmitter |  |  |
| (1) Number of transmit antennas. (The number shall be within the indicated range in Table 6 of Report ITU-R M.2135) | 4 | 2 |
| (2) Maximal transmit power per antenna (dBm) | 43 | 21 |
| (3) Total transmit power = function of (1) and (2) (dBm) (The value shall not exceed the indicated value in Table 6 of Report ITU-R M.2135) | 49 | 24 |
| (4) Transmitter antenna gain (dBi) | 17 | 0 |
| (5) Transmitter array gain (depends on transmitter array configurations and technologies such as adaptive beam forming, CDD (cyclic delay diversity), etc.) (dB) | 0 | 0 |
| (6) Control channel power boosting gain (dB) | 0 | 0 |
| (7) Data channel power loss due to pilot/control boosting (dB) | 0.2734 | 0 |
| (8) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for downlink) | 3 | 1 |
| (9a) Control channel EIRP = (3) + (4) + (5) + (6) – (8) dBm | 63 | 23 |
| (9b) Data channel EIRP = (3) + (4) + (5) – (7) – (8) dBm | 62.73 | 23 |
| Receiver |  |  |
| (10) Number of receive antennas (The number shall be within the indicated range in Table 6 of Report ITU-R M.2135) | 2 | 4 |
| (11) Receiver antenna gain (dBi) | 0 | 17 |
| (12) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for uplink) | 1 | 3 |
| (13) Receiver noise figure (dB) | 7 | 5 |
| (14) Thermal noise density (dBm/Hz) | –174 | –174 |
| (15) Receiver interference density (dBm/Hz) | -165 | -166 |
| (16) Total noise plus interference density = 10 log (10^(((13) + (14))/10) + 10^((15)/10)) dBm/Hz | -162.88 | -164.24 |
| (17) Occupied channel bandwidth (for meeting the requirements of the traffic type) (Hz) | 18904320 | 787680 |
| (18) Effective noise power = (16) + 10 log((17)) dBm | -90.11 | -105.27 |
| (19a) Required SNR for the control channel (dB) | -1.95 | -3.97 |
| (19b) Required SNR for the data channel (dB) | -0.21 | -0.82 |
| (20) Receiver implementation margin (dB) | 2 | 2 |
| (21a) H-ARQ gain for control channel (dB) | 0 | 0 |
| (21b) H-ARQ gain for data channel (dB) | 0.5 | 0.5 |
| (22a) Receiver sensitivity for control channel = (18) + (19a) + (20) – (21a) dBm | -90.06 | -107.24 |
| (22b) Receiver sensitivity for data channel = (18) + (19b) + (20) – (21b) dBm | -88.82 | -104.59 |
| (23a) Hardware link budget for control channel = (9a) + (11) − (22a) dB | 153.06 | 147.24 |
| (23b) Hardware link budget for data channel = (9b) + (11) − (22b) dB | 151.55 | 144.59 |
| Calculation of available pathloss |  |  |
| (24) Lognormal shadow fading std deviation (dB) | 7.8 | 7.8 |
| (25) Shadow fading margin (function of the cell area reliability and (24)) (dB) | 8.1 | 8.1 |
| (26) BS selection/macro-diversity gain (dB) | 0 | 0 |
| (27) Penetration margin (dB) | 9 | 9 |
| (28) Other gains (dB) (if any please specify) | 0 | 0 |
| (29a) Available path loss for control channel = (23a) – (25) + (26) – (27) + (28) – (12) dB | 134.96 | 127.14 |
| (29b) Available path loss for data channel = (23b) – (25) + (26) – (27) + (28) – (12) dB | 133.45 | 124.49 |
| Range/coverage efficiency calculation |  |  |
| (30a) Maximum range for control channel (based on (29a) and according to the system configuration section of the link budget) (m) | 891.93 | 562.90 |
| (30b) Maximum range for data channel (based on (29b) and according to the system configuration section of the link budget) (m) | 816.07 | 481.56 |
| (31a) Coverage Area for control channel = (π (30a)2) (m2/site) | 2499264.75 | 995447.89 |
| (31b) Coverage Area for data channel = (π (30b)2) (m2/site) | 2092190.06 | 728546.98 |

(1) 	Cell area reliability is defined as the percentage of the cell area over which coverage can be guaranteed. It is obtained from the cell edge reliability, shadow fading standard deviation and the path loss exponent. The latter two values are used to calculate a fade margin. Macro diversity gain may be considered explicitly and improve the system margin or implicitly by reducing the fade margin.

(2) 	The spectral efficiency of the chosen modulation scheme.

(3)  The pathloss models are summarized in TABLE A1-2 of Report ITU-R M.2135.

Table 7 Link budget template for UMa, FDD

| Item | Downlink | Uplink |
| --- | --- | --- |
| System configuration |  |  |
| Carrier frequency (GHz) | 2 | 2 |
| BS antenna heights (m) | 10 | 10 |
| UT antenna heights (m) | 1.5 | 1.5 |
| Cell area reliability(1) (%) (Please specify how it is calculated.) | 95% | 95% |
| Transmission bit rate for control channel (bit/s) | 89,600 | 1,200 |
| Transmission bit rate for data channel (bit/s) | 5,849,421 | 611,334 |
| Target packet error rate for the required SNR in item (19a) for control channel | 10-2 | 10-2 |
| Target packet error rate for the required SNR in item (19b) for data channel | 10-1 | 10-1 |
| Spectral efficiency(2) (bit/s/Hz) for data | 0.619 | 0.776 |
| Pathloss model(3) (select from LoS or NLoS) | NLoS | NLoS |
| Mobile speed (km/h) | 30 | 30 |
| Feeder loss (dB) | 2 | 2 |
| Transmitter |  |  |
| (1) Number of transmit antennas. (The number shall be within the indicated range in Table 6 of Report ITU-R M.2135) | 4 | 2 |
| (2) Maximal transmit power per antenna (dBm) | 40 | 21 |
| (3) Total transmit power = function of (1) and (2) (dBm) (The value shall not exceed the indicated value in Table 6 of Report ITU-R M.2135) | 46 | 24 |
| (4) Transmitter antenna gain (dBi) | 17 | 0 |
| (5) Transmitter array gain (depends on transmitter array configurations and technologies such as adaptive beam forming, CDD (cyclic delay diversity), etc.) (dB) | 0 | 0 |
| (6) Control channel power boosting gain (dB) | 0 | 0 |
| (7) Data channel power loss due to pilot/control boosting (dB) | 0.2734 | 0 |
| (8) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for downlink) | 3 | 1 |
| (9a) Control channel EIRP = (3) + (4) + (5) + (6) – (8) dBm | 60 | 23 |
| (9b) Data channel EIRP = (3) + (4) + (5) – (7) – (8) dBm | 59.73 | 23 |
| Receiver |  |  |
| (10) Number of receive antennas (The number shall be within the indicated range in Table 6 of Report ITU-R M.2135) | 2 | 4 |
| (11) Receiver antenna gain (dBi) | 0 | 17 |
| (12) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for uplink) | 1 | 3 |
| (13) Receiver noise figure (dB) | 7 | 5 |
| (14) Thermal noise density (dBm/Hz) | –174 | –174 |
| (15) Receiver interference density (dBm/Hz) | -165 | -166 |
| (16) Total noise plus interference density = 10 log (10^(((13) + (14))/10) + 10^((15)/10)) dBm/Hz | -162.88 | -164.24 |
| (17) Occupied channel bandwidth (for meeting the requirements of the traffic type) (Hz) | 9452160 | 787680 |
| (18) Effective noise power = (16) + 10 log((17)) dBm | -93.12 | -105.27 |
| (19a) Required SNR for the control channel (dB) | -1.95 | -3.97 |
| (19b) Required SNR for the data channel (dB) | -0.24 | -0.59 |
| (20) Receiver implementation margin (dB) | 2 | 2 |
| (21a) H-ARQ gain for control channel (dB) | 0 | 0 |
| (21b) H-ARQ gain for data channel (dB) | 0.5 | 0.5 |
| (22a) Receiver sensitivity for control channel = (18) + (19a) + (20) – (21a) dBm | -93.07 | -107.24 |
| (22b) Receiver sensitivity for data channel = (18) + (19b) + (20) – (21b) dBm | -91.86 | -104.36 |
| (23a) Hardware link budget for control channel = (9a) + (11) − (22a) dB | 153.07 | 147.24 |
| (23b) Hardware link budget for data channel = (9b) + (11) − (22b) dB | 151.59 | 144.36 |
| Calculation of available pathloss |  |  |
| (24) Lognormal shadow fading std deviation (dB) | 7.8 | 7.8 |
| (25) Shadow fading margin (function of the cell area reliability and (24)) (dB) | 8.1 | 8.1 |
| (26) BS selection/macro-diversity gain (dB) | 0 | 0 |
| (27) Penetration margin (dB) | 9 | 9 |
| (28) Other gains (dB) (if any please specify) | 0 | 0 |
| (29a) Available path loss for control channel = (23a) – (25) + (26) – (27) + (28) – (12) dB | 134.97 | 127.14 |
| (29b) Available path loss for data channel = (23b) – (25) + (26) – (27) + (28) – (12) dB | 133.49 | 124.26 |
| Range/coverage efficiency calculation |  |  |
| (30a) Maximum range for control channel (based on (29a) and according to the system configuration section of the link budget) (m) | 892.47 | 562.90 |
| (30b) Maximum range for data channel (based on (29b) and according to the system configuration section of the link budget) (m) | 817.87 | 475.03 |
| (31a) Coverage Area for control channel = (π (30a)2) (m2/site) | 2502298.50 | 995447.89 |
| (31b) Coverage Area for data channel = (π (30b)2) (m2/site) | 2101435.73 | 708919.02 |

(1) 	Cell area reliability is defined as the percentage of the cell area over which coverage can be guaranteed. It is obtained from the cell edge reliability, shadow fading standard deviation and the path loss exponent. The latter two values are used to calculate a fade margin. Macro diversity gain may be considered explicitly and improve the system margin or implicitly by reducing the fade margin.

(2) 	The spectral efficiency of the chosen modulation scheme.

(3)  The pathloss models are summarized in TABLE A1-2 of Report ITU-R M.2135.

Table 8 Link budget template for RMa, TDD

| Item | Downlink | Uplink |
| --- | --- | --- |
| System configuration |  |  |
| Carrier frequency (GHz) | 0.8 | 0.8 |
| BS antenna heights (m) | 35 | 35 |
| UT antenna heights (m) | 1.5 | 1.5 |
| Cell area reliability(1) (%) (Please specify how it is calculated.) | 95% | 95% |
| Transmission bit rate for control channel (bit/s) | 89,600 | 1,200 |
| Transmission bit rate for data channel (bit/s) | 7,529,917 | 212,755 |
| Target packet error rate for the required SNR in item (19a) for control channel | 10-2 | 10-2 |
| Target packet error rate for the required SNR in item (19b) for data channel | 10-1 | 10-1 |
| Spectral efficiency(2) (bit/s/Hz) for data | 0.637 | 0.720 |
| Pathloss model(3) (select from LoS or NLoS) | NLoS | NLoS |
| Mobile speed (km/h) | 120 | 120 |
| Feeder loss (dB) | 2 | 2 |
| Transmitter |  |  |
| (1) Number of transmit antennas. (The number shall be within the indicated range in Table 6 of Report ITU-R M.2135) | 4 | 2 |
| (2) Maximal transmit power per antenna (dBm) | 43 | 21 |
| (3) Total transmit power = function of (1) and (2) (dBm) (The value shall not exceed the indicated value in Table 6 of Report ITU-R M.2135) | 49 | 24 |
| (4) Transmitter antenna gain (dBi) | 17 | 0 |
| (5) Transmitter array gain (depends on transmitter array configurations and technologies such as adaptive beam forming, CDD (cyclic delay diversity), etc.) (dB) | 0 | 0 |
| (6) Control channel power boosting gain (dB) | 0 | 0 |
| (7) Data channel power loss due to pilot/control boosting (dB) | 0.2734 | 0 |
| (8) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for downlink) | 3 | 1 |
| (9a) Control channel EIRP = (3) + (4) + (5) + (6) – (8) dBm | 63 | 23 |
| (9b) Data channel EIRP = (3) + (4) + (5) – (7) – (8) dBm | 62.73 | 23 |
| Receiver |  |  |
| (10) Number of receive antennas (The number shall be within the indicated range in Table 6 of Report ITU-R M.2135) | 2 | 4 |
| (11) Receiver antenna gain (dBi) | 0 | 17 |
| (12) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for uplink) | 1 | 3 |
| (13) Receiver noise figure (dB) | 7 | 5 |
| (14) Thermal noise density (dBm/Hz) | –174 | –174 |
| (15) Receiver interference density (dBm/Hz) | -165 | -166 |
| (16) Total noise plus interference density = 10 log (10^(((13) + (14))/10) + 10^((15)/10)) dBm/Hz | -162.88 | -164.24 |
| (17) Occupied channel bandwidth (for meeting the requirements of the traffic type) (Hz) | 18904320 | 787680 |
| (18) Effective noise power = (16) + 10 log((17)) dBm | -90.11 | -105.27 |
| (19a) Required SNR for the control channel (dB) | -1.19 | -2.42 |
| (19b) Required SNR for the data channel (dB) | -0.70 | 0.37 |
| (20) Receiver implementation margin (dB) | 2 | 2 |
| (21a) H-ARQ gain for control channel (dB) | 0 | 0 |
| (21b) H-ARQ gain for data channel (dB) | 0.5 | 0.5 |
| (22a) Receiver sensitivity for control channel = (18) + (19a) + (20) – (21a) dBm | -89.30 | -105.69 |
| (22b) Receiver sensitivity for data channel = (18) + (19b) + (20) – (21b) dBm | -89.31 | -103.40 |
| (23a) Hardware link budget for control channel = (9a) + (11) − (22a) dB | 152.30 | 145.69 |
| (23b) Hardware link budget for data channel = (9b) + (11) − (22b) dB | 152.04 | 143.40 |
| Calculation of available pathloss |  |  |
| (24) Lognormal shadow fading std deviation (dB) | 9.4 | 9.4 |
| (25) Shadow fading margin (function of the cell area reliability and (24)) (dB) | 10.4 | 10.4 |
| (26) BS selection/macro-diversity gain (dB) | 0 | 0 |
| (27) Penetration margin (dB) | 9 | 9 |
| (28) Other gains (dB) (if any please specify) | 0 | 0 |
| (29a) Available path loss for control channel = (23a) – (25) + (26) – (27) + (28) – (12) dB | 131.90 | 123.29 |
| (29b) Available path loss for data channel = (23b) – (25) + (26) – (27) + (28) – (12) dB | 131.64 | 121.00 |
| Range/coverage efficiency calculation |  |  |
| (30a) Maximum range for control channel (based on (29a) and according to the system configuration section of the link budget) (m) | 2361.39 | 1412.39 |
| (30b) Maximum range for data channel (based on (29b) and according to the system configuration section of the link budget) (m) | 2323.96 | 1232.23 |
| (31a) Coverage Area for control channel = (π (30a)2) (m2/site) | 17517973.59 | 6267014.62 |
| (31b) Coverage Area for data channel = (π (30b)2) (m2/site) | 16967146.48 | 4770131.41 |

(1) 	Cell area reliability is defined as the percentage of the cell area over which coverage can be guaranteed. It is obtained from the cell edge reliability, shadow fading standard deviation and the path loss exponent. The latter two values are used to calculate a fade margin. Macro diversity gain may be considered explicitly and improve the system margin or implicitly by reducing the fade margin.

(2) 	The spectral efficiency of the chosen modulation scheme.

(3)  The pathloss models are summarized in TABLE A1-2 of Report ITU-R M.2135.

Table 9 Link budget template for RMa, FDD

| Item | Downlink | Uplink |
| --- | --- | --- |
| System configuration |  |  |
| Carrier frequency (GHz) | 0.8 | 0.8 |
| BS antenna heights (m) | 35 | 35 |
| UT antenna heights (m) | 1.5 | 1.5 |
| Cell area reliability(1) (%) (Please specify how it is calculated.) | 95% | 95% |
| Transmission bit rate for control channel (bit/s) | 89,600 | 1,200 |
| Transmission bit rate for data channel (bit/s) | 5,849,421 | 611,334 |
| Target packet error rate for the required SNR in item (19a) for control channel | 10-2 | 10-2 |
| Target packet error rate for the required SNR in item (19b) for data channel | 10-1 | 10-1 |
| Spectral efficiency(2) (bit/s/Hz) for data | 0.619 | 0.776 |
| Pathloss model(3) (select from LoS or NLoS) | NLoS | NLoS |
| Mobile speed (km/h) | 120 | 120 |
| Feeder loss (dB) | 2 | 2 |
| Transmitter |  |  |
| (1) Number of transmit antennas. (The number shall be within the indicated range in Table 6 of Report ITU-R M.2135) | 4 | 2 |
| (2) Maximal transmit power per antenna (dBm) | 40 | 21 |
| (3) Total transmit power = function of (1) and (2) (dBm) (The value shall not exceed the indicated value in Table 6 of Report ITU-R M.2135) | 46 | 24 |
| (4) Transmitter antenna gain (dBi) | 17 | 0 |
| (5) Transmitter array gain (depends on transmitter array configurations and technologies such as adaptive beam forming, CDD (cyclic delay diversity), etc.) (dB) | 0 | 0 |
| (6) Control channel power boosting gain (dB) | 0 | 0 |
| (7) Data channel power loss due to pilot/control boosting (dB) | 0.2734 | 0 |
| (8) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for downlink) | 3 | 1 |
| (9a) Control channel EIRP = (3) + (4) + (5) + (6) – (8) dBm | 60 | 23 |
| (9b) Data channel EIRP = (3) + (4) + (5) – (7) – (8) dBm | 59.73 | 23 |
| Receiver |  |  |
| (10) Number of receive antennas (The number shall be within the indicated range in Table 6 of Report ITU-R M.2135) | 2 | 4 |
| (11) Receiver antenna gain (dBi) | 0 | 17 |
| (12) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for uplink) | 1 | 3 |
| (13) Receiver noise figure (dB) | 7 | 5 |
| (14) Thermal noise density (dBm/Hz) | –174 | –174 |
| (15) Receiver interference density (dBm/Hz) | -165 | -166 |
| (16) Total noise plus interference density = 10 log (10^(((13) + (14))/10) + 10^((15)/10)) dBm/Hz | -162.88 | -164.24 |
| (17) Occupied channel bandwidth (for meeting the requirements of the traffic type) (Hz) | 9452160 | 787680 |
| (18) Effective noise power = (16) + 10 log((17)) dBm | -93.12 | -105.27 |
| (19a) Required SNR for the control channel (dB) | -1.19 | -2.42 |
| (19b) Required SNR for the data channel (dB) | -1.06 | 1.03 |
| (20) Receiver implementation margin (dB) | 2 | 2 |
| (21a) H-ARQ gain for control channel (dB) | 0 | 0 |
| (21b) H-ARQ gain for data channel (dB) | 0.5 | 0.5 |
| (22a) Receiver sensitivity for control channel = (18) + (19a) + (20) – (21a) dBm | -92.31 | -105.69 |
| (22b) Receiver sensitivity for data channel = (18) + (19b) + (20) – (21b) dBm | -92.68 | -102.74 |
| (23a) Hardware link budget for control channel = (9a) + (11) − (22a) dB | 152.31 | 145.69 |
| (23b) Hardware link budget for data channel = (9b) + (11) − (22b) dB | 152.40 | 142.74 |
| Calculation of available pathloss |  |  |
| (24) Lognormal shadow fading std deviation (dB) | 9.4 | 9.4 |
| (25) Shadow fading margin (function of the cell area reliability and (24)) (dB) | 10.4 | 10.4 |
| (26) BS selection/macro-diversity gain (dB) | 0 | 0 |
| (27) Penetration margin (dB) | 9 | 9 |
| (28) Other gains (dB) (if any please specify) | 0 | 0 |
| (29a) Available path loss for control channel = (23a) – (25) + (26) – (27) + (28) – (12) dB | 131.91 | 123.29 |
| (29b) Available path loss for data channel = (23b) – (25) + (26) – (27) + (28) – (12) dB | 132.00 | 120.34 |
| Range/coverage efficiency calculation |  |  |
| (30a) Maximum range for control channel (based on (29a) and according to the system configuration section of the link budget) (m) | 2362.84 | 1412.39 |
| (30b) Maximum range for data channel (based on (29b) and according to the system configuration section of the link budget) (m) | 2375.49 | 1184.79 |
| (31a) Coverage Area for control channel = (π (30a)2) (m2/site) | 17539513.53 | 6267014.62 |
| (31b) Coverage Area for data channel = (π (30b)2) (m2/site) | 17727874.98 | 4409968.99 |

(1) 	Cell area reliability is defined as the percentage of the cell area over which coverage can be guaranteed. It is obtained from the cell edge reliability, shadow fading standard deviation and the path loss exponent. The latter two values are used to calculate a fade margin. Macro diversity gain may be considered explicitly and improve the system margin or implicitly by reducing the fade margin.

(2) 	The spectral efficiency of the chosen modulation scheme.

(3)  The pathloss models are summarized in TABLE A1-2 of Report ITU-R M.2135.

- References

[1] REPORT ITU-R M.2133, “Requirements, evaluation criteria and submission templates for the development of IMT-Advanced.”

[2] IEEE 802.16m-09/0047, “System Evaluation Details for IEEE 802.16 IMT-Advanced Proposal.”

[3] REPORT ITU-R M.2135, “Guidelines for evaluation of radio interface technologies for IMT-Advanced.”