| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on Working Draft for 802.16n System Requirements Document |  |
| Date Submitted | 2010-08-31 |  |
| Source(s) | Youngbin Chang, Yeongmoon Son, Jaehyuk Jang, Kyungkyu Kim, Rakesh Taori Samsung Electronics | yb.chang@samsung.com ym1004.son@samsung.com rakesh.taori@samsung.com |
| Re: | Call for comments on 802.16n SRD (80216gman-10_0024r2) |  |
| Abstract | This document includes comments on the 802.16n SRD. |  |
| Purpose | Adopt comments on 802.16n SRD ( |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Overview

The 802.16n amendment shall be developed in accordance with the P802.16 project authorization request (PAR) [1] with its accompanying Five Criteria Statement.

The standard shall be developed as an amendment to 802.16. The resulting standard shall fit within the following scope:

This amendment specifies protocol enhancements to the IEEE 802.16 MAC for enabling increased robustness and alternate radio path establishment in degraded network conditions. Limited OFDMA PHY extensions are included for enabling operation with radio path redundancy and direct communication between subscriber stations. Also mobile base stations and mobile relay stations are supported. Support for enabling application specific specialized security suites is also provided.

With the explanatory note:

Operation in licensed, unlicensed and lightly licensed spectrum bands below 6 GHz with means and mechanisms to coexist with other radio access technologies (RATs) is supported.

This document represents the high-level system requirements for the 802.16n amendment.  All content included in any draft of the 802.16n amendment shall meet these requirements. This document, however, shall be maintained and may evolve. These system requirements embodied herein are defined to ensure competitiveness of the amended standard against other mobile broadband radio access technologies in those areas defined by the PAR and Five Criteria Statement. These system requirements also call for significant gains and improvements relative to the preexisting IEEE 802.16 system that would justify the creation of the amendment. To accelerate the completion and evaluation of the standard, to improve the clarity and reduce complexity of the standard specification, and to further facilitate the deployment of new systems, the number of optional features should be minimized.

# References

- IEEE P802.16n PAR and 5C in 80216gman-10_0018r2.doc

- IEEE P802.16m System Requirements Document (SRD)

# Definitions

|  | The definitions below have been agreed in the PAR |
| --- | --- |
| Degraded Network | The failure of one or more 802.16 network infrastructure nodes or network connectivity |
| Robustness | The capability of the network to withstand and automatically recover from degradation to provide the required availability to support mission critical applications (essential to the core function of society and the economy). E.g. the ability to recover from a single point of failure |
| Mobile Base Station | A base station which is capable of maintaining service while moving |
| Radio Path Redundancy | The ability to provide alternative paths between base stations, relay stations, and subscriber stations |
|  | Other definitions |
| HR-MS | A subscriber station that complies with the requirements for subscriber stations in this amendment |
| HR-BS | A base station that complies with the requirements for base stations in this amendment |
| HR-RS | A relay that complies with the requirements for relays in this amendment |
| HR-network | A network whose nodes comply with their respective HR requirements in this amendment |
| HR-station | A station served by HR-Network |
| Minimal QoS | The combination of data rate, latency and other relevant parameters that are considered necessary to establish a radio link |
| Radio Connectable | The ability of any two nodes to communicate directly at an acceptable certain minimal QoS |
| Infrastructure station | An HR-BS or HR-RS |

# Abbreviations and Acronyms

| SPOF | Single point of failure |
| --- | --- |

Table of Contents

1 Overview	1

2 References	1

3 Definitions	1

4 Abbreviations and Acronyms	2

5 General Requirements	6

5.1 Compatibility with other 802.16 equipment	6

5.2 Complexity	6

5.3 Services	6

5.4 Operating Frequencies	6

5.5 Operating bandwidths	6

5.6 Duplex schemes	6

5.7 Support for Government mandates and public safety	6

6 Functional Requirements	7

6.1 Requirements related to construction and maintenance of network	7

6.1.1 General	7

6.1.2 Requirements related to multi-mode operation	7

6.1.2.1 Relay function for HR-BS	7

6.1.2.2 Relay function for HR-MS (RS Mode)	7

6.1.2.3 Base Station function for HR-MS (BS Mode)	7

6.1.2.4 Topology for Multi-Mode	7

6.1.3 Requirements related to infrastructure SPOF immunity	7

6.1.3.1 Inter-cell relaying	8

6.1.3.2 Standalone networks (No connection to core network. Includes both cases: Inter-BS connectivity, and no inter-BS connectivity)	8

6.1.3.3 Multi-hop relay	8

6.1.3.4 Shared relay (a RS is communicating with multiple BS)	8

6.1.3.5 HR-RS Sourcing and Sinking of Data	9

6.1.4 Requirements related to link reliability	9

6.1.5 Requirements related to link existence	9

6.1.5.1 MS to MS Direct Communication	9

6.1.5.2 Multi Path Routing	10

6.1.5.3 Link Redundancy	10

6.1.5.4 Local Forwarding for RS and BS	10

6.1.5.5 Neighbor Discovery	10

6.1.5.6 MS-MS association establishment (not including service flows)	10

6.1.6 Requirements related to mobility	10

6.1.6.1 Mobile Base Stations	10

6.1.6.2 Mobile relay stations	10

6.1.7 Requirements related to security	10

6.1.7.1 New security procedures for MS-MS direct communication	11

6.1.7.2 Group Key Management	11

6.1.8 Coexistence requirements	11

6.1.8.1 Operation in unlicensed and lightly licensed bands	11

6.1.8.2 Support for Multi-carrier operation in different licensing regimes	11

6.2 Requirements related to Services provided on network	12

6.2.1 Point to Multipoint data transfer	12

6.2.1.1 Enhancements to Multicast and Broadcasting	12

6.2.1.2 Group call 	12

6.2.1.3 Two-way group call (mobile to mobile endpoints: virtual path)	12

6.2.2 Multipoint to Multipoint data transfer	12

6.2.3 Improved services	12

6.2.3.1 Enhanced VoIP service	12

6.2.3.2 Data services (divided into short packet and normal packet)	12

6.2.4 Sensing and reporting	13

6.3 Requirements related to flow/connection management	13

6.3.1 Large volumes of data flow control	13

6.3.2 High frequency of data exchange	13

6.3.3 Real-time connection control	13

6.3.4 Non-real time connection control	13

6.4 MAC Layer Routing	13

<TBD>Performance Requirements	13

Performance Requirements	14

6.5 General	14

6.6 Requirements on network robustness	14

6.6.1 Recovery time from HR-BS backhaul failure	14

6.6.2 Recovery time from HR-BS failure	14

6.6.3 Recovery time from HR-RS failure	14

6.7 Mobile to mobile performance requirements	14

6.7.1 Peak throughput	14

6.7.2 Cell-average throughput	14

6.8 Cell coverage	14

6.8.1 Cell coverage reliability with mobile relaying	14

6.9 Cell average throughput	14

6.9.1 Cell average throughput with MS-MS relaying	14

6.9.2 Cell average throughput with local traffic	14

6.10 Cell edge throughput	15

6.10.1 Cell edge throughput with MS-MS relaying	15

6.10.2 Cell edge throughput with local traffic	15

6.11 Node discovery performance	15

6.11.1 Node discovery success rate	15

6.11.1.1 MS-MS node discovery	15

6.11.1.2 Other nodes	15

6.11.2 Node discovery time	15

6.12 Direct connection establishment performance	15

6.12.1 Direct connection establishment time between MS	15

6.12.2 Direct connection establishment time between base stations and relays	15

6.13 Traffic performance	15

6.13.1 Peak spectral efficiency	15

6.13.2 Link Throughput	15

6.13.3 End-to-end Throughput	15

6.14 Latency	15

6.14.1 Link latency	15

6.14.2 End-to-end latency	15

6.15 QoS	15

6.16 Security	16

6.17 Mobility	16

6.18 Reliability peformance	16

6.18.1 Fault detection time	16

6.18.2 Fault healing/isolation time	16

6.18.3 Failure detection time	16

6.18.4 Recovery/Self-healing time from failure	16

6.18.5 Restoration time	16

6.18.6 Path routing time	16

6.19 Multicast Performance	16

7 Operational Requirements	16

List of Figures

Figure 1: HR-Network robustness examples (informative)	7

Figure 2: Illustration of MS-MS topologies (Informative).	9

# General Requirements

## Compatibility with other 802.16 equipment

<tbd>

## Complexity

IEEE 802.16n amendment should minimize complexity of the architecture and protocols and avoid excessive system complexity.  It should enable interoperability of access networks, support low cost devices and minimize total cost of ownership.

## Services

IEEE 802.16n should support services that require a higher degree of assurance of maintaining sufficient connectivity than can be provided by IEEE 802.16 legacy systems.  Examples of such services can be found in PPDR (Public Protection and Disaster Recovery) and M2M (Machine to Machine) communication networks for utility monitoring and control.

## Operating Frequencies

The HR-Network shall be specified to allow operation in all radio frequencies where 802.16 operates.

The HR-Network shall also be specified to allow operation in unlicensed and lightly licensed spectrum bands below 6 GHz with means and mechanisms to coexist with other radio access technologies (RATs)

## Operating bandwidths

The HR-Network shall be specified to allow scalable bandwidth, from 5 MHz to 40 MHz. This bandwidth may be supported by single or multiple RF carriers. Other bandwidth shall be considered as necessary to meet the operator requirements.

## Duplex schemes

The HR-Network shall be specified to support TDD and FDD. {Not every feature is necessarily common to both TDD and FDD}

# Functional Requirements

HR-Network devices shall comply with all Advanced Air Interface or 802.16 OFDMA functional requirements unless indicated below. Additional functional requirements are indicated in this section.

## Requirements related to construction and maintenance of network

This section contains requirements for IEEE 802.16n related to construction and maintenance of network. These requirements are intended to address multi-mode operation, link existence, infrastructure SPOF immunity, link reliability, mobility, security, and coexistence

### Requirements related to multi-mode operation

An ability to dynamically change roles shall be included within 802.16n...

#### Relay function for HR-BS

p

When the backhaul of an HR-BS is disrupted, the HR-BS shall continue to provide network connectivity to the HR-MSs by using neighboring HR-BS’s backhaul.

#### Relay function for HR-MS (RS Mode)

shall proon n

In case of HR-BS failure, HR-Network shall support an HR-MS, connected to a functioning HR-BS, to relay data from other HR-MSs connected to a disrupted HR-BS.

#### shall proH multimod RS Mode MS, and MS. IEEE 802.16nHRNrk sn HR-Netwo RS-mode MSs move

### Requirements related to infrastructure SPOF immunity

An HR-network shall be able to recover from any single point failure in any of its infrastructure nodes (i.e. all nodes excluding the subscriber station) or any of its radio links.

#### Inter-cell relaying

The HR-network shall support inter-cell relaying. In case of a failure of HR-BS backhaul, the HR-BS shall be able to maintain service to its HR-MS by conveying user data to and from another HR-BS.

#### Standalone network (No connection to core network. Includes both cases: Inter-BS connectivity, and no inter-BS connectivity)

IEEE 802.16n should provide that the BS provides connectivity to the MSs within the coverage of the BS without BS’s connectivity to the backbone network.

When the BS loses the backbone connection, the established service flow between MSs within the coverage of the BS should be maintained.

#### Multi-hop relaying for HR-MS

HR-MS may have a multi-hop relaying function

### Requirements related to link existence and reliability

#### MS to MS Direct Communication

The HR-MS shall provide direct communication; defined as communication among HR-MSs without the data passing through any other HR-station. The control signaling and the data transmission for the direct communication shall at least be capable of operating within the frequency band that BSs operate.  MS to MS direct communication shall only take place in the absence of infrastructure station.  MS-MS association establishment procedure shall be supported.

#### Multi Path Routing

HR-Network shall support pre-establishment of alternative paths to support fast recovery in the event of disruption. Stations having a routing functionality should choose the most reliable paths when the station performs HR-network entry.

#### Link Redundancy

HR-Network may provide multiple communication links between two HR-stations.

#### Local Forwarding for RS and BS

HR-Network should allow local forwarding, which allows one HR-MS to communicate to another HR-MS via infrastructure station without going through the backhaul.

#### Neighbor Discovery

The HR-Network shall support the HR-MS discovery by other HR-MS, defined here as the capability to establish the presence and capabilities of other HR-MS, including their connectivity status to other nodes. Discovery can be accomplished with or without network support.

### Requirements related to mobility

#### Mobile Base Stations

#### Mobile relay stations

### Requirements related to security

The HR-Network topology shall not degrade the security performance achieved with AAI.

#### New security procedures for HR-Networks

HR-Network shall support secure communication among HR-stations, and between HR-station and external AAA-servers

an

Autonomous (limited) mutual authentication of HR-MS and data security

HR-MSs shall be able to mutually authenticate themselves without access to a security server.

HR-MSs shall be able to establish encrypted communication without access to a security server.

##### Security requirements for HR-station acting as relays

HR-station that function as relay shall pass security related messages between other HR-station and a security server, both during security association establishment and ongoing communications.

Any HR-station, acting as a relay shall be able to attest to the supplicant that it has forwarded the data as requested.

### Coexistence requirements

#### Operation in unlicensed and lightly licensed bands

HR devices shall comply with regulators’ respective requirements for operation in unlicensed and lightly licensed spectrum.

#### Support for Multi-carrier operation in different licensing regimes

The HR-Network shall support multicarrier operation in licensed, unlicensed and lightly-licensed licensing environments.

An HR-MS that supports MC operation in different licensing environments shall be able of operating in all three types of spectrum at the same time.

## Requirements related to Services provided on network

HR-Network shall support both Point-to-point and Point-to-Multipoint transmission services which are described in this section..

### Enhanced  Multicast and Broadcasting

HR-Network shall provide multicast and broadcast transmission with reduced signaling overhead..

HR-Network shall provide support for enhanced power saving functionality of multicast and broadcast transmission to help reduce power consumption in devices.

HR-Network shall provide MAC level traffic encryption to bursts on the multicast and broadcast connection.

### Group call

IEEE 802.16n shall provide support for group call, including two-way communication

Multiple groups shall be supported, with each group having a unique identity.

The group should be organized dynamically.

Each MR-MS may belong to multiple groups

Each group shall have a priority value

IEEE 802.16n shall support for multicast and broadcast transmission of signaling messages to the group members at a time.

### Two-way group call (mobile to mobile endpoints: virtual path)

IEEE 802.16n shall provide the group call and the PTT-like half-duplex communication in the direct communication.

### Enhanced VoIP service

IEEE 802.16n shall support Emergency Call

### Data services (divided into short packet and normal packet)

IEEE 802.16n shall maintain the QoS data service of 802.16 to the best extent possible during reconfigurations of infrastructure due to disruption.

#### 3 Pusht-Talk (PTT) service

IEEE 802.16n shall provide PTT service, which is a two-way form of communications that allows the HR_MS to engage in communication with one or more HR-MSs (i.e., Point to Point data transfer or Point to Multipoint data transfer).

Applications to be used in PTT service are as follows:

- audio (e.g., speech, music)

- video

- still image

- text (formatted and non-formatted)

- file

### Sensing and reporting

The HR-network shall support applications with real time sensing and reporting functions.

## Requirements related to flow/connection management

As the HR-Network is reconfigured to support disruptions, the network needs to maintain the following requirements

### Large volumes of data flow control

HR-Network shall provide functionality to control large volume of data.

### High frequency of data exchange

HR-Network should provide functionality to control high frequency of data exchange efficiently.

### Real-time connection control

HR-Network shall support real-time data and control message connection.

### Non-real time connection control

HR-Network shall support non-real-time data and control message connection.

## MAC Layer Routing

<TBD>

# Performance Requirements

## General

## Requirements on network robustness

A subscriber node shall be considered always serviceable if there exists at least one node that can establish an alternative path of acceptable QoS between that node and a messaging correspondent node.

## Multicast Performance

HR-Network shall provide support for Multicast and Broadcasting for at least 10000 people in one connection.

HR-Network shall support a number of multicast groups that is 10 (connections / MHz / sector).

# Operational Requirements