| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Propose to Keep the Function Requirement of Allowing Relay Source and Sink Data |  |
| Date Submitted | 2010-09-13 |  |
| Source(s) | Haiguang Wang, Jaya Shankar, Joseph Teo Chee Ming Institute for Infocomm Research | hwang@i2r.a-star.edu.sg jshankar@i2r.a-star.edu.sg cmteo@i2r.a-star.edu.sg |
| Re: | 802.16n requirements |  |
| Abstract | This document proposes text changes to 802.16n SRD. |  |
| Purpose | Discussion and inclusion |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Propose to Keep the Function Requirement of Allowing Relay Source and Sink Data

Haiguang Wang, Jaya Shankar and Joseph Teo Chee Ming

Institute for Infocomm Research (I2R)

#### In the maritime safety communication environment as stated in the 802.16n PAR, a base station will be deployed on the shore and ships on the sea may equip with a relay station on each of them and relay data for each other. At the same time, each ship may also need send and receive data to and from base station. Therefore, it is necessary to allow the relay station to source and sink data. However, with the current standard, there is no statement to address how BS station will handle the Dynamic Service Requests (DSA), Dynamic Service Change (DSC) and Dynamic Service Delete (DSD) from relay station.

#### One possible method to implement this feature is to logically implement the MS and RS functionality in the same device and enable both instances at the same time although the device operates as an RS to all other device. However, we believe that this will introduce a non-standard approach as this would introduce many other challenges such as MS association and internal communication with the RS. Resource allocation will also be an issue for this logical MS. It may not be optimal in resource allocation when the relay station is an 802.16j RS. Finally, in our view, the required change to the existing standard is minor and it would be simpler to implement a RS stack that source and sinks the data without requiring a device to unnecessarily run a MS stack.

#### Therefore, we propose to keep the requirement as stated in 802.16n system requirement document, C80216gman-10_0024r3.doc, in section, 6.1.2.4 HR-RS Sourcing and Sinking of Data, which allow relay stations to source and sink data with the 802.16n standard amendment.

Proposed text (to keep the requirement on relay source and sink data as follows)

==============================================================================

# Functional Requirements

## Requirements related to construction and maintenance of network

### General

### Requirements related to multi-mode operation

#### Relay function for HR-BS

#### Relay function for HR-MS (RS Mode)

#### Base Station function for HR-MS (BS Mode)

#### Topology for Multi-Mode

### Requirements related to infrastructure SPOF immunity

#### Inter-cell relaying

#### Standalone networks (No connection to core network. Includes both cases: Inter-BS connectivity, and no inter-BS connectivity)

#### Multi-hop relay

#### Shared relay (a RS is communicating with multiple BS)

#### HR-RS Sourcing and Sinking of Data

HR-RS shall be allowed to source and sink data besides relaying data that do not belong to itself. In many scenarios, Relay Stations themselves need to consume data locally..