| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modifications of Security Requirements in 802.16n System Requirements Document |  |
| Date Submitted | 09/13/10 |  |
| Source(s) | Joseph Teo Chee Ming, Jaya Shankar, Haiguang Wang Institute for Infocomm Research | E-mail: cmteo@i2r.a-star.edu.sg |
| Re: | Call for Contributions for P802.16n System Requirements Document : IEEE 802.16gman-10/0024r3 |  |
| Abstract | This provides proposed texts to update 802.16n System Requirements Document and reasons to retain the Group Key management in existing draft. |  |
| Purpose | To be discussed and adapted by TGn for draft 802.16n SRD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Modifications of Security Requirements in 802.16n System Requirements Document

Joseph Teo Chee Ming, Jaya Shankar, Haiguang Wang

Institute for InfoComm Research

# Introduction

The current draft IEEE 802.16gmain-10_0024r3 [1] discusses on multicast support and point-to-Multipoint/multipoint-multipoint data transfer (e.g. Group call, push-to-Talk etc). Some of these applications require secure communications between the stations. Furthermore, in some sensitive applications (e.g. Military or private business), the data in these applications has to be protected (confidentiality and authenticated).

Group Key Management is used to establish keys for secure group communications, especially in applications such as secure multicast and secure group data transfer (secure point to multipoint and multipoint to multipoint data transfer. It is therefore necessary to provide some form of Group Key Management in the IEEE 802.16n standard to enable support for secure multicast and secure group data transfer applications.

As such, we would like to propose to keep Section 6.4.6.2 Group Key Management of the current draft IEEE 802.16gmain-10_0024r3.doc [1].

In addition, we would like to propose the addition of the follow text into the Section 6.8: Performance Requirement  of the current draft [1].

# Text Proposal for draft 802.16n SRD

--------------Start of Proposed Text---------------------------------------------------------------------

6.8 Performance Requirement

6.20	Security

    - Security Association (SA) Establishment (Authentication and Key Establishment) 	time (Signaling) for MS-MS direct 	communication

MSs shall authenticate and establish their session key within a certain specified time for secure MS-MS direct communications.

6.20.1.1 Security Association (SA) establishment time with Security Server

Two MSs shall establish the SA (with the help of a Security Server) within a specified 			time.

6.20.1.2 Security Association (SA) establishment time without Security Server

Two MSs shall establish the SA (without the help of a Security Server) within a specified 			time.

6.20.1.3 Security Association (SA) expiration time

The SA for secure MS-MS direct communication should have a expiration time.

    - Group Key Management

The HR-network shall authenticate and establish a group key within a specified time for 			secure group communications.

      - Initial Group Key establishment time

The HR-network shall establish the initial group key within a specified time.

      - Dynamic Group Key Establishment (Rekeying) time

The group key has to be changed/updated whenever dynamic group events such as new 			BS/RS/BS joining the group or BS/RS/MS leaving the group. The group key establishment time 		for such operations has to be within a specified time.

        - BS/RS/MS Join Group Event

The group key has to be changed/updated within a specified time whenever a BS/RS/MS 			joins the network.

        - BS/RS/MS Leave Group Event

The group key has to be changed/updated within a specified time whenever a BS/RS/MS 			leaves the network.

--------------End of Proposed Text ---------------------------------------------------------------------

# References

- IEEE 802.16gmain-10_0024r3, “Working Draft for 802.16n System Requirements Document,” 12 September 2010.