| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text for 802.16n System Architecture Reference Model |  |
| Date Submitted | 2010-11-10 |  |
| Source(s) | Eldad Zeira InterDigital Communications LLC | Eldad.zeira@interdigital.com |
| Re: | CfC for 802.16n SARM |  |
| Abstract | This document proposes text for the 802.16n SARM. |  |
| Purpose | To be discussed and decided by GRIDMAN. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed text for 802.16n System Architecture Reference Model

Eldad Zeira, InterDigital

# Discussion

In session #69 in St. Petersburg GRIDMAN has discussed the contents of the 802.16n SARM. The following content has been agreed (see 80216gman-10_0039r1):

- Overall Network architecture model(s) – describing network entities and their links for e.g.

  - MS-MS communications

  - Relays

- System reference model

- Protocol structure e.g. for

  - MS-MS communications

  - Security

- Operational states (if different from current 802.16)

- Specific for 2009+802.16j / 802.16m

- Specific for feature(s)

This document proposes SARM text for these items.

# >>>>>>>>> Proposed Text <<<<<<<<<<<

# Network reference model

## Over-all Network reference model

The Network Reference Model (NRM) is a logical representation of the network architecture. The NRM identifies functional entities and reference points over which interoperability is achieved between functional entities. Error: Reference source not found illustrates the NRM of HR-Network , consisting of the following functional entities:

- Mobile Station (MS) which signifies 802.16m or WirelessMAN-OFDMA compatible subscriber station

- HR-MS, HR-RS and HR-BS which are compatible with 802.16n requirements

Figure 1: HR-Network Reference Model; Numbers referenced in the text below

Legend:

R1 refers to "legacy" baseline interface which could be either WirelessMAN-OFDMA (2009) including 802.16j or 802.16m. Cases 1 and 2 are therefore not new to this amendment and are brought here for reference only.

Cases 3 & 4 represent the multi-mode feature of HR-Network. Note that legacy interface is used.

Case 5 defines a new interface between HR-MS, denoted R10. It is based on HR-MS with limited relaying functionality. R10 cannot serve legacy MS. The interface between the HR-MS in line 6 and the HR-BS is legacy, however due to the need to support the HR-MS to HR-MS connection it is denoted R10’.

Cases 6 and 7 represent a new interface, denoted R11, required for HR-MS to HR-MS direct communications in the absence of infrastructure (single and dual hop cases).

## Interoperability table for Multi-mode operation

The HR-Network supports role changes of some network entities to support recovery from network entity or backhaul failure. Table 1 summarizes the role changes and resulting connectivity.

Table 1:  Interoperability Table

| Original entities | e-HR-MS | m-HR-MS | e-HR-MS | m-HR-MS | e-HR-BS | m-HR-BS | Connected to… |
| --- | --- | --- | --- | --- | --- | --- | --- |
| New Roles | Relay | Relay | BS | N/A | Relay | Relay |  |
|  |  | AAI |  |  | - | AAI | m-HR-BS |
|  | OFDMA |  |  |  | OFDMA | - | e-HR-BS |
|  |  | AAI |  |  | - | AAI | m-BS |
|  | OFDMA |  |  |  | OFDMA |  | e-BS |
|  |  |  |  |  | - |  | m-HR-RS |
|  | OFDMA (multihop) |  | OFDMA |  | OFDMA (multihop) |  | e-HR-RS |
|  |  |  |  |  | - |  | m-RS |
|  | OFDMA (multihop) |  | OFDMA |  | OFDMA (multihop) |  | e-RS |
|  |  | AAI |  |  |  | AAI | m-HR-MS |
|  | OFDMA |  | OFDMA |  | OFDMA | - | e-HR-MS |
|  |  | AAI |  |  |  | AAI | m-MS |
|  | OFDMA |  | OFDMA |  | OFDMA |  | e-MS |

Legend:

- OFDMA is shorthand for WirelessMAN-OFDMA (2009) + 802.16j

- AAI stands for 802.16m

- An e-HR-xS or m-HR-xS stand for HR-xS node (where xS could be MS, BS or RS) that are based on WirelessMAN-OFDMA or AAI respectively.

# IEEE 802.16n System Reference Model

## Multi-mode operation

This section shows the system reference model for HR-BS and HR-MS role changes. An example is given in Figure 2. The Role Change Management block communicates with other network nodes. It interfaces with the MAC e.g. to request and receive reports of various measurements. Finally it causes the switching of functionality as specified in the SRD e.g. between HR-MS to HR-RS. Note that the role change management and switching mechanism are outside of the scope of HR-Network specifications and are brought here for illustrative purposes only.

Figure 2 : System reference model for HR-MS and HR-BS role change

## System reference model for other modes

The System reference model for modes other than multi-mode devices is unchanged from 802.16m system reference model.

# HR-MS State Diagrams

## States of HR-MS

Figure 3 depicts the states of an HR-MS. Note the distinct separate states of HR-MS that HR-MS enters depending if it is associated with an infrastructure node (BS or relay), with another HR-MS without presence of infrastructure (direct MS-MS) or to infrastructure through a forwarding HR-MS.

Note that direct MS-MS mode does not include Access or Idle states. The former is provided by the HR-MS forwarding mode while the other isn't necessary for this mode.

Figure 3: States of an HR-MS

## Initialization State

Figure 4 depicts the initialization stage for an HR-MS. During this stage the HR-MS first attempts to find a BS or relay in the normal manner (i.e. same as baseline). If one is found, the HR-MS proceeds to the corresponding access state for the baseline interface.

If none is found, the HR-MS attempts to find another HR-MS to forward its data and proceeds to access state with that neighbor.

Figure 4 : Initialization State

## Access States

Access states are depicted in Figure 5.

Access state with infrastructure nodes is identical to the corresponding legacy mode (WirelessMAN-OFDMA (2009) or AAI).

Access state to HR-MS connected mode includes:

- Basic access to an HR-MS found in the initialization state.

- Network information exchange

- If the network does not include a BS then HR-MS switches to connected state for MS-MS direct communication (no BS).

- Otherwise a link to infrastructure node is established through the forwarding HR-MS.

Figure 5: Access State Procedures for non-infrastructure modes

## Connected States

Connected states for infrastructure and HR-MS forwarding modes are identical to legacy.

There is no connected state for direct communication mode.

## Idle States

Idle states for infrastructure and HR-MS forwarding modes are identical to legacy.

There is no idle state for direct communication mode.

# IEEE 802.16n High Reliability Network Protocol Structure

## Multi-mode operation

HR-Network stations protocol structure is identical to the baseline protocol.

## MS-MS direct communication and HR-MS forwarding protocol structure

FFS.